IALA Recommendation

R0139 (O-139)

The Marking of man-made offshore structures

Edition 2.1

December 20xx

Revisions to this IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |
| December 2013 | Entire document | Due to the technological and operational developments that occurred in the offshore structures activities in the sea, it was necessary to update this recommendation in order to respond to the identified marking. |
| October 2019 | Entire Document | Converted text to new IALA format. Edited entire document including adding new section 2.6 on Breakwaters. |

THE IALA COUNCIL

RECALLING:

- the function of IALA with respect to Safety of Navigation, the efficiency of maritime transport and the protection of the environment.

RECOGNISING:

- that there is an increase in new and emerging uses of ocean and coastal waters, subsoil and seabed, an increase of seaborne trade, increasing demands of energy resources, increasing recreational use and increasing pollution threats from both ocean uses and an expansion of coastal populations and increasing need for breakwaters to protect ports and coastal areas.

- that the number and types of man-made structures being built in the maritime environment are increasing.

- the need to provide consistency in marking different types of man-made structures which may be a danger to navigation.

- that it is a matter for a National Authority to decide on whether a man-made structure needs to be marked, depending on the risk involved and the level of traffic.

- that IMO Resolution A.672(16), dated 6th December 1989, established Guidelines and Standards for the Removal of Offshore Installations and Structures on the Continental Shelf and in the Exclusive Economic Zone, which incorporate requirements for such installations and structures, whilst being removed, to be marked in accordance with IALA Recommendations.

- that marking is to improve the safety to navigation and protect the structures themselves.

ADOPTS:

- the recommendation on the marking of man-made structures as set out in the following Sections.

NOTING:

RECOMMENDS that Members ensure that the marking of man-made structures conforms to the standards and practices specified in the following Sections of this recommendation.

# Table of Contents

Document Revisions	21

Recommendation on The Marking of Man-Made Man made Structures	3

Table of Contents	4

Index of Figures	5

1.	INTRODUCTION	  7

1.1	Scope	7

1.2	Field of Application	7

1.3	Information and promulgation	7

1.4	Emergency provisions and contingency plans	7

2.	Marking of man made STRUCTURES	8

2.1	MARKING OF MAN MADE STRUCTURES / PLATFORMS IN GENERAL	8

2.1.1	General	8

2.1.2	Marking	9

2.1.3	Considerations during construction and decommissioning	10

2.2	MARKING OF OIL AND GAS PLATFORMS	11

2.3	MARKING OF OFFSHORE WINDFARMS	11

2.3.1	Marking of Isolated WTG, Meteorological Masts and other Individual Structures	12

2.3.2	Marking of Floating Wind Turbine Structures	13

2.3.3	Marking of Group of Structures (Man made Wind Farms)	13

2.4	MARKING OF  WAVE AND TIDAL ENERGY DEVICES	14

The level of marking should be decided after a risk assessment has been conducted.	15

2.4.1	2.4.1 Marking	15

2.5	MARKING OF MAN MADE AQUACULTURE FARMS	16

2.5.1	Marking Examples	17

2.6	  MARKING OF BREAKWATERS

2.6.1

2.6.2

3.	FUTURE REQUIREMENTS	19

4.	Definitions and Acronyms	20

APPENDIX 1	INVENTORY OF MAN MADE STRUCTURES	21

1.	Aquaculture Farms	21

2.	Articulated Loading Platform (ALP) (Oil & Gas Permanent Installations)	22

3.	CALM (Oil & Gas Permanent Installations - Loading / Discharge Buoys)	23

4.	Floating Production Storage Offloader (FPSOs) (Loading / Discharge Buoys)	24

5.	Floating Wind Turbine	24

6.	Flotel (Oil & Gas Temporary Structures)	25

7.	LNG Offloading Points	25

8.	Meteorological Masts	26

9.	Minimum Facility Platform (MFP)	26

10.	Man made docks / Loading Islands	27

11.	Pipes	27

12.	Production Platform / Drilling Rigs	28

13.	Seaplane berth	28

14.	Seawater Intakes / Sewage Outfalls	29

15.	Single Point Mooring (SPM) (Loading / Discharge Buoys)	29

16.	Tension Leg Platform (TLP) (Oil & Gas Permanent Installations)	30

17.	Tidal Generator	31

18.	Tidal Generator Field	31

19.	Underwater manifolds / obstructions	32

20.	Water Injection Platforms or booster station (WIPs)	32

21.	Wave Generator	33

22.	Wave Generator Field	33

23.	Wind Generator	34

24.	Wind Farm	34

25. 	Breakwaters	35

# Index of Figures

Figure 1	Sample marking of an individual wind turbine	13

Figure 2	Sample marking of an OWF	14

Figure 3	– Wave and tidal devices	16

Figure 4	A. Rectangular Aquaculture Farms	18

Figure 4	B.  Circular Aquaculture Farms	19

Figure 5	A fish farm	21

Figure 6	A diagram of an Articulated Platform	22

Figure 7	A Catenary Anchor Leg Mooring buoy	23

Figure 8	A Floating Production Storage Offloader	24

Figure 9	A floating wind turbine	24

Figure 10	An example of a Flotel	25

Figure 11	An LNG Offloading Point	25

Figure 12	Meteorological Masts	26

Figure 13	An example of a Minimum Facility Platform	27

Figure 14	Man made docks / Loading Islands	27

Figure 15	Typical underwater piping	27

Figure 16	Underwater pipe maintenance	27

Figure 17	An example of a Production Platform	28

Figure 18	An example of a Seaplane berth	28

Figure 19	Seawater Intake / Sewage Outfall	29

Figure 20	Single Point Mooring buoy	29

Figure 21	diagram of a Tension Leg Platform	30

Figure 22	Tidal Generators	31

Figure 23	Tidal Generator Field	31

Figure 24	example of an underwater obstruction	32

Figure 25	Water Injection Platform	32

Figure 26	Wave Generator	33

Figure 27	Wave Generator Field	33

Figure 28	Wind Generator	34

Figure 29	 Offshore Wind Farm	34

Figure 30	Breakwater Example

Figure 31	Breakwater Example 2

# INTRODUCTION

There is increasing development of man-made structures at sea, which may affect shipping.  These structures can be isolated or in groups, small or large, and close to or far from shipping routes.

IALA is monitoring the developments of these structures and will continue to create and update documentation as required to ensure clear and unambiguous marking of waterways for safe navigation, protection of the environment and protection of the structures themselves.  Authorities facing problems in this field are invited to bring them to the attention of IALA to obtain advice on current practice.

The following sections of this document detail the updated IALA recommendations for the marking of each structure type.  Appendix 1, contains  an inventory and examples of man-made offshore structures.

The marking of structures as defined in these recommendations may be considered as a minimum requirement to ensure the safety of navigation in the vicinity of the structures, however, National Authorities may require more stringent marking.

### SCOPE

These recommendations are for the guidance and information of stakeholders such as National Authorities, Lighthouse, Port, and, Aviation Authorities and other competent Authorities, Aids to Navigation providers, and the Contractors, Developers and Operators involved in each type of the structures mentioned in the following sections. They are further called National Authorities in this Recommendation.

### FIELD OF APPLICATON

The guidance contained in this document applies to all structuresfixed in position temporarily or permanently which extend above or below the surface of the sea and which are obstructions to navigation, e.g. structures used for drilling or exploring for oil and/or minerals, oil production platforms, oil well protective jackets, renewable offshore energy installations, ocean data platforms, breakwaters or offshore aquaculture farms.

### INFORMATION AND PROMULGATION

National Authorities must ensure that all stakeholders are informed of installed Aids to Navigation (AtoN) and markings in accordance with these Recommendations.  These are published in nautical charts and publications and by promulgation of Maritime Safety Information (MSI).

### EMERGENCY PROVISIONS AND CONTINGENCY PLANS

- In case of main power failure, an adequate backup system is recommended to maintain the function and availability of AtoN, including Racon and AIS, for time specified by the National Authority; typically 96 hours.

AtoN and AtoN systems should have availability in accordance with IALA Recommendation R0130 on Categorisation and Availability Objectives for Short Range AtoN and Guideline 1035 on Availability and Reliability of AtoN.

Remote monitoring of the AtoN system is recommended.

National Authorities are responsible for providing Maritime Safety Information (MSI) when an operator reports any AtoN malfunction.

Operators of man-made structures are recommended to develop contingency and emergency response plans which address the possibility of individual devices breaking loose and becoming floating hazards.

Operators are recommended to have a reliable maintenance and AtoN defect response regime in place to ensure the required availability targets are met.  This will include having the necessary AtoN spares on hand, with provision made at the design stage, where necessary, to ensure safe access.

# 2 	MAN-MADE STRUCTURES

Man Made structures present very different characteristics.  Therefore, these structures have been grouped as follows:

- Section 2.1: Offshore Structures in General

- Section 2.2: Oil and Gas Platforms

- Section 2.3: Offshore Wind Farms

- Section 2.4: Wave and Tidal Energy Devices

- Section 2.5: Aquaculture Farms

- Section 2.6: Breakwaters

2.1	MARKING OF MAN-MADE STRUCTURES

    - IN GENERAL

The marking requirements defined in this section must be complemented with those in sections 2.2 to 2.6 for the specific types of man-made structures.

Consultation between the stakeholders should take place at an early stage.  In general, development of all structures mentioned in this section must not prejudice the safe use of Traffic Separation Schemes, Inshore Traffic Zones, recognised sea-lanes and safe access to anchorages, harbours and places of refuge.

On a case-by-case basis, National Authorities may consider establishing Exclusion or Safety Zones and areas to be avoided in order to prohibit or restrict vessels from entering areas of man-made Structures.. Such information must be identified on the nautical charts and publications and promulgated through Maritime Safety Information (MSI).

The National Authority shall bear in mind that the marking recommendations herein may be adjusted based on risk assessments that consider background lighting, traffic density, proximity to ports, proximity to dangers, tidal considerations and other factors.

In order to avoid confusion from a high-density of AtoN (and other general lighting), it is recommended that full consideration be given to the use of synchronised lights, different light characters and varied light ranges.

- There has been some evidence that sea-bed erosion, at the bases of offshore renewable energy installations in areas of strong tides or currents has resulted in significant deposits of material in other locations.  National Authorities may consider fitting depth-monitoring devices to such installations to measure erosion.  This may need to be considered when approving wave and tidal energy extraction proposals / locations.

Power cables between offshore energy devices and the Offshore Sub Station, and between the Offshore Sub Station and the shore should be sufficiently trenched to avoid exposure from erosion  / sand migration or trawling activities.  Where burial depth is not achieved additional marking requirements are recommended.

    - MARKING

The general rules for the marking of Offshore Structures are as follows:

- If implemented, it is recommended that fog signals meet the minimum requirements identified by the National Authority authorizing the development of the off shore structure.

- Where there is a requirement to identify a particular structure, a radar beacon (Racon) or Automated Identification System (AIS) AtoN may be fitted.  The character and code length shall be determined by the National Authority.

- The National Authority may consider that a group of structures located close together can be marked as one single platform or structure.

- The National Authority may consider that buoys or beacons are placed to mark the perimeter of a group of structures, to mark channels through a group of structures, or to mark any fixed structure while being erected or dismantled.  The characteristics of such marks shall be determined by the National Authority in accordance with the IALA Maritime Buoyage System (MBS).

- Where underwater obstructions, such as submerged wells, pipelines or breakwaters, are considered to be a hazard to surface borne vessels, it is recommended that they are adequately marked in accordance with the MBS.

- The Hydrographic Office must be informed of the marking, location and extent of any man-made structure, to permit the appropriate charting.

- Notices to Mariners must be issued to publicise the establishment of a man-made structure(s) / field.  The Notice to Mariners has to include the marking, location and extent of such structure(s) / fields.

- The National Authority should be satisfied that the selected lighting has a suitable Nominal Range and sufficient autonomy to remain powered through all seasonal conditions  – especially in higher latitudes.

- The aviation authorities may require additional marking of the structure(s).

- The table below lists the marking recommendations and considerations for offshore structures:

| * = RECOMMENDED + = TO BE CONSIDERED | Lights (white) | Lights (yellow) | Subsidiary Lights (red) | Intermediate Lights (yellow) | Fog Signal | Radar Beacon | AIS AtoN | Floating AtoN | Pathway or Street Lighting |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Offshore Oil or Gas Platform – Temporary or Fixed | * |  | * |  | + | + | + | + |  |
| Floating Production Storage Offloading | * |  | + |  | + | + | + |  |  |
| Floating Petrochemical Offloading Points / Single Point Mooring | * |  | + |  | + | + | + |  |  |
| Aquaculture |  | * |  |  |  | + | + | * |  |
| Meteorological Mast | * |  |  |  | + | + | + | + |  |
| Minimum Facility Platform | * |  | + |  | + | + | + | + |  |
| Offshore Docks / Loading Islands | * |  | * |  | + | + | + | + |  |
| Underwater Pipes, Underwater Manifolds | + |  |  |  |  |  |  | + |  |
| Isolated Tidal / Wave Generator | * |  | + |  | + | + | + | + |  |
| Tidal/Wave Generator Field |  | * |  |  | + | + | + | * |  |
| Offshore Wind Farm |  | * |  | + | + | + | + | + |  |
| Isolated WTG | * |  |  |  | + | + | + | + |  |
| OWF Transformer / Sub-Station | * |  | + |  | + | + | + |  |  |
| Breakwaters |  | * |  |  | + | + | + | + | + |

    - CONSIDERATIONS DURING CONSTRUCTION AND DECOMMISSIONING

It is essential to consider the marking of man-made  structures during the different phases of their existence, i.e. construction, operation and decommissioning, when the structure may be a hazard to navigation.

During the construction and decommissioning of man-made structures, it is recommended that working areas are established and marked as appropriate.  National Authorities shall also consider the use of guard ships, temporary VTS or the establishment of temporary work vessel routes in areas of high traffic density.

MSI must be promulgated in advance of and during any man-made structure / field construction or decommissioning.

When decommissioning such devices, it is recommended that the National Authority ensures that the operator / contractor remove all obstructions, so that the seabed is verified as being returned to its original depth and topography.

In the event that an obstruction remains which constitutes a danger to navigation, then it is recommended to mark the location based on risk assessment.

### MARKING OF OIL AND GAS PLATFORMS

- This section supplements the general rules for marking defined in section 2.1 and must be read in conjunction with it.

The Offshore Structures mentioned in this section are recommended to be marked as a single unit, a block or field, as appropriate, as follows:

- Any structure shall be marked at night by one or more white lights so constructed and fixed as to ensure that at least one light is visible upon approaching the structure from any direction. Minimum nominal range is to be 10 Nautical miles.

- Subsidiary red flashing lights shall also be provided and show the same characteristics as the main white lights, i.e. synchronized Mo (U) R ≤15s.  These are to be located to mark the horizontal extremities of the structure, excepting those marked with white lights, as well as interconnecting bridges.  Minimum nominal range is to be 3 Nautical miles.

- Each structure, where practicable, displays identification panels with black letters or numbers 1 metre high on a yellow background visible in all directions.  These panels shall be easily visible in daylight as well as at night, either by using appropriate illumination or retro-reflecting material.

### MARKING OF OFFSHORE WINDFARMS

- This section supplements the general rules for marking defined in section 2.1 and must be read in conjunction with it.

When mentioning Offshore Wind Farms (OWF), the following are included: Meteorological Mast, Wind Turbine Generator (WTG) and Offshore Transformer / Sub-Station.

- It is recommended that each structure, where practicable, displays identification panels with black letters or numbers 1 meter high on a yellow background visible in all directions.  These panels shall be easily visible in daylight as well as at night, either by using illumination or retro-reflecting material.

The structures should be painted yellow all around from the level of HAT up to 15 metres. On a case-by-case assessment alternative marking, where applicable, may include horizontal yellow bands of not less than 2 metres in height and separation. The addition of retro-reflective material may be considered (see figure 1).

When using working lights, such as down lighting on ladders and access platforms, they must not reduce the conspicuity of marking lights.

- National Authorities should consider that:

- OWF structures may affect shipborne and shore based radar systems, which in some cases through inherent system limitations, cause interference strong enough to produce significant degradation of the radar display;

- Passage close to an OWF boundary, or within the OWF itself, could affect vessels’ capability to manoeuvre;

- The safety of navigation shall be ensured when approving an OWF;

- Marking lights should be visible from all directions in the horizontal plane.

- Consideration may also be given to the provision of fog signals where appropriate, taking into account the prevailing visibility, topography and vessel traffic conditions. The range of such a fog signal should not be less than two 2 Nautical miles.

    - MARKING OF ISOLATED WIND TURBINE GENERATORS (WTG), METEOLOGICAL MASTS AND OTHER INDIVIDUAL STRUCTURES

- It is recommended that these structures:

- Are marked with a white light flashing Mo (U) W ≤15s, and with a nominal range of 10 Nautical miles;

- Have AtoN mounted below the lowest point of the arc of the rotor blades.  They shall be located at a height of at least 6 metres above HAT;

- Have AtoN that comply with IALA Recommendations and have an availability of not less than 99.0% (IALA Category 2).

Figure 1 Sample marking of an individual wind turbine

    - MARKING OF FLOATING WIND TURBINE STRUCUTURES

- Due to the specific movement of the floating wind structures, it is recommended that:

- National Authorities take into account the interaction between aviation lights and the shipping in the area;

- The marine lights should have a larger vertical divergence compared to a fixed structure, in order to maximize visibility at range to the mariner (e.g. 30º at 50%).

    - MARKING OF GROUP OF STRUCTURES (OFFSHORE WIND FARMS)

- A Significant Peripheral Structure (SPS) is the ‘corner’ or other significant point on the periphery of the OWF. It is recommended that:

- These lights display a Special Mark characteristic, flashing yellow, with a nominal range of 5 Nautical miles;

- The National Authority may consider the synchronisation of all SPS;

- In the case of a large or extended OWF, the distance between SPS should not normally exceed 3 Nautical miles.

It is recommended that Intermediate Peripheral Structures (IPS) selected on the periphery of an OWF:

- Are marked with flashing yellow lights;

- The flash character of these lights shall be distinctly different from those displayed on the SPS, with a nominal range of 2 Nautical miles;

- Have a lateral distance between IPS or the nearest SPS which will not normally exceed 2 Nautical Miles.

The National

SPS

SPS - lights visible from all directions in the horizontal plane.  It is recommended to synchronize these lights in order to display a Special Mark characteristic, flashing yellow, with a range of not less than 5 Nautical miles

IPS

Intermediate structures on the periphery of an OWF other than the SPS - marked with flashing yellow lights which are visible to the mariner from all directions in the horizontal plane with a flash character distinctly different from those displayed on the SPS and with a range of not less than 2 Nautical miles

Figure 2 Sample marking of an OWF

< 3 nm

< 2 nm

SPS

SPS

SPS

SPS

SPS

IPS

IPS

Depending on the marking, lighting and lateral separation of the peripheral structures, the additional marking of the individual structures in general within an OWF may be considered as follows:

- Lighting or marking of each structure;

- Unlit individual structures can be made more conspicuous with illumination and retro-reflecting material;

- Use of flashing yellow lights with a nominal range of 2 Nautical Miles;

- Racons;

- AIS AtoN.

An Offshore Transformer / Sub-Station or a Meteorological Mast, if considered to be a composite part of the OWF, shall be included as part of the overall OWF marking.  If not considered to be within the OWF block it shall be marked as an isolated offshore structure (see 2.3.1).

### MARKING OF WAVE AND TIDAL ENERGY DEVICES

- This section supplements the general rules for marking defined in section 2.1 and must be read in conjunction with it.

Wave and Tidal Energy Devices include: Tidal Generator, Tidal Generator field, Wave Generator, Wave Generator field as defined in Appendix 1.

It must be borne in mind that many wave and tidal devices are low freeboard floating structures moored to the seabed.  They may be moored in deep or shallow water and some may be located on the seabed or just below the surface.  Surface piercing and subsurface elements may extend laterally beyond the surface elements.  This could include shared moorings and mid-water connections between units that may also carry electricity, control signals, hydraulics or pneumatics associated with the units.

When identifying the marking requirements, it must be taken into consideration that some tidal devices:

- Have fast-moving sub-surface elements such as whirling blades;

- Do not allow for safe under keel clearance (UKC).

The level of marking should be decided after a risk assessment has been conducted.

    - MARKING

- Wave and Tidal energy extraction devices should be marked as a single unit or as a block or field as follows:

- When structures are fixed to the seabed or in the water column and extend above the surface, they shall be marked in accordance with the guidance contained in Section 2.3.

- It is recommended that:

  - Subject to the proper risk assessment, areas containing on surface or sub-surface wave or tidal devices are marked by appropriate AtoN.  In addition, radar reflectors, retro-reflecting material, Racons and / or AIS transponders should be considered where the level of traffic and degree of risk requires;

  - The AtoN must be visible to the mariner from all relevant directions in the horizontal plane, by day and lighted at night;

  - ;

cTaking the results of a risk assessment into account, lights must have an appropriate nominal range and vertical divergence and may be synchronized.

  - Individual wave and tidal energy devices within a site that extend above the surface are painted yellow above the waterline and have yellow retro-reflective tape.  If navigation is permitted within the site, marking of individual devices may be required;

  - If marked, the individual devices should have flashing yellow lights.  The flash character of such lights must be sufficiently different from those displayed on the boundary lights with a nominal range of not less than 2 Nautical miles;

  - Floating AtoN should be located outside the moorings of the floating structures.

- Based on risk assessment, a single wave or tidal energy extraction structure, standing alone, may be marked as follows:

- An Isolated Danger Mark or

- Special Mark.

- Specific guidance to small craft needs early consideration.

- The AtoN described herein should comply with IALA Recommendations and have an appropriate availability, normally not less than 99.0% (IALA Category 2).

- Recommended principles for marking of area for wave energy devices are referred to in the figure below. The National Authority may consider the distances between lit and unlit special marks on a case by case basis and on a risk assessment.

Figure 3 Marking of Wave and tidal devices

## MARKING OF OFFSHORE AQUACULTURE FARMS

- This section supplements the general rules for marking defined in section 2.1 and must be read in conjunction with it.

The farm, or group of farms, should be marked depending on their size, extent and location. In some cases it may be sufficient to mark only part of the perimeter, or the centre. The use of Racons or AIS AtoN may also be considered.

Recommended principles for marking of area aquaculture farms are referred to in the table below. National Authority may consider the distances between lit and unlit special marks on a case by case basis after conducting  a risk assessment.

It should be borne in mind that many aquaculture farms are low freeboard floating structures that are moored to the seabed.  They may be moored in deep or shallow water and some may be located on the seabed or just below the surface.  Surface piercing and subsurface elements may extend laterally beyond the surface elements.  This could include shared moorings and mid-water connections between units that may also carry electricity, control signals, hydraulics or pneumatics associated with the units.

The National Authority should bear in mind that the marking recommendations herein may be adjusted in consideration of traffic density, proximity to ports, proximity to dangers, tidal considerations and other factors.

It is recommended to mark offshore aquaculture farms as follows:

- Aquaculture farms are normally marked by Special Marks;

- If there is a requirement for vessel traffic between aquaculture farms, then such channels are normally marked by Lateral Marks;

- If the prevailing situation warrants, Cardinal Marking alone may be used to direct vessel traffic away from the aquaculture farm(s);

- It is recommended that areas of aquaculture farms are marked by appropriate AtoN.  In addition radar reflectors, retro reflecting material, Racons and AIS AtoN may be considered;

- To improve the effectiveness of marking and taking into account background lighting, synchronisation of the lights is recommended.  Taking the results of a risk assessment into account, lights must have an appropriate nominal range;

- Specific guidance to small craft needs early consideration;

- The AtoN described herein should comply with IALA Recommendations and have an appropriate availability, normally not less than 99.0% (IALA Category 2).

### Marking Examples

Examples can be found in the following tables and figures that illustrate the minimum recommended marking arrangement with Special Marks.

- It is recommended that Rectangular Aquaculture Farms are marked according to the length of their sides.

| Example | X Axis (m) | Y Axis (m) | Area (m²) | Minimum Marking Requirements |
| --- | --- | --- | --- | --- |
| A | ≤ 500 | ≤ 500 |  | One light in centre of farm (consider radar reflector) |
| B | ≤ 2 500 | ≤ 500 |  | One light on each sea corner; one daymark on each coast corner (consider radar reflector) |
| C | ≤ 500 | ≤ 2 500 |  | One light on one sea corner; one light on the diagonally opposite coast corner; one daymark on one sea corner and one daymark on the diagonally opposite corner (consider radar reflector) |
| D | > 500 | ≤ 2 500 | ≤ 1 250 000 | One light on diagonally opposite corners; daymark on diagonally opposite corners (consider radar reflector) |
| E | > 900 | ≤ 2 500 | > 1 250 000 | One light on each corner (consider radar reflector) |

Special Mark LIT (Fl Y 3s 5NM) synchronized

Special Mark UNLIT

Area borders

Safety borders

T

Figure 4 A. Rectangular Aquaculture Farms

B

shore

sea

≤ 500 m

≤ 2500 m

A

≤ 500 m

≤ 500 m

D

shore

sea

> 500 m

≤ 2500 m

C

shore

sea

≤ 500 m

≤ 2500 m

E

shore shoreeeee shoreore shore

shore

ee

e shore eeeeeeeeeee

sea

> 900 m

≤ 2500 m

≤ 2500 m

- Circular Aquaculture Farms should be marked according to their diameter.

| Example | Diameter (m) | Diameter (m) | Minimum Marking Requirements |
| --- | --- | --- | --- |
| F |  | ≤ 500 | One light in centre of farm (consider radar reflector) |
| G | > 500 | ≤ 1000 | Two lights 180° apart on the circumference; two daymarks positioned 90° to the lights (consider radar reflector) |
| H | > 1000 | ≤ 2000 | Three lights 120° apart on the circumference (consider radar reflector) |
| I | > 2000 |  | Three lights 120° apart on the circumference, three daymarks positions 60° to the lights (consider radar reflector) |

Figure 5 B. Circular Aquaculture Farms

# 2.6 MARKING OF BREAKWATERS

This section provides information that will assist in the development of guidance for the marking of ‘above water’, partially or wholly submerged breakwaters.

Breakwaters are large artificial offshore structures designed  to serve as a barrier that protects a coast, port,  its facilities (e.g. a marina). .

Breakwaters can present challenges to safe navigation if not marked appropriately. A lack of lighting or inconsistent marking can create confusion for transiting ships, domestic vessels and recreational vessels, particularly at night.

2.6.1 –EXPOSED BREAKWATERS

Other breakwaters are exposed or ‘above water’, and are associated with smaller, regional ports for the domestic and recreational industries.  The vast majority of these breakwaters are marked using a combination of lateral marks, and in many cases, a set of lead and/or sector lights to assist approaching vessels in navigating between breakwaters.

Small and regional ports - Other breakwaters are exposed or ‘above water’, and are associated with smaller, regional ports for the domestic and recreational industries.  The vast majority of these breakwaters are marked using a combination of lateral marks, and in many cases, a set of lead and/or sector lights to assist approaching vessels in navigating between breakwaters.

The following example illustrates the layout of AtoNs for marking the breakwaters at Wyndham Port, Victoria.

Figure 1 - Wyndham Harbour, Victoria. (Photo courtesy: Sail Escapade)

The unlit portion of the breakwater (in between AtoN) presents a hazard to navigation for mariners. Where pedestrian access is permitted, pathway or street lighting is sometimes, but not always, provided.  Pathway or street lighting should be considered to provide a non-conventional means to maximize the area illuminated of a breakwater, thus increasing safety for the mariner.

Large Commercial Ports – Some larger ports incorporate a combination of special marks, spaced at an equal distance apart (no more than 200m), in a similar fashion to that illustrated in section 3.3. Examples of where ‘Special Marks’ have been used extensively to mark breakwaters include the Port of Brisbane, (Australia?)  and Port of Townsville (Australlia?).  Examples of marking arrangements are included below:

Figure 2 - Fisherman’s Island, Port of Brisbane

2.6.2 – SUBMERGED BREAKWATERS

There are many breakwaters designs, among them, submerged breakwaters present high risk as they are underwater at high tide, and only partially above water at low tide.  Usually, vessels that transit in the proximity of such submerged breakwaters are small vessels, such as fishing boats and leisure boats. Many of these vessels have no electronic chart system or advanced navigational equipment.  If a submerged breakwater is not visible to the mariner, it can be very dangerous.

- The deployment of submerged breakwater marking shall be recommended in accordance with the following criteria, taking into account various factors, including the structure of submerged breakwater and maritime conditions near the deployment site, traffic density and proximity to the port.

- 1. Submerged breakwater markings for prevention of collision are generally deployed as special marks

- 2. If there is a passageway of a vessel around a submerged breakwater, mark the waterway according to the IALA MBS

- 3. If markings are available on the top of submerged breakwater deploy on the top, if not deploy on the vessel passageway side near the submerged breakwater.

- 4. For single submerged breakwater, deploy at the centre or end of a submerged breakwater depending on the sea conditions.

5. If 2 or more of submerged breakwaters are exist, consider the entire submerged breakwaters as one and deploy it at both ends of the submerged breakwater at regular intervals depending on the conditions of the sea

- The criteria for the AtoN arrangement of submerged breakwater will be considered on a case by case basis after close consultation with owner of project and the risk assessment.

- Examples:1.Marking with fixed AtoN.

- 2.Marking with floating AtoN

# Future requirements

IALA is aware that there is an increasing quantity of energy devices and structures already in place and many more planned that may affect shipping.

It is therefore recommended that National Authorities continuously monitor developments to ensure that any navigational problems caused by offshore structures are solved in a satisfactory manner.

# definitions and acronyms

National Authority - the competent Authority for determining the marking of offshore structures.

Energy Extraction Device (EED) - a wave or tidal generator.

Highest Astronomical Tide (HAT) - is the highest level that can be expected to occur under average meteorological conditions and under any combination of astronomical conditions.  HAT is not an extreme level, as certain meteorological conditions can cause a higher water level.

Nautical mile – 1852m.

Significant Peripheral Structure (SPS) – the corner wind generator on a rectangular OWF or other significant point on the periphery of an OWF.

Transformer Station (hub) – a special structure within or outside the wave and/or tidal energy extraction field and/or OWF to which the individual generators are connected via a power cable.  Power is transferred ashore from the transformer station by submarine cable.  A ‘hub’ may be a separate fixed or floating platform, a unit very similar to the generators but carrying additional power conversion equipment.

Usual Range – the usual range of the fog signal shall be calculated in accordance with IALA Recommendation E-109, on the calculation of the range of a sound signal.

Subsidiary Light – additional red light(s) located on offshore platform(s) used to mark the extremities of extensive installations and their interconnecting bridges.

Promulgation – to make known by open declaration; publish; proclaim formally or put into operation (a law, decree of a court, etc.).

| AIS | Automatic Identification System |
| --- | --- |
| AIS AtoN | AIS as an Aid to Navigation |
| AtoN | Aid(s) to Navigation |
| CALM | Catenary Anchor Leg Mooring |
| cd | Candela |
| FPSO | Floating Production Storage Offloading |
| HAT | Highest Astronomical Tide |
| IPS | Intermediate Peripheral Structure [OWF] |
| MBS | IALA Maritime Buoyage System |
| MER | Minimum Effective Range |
| MFP | Minimum Facilities Platform |
| MHWS | Mean High Water Springs |
| MSI | Maritime Safety Information (e.g. NAVTEX, Notices to Mariners) |
| OREI | Offshore Renewable Energy Installation |
| OWF | Offshore Wind Farm(s) |
| SOLAS | Safety of Life At Sea [convention]. |
| SPM | Single Point Mooring |
| SPS | Significant Peripheral Structure [OWF] |
| UKC | Under Keel Clearance |
| WTG | Wind Turbine Generator |

INVENTORY OF MAN MADE STRUCTURES

This Appendix states definitions and gives examples of current man made structures (in alphabetical order).

# Aquaculture Farms

A fish farm

Aquaculture is the cultivation of fresh-water and marine species, including fish, molluscs, crustaceans and aquatic plants.  Unlike fishing, aquaculture, also known as aquafarming, implies the cultivation of aquatic populations under controlled conditions.

Particular kinds of aquaculture include algaculture (the production of kelp/seaweed and other algae); fish farming; shrimp farming, shellfish farming, and the growing of cultured pearls.

The worldwide practice of aquaculture ranges from low-technology extensive methods to highly intensive systems.

Aquaculture farms take on a variety of forms including huge tanks, freshwater ponds, and shallow- or deep-water marine environments. This document relates to farms in marine environments.

# Articulated Loading Platform (ALP) (Oil & Gas Permanent Installations)

A diagram of an Articulated Platform

An ALP is a metal lattice tower, buoyant at one end and attached at the other by a universal joint to a concrete filled base on the sea bed.  The platform may be fitted with a helicopter platform, emergency accommodation and hawser/hose retrieval.

# CALM (Oil & Gas Permanent Installations - Loading / Discharge Buoys)

A Catenary Anchor Leg Mooring buoy

CALM systems, are named so for the characteristic curve of the anchor legs that hold the buoy in position.  These buoys are also often referred to as a single buoy mooring, monobuoy or loading buoy.

CALM buoys can be designed to berth any size tanker up to and including Ultra Large Crude Carriers (ULCCs).

The main applications of a CALM system are:

- Short term mooring: for import and export of fluids between onshore or man made facilities and a tanker;

- Permanent mooring: for production and storage systems;

- Semi-permanent mooring: permanent mooring with easy disconnect capability to evacuate the facility in case of severe weather conditions.

# Floating Production Storage Offloader (FPSOs) (Loading / Discharge Buoys)

A Floating Production Storage Offloader

These are vessels which are usually self-powered and make their own way from the builder yard to the oil fields where they are permanently anchored, and act as floating tanks.

They may be purpose built or can be ships (VLCC, for example) that have passed their active trading life and were refitted into a FPSO.

# Floating Wind Turbine

A floating wind turbine

A wind turbine that is moored to the seabed.

# Flotel (Oil & Gas Temporary Structures)

An example of a Flotel

This type of structure is a platform that is used as an R&R (rest and recuperation) location for workers; it is not an active rig.

# LNG Offloading Points

An LNG Offloading Point

These Offloading Points, which are used for loading / unloading LNG, are linked or fixed to the seabed and include many types of transference structures.

# Meteorological Masts

Meteorological Masts

Any individual surface structure, usually consisting of an embedded mast or tower with meteorological measuring instruments.

# Minimum Facility Platform (MFP)

An example of a Minimum Facility Platform

These are unmanned platforms for a variety of purposes, such as pipeline booster stations and transformer stations.

# Man made docks / Loading Islands

Man made docks / Loading Islands

Floating structures of various types and sizes, moored to the seabed and used for berthing and loading / unloading cargo.

# Pipes

Underwater or subsea pipes are used worldwide.  They are usually made of steel, have a concrete coating, and depending on the conditions, can be placed by barges with divers assistance.

| Typical underwater piping | Underwater pipe maintenance |
| --- | --- |

# Production Platform / Drilling Rigs

An example of a Production Platform

Production Platforms or Drilling Rigs are large structures used to house workers and machinery needed to drill and/or extract oil and natural gas through wells in the ocean bed.  The platform may be attached to the ocean floor, consist of an artificial island, or be floating.

Many platforms also have remote wellheads attached by umbilical connections, these may be single wells or a manifold centre for multiple wells.

# Seaplane berth

An example of a Seaplane berth

A Seaplane is a fixed-wing aircraft designed to take off and land on water.  A seaplane berth is a structure that has berthing facilities for these kind of planes, which are generally used for connecting islands to the mainland.

# Seawater Intakes / Sewage Outfalls

A Seawater Intake / Sewage Outfall

A seawater supply system can be found in many projects, and it includes Seawater Intakes and a discharge system.  An outlet to a drainage network or a wastewater treatment is the Sewage Outfall.

# Single Point Mooring (SPM) (Loading / Discharge Buoys)

A Single Point Mooring buoy

Loading Buoys are anchored man made and serve as a mooring point for tankers to (off)load gas or fluid products.  They are the link between the geostatic subsea manifold connections and the tanker.

The main purpose of the buoy is to transfer fluids between onshore or man made facilities and the moored tanker.

These Buoys are generally referred to as Single Point Mooring systems or ‘SPMs’.  They are also often referred to as Single Buoy Moorings or ‘SBMs’.

# Tension Leg Platform (TLP) (Oil & Gas Permanent Installations)

A diagram of a Tension Leg Platform

A Tension Leg Platform (TLP) is a buoyant platform held in place by a mooring system.  The TLP’s are similar to conventional fixed platforms except that the platform is maintained on location through the use of moorings held in tension by the buoyancy of the hull.

The topside facilities (processing facilities, pipelines, and surface trees) of the TLP and most of the daily operations are the same as for a conventional platform.

# Tidal Generator

Tidal Generators

Any individual surface or sub-surface structure incorporating a generator, fixed or moored to the seabed and connected to an electrical terminal via cable(s).

# Tidal Generator Field

A Tidal Generator Field

A group of individual tidal generators, which are located in one block and are considered to be a unit, fixed or moored to the seabed and/or each other and connected to an electrical terminal via cable(s).

# Underwater manifolds / obstructions

An example of an underwater obstruction

This group of structures comprises manifolds and various types of man-made obstructions placed on the seabed.

# Water Injection Platforms or booster station (WIPs)

A Water Injection Platform

Water injection platforms usually comprise a fixed steel platform, linked to a wellhead platform.  The integrated topside has water injection facilities, water treatment facilities and power generation.  This platform can also lodge a drilling rig.

# Wave Generator

Production models will be coloured yellow, in accordance with the Recommendation.

A Wave Generator

Any individual surface or sub-surface structure incorporating a generator, moored to the seabed and connected to an electrical terminal.

# Wave Generator Field

Production models will be coloured yellow, in accordance with the Recommendation.

A Wave Generator Field

A group of individual wave generators, which are located in one block and are considered to be a unit, moored to the seabed and/or each other and connected to electrical hub.

# Wind Generator

A Wind Generator

Any individual surface structure, usually consisting of an embedded mast or tower with rotating blades and incorporating a generator.

# Man made Wind Farm

An Man made Wind Farm

A group of individual wind generators, which are located in one block and are considered to be a unit.