IALA Guideline

Note:

- This version is based on the tracking of ARM12-7.3.5 WP 1078 - The use of AtoN in the design or fairways (ARM11-13.2.10) having been accepted.  The final this version needs to be combined with the tracked version of ARM12-7.3.5 WP 1078 - The use of AtoN in the design or fairways (ARM11-13.2.10) to show the overall tracked amendments to the original document

- With the multiple authors involved in the amendments, an overall check needs to be done to see if

- The relevant input appears under the appropriate headings

- There are no duplications

- Develop descriptions for inclusion in the IALA Dictionary:

- Fairway

- Channel

- Waterway

1078

The use of AtoN in the design of fairways and CHANNELS

Note: Mention “channel” where appropriate in conjunction with “fairway” throughout the document

Edition 1.0

June 2011

Revisions to this IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |
| XXX | XXX | XXX |

1.	INTRODUCTION	9

- 1.1.	Background	9

- 1.2.	e-Navigation	9

2.	USER REQUIREMENTS	9

- 2.1.	General	9

- 2.2.	Accuracy	10

- 2.3.	Reliability	10

- 2.4.	Special Requirements for different user groups	10

- 2.5.	USER ENGAGEMENTS	10

3.	Considerations	10

4.	PERFORMANCE PARAMETERS OF AtoN SYSTEMS	13

- 4.1.	Positioning Accuracy	13

4.1.1.	Radionavigation Systems	13

4.1.2.	Visual AtoN	13

- 4.2.	Redundancy	13

- 4.3.	Perception	13

4.3.1.	General	13

4.3.2.	Lights	14

4.3.3.	Daymarks	14

4.3.4.	Radar	14

4.3.5.	Automatic Identification System (AIS)	15

5.	LAYOUT OF AtoN FOR MARKING A FAIRWAY	15

- 5.1.	General	15

- 5.2.	AtoN Marking the Fairway Boundaries	15

- 5.3.	Other AtoN Marks in THE APPROACH TO, OR IN the Fairway	16

5.3.1.	Approach to a fairway	17

5.3.2.	Audible signals	17

5.3.3.	Radar Beacons	17

5.3.4.	MAtoN	17

5.3.5.	Tide lanterns	17

5.3.6.	traffic separation scheme (TSS)	17

5.3.7.	Bridges, overhanging cables, etc.	17

5.3.8.	Breakwaters, surface- and underwater	18

5.3.9.	Other boundary areas	18

- 5.4.	Fixed visual AtoN Outside the Fairway	18

5.4.1.	Leading and Transit lines	18

5.4.2.	Sector lights	18

5.4.3.	Major floating AtoN	19

5.4.4.	Position, navigation and timing (PNT) and DGNSS 	19

5.4.5.	Vessel Traffic Services	19

5.4.6.	Pilotage	19

6.	DESIGN METHODOLOGY/PROCEDURE	19

- 6.1.	A simple procedure for the establishment of a Marked Fairway	19

- 6.2.	Channel Design and Maintenance	20

- 6.3.	Risk assessment	20

- 6.4.	Simulation	20

7.	EXAMPLES OF MARKING OF FAIRWAYS	20

8.	CONCLUSION	20

9.	ACRONYMS & DEFINITIONS	20

- 9.1.	Acronyms	20

- 9.2.	Definitions	21

ANNEX A	RELEVANT IALA DOCUMENTATION	23

ANNEX B	DRIFT DETECTION – JAPANESE METHOD	24

ANNEX C	AtoN DESIGN - THE RIO DE LA PLATA NAVIGATION CHANNEL	28

ANNEX D	AtoN DESIGN - FINLAND	32

ANNEX E	AtoN DESIGN - LE HAVRE AND PORT 2000, FRANCE	39

ANNEX F	AtoN DESIGN - THE ‘SEEKANAL ROSTOCK’	42

ANNEX G	AtoN DESIGN - THE APPROACH TO MALMÖ, SWEDEN	46

List of Tables

Table 1	Definitions	20

Table 2	Synchronization of AtoN	45

List of Figures

Figure 1	Undetectable zone	24

Figure 2	Detectable manoeuvring lane for light buoys on the both sides of fairway	25

Figure 3	AtoN design on the Rio de la Plata	31

Figure 4	Mussalo Channel	33

Figure 5	Kotka inner harbours	34

Figure 6	Mussalo Harbour	35

Figure 7	Rauma Channel	37

Figure 8	Port of Rauma	38

Figure 9	Port of Le Havre	41

Figure 10	Seekanal Rostock – canal section	44

Figure 11	Example for synchronized AtoN: Inner part of ‘Seekanal Rostock’	44

Figure 12	Sea Chart of a part of the outer and inner channel ‘Seekanal Rostock’	45

Figure 13	Outer part of channel - Malmo	47

Figure 14	Inner part of channel - Malmo	47

Figure 15	Entrance to Malmo Harbour	48

Figure 16	Malmo Harbour	48

List of Equations

Equation 1	Width of basic manoeuvring lane:	24

Equation 2	Channel width required for drift detection using the naked eye or radar	25

Equation 3	Channel width needed for drift detection by observing light buoys with the naked eye	26

Equation 4	Definition of αmax	26

Equation 5	Channel width needed for drift detection by observing light buoys with radar	26

Equation 6	Channel width needed with error of direction of 2 degrees	27

Equation 7	Channel width needed with error of direction of 1 degree	27

Equation 8	Channel width required with drift detection by GPS	27

Equation 9	Channel width required with drift detection by DGPS	27

# INTRODUCTION

The purpose of this Guideline is to give guidance for Marine Aids to Navigation (AtoN) providers and competent authorities on the:

use of AtoN in the design of fairways, including dredged channels and canals;

review of existing AtoN for fairways including dredged channels and canals.

The objective is to define a suitable AtoN mix that enables safe and efficient vessel passage in the most cost effective way for AtoN providers.

This Guideline shall be used as a general overview.  For more detailed AtoN planning, this document should be used in conjunction with other IALA documentation, especially the IALA Maritime Buoyage System (MBS) (New name?).

## Background

The art of navigation has evolved over many hundreds of years, as well as the design of vessels.  For instance, vessels are bigger and faster; vessel equipage has become much more complex.

In principle, navigation comprises:

Planning a safe passage for a vessel, through the use of nautical charts and relevant publications;

Monitoring or establishing a vessel’s position or movement along a planned passage;

Controlling a vessel, to ensure that it follows a planned passage.

The navigator will combine various tools providing  navigational information to control the vessel.   This places great demand on the accuracy of the chart, the navigation system and the vessel’s control systems.

The availability, reliability and relative low cost of high precision electronic position fixing systems provide the modern mariner with accurate real time positioning from open ocean into restricted waterways.  Modern electronic AtoN, such as AIS AtoN, continue to become more available and, along with traditional physical AtoN, give the modern AtoN manager even more tools in designing and deploying AtoN systems for fairways.  Even as electronic aids evolve and improve, physical AtoN will continue to be the essential component of every waterway.

## e-Navigation

e-Navigation now has considerable impact on the mix of AtoN for an existing or a planned fairway.  e-Navigation will help to improve the efficiency of fairway marking with AtoN, providing :

improved safety, through promotion of standards in safe navigation;

better protection of the marine environment;

the potential for higher efficiency and reduced costs;

a potential reduction in bureaucracy - e.g. standardised reporting requirements.

Issues relating to the presentation of this e-Navigation information are still being determined.

# USER REQUIREMENTS

## General

According to SOLAS, Chapter V, Regulation 13, each of the Contracting Governments is required to provide, as it deems practical and necessary, AtoN, as the density of traffic and the level of risk requires.  IMO member states commit themselves to take into account international recommendations and guidelines when establishing such aids.

## Accuracy

The assigned position of the AtoN should be accurate and in accordance with International Hydrographic Organization (IHO) standards, in order that a vessel can establish its position sufficiently and follow a route in the fairway by visual or radionavigation means.

The AtoN should be surveyed and positioned with at least the same accuracy as the nautical chart.  This is determined in the IHO Standards for Hydrographic Surveys (S-44) 5th Edition February 2008 with accuracies of:

2 m for fixed aids (5m when water depth more than 100 m);

10 m for floating aids (20m when water depth more than 100 m).

In many cases, such as leading lights, these minimum IHO requirements are not accurate enough.

## Reliability

In determining AtoN Reliability, such issues as Integrity, Availability, Continuity and mean time to repair (MTTR) need to be taken into account.

The required level of AtoN Reliability is determined by the level of risks to the mariner, the vessel and the marine environment that are mitigated through the use of a particular AtoN.  In those areas in which the level of risk has been determined to be high, the use of certain types of AtoN may prove to provide greater risk mitigation.

The planner must consider availability objectives.  AtoN providers should refer to IALA Recommendation O-130 ‘on Categorisation and Availability Objectives for Short Range AtoN’ for additional information related to the categorization of individual AtoN, the calculation of availability targets, and recommended availability objectives.

If continuity is used for defining the requirements for a specific system, it has to be calculated for the time that a vessel takes to pass through the fairway or area.

## Special Requirements for different user groups

The level of on-board navigational equipment can vary significantly on different types of vessels.  SOLAS vessels are equipped with certified on-board navigational equipment.  This is suitable to support long-range and/or low visibility navigation.  These vessels are operated by professionally trained and certified personnel.  For these vessels, visual AtoN are used to enhance navigation and as a valuable way to verify the vessel’s position, thus providing essential redundancy to electronic navigation systems.  It is recommended that whenever possible, the reliance on a single method of determining position be avoided.  However, for vessels that are not SOLAS equipped, visual AtoN are essential.

For example High Speed Craft and other fast vessels in coastal areas, using visual AtoN for a safe passage, the speed of these vessels can be the defining factor for the AtoN system .

## USER ENGAGEMENTS

User consultation is an integral part of all AtoN planning and also applies with the use of AtoN in the design of fairways.  Refer to Guideline G1079 - Establishing and Conducting User Consultancy by Marine Aids to Navigation Authorities for guidance.

# Considerations

When the use of AtoN in the design of fairways are considered, or reviewed, the following should be taken into account:

- Approach to and exiting from a Port or Harbour

Port and Harbour AtoN should comply with the MBS wherever possible, in order that mariners can achieve a seamless transition between different areas.

- Through an area with specific features, e.g. :

- Depths of water, or lack thereof in certain areas and vessel under keel clearance

This would have an influence on the size of vessels that would be able to safely navigate the fairway, and subsequently the type of floating AtoN (normal buoy, resilient/elastic type, focal height, draft, diameter, with, or without a tail) to be made provision for, the swinging circle, the type of mooring (traditional or elastic) and the divergence of the buoy lanterns.

- Underwater- or above water obstructions, such as submerged breakwaters, isolated dangers, wrecks, that would require to be marked accordingly.

- Special Mark applications (Ocean Data Acquisition Systems (ODAS) marks, traffic separation marks, spoil ground marks, military exercise zone marks, cable or pipeline marks, recreation zone marks, boundaries of anchorage areas, structures such as offshore renewable energy installations, aquaculture and mooring buoys.

- Fairway cutting across a fishing, environmental and / or protected area (including dynamic marine mammal protected areas)

When it is necessary to create a throughway across a fishing, protected, or another restricted area to reach e.g. a slipway, beach , or any other destination for mariners, the implementation of a fairway and AtoN should be take into consideration.

In the case of a mammal (whales) protected area, it should be noted that these areas may change due to the natural movement of the mammals, and amendments to the layout of the fairway could be considered (may be very expensive, or impractical), or safety precautions taken through the promulgation of relevant Maritime Safety Information (MSI) messages.

The MBS and the IALA Recommendation O-139 - Marking of Man-Made Offshore Structures should be consulted as to the marking of such a fairway.

- Zoning

Special Marks with relevant pictograms should be considered to identify the zoning area, as well as the IALA Recommendation O-139 - Marking of Man-Made Offshore Structures should be consulted as to the marking of such areas.

- Split in fairway / channel

In the design of a fairway, and where a split is required,  e.g. a preferred channel, the following should be taken into consideration.

A split in a fairway / channel can be identified by:

- A traffic separation scheme

- Preferred channel marking

- Incoming traffic from a different channel with mandatory obligations to choose one direction (“T” connection)

- Fairways Intersect

Note: A split in a fairway / channel should only be established when safety of navigation in the area can thereby be clearly improved (IMO Resolution A.572(14)).

- Limited width of Fairway (swinging circle / Resilient/elastic type)

In a fairway with limited width, consideration should be taken that floating AtoN positions might be subjected to variations (swinging circle) because of depth, tides, current and mooring types.  The mooring system must be able to maintain the floating aid in a sufficiently accurate position for it to perform its function as an AtoN in order to ensure that the effective width of the fairway is not reduced.  The use of resilient / elastic buoys are particularly suitable in fairways where the swinging radius is limited by the width of the fairway and heavy vessel traffic.

- IMO International Regulations for Preventing Collisions at Sea 1972 (COLREGS)

COLREGS apply to all vessels upon the high seas and in all waters connected therewith navigable by seagoing vessels.

Narrow and shallow waters in waterways such as channels and fairways imposes a vessel her ability to manoeuvre, according to the nature of her work, or because of her draught in relation to the available depth and width of navigable water, or severely restrict her ability to deviate from the course she is following.

Also, ship-to-ship interaction can cause vessels to drift toward each other, resulting in a risk of collision.

The design of the fairway should ensure ship’s safe manoeuvring and assess the navigational risk to improve the safety of ship underway.

The following are some examples of the COLREGS rules that an AtoN  competent  authority may have to consider when designing a fairway.

Rule 1 - Nothing in these rules shall interfere with the operation of special rules made by an appropriate author for roadsteads, harbours, rivers, lakes or inland waterways connected with the high seas and navigable by seagoing vessels.

Rule 6 Safe speed.  Every vessel shall at all times proceed at a safe speed so that she can take proper and effective action to avoid collision and be stopped within a distance appropriate to the prevailing circumstance and conditions, taken into account the draught in relation to the available depth of water.

Rule 9 Narrow channels - Vessels less than 20 meters or sailing vessel shall not impede a vessel which can safely navigate within a narrow channel. Fishing vessels shall not impede. Vessels shall avoid anchoring. Consider options for smaller vessels, fishing vessels, emergency anchorages, etc.

A vessel nearing a bend or an area of a narrow channel or fairway where other vessels may be obscured by an intervening obstruction, shall navigate with particular alertness and caution and shall sound the appropriate signal.

Rule 10 Traffic Separation Schemes – multiple lanes, higher risks at entrance and terminus, mandatory usage by certain vessel types/sizes.

Rule 18 Responsibilities between vessels - any vessel other than a vessel not under command or a vessel restricted in her ability to manoeuvre shall, if the circumstances of the case admit, avoid impeding the safe passage of a vessel constrained by her draught exhibiting the signals.

Rule 28 Vessels constrained by their draught – must navigate in the channel.

- Audible signals

If the conditions include limited visibility (e.g. 10 days of visibility under 1 nautical mile (NM) per year)) installation of an audible, or sound signal may be justified.

- Background environment (height & colour of buildings, etc.)

The visibility of a visual AtoN may be  affected during day-and night-time by the type of background such as lighting, vegetation, snow, buildings (height & colour), etc. and should be considered in the design of a fairway.  Refer to the IALA NAVGUIDE and relevant IALA Recommendations and Guidelines in this regard.

- Nature of Seabed

The nature of the seabed may have an influence on the type of mooring lines and components, including the sinkers of buoys to be deployed.  Seabed composition can additionally have an influence on the designed positioning when laying the anchors / sinkers.

In areas with dynamic sediments, studies should be considered in the design of AtoN.

- Two-way traffic

Should markings be required to indicate two-way traffic areas on a fairway, appropriate AtoN are to be used to avoid misunderstanding by the users, noting that there are no specific AtoN relating to two-way traffic.

The MBS and the IALA Recommendation O-139 on the Marking of Man-Made Offshore Structures should be consulted as to how to mark such areas.

- Other local inland navigation rules or instructions, e.g. the European Code for Signs and Signals on Inland Waterways (SIGNI)

Refer to point 6.1.

# PERFORMANCE PARAMETERS OF AtoN SYSTEMS

## Positioning Accuracy

### Radionavigation Systems

The position accuracy for a vessel using radionavigation systems can be assumed to be 10m (95%) or better therefore, the accuracy of the chart has to be taken into account when using radionavigation systems.

### Visual AtoN

Generally, visual AtoN are not used to provide absolute accuracies, however, and importantly, they provide a good relative accuracy.  Thus, they are a good tool to determine the vessel’s position relative to relevant objects, such as fairway boundaries and hazards.

Due to differing mooring arrangements, the position accuracy of floating short range AtoN is sometimes difficult to define.  Floating AtoN positions are subject to variation because of water depth, tides, current, mooring type, and the capabilities of the servicing vessel in positioning the AtoN anchoring device.

## Redundancy

Reliance upon a single AtoN may result in a higher availability requirement which may prove difficult for the provider to meet.  Therefore, implementation of multiple AtoN should be considered to provide redundancy and reduce risk.

Duplication of the navigational functions of a single AtoN may be appropriate to provide a degree of redundancy to avoid the excessive cost of emergency repairs.  Moreover, temporary duplication may be provided when new or alternative types of aids are being introduced in order to allow a safe transition period.

## Perception

### General

When designing a fairway, the distance from which the AtoN can be detected, recognised and identified by the mariner, is a critical consideration.  For visual perception, this is called the useful range.

The useful range does not only depend upon the properties of the AtoN itself.  The atmosphere and the human eye are the other determining factors.  Therefore, useful range can be calculated using characteristics of the AtoN, atmosphere and the human eye (this is further determined by the experience of the observer).  The conspicuity of an object is also relevant.  An object is conspicuous if it appears outstanding in a complex visual scene.

To identify an AtoN, it is important that the visual information provided by the AtoN is verified.  The mariner achieves this task by comparing the characteristics of the AtoN (e.g. shape, colour, daymark, lettering and light character).  This process can take time due to interference by intermittent influences such as wave movement and visibility.

Audible signals, also referred to as sound signals, should only be used as a hazard warning.  These hazards refer to certain man-made structures such as offshore structures, renewable energy infrastructure, bridges, breakwaters, and isolated AtoN. The competent authority shall determine whether a hazard requires an audible signal.  Where provided, audible signals for navigation hazards should have a nominal range of at least 1 NM. (Refer to Recommendation E-109 and Guideline G1090 On The Use of Audible Signals)

### Lights

An AtoN light can be defined by its intensity, colour and divergence.  When this light is measured in its practical application a further parameter is derived; the luminous range.  The luminous range is the distance from which, under defined conditions, the user can identify the light.  To identify the light, the mariner must be able to confirm its colour and character.

The prevailing visibility conditions will vary for different geographical locations.  Therefore, when selecting an AtoN light, these conditions should be taken into consideration.  The nominal range will be promulgated in nautical charts and publications for mariners, lists of lights etc.

The IALA E-200 series of Recommendations provides further information on the determination and measurement of marine signal lights.

### Daymarks

The distance at which a daymark can be identified depends on size, shape, colour, contrast to the background, environmental conditions, background and geographical range.  The object can be normally identified when it subtends, at the eye, an angle of more than 3’ (three minutes of arc).

The contrast between the background and the AtoN depends on the:

chromaticity of the paint of the AtoN;

specific meteorological visibility in the area;

colour and illumination of the background;

conspicuity.

Reflective sheets / retro-reflective material on the aid can be used at night to enhance the daymark in low visibility conditions.

### Radar

The perception of an AtoN by a vessel borne radar is determined by the:

height of the vessel’s radar;

height of AtoN above actual sea level;

Radar Cross Section of the AtoN (including any radar reflector);

environmental conditions:

radar clutter due to sea state and weather;

movement of AtoN due to environmental conditions;

active radar devices on the AtoN.

### Automatic Identification System (AIS)

Perception can be improved by means of AIS on AtoN, provided that the vessel’s on-board equipment allows presentation of such information.

Some of the benefits of AIS include:

unambiguous indication of AtoN identity;

day/night and all weather operation;

greater range than most visual signals;

enhanced perception, showing the position on the vessel’s electronic chart;

verification of the integrity of the aid, including Off-Position and operational status indication / malfunction alert to the mariner and AtoN provider;

additional broadcast of meteorological and hydrological data and safety related information in real time if suitable equipment is fitted to the AtoN.

More details on AIS and its implementation as an AtoN can be found in IALA Recommendation A-126 on the use of AIS in Marine AtoN Services.

# LAYOUT OF AtoN FOR MARKING A FAIRWAY

## General

When considering the design of a system of AtoN, the MBS (new name?) must be adhered to.  However, for inland waterways, there may be different legislation, rules and marking systems established by national authorities; for example, SIGNI.  Any risk of conflict or confusion between the two systems (IALA MBS and that of the inland waterway) should be avoided as far as possible.

The Hydrographic Office is responsible to mark AtoN on nautical charts (ENC and paper charts), whilst the AtoN provider is responsible for verifying that AtoN are identified and marked on these charts..

In narrow, winding or meandering passages, it may be difficult for mariners to correlate the vessel’s position with chart information in a timely manner.  In these circumstances, visual AtoN will be the primary means of navigation.

## AtoN Marking the Fairway Boundaries

The following general principles apply to the design of fairways:

A fairway shall be marked by lateral and cardinal marks.

There shall be AtoN at least at bends and junctions of the fairway.

Lit AtoN should be generally used for:

the beginning and end of the fairway;

changes of direction.

AtoN should be spaced evenly along the fairway, where practicable.

In general, the useful range of buoys at day and night should be greater than the distance between the buoys.  The AtoN appearance on the vessel’s radar screen should also be considered.

The distances between unlit AtoN are based on their size and daytime visibility.

The AtoN on one or both sides of the fairway, should be positioned equal distance from the central axis of the fairway, where practicable.

The use of synchronized and sequential lights should be considered.

If high navigational accuracy or a very clearly distinct fairway with continuous buoyage is required, ideally, the AtoN marking the fairway shall be established as pairs (‘gates’); however, if this degree of accuracy is not required, then a ‘staggered’ arrangement of AtoN could be considered.  Moreover, AtoN could be positioned to one side of the fairway only, or where relevant, marking the deepest part of the fairway with safe water marks.

If a fixed AtoN separation distance is required, then the appropriate mark and size has to be established.  If a certain type of mark/size of AtoN is preferred for a particular purpose, then the separation distance of the AtoN has to be determined.  Usually, this will be an iterative process.  More than one option should be examined and assessed against the level of risk, from the navigational, economical, technical and operational perspectives.

In general, a high density of fairway AtoN ensures an easy and more accurate level of navigation.  However, there is a saturation point where adding AtoN does not help positioning any further.  To find the ideal AtoN density, simulation and risk assessment will be necessary, or at least very useful in fairway design.

To make AtoN service provision more economical, providers should consider defining a restricted number of various types and classes of AtoN (by size and shape) with certain identification distances and useful ranges of the lights.  Thus, the most suitable AtoN can be chosen from this ‘toolbox’ instead of designing a new one for each case.

## Other AtoN Marks in THE APPROACH TO, OR IN the Fairway

In addition to marking the boundaries of the fairway, short range AtoN and electronic AtoN may be used in the right mix to indicate:

critical points;

the centre of the fairway;

change of direction;

marking of isolated dangers;

marking of different areas;

marking specific type of objects, e.g. Ocean Data Acquisition Systems (ODAS) and wrecks (fixed and / or moving).

The following marks can be used:

Lateral, cardinal or safe water marks, which are physically deployed directly in the fairway, or on the point they indicate (or as close as possible to it).

Special Mark applications, both in the approach to, and / or in the fairway – refer to the MBS what the Special Mark applications comprise

These can be floating (buoys) or fixed (beacons) AtoN.

### Approach to a fairway

The phase of navigation called restricted waters, relates to the approach to a harbour, restricted waters, or a fairway, noting that although the harbour approach is an aspect of the restricted waters phase, it is treated separately as its own phase.  Consideration could be given to position a safe water mark in an appropriate position to indicate the approach to a fairway channel.

### Audible signals

Audible signals may also be used to augment the AtoN identification, both lighted and unlighted, to enhance their effectiveness to the mariner in reduced visibility.  Audible signals on buoys should be used to warn mariners of a particular hazard, such as proximity to shoals, rocks or other hazards; or to alert the mariner to a change in navigational requirements, such as the entrance to a restricted channel.  Where electronic audible signals are used to augment buoys, they should have a nominal range of 0.25 to 0.5 NM (refer to Guideline G1090 - Use of Audible Signals).

### Radar Beacons

The use of radar beacons (racons) could be considered as it enhances the detection and identification of AtoN,  thereby improving their effectiveness to the mariner in reduced visibility.  Placing a racon on safe water mark in the approach to a fairway / channel, and / or bends and junctions of the fairway could be considered, noting that racons are not limited to be placed on buoys only.

### MAtoN

MAtoN may be used to mark moving / drifting applications and would not normally be a permanent part of a marked fairway.

Refer to IALA Guideline XXXX - Use of Mobile Aids to Navigation

### Tide lanterns

Although a tide lantern is not an AtoN, it provides information to the mariner on the direction of the tide, e.g. on estuaries and may be a consideration -  refer to Annex C

### traffic separation scheme (TSS)

A Traffic Separation Scheme (TSS) is a routeing measure aimed at the separation of opposing streams of traffic by appropriate means and by the establishment of traffic lanes.  A TSS is an important instrument for managing shipping traffic and are intended to achieve the most unambiguous traffic flow possible.  In a TSS, shipping traffic in the opposite direction each has its own waterway.  This principle is applied in busy sea areas and ensures fewer intersecting movements and thus a clearer and safer traffic image.  The connecting lines between TSS form a route structure which is followed by the majority of ships.  In practice, mainly large ships make use of a TSS at it provides for a safe structure to their sea voyage.  TSS’ are less suitable for small recreational craft.

A TSS  is usually marked with safe water and / or lateral marking, unless the use of conventional channel marking may cause confusion, in which case Special Marks are used.

### Bridges, overhanging cables, etc.

Fixed bridges and other structures, including floating bridges, overhead pipelines as well as structures under construction that cross fairways should be marked to ensure their safety and that of vessels navigating beneath them, for example by reason of limited headroom, water depth or the possibility of collision.

IALA Recommendation O-113 - Marking of Fixed Bridges over Navigable Waters provides further guidance on the marking of fixed bridges and other structures over navigable waters.

### Breakwaters, surface- and underwater

Fairways and / or channels in many cases provide safe passage to vessels entering and exciting a Port or Harbour.  It is therefore necessary to also take the following into consideration in the design thereof. Breakwaters are usually marked using a combination of lateral marks to assist approaching vessels in navigating between breakwaters.  In those instances where both “above water”, partially or wholly submerged breakwaters present a hazard to navigation, a small network of special marks (fixed and/or floating) spaced at an equal distance apart, could provide an appropriate and recognised means of marking these hazards.

The unlit portion of the breakwater (in between AtoN) could present a hazard to navigation for mariners.  Pathway, or street lighting could be considered to provide a non-conventional means to maximize the area illuminated of a breakwater, thus increasing safety for the mariner (refer to IALA Recommendation O-139 and IALA Guideline G1078 refer.

### Other boundary areas

Other boundary areas adjacent, or close to a fairway, e.g. where dredging material is deposited, an anchorage area, offshore renewable energy fields, waiting and manoeuvring areas, etc. and which are usually marked by Special Marks, would need to be taken into consideration to ensure that there is no interference or ambiguity between those markings and that of the fairway.

## Fixed visual AtoN Outside the Fairway

### Leading and Transit lines

Leading lines (lit or unlit) generally provide high accuracy for the middle of the fairway and or .

They can be established when there are straight stretches in the fairway or if:

the middle of a fairway needs to be indicated;

may reduce the need for physical AtoN;

other AtoN could be affected by ice, severe weather or tide;

there is a channel inside the fairway, which has to be used by vessels with deep drafts;

strong cross currents occur (for instance in harbour entrances);

leading and / or transit lines can also be used to mark the boundaries of the fairway, provided this function is clearly shown in the relevant charts (see example in ANNEX G).

The preliminary decision in designing a leading and / or transit line is to define length and breadth of the section of fairway to be defined by the line.  Generally, it is costly to build a leading line to serve a long channel, as the rear leading light must be of sufficient height to be clearly visible above the front structure.  Leading marks should also be large enough to be visible from the far end of the section of fairway marked.  Both these conditions result in increases to the required height of the rear structure marking a long channel.

More details can be found in IALA Guideline No. 1023 for the design of leading lines and in the IALA NAVGUIDE.

### Sector lights

A sector light is an AtoN that displays different colours and/or rhythms over designated arcs.  The colour of the light provides directional / positional information to the mariner.

A sector, or a limit between two sectors, may indicate a fairway, a turning point, a junction with other fairways, a hazard or something else of importance for the navigator.

When a fairway is covered by a sector light, colours indicating safe passage and danger areas are defined in the MBS.

A sector light may indicate one or more of the following boundaries and aspects of a fairway; for example:

position at which a change of course should be made;

location of shoals, banks, etc.;

an area or position (e.g. an anchorage);

the deepest part of a fairway.

More details can be found in IALA Guideline No. 1041 on Sector Lights.

### Major floating AtoN

Major floating aids include lightvessels, light floats and large navigational buoys.

Consideration could be given to deploying major floating aids.  Such aids are generally deployed at critical locations, intended to mark approaches from offshore areas, where shipping traffic concentrations are high.  They may provide a platform for other AtoN such as racons and / or AIS AtoN.

### Position, navigation and timing (PNT) and Differential Systems

Position, navigation and timing (PNT) and Differential Global Navigation Satellite System (DGNSS) are widely used both for navigation and as an AtoN.  The availability and type of positioning fixing system in a particular area should be considered.

### Vessel Traffic Services

Vessel Traffic Services (VTS) managing traffic volume and density in a fairway may mitigate some of the risk and therefore may influence the type and amount of AtoN to be deployed.

### Pilotage

Pilotage is a specialised service to navigation, specifically in restricted and / or navigationally challenging waters. The pilot draws on local knowledge related to geographic and hydrographic points of interest. Deployment of AtoN may be designed considering pilots’ knowledge or requirements.

# DESIGN METHODOLOGY/PROCEDURE

## A simple procedure for the establishment of a Marked Fairway

The design of AtoN marking a fairway which is formed by straight lines and bends can be undertaken considering the following:  Jiangna (intersessional)

- Establish a conspicuous AtoN, or pair of AtoN, at the beginning of the fairway.

Place AtoN at points where:

  - vessels have to alter their course;

  - the fairway boundary line or the middle line has a bend or curve;

  - critical shallows and rocks, or other hazards, form the boundary of the fairway;

  - fairway and channel intersect, or split.

Distribute buoys between these points with regard to the distance at which they can be detected and identified (refer to the section on perception of AtoN in this Guideline (section 4.3) and other relevant IALA documents).

The distance, at which the marks should be detected and identified, may be different in relation to the length of the fairway for different cases as follows:

after the initial assumption of requirements made from section 4.3, the buoy separation distance should now be chosen as follows; when the nearest buoy is being approached, the mariner should be able to see at least the next buoy along the fairway (the next two buoys in more accurate applications)

## Channel Design and Maintenance

Fairway planners in the relevant authorities should consider AtoN solutions to minimise the amount of dredging and surveying that is required in fairway construction and maintenance.  Further details on these matters can be found in the PIANC ‘Report WG 121- 2014, Harbour approach channels design guideline.’

## Risk assessment

Advice on risk assessment and risk management can be found in the IALA Guidelines  G1018 -Risk Management, G1123 – The Use of IALA Waterway Risk Assessment Programme (IWRAP MkII), G1124 – The Use of Ports and Waterways Safety Assessment (PAWSA) MkII Tool and s. G1138 - The Use of the Simplified IALA Risk Assessment Method (SIRA)

## Simulation

Simulation can be used as a tool in the design and planning of fairways.  Refer to IALA Recommendation O-138 - The Use of GIS and Simulation by AtoN Authorities and IALA Guideline 1058 - The use of simulation as a tool for waterway design and AtoN planning, for discussions on the use of simulation in fairway design and placement of AtoN.

# EXAMPLES OF MARKING OF FAIRWAYS

Some examples for the marking of fairways are given in ANNEX C to ANNEX G.

# CONCLUSION

When designing an AtoN system for a fairway, many considerations are necessary.  The requirements can be defined in parameters such as accuracy, reliability, conspicuity, etc.   The parameters vary depending on the type of fairway, the vessel traffic and on other factors such as risks.  The design must include the considerations and requirements of international bodies such as the IMO, IHO, PIANC and IALA.

e-Navigation will integrate information on visual AtoN in the fairway with all the information available on the bridge of a vessel and thus contribute to the optimal use of the fairway.  This is subject to further development, as currently many vessels are not equipped to use e-Navigation applications.  A systematic approach to the design of AtoN systems and fairways is an initial step towards AtoN integration in e-Navigation.

Continuous improvement of marking principles and AtoN positioning will optimise the fairway design, drive efficiencies and protect the marine environment.

# ACRONYMS & DEFINITIONS

## Acronyms (Check For Completeness)

AIS	Automatic Identification System

AtoN	Marine Aid(s) to Navigation

COLREGS	IMO International Regulations for Preventing Collisions at Sea

DGPS	Differential Global Positioning System

DGNSS	Differential Global Navigation Satellite System

DWT	Deadweight tonnage

ENC	Electronic Navigation Chart

EWF	Effective Width of the Fairway

GPS	Global Positioning System

HSC	High Speed Craft

IALA	International Association of Marine Aids to Navigation and Lighthouse Authoritie

IHO	International Hydrographic Organisation

IMO	International Maritime Organization (Acronym style)

m	metre

MARCOM	A PIANC technical working group

MAtoN	Mobile AtoN

MBS	IALA Maritime Buoyage System

MSI	Maritime Safety Information

MTTR	Mean Time To Repair

NM	Nautical mile(s)

ODAS	 Ocean Data Acquisition Systems

PEL	Port Entry Light

PIANC	The World Association for Waterborne Transport Infrastructure

PNT	Positioning, Navigation and Timing

Racon	Radar beacon

SC	Swinging Circle

SIGNI	Signs and Signals on Inland Waterways, UN, ECE, Resolution No 22

SOLAS	International Convention for the Safety of Life at Sea

S-44	Standards for Hydrographic Surveys (IHO)

TEU	Twenty-foot equivalent unit

TSS	Traffic Separation Scheme

VTS	Vessel Traffic Service(s)

WIG	Wing in Ground

## Definitions (Check For Completeness

Definitions

| Term/Acronym | Definition/Expansion |
| --- | --- |
| Absolute Accuracy | The accuracy of a position estimate with respect to the geographic or geodetic co-ordinates of the Earth |
| Absolute Position | A position estimate with respect to the geographic or geodetic co-ordinates of the Earth |
| AtoN Provider | Any organisation, public or private, which is required to deploy AtoN as part of its duties |
| Availability | The percentage of time that an aid, or system of aids, is performing a required function under stated conditions. Non-availability can be caused by scheduled and/or unscheduled interruptions |
| Canal | A narrow stretch of inland waterway created artificially to facilitate navigation |
| Channel | Part of an (otherwise shallow) water having sufficient depth for vessels and designated or customarily used for vessels. The borders of the channel may be defined by natural or artificial banks or by AtoN |
| Conspicuity | The ability of an object to stand out from its surroundings |
| Continuity | The probability that, assuming a fault-free receiver, a user will be able to determine position with specified accuracy and is able to monitor the integrity of the determined position over the (short) time interval applicable for a particular operation within a limited part of the coverage area |
| e-Navigation | The harmonised collection, integration, exchange, presentation and analysis of maritime information on board and ashore by electronic means to enhance berth to berth navigation and related services for safety and security at sea and protection of the marine environment |
| Fairway | A charted and marked waterway recommended by a competent authority |
| High Speed Craft (HSC) | Craft capable of maximum speed equal to or exceeding 3.70.1667 ms-1 where:  = volume of displacement corresponding to the design waterline (m3) This excludes WIG vessels |
| Integrity | The ability to provide users with warnings within a specified time when a system should not be used for navigation |
| Luminous range | The distance from which, under defined conditions, the user can identify the light |
| Nominal range | Luminous range in a homogeneous atmosphere in which the meteorological visibility is 10 NM |
| Radionavigation | Radiodetermination used for the purpose of navigation, including obstruction warning |
| Radionavigation system | Navigational system that uses radiofrequencies to determine a position |
| Relative Accuracy | The accuracy with which a user can determine position relative to that of another user of the same navigation system at the same time |
| Reliability | Ability of a device, or system, to satisfy the requirements of its intended use within defined limits, and for a stated period of time |
| Useful range | The practical convenient range for a mariner to identify an AtoN |
| Waterways | Navigable waters |
| Wing in Ground (WIG) | Vessels, the hull of which is supported completely clear above the water surface in non-displacement mode, by aerodynamic forces generated by ground effect |

RELEVANT IALA DOCUMENTATION

IALA AtoN GUIDE (‘NAVGUIDE’)

This document provides overall guidance, in particular chapters on AtoN and Risk Management.

IALA RECOMMENDATIONS (Check For Completeness

R-101 on Maritime Radar Beacons

E-105 on the need to follow national and international standards

E-110 for the Rhythmic Characters of Lights on AtoN

E-111 for Port Traffic Signals

E-112 for Leading Lights

O-113 for the Marking of fixed bridges over navigable waters

A-126 on the use of AIS in Marine AtoN Services

O-130 on categorization and availability objectives for short range AtoN

O-138 on the Use of GIS and Simulation by AtoN Authorities

E-200-0 on Marine Signal Lights - Overview

E-200-1 on Marine Signal Lights - Colours

E-200-2 on Marine Signal Lights - Calculation, Definition and Notation of Luminous Range

E-200-4 on Marine Signal Lights - Determination and Calculation of Effective Intensity

E-200-5 on Marine Signal Lights - Estimation of the Performance of Optical Apparatus

IALA GUIDELINES (Check For Completeness

- 1010 on Racon range performance

1018 on Risk Management

1023 for the Design of Leading Lines

1033 on the Provision of AtoN for different classes of vessels, including high speed craft

1041 on Sector Lights

1046 on a Response Plan for the Marking of New Wrecks

1051 on the Provision of AtoN in Built-up Areas

1058 on the Use of Simulation as a Tool for Waterway Design and AtoN Planning

1061 on Light Application – Illumination of Structures

Other IALA Recommendations and Guidelines on VTS, AIS, Racons and DGPS should also be considered.

DRIFT DETECTION – JAPANESE METHOD

Extract from Japanese Method (contribution to Section 6 ‘Fairway Layout and Channel Width’ to the Draft of the report of PIANC MARCOM Working Group 49 ‘Horizontal and vertical dimensions of channels’).

Channel Classification and Channel Width Elements

Channel width can generally be assumed to consist of the following fundamental elements:

Wmi=Wmβ+Wmy+WmS

Width of basic manoeuvring lane:

where

Wmβ is the width needed against wind and current forces

Wmy is thewidth needed against yawing motion

WmS is thewidth needed for drift detection.

Additional width needed against bank effect forces: Wbi;

Additional width needed against two-vessel interaction forces in passing condition: Wc;

Additional width needed against two-vessel interaction forces in overtaking condition: Wov.

It is noted that the total channel width may be obtained by summing up necessary elements mentioned above, not necessarily all, according to design purposes and detailed conditions for the subject channel.

Equipment and Systems of Navigation Aids

A vessel sailing in a channel usually makes some amount of drift from its course line due to various causes together with external forces even if a vessel handler does believe that his vessel is running ‘on track’.  Drift detection may be impossible when the drift amount is small, but a vessel handler can recognize a drift when a vessel makes some considerable amount of lateral deviation from its course line as shown in Figure 1.  The drift amount which a vessel handler can detect depends on the type of equipment and systems of navigation aids together with the way in which they are utilized.  It should be noted that the drift quantity to be detected plays an important role in the design of channel width.  A narrower width may generally be adopted for a channel with a higher level of equipment and systems where the drift detection can be made more easily.  Moreover, a vessel of larger size may be allowed to sail by installing a higher level of equipment and systems even in an existing channel which cannot be widened due to some topographical limitations.

Undetectable zone

As for the drift detection, in general, three types of equipment and systems of navigation aids are available as follows.

Drift detection by observing light buoys ahead on both sides of channel with the naked eye.

Drift detection by observing light buoys ahead on both sides of channel with RADAR.

Drift detection by GPS or D-GPS.

The channel width needed for the drift detection can be estimated with a detectable deviation from a course line with the use of the equipment and systems of navigation aids, as above, in each channel to be designed.

Width Needed for Drift Detection

As for the drift detection means by observing light buoys ahead on both sides of channel either with the naked eye or with radar, the channel width needed for the drift detection may be estimated on the basis of an angle made by two lines from a vessel to two buoys ahead on both sides θ shown in Figure 2, which is defined as:

θ=2arctanwbuoy2LF

Channel width required for drift detection using the naked eye or radar

where

Wbuoy is the clearance between two buoys

LF is the distance along channel centre line from vessel to light buoys ahead.

In Equation 2, amounts of Wbuoy and LF are given in the following manner according to the subject channel of a newly designed channel or an existing channel.

Detectable manoeuvring lane for light buoys on the both sides of fairway

Newly designed channel

In this case, Wbuoy is to be the channel width finally determined which is unknown in the beginning of channel width design.  Therefore, an iteration technique as described in section 6.1 is employed where assuming some amount of Wbuoy iterations are made until computed value is to be finally identical to the assumed one.  Amounts of LF may be set empirically to be 7LOA for a one-way channel and (3.5-7)LOA for a both-way channel.  Moreover, LF may be set with a value of (0.5 – 1.0) times of the distance between two successive buoys along a channel when buoy locations are given.

Existing channel

Wbuoy is set with a clearance between two buoys on both sides of the channel, and LF is set with a distance between two successive buoys along a channel.  However, when the distance between two successive buoys is thought to be somewhat long or more, LF may be set, in a similar way to the case of newly designed channel, to be  7LOA for a one-way channel and (3.5-7)LOA for a both-way channel.

Drift Detection by Observing Light Buoys with Naked Eyes

As shown in Figure 2, an angle made by two lines of a channel centre line and a line from a vessel to a midpoint of two buoys is denoted with α.  A concept of the maximum deviation is introduced which is defined that almost all vessel handlers are able to recognize a drift from its course line.  Corresponding to this maximum deviation, the angle of α is denoted with αmax as shown in Figure 2.  Making use of the above concept of αmax, the channel width needed for the drift detection by observing light buoys with the naked eye can be calculated by:

Wmα=LFtanαmax

Channel width needed for drift detection by observing light buoys with the naked eye

Where αmax may practically be estimated with an empirical formula developed on the basis of statistical data by full scale experiments, which is given by:

αmax=0.00176θ2+0.008θ+2.21372

Definition of αmax

Drift Detection by Observing Light Buoys with RADAR

The channel width needed for the drift detection by observing light buoys with RADAR can be calculated by:

WmR=wbuoysinθsinγ

Channel width needed for drift detection by observing light buoys with radar

Where:

γ is the error of direction observation by radar

The following expressions are easily written from Equation 5 for two cases of γ = 2deg. and 1 deg. respectively.

WmR=0.0349wbuoysinθ

Channel width needed with error of direction of 2 degrees

WmR=0.0175wbuoysinθ

Channel width needed with error of direction of 1 degree

Drift detection by GPS

In vessel manoeuvring operations with the utilization of GPS, a vessel handler may judge and recognize the vessel position by an image of GPS information displayed on an electric chart.  Although image information on the electric chart is sufficiently accurate, the drift detection is made solely by perceiving a vessel movement on the display with the naked eye, and some amount of perception error should be taken into account.  As for drift detection by means of GPS, an assumption is made for the above perception error to be a half of the vessel’s breadth.  In addition, the margin of error for GPS information error is assumed to be 30 metres for a standard GPS and none for a DGPS.  Therefore, the channel width needed for drift detection by GPS and DGPS can be calculated by the following equations, respectively:

WmGPS=0.5B+30 unit:metre

Channel width required with drift detection by GPS

WmDGPS=0.5B unit:metre

Channel width required with drift detection by DGPS

In addition, it is noted that the channel width needed for drift detection by GPS should be designed with careful consideration of the risks involved in using GPS and GPS-related equipment.

References

Ohtsu,K., Yoshimura,Y., Hirano,M., Tsugane,M.and Takahashi,H.: Design standard for fairway in next generation.  Asia Navigation Conference 2006, No. 26, 2006.

The Japan Port and Harbour Association: Technical Standards and Commentaries of Port and Harbour Facilities in Japan, 2007. (in Japanese)

AtoN DESIGN - THE RIO DE LA PLATA NAVIGATION CHANNEL

The Rio de La Plata Navigation Channel is a partly straight and partly curved channel with buoys from a position of 129.1 miles seawards of the harbour entrance of Buenos Aires.

More data can be seen on the following chart.

Some basic data

Waterway in analysis

Length:	63.8 NM

Width:	100 m (one-way channel)

(there is a two-way channel sector of 160 m)

Design Depth:	34 feet

Tidal range	0.60 m

The Vessel Traffic

Total Traffic in the Area in 2006:	7.760 vessels

including

Container vessels:	1,726

Bulk Carriers:	3,134

General cargo:	    962

Oil Tanker:	1,386

Other:	   552

Maximum size of Vessels Dimensions

Length BP	Beam	Design Draught

Container vessel 	261 m	40 m	41 feet

Bulk carrier	260 m	42 m	42 feet

Tanker	241 m	32.2 m	49 feet

AtoN

The buoyage system is designed as ‘paired buoys’ with the following parameters:

Buoy separation distance	average 3000 m (max 5.000 m, min 1.100 m)

Buoy types	Floating buoys and Spar Buoys

Floating Buoy size above water level	4 m

Spar Buoys size above water level:	8 m

Additionally there are the following AtoN:

- DGPS coverage (on demand- private service)

- VTS and AIS control by Prefectura Naval Argentina (National Coast Guard)

- Mandatory Pilotage services

Tide Lanterns

Tide lanterns are deployed in 10 buoys; 9 on Port Hand buoys, and one on a Safe Water Mark buoy and provide information to the mariner on the direction of the tide.  The buoys are equipped with a rudder and change position when the flow of the tide changes, displaying a fixed, low intensity green light to vessels transiting from the open sea side when the current direction is “going out of the estuary” and vice versa a red light when the current direction is “coming into the estuary”

The lens of the lantern has two 60 degrees (one green and other red) and two 120 degrees dark sectors

The green and red sectors need to be aligned with the buoy rudder.

Tide lanterns are placed in the middle of the buoy super structure to ensure that there is no interference with the main lantern.

Tide Lantern (fixed, low intensity light)

Main lantern /flash)

AtoN design on the Rio de la Plata

AtoN DESIGN - FINLAND

MUSSALO

FAIRWAY DATA Alignment and buoyage: The channel starts SW of Kotka Lighthouse and runs in a NE direction.  E of the edge mark Kaakkoniemi it turns northwards towards the Deep Harbour of Kotka.  Length 51 km/28 nm.  5 navigation lines, marked by boards and sector lights.  Cardinal marks in the channel, lateral marks in the harbour.  Lit.  Dimensions: Design vessel: bulk carrier 125 000 DWT, l = 300 m, b = 48 m, t = 15.3 m. Authorised draught 15.3 m, safe clearance depths (MW 90) 18.4 m, along the navigation line in the harbour 17.5 m.  Minimum width 200 m. Anchorage areas etc.: Anchorage NE of Kaunissaari, close to crossroads.  Safe clearance depth 18.4 m.

NAVIGABILITY Navigational conditions: The approach as far as the island Viikarinsaari consists of open sea, unsheltered against E–S–SW winds.  Navigation may be hampered by strong winds and sea state.  The channel is at its narrowest at the edge mark Elo 2, where it is only 510 m wide.  Along the navigation line in the harbour the width is 200 m.  RECOMMENDATIONS (channel) Speed: Vessels sailing at maximum draught should take the squat effect into account; design speed 16.5 knots (Sc 18.4) in the approach, 13 knots (Sc 17.5 m) in the harbour.

TRAFFIC SERVICE Pilotage: Pilot order, tel. +358 204 48 5604.  Pilot boarding position 60°07.49\, 26°29.65\.  Pilotage distance 25 nm.  VTS: Kotka VTS, VHF Channel 67.  Tugs: Provided by Alfons Håkans Ltd.  Ordered by pilot, if required

PORT Quays: A Quay: length 609 m, safe clearance depth 15.0 - 17.5 m; B Quay: length 500 m, safe clearance depth 11.7 m; C Quay: length 936 m, safe clearance depth 11.7 m; Liquid Bulk Terminal, berth N 1: length 69 m, safe clearance depth 15.0 m; Liquid Bulk Terminal, berth N 2: length 60 m, safe clearance depth 11.5 m. Cargo handling: A Quay – four cranes (40 t), B Quay – two cranes (40 t) and a mobile crane, C Quay – four cranes (50 t).  Harbour basin: Speed of vessel to be regulated to ensure that no harm or damage is caused.

CONTACTS VTS: Kotka VTS tel. +358 204 48 5604 fax +358 204 48 5600 Port: Port of Kotka tel. +358 5 2344 280 fax +358 5 2181 375

Mussalo Channel

Kotka inner harbours

Mussalo Harbour

RAUMA

Rauma Channel

CHANNEL DATA

Alignment and buoyage: V of Rauma Lighthouse – port.  Four lines.  Length approx. 26 km/14 nautical miles.  Lateral marks.  Lit.

Dimensions: Design vessel: Ro-Ro vessel, L = 210 m, B = 30 m, T = 10.0 m.  Maximum authorised draught 10.0 m, safe clearance depth (MW95) in the outer channel -12.0 m, in the inner channel -11.5 m.  Minimum width 120 m, in the passage of Kovankivet 160 m; minimum bend radius 1000 m; design speed in dredged passages 12 knots.

Anchorage and other special areas: In the outer channel, anchorage W of Rauma Lighthouse; beware of cable S of lighthouse.  In the inner channel, anchorage and passage either in the widened area N of Rihtniemi or SW of Iso Järviluoto, approx. 1.5 km before arrival into port.

NAVIGABILITY

Navigational conditions: The outer channel to Rihtniemi is unsheltered and open to S-W-N winds.  From Rihtniemi on, the channel continues as a narrow and densely marked channel, sheltered by isles, islands and mainland.  Cross currents, which make the manoeuvring of large vessels more difficult, may occur when navigating the Urmluoto line in the passage of Kovankivet.  Strong side winds also aggravate the side drift.

Ice conditions: In winter ice fields tend to move in the channel, outside Hylkikarta.  Ice movement may cause buoys to be pressed beneath the surface and their lighting devices may be damaged.

OPERATIONAL RECOMMENDATIONS (channel and harbour)

Wind: Max. speed of drifting wind gusts 18 m/s in daytime and 15 m/s at night.  Limits lower for Ro-Ro vessels and vessels in ballast.  Max. wind gusts 11 m/s for vessels in ballast, larger than the design vessel.  Drifting wind means a wind which differs from the Urmluoto line by more than 30°.  Pilotage is discontinued when the wind speed exceeds 20 m/s.

Visibility: The Urmluoto lines should be visible at night.

Vessel-specific recommendations: Vessels larger than the design vessel and vessels with poor manoeuvrability are piloted only in daytime.

New authorised channel draught practice applied in the channel as from July 15th, 2005 (NtM 17/2005, 20.6.2005).

TRAFFIC SERVICE

Pilotage: Pilot order, tel. +358 204 48 6645. Pilot boarding position 61°07,5’, 21°10,4’.  Pilotage distance 10 nautical miles.

VTS: West Coast VTS, Channel 67

PORT

Quays: Petäjäs: length 445 m, safe clearance depth -11.0 m; Iso-Hakuni: 6 berths alongside/Ro-Ro berths, safe clearance depth -11.0 m; Oil harbour: safe clearance depth -9.15 m; Central harbour: length 665 m, Ro-Ro berth, safe clearance depth -6.70..-7.30 m; Laitsaari: length 246 m, safe clearance depth - 9.05 m; Inner harbour: 2 chemical piers, safe clearance depth -5.10..-7.05 m.

Cargo handling: Petäjäs: cranes 40 t, 45 t and 16 t, pneumatic grain suction device; Iso-Hakuni: cranes and reach stackers; Oil harbour: piping, pumping power 1000 t; Central harbour: crane 6 t. Vehicle mounted cranes (50 and 100 t) used in all parts of the port.

CONTACT INFORMATION

VTS: West Coast VTS, Pori	Tel. +358 204 48 6645 Fax +358 204 48 6646

Port: Port of Rauma, Rauma	Tel. +358 2 83 44 710 Fax +358 2 822 63 69

Port Director Hannu Asumalahti

Rauma Channel

Port of Rauma

AtoN DESIGN - LE HAVRE AND PORT 2000, FRANCE

Le Havre (1st port in France) is a deep-water port with day and night access for the largest container carriers (10,000 TEUs and more).

Located at the south end of the port, the facilities of the Le Havre oil port are made up of 8 specialised berths, including two for the reception of 230- 280,000 DWT tankers.

Recently, an external port (Port 2000 terminals) able to welcome and rapidly handle the largest container carriers in the world in optimal logistic and nautical conditions has been created.

The area is also one of the major places for pleasure and fishing craft in the region.

THE VESSEL TRAFFIC (PER YEAR)

Total traffic flow:	80 million tons

Container traffic flow:	2.5 million TEUs

Oil: 	40 million tons

Roll-on/roll-off services:	472 000 vehicles

Passengers: 	355 000

About 7 500 vessel calls

MAIN WATERWAY IN ANALYSIS

Length: 	12 NM from the landfall buoy to the main entrance

Width:	450 metres (two-ways channel)

PORT 2000 WATERWAY

Length:	3 NM

Width: 	350 metres (two-ways channel for vessels up to 55 m beam)

Design Depth: 	15 meters + tide

Tidal range:	8,00 metres

AtoN

The buoyage system is designed as ‘paired buoys’ with the following parameters:

Buoy separation distance average 1400 m (max 2 000 m in the entrance, minimum 1.000 m at Port 2000);

Buoy types: 18 Floating buoys and 3 fixed beacons (LH 17, 18 & 21) for the marking of the effective width of the fairway (EWF);

Floating Buoy size above water level: 3.5 m with a luminous range of 3 miles by night;

Beacons size above the highest water level: 5 m.

All the lights are synchronised by paired buoys and sequenced (timing method GPS)

Additionally, there are the following AtoN:

Leading lines covering the main channel up to the landfall buoy (front light height 36 meters and range 25 Miles by night rear light height 78meters and range 25 miles by night) operated night and day;

A very precise PEL Sector light with oscillating boundary for the Port 2000 channel (5 sectors in 5°) operated night and day;

Various Sector lights in the port;

DGPS coverage;

VTS and AIS control by The port of Le Havre Authority;

AIS Aton on the landfall buoy;

Mandatory pilotage services.

Port of Le Havre

AtoN DESIGN - THE ‘SEEKANAL ROSTOCK’

The Seekanal Rostock is a straight channel with buoys from a position 6 miles seawards of the harbour entrance of Rostock/ Warnemünde leading into the mouth of the river Warnow with following design data:

WATERWAY

Outer part:

Length:	5.7 NM

Width:	225 … 120 m

Depth:	≥ 14.5 m

Inner part:

Length:	1.5 NM

Width:	120 m

Depth:	≥ 14.5 m

VESSEL TRAFFIC

Total Traffic in the Area in 2006:	25.200 vessels including:

Passenger- and Ro/Ro Ferries:	13,000

Cargo vessels:	  4,500

Oil Tanker: 	  1,800

The area is also one of the major places for pleasure and fishing craft in the region.

VESSELS DIMENSIONS

Terms for two lane traffic passing 120 m width area:

- Beam	< 40 m – sum of both passing vessels and

Draught	< 8.5 m

If wind < Force 6 and both masters accept a two lane traffic passing and

Beam	< 60 m – sum of both passing vessels beams and

Draught	< 8.5 m

Vessel with draft > 8,5 m and using the leading line

X = 60 m – 0.5 * beam of this vessel

MAX beam of the meeting vessel X/5 (opposite direction)

All other vessels have to pass by ‘one lane traffic’ under the responsibility of the captain with assistance of VTS.

MAXIMUM SIZE OF VESSEL IN ‘SEEKANAL ROSTOCK’

Length	296.0 m

Beam	  32.0 m

Draft	  10.5 m

The channel is narrow in relation to the vessels dimensions.

AtoN

The buoyage system is designed as ‘paired buoys’ with the following parameters:

Buoy separation distance	Outer Part	1500 … 1000 m

Inner Part	600 m

Buoy type	Outer Part: deep water light buoy ‘LT 81’, steel mooring chains

Inner Part: hinge beacon with steel tube, with a minimal swinging circle radius, synchronised lights

Buoy size above water level	Outer Part:	2.5 x 5 m

Inner Part:	1 x 4 m

Notes

- EWF = effective width of fairway

SC = swinging circle

SC Tonne

EWF Bake

Middle Line Fairway

Canal section

EWF Bake

SC Tonne

Seekanal Rostock – canal section

The inner part of the canal is designed with Hinge Beacons because they have a smaller swinging circle and thus cause less reduction of the effective width of fairway.

Additionally there are the following AtoN:

Leading Lights over 4 NM, DGPS coverage, VTS coverage, AIS coverage;

The Leading Lights are synchronized (same character for front light and rear light).

To avoid misleading information in case of failure of one of the lights by mirroring a light with the same character on the water surface and be interpreted as a front light in the wrong place, in case of failure of either the front light or the rear light the other one will also be switched off.  This is ensured by connecting the remote control devices of the front light and the rear light.

Example for synchronized AtoN: Inner part of ‘Seekanal Rostock’

The Leading Lights are synchronized with the mole lights and the buoys as shown in Table 2.

Synchronization of AtoN

|  | 1s | 2s | 3s | 4s | Flash code |
| --- | --- | --- | --- | --- | --- |
| Leading lights | X | On | On | On | Occ. 4s |
| Mole lights at canal entrance | X | X | On | On | Iso. 4s |
| Articulated lights on buoys | X | X | X | On | Fl. 4s |

Sea Chart of a part of the outer and inner channel ‘Seekanal Rostock’

AtoN DESIGN - THE APPROACH TO MALMÖ, SWEDEN

The Port of Malmö is located on the west coast of the southern part of Sweden.  The port is approached from the sea through a channel marked by buoys, light buoys, lights and beacon, on the alignment of leading lights.  The channel layout is based on a simulation that was carried out in November 2006 in order to evaluate risk and conditions in terms of vessels types and sizes, tug capacity, port and channel layout.

Waterway

Outer part

Length: 	3.9 NM

Width:	The channel has been widened to 162 m for its full length.

Depth: 	13.5 m

Inner part

Length:	0.6 NM

Width:

Depth:	13.5 m

Vessel Traffic

Total Traffic in 2008: 	1102 arr/dep

Vessel Dimensions:	260 x 40 x 12.5 m

AtoN

The buoyage system is designed as ‘paired buoys’.

Buoy separation distance	Outer part: 0.3 NM

Inner part: 0.1 NM

All buoys in the channel have synchronized lights with the same frequency at the same time.  The light rhythm for the buoys in the channel is Q (0,2s + (0,8s) = 1s) on both sides of the channel.  The buoys are of type ‘S-7’ (light is 4 m above surface of water).  The placement and the synchronised flash on the buoys in the layout is an advantage for safe navigation during darkness as it is much easier to see as the darkness period is very short.

The position of the leading light has been optimized according to new channel layout and the blink rate has changed to Oc 6s.  There are two lights in line, marking the sides of the channel with the light rhythm Q.  The quality of the centre line has improved with a modern high visible light. This improves safety for navigation in restricted visibility.

These improvements have made navigation in darkness much safer.

Additionally

DGPS coverage, VTS, Mandatory pilotage services etc.

Outer part of channel - Malmo

Inner part of channel - Malmo

Entrance to Malmo Harbour

Malmo Harbour

For instance, fishing vessels and leisure craft

IALA Guideline No. 1033 ‘on the provision of AtoN for different classes of vessels, including high speed craft’ addresses the specific AtoN requirements of HSC.

IALA Guideline 1116