## GUIDELINES ON VTS OPERATOR WORKLOAD STAFFING LEVELS

This Working Paper has been produced by WG3 as a pro-forma for work to be carried out on the above subject (Task 8) over the next four sessions.  There is still considerable work to be done and it has been agreed that members will be carrying our their own research inter-sessionally, for contribution at VTS18 (March 2003).

----------------------------------

## Format of document

A.1		Introduction

A.1.1		Implementation of a VTS

A.1.2		VTS Personnel

A.1.3		VTS Personnel Positions

A.2		Scope of Guidelines

A.3		Aim

A.4		Objectives

A.5		International Regulations

B		Staffing Levels

B.1		Comparison of outcomes

B.2		Factors for Consideration

B.3		Extent of technological Automation

B.4		Communication requirements

B.5		Risks of poor performance

B.6		Navigational Assistance Service (NAS)

C		Factors affecting Workload

C.1		Technical / Equipment

C.2		Procedures

C.3		Environment

C.4		Human Resources

C.5		Interaction with Traffic

C.6		Interaction with Allied Services

Annexes	Risk Assessment

CCG formula – number of persons per workstation

Hours working at display

References to various Regulations

Scope

These Guidelines are only intended to provide guidance to Authorities intending to establish a VTS Centre(s) and to those Authorities with existing Centres who need to realise the ever-increasing changes in technology.  They are by no means prescriptive.  When using these Guidelines, readers are strongly recommended to make themselves familiar with the various works of Reference as shown in Annex xx.

# Introduction

IMO Resolution A.857(20) 'Guidelines for Vessel Traffic Services' (VTS) describes the skill and knowledge qualifications required by VTS operators to provide the required services to ensure the safety of navigation and protection of the environment.

The Guidelines are intended for application in both planned and existing VTS.  They provide guidance in determining how VTS Authorities can recruit, select and train personnel in order to carry out their tasks thereby providing the required VTS standards.

In planning and establishing a VTS, the Contracting Government(s) or the Competent Authority should:

- ensure that the VTS Authority is provided with the equipment and facilities necessary to effectively accomplish the objectives of the VTS and;

- ensure that the VTS Authority is provided with sufficient staff, appropriately qualified, suitably trained and capable of performing the tasks required, taking into consideration, the type and level of services to be provided and the current IMO Guidelines on the recruitment, qualifications and training of VTS operators (Annex 2).

Implementation of a VTS

The following steps need to be considered:

- Implementing a VTS - make a decision, or have made a decision to implement a VTS.

- Identification of VTS external and internal functions - identify and describe the detailed functions relevant to the proposed VTS.

- Assignation of VTS Centre functions according to how they are to be carried out in accordance with the VTS operation.

- Establishment of VTS operator positions within a VTS, determine what functions will be carried out from which positions and be prepared to ensure that there will be personnel occupying those positions who have been given responsibility for carrying out the identified functions.

VTS Operational Staff

The role of VTS in the provision of safety and efficiency services to shipping and in the protection of the environment, the need to avoid confusion on the part of users travelling from one VTS to another and the importance of professionalism on the part of operators in determining the extent of trust placed in the functioning and effectiveness of a VTS.  It is essential that VTS operational staff be appropriately qualified and trained to carry out their functions, and that the standards for such qualification and training be according to the internationally agreed IALA V-103 standard.

A major factor in the efficient operation of a VTS Centre is the standard of competence of its personnel.  Recognising that VTS personnel are members of a profession whose principal interaction is with mariners and maritime pilots in the safe management of maritime traffic, their competence needs to reflect that professional responsibility.

VTS personnel should be capable of providing VTS information, navigational assistance when required and establishing a traffic organisation service in a VTS area as specified by the relevant VTS Authority.  Depending on the size, activities and location of a VTS, the Centre may comprise VTS Operators, VTS Supervisors and a VTS Manager.  It is for the VTS Authority to determine the levels of manning sufficient to meet its obligations and to ensure that trained personnel are available to undertake these commitments.

The purpose of standards for training VTS personnel has two main aspects.  First, to ensure the competence of personnel that occupy key positions in VTS in which critical situations can occur and, secondly, to provide uniform consistency of procedural communications with ships throughout the world.

# VTS Personnel Positions

Experience has shown that effective VTS Centres have invariably devoted considerable time and effort in developing comprehensive 'Job Descriptions' for the personnel they intend to train and employ.  The key person in any VTS operation is the VTS Operator (VTSO).  Regardless of the size or complexity of VTS area, all VTS Centres are likely to experience situations that will involve many of the functions for which the VTSO has been trained and, following endorsement, considered competent.

In Centres where more than one VTSO is on watch simultaneously, the VTS Authority may determine a requirement to establish the position of VTS Supervisor to assist, oversee and/or co-ordinate the activities of the watchkeeping VTSO's.

Where VTS Centres cover a large geographical area, a VTS or Port Authority may elect, as part of its management infrastructure, to establish the position of VTS Manager.

# Staffing Levels

The availability of trained VTS staff is an essential resource without which VTS

operations cannot safely be managed.  Determining the adequacy of staff levels is often difficult to quantify with any degree of accuracy.  Invariably this will be a balance between a number of factors which a VTS authority will need to keep under periodic review, not least of which will be their acquired experience both collectively and at individual level. These Guidelines aim to analyse some of the criteria that will need to be taken into account.

There are at least six criteria that should be considered, namely:

- The comparison of outcomes against the VTS service level agreement

- The optimum time that a VTS Operator should spend at a work station

- The extent of machine assistance available to the VTS Operator

- Communication requirements

- The ability of the duty watch to deal with unplanned events

- The risks of poor performance associated with workload stress

# Comparison of Outcomes with respect to the Service Level Agreement

A VTS should operate within agreed terms of reference, which should be incorporated into the mission statement or service level agreement between the VTS organisation and either a port or governmental administration.  In every case VTS Managers, Supervisors and Operators should be aware of the aims and objectives of the VTS.

It follows that once the aims and objectives are defined, measurement of performance should be carried out to determine whether these are being met. Measurement might be determined by the evaluation of internal management information or by the polling of end-users or by a combination of both.

The outcomes measured will be driven by the terms of reference and may include:

- response times to calls and enquiries to the VTS

- number of targets that each VTS Operator has to plot at any one time

- traffic patterns, density, geography of the area and potential risk of incident

- risk assessments in terms of probability and impact

- actual numbers of incidents historically, their distribution in time, and their frequency and impact indicators

Factors for consideration when determining periods of duty for VTS Watchkeepers

There are several considerations to determine a period of duty, these include:

- forecast or actual ship movements

- the number of interventions expected of the VTSO for every ship movement

- Health and Safety requirements, particularly when working with visual display units

- appropriate working environment

- the ability to maintain situational awareness

# The extent of technological automation available to VTS Personnel

Aboard ships, technology has been used to reduce the workload of staff on vessels in order to make marine transportation economically feasible for a wide range of industry practices.  Electronic Navigational Chart (ENC) data is the core element of a modern integrated navigation information system.  ENC based systems can improve safety in waterways because they present mariners with real-time information quickly and with minimal effort.  An accurate position based on the Differential Global Positioning System (DGPS), when presented on an electronic chart, provides mariners with the positioning accuracy they need to support navigation decisions.

Likewise, there are elements of equipment or system design in VTS Centres that can significantly ease the workload on the VTSO.  Examples of this include:

- automatic tracking

- deviation alarms

- predetermined area warnings and;

- information presented by Automatic Identification System (AIS) transponders.

Systems and equipment that relieve the VTS personnel of repetitive or mundane tasks, and effective man-machine interfaces allow the VTS personnel to take on more work or modify an existing workload to something much more achievable.

When drawing up the specification for VTS equipment, careful consideration should be given to ensure that the equipment flexible and capable of maintaining a high level of reliability in order to keep up with technological advances.  If this is not allowed for, the system will become less effective and thereby increases the workload of the user.  The experience of the VTSO in operating the equipment may well be drawn on for this purpose.

When new or updated equipment is introduced, consideration should be given to the training of the VTSO in order to maintain the expertise commensurate with the level of service being provided.

# Communication Requirements

# The level of radio traffic to and from the VTS Centre

Communications between ship and shore are an essential component.  Each item of radio traffic handled by the VTS Operator demands interpretation and action – it generates a workload.  To minimise the effect of this, radio traffic should be conducted in a manner that is clear, concise and procedurally correct.  Equipment considerations that can facilitate efficient radio traffic include good quality transceivers, adequate radio coverage (i.e. with a minimum of 'blind spots'), use of good quality loudspeakers and headsets in the VTS Centre and acceptable background noise levels in the VTS Centre.

Radio workload can also be eased by the use of alternative means of exchanging

information between the ship and shore, such as VHF/DSC and AIS.  Text based systems often require less operator attention and can ease communications difficulties caused by differing standards in spoken English.

A reduction in radio workload can also be achieved by the introduction of written procedures as prescribed by IMO and the International Telecommunication Union (ITU).

As navigational and safety communications between ship and shore must be precise, simple and unambiguous, so as to avoid confusion and error, there is a need to standardize the language used.  To meet this requirement IMO has introduced 'Standard Marine Communication Phrases' (SMCP) - IMO Resolution A.918(22).  The habitual use of these Phrases will eventually ensure an acceptable safety language, using English for the verbal interchange of intelligence among individuals of all maritime nations on the many and varied occasions when precise meanings and translations are in doubt.  This habitual use should also ensure an eventual reduction in radio workload.

# The volume of landline telephone traffic to and from the VTS Centre

Each telephone call made or received generates workload for the VTS Operator. The VTS Centre is at the hub of a complex web of information, which can result in the reception of a large number of enquiries by telephone not directly connected with the primary mission.

As VTS Centres are usually in continuous operation, using them as an out-of-hours point of contact for other tasks and purposes should be avoided.  The contact may either be official or unofficial.  Very often the main harbour administrative office, redirects all the out of hours telephone lines through to the VTS Centre.  This practice may well increase the VTSO workload to an unacceptable level and should not be allowed to happen.

VTS Authorities should develop clear policies on the nature of telephone traffic to and from the VTS Centre, and to whom key numbers are promulgated externally.

One way of reducing telephone traffic is to ensure that appropriate means are available to facilitate the access of relevant shipping data to all parties concerned, for example, though a computer network.

The ability of the staff on duty to deal with Emergencies and Unplanned Events

It should be relatively simple for a VTS Authority to determine the routine level of

activity at the respective VTS Centre and any foreseeable variations driven by factors such as tide, severe weather, normal office hours and day/night operations.  However, the potential for an emergency to occur is always present.   Any emergency within the VTS service area will create additional workload for the staff on duty, and the more serious the emergency, the greater the rise in workload.

The VTS Centre should have a contingency plan to deal with major internal and external incidents.  The activation and operation of these plans takes time to implement. There should be sufficient personnel to deal with the rise in workload until the contingency plan is fully effective, accepting that during this period all tasks must be strictly prioritised.  A manning level that is only able to deal with the normal forecast workload is potentially overloaded, and will certainly be overloaded should an emergency or other unplanned event occur.  Debriefing of these events contributes to the ability of the staff on duty to deal with them.

# The risks of poor performance associated with workload stress

Workloads either in excess or below normal levels is potentially damaging and may have health implications for the individual, colleagues and managers if this damage is allowed to occur repeatedly.

Short term or acute overload (hyperstress) will impair or prevent personnel from

interpreting information, making effective decisions and taking the proper action. Usually, augmenting the number of people available to do the work and the clear prioritisation of tasks will rapidly ease the overload.  Prolonged overload will produce a more permanent degradation on the performance of an individual and may result in damage to that person’s health.

However, prolonged inactivity or under employment (hypostress) is considered more destructive than hyperstress. Over-staffing can create boredom and monotony in the workplace, and reduce the exposure of VTS personnel to work, that would normally maintain and develop their skills.  An underemployed individual or team will soon develop attitudes that may well be unacceptable within the organisation.

# Navigational Assistance Service (NAS)

NAS is one example of a service that requires considerable skill and experience to execute properly.  It also demands considerable workload in so much that the NAS is usually provided on a one-to-one basis.

The achievable benefits depend not only on the skill and knowledge of the VTSO but also on the resolution and accuracy of the VTS equipment and the reliability of the VTS information being processed.  If information on the fairway related position, the adjacent traffic and on demand course advice is provided on a continuous basis, this service has proven to make a positive contribution to the safety of vessel traffic.  It also reduces the 'down time' and delays for both vessel and other activities in the area concerned.

# Factors Affecting VTSO Workload Staffing Levels

Technical / Equipment

Procedures

Environment

Human Resources

Interaction with Vessel Traffic

Interaction with Allied Services