Recommendation on XXX [ Title Here]

(Recommendation [leave number blank, to be filled by IALA Secretariat])

THE COUNCIL:

RECALLING the function of IALA with respect to Safety of Navigation, the efficiency of maritime transport and the protection of the environment;

RECOGNISING that [to be completed by Secretariat - for example - IMO has concluded that AIS will improve the safety of navigation and the protection of the environment];

RECOGNISING ALSO that [to be completed by Secretariat - for example - there is a mandatory carriage requirement for AIS equipment on SOLAS Convention vessels that entered into force on 1 July 2002 and will be complete by [2008]];

RECOGNISING FURTHER that [to be completed by Secretariat - for example - documentation of IMO, ITU and IEC refer to the provision of an AIS shore infrastructure as part of the overall operational system];

NOTING the [as above];

NOTING ALSO that;

NOTING FURTHER that;

CONSIDERING that to be completed by Secretariat - for example.

ADOPTS the [name of document] in the annex of this recommendation; and,

RECOMMENDS that National Members and other appropriate Authorities providing marine aids to navigation services [action to be taken].

*    *    *

# DRAFT GUIDELINES ON A COMMON STRUCTURE FOR PERFORMANCE STANDARDS FOR NAVIGATIONAL EQUIPMENT

### INTRODUCTION

Anchor the document in regulations/documentation

### BACKGROUND

Provide any background information required to ensure complete understanding of the document

### OBJECTIVES OF...

In the headline following the word "OF" the name of the individual navigational equipment should be inserted.

In this section the purpose of the navigational equipment for navigation and/or the ship's safety should be described.

### BENEFITS

Include as appropriate the benefit of adhering to the information contained in the document

### APPLICATION [OF THESE PERFORMANCE STANDARDS]

In this section the application and the limits of application of the individual performance standards for the navigational equipment should be determined.

### RELATED REQUIREMENTS INVOLVED

In this section all additional applicable IMO requirements for the respective navigational equipment should be listed. This may be:

Relevant SOLAS requirements;

General resolutions, such as resolution A.694(17) - General Requirements for Shipborne Radio Equipment forming Part of the Global Maritime Distress and Safety System (GMDSS) and for Electronic Navigational Aids;

Resolutions with more specific requirements, such as:

Resolution A.529(13) - Accuracy Standards for Navigation;

Resolution A.688(17) - Code on Alarms and Indicators;

Resolution MSC.39(63) - Adoption of amendments to the Code on Alarms and Indicators.

### DEFINITIONS

If necessary in this section terms used in the individual performance standards should be defined.

### FUNCTIONAL REQUIREMENTS

8.1	Required functions and their availability

In this section functional requirements, such as modes of operation, functional properties and capabilities and output of information should be determined.

8.2	Reliability, accuracy and discrimination

In this section the reliability, minimum requirements for accuracy and discrimination, and other relevant information which are necessary for the safe use of the navigational equipment should be clearly determined.

8.3	Malfunctions, warnings, alarms and indications

In this section reference should be made to relevant requirements of resolution A.686(17) and resolution MSC.39(63). Depending on the individual type of the navigational equipment requirements in addition to resolutions A.686(17) and MSC.39(63) should be determined if necessary.

8.4	Software requirements

Until the Organization has adopted software requirements for navigational equipment, if necessary in this section software requirements concerning the following items should be determined for the individual navigational equipment:

demarcation of closed systems from open systems; correctness, stability and consistency of software; stability, security of certain system data; ability for a new startup of the system after a breakdown of power supply over a short period;

data and software update management; test procedures;

failure handling and release of warnings and alarms.

### OPERATIONAL REQUIREMENTS

9.1 Ergonomy

In this section general ergonomic requirements for the operation of the navigational equipment and the presentation of the information should be determined until the Organization has adopted applicable ergonomic requirements.

9.2	Operational controls

Depending on the individual type of the navigational equipment, requirements in addition to resolution A.694(17), such as direct operations and automatic controls should be determined if necessary.

9.3	Presentation of information

Depending on the individual type of the navigational equipment, requirements in addition to resolution A.694(17) should be determined, such as settings (e.g. continuous, variable and default settings), maximum permissible time for operation, information to be displayed at all times and dedicated display area.

9.4	Software requirements

Until the Organization has adopted software requirements for the navigational equipment, if necessary in this section software requirements concerning the following items should be determined for the individual navigational equipment:

design of user interfaces according common ergonomic principles; support of virtual operations;

self-descriptiveness-,

status information of the user interface; protection against unauthorized use.

### DESIGN AND INSTALLATION

10.1	Durability and resistance to environmental conditions

As regards environmental conditions the IEC requirements' should be applied as far as relevant. Depending on the individual type of the navigational equipment requirements concerning the durability and resistance to environmental conditions in addition to resolution A.b94(17) and the relevant IEC requirements' should be determined if necessary.

10.2 Interference

In this section reference should be made to [NAV 40125, annex 17 - General Requirements for Electromagnetic Compatibility (EMC) for Electrical and Electronic Ship's Equipment -] and as regards interference the IEC requirements' should be applied as far as relevant. Depending on the individual type of the navigational equipment, requirements in addition to resolution A.fi94(17), [NAV 44125, annex 17] and relevant IEC requirements' should be determined if necessary.

10.3	Power supply

As regards power supply, the IEC requirements' should be applied as far as relevant. Depending on the individual type of the navigational equipment, requirements in addition to resolution A.b94(17) and relevant IEC requirements' should be determined if necessary.

10.4 Installation

Depending on the individual type of the navigational equipment, requirements concerning the installation of sensors, wiring and the arrangement of the navigational equipment in the wheelhouse should be determined.

'IEC 529 "Degrees of protection provided by enclosures (IP Code)"

IEC 721-3-6 "Classification of environmental conditions, Part 3: Classification of groups of environmental parameters and their severities; Ship environment"

IEC 945 "Maritime navigation and radiocommunication equipment and systems - General requirements, methods of testing and required test results"

10.5 Maintenance

Depending on the individual type of the navigational equipment, requirements in addition to resolution A.694(17) concerning maintenance should be determined if necessary.

### INTERFACING

Requirements concerning the correctness and unambiguity of input and output data of the individual components should be determined. Relevant existing standards' should be taken into account until the Organization has adopted relevant requirements.

### BACK-UP AND FALL-BACK ARRANGEMENTS

Depending on the individual type of the navigational equipment, requirements concerning back-up and fall-back arrangements should be determined if necessary.

### SAFETY PRECAUTIONS

Depending on the individual type of the navigational equipment, requirements in addition to resolution A.694(17) concerning safety precautions should be determined if necessary.

### MARKING AND IDENTIFICATION

Depending on the individual type of the navigational equipment, requirements in addition to resolution A.694(17) should be determined if necessary, e.g., for the identification of software.

### DOCUMENTATION

In this section the contents of the documentation (e.g. Operator's Manual, Installation Manual, Service Manual) necessary for the operation of the navigational equipment should be determined. Depending on the individual type of the navigational equipment, the documentation should contain the following items:

General:

manufacturer, type of equipment and the serial number; system description;

specification of the navigational equipment; supplementary equipment;

'For example:

National Marine Electronics Association (NMEA)

Radio Technical Commission for Maritime Services (RTCM) Radio Technical Commission for Aeronautics (RTCA) American National Standard Institute (ANSI)

Electronic Industries Association (EIA)

Installation:

physical characteristics of the navigational equipment;

requirements concerning installation (e.g. supply voltage and frequency, earthing, power consumption, heat dissipation);

safety information;

Operation:

operational controls, functions and presentation of information; startup and shutdown procedures;

tests, failures, calibration procedures and alarms; interfaces and software description;

of updating procedures; safety information;

Failure Elimination, Maintenance and Service:

required tools and materials for failure elimination and maintenance on board; access for maintenance;

preventive maintenance;

replacement, repair, failure elimination, adjustment; information concerning external service.

### MISCELLANEOUS

In this section additional requirements, which are necessary for the individual navigational equipment but could not be subsumed under another headline should be determined here.

### APPENDIX

### REFERENCES

In this appendix all references used in each individual performance standards should be listed.