VTS18 / output / 03

Vessel Traffic Services Risk Mitigation Levels

A risk-based assessment of the waterway should indicate the traffic, navigation and environmental conditions that exert the greatest influence on the probability and the consequences of a marine accident.  Taking what is learned in such an assessment, the competent authority can then determine what measures should be applied to mitigate the impact those risk inducing factors have on overall risk.  (IALA Guidelines on Risk Management and IALA VTS Manual Chapter 2 )

In many waterways vessels can operate independently under all conditions of traffic density, traffic mix and weather likely to be encountered.  However, there are many waterways in which vessels must rely on some level of interaction with persons ashore to safely conduct their movements due to traffic congestion, physical limitations, adverse weather or vulnerable environments.  In the extreme case, there are areas in which vessels must rely entirely on information, advice or directions from ashore in order to complete a safe transit. It is therefore necessary to take all these factors into account, in order to properly assess how they impact with each other in contributing to the level of risk. For instance, in some circumstances the need for a VTS may be deemed inappropriate or excessive, based on the Competent Authority’s risk assessment.

Waterways were therefore initially broken down into non- VTS areas and VTS areas. The distinction between the two being based on the following key attributes of a VTS:

  - Ability to interact

  - Maintain a traffic image

  - Respond to developing situations

  - Forward Movement planning

  - Authority to direct

This is illustrated in the figure below

The level and type of services a non- Vessel Traffic Service and a Vessel Traffic Service should deliver is determined by the waterway, traffic and navigation conditions in a particular area.  Because these conditions can vary considerably, there are also varying levels and types of services among non- VTSs and VTSs, as is illustrated in the figure above.  A systematic means is needed to classify these according to the services they provide and consequently the waterway conditions they address.

For instance, a Vessel Traffic Service, if properly equipped, staffed and operated, can mitigate certain risks inherent in navigation.  However, a VTS can be established and operated with varying levels of capability and authority.  Determining the appropriate level of service and capability to apply to certain conditions is important from a cost benefit standpoint.

Taking the two extremes of vessels operating completely independent from shore-side influences and the situation in which navigation cannot proceed without intervention from ashore, gives two anchor points from which a continuum of VTS capability levels can be established.  Within this continuum are defining intervals that describe various levels of VTS according to their respective capability to mitigate risk.  These are the key attributes described earlier.

These levels of non-VTS and VTS can be used in the course of a risk assessment/mitigation exercise to determine in conjunction with other risk mitigation mechanisms, whether a VTS is necessary and at what level of capability and competence.  The levels are defined as:

I	Unaided (Non VTS)  Vessels navigating in these areas can safely proceed in all likely conditions of traffic and weather without any assistance from ashore.  In this scenario, the vessel can obtain all the information it needs to complete a passage by using its own on board resources, including pilots.

II	Port/Local Information (non VTS)  This case introduces a resource ashore that provides information to the bridge team.  However, services from such a resource are limited to information only. This organization does not maintain a traffic image, does not interact with traffic or respond to developing traffic situations.  The organization does not participate in on board decision-making and lacks the authority to direct the movement of a vessel.

III.  Information Service (VTS)  This level introduces a competent authority with resources to maintain a traffic image, interact with traffic and respond to developing traffic situations.  However, this organization does not participate in on board decision-making and lacks the authority to direct the movement of a vessel.  All or certain classes of vessels may be required to provide information to this organization and to establish and maintain communication during their transit.

IV.  Traffic Organization Service (VTS)  This level has all the capability of an Information Service (level III) and authority to direct the movement of a vessel.  All or certain classes of vessels will be required to participate in this service and shall not proceed without clearance.

Information and Traffic Organization Services may have the additional capability to provide a Navigation Assistance Service (NAS) to individual vessels.  A contributory navigational assistance service is solely the provision of factual navigational information to assist the onboard decision making process. In a participatory navigational assistance service, the VTS becomes involved in the onboard decision making process by providing navigational advice.

Annex 1 depicts an example of the Risk Mitigation Levels that can be attributed to both non-VTSs and VTSs.

# Proposal

- The VTS Committee give further consideration to this paper at their next session VTS19.

- The IALA Secretariat bring this interim paper to the attention of the next session of the Port and Waterways Risk Management Working Group.

Annex 1

VTS Risk Mitigation Levels

VTS as a tool to assist in the Reduction of Risk

|  | ## Class | Name |
| --- | --- | --- |
| Non-VTS | I | Unaided |
|  | II | Port / Local Information |
| VTS | III | Information Service (IS) |
|  | IV | Traffic Organisation Service3 (TOS) |

Navigational Assistance Service (NAS)

|  | ## Class | Name | TYPE OF SUPPORT |  |  | FUNCTION |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ### Information | ### Advice | Instruction1 2 | ### Interaction | Maintain a Traffic Image | Respond to developing situation | ### Shore Participation in On-Board Decision Making1 | ### Movement Planning |
| Non-VTS | ### I | Unaided | N | N | N | N | N | N | N | N |
|  | II | Port Information | ###### Y | N | N | ###### N | N | N | ###### N | ###### N |
| VTS | III | IS | Y | N | N | Y | Y | Y | N | N |
|  | III (a) | IS and NAS (Contributory) | Y | Y | N | Y | Y | Y | N | N |
|  | III(b) | IS and NAS (Participatory) | ###### Y | Y | N | Y | Y | Y | Y | N |
|  | IV | TOS | ###### Y | N | Y | Y | Y | Y | N | Y |
|  | IV(a) | TOS and NAS (Contributory) | Y | Y | Y | Y | Y | Y | N | Y |
|  | IV(b) | TOS and NAS (Participatory) | Y | Y | Y | Y | Y | Y | Y | Y |

1 Instructions should be result orientated

2 Where the VTS has the regulatory power to do so

3 A TOS includes an IS