# Radar Service / Other Sensor Services

Summary of discussion during the breaks of VTS Committee /Technical Group

The discussion in the Technical Group got the result that it is useful to describe the requirement to a VTS system the functional way rather technical way.

Functional means to answer the question “What must the system be able to do”  while the technical question is “ how does the system work to meet the requirements”.

Following this it make sense to talk about  the technical service which meets the user´s application instead of defining the parameters of technical system .

During the committee meeting  we assumed we to know the use cases of the VTS personal and started with our knowledge  of technical systems the design of services.

Association Diagram for Position Sensor Service and Processing the Sensor Data:

Multisensor Fusion

Optical Sensors

div. DSC

RDF-Service?

AIS Sensors

Or

AIS Service?

Radar service with single tracker

Tracker

Sensor

Tracker

Sensor

Tracker

Sensor

DF-Tracker

DF

Design of Radar service structure with UML:

Multisensorfusion

1

1..n

Tracker

1..n

1..n				Radarservice

Sensor

Read: One Multisensorfusion is associated with min 1 thru max n Trackers

Each  Tracker is associated with minimum one  sensor

Radar service interface towards multisensor fusion:

- Integrity data

- Track quality

- Position of target

- Quality and Status of each target

- Track (course and speed)

- Radar video

Multisensor fusion interface toward other non sensor services:

- One track per target/ multisensor fusion result

- Total quality of fusioned target and quality of each sensor

- Deviation of each sensor from the fusion position

- Heading

- Each sensor target velocity and position

- Radar video ?

I have added some explanations and some question marks. We should discuss that during the next committee meeting.

Dirk Eckhoff, Kiel, Feb. 11   2003