## GUIDELINES ON STAFFING LEVELS AT VTS CENTRES

This Working Paper has been produced by WG3 as version 3.  There is still considerable work to be done and it has been agreed that members will be carrying out their own research inter-sessionally, for contribution at VTS20 (February 2004).

----------------------------------

Table of Contents

## Introduction

    - Scope

    - Aims and objectives

    - International Regulations

    - Definitions and glossary

  - Background

    - Implementation of VTS

    - VTS personnel

    - VTS personnel positions

3.1	Factors affecting workload

    - Technical and equipment

    - Procedures

    - Environment

    - Human resources

    - Interaction with traffic

    - Interaction with allied services

    - Comparison of outcomes

    - Extent of technological automation

    - Type and levels of service provided

  - Staffing Levels

    - General formula for the number of operators per VTS operating position

    - ………………..

    - ………………..

  - Annexes

    - Examples

    - References

# 1.1	Introduction

IMO Resolution A.857(20) 'Guidelines for Vessel Traffic Services' (VTS) describes the skill and knowledge qualifications required by VTS operators to provide the required services to ensure the safety of navigation and protection of the environment.

The IMO Guidelines are intended for application in both planned and existing VTS.  They provide guidance in determining how VTS Authorities can recruit, select and train personnel in order to carry out their tasks thereby providing the required VTS standards.

In planning and establishing a VTS, the Contracting Government(s) or the Competent Authority should:

- ensure that the VTS Authority is provided with the equipment and facilities necessary to effectively accomplish the objectives of the VTS and;

- ensure that the VTS Authority is provided with sufficient staff, appropriately qualified, suitably trained and capable of performing the tasks required, taking into consideration, the type and level of services to be provided, as per the current IMO Resolution A.857 (20) Guidelines on the Recruitment, Qualifications and Training of VTS Operators (Annex 2).

1.1.1	Scope

These Guidelines are only intended to provide guidance to Authorities intending to establish a VTS Centre(s) and to those Authorities with existing Centres who need to realise the ever-increasing changes in technology and their effect on workload.  They are by no means prescriptive.  When using these Guidelines, readers are strongly recommended to make themselves familiar with the various works of Reference as shown in Annex xxx.

2.1.1	Implementation of a VTS

The following steps need to be considered:

- Implementing a VTS - make a decision, or have made a decision to implement a VTS.

- Identification of VTS external and internal functions - identify and describe the detailed functions relevant to the proposed VTS.

- Assignation of VTS Centre functions according to how they are to be carried out in accordance with the VTS operation.

- Establishment of VTS Operator posts within a VTS, determine what functions will be carried out from which positions and the VTS Authority must be prepared to ensure that there will be qualified and certified personnel occupying those positions who have been given responsibility for carrying out the identified functions.

2.1.2	VTS Personnel

The role of VTS is the provision of safety and efficiency services to shipping and the protection of the environment.  The need to avoid confusion on the part of users and the importance of professionalism on the part of VTS Operators, makes it essential that the VTS operational staff be appropriately qualified and trained according to the internationally agreed IALA V-103 standard.

A major factor in the efficient operation of a VTS Centre is the level of competence of its personnel.  Recognising that the main task of VTS personnel is to interact with mariners and maritime pilots for the safe management of maritime traffic, their competence must reflect that professional responsibility.

VTS personnel should be capable of providing VTS information, navigational assistance when required and establishing a traffic organisation service in a VTS area as specified by the relevant VTS Authority.  Depending on the VTS area characteristics, such as traffic patterns, size and location, the VTS Centre may comprise VTS Operators, VTS Supervisors and a VTS Manager.  It is for the VTS Authority to determine the levels of manning sufficient to meet its obligations and to ensure that trained personnel are available to undertake these commitments.

The purpose of standards for training VTS personnel has two main aspects.  First, to ensure the competence of personnel that occupy key positions in VTS in which critical situations can occur and, secondly, to provide uniform consistency of procedural communications with ships throughout the world.

# 2.1.3	VTS Personnel Positions

Experience has shown that effective VTS Centres have invariably devoted considerable time and effort in developing comprehensive 'Job Descriptions' for the personnel they intend to train and employ.  The key person in any VTS operation is the VTS Operator (VTSO).  Regardless of the size or complexity of the respective VTS area, all VTS Centres are likely to experience situations that will involve many of the functions for which the VTSO has been trained and, following endorsement, considered competent.

In VTS Centres where more than one VTSO is on watch simultaneously, the VTS Authority may determine a requirement to establish the position of VTS Supervisor to assist, oversee and/or co-ordinate the activities of the watchkeeping VTSO's.

Where VTS Centres cover a large geographical area, a VTS Authority may elect, as part of its management infrastructure, to establish the post of VTS Manager.

3.1	Factors for consideration when determining periods of duty for VTS Operators and Supervisors

There are several considerations to determine a period of duty, these include:

- forecast or actual ship movements;

- the number of interventions expected of the VTSO for every ship movement;

- Health and Safety requirements, particularly when working with visual display units;

- appropriate working environment;

- the ability to maintain situational awareness.

3.1.1	Technical Equipment

An assessment should be made to assess the need and degree of equipment redundancy necessary for specific purposes offered.  For example, appropriate redundant sensing, tracking and communications should support the provision of a Navigational Assistance Service.

An analysis should be included of the optimum ergonomic design of workstations and placement of equipment within the VTSO work area.

3.1.2	Procedures

These should eliminate needless redundancy and should support decision-making and action at the VTS position.

They should be reviewed and updated at regular intervals in accordance with VTS quality management system standards to ensure their relevance to VTS aims and objectives.

They should emphasize the relationship between standard communications practices and reduced workload at the VTS Centre and on participating vessels.

3.1.3	Working Environment

An analysis should be of the immediate environment around the display screen equipment including the room (artificial and/or natural) and workstation lighting together with air quality and ambient temperature.  Consideration should also be given to possible internal and external noise and other distractions which may affect concentration in monitoring the traffic image and radio communications.  With respect to lighting, consideration should also be given to possible glare and reflection

3.1.4	Human Resources

Allowance should be made for additional staffing during critical phases of training and assessment.  The possibility of the VTSO performing the role of OJT Instructor should also be considered.

Examples of factors for consideration, that could influence workload in the VTS Centre, are predictable peak and lull periods in vessel traffic, which may be daily or seasonal, darkness restrictions on certain classes of vessel and cargo, seasonal restrictions due to ice and tidal range variations.

Another factor for consideration is the availability of additional or ‘call-in’ personnel in case of emergencies.

An analysis should be made of the VTSO’s job description, in particular, responsibilities that may pose conflicts with the VTSO’s attention to the monitoring of traffic and radio communications.

It should be noted that the performance requirements of the VTS system often demands high levels of attention and concentration by the user, for example, where the consequences of error may be critical.

Considerations should also include collective agreements, contracts and terms of employment for VTS personnel.

Although consideration needs to be given to the period that sustained attention to monitoring displays and other instruments is necessary, it is not appropriate to lay down requirements for breaks, which apply to all types of work.  It is the nature and mix of demands made by the job, which determine the length of break necessary to prevent fatigue.

There may also be an additional workload with respect to dealing with Allied Services.  Interaction with traffic should include an analysis of the number of reporting Points together with the type and amount of information necessary, with a view to possibly reducing them.

# 3.1.7	Comparison of Outcomes with respect to the Service Level Agreement

A VTS should operate within agreed terms of reference, which should be incorporated into the mission statement or service level agreement between the VTS authority and either a port or governmental administration.  In every case VTS Managers, Supervisors and Operators should be aware of the aims and objectives of the VTS.

It follows that once the aims and objectives are defined, measurement of performance should be carried out to determine whether these are being met. Measurement might be determined by the evaluation of internal management information or by the polling of end-users or by a combination of both.

The outcomes measured will be driven by the terms of reference and may include:

- response times to calls and enquiries to the VTS;

- number of targets that each VTS Operator has to plot at any one time;

- traffic patterns, density, geography of the area and potential risk of incident;

- risk assessments in terms of probability and impact;

- actual numbers of incidents historically, their distribution in time, and their frequency and impact indicators.

# 3.1.8	The extent of technological automation available to VTS Operators and Supervisors

Aboard ships, technology has been used to reduce the workload of staff on vessels in order to make marine transportation economically feasible for a wide range of industry practices.  Electronic Navigational Chart (ENC) data is the core element of a modern integrated navigation information system.  ENC based systems can improve safety in waterways because they present mariners with real-time information quickly and with minimal effort.  An accurate position based on the Differential Global Positioning System (DGPS), when presented on an electronic chart, provides mariners with the positioning accuracy they need to support navigation decisions.

Likewise, there are elements of equipment or system design in VTS Centres that can significantly ease the workload on the VTSO.  Examples of this include:

- automatic tracking;

- deviation alarms;

- predetermined area warnings and;

- information presented by Automatic Identification System (AIS) transponders.

Systems and equipment that relieve the VTS Operators of repetitive and routine tasks, and effective man-machine interfaces, allow the VTS personnel to take on more work or modify an existing workload to something much more achievable.

When drawing up the specification for VTS equipment, careful consideration should be given to ensure that the equipment is flexible and capable of maintaining a high level of reliability in order to keep up with technological advances.  If this is not allowed for, the system will become less effective and thereby increases the workload of the user.  The experience of the VTSO in operating the equipment may well be drawn on for this purpose.

When new or updated equipment is introduced, consideration should be given to the training of the VTSO in order to maintain an adequate expertise to the level of service being provided.

# 4.1	Staffing Levels

The availability of qualified VTS staff is an essential resource without which VTS

operations cannot safely be managed.  Determining the adequacy of staff levels is often difficult to quantify with any degree of accuracy.  Invariably this will be a balance between a number of factors which a VTS authority will need to keep under periodic review, not least of which will be their acquired experience both collectively and at individual level.  These Guidelines aim to analyse some of the criteria that will need to be taken into account.

There are a number of criteria that should be considered, for example:

- The comparison of outcomes against the VTS service level agreement;

- The optimum time that a VTS Operator should spend at a work station;

- The extent of machine assistance available to the VTS Operator;

- Communication requirements;

- The ability of the staff on duty to deal with emergencies and unplanned events;

- The risks of poor performance associated with workload.

# 4.1.1	Communication Requirements

# The level of radio traffic to and from the VTS Centre

Communications between ship and shore are an essential component.  Each item of radio traffic handled by the VTS Operator demands interpretation and action – it generates a workload.  To minimise the effect of this, radio traffic should be conducted in a manner that is clear, concise and procedurally correct.  Equipment considerations that can facilitate efficient radio traffic include good quality transceivers, adequate radio coverage (i.e. with a minimum of 'blind spots'), use of good quality loudspeakers and headsets in the VTS Centre and acceptable background noise levels in the VTS Centre.

Radio workload can also be eased by the use of alternative means of exchanging

information between the ship and shore, such as VHF/DSC and AIS.  Text based systems are often easier to be understood and can ease communications difficulties caused by differing standards in spoken English.

A reduction in radio workload can also be achieved by the introduction of written procedures as prescribed by IMO and the International Telecommunication Union (ITU).

As navigational and safety communications between ship and shore must be precise, simple and unambiguous, so as to avoid confusion and error, there is a need to standardize the language used.  To meet this requirement IMO has introduced 'Standard Marine Communication Phrases' (SMCP) - IMO Resolution A.918(22).  The habitual use of these Phrases will eventually ensure an acceptable safety language, using English for the verbal interchange of intelligence among individuals of all maritime nations on the many and varied occasions when precise meanings and translations are in doubt.  This habitual use should also ensure an eventual reduction in radio workload.

# The volume of landline telephone traffic to and from the VTS Centre

Each telephone call made or received generates workload for the VTS Operator. The VTS Centre is the heart of a complex web of information, which can result in the reception of a large number of enquiries by telephone not directly connected with the primary mission.

As VTS Centres are usually in continuous operation, using them as an out-of-hours point of contact for other tasks and purposes should be avoided.  The contact may either be official or unofficial.  Very often the main harbour administrative office, redirects all the out of hours telephone lines through to the VTS Centre.  This practice may well increase the VTSO workload to an unacceptable level and should not be allowed to happen.

VTS Authorities should develop clear policies on the nature of telephone traffic to and from the VTS Centre, and to whom key numbers are promulgated externally.

One way of reducing telephone traffic is to ensure that appropriate means are available to facilitate the access of relevant shipping data to all parties concerned, for example, though a computer network.

The ability of the staff on duty to deal with emergencies and unplanned events

It should be relatively simple for a VTS Authority to determine the routine level of

activity at the respective VTS Centre and any foreseeable variations driven by factors such as tide, severe weather, normal office hours and day/night operations.  However, the potential for an emergency to occur is always present.   Any emergency within the VTS service area will create additional workload for the staff on duty, and the more serious the emergency, the greater the rise in workload.

The VTS Centre should have a contingency plan to deal with major internal and external incidents.  The activation and operation of these plans takes time to implement. There should be sufficient personnel to deal with the rise in workload until the contingency plan is fully effective, accepting that during this period all tasks must be strictly prioritised.  A manning level that is only able to deal with the normal forecast workload is potentially overloaded, and will certainly be overloaded should an emergency or other unplanned event occur.  Debriefing of these events contributes to the ability of the staff on duty to deal with them.

# The risks of poor performance associated with workload stress

Workloads either in excess or below normal levels is potentially damaging and may have health implications for the individual, colleagues and managers if this damage is allowed to occur repeatedly.

Short term or acute overload (hyperstress) will impair or prevent personnel from

interpreting information, making effective decisions and taking the proper action. Usually, augmenting the number of people available to do the work and the clear prioritisation of tasks will rapidly ease the overload.  Prolonged overload will produce a more permanent degradation on the performance of an individual and may result in damage to that person’s health.

However, prolonged inactivity or underemployment (hypostress) is considered more destructive than hyperstress. Over-staffing can create boredom and monotony in the workplace, and reduce the exposure of VTS personnel to work, that would normally maintain and develop their skills.  An underemployed individual or team will soon develop attitudes that may well be unacceptable within the organisation.

# 3.1.9	Navigational Assistance Service (NAS)

NAS is one example of a service that requires considerable skill and experience to execute properly.  It also demands considerable workload in so much that the NAS is usually provided on a one-to-one basis.

The achievable benefits depend not only on the skill and knowledge of the VTSO but also on the resolution and accuracy of the VTS equipment and the reliability of the VTS information being processed.  If information on the fairway related position, the adjacent traffic and on demand course advice is provided on a continuous basis, this service has proven to make a positive contribution to the safety of vessel traffic.  It also reduces the 'down time' and delays for both vessel and other activities in the area concerned.

# EXAMPLES OF STAFFING LEVELS ON A VTS CENTRE (24 hours service)

| Type of VTS | Sectors | VTS area (size) | Radar coverage | Participating vessel movements a day | Working hours |  | VTS Operators |  | Supervisors |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Per week | Per shift | Positions | Total | Positions | Total |
| Port & Coastal | 2 | 55 NM | 100 % | 200-250 | 38 | 8/12 | 1 | 6 | 1 | 5 |
| Port & Coastal | 4 | 74 NM | 100 % | 150 | 42 | 12 | 4 | 32 | 1 | 5 |
| Estuarial & Port | 1 | 300 NM | 7 % | 70 | 34,25 | 8 | 2 at daytime 1 at night | 7 | 0 | 0 |
| Bridge protection | 1 | 35 NM | 100 % | 65 | 37 | 12 | 2 | 12 | 1 | 6 |
| Coastal | 3 | 90 NM | 100 % | 200 | 38 | 12 | 3 | 7 | 1 | 8 |
| Inland | 3 | 450 km | 10 % | 600-700 | 36 | 8 | 4 | 25 | 1 at daytime | 1 |
| Coastal | 1 | 35 NM | 100 % | 500-600 | 35 | 8 | 3 | 21 | 1 | 7 |
| Port | 2 | 35 NM | 100 % | 100 | 35 | 8 | 4 | 28 | 2 | 14 |