Note from the editor

Dear members of the IALA VTS committee

Please find an updated version of the draft performance requirements for VTS radar services included.

All the comments from VTS 19 have been included in the document or highlighted in yellow if they are to be readdressed.

Information highlighted in green shall be moved to performance requirements for other services as they become available.

Outstanding work is that sections and tables with TBD are to be filled in and chapters 5+3 are to be updated for consistency with chapter 4.

The aim is to finalise the document during VTS 20 and I will be happy receive any comments by e-mail jcp@terma.com BEFORE December 1 2003 and incorporate such comments in a new input paper for VTS 20.

Best regards

Jens C. Pedersen

Draft recommendation on performance requirements for VTS radar services

(Recommendation [leave number blank, to be filled by IALA Secretariat])

THE COUNCIL:

RECALLING the function of IALA with respect to Safety of Navigation, the efficiency of maritime transport and the protection of the environment;

RECOGNISING that [to be completed by Secretariat - for example - IMO has concluded that AIS will improve the safety of navigation and the protection of the environment];

RECOGNISING ALSO that [to be completed by Secretariat - for example - there is a mandatory carriage requirement for AIS equipment on SOLAS Convention vessels that entered into force on 1 July 2002 and will be complete by [2008]];

RECOGNISING FURTHER that [to be completed by Secretariat - for example - documentation of IMO, ITU and IEC refer to the provision of an AIS shore infrastructure as part of the overall operational system];

NOTING the [as above];

NOTING ALSO that;

NOTING FURTHER that;

CONSIDERING that to be completed by Secretariat - for example.

ADOPTS the [name of document] in the annex of this recommendation; and,

RECOMMENDS that National Members and other appropriate Authorities providing marine aids to navigation services [action to be taken].

*    *    *

Annex

Performance requirements for VTS radar services

List of contents

1.	Introduction	6

1.1	Referred documents	6

1.2	SW tools	6

1.3	Abbreviations	6

2.	Background	7

2.1	Implementation of radar services	7

2.2	Radar Target Characteristics	9

2.2.1	Radar cross section	9

2.3	Radar Range performance	11

2.3.1	Antenna height	11

2.3.2	Propagation conditions	11

2.3.3	Lobing	13

2.3.4	Shadows	14

2.3.5	Tide and its influence on river banks etc	14

2.4	Noise and clutter affecting radar performance	15

2.4.1	Thermal noise	15

2.4.2	Surface and volume clutter	15

2.4.3	Ice	16

2.4.4	Precipitation	17

2.4.5	Man made noise	17

2.4.6	Multiple reflections (ghost images)	17

2.5	Target discrimination and position Accuracy	18

2.5.1	Range resolution	18

2.5.2	Azimuth resolution	18

2.5.3	Range Accuracy	19

2.5.4	Azimuth accuracy	19

2.6	Radar Parameters	19

2.6.1	Antenna parameters	19

2.6.2	Transmitter characteristics	20

2.6.3	Receiver characteristics	20

2.6.4	System Losses	21

2.6.5	Signal processing, extraction and tracking	22

3.	Definitions	24

4.	Recommendation on performance standards for VTS radar services	26

4.1	General requirements	26

4.2	Objects to be detected	26

4.3	Reduction of operator workload	27

4.4	Radar coverage and detection performance	27

4.4.1	Detection range	27

4.4.2	Dynamic range	27

4.4.3	Clutter and noise reduction facilities	28

4.4.4	Antenna Scan	29

4.4.5	Target Discrimination	29

4.4.6	Range performance	31

4.5	Plot Extraction and Tracking	32

4.5.1	Plot Extraction	33

4.5.2	Track Initiation	33

4.5.3	Maintaining Track	33

4.5.4	Track Termination	33

4.5.5	Track Data Output	33

4.6	Reliability and availability	35

4.6.1	Redundancy	35

4.6.2	Security functions (optional)	35

4.7	Environmental Specifications and Safety	35

4.8	Sections to be moved to performance standards for other VTS services	37

4.8.1	Fusion of radar data with other sensors	37

4.8.2	Display	37

4.8.3	Measurements	38

5.	Performance verification methods	39

5.1	Methods recommended for system design	40

5.1.1	Type tests	40

5.1.2	Determination of radar coverage and detection performance	40

5.2	Factory Acceptance Test	40

5.2.1	System functional tests and measurements	40

5.2.2	Antenna measurements	40

5.2.3	Transceiver measurements	41

5.3	Site Acceptance Test	41

5.3.1	Functional Tests and Checks	41

5.4	In operation tests	41

5.4.1	Acquisition and analysis of statistical data from the system	41

5.4.2	Record and analysis of events	41

# Introduction

[anchor the document in regulations / documentation]

## Referred documents

IEEE Std 686-1997	IEEE Standard Radar Definitions

## SW tools

CARPET

AREPS

## Abbreviations

AIS		Automatic Identification System

ASL		Above Sea Level

CARPET	Computer Aided Radar Performance Evaluation Tool

DF		Directional Finder

GIT		Georgia Institute of Technology

ICAO		International Civil Aviation Organization

PD		Probability of Detection

PFA		Probability of False Alarm

RCS 		Radar Cross Section

TBD		To be defined

VTS		Vessel Traffic Services

# Background

Performance requirements to radars for VTS are often very different to the requirements for marine navigational radars. The aim of this chapter is to provide comprehensive guidance on the subjects relevant for the VTS application.

VTS radars normally need to operate simultaneously on short and long range leading to dynamic requirements far beyond that required onboard a ship.  The geometry, elevated antennas with high gain looking into the sea, surroundings with large structures may further increase dynamic and resolution requirements.

Furthermore, VTS radars will often be more influenced by weather related phenomena's such as ducting, than a ships radar and this can have a serious positive or negative influence on the performance.

Good clutter suppression is needed for sea clutter and, for most parts of the world, for rain clutter too.  In addition, the need to see small targets in rough weather conditions becomes essential if objectives include detection of targets for security purposes.

The introduction of AIS further develops the VTS into a modern information system, and the presentation of radar information needs to follow the trend, setting new demands for the radar performance. Antenna side lobes and ghost targets (multiple reflections) may lead to false and dangerous results when Radar returns and AIS plots are associated.  High precision is therefore a must to allow for unambiguous correlation of position obtained from the two information sources.

## Implementation of radar services

Furthermore, the implementation of radar services for VTS requires a broad spectrum of knowledge about:

- VTS operation

- General physical capabilities and constraints

- Target characteristics and coverage

- Accuracy and resolution

- Assessment of interference risks

- Weather influence including propagation

- External interface design

- System design and program planning

The aim of this document is to provide background information, set generic performance standards and to provide guidelines for implementation of Radar Services in VTS systems based on the VTS functional model illustrated in  Figure 1.1

Requirements of radar services may have a high impact on acquisition and life-cycle costs of a VTS, therefore the performance recommendations are divided into 3 levels, where:

- Standard recommendations are applicable to all 3 levels of VTS, Information Service, Navigational Assistance Service and Traffic Organisational Service for areas with medium to high traffic density and/or without specific navigational hazards.

- Relaxed recommendations are applicable to VTS, Information Service and/or Navigational Assistance Service for areas with low to medium traffic density.and without specific navigational hazards.

- Extended recommendations are applicable to VTS areas with high traffic density and/or specific navigational hazards

The 3 levels of recommendations may be used as applicable within a VTS, e.g. part of a VTS area may call for Standard recommendations and another part may call for relaxed recommendations

The individual VTS authority should decide on the level of specification and define requirements related to climate and topography as further advised in this document.   Consideration of other local conditions may dictate individual performance requirements that are independent of the level of recommendation.

Requirements to detect very small vessels for safety or security reasons may further extend requirements, beyond those for extended recommendations.

Figure 1.1 VTS functional model

## Radar Target Characteristics

Understanding the behaviour of radar targets is essential to determine expected performance from radar services.

The strength of the echo returned from a target is largely governed by the width of the target and by its height above the radar horizon. However, this is not the only factor and a small target of good reflecting properties may well give a stronger echo than a much larger target of poor reflecting properties.

Aspect angle (i.e. the angle at which the radar pulse “hits” the target) is an important factor affecting echo strength. Certain types of objects which might be expected to give a good echo prove to be very poor radar targets because they reflect most of the energy that strikes them in a direction other than straight back to the radar. The worst of these are gently curved smooth surfaces, such as sandy beaches, sandbanks, mud banks, and gently sloping hills which are regular in shape and not covered with trees or thick undergrowth. For the same reason lighthouses that are conical in shape do not give as good a response as might be expected.

Vertical, comparatively smooth surfaces, such as cliffs and isolated buildings, give strong reflection provided they face the radar.

Complex structures such as metal ships with several corners do also give strong reflections, whereas ships made of other material typically give weaker reflections.

### Radar cross section

The characteristics of a target are defined by its radar cross section (RCS) and the fluctuations in RCS.

Formally, the radar cross section is defined as the ratio between the power [in W] scattered by the target back towards the radar receiver and the power density [in W/m2] hitting the target. Thus RCS is measured in m2 and has the dimension of an area. There is, however, no simple relation between the physical area of the target and the RCS, as the reflected power depends on the angle of incidence as well as target properties such as material (conducting/non-conducting), coating, physical shape (smooth/sharp corners) etc.

The total reflection from a target is made up from the “elementary” reflections from different parts of the targets. These elementary reflections will interfere constructively or destructively to form the total reflection. Therefore, small changes in attitude of the target may change the amount of reflected power dramatically. The consequence of this is that in radar performance evaluations the RCS of a target is best described by a statistical distribution function. Different kinds of wanted targets (ships, buoys, etc.) and unwanted targets (sea clutter) have different distribution functions. These differences can be exploited in the radar signal processing to enhance target visibility.

A good scientific reference giving information about the RCS of objects relevant for VTS is the experimental study of the radar cross section of maritime targets by P.D.L. Williams, H.D, Cramp and Kay Curtis, in ELECTRONIC CIRCUITS AND SYSTEMS, July 1978. Vol 2. No 4.

In combination with measurements from trials made by an IALA industrial member this gives rules of thumb for determination of average RCS as outlined in Table 2.1.

Guidance on which statistical distribution function to for different kinds of targets and clutter can be found in e.g. Merrill I. Skolnik: “Introduction to Radar Systems” 3rd Ed. McGraw-Hill (2001).

Table 2.1 , Typical target characteristics

| Target types |  | Typical characteristics at I (X) band |  |  |
| --- | --- | --- | --- | --- |
|  |  | RCS | Height | Fluctuations etc. |
| 1 | Navigational aids without radar reflector | Up to 1 m2 | 1 - 4 m ASL | Rapidly fluctuating, highly dependent on sea characteristics. |
| 2 | Navigational aids with radar reflector | 10 – 100 m2 |  | Rapidly fluctuating, wind and currents may tilt to blind angles and lobing may cause reflectors to be in blind spots |
| 3 | Small open boat, fiberglass, wood or rubber with outboard motor and at least 2 persons onboard, small speedboat, small fishing vessels or small sailing boats. | 0.5 – 5 m2 | 0.5 to 1 m ASL | Rapidly fluctuating may be hidden behind waves up to 50 % of the time. Slow moving targets tend to lie lower in the water than fast moving ones and therefore RCS visible to the radar tends to increase with speed. |
| 4 | Inshore fishing vessels, sailing boats and speedboats, equipped with radar reflector of good quality. | 3 – 10 m2 | 1 – 2 m ASL | Rapidly fluctuating |
| 5 | Small metal ships, fishing vessels, patrol vessels and the like | 10 – 100 m2, | 2 - 4 m ASL |  |
| 6 | Coasters and the like | 100 – 1000 m2 |  | RCS is highly dependent on aspect angle of the individual vessel. Rate of fluctuations is typically moderate |
| 7 | Large coasters, Bulk carriers, cargo ships and the like | 1000 – 10000 m2 |  |  |
| 8 | Container carriers, tankers etc. | 10000 – 1000000 m2 |  |  |
| 9 | Buildings, cranes. Stacks of containers and other large structures | Up to 1000000 m2 | Depend on site | Insignificant |
| 10 | Floating items, oil drums and the like. Birds, floating or flying. | Up to 1 m2 | 0 to 0.5 m ASL | Rapidly fluctuating, highly dependent on sea characteristics. |
| 11 | Flocks of birds. | Up to 3 m2 | Sea level and up | Rapidly fluctuating, flight paths tend to be characteristic of given species in given areas of interest |

Comprehensive RCS data for other frequency bands was not available to be included in this document; however, there is no indication of large variations in RCS with frequency for the listed types of targets for the frequencies traditionally used by Radars for VTS.  Excluded from this is small objects such as  birds and flocks of birds where the RCS of the individual species is significantly dependent on frequency.

## Radar Range performance

The range performance of a radar system is the result of many effects. In calm weather and under normal atmospheric conditions it depends mainly on the radar specifications and the antenna height, but sea and weather conditions affect the radar range as described in the following.

### Antenna height

The figures below illustrate how the height of the antenna above the water line affects the maximum and minimum detection range performance.

Figure 2.2 Target range and visibility

### Propagation conditions

In some areas of the world, especially those with hot, dry climate, propagation effects cause radar performance to vary drastically over time – sometimes from hour to hour.

The density of the atmosphere is not uniform with altitude. The gradient of density results in a bending of the radar waves in a manner similar to the bending of light waves by an optical prism. Water vapour is the atmospheric component chiefly responsible for the bending of radar waves in the lower atmosphere as illustrated in Figure 2.3. Temperature also has an influence.

The temperature and humidity gradients in the air above the sea surface cause the radar beam to be bent up or down - in most cases down and to follow the earth’s curvature to some extent, thereby elongating the range for potential target detection. This condition is called super-refraction, or ducting

The air in contact with the ocean’s surface is saturated with water vapour. A few meters above the surface the air is not usually saturated, so there is a decrease of water vapour pressure from the surface to some value well above the surface. Such a change in the moisture distribution without an accompanying temperature change forms a so-called evaporation duct, always leading to extended radar range if present.

The atmosphere may also form a trapping layer and generate a wave guide like duct – a surface based duct or an elevated duct.

Under trapping layer conditions a radar antenna situated within the duct may receive extremely long-range returns. The result is also considerably increased amounts of clutter and false echoes generated by objects beyond the radar ambiguity range (2nd and multiple time around echoes).

Above a strong duct (a trapping layer) one often finds a layer of low refraction (sub refraction). An antenna situated above a trapping layer, or even worse above more than one trapping layer, will radiate most of its energy into the sub-refractive layer, which will not direct the energy down towards targets on the sea or on the ground. Therefore, the radar will detect only very short range targets. This may limit usable antenna height in dry hot regions of the world, especially in Middle East countries.

Severe atmospheric conditions, such as thunderstorms or acute build-up of static electricity during pre-storm situations also have the affect of distorting the radar presentation owing to the contorting effect of the electrical disturbances in the atmosphere.

Figure 2.3 Effects of propagation conditions

Figure 2.4 shows the radar coverage in standard atmospheric conditions (defined by ICAO) as a function of antenna elevation and target height, based on the equation:

R (NM) = 2.2 (hA + hT),

1

where hA and hT are measured in metres.

However, note that only large targets can be expected to be detected to the theoretical radar horizon. Complex calculations such as that illustrated in Figure 2.2 gives a much better impression of what to expect. Software tool such as CARPET can be used for complex calculations.

Figure 2.4. Radar horizon in standard atmospheric conditions

### Lobing

The presence of the surface of the earth or the sea not only restricts the line of sight, but it can also have an effect on the radar coverage of targets within the line of sight. Two waves arrive at the target via two separate paths: One travels along the direct path from radar to target; the other path includes a reflection from the water surface. The two waves can interfere destructively or constructively according to their difference in phase.

This effect is known as lobing and one of the effects is that point targets, such as buoys may become invisible to a radar at certain ranges and in certain conditions as illustrated by Figure 2.5 .

Figure 2.5 Coverage diagram for a typical VTS radar. Detection probability PD is given in percentage as shown in the lower bar in the figure.

### Shadows

TBD

### Tide and its influence on river banks etc

TBD

## Noise and clutter affecting radar performance

Noise is the part of the radar signal which is uncorrelated to the transmitter pulse, whereas clutter is unwanted radar return signals from e.g. sea waves or land areas.

Targets as well as various sources of noise or clutter behave statistically in different ways. Knowledge of this is essential to obtain adequate performance of the radar system. The ability to detect target echoes depends not only on the signal power, but also on the competing noise and clutter power present in the receiver. The sources of this noise/clutter lie both within the radar and in the outside environment as illustrated in Figure 2.6.

Figure 2.6 Noise sources.

### Thermal noise

Historically, the sensitivity (or Minimum Detectable Signal, MDS) of radar systems has been limited by the thermal noise of the receiver, expressed by the noise figure. However, processing techniques becoming common during the 1990’s can remove the white noise and reduce the radar system MDS below the level determined by the noise figure.

### Surface and volume clutter

A radar set which is sensitive enough to detect small targets like buoys and small boats will also be sensitive enough to detect signals from land, sea waves, raindrops, snowflakes, and hail.

Such signals are known as clutter and represent a serious limitation detection by radar.

#### Sea Clutter

Sea return is caused by the reflection of the radar signal from water surface irregularities and sea return is therefore composed of a very large number of echoes, originated by the random wave reflection from scan to scan.

Sea and wind relations vary dependent on the geographical location. The Ocean differs from other waters and the same seems to be the case for the definition of sea states, which have been seen to be defined by the Beaufort scale (wind), the GIT model (used by the CARPET radar performance evaluation program) or the Douglas scale. The approximate relations between the scales, based on various publications are illustrated in Table 2.2.

Table 2.2 : Sea state – approximate relations between Beaufort scale, the Douglas scale and CARPET Definitions

#### Sea breezes

The large temperature variations between day and night in desert areas and associated pressure differences between land and sea tend to cause strong sea breezes during summertime. This generates, in combination with layered distribution of air, eddies over the sea, often forming distinctive sea clutter patterns.

### Ice

The echoes obtained from ice depend largely on the form and shape the ice encountered.

The typical characteristics of the various types of ice floes are as follows:

- Smooth flat ice: Most of the radar energy is reflected at its angle of incidence, providing little or no return signal.

It is sometimes an advantage to set up the radar controls to obtain sea clutter right up to the edge of the ice. If there is sufficient wind to disturb the surface of the water, patches of water in a smooth ice field are often revealed by sea clutter return.

- Pack ice: Strong multiple echoes are usually obtained from this condition producing a pattern on the display not unlike excessive sea clutter. The ice left in the wake of a vessel passing through an ice field can be distinguished clearly.

- Ice walls: These objects will give strong echoes depending on the angle that they are to the sea surface to scatter the reflected energy.

- Icebergs: The sharp angles of the faces of icebergs usually give a poor return signal. Their smooth surface will often scatter the reflected signal at acute angles.

- Growlers: The detection of growlers by radar is uncertain due to their small surface area above water.

Detection of icebergs and growlers may therefore require sophisticated techniques similar to those for detection of small targets performing illegal activities.

#### Land clutter

Clutter caused by land will normally not affect the VTS radar service.

### Precipitation

#### Rain clutter and penetration

The amounts of rain clutter return are dependent on:

- The resolution cell size, meaning that influence is reduced with decreasing antenna beam width and decreased with decreased transmitter pulse width.

- The antenna polarization, where circular polarization is preferred over vertical or horizontal polarization.

- Frequency, where lower frequencies penetrate precipitation better than higher frequencies and give less clutter return: However, lower frequencies are in most cases associated with severe drawbacks in respect to resolution and cost.  Therefore, the use of higher frequencies and circular polarized antennas in combination with processing techniques tend to be preferred for VTS except for long range installations in areas with extreme tropical rainfall.

#### Hail and snow

Hail and snow produce effects similar to that of rain clutter. Dense snow has a greater effect than that of light flurries which, owing to its small reflecting surface, has minimal effect.

### Man made noise

Man made noise consist primary of signals from other nearby radars. However, applying noise reductions techniques allow for several radars near each other to operate on the same frequency without interference.

### Multiple reflections (ghost images)

Multiple reflections can result in several types of ghost images. Sometimes these have the appearance of normal targets, but in most cases they are intermittent and poorly defined. Ghost images maintain a fixed relationship with respect to the true image and have more of an arc-like appearance than the true image, plus a tendency to smear. This is especially the case when the radar is located beside large structures.

One type of ghost is caused by multiple reflections from objects which are close together. For example, two vessels which are passing within a few hundred metres of each other may, because of reflections of the electrical impulse between the vessels, produce a smaller second, third, and fourth image at a longer range. These images will usually appear only momentarily beyond the vessel.

A second type of ghost image is caused by secondary reflection from other buildings and structures. Such reflections are especially common when the antenna is mounted close to other big buildings and iron structures.

Careful notice should be taken of any false image encountered, in order to determine its probable cause and to assist in future recognition. For safety's sake, if there is ever any question as to whether a particular image is true or ghost, always assume it to be a true image corresponding to an actual object.

## Target discrimination and position Accuracy

The ability to discriminate targets depends on the radar range and azimuth resolution as shown in Figure 2.7.

Figure 2.7 Range and Azimuth resolution

### Range resolution

For traditional pulse radars the range resolution depends on Transmitter Pulse Width (PW), rise time, fall time, allowance for jitter and stretch in the receiver.

In a pulse compression or continuous wave radar the effective pulse width can be made very short giving fine resolution and the equivalent peak power may be very big. The drawback is added complexity and unwanted effects such as time side lobes.

### Azimuth resolution

The azimuth characteristics of a radar return from a point target is almost entirely determined by the characteristics of the antenna. The azimuth beam width is inversely proportional to the antenna length which in turn directly affects the radar angular resolution (and range performance as antenna gain increases with length).

The angular width of small target echoes (at the - 3 dB) is equal to the horizontal beam width of the antenna (defined at the - 3 dB points). However this is a highly theoretical definition of azimuth resolution which mainly can be used for comparison of equipments.

Most targets, especially at short or medium ranges, are detected when they are also below the - 3 dB points and, as a result, most echoes appears with an angular width in excess of the antenna beam width.

### Range Accuracy

Determination of the range from an antenna to a target (the slant range) is normally dependant on precise, reliable clocks and the range accuracy is accordingly precise in most modern radar systems.

### Azimuth accuracy

The azimuth accuracy depends mainly on the turning gear and on the resolution and accuracy of its associated azimuth bearing transmitter.

## Radar Parameters

The limiting factors with respect to radar coverage is a combination of radar location, antenna height,  local weather, transmitted power, antenna characteristics, receiver sensitivity, receiver dynamic characteristics, processing capabilities and system losses, all determining the minimum magnitude of radar returns which can be detected.

The parameters are often dependent on each other and different vendors may choose different methods to solve the same issue.  It is therefore recommended to specify overall radar performance required, rather than radar parameters.

However, in order to understand how the parameters affect radar range each of the parameters is described here on the basis of Figure 2.8.

Figure 2.8 Radar functions TBD

### Antenna parameters

The antenna gain is one of the most important factors determining the radar range. The gain is mainly determined by the operating frequency and the horizontal and vertical beam widths of the antenna array.

The horizontal beam with is furthermore the governing parameter in respect to azimuth resolution.

The vertical beam characteristics has, furthermore, influence on coverage and performance in precipitation.

Use of circular polarization will reduce unwanted returns from precipitation while maintaining good returns from targets of interest to the VTS operator. If targets of interest to security, such as small rubber boats, are an issue the selection of antenna polarization is more complex and should be studied carefully.

Use of high antenna speed increases the scan data rate, but the hits per scan (echo returns per scan from a particular target) is decreased correspondingly. In some cases this may mask a weak target which would otherwise be detected by using a lower antenna speed.

#### Side lobes

A small part of the RF energy from each transmitted pulse is radiated outside the main antenna beam, producing side lobe patterns whose axis in the horizontal plane have various angles with respect to the main beam.

Side lobes have no effect in case of distant or small surface targets, but the echo from a large target at short range may produce an arc similar to a part of a range ring or they may appear as a series of echoes forming a dotted arc.

Side lobes may also occur far from the main beam producing targets which are very difficult to discriminate from real targets.

The side lobe levels are entirely dependent on the antenna used and far side lobe characteristics are of very high importance as side lobes far from the main beam may result in false targets, appearing to the operator with characteristics similar to the characteristics of valid targets.

### Transmitter characteristics

For traditional pulsed radars the average power and peak pulse power has an influence on the detection range, however, its importance shall not be overestimated. The reason is that high peak power often results in higher losses in the transmission lines of the radar and it will always result in larger amount of unwanted information as e.g. sea clutter to be returned to the radar.

The average RF power depends upon peak power, pulse width, and pulse repetition frequency. Average power on long pulses is higher than the average power on short pulses. For this reason there is a possibility of using the long pulse to pick up signals from small targets such as buoys and wooden boats, especially in clear weather.

Shorter pulse lengths will, however, result in better range resolution and result in reduced returns from rain and sea clutter. The result is better overall performance and use of relatively short pulses (50 to 300 nanoseconds) is often beneficial for VTS applications.

### Receiver characteristics

The receiver sensitivity (often expressed by the receiver noise factor) is one of the main factors with respect to detection of small distant targets in clear weather. However, notice that the noise figure may be expressed in different terms such as LNFE noise figure and overall noise figure, making it difficult to compare radars from different vendors.

A better expression for receiver sensitivity is the Minimum Detectable Signal or MDS; however, this is not ideal either as it may be difficult to measure this figure in a reliable manner.

The dynamic characteristics are of high importance, especially if operational conditions require:

- Simultaneous detection of targets on short and long range.

- Supply of signals for more than one display with different range settings.

- Simultaneous supply of signals for different operational roles.

- Supply of signals for automatic video processing and automatic tracking.

The RCS of targets of interest are often seen to range from below 1 m2 to 1.000.000 m2. Ranges vary from typically 50 meters to 15 – 20 NM

Minimum desirable signals to be seen at the receiver input and processed onwards lie typically at -90 to - 105 dBm in VTS applications or maybe as low as –115 dBm for security applications.

Very large targets at close range are, however, not fully within the beam at short range, limiting the input at the receiver to about + 10 dBm for the typical case.

Consequently, the overall dynamic range needed for the individual radar for a VTS application is typically between 100 and 125 dB.

#### Time side lobes

For pulse compression radars, special attention should be made to time side lobes which may cause severe false echoes in front of and behind large ships and other structures.

### System Losses

System losses reduce performance, and especially the ability to detect small distant targets. Common sources are:

- Long waveguide runs

- Moisture in the waveguide runs

- Flexible and especially flexible/twistable waveguide parts. (Not to be confused with elliptical waveguides which in general are good).

- Teflon or plastic gaskets used instead of proper pressure windows, especially if mounted between plain and choke flanges

- Excessive number of joints in the waveguide system

- Poor rotary joints

- Poor workmanship in the manufacture of waveguide parts - it is a skilled persons job

- Matching losses in receivers and distribution of signals with inadequate bandwidth / speed.

- Digitalization of signals using too few bits.

- Losses in detection and tracking

Furthermore, discontinuities in wave guide installations can result in reflections causing the centre of the radar picture to be totally saturated. Bad cases can even disturb the transmitted pulse, giving reduced angular resolution as well as reduced performance with respect to discrimination of targets from clutter.

### Signal processing, extraction and tracking

As mentioned above the radar return signal down-converted to video level contains wanted signals from objects of interest, unwanted signals (clutter) from e.g. the sea and some basic system noise as e.g. thermal noise – all mixed together. The purpose of the signal processing is to enhance the signals from the objects of interest relative to the clutter and noise. Different techniques are used. Some of the more important ones are outlined below:

- Sweep-to-sweep correlation: Normally, the pulse repetition rate of a radar is so high, that a target is hit several times while the antenna beam turns past the target. In contrast, noise will pop up randomly. Therefore, by correlation of radar return signals from a number consecutive transmissions noise can be suppressed efficiently relative to targets (and clutter).

- Signal differentiation: Extended areas of clutter from e.g. rain showers can be reduced by signal differentiation (in radar this is called FTC).

- Signal integration: The multiple hits on a target described above can be added together. This will enhance the returns from the objects of interest relative to noise and, to a lesser extent, relative to clutter. Further, it will reduce the fluctuations of target echoes.

- Frequency Diversity: Frequency diversity radars employ two (nearby) transmission frequencies. The radar returns on the two frequencies from a target will normally be similar, while radar returns from clutter will be different. If an antenna with frequency dependent squint such as a slotted wave guide antenna is used, there will also be a temporal difference between the returns on the two frequencies. This means that the target will be present in both radar returns, while it is very likely that the clutter will be present in only one of the returns. The total effect of using frequency diversity will be an enhancement of target radar returns relative to clutter radar returns.

- Scan-to-scan correlation: By comparing radar returns from consecutive rotations of the antenna fast disappearing echoes from e.g. sea waves can be filtered away while persisting echoes from e.g. ships can be retained for further processing. Again, the overall effect is to enhance target radar returns relative to clutter radar returns.

The output of the signal processing is the processed radar data in a polar grid i.e. a circular area divided into radar cells, each cell being characterized by a range value, a range depth, an azimuth value, an azimuth width and a radar return intensity value. Plot extraction is the process of identifying groups of neighboring radar cells containing radar return intensity values above some threshold value. Such a group of radar cells is called a plot and is supposed to correspond to a physical target.

Figure, cells and targets TBD

Plots are updated on each rotation of the radar. Plots which persist from rotation to rotation and which move within reasonable physical limits of speed and acceleration are supposed to correspond to physical targets. A series of plots associated in this way is called a track.  Erratically appearing plots are rejected as representatives for physical objects and therefore as candidates for tracks.

Tracking is the process of associating plots from consecutive rotations of the radar to existing tracks and to initiate new, tentative tracks on plots not associated to existing tracks. Once tracks are formed prediction of future behavior can be predicted as an aid for further plot association and as information for navigation (Closest Point of Approach (CPA), Time to CPA etc.) .

# Definitions

Reference to IEEE Standard Radar Definitions

- Normal weather and propagation conditions are the conditions persisting 99% - 99.9 % of the time as defined by the individual VTS authority. The rest of the time is considered having adverse weather and propagation conditions

- Target means any object fixed or moving whose position and motion is determined by measurements of range and bearing on radar.

- True course means the true direction of motion of a target expressed as an angular displacement from north.

- True bearing means the direction of a target from own ship or from another target expressed as an angular displacement from north.

- North-up display means a display in which a line connecting the centre with the top of the display is north true bearing.

- Target's predicted motion means a prediction of future target motion based on linear extrapolation from its present motion as determined by past measurements of its range and bearing on the radar.

- True vector means the predicted true motion of a target.

- Acquisition means the process of selecting a target or targets and initiating their tracking.

- Tracking means the computer process of observing the sequential changes in the position of a target in order to establish its motion.

- Target swap means a situation in which the incoming radar data for a tracked target becomes incorrectly associated with another tracked target or a non-tracked radar echo.

- Acquisition area means an area set up by the observer which should automatically acquire a target when it enters such an area.

- History means equally time spaced past position of a target which is being tracked.

- Trails means tracks displayed by the radar echoes of targets in the form of a synthetic afterglow.

- Echo reference means a facility for indicating that a particular fixed navigational mark which is being tracked is to be used as a ground stabilized reference.

- Trial manoeuvre means a facility to assist the observer in making the correct manoeuvre for navigation and collision avoidance purposes.

- Suppressed area means an area set up by the observer within which targets are not acquired.

- ERBL means the electronic range and bearing line used to measure bearings and/or ranges.

- CPA/TCPA stands for closest point of approach (CPA) and time to closest point of approach (TCPA) limits from an object as defined by the observer, to give warning of when a tracked target or targets will close to within these limits.

- Bow passing prediction is the situation associated with a target which is crossing or predicted to cross ahead of own ship.

- Bad echo is the name associated with a tracked target which appears to have been temporarily lost or which has a poorly defined radar aspect, so that it does not have tracking ability.

- Lost target is the name associated with a target that is no longer being tracked due to having been lost or obscured.

- Predicted points of collision is a graphical representation of where predicted collision intercept points lie with respect to own ship and other targets.

- PAD means the predicted area of danger defined around a predicted close quarter situation area. The size is determined by speed ratios between a ship and another target in question and CPA distance limits as defined by the observer.

- Map lines means the navigational facility whereby the observer can define lines to indicate channels or traffic separation schemes. Sometimes called Nav lines.

Additional definitions TBD

# Recommendation on performance standards for VTS radar services

## General requirements

The radar facilities provided for VTS should comply with the minimum performance standards defined by this document

The individual VTS authority shall ensure that the selected system architecture, the numbers of equipments, the network capacity etc. is capable of meting the performance standards.

## Objects to be detected

The radar service in a VTS shall be capable to detect and to track the following type of surface objects in weather conditions normal for the individual site:

Table 3.3 Types of targets (surface objects)

|  | Recommendation level |  |  |
| --- | --- | --- | --- |
|  | Relaxed | Standard | Extended |
| Navigational aids without radar reflector |  |  | X |
| Navigational aids with radar reflector | X | X | X |
| Small open boat, fiberglass, wood or rubber with outboard motor and at least 2 persons onboard, small speedboat, small fishing vessels or small sailing boats. |  |  | X |
| Inshore fishing vessels, sailing boats and speedboats, equipped with radar reflector of good quality. |  | X | X |
| Small metal ships, fishing vessels, patrol vessels and the like | X | X | X |
| Coasters and the like | X | X | X |
| Large coasters, Bulk carriers, cargo ships and the like. | X | X | X |
| Container carriers, tankers etc. | X | X | X |

In addition the radar service should be available to the extent possible in adverse weather conditions. However, the individual VTS authority may find it desirable to discontinue radar services temporary in the event of adverse conditions, such as extreme wind or icing conditions.

The output from a radar service should be radar image and track data.

## Reduction of operator workload

Radar services functions shall be designed and implemented to optimize performance and minimize operator workload to the level practical.

Ideally, only basic functions such as start and stop should be controllable by operators and it is recommended to make automatic adaptation to changing weather conditions etc.

Built in test features should include monitoring of functions and performance and it is recommended to make the results accessible for remote monitoring, especially in the case of radars being installed at locations difficult to access.

## Radar coverage and detection performance

The requirements to radar coverage (instrumented area) and range performance are by nature individual from site to site and the requirements shall be determined by the individual VTS authority.

The suggested method for determination of radar coverage and range performance is a combination of site inspections and radar system performance calculations, made by experts with a broad operational and technical knowledge about the subject.

### Detection range

In normal weather and propagation conditions the surface objects specified in Table 3.3 and within the VTS area should be:

- Clearly displayed from a minimum horizontal range (e.g., 50 meters) from the antenna position to the maximum detection range determined in accordance with section 4.4.6

- Stable tracked from a minimum horizontal range (e.g.,100 meters) from the antenna position to the maximum detection range determined in accordance with section 4.4.6

Excessive antenna heights may increase the above minimum values or require dedicated vertical radiation patterns (inverse cosecant square) to be used for the antenna.

Locating the radar outside the VTS coverage area could avoid loss of coverage due to minimum horizontal range limitations.

### Dynamic range

The dynamic range of the radar service should, in normal weather and propagation conditions, detect and process the surface objects specified in Table 3.3 and within the VTS area  without distortion such as:

- Pulse stretch making any significant disturbance of the radar image or the tracking performance

- Masking  of small targets by larger targets (except if shaded)

- Masking of small targets by the effects of time side lobes (in the case of pulse compression or CW radars)

However, there is no requirement to display the full dynamic characteristics of the information received.

### Clutter and noise reduction facilities

Appropriate, clutter reduction facilities should be available to meet the performance criteria as defined per section 4.4.6.

This shall typically include:

- White noise suppression

- Sea clutter suppression

- Rain penetration and volume clutter suppression; and in some cases

- Adaptation to varying propagation conditions

The features should preferably be automatic for systems requiring standard or extended specifications.

#### Performance in sea clutter

Depending on site conditions the ability to detect targets in higher sea states should be defined by the VTS authority

Table 3.4 give examples of characteristics suitable for the 3 recommendation levels.

Table 3.4  Examples of performance in high sea states

| Antenna elevation | Target |  | Maximum sea states (GIT model) for detection and tracking of fluctuating point targets in clear weather, and standard atmosphere |  |  |
| --- | --- | --- | --- | --- | --- |
|  | RCS | Height | Relaxed specification | Standard specification | Extended specification |
| 50 m ASL | 1 m2 | 1 m ASL | N/A | N/A | 3-4 |
|  | 3 m2 | 2 m ASL | N/A | 3 | 4-5 |
|  | 10 m2 | 3 m ASL | 3 | 4 | 6 |
|  | 100 m2 | 5 m ASL | 4 | 5 | 7 |
|  | 1000 m2 | 8 m ASL | 5 | 6 | 8 |

#### Rain penetration and volume clutter

The ability to detect targets in precipitation should be defined by the VTS authority on the basis of statistical information about local weather conditions including:

- the frequency of precipitation

- the density of precipitation, and

- the size of rain cells etc.

Such data will normally be available from local meteorological services

For precipitation rates normal to the individual site it may i.e. be acceptable to reduce detection range by 25 % relative to those defined per Table 3.7. This assumes that volume clutter (rain clutter) has been suppressed to obtain acceptable false alarm rates.

#### Propagation conditions

For areas of the world having:

- Tropical climate

- Dry, hot climate

The influence from adverse propagation effects should be considered.

### Antenna Scan

The scan shall be clockwise, continuous and automatic throughout the coverage defined for the individual radar

The antenna rotation rate may be optimized for the individual site. Slow rotation rates (6 - 20 RPM) will in general provide the best performance in long range applications. Faster rotation rates (20-45 RPM) may be preferred for short range applications such as harbour and inland VTS.

Alternative methods of scanning and higher rotation rates may be used providing that the expected performance is met.

#### Side lobe suppression

Antenna side lobes shall be sufficiently low to avoid false targets, especially those far from the antenna main lobe.

Table 3.5 give examples of characteristics suitable for the 3 recommendation levels.

Table 3.5  Side lobe suppression

| Antenna side lobe suppression | Relaxed specification | Standard specification | Extended specification |
| --- | --- | --- | --- |
| Minimum first side lobe level suppression | 26 dB | 27 dB | 28 dB |
| Increasing to, at angles +/- 10 or more outside the main lobe. | 33 dB | 34 dB | 35 dB |

    - Target Discrimination

In normal weather and propagation conditions surface objects within the VTS area should be

- separated in presentation

- and individually tracked without track swap at any applicable target speed

when they are positioned apart and with distances as defined by the individual VTS authority.

Table 3.6 and Error: Reference source not found.give examples of characteristics suitable for the 3 recommendation levels. The values are not only related to antenna beam width, but shall also allow for tolerances and resolution in displays etc.

For larger (non-point) targets the definition of separation is highly depending on aspect angles, pulse stretch  etc. Proper transmitter and receiver characteristics, rather than specification of absolute separation, shall ensure proper separation of such targets.

Table 3.6  Target separation

| Physical separation between small point targets (< 100 m2) RCS) for discrimination in display and tracking |  | Relaxed specification |  | Standard specification |  | Extended specification |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Presentation | Tracking | Presentation | Tracking | Presentation | Tracking |
| In range (targets aligned in azimuth) | Short range applications (< 5 NM coverage - Inland waterways, harbours etc) | 25 m | 40 m | 20 m | 30 m | 15 m | 25 m |
|  | Long range applications (up to 20 NM coverage - littoral waters, off-shore etc.) | 75 m | 100 m | 60 m | 75 m | 50 m | 60 m |
|  | Very long range applications (> 20 NM coverage) | N/A |  | 100 m | 125 m | 80 m | 100 m |
| In azimuth (targets aligned in range) | Angle between targets as seen from the radar | 0.75 | 0.85 a | 0.6 | 0.7 | 0.5 | 0.55 |
|  | Or distance in meters, whichever is the greater | 25 m | 40 m | 20 m | 30 m | 15 m | 25 m |

### Range performance

The operational requirement under the weather and propagation conditions normal for the individual site should be defined by the VTS authority. The equipment shall give a clear indication and tracking of targets out to such specified ranges.

Table 3.7 give examples of calculated range performance for system characteristics suitable for the 3 recommendation levels.

Table 3.7  Examples of calculated range performance

| Antenna elevation | Target |  | Calculated detection and tracking ranges for fluctuating point targets in clear weather, standard atmosphere and sea state 3 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | RCS | Height | Relaxed specification | Standard specification | Extended specification |
| 20 m ASL | 1 m2 | 1 m ASL | N/A | N/A | 7 NM |
|  | 3 m2 | 2 m ASL | N/A | 6 NM | 8 NM |
|  | 10 m2 | 3 m ASL | 7 NM | 9 NM | 9 NM |
|  | 100 m2 | 5 m ASL | 9 NM | 11 NM | 12 NM |
|  | 1000 m2 | 8 m ASL | 12 NM | 13 NM | 15 NM |
| 50 m ASL | 1 m2 | 1 m ASL | N/A | N/A | 10 NM |
|  | 3 m2 | 2 m ASL | N/A | 10 NM | 12 NM |
|  | 10 m2 | 3 m ASL | 10 NM | 12 NM | 14 NM |
|  | 100 m2 | 5 m ASL | 13 NM | 15 NM | 17 NM |
|  | 1000 m2 | 8 m ASL | 16 NM | 18 NM | 20 NM |
| 100 m ASL | 1 m2 | 1 m ASL | N/A | N/A | 12 NM |
|  | 3 m2 | 2 m ASL | N/A | 13 NM a | 16 NM |
|  | 10 m2 | 3 m ASL | 10 NM a | 17 NM | 19 NM |
|  | 100 m2 | 5 m ASL | 15 NM | 20 NM | 22 NM |
|  | 1000 m2 | 8 m ASL | 22 NM | 23 NM | 25 NM |

## Plot Extraction and Tracking

The requirements in respect to plot extraction and tracking shall be defined by the individual VTS authority on the basis of local conditions, number of radar sensors in a system etc. Table 3.8 suggest values for each individual radar sensor in a system.

Table 3.8 Single radar performance parameters

| Plot Extraction and Tracking Performance |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter |  | Relaxed specification | Standard specification | Extended specification |
| Number of plots per antenna rotation |  |  1000 |  2500 |  5000 |
| Number of tentative tracks |  |  250 |  500 |  1000 |
| Number of confirmed tracks |  |  100 |  200 |  300 |
| PD for automatic track initiation |  |  0.9 |  0.8 |  0.7 |
| PD for maintaining track |  |  0.8 |  0.7 |  0.6 |
| Time for confirmation of tentative track |  |  1 min |  |  |
| Time from track confirmation to track display |  |  15s |  |  |
| Time from track confirmation to achievement of specified accuracy |  |  2 min |  |  |
| Time from data loss to automatic track termination |  |  1 min |  |  |
| Speed of tracked objects |  |  50 knots |  | 70 knots |
| Turn rate of tracked objects |  |  10°/s |  |  20°/s |
| Accuracy in track position | Range |  0.75 % of range covered by the individual radar or 30 meters, whichever is the greater |  |  0.5 % of range covered or 15 meters |
|  | Bearing a |  1 |  |  0,5 |
| Accuracy of track data | Speed a |  2 knots |  1 knot |  1 knot |
|  | Course a |  5° |  2° |  2° |

### Plot Extraction

Plot extraction (see section 2.6.5) shall be automatic. The plot extraction process shall be able to handle a minimum of plots per rotation (see Table 3.8)

### Track Initiation

Track initiation shall be automatic, automatic in selected areas, or manual depending on the concept of operations.. In automatic track initiation mode, all plots in a scan shall be considered potential targets. Some of the plots will be associated with previously established tracks, while the remaining plots shall be considered as candidates for new tracks i.e. tentative tracks. Tentative tracks are propagated if plots from consecutive scans “fit into the picture” within reasonable physical manoeuvrability limits, otherwise the tentative tracks are discarded. The tracking system shall be able to handle at least a certain number of tentative tracks (Table 3.8) and to initiate tracks and eventually to confirm tracks under certain conditions of PD  (Table 3.8) and PFA ≤ 0.01. Automatically generated tentative tracks shall not be shown on the display.

It shall also be possible to initiate a track manually.  In manual track initiation a plot on the radar display is selected by the operator using a graphical tool. When selected this plot shall form the starting point for a tentative track which eventually shall be confirmed or discarded as in the automatic case described above.  Manually generated tentative tracks shall be shown on the display, but they shall be discernible from confirmed tracks by using e.g. a blinking indication or a different colour.

### Maintaining Track

If automatically or manually created tentative tracks persist over a certain length of time the tracks shall be promoted to confirmed tracks.  Confirmed tracks shall be shown on the display. The tracking system shall be able to handle at least a certain number of confirmed tracks (Table 3.8) and to maintain tracks under certain conditions of PD (Table 3.8) and PFA ≤ 0.01.

### Track Termination

If a confirmed track either moves outside a user defined maximum range, into a user defined non-tracking area, or if the track cannot be updated with new plots over a certain length of time, then the track shall be terminated and the operator shall receive a warning on this.

### Track Data Output

Track data output shall be contained in track records. A track record shall at least contain the following data:

- Track identification: an automatically generated integer or alphanumerical label

- Current track date and time: UTC or GPS date and time and/or local date and time

- Current target position: Numerous formats: geographical (Latitude, Longitude), UTM (Northing, Easting), local (Range, Bearing), ...

- Current target speed

- Current target course: in degrees relative to North

The track record shall be extendable with user defined information. It shall be possible to save track record history lists.

## Reliability and availability

The requirements to availability of radar services for any given area in a VTS, and thereby the requirements to reliability, redundancy and maintenance shall be defined by the individual VTS authority.

### Redundancy

The redundancy listed in Error: Reference source not found is appropriate for the majority of radar sites.

Table 3.9 Redundancy

| Redundancy |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Relaxed specification |  |  |  | Standard specification |  |  |  | Extended specification |  |  |  |
| VTS application |  | Inland Waterways | Harbors | Littoral Waters | OFF Shore | Inland Waterways | Harbors | Littoral Waters | OFF Shore | Inland Waterways | Harbors | Littoral Waters | OFF Shore |
| Redundancy | Not required | X | X | X | X | X | X |  |  |  |  |  |  |
|  | Dual Transmitters |  |  |  |  |  |  | X | X | X | X | X | X |
|  | Stereographic view |  |  |  |  |  |  |  |  | X | X |  |  |

### Security functions (optional)

To be specified

## Environmental Specifications and Safety

TBD including:

- OOB emissions and spurious

- EMC/EMI

- Temperature, humidity, etc

- Acoustic Noise

- Storage and transportation

- Electric shock

- Lightning protection

- Safety (Rotating machines etc.)

- Radiation Hazards

- Wind speed (robustness of antennas)

Remember

Protection against other radars

De-icing of antennas may additionally be required in areas with icing conditions.

Stability, UPS etc.

Preventive and corrective maintenance

## Sections to be moved to performance standards for other VTS services

### Fusion of radar data with other sensors

When a target is seen by more than one sensor (including multiple radars), there shall be a capability to derive a single track to be presented to the operator.

### Display

The VTS equipment should provide daylight display of radar information from the VTS area covered by radar. The equipment should provide range scales or areas of display as defined by the individual VTS authority.

The range scale displayed should be clearly indicated at all times. The range scale shall be linear.

Within the effective display radar area, the display should only contain information which pertains to the use of the radar display for VTS services and which has to be displayed there because of its association with a target (e.g. target identifiers, data base information, vectors) or because of some other direct relationship with the radar display.

Multi color displays are preferred but the following requirements should be met:

- Radar target information should be displayed by means of the same basic colors and the echo strength should not be displayed in different colours.

- AIS information may be combined with tracked target radar information or displayed individually; and:

- Additional information may be shown in different colors.

If a light shield is necessary to facilitate operation of the display in high ambient light levels, then means should be provided for its ready attachment and removal.

After changing display area full information should be displayed within a period of time not exceeding 3 seconds.

#### Target information

The course and speed information generated by for acquired targets and for AIS targets should be displayed in a vector or graphic form which clearly indicates the target's predicted motion with relevant symbols. In this regard:

- Predicted information in vector form should be in the form of true vectors.

- Equipment which is capable of presenting target course and speed information in graphic form should also, on request, provide the target's true vector.

- Vectors displayed should be time-adjustable; and:

- A positive indication of the time-scale of the vector in use should be given

The tracked target information should not obscure the visibility of radar targets not being tracked or AIS targets. The display of tracked targets data should be under the control of the radar observer. It should be possible to cancel the display of unwanted data within 3 s.

Means should be provided to adjust independently the brilliance of the tracked target information, AIS information and radar information

When a target appears on the radar display and, in the case of automatic acquisition, or, in the case of manual acquisition, the equipment should present the target's motion trend and the target's predicted motion within the time and accuracy specified by section 4.5

#### Electronic chart information

Electronic chart information may be displayed in such a way that the radar information is not masked, obscured or degraded. If electronic chart information is made available for a radar display it should at least include coastlines, dangers to navigation and fixed and floating aids to navigation. The VTS operator be able to select those parts of the chart, which can be made available and the VTS operator requires to be displayed.

For the superimposition of selected parts of the electronic chart:

- Reference management is required to ensure that the information displayed is correlated and in the same reference and co-ordinate system;

- The whole effective display area should contain the available radar and chart information;

- The display of radar and AIS information should have priority;

- The chart information should not have an adverse effect on the radar or AIS information.

- Radar / AIS information and chart information should be clearly recognizable as such; and in the case of a malfunction of one component, the function of the other component should not be affected.

### Measurements

Provisions should be made to obtain quickly the position of any object which appears on the VTS display

Electronic measurement tools shall be provided for:

- The measurement of range and azimuth relative to any position within the selected display area.

- The measurement of  geographic position at any point within the selected display area.

Accuracy TBD

#### Operational warnings

TBD

# Performance verification methods

The high dynamic characteristics and dependence on propagation conditions furthermore adds uncertainty to tests on installed radar systems and it can be difficult to reproduce tests as conditions may vary substantially from day to day. Therefore a combination of tests and calculations is recommended for verification of radar services.  Please refer to Table 4.10

Table 4.10, Performance verification methods

|  | Design / verification method |  |  |  |
| --- | --- | --- | --- | --- |
|  | During system design | Factory Acceptance Test | Site Acceptance Test | In Operation Test |
| Overall functionality | Analysis | Functional test | Functional test |  |
| Coverage | Analysis including weather influence and propagation |  | Functional test |  |
| Detection performance and dynamic requirements | Simulation including weather influence and propagation | Measurement of antenna and transmitter-receiver parameters, losses etc. | Functional evaluation, compared to weather condition |  |
| Target separation | Analysis | Measurement of individual antenna and transceiver parameters | Check using known structures or controlled targets. |  |
| Tracking performance | Analysis or Simulation |  | Functional test | Acquisition and analysis of statistical data from the system |
| Environmental and Safety issues | Analysis and type tests | Check of workmanship and measurement of parameters | Check of Installation |  |
| Availability | Analysis |  |  | Record and analysis of events |

Methods suggested for the individual item is described in the following paragraphs.

## Methods recommended for system design

TBD

### Type tests

TBD

### Determination of radar coverage and detection performance

TBD

#### Minimum range

TBD

#### Dynamic requirements

TBD

#### Performance with noise and clutter present

TBD

#### Performance in adverse propagation conditions

TBD

#### Target Discrimination

TBD

#### Range performance calculation

TBD

## Factory Acceptance Test

### System functional tests and measurements

To be defined, possibly by references to other standards

### Antenna measurements

TBD, at least Including:

- Gain or directivity

- Radiation patterns, side lobes

- Integrated cancellation ratio, in main beam, outside main beam (only circular polarized antennas)

### Transceiver measurements

TBD, at least including:

- Transmitter pulse characteristics

- Transmitter peak power,

- Transmitter rise and fall time

- Receiver Noise Figure (Overall) and/or Minimum Detectable Signal  (Relations to noise floor to be described)

- Receiver dynamic range , being the ability to receive pulses without degradation (collapse & pulse stretch)

## Site Acceptance Test

TBD

### Functional Tests and Checks

- Check of Transmission line losses etc.

## In operation tests

### Acquisition and analysis of statistical data from the system

TBD

### Record and analysis of events

TBD

Except if targets are hidden by waves

Note that performance in respect to small targets detection is influenced by antenna height as indicated by Table 3.7

1.2 / 1.3  for small radars e.g. used as gap fillers between larger radars in harbors etc.

Target may be difficult to detect and track  in large parts of the clutter region

May be too high in dry, hot regions of the world due to propagation conditions

95% confidence limit when sailing on a straight course

Speed ≥ 5 knots

The use two or more radars to overlook an area (stereographic view) will increase the availability of radar service and it is beneficial in congested areas to eliminate effects of multi path propagation