DRAFT GUIDELINES ON THE EXCHANGE OF REAL TIME DATA BETWEEN VTS SYSTEMS

# Introduction

There is an increasing interest in exchanging data between VTS systems, although it needs to be recognised that it will form only a part of the overall data flow into and out of a VTS.  These systems may be locally or regionally grouped and will be using a variety of sources (including external databases) and which may be site dependent / application specific.  They may also include an interface to other regional systems, for instance SafeSeaNet

Even if several VTS systems have been interfaced, there is currently no standard available covering the different types of data available in a VTS.  To make a standard describing data exchange between VTS systems, it is important to choose a way forward that will take into account the ever evolving needs of new types of information required to be exchanged.  In addition a static description will only lead to proprietary solutions needed to cater for different types of data not covered by it.  However, it is necessary to create a baseline so that interfaces between different vendors will work together without further refinement.

# Objective

This paper provides guidance on the content and type of real time data which may be exchanged between VTS systems and does not seek to address the wider scope of all information likely to be exchanged.

# Data exchange

It is highly desirable to exchange validated data to avoid operational conflicts between VTS systems.  It is essential that the system network architecture prevents data loops.

# TYPES / CLASSES OF REAL TIME DATA THAT MAY BE EXCHANGED

These may be contained in the following types of data :

## Track information

This is dynamic information related to a target being followed by a tracking system, which may be derived from multiple sensors and other sources.  Currently, the most common sources are radar and AIS.  Position, course and speed over the ground and identification with a few additions comprise the parameter set involved in tracking.

### Considerations relating to track data exchange

Regional track data needs special consideration because it is intended to be integrated into the receiving processing system and so latency is critical.

Track data exchange is used to :

- Extend the coverage of own system;

- Avoid the new identification of targets which have already been identified by the neighbouring system.

The following minimum data set has been identified as sufficient for the integration, correlation and display in the receiving VTS system as well as for the storage in an associated database system :

- IMO number (Unique identification)

- Name of vessel;

- Call Sign;

- Position (Latitude / Longitude);

- Course (COG);

- Speed (SOG);

- Heading (if available from AIS);

- Time stamp;

- Data source;

- Quality figure.

A quality figure is an assessment of the reliablity of the transmitted track data.  There is currently no existing standard for such a quality figure.  However, as a preliminary indicator, the sensor data source should suffice.

## Voyage related and static ship information

This information is not necessarily restricted to that provided by AIS and may be application specific to regional or national requirements.  For a comprehensive list of AIS data elements, refer to ITU-R Recommendation M.1371-1.

## Meteorological and hydrological information

This information can vary according to the different types of sensors and information, although NMEA has provided a form of standardisation.

## Digitised radar video data

The radar video data is used both for presentation and as the basis for tracking in a VTS system.  The tracking system should take care of all targets of interest in a system and therefore the exchange of radar video between VTSs generally is not required.  However, there might be different levels of interest, i.e. a commercial operator may look for something different to a military / security focused operator.

# TECHNOLOGIES

There are several methods of data exchange, including NMEA, EDI, XML, TCP/IP, as well as NATO standards.  A format suitable for real time data exchange needs to be selected.

# Abbreviations and definitions

AIS	Universal Automatic Identification System

COG	Course Over the Ground

EDI	Electronic Data Interchange

IMO	International Maritime Organisation

ITU-R	International Telecommunications Union – Radiocommunications Bureau

NATO	North Atlantic Treaty Organisation

NMEA	National Marine Electronics Association

SOG	Speed Over the Ground

TCP/IP	Transmission Control Protocol / Internet Protocol

VTS	Vessel Traffic Services

XML	eXtensible Markup Language

SafeSeaNet is a group of representatives of European Union member states and Norway, co-ordinating the implementation the trans-European information exchange resulting from the Directive on Vessel Traffic Monitoring and Information System (2002/59) and other associated Directives.

# References

- IALA VTS Manual, 2002	3.3.4.2

- IALA VTS Manual, 2002	4.3.2

- ITU-R M.1371-1

- Regional Directives, e.g. EU Directive 2002/59