### 2.1.6 Implementation phase

The main tasks during the Implementation phase will be associated with the:

- preparation and publication of tenders to carry out the construction and installation of the equipment and any construction or alterations to buildings;

- evaluation of tenders and awarding of contracts;

- monitoring of progress of the contracted work; and,

- verification or conduct of tests on individual items of equipment of sub-systems

- [Integrated Logistics Support, documentation,etc.]

- [training]

While it can be expected that individual items of equipment will have been tested before installation, during this phase the system should be tested as an entity.  This should include the integration of VTS Personnel and therefore implies that the recruitment and training programme should be conducted before final acceptance of the system is undertaken.

All documentation necessary for the proper operation of the VTS, including operational rules, procedures and contingency plans, should be developed during this phase. In addition, all national and international administrative and legal matters concerning the establishment and operation of the VTS should be resolved, including the availability of radiocommunication frequencies and, if necessary, the approval by IMO for any traffic routeing schemes or ship reporting systems.

This phase will be complete when the VTS has satisfactorily finished all necessary testing and trials and has full operational capability.