International Maritime Information Systems Ltd.

| Telephone: Fax: E-mail: Internet: | +44 (0) 23 80252952 +44 (0) 1670 821142 info@IMISGlobal.com www.IMISGlobal.com | Cams Hall Cams Hill Fareham Hampshire PO16 8AB England |
| --- | --- | --- |

## VTS AIS INTEGRATION

# INTRODUCTION

The integration of VTS and AIS terminals or systems is now possible. Doing this, however, requires the resolution of a number of issues as listed below (a non-exhaustive list):

- VTS – AIS network function separation

- VTS target selection

- AIS target update rate

- VTS TTI target display on the vessel

- VTS TTI target display on another VTS within the coverage area or during periods of propagation anomaly

# VTS – AIS NETWORK SEPARATION

The VTS is the prime positioning tool used by a VTS due to its ability to deal with all vessels and other targets within the radar coverage area.

AIS systems, although more accurate and offering a feature rich position set of information, requires the complicity of the target or vessel.

AIS systems or networks can generate significant streams of data especially when systems in the coverage area are large and include VTS TTI transmissions and Aids to Navigation AIS based systems.

It may be logically reasoned then that AIS networks and VTS systems, although carefully integrated, should be able to operate as separate and fully independent systems thus enabling either system to offer a level of functionality during a failure of the other system for whatever reason. The design of AIS networks and the topology recommended by the AIS working groups easily enables this functionality thus allowing the benefits of separate but integrated systems to be achieved.

Another item that requires some resolution is the ability of the VTS operators to transmit commands to the AIS network. These AIS network commands could include:

- The AIS assign message where frequencies and update rates of AIS terminals on vessels can be changed.

It is clear from this simple example that network functionality and VTS operation environments need to be carefully defined and separated, if required.

# TARGET SELECTION

The AIS message set allows for the transmission of the VTS Tactical Traffic Image [TTI]. For this to be done, a number of VTS targets need to be selected, the AIS message constructed and then passed to the AIS terminal or AIS network for transmission.

If a large set of targets is chosen, then a significant portion of the AIS slot map is going to be occupied by VTS TTI transmissions. If a small set or a very low update rate is selected, then the value of the TTI transmission is diminished.

This selection process and the methods and criteria for target selection, provides for an interesting discussion.

These targets could include one or more of the following sets of targets:

- All targets within an area of interest

- All radar targets (since all AIS targets are already seen by vessels fitted with AIS)

- All fused targets (where the VTS has both an AIS and Radar set of data)

# AIS TARGET UPDATE RATE

The transmission of the TTI can occupy a significant portion of the available AIS slot map. In a harbour environment, this could be dealt with by increasing the AIS slot capacity through an increase in the number of AIS base stations. These base stations would be installed in a manner that would limit their coverage to small areas of interest only.

The design and integration of a VTS / AIS network requires that the AIS network have processes in place that prevent a VTS operator from selecting a set of targets for broadcast as the TTI that would overload the AIS protocol or would limit the available free slots and thus reduce the coverage or apparent coverage of the AIS network.

# VTS TTI DISPLAY ON A VESSEL

If the TTI is transmitted using an AIS base station or AIS network connected to the VTS, then the vessels must have a method of displaying these targets in unambiguous way. This requires that AIS targets be either fused on the vessel side with VTS TTI targets are the TTI targets will have to be easily distinguished from direct AIS targets.

If this is not done, the mariners may see a display that is overloaded with target information, most of which could be duplicates.

# VTS TO VTS DISPLAY ISSUES

During periods of anomalous propagation, coverage of AIS networks has increased from a normal 40NM to over 200NM. This is not a problem when there is only one VTS in the area or if the VTS’s that fall within the coverage area of each other during anomalous propagation do not transmit their TTI as an AIS message.

If they do, the result will be that each of the VTS systems being able to decode and display the VTS TTI of the other VTS.

Should the target selection criteria for retransmit as part of the TTI not be well thought through, this situation could result in the VTS systems exchanging VTS targets through AIS TTI messages and overloading both systems.