# Service Design for the Provision of AtoN Information Service to End-users using IEC 63173-2 (SECOM)

Contents

1	Introduction	5

1.1	Purpose of the Document	5

1.2	Intended Readership	5

1.3	Inputs from Other Sources	5

2	Service Identification	7

3	Technology Introduction	8

4	Service Design Overview	10

4.1	General	10

4.2	Service interfaces	11

4.3	Service Discovery	12

5	Physical Data Model	13

6	Service Interface Behaviour	15

6.1	Get Interface	15

6.1.1	Operation Get	16

6.2	Get Summary Interface	18

6.2.1	Operation Get Summary	19

6.3	Subscription Interface	21

6.3.1	Operation Subscribe	21

6.3.2	Operation Remove Subscription	23

6.4	Acknowledgement Interface (Optional)	24

6.4.1	Operation Acknowledgement	25

6.5	Upload interface (Consumer Interface - Optional)	26

6.5.1	Operation Upload	27

6.6	Subscription Notification Interface (Consumer Interface - Optional)	30

6.6.1	Operation Subscription Notification	30

7	Service Dynamic Behaviour	32

7.1	Client-initiated Retrieval of AtoN Information	32

7.2	Subscription-based AtoN Information Propagation	33

8	References	36

9	Acronyms and Terminology	37

1.4	Acronyms	37

1.5	Terminology	37

Appendix A	Service implementation XML template	39

Appendix B	Service instance OpenAPI definition template	40

Table of figures

Figure 1: Overview of SECOM as in [2].	8

Figure 2: Envisioned S-125 Operational Context	10

Figure 3: The Provision of AtoN Information Service Interfaces Overview.	11

Figure 4: Overview of S-125 Feature types.	13

Figure 5: An example REST GET request on the Get interface.	17

Figure 6: An example response to the REST GET request on the Get interface.	18

Figure 7: An example REST GET request on the Get Summary interface.	20

Figure 8: An example response to the REST GET request on the Get Summary interface.	20

Figure 9: An example REST POST request on the Subscription Interface.	23

Figure 10: An example response to the REST POST request on the Subscription Interface.	23

Figure 11 : An example REST DELETE request on the Subscription interface.	24

Figure 12 : An example response to the REST DELETE request on the Subscription interface.	24

Figure 13 : An example REST POST request on the Acknowledgement interface.	26

Figure 14 :  An example response to the REST POST request on the Acknowledgement interface.	26

Figure 15 : An example REST POST request on the Upload interface.	29

Figure 16 : An example response to the REST POST request on the Upload interface.	29

Figure 17 : An example REST POST request on the Subscription Notification interface.	30

Figure 18 : An example response to the REST POST request on the Subscription Notification interface.	31

Figure 19: Client-Initiated AtoN Information Retrieval through the Get operation.	32

Figure 20: Client-Initiated AtoN Information Retrieval through the Get Summary operation.	33

Figure 21: Subscription requested by external service.	34

List of tables

Table 1 : Technical Design Identification	7

Table 2: Service Interface Mapping	12

Table 3: Operation GET Parameters	16

Table 4 : Operation GET SUMMARY Parameters	19

Table 5: Operation SUBSCRIPTION Parameters	22

Table 6: Operation REMOVE SUBSCRIPTION Parameters	24

Table 7 : Operation ACKNOWLEDGEMENT Parameters	25

Table 8 : Operation UPLOAD Parameters	27

Table 9 : Operation SUBSCRIPTION NOTIFICATION Parameters	30

# Introduction

The document is structured according to the IALA Guideline “G1128 - The Specification of e-Navigation Technical Services” [1].

## Purpose of the Document

The purpose of this service design is to provide a design for the implementation of the digital service for the Provision of AtoN information to end-users, IEC 63173-2 (SECOM) and S-125 product used as the S-100 series data model for the actual message payload.

The aim is to document the key aspects of the service name so that implementers know how the specification is to be implemented in an interoperable way and how the interaction between the actors defined in the specification is implemented using the APIs defined in SECOM. For this purpose, we define:

- Why SECOM was chosen to facilitate the implementation.

- The main elements of the service:

- the components it is composed of,

  - interfaces provided,

  - the operations of the service,

  - and the parameters in the operations.

  - The data model of the service

- The dynamic behaviour of the service, i.e. how the use cases defined in the specification are actually technically implemented.

## Intended Readership

This service specification is intended to be read by service architects, system engineers and developers in charge of designing and developing an instance of the VTS Traffic Clearance Service using SECOM.

Furthermore, this service specification is intended to be read by enterprise architects, service architects, information architects, system engineers and developers in pursuing architecting, designing and development activities of other related services.

## Inputs from Other Sources

This section provides an overview of activities, which are dealing with similar topics and lists already finished ones that provided inputs to this activity.

This technical design specification is based on the IEC 63173-2 (SECOM) [2] standard and is written in accordance to the IALA G1128 guideline. As such, the Service Design Description for SECOM REST template [3] document is assumed to be followed, whenever relevant definitions and operations are mentioned. This provides a generic template for all service design specifications that adhere to SECOM, in order to reduce the complexity of the documentation, as well as to keep them in line with any future changes in the SECOM standard.

In addition, the General Lighthouse Authorities of UK and Ireland (GLA), through their Research and Development Directorate (GRAD) have developed a prototype implementation of a technical service able to provide AtoN information in the S-125 data product format, and in line with the requirements of the IEC 63173-2 (SECOM) standard. This was taken into account while compiling this service design specification document.

# Service Identification

The purpose of this section is to provide a unique identification of the service and describe where the service is in terms of the engineering lifecycle.

Table 1 : Technical Design Identification

| Name | Provision of AtoN Information Service to End-users |
| --- | --- |
| Implements | Service Specification for the Provision of AtoN Information Service to End-users <service specification - MRN to be assign> |
| ID | urn:mrn:iala:techsvc:sd:spec prefix:technology:version |
| Version | 0.0.1 |
| Description | Technical Design for the provision of AtoN information to end-users via web services, using the IEC 63173-2 (SECOM) communication standard. |
| Keywords | AtoN Information, REST, SECOM, S-125, web service |
| Architect(s) | Nikolaos Vastardis (GRAD) |
| Status | PROVISIONAL |

# Technology Introduction

The IEC 63173-2 (SECOM) in general provides a set of requirements for secure data exchange, as well as a technical service interface design. It is using REST (Representational State Transfer) as its architectural style, which is an approach to communications that is often used in the development of Web services. According to [2], SECOM includes information services interfaces (APIs) for data exchange, information security measures by a SECOM Public Key Infrastructure (PKI), communication channel security and data protection to enable secure communication. Further, SECOM includes interfaces for service discoverability as shown in Figure 1. The purpose with these included components is for SECOM to provide technical interoperability, where the same service interface is used for exchanging the information regardless of its operational use, up to the level of exchanging information securely online. Although designed for IHO S-100 [4] based products, SECOM is technically payload agnostic and applicable also for other types of data.

An overview of the scope of SECOM in the content of vendor specific private API towards an example end-user such as a ship presented bellow in Figure 1.

Figure 1: Overview of SECOM as in [2].

The SECOM information service interface includes the public side exposed on the Internet as depicted in Figure 1. The "last mile" links between a SECOM information service interface and the end-user application is not defined in the standard and hence different solutions between the service instance and shore/ship’s system are possible.

SECOM information security includes the communication channel security, a variant of PKI and data protection scheme alternatives for the information exchange with full or partial compliance with IHO S-100. The data protection is provided between end-users. SECOM PKI includes the definition of a set of service interfaces for key management.

# Service Design Overview

## General

This section provides an overview of the main elements of the service design and a mapping of the design elements to the service specification elements.

The envisioned operational context of the Provision of AtoN Information to End-users service, as defined in [5], is presented in the Figure 2.

Figure 2: Envisioned S-125 Operational Context

This scenario depicts an envisaged future of S-125 data exchange. Based on a standardized structured AtoN information (List of Lights) format, compatible systems will be able to exchange AtoN Data seamlessly. As depicted, each AtoN Administration may have a unique AtoN Information Management System. This system should automatically promulgate S-125 Data from the authoritative source for use by national and local authorities (e.g.  Coastal State Authorities, Harbor and Port Authorities), the mariner public, as well as being available for use by commercial value-added services providers.  The provision of an “S-125 Service” described in this document, is depicted as those activities after production of the S-125 data by AtoN administrations in the above diagram. It is envisioned that the legacy IHO S-12 data is a subcomponent and replaced by S-125. Administrations may desire to continue providing S-12 format human readable “paper reference” List of Lights in addition to the S-125 Marine Navigation Services, either transitionally, or indefinitely.

This document however, is intended to provide a much clearer definition of the Provision of AtoN Information to end-users technical service. The service itself, even if it comprises of multiple interacting components, can be seen as a single functional entity with well-defined inputs and outputs. Any implemented instance, in order to conform to the current specification, it must be a web-based service, exposing a set of REST interfaces to communicate with the end-users. These interfaces must be compatible with requirements specified in SECOM and make the Aton Information available in a data format that follows the S-125 data product specification.

## Service interfaces

Figure 3 gives an overview of the interfaces of the service design. Note that these are for the most part inherited from the respective service specification document [5]. The interfaces for both the service consumer and the service provider are displayed, as both are required for the dynamic behaviour of the service. More information on that is provided in the respective Section 7 of this document.

Figure 3: The Provision of AtoN Information Service Interfaces Overview.

The table below provides the mapping of service design to service specification, as well as references to the formal descriptions of the service interfaces and operations. Evidently the interfaces defined in this document are almost identical to the ones in the service specification, with a notable exception on the Subscription and Remove Subscription service specification interfaces which in the service design case are performed by different operations in the same Subscription interface. It should also be noted that all the interfaces specified in the service design adhere to the respective definitions provided by the SECOM REST service design template [3].

Table 2: Service Interface Mapping

| Service Specification |  | Service Design |  |
| --- | --- | --- | --- |
| Service Interface | Service Operation | Service Interface | Service Operation |
| Upload | upload | Upload | upload |
| Acknowledgement | acknowledgement | Acknowledgement | acknowledgement |
| Get | get | Get | get |
| Get Summary | getSummary | Get Summary | getSummary |
| Subscription | subscribe | Subscription | subscribe |
| Remove Subscription | removeSubscription |  | removeSubscription |
| Subscription Notification | subscriptionNotification | Subscription Notification | subscriptionNotification |

## Service Discovery

Services implemented according to this design must submit their instance description to a valid service registry that follows the Maritime Connectivity Platform (MCP) Service Registry definition as per [6].

An XML template for the instance description is provided as an annex to this design. Appendix A.

# Physical Data Model

This section provides an overview of the main elements of the service technical design and a mapping of the design elements to the service specification elements.

The data model of the service is a combination of Javascript Object Notation (JSON) and XML (the S-125 payload).  At the top level, SECOM dictates that the services interface are designed according to REST and the type of data is based on JSON. The SECOM JSON structure is defined in [2], in Section 5. As such, the top level SECOM JSON objects can accept payload data that can be either XML, compressed XML or an encrypted binary message, typically compressed, as described in the exchange metadata object. The data can contain either one message, such as an S-125 dataset, or a set of messages (files) compressed in a ZIP container and described by an “ExchangeSet” as described in IHO S-100. The data is encoded as one row Base64 before transferred.

Complete and updated documentation can be found in the latest S-125 Data Product Specification and SECOM and the respective service design template [3]. Therefore for the sake of brevity, the details will be omitted from this document. A short overview of S-125 however can be seen in Figure 4. Information on the individual SECOM JSON objects exchanged will also be briefly presented in the respective interface definition sections.

Figure 4: Overview of S-125 Feature types.

An additional data structure that is used in the context of the service in question, is the concept of the Maritime Resource Name (MRN). This is defined clearly by IALA under the guideline G1143 [7], and all subsequent references to MRN are assumed to be governed by it. In a nutshell, MRN are unique identifiers based on URN, necessary for the development of e-Navigation in order to maintain harmonization across domains and services.

.

# Service Interface Behaviour

This section describes the details of each service interface. One sub-section is provided for each Service Interface. The Service Interface design covers the static design description, while the dynamic design (behaviour) is described in Section 7.

The operations defined in this section directly refer to their counterpart in the service specification and use the same operation principles, parameters and return values, unless otherwise stated. The encoding of all data model instances must be JSON as specified in SECOM and the respective REST design template [3]. The basic types included in the JSON objects must also conform to the specifications of design template, unless otherwise stated. Geometry data in particular, must be defined in a WKT format, which is a human readable representation for spatial objects like points, lines, or enclosed areas on a map. SECOM uses a subset of the objects in WKT CRS 2 (see ISO 19162) with EPSG projection 4326 – WGS 84.

The S-125 payload data included in the SECOM JSON messages must all be encoded in Base64 format before being transmitted to the client. Additional compression and encryption operations are optional and should take place beforehand, and in accordance with the SECOM requirements.

It is strongly recommended that service providers use the secure Hypertext Transfer Protocol (HTTPS) for access to their service interfaces. The protocol, host, port and the root path under which the service is available is referred to as base URL. Specific service interfaces and information can be retrieved by adding a sub-path to the base URL. The sub-path is specified for all interfaces provided by the service provider.

Example:

base URL: https://aton-service:8765/api/secom/v1/

sub-path: object/

For example, sending a HTTP GET (see subsection 6.1.1) request to:

https://aton-service:8765/api/secom/v1/object/

The request should be performed with the correct parameters would return a SECOM-defined response containing one or more S-125 datasets.

## Get Interface

The Get interface is used by service consumers for retrieving AtoN information from a service provider, e.g., S-125 datasets that are applicable for a specified geographical area. The consumer can filter the requested S-125 dataset results by the dataset identifier, validity period and/or geometry.

Name: Get Interface

Sub-path: object/

HTTP-Method: GET

Cookies: Session-Cookies (optional)

Content-Type: application/json

### Operation Get

This operation can be used for retrieving and filtering S-125 datasets from a service provider. It follows a request-response paradigm (REQUEST_RESPONSE), where a client queries the server, and based on a set of filtering criteria, receives an appropriate response back. The service response should be formatted as a list of all applicable S-125 datasets that match all the provided query parameters. Each S-125 dataset contained in the response should include all AtoN entries allocated to it, not just the ones matching the query parameters.

#### Operation Functionality

This operation is performed by a REST GET request, carrying a number of query parameters. If multiple query parameters are provided, only results that match all requested filters are returned. If no parameters are given, the response is either an empty list or all available datasets the consumer has been given access to by the service provider. A service provider must offer the appropriate encoding of AtoN information in accordance to the S-125 standard. In addition, as per the requirements of SECOM, the provider must sign the data response using a valid certificate generated by the employed SECOM PKI. The signature, the public certificate and the root certificate of the PKI must all be present in the response.

All instances of services providing AtoN information are required to support all query parameters presented in the following sub-section 6.1.1.2. In most cases the parameter description provides an adequate definition. The <AreaName> parameter however is intentionally left more abstract, so that a more generic server-side area naming convention can be utilised.

#### Operation Parameters

The operational parameters and generated responses follow the respective interface definition in the SECOM REST service design template [3]. Table 3 however includes only the operational parameters applicable for this operation, along with any additional ones.

Table 3: Operation GET Parameters

| Parameters (in) | Encoding | Mult. | Description |
| --- | --- | --- | --- |
| containerType | Enumeration as defined in [3] | 0..1 | Requested encoding of the data. If not provided, the service provider determines the standard encoding. |
| dataReference | UUID | 0..1 | Dataset reference identifier to a specific S-125 dataset. The reference can be provided as search criteria. A list of references can be retrieved via Get Summary interface. If no references are provided, it's up to the service to decide what to return. |
| geometry | WKT | 0..1 | Geometry condition for geo-located information objects. This can be used to filter on geometric shapes (e.g., filter S-125 datasets by a bounding box). |
| unlocode | String | 0..1 | UN/LOCODE (United Nations Code for Trade and Transport Locations) of a defined object (e.g., “CN” for China). |
| areaName | String | 0..1 | Name or identifier of a server-side defined area (e.g., “German Bight”). |
| atonIdentifier | MRN | 0..1 | An AtoN MRN can be provided to query information related to a specific AtoN. |
| page | Integer | 0..1 | The number of the page for retrieving paged results. If not provided the default value of this parameter is set to zero (0). |
| pageSize | Integer | 0..1 | The size of each page in a paged result. If not provided the default value of this parameter is not set and the resulting S-125 datasets are not paged. |

An example of this operation can be found in the following Figure 5 and Figure 6. Note that since the client request is a GET, it does not contain a request body.

Figure 5: An example REST GET request on the Get interface.

Figure 6: An example response to the REST GET request on the Get interface.

## Get Summary Interface

The Get Summary interface is used by service consumers to discover the AtoN information that is made available (and is accessible) by a service provider. The service provider will respond by listing the available (and accessible) S-125 datasets that are of interest to the consumer. This list should only contain summary information on each dataset, not the dataset content itself. The dataset reference identifiers can be used to retrieve the full S-125 datasets via the Get interface. The consumer can filter the requested S-125 dataset summary results by the validity period and/or geometry.

Name: Get Summary Interface

Sub-path: object/summary/

HTTP-Method: GET

Cookies: Session-Cookies (optional)

Content-Type: application/json

### Operation Get Summary

This operation can be used for discovering the S-125 datasets that are available and accessible by a service provider. This operation is typically used to conserve bandwidth, and make the query process faster by retrieving a summary of the information and then only query specific datasets from the service provider via the Get interface. It follows a request-response paradigm (REQUEST_RESPONSE), where a client queries the server, and based on a set of filtering criteria receives an appropriate response back. The server response should be formatted as a summary list of all applicable S-125 datasets that match all the provided query parameters. Each item contained in the response should only include summary information on the corresponding S-125 dataset, such as the data reference identifier, the status, a short description and the dataset size.

#### Operation Functionality

This operation is performed by a REST GET request, carrying a number of query parameters. If multiple query parameters are provided, only results that match all requested filters are returned. If no parameters are given, the response is either an empty list or all available dataset summaries the consumer has been given access to by the service provider. According to SECOM, the service provider is not required to sign the returned summary information.

All instances of services providing AtoN information are required to support all query parameters presented in the following sub-section 6.2.1.2. In most cases the parameter description provides an adequate definition. The <AreaName> parameter however is again intentionally left more abstract, so that a more generic server-side area naming convention be utilised.

#### Operation Parameters

The operational parameters and generated responses follow the respective interface definition in the SECOM REST service design template [3]. Table 4 however includes only the operational parameters applicable for this operation, along with any additional ones.

Table 4 : Operation GET SUMMARY Parameters

| Parameters (in) | Encoding | Mult. | Description |
| --- | --- | --- | --- |
| containerType | Enumeration as defined in [3] | 0..1 | Requested encoding of the data. If not provided, the service provider determines the standard encoding. |
| geometry | WKT | 0..1 | Geometry condition for geo-located information objects. This can be used to filter on geometric shapes (e.g., filter AtoN information by a bounding box). |
| unlocode | String | 0..1 | UN/LOCODE (United Nations Code for Trade and Transport Locations) of a defined object (e.g., “CN” for China). |
| areaName | String | 0..1 | Name or identifier of a pre-defined area (e.g., “German Bight”). |
| atonIdentifier | MRN | 0..1 | An MRN can be provided to query information related to a specific AtoN. |
| page | Integer | 0..1 | The number of the page for retrieving paged results. If not provided the default value of this parameter is set to zero (0). |
| pageSize | Integer | 0..1 | The size of each page in a paged result. If not provided the default value of this parameter is not set and the resulting S-125 datasets are not paged. |

An example of this operation can be found in the following Figure 7 and Figure 8. Note that since the client request is a GET, it does not contain a request body.

Figure 7: An example REST GET request on the Get Summary interface.

Figure 8: An example response to the REST GET request on the Get Summary interface.

## Subscription Interface

The purpose of the interface is for service consumers to request or cancel a subscription on AtoN information. Subscriptions are made either on specific parameters, or the information decided upon by the service provider.

Name: Subscription Interface

Sub-path: subscription/

HTTP-Method: POST, DELETE

Cookies: Session-Cookies (optional)

Content-Type: application/json

### Operation Subscribe

The Subscription interface can be used to subscribe to a set of S-125 datasets. The specific set of datasets that are covered by the subscription is identified by the provided query parameters. Note that the dataset reference identifiers of the S-125 datasets can be retrieved by the Get Summary interface. It follows a publish-subscribe paradigm (PUBLISH_SUBSCRIBE), in which the consumers register their interest in a set of information and receive updates whenever one of the corresponding S-125 datasets changes.

#### Operation Functionality

This operation is performed by a REST POST request, containing the subscription parameters in the request body. If no parameters are specified, the consumer is assumed to intend to subscribe to updates on all active datasets.

The service needs to keep track of the following operations:

- New subscriptions

- Changes between two points in time (Service-level Delta)

- Cancellation of datasets

A subscription can be cancelled through the Remove Subscription operation or can be limited by specifying a subscription period. In a subscription, only datasets for which updates exist are sent to the service consumer. Therefore, it is recommended that before a consumer subscribes to updates, it initially requests all applicable S-125 datasets via the Get interface.

This interface consumes the following consumer interfaces:

- Upload

- Subscription Notification

The Upload consumer interface is utilised by the service provider to push the updated S-125 datasets to the subscribed service consumer. The Subscription Notification consumer interface on the other hand, is used to inform the consumer that on the status of the subscription, i.e., when it has been successfully activated or removed. More information on this operation can be found in the section describing the dynamic behaviour of the Subscription interface. Note that for each new subscription, a Subscription Identifier should be made available to the consumer though the interface’s response.

All instances of services providing AtoN information are required to support all query parameters presented in the following sub-section 6.3.1.2. In most cases the parameter description provides an adequate definition. The <AreaName> parameter however is again intentionally left more abstract, so that a more generic server-side area naming convention be utilised.

#### Operation Parameters

The operational parameters and generated responses follow the respective interface definition in the SECOM REST service design template [3]. Table 5 however includes only the operational parameters applicable for this operation, along with any additional ones.

Table 5: Operation SUBSCRIPTION Parameters

| Parameters (in) | Encoding | Mult. | Description |
| --- | --- | --- | --- |
| dataReference | UUID | 0..1 | Dataset reference identifier to a specific S-125 dataset. The reference can be provided as a query parameter. A list of references can be retrieved via Get Summary interface. If no references are provided, it's up to the service to decide what to return. |
| geometry | WKT | 0..1 | Geometry condition for geo-located information objects. This can be used to filter on geometric shapes (e.g., filter AtoN information by a bounding box). |
| unlocode | String | 0..1 | UN/LOCODE (United Nations Code for Trade and Transport Locations) of a defined object (e.g., “CN” for China). |
| areaName | String | 0..1 | Name or identifier of a pre-defined area (e.g., “German Bight”). |
| subscriptionPeriod | DateTime | 0..1 | The period for which the subscription is active (start and end time). |
| atonIdentifier | MRN | 0..1 | An MRN can be provided to query information related to a specific AtoN. |

An example of this operation can be found in the following Figure 9 and Figure 10. Note that the client request is a POST, and the associated request body is displayed under the request.

Figure 9: An example REST POST request on the Subscription Interface.

Due to the design of the SECOM subscription operation, the service does not necessarily know in advance the client which requested the subscription. This can be identified by the client service MRN, which however is not available in the subscription request body. Therefore in the current service design, the client MRN is provided through the request HTTP headers. This workaround is compatible with the specifications of SECOM and allows the service to identify the client and locate its uploading interface using an appropriate SECOM-compliant service registry. More information on the dynamic behaviour of the subscription operation and its relation to the SECOM-compliant service registry is provided in the next section of this document.

Figure 10: An example response to the REST POST request on the Subscription Interface.

Once the subscription request is received and processed successfully, the service will create a new subscription record for the respective client and return a unique UUID identifier for that record. The response containing with the generated are presented in the example service response of Figure 10.

### Operation Remove Subscription

The Remove Subscription interface can be used to remove subscriptions that were created earlier through the Subscribe operation. It follows a one-way paradigm (ONE_WAY), where a client sends data to consumer without expecting information back, other than a technical response.

#### Operation Functionality

This operation is performed by a REST DELETE request, containing the identifier of an existing subscription in the request body. The SECOM interface specification is not quite clear on whether the MRN of the client that originally requested the subscription is to be checked against the MRN of the client who performs this request. It is therefore assumed that any valid client can terminate an existing subscription. If no subscription identifier parameter is specified, the service will respond with an error code and an appropriate message.

This interface consumes the following consumer interfaces:

- Subscription Notification

The Subscription Notification consumer interface is used to inform the consumer that on the status of the subscription, i.e. when it has been successfully removed.

#### Operation Parameters

The operational parameters and generated responses follow the respective interface definition in the SECOM REST service design template [3]. Table 6 however includes only the operational parameters applicable for this operation, along with any additional ones.

Table 6: Operation REMOVE SUBSCRIPTION Parameters

| Parameters (in) | Encoding | Mult. | Description |
| --- | --- | --- | --- |
| subscriptionIdentifier | UUID | 1 | Specific identifier of the information object to remove the subscription for. Received in subscription request or subscription notification. |

An example of this operation can be found in the following Figure 11 and Figure 12. Note that the client request is a POST, and the associated request body is displayed under the request.

Figure 11 : An example REST DELETE request on the Subscription interface.

Figure 12 : An example response to the REST DELETE request on the Subscription interface.

## Acknowledgement Interface (Optional)

Optional interface provided to cater for acknowledgement of received AtoN information.

Name: Acknowledgement Interface

Sub-path: subscription/

HTTP-Method: POST, DELETE

Cookies: Session-Cookies (optional)

Content-Type: application/json

### Operation Acknowledgement

During the upload of information (see Subsection 6.5), an acknowledgement can be requested if the information has been delivered successfully to the service consumer. This interface follows a one-way paradigm (ONE_WAY), where a service sends data to consumer without expecting information back, other than a technical response.

#### Operation Functionality

This operation is performed by a REST POST request, containing the UUID identifier and the type of the transaction to be acknowledged, delivered in the request body. These parameters are packaged inside a SECOM envelope acknowledgement object, which is further signed to ensure that the acknowledgement notification has not been tampered with.

#### Operation Parameters

The operational parameters and generated responses follow the respective interface definition in the SECOM REST service design template [3]. Table 7 however includes only the operational parameters applicable for this operation, along with any additional ones.

Table 7 : Operation ACKNOWLEDGEMENT Parameters

| Parameters (in) | Encoding | Mult. | Description |
| --- | --- | --- | --- |
| envelope | EnvelopeAckObject as defined in [3] | 1 | The acknowledgement payload to be signed |
| digitalSignature | String | 1 | The signature of the enveloped in HEX format without whitespace or line breaks. |
| EnvelopeAckObject |  |  |  |
| createdAt | DateTime | 1 | Creation time for the acknowledgement. |
| envelopeCertificate | String | 1 | The public certificate of the sender used to verify the envelope object signature. |
| envelopeRootCerficateThumbprint | String | 1 | Claimed thumbprint for signed root key (X.509 Certificate). |
| transactionIdentifier | UUID | 1 | Reference identifier given in upload. |
| ackType | AckTypeEnum as defined in [3] | 1 | Type of acknowledgement. |
| nackType | NackTypeEnum as defined in [3] | 0..1 | Information details if an error occurs. |
| envelopeSignatureTime | DateTime | 1 | Timestamp when the envelope is signed. |

An example of this operation can be found in the following Figure 13 and Figure 14. Note that the client request is a POST, and the associated request body is displayed under the request.

Figure 13 : An example REST POST request on the Acknowledgement interface.

Figure 14 :  An example response to the REST POST request on the Acknowledgement interface.

## Upload interface (Consumer Interface - Optional)

The purpose of this interface is to upload (push) AtoN information to a service consumer. Hence, a consumer needs to implement this interface in order to receive S-125 datasets from a subscription or a broadcast.

Name: Upload Interface

Sub-path: object/

HTTP-Method: POST

Cookies: Session-Cookies (optional)

Content-Type: application/json

### Operation Upload

This interface allows service consumers to be directly informed in regards to new or updated S-125 datasets made available. It follows a request callback paradigm (REQUEST_CALLBACK), where sent messages are expected to result in a message sent asynchronously back.

#### Operation Functionality

This operation is performed by a REST POST request, containing the S-125 datasets to be uploaded onto the application that supplies the interface. In the context of the Provision of AtoN Information to end-users service, this is the service consumer. The S-125 dataset content, as in the case of the Get interface, is signed by the service provider and the signature information is contained inside a SECOM metadata object. In the case of the Upload interface however, the generated response is further packaged inside a SECOM envelope object, in order to ensure that the envelope has not been tampered with (e.g., in terms of the envelope creation time, the subscription flag etc.). The upload operation is used both in single uploads and uploads under a subscription. The parameter <fromSubscription> indicates whether the upload took place within or outside a subscription by the consumer.

This interface consumes the following provider interfaces:

- Acknowledgement

When uploading the message, an acknowledgement can be requested. This is expected to be received when the uploaded message has been delivered to end system (technical acknowledgement), and if supported, when the message has been opened (read) by the end user (operational acknowledgement). More information on this operation can be found in Section 7.2, describing the dynamic behaviour description of the subscription on AtoN information.

#### Operation Parameters

The operational parameters and generated responses follow the respective interface definition in the SECOM REST service design template [3]. Table 8 however includes only the operational parameters applicable for this operation, along with any additional ones.

Table 8 : Operation UPLOAD Parameters

| Parameters (in) | Encoding | Mult. | Description |
| --- | --- | --- | --- |
| envelope | EnvelopeUploadObject as defined in [3] | 1 | The complete envelope object being uploaded to the receiver, including data and message properties. |
| envelopeSignature | String | 1 | The signature of the enveloped in HEX format without whitespace or line breaks. |
| EnvelopeUploadObject |  |  |  |
| data | Base64 | 1 | The S-125 XML payload, Base64 encoded. The data can be open, protected and/or compressed. |
| containerType | ContainerTypeEnum as defined in [3] | 1 | Container type of message. |
| dataProductType | SECOM_DataProcuctType.Name as defined in [3] | 1 | The name column of the SECOM_DataProductType. |
| exchangeMetadata | SECOM_ExchangeMetadata as defined in [3] | 1 | The exchange metadata that contains information regarding the protection scheme, compression, signature and claimed identity. |
| fromSubscription | Boolean | 1 | A flag that indicates whether the data has been uploaded within an active subscription or not. |
| ackRequest | AckRequestEnum as defined in [3] | 1 | A flag to indicate that an acknowledgement is expected to be returned when the data is delivered to the end-user, and/or when the content is processed (opened) by the end user. |
| transactionIdentifier | UUID | 1 | Transaction identifier to be used e.g., in the acknowledgement. |
| envelopeSignatureCertificate | String | 1 | The public certificate of the sender, used to verify the envelope object signature. |
| envelopeRootCertificateThumbprint | String | 1 | Claimed thumbprint for signed root key (X.509 Certificate). |
| envelopeSignatureTime | DateTime | 1 | Timestamp when the envelope is signed. |

An example of this operation can be found in the following Figure 15 and Figure 16. Note that the client request is a POST, and the associated request body is displayed under the request.

Figure 15 : An example REST POST request on the Upload interface.

Figure 16 : An example response to the REST POST request on the Upload interface.

## Subscription Notification Interface (Consumer Interface - Optional)

The interface receives notifications by service provider, when a subscription is created or removed.

Name: Upload Interface

Sub-path: subscription/notification

HTTP-Method: POST

Cookies: Session-Cookies (optional)

Content-Type: application/json

### Operation Subscription Notification

The interface receives notifications by the service provider, when subscription is created or removed, either internally by information owner, or externally on request. This interface follows a one-way paradigm (ONE_WAY), where a service sends data to consumer without expecting information back, other than a technical response. More information on this operation can be found in Section 7.2, describing the dynamic behaviour description of the subscription on AtoN information.

#### Operation Functionality

This operation is performed by a REST POST request, containing the UUID identifier of the subscription that is involved in the notification and the event that took place.

#### Operation Parameters

The operational parameters and generated responses follow the respective interface definition in the SECOM REST service design template [3]. Table 9 however includes only the operational parameters applicable for this operation, along with any additional ones.

Table 9 : Operation SUBSCRIPTION NOTIFICATION Parameters

| Parameters (in) | Encoding | Mult. | Description |
| --- | --- | --- | --- |
| subscriptionIdentifier | UUID | 1 | Specific identifier of the subscription for which the status has being updated. |
| eventEnum | SubscriptionEventEnum as defined in | 1 | Type of subscription status update event e.g., Created or Removed. |

An example of this operation can be found in the following Figure 17 and Figure 18. Note that the client request is a POST, and the associated request body is displayed under the request.

Figure 17 : An example REST POST request on the Subscription Notification interface.

Figure 18 : An example response to the REST POST request on the Subscription Notification interface.

# Service Dynamic Behaviour

This section describes the interactive behaviour of the operations between the service provider and the service consumers. The AtoN Information service supports two different types of operations, namely the client-initiated retrieval of AtoN information, and the subscription-based AtoN information propagation. Both are discussed in the following sections respectively.

## Client-initiated Retrieval of AtoN Information

The client-initiated retrieval is triggered by the service consumer, when it issues request on the appropriate service provider operations (i.e. Get and Get Summary). This operation covers Use Case #1, Use Case #2 and Use Case #4, as outlined in Section 3.4.4 of the respective service specification document. All three use cases can be achieved by two methods; either by directly requesting specific S-125 datasets, using the operation parameters of the Get operation, or by first requesting the S-125 dataset summary information through the Get Summary operation, and then using the provided dataset reference identifiers, to retrieve the corresponding S-125 datasets.

The first method is presented in Figure 19, where the service consumer contacts directly the Get operation of the service provider. By utilising the available operation parameters such as the geometry, area name, time-period and AtoN identifier, only specific S-125 datasets matching the specified criteria and/or have updates on the specified time-period interval, will be retrieved, packaged into an S-100 Exchange Set, and returned as a response.

Figure 19: Client-Initiated AtoN Information Retrieval through the Get operation.

In cases where the service consumer requires more information, before making a decision on which datasets should be downloaded, the latter “Client-initiated AtoN Information Retrieval” method can be utilised, by contacting the Get Summary operation beforehand. This is demonstrated in Figure 20, where the service consumer requests the list of the available S-125 dataset metadata via the summary operation and directly receives a response from the service provider. The service consumer can then select a dataset reference identifier from the returned list and request the detailed information (i.e. the complete S-125 dataset) back, through the Get operation of the service provider. The corresponding S-125 dataset (and/or its corresponding updates) will be packaged as an S-100 Exchange Set and then get sent back to the service consumer as direct response.

Figure 20: Client-Initiated AtoN Information Retrieval through the Get Summary operation.

As demonstrated in Figure 20, the Get Summary operation accepts similar operation parameters as the Get operation, which can be employed to filter the summary response list. This feature improves the flexibility of the whole operation as for example, even if a provided geometry operation parameter matches more than one S-125 datasets, only the required dataset will eventually be retrieved by the service consumer, by using its respective dataset reference identifier.

## Subscription-based AtoN Information Propagation

The subscription on AtoN Information is an alternative method of data propagation, in which a service consumer actively registers its interest to receive updates either on a specific set or alternatively, on all available S-125 datasets. This operation covers Use Case #3, as outlined in Section 3.4.4 of the service specification document.

Figure 21 illustrates the typical behaviour the subscription operation. Initially, the service consumer is required to initiate the subscription through the Subscription operation of the service provider. By utilising the available operation parameters such as the geometry, the area name and the subscription period, only the specific S-125 datasets matching the specified criteria and have updates during the specified subscription interval will be covered. Once the subscription request has been received, processed and accepted by the service provider, a notification should be sent back to the service consumer, using its Subscription Notification operation. This notification will indicate that the requested subscription has commenced and should include the identifier reference of the established subscription.

Whenever an update is made available on the set of S-125 datasets covered by a subscription, the service consumer Upload operation is used to deliver these updates, packaged as S-100 Exchange Sets, to the service consumer. Depending on the implementation of the service, the consumer may be required to respond to this operation by sending an acknowledgement message back to the service provider. The subscription updates will continue to be provided until one of the following conditions are met:

- The subscription period initially set by the service consumer has expired.

- The service consumer requests an end to the active subscription, using the Remove Subscription operation of the service provider.

- The service provider removes the information objects covered by the subscription.

- An expected incident/unexpected error forces the service provider to terminate the subscription.

Figure 21: Subscription requested by external service.

In the sequence diagram presented in Figure 21, the established subscription is terminated though a service consumer request. This should include the identifier reference of the subscription to be terminated, otherwise the service provider will terminate all active subscriptions for the service consumer performing the request. Once the service consumer’s request has been received, processed, and accepted by the service provider, another notification will be sent back to the service consumer’s Subscription Notification operation, to terminate the subscription process.

Note that the full contents of the S-125 datasets covered by a subscription, are not send back to the service consumer, unless a new dataset is created within the active period of an existing subscription that covers it. In all other cases, only the S-125 dataset changes will be transferred, in order to conserve bandwidth. Therefore, service consumer needs to utilise the Get (and Get Summary) operations, if the full contents of the S-125 datasets are required.

# References

| [1] | IALA, “G1128 The Specification of e-Navigation Technical Services,” 2021. |
| --- | --- |
| [2] | IEC, “63173-2 Maritime navigation and radiocommunication equipment and systems - Data interfaces - Part 2: Secure communication between ship and shore (SECOM),” IEC, 2022. |
| [3] | IALA, “Service Design Description for SECOM REST Template,” IALA, 2023. |
| [4] | IHO, “Universal Hydrographic Data Model,” May 2022. [Online]. Available: https://iho.int/uploads/user/pubs/Drafts/S-100%20Ed%205.0.0_Final.pdf. [Accessed 10 March 2023]. |
| [5] | IALA, “Technical Specification for the Provision of AtoN Information Service to End-Users,” IALA, 2023. |
| [6] | IALA, “GXXX - Guideline on the Provision of MCP Identities,” IALA, 2024. |
| [7] | IALA, “G1143 Unique Identifiers for Maritime Resources,” IALA, 2021. |

# Acronyms and Terminology

## Acronyms

| Term |  | Definition |  |
| --- | --- | --- | --- |
| API | Application Programming Interface |  |
| MRN | Maritime Resource Name |  |

## Terminology

| Term | Definition |
| --- | --- |
| AtoN | A logical entity that performs activities. Note: nodes are specified independently of any physical realisation. Examples of operational nodes in the maritime context are: Maritime Control Center, Maritime Authority, Ship, Port, Weather Information Provider, … |
| Maritime Safety Information (MSI) | Navigational and meteorological warnings, meteorological forecasts and other urgent safety-related messages. |
| Navigational Warning (NW) | A broadcast message containing urgent information relevant to safe navigation. |
| Navaid | An instrument, device or nautical publication carried on board a vessel for the purpose of assisting navigation. |
| NAVAREA | A geographical sea area, as shown in the appendix (IMO A.706(17) established for the purpose of co-ordinating the transmission of radio navigational warnings. Where appropriate, the term NAVAREA followed by an identifying roman numeral may be used as a short title. The delimitation of such areas is not related to and shall not prejudice the delimitation of any boundaries between States. |
| NAVTEX | Single frequency time-shared broadcast system with automated reception and message rejection/selection facilities. Use of NAVTEX is regulated by the IMO NAVTEX Manual (IMO publication 951). |
| SafetyNET Service | Dedicated international satellite broadcast system with automated reception and message rejection/selection facilities. |
| SafetyCast Service | Dedicated international mobile satellite service in the Global Maritime Distress and Safety System (GMDSS) for broadcasting navigational warnings, meteorological warnings and forecasts, and Search and Rescue (SAR) related information. |
| Service | The provision of something (a non-physical object), by one, for the use of one or more others, regulated by formal definitions and mutual agreements. Services involve interactions between providers and consumers, which may be performed in a digital form (data exchanges) or through voice communication or written processes and procedures. |
| Service Consumer | A service consumer uses service instances provided by service providers. All users within the maritime domain can be service customers, e.g., ships and their crew, authorities, VTS centres, organizations (e.g., meteorological), commercial service providers, etc. |
| Service Data Model | Formal description of one dedicated service at logical level. The service data model is part of the service specification. Is typically defined in UML and/or XSD. If an external data model exists (e.g., a standard data model), then the service data model shall refer to it: each data item of the service data model shall be mapped to a data item defined in the external data model. |
| Service Interface | The communication mechanism of the service, i.e., interaction mechanism between service provider and service consumer. A service interface is characterised by a message exchange pattern and consists of service operations that are either allocated to the provider or the consumer of the service. |
| Service Operation | Functions or procedure which enables programmatic communication with a service via a service interface. |
| Service Physical Data Model | Describes the realisation of a dedicated service data model in a dedicated technology. This includes a detailed description of the data S-212 to be exchanged using the chosen technology. The actual format of the service physical data model depends on the chosen technology. Examples may be WSDL and XSD files (e.g., for SOAP services) or swagger (Open API) specifications (e.g., for REST services). If an external data model exists (e.g., a standard data model), then the service physical data model shall refer to it: each data item of the service physical data model shall be mapped to a data item defined in the external data model. In order to prove correct implementation of the service specification, there shall exist a mapping between the service physical data model and the service data model. This means, each data item used in the service physical data model shall be mapped to a corresponding data item of the service data model. (In case of existing mappings to a common external (standard) data model from both the service data model and the service physical data model, such a mapping is implicitly given.) |
| Service Provider | A service provider provides instances of services according to a service specification and service instance description. All users within the maritime domain can be service providers, e.g., authorities, VTS centres, organizations (e.g., meteorological), commercial service providers, etc. |

Service implementation XML template

Service instance OpenAPI definition template