Input paper:	     ENG2-5.5

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	X  ENG		□  PAP					□   Input

□  ENAV	□  VTS							X   Information

Agenda item			5.5

Technical Domain / Task Number	TD#2

Author(s) / Submitter(s)			Dr. Jong-Uk Kim /Korea Association of Aids to Navigation

# Developments in AtoN Simulation Systems

# - Monitoring development of simulation techniques

# for AtoN simulation in Korea(2) -

The objectives of Aids to navigation(AtoN) simulator are to develop the design and construction technology of AtoN simulator system with full mission bridge. AtoN simulator system is developed to assist the decision making for AtoN design and AtoN placement plan in consideration of the effects of topographical, environmental and maritime traffic characteristics of target navigation area.

Aids to navigation(AtoN) is a important facility to give a navigation information to a ship. The previous distribution planning of AtoN was carried out by using a expert's experience and a marine map. Recently the size of ship, the vessel traffic and the complexity of harbour area are increasing. So the need for a scientific design technique of distribution planning of AtoN is increasing. The AtoN simulator provides the simulation circumstance which includes the topographical and environmental characteristics of a primary harbour and the characteristics of a navigating ship and maritime traffic. The expert of AtoN distribution planning can design a safer and more efficient distribution planning of AtoN using the simulator. To get the goal the AtoN simulator must have further function than a ship handling simulator.

The main operating program and data for all sub-systems developed by KRISO(Korea Research Institute of Ships & Ocean Engineering) and KAAN(Korea Association of Aids to Navigation). Especially, AtoN Manager SW for operating to AtoN simulation was developed including AtoN database management(Properties, Placement Scenario), AtoN placement(real-time control of properties and placement), AtoN database print and import/export  and quantification of AtoN visibility .

Fig. 1. Configuration of operation SW of AtoN simulator system in Korea

AtoN is artificial facilities to help to sail safely by using light, shape, color, sound and radio. Especially, because AtoN provides a lot of visual information, AtoN simulator must be developed to reflect this point. AtoN Simulator has to visualize the maritime environment and AtoN realistically. This system was performed the design and implementation of a visualization system that is a sub-system of the AtoN simulator. This system reflects a variety of visualization factors related to AtoN. Also the AtoN experts can make decision and evaluate visual effects of the AtoN.

This system is necessary to visualize the picture was carried through the 3D modeling of the AtoN such as Manned Lighthouse, Unmanned Lighthouse, Beacon, Leading Light, Projector Light, Direction Light, Light Staff, Light Buoy, Traffic Signal Light, Unlighted Beacon and Bridge Daymark etc. The Visual System provides daylight, dusk and night scenes in true perspective and true colour. It shows terrain features, waterways, AtoN such as lighthouses, navigation marks and buoys etc.

AtoN Manager SW was developed as a virtual/real AtoN management for AtoN simulation.

The AtoN data are the difficulties in managing the conversion to electronic data because it is complicated large data including various properties of AtoN systems. Also, operation of  AtoN simulation system is needed for effective database management of AtoN properties. Therefore this system was designed database structural of AtoN properties based on the analysis of AtoN properties for Implementation of  visual  simulation.

Fig. 2 Picture of operation of AtoN Manager for AtoN simulation

IALA recommends the required illuminance which is needed to identify the aids to navigation. It does not mean the quantities for navigator to be able to figure out the existence of the AtoN, but the quantities for one to able to figure out the instruction of AtoN. Visibility of AtoN assumes that if the illuminance acquired on the eyes of the navigator is bigger than the required illuminance, the navigator can identify what the AtoN informs one of. The required illuminance for visibility is given by 2x10-7 lux for night and given by 1x10-3 lux for day. One for night is based on the assumption of no background light. One for day is based on the assumption of meteorological conditions with bright cloud or clear sky close to the direction of the sun.

The required illuminance for night needs to be compensated by containing the effect of the background light since the visual recognition is carried out by getting the light contrast between the interested light source and the background light. The required illuminance has to be determined according to the status of the background light. The state of the background light is mainly changed with the existence of the background light, the distributed area, and the intensity of the light source.

While the required illuminance is given by 2x10-7 lux for no background light, the required illuminances are recommended by 2x10-6 lux for minor background light and by 2x10-5 lux for substantial background light, respectively. Let   denote the required illumination and   denote the ratio of the background light. It is defined to employ the effect of the background light and is in the range from 0 to 1. It is defined that no background light corresponds to ; minor and substantial background lights correspond to   and  , respectively. Based on the relationship which Figure 1 shows and using a liner interpolation method, the logarithm of the required illuminance is formulated by employing the ratio of background light as follows.

(1)

Fig. 3 Relationship between the required illuminance and the background light

The performance measures of visibility is introduced in order to quantify how well the navigator recognizes the instruction of AtoN light. In order to evaluate the performance measures, the simulation calculating the performance measures is carried out using the AtoN placement. Simulations should be carried out based on the AtoN database to be reliable. For reasonable simulations, AtoNs are selected from the pre-installed AtoNs at Busan port in South Korea. Since it has been installed for a long time and has guided many navigators, the placement can be considered to have been well adjusted. For simplification, the background effect disturbed by facilities of land is ignored.

Fig. 4 Selected AtoN Placement at Busan Port

Fig. 5 shows the example of AtoN placement selected from Busan port. AtoNs to be investigated for visibility appear to be circled by dotted line and 12 numbers. Ship’s virtual path along with the known waterway is established as the arrow line of the carter-corner whose heading angle is -55.29°. The virtual cruise starts in the bottom right-hand corner where is at latitude of 35.0785285°(north) and longitude of 129.11338°(east) and ends at latitude of 35.105793°(north) and longitude of 129.065496°(east). For calculating the amount of recognition, the specified waterway is divided into the small waterway which has the length of 10 meter and whose number is about 550. The details information of performance measures based on visibility for effective placement of AtoN will submit to the next meeting.

Fig. 5 Performance measures of AtoN Placement

Future Tasks

In future tasks, it is suggested that the AtoN simulator would be an item of an AtoN Accountability toolbox for work programme(2014-2018) and the Ministry of Ocean and Fisheries(MOF) of Korea/Korea Association of Aids to Navigation(Jong-Uk Kim) will provide information about simulation techniques(3) for AtoN simulation, the possible availability of the simulator to the IALA WWA and detail information of performance measures based on visibility for effective placement of AtoN to the next meeting(ENG-3). Also we will propose new guideline on the operation techniques for AtoN simulation.