Input paper:	     ENG2-9.9

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	X  ENG		□  PAP					□   Input

□  ENAV	□  VTS							X   Information

Agenda item			9

Technical Domain / Task Number	TD#1

Author(s) / Submitter(s)			Mr. Yong-Su Yu/Korea Association of Aids to Navigation

Development of LED Large Beacon in Korea

Aids to Navigation(AtoN) is important facility to give a navigation information to a ship. The previous distribution planning of AtoN was carried out by using a expert's experience and a marine map. AtoN beacon is important facility to improve safety and Sailing efficiency of Ship during day and night. Metal halide lamp, as the main light source of the beacon, is applied high voltage and has short lifetime due to high lamp voltage. The necessity  for AtoN technology has through the application of modern light sources.

The LED light source by replacing the pre-existing old group was clear and reducing the power and maximize the available light. This is a system that can support the effectiveness of the safe navigation and operation of the vessel. A metal halide lamp used as a light source, such as a large conventional rotating beacon is take typically 5 to 10 minutes until lighting and restart time is required 8 to 15 minutes. In case of situation of severe cold, it takes up to 20 minutes until normal lighting. As above disadvantage of pre-heating and cooling time required by external environment is difficult to quick lighting and turn off. To compensate for this, there is a case of using separate high temperature re-lighting unit. There is fatal disadvantage that the life is shortened according of high lamp voltage.

We applied LED modules and a large reflecting mirror and developed the LED large beacon with Variable luminous intensity from 20M to 27M (T=0.74) including the light output and flashing control system.

Korea has developed a rotating LED large beacon by Korea Photonics Technology Institute (KOPTI) and Korea Association of Aids to Navigation (KAAN).

Large LED beacon was applied to light optic technology, optimized heat dissipation and compact mechanical design technologies.

The vertical divergence angle is proposed of ±2 degrees and the horizontal divergence angle ±4.5 degrees (FWHM).  Optical system of beacon is configured to 2 converging lens, reflector, and compound optical system of the Fresnel lens.

For efficient LED heat dissipation, Non-directional rapidly inverse spread devices and using a reflector rotational force convection LED heat dissipation is developed. As a structure capable of cooling the heat sink of the outside air flows into the housing by the rotating force, it is possible to minimize of weight and volume the heat sink like a forced convection system.

2nd optical lens

1st optical lens

LED modules

Heat pipes

BLDC motor

Fig.1 Scheme of rotating LED large beacon

Future Tasks

In future tasks, we will provide information detail properties of rotating LED large beacon.