To

IALA VTS Committee

From

Marten Koopmans

Date - 8 January 2004

Telephone - +31 70 351 1562

Enclosure(s) - 1

Subject

VTS Manual - Chapter 2

Dear Colleagues,

Please find attached a draft version of a revised Chapter 2 of the VTS Manual for consideration at the next meeting of the IALA VTS Committee in February 2004.

As you may recall an inter-sessional group was set up at the last meeting to be chaired by me to review this chapter. Through an exchange of views and text proposals by email the attached new draft version was produced. I would like to take this opportunity to thank my colleagues in this group for their valuable contributions.

This chapter should be read in conjunction with the IALA Recommendation on the Implementation of VTS (V-119) as this chapter is intended to offer an elaboration and should not deviate from this Recommendation.

In general no major rewrite has considered necessary as this chapter was relatively recently written when Recommendation V-119 was developed. Apart from clearing up the structure and text slightly only some additional remarks on security, shore based pilotage and AIS have been introduced.

Parallel to the review of the content of this chapter an overall review of the general structure of the VTS Manual is being undertaken by others in the VTS Committee. This might result in a relocation of (parts of) this chapter when a new structure for the VTS Manual as a whole has been decided upon.

Definitely paragraph 2.5 on VTS in inland waters will need to be relocated in the VTS Manual as it did not fit well anymore in the revised chapter 2, but is worthwhile retaining in the VTS Manual in another chapter.

Yours sincerely,

# Marten Koopmans

# CHAPTER 2 - PLANNING A VTS

## 2.1	IMPLEMENTATION ASPECTS

2.1.1.	General

The purpose of a VTS is to improve the maritime safety and efficiency of navigation, safety of life at sea and the protection of the marine environment and/or the adjacent shore area, work sites and offshore installations from possible adverse effects of marine traffic in a given area. VTS can also contribute to maritime security and other objectives. Implementation of a new VTS or the re-assessment of an existing VTS may be a practical solution, if there is concern about the levels of safety and if, as a result of reviewing the existing safety measures, these have been found to not adequately meet the requirements.

When considering the implementation of a VTS, the Government or the Competent Authority should take full account of IMO Assembly Resolution A.857(20) on Guidelines for Vessel Traffic Services (see Annex 1) and, in particular:

1	ensure that legislation is in place for the establishment and operation of a VTS, in accordance with national or international law;

2	ensure that a mission statement and objectives for the VTS are set;

3	ensure that a VTS Authority is appointed and legally empowered;

4	ensure that the geographic service area is delineated and declared a VTS area; including, where appropriate, defining VTS sub areas and sectors;

5	determine the services to be provided by the VTS with regard to the objectives of the VTS;

6	establish appropriate standards for shore and offshore based equipment

7	ensure that the VTS authority is provided with the equipment and facilities necessary to effectively accomplish the objectives of the VTS;

8	ensure that the VTS Authority is sufficiently staffed with appropriately qualified, suitably trained personnel capable of performing the tasks required;

- instruct the VTS Authority to operate the VTS in accordance with relevant IMO resolutions.

A VTS is a considerable investment. The local needs and requirements should form a starting point in order to establish whether the investment required is justified. If implementation of a new VTS or enhancement of an existing VTS is justified considerable insight is required to be able to judge the scope of activities and capabilities of this VTS.

Whilst the traditional approach to safety in the maritime industry has been to react to problems and issues as they occur, informal risk assessment techniques to support the relevant authorities have been in existence for many years. IALA has developed Guidelines on Risk Management Guidelines, to provide its members with a broad outline on the use of such risk assessment and risk management techniques. These Guidelines break down the Risk Management process into five clearly identifiable steps: Risk/ Hazard Identification, Risk Assessment, Risk Control Options, Decision Making and  Action.

Before considering the establishment of a new VTS or the enhancement of an existing VTS, the Competent Authority and/or future VTS  Authority should launch a Feasibility Study, which should contain the following six phases, in which these Risk Management process steps are addressed:

Inception phase

Feasibility and Design Study phase

Risk Assessment phase

Cost/benefit phase

Implementation phase

Evaluation phase

The activities to be undertaken in each phase are shown in Figure 2

In addition the Competent Authority will need to decide whether the VTS Authority has sufficient technical competence in house to undertake the detailed planning of the intended service.  Where the VTS Authority does not have such competence immediately available use should be made of external expertise.  When employing external expertise care should be taken to ensure that the persons/companies involved are independent and impartial. It should also be noted that the expertise to describe the system is different from the expertise required to supervise the implementation and the quality requirements of the system.

Broadly the expertise required can be considered as falling into the following three categories:

- operational;

- technical;

- legal.

INCEPTION

Problems

Constraints

Area

Traffic Management

Tools

Evolve options

Weightings

Importance

Risk

Organisation

Framework

Structure

Legal

Security

Funding

Operational objectives

Areas

Sub Area

Sub Area

Delineate areas

Measures to be taken

Determine level of service

FEASIBILITY AND

Functional Requirements

Functional Specification

Traffic Management Measures

Finalise Inception Proposals

Outline Procedures

Manning Training

Cost/Benefit analysis (preliminary)

…DESIGN STUDY

Geotechnical Data • Project Plan

Resource Allocation • Spend Profile

Timescale • Design Review/Audit

Equipment Technical Specification

Civil

Mechanical

Electrical

IT Hardware/Software

Training

RISK ASSESSMENT

Navigational Risks:

• Environmental Risks

FSA Techniques

Estimation

Evaluation

Alternative mitigation measures

Estimated reduction

COST/BENEFIT

Analysis-Investment v risks reduction

Direct benefits

Indirect benefits

Prospects

Value Added Services

IMPLEMENTATION

Implement other traffic management measures

Preparation • Tender procedure

Purchasing           • Building

Legislation/organisation/personnel   • Trial operations

EVALUATION

Technical Performance

Operational Performance

Operational Objectives

Problems alleviated

Not acceptable               Acceptable

Figure 2  Main activities of the various phases in the development and procurement of a VTS

### 2.1.2	Inception phase

The Inception Phase should answer the question of whether active traffic management is the appropriate means to address the local traffic problems. Active traffic management should only be used in those areas where other means are inadequate to provide the desired level of safety and protection of the environment. This section gives guidance on how the Inception Phase should be conducted. The reports of the Inception and Feasibility Study Phases together are sometimes referred to as the Baseline document.

In the Inception Phase, all relevant problems in the maritime area concerned should be defined and analysed. Further, as a second step in the process, operational objectives should be established with the ultimate aim of alleviating the defined problems. The last step in this phase is to identify the most appropriate traffic management tools, in terms of effectiveness and costs, to alleviate the defined problems. Implementing the rather expensive VTS tool may be one of these solutions.

The possible local traffic problems could be related to:

- The interaction of maritime traffic,

- Volume and mix of traffic;

- The local conditions like geography, hydro/meteo, tides and weather;

- Protection of the marine environment; and

- Protection of the surrounding area (safety and security).

In addition, future developments in the port infrastructure and the resulting changes in traffic volumes and composition, including dangerous cargoes and any other relevant future development in the area concerned should be considered in this phase.

In the specific case of a Coastal VTS, future trends in traffic volume and other activities in the coastal area, such as fishing, recreation and offshore activities should be taken into account.

Further developments in VTS technology and SOLAS requirements for navigational and communication equipment on board vessels should be considered as well.

Furthermore in the case of a decision to establish a VTS, the following aspects need to be addressed;

- Organisational framework of the national and local maritime authorities in relation to implementing new traffic management solutions, VTS in particular; and

- The present regulatory or legislative framework, including local by-laws, rules and recommendations. Special attention has to be devoted to ascertain any requirement for adjusting the framework to allow for effective implementation of a VTS.

Attention is drawn to IALA Recommendation V-119 on the Implementation of Vessel Traffic Services, which provides a comprehensive list of all the relevant items to be addressed.

### 2.1.3 Feasibility and Design Study Phase

- If the previous Inception Phase has indicated that just passive measures are inadequate to attain the desired level of safety and efficiency of the maritime traffic, in the area under consideration, the effect of establishing a VTS should be tested.

- The foundation for the work in the Feasibility and Design Study Phase is the approved report on the Inception Phase and the expected functions and benefits of a future VTS. This input may also give an indication of the desired level of service to be provided by the VTS.

The overall objective is to produce a VTS that meets exactly the set goals and resulting requirements of the VTS Authority, and against which the design can be validated fully, verified and tested before development.  There are normally two levels or sub-phases in defining the requirements of a VTS, these are:

- the development of an overview of the system and an intermediate level of detail about the system’s function, operation and environment, referred to in this Chapter as the feasibility sub-phase; and

- the development of the detailed technical specification of requirements, referred to in this Chapter as the technical specification sub-phase.

One of the main tasks of this Feasibility sub-phase is to establish the functional requirements for the VTS. This Phase is of critical importance to the establishment of a VTS optimised for the area that it will cover and it is essential that the correct balance of input by the VTS Authority, users, local services and allied services is achieved. Examples of some aspects that need to be taken into account are listed below.

#### 2.1.3.1   Operational aspects

  - Local Geography.   The local geography will be the determining influence on the size of the area to be covered by a VTS.  In the case of ports these vary enormously in their geography.  Some ports, are extremely simple and are little more than an indentation in the coast protected by breakwaters.  Entry and exit is through a passage between the breakwater heads, which give direct access to the open sea.  Vessels are only restricted in their freedom to manoeuvre as they pass through the breakwater and into the port itself.  At the other extreme are estuarial ports, often far from the open sea with long approaches encumbered by shallow, shifting sandbanks.  Vessels using these ports will be restricted navigationally and possibly be unable to anchor or reverse course over long stretches of their passage.

  - Local Conditions:  The prevailing weather, in particular visibility and wind together with the tidal range and stream, may impose difficulties on the ability to navigate safely.  Together with the local geography they determine the degree of navigational difficulty likely to be encountered by a vessel.

An appreciation of these physical factors, plus any interface with local or regional services, leads to a first assessment of services which would be of most benefit to vessels navigating in the area.

  - Numbers of vessels and types:  The numbers of vessels, including local traffic, and their type is significant.  A simple count of vessels, although of value, is not sufficient.  The vessels need to be considered with regard to their size, type, equipment, manoeuvrability, spatial distribution and (hazardous) cargo so that the optimum service meeting the needs of all users and without placing unnecessary constraints on the movement of any of the vessels can be identified. In combination with the local geography traffic densities in time should be determined.

  - Commercial factors:  Any VTS must make due allowance for any potential conflict between safety and commercial operation and pre-empt such conflicts before they arise.  Ports must operate in an efficient and timely manner and meet the needs of their users, but this must be done without impinging on the safe operation of the port.  The distribution of ship arrivals and departures may be an important factor influencing the port management resources. Unannounced arrivals and departures can have a considerable and adverse effect on the viability of a port.  Some ports, such as ferry ports and container terminals, operate to a schedule which has to be maintained in virtually all weathers.

  - Other Activities: Naval operations, oil and gas exploration and production, fishing, research, recreational activities, wind and fish farming and other activities may take place within the area to be covered by a Vessel Traffic Service.  These activities will also influence the operation of the scheme and must be taken into account.  A good working relationship needs to be established and maintained with other users of the area.

#### 2.1.3.2  Environmental Aspects

Many incidents in recent years have highlighted the hazard to the marine environment presented by large tankers and other vessels carrying dangerous substances and has drawn political attention to VTS, ship reporting systems and vessel identification.  As a consequence of these incidents protecting the environment from marine casualties has attracted significant public attention from outside the marine industry.  It may be that pressures to protect the environment will continue to grow, causing a change in the balance between efficiency, safety, security and the environment.  Annex 3 contains further guidance on the role VTS can play in the protection of the environment.

There are areas where the consequences of an incident would be such that extra safety provision, over that normally applied, may be appropriate at an early stage.  These areas must be identified so that the VTS can accommodate them.  In this context, suitable measures may include routeing vessels clear of environmentally sensitive areas and the careful organising of traffic flow, so that vessel numbers in particularly sensitive areas are limited.

A VTS will also need to take account of, and probably operate in conjunction with, mandatory ship reporting systems which have been adopted by IMO and are completely or partly within the VTS area.

#### 2.1.3.3    User requirements

In any consideration of VTS the views of the various users must be included.  In the past a VTS was designed primarily to meet the immediate needs of vessel operators.  As VTS grew and became recognised as a valuable information gathering and processing resource, the list of users of Vessel Traffic Services expanded to include many in the shore-side marine industry.

While the benefit to the shipping community is tangible and immediate, the public benefits as well.  The VTS Authority must consider the collective, and at times conflicting, needs of these entities when designing and operating a VTS.  The following list, by no means exhaustive, identifies a number of potential users and participating bodies.

###### Maritime Users

Commercial vessels of all types 	       	Support vessels (buoy tenders, tugs, dredgers)

Fishing vessels			       	Recreational vessels

Military or other public vessels	       	Regatta or racing vessels

Oil and gas exploration and production	Wind and fish farming

Aircraft (amphibious, seaplanes and helicopters)

###### Allied Services, Port Operations & Adjacent VTSs

Pilotage Service, Tugs and linesmen	Other VTS		Terminal Operators

###### Emergency Services

Search and Rescue	Fire Service	Pollution Control	Security Agencies

Other agencies which could be involved

Terminal Operators			Foreign Governments (Flag State)

Port State Control			State or Municipal Governments

Shipping Agents				Customs

Immigration				Shipyards

Industry or Trade Organisations		Environmental Organisations

Persons living along the waterway	International organisations

Media					Quarantine services

Tourism

The feasibility and design study phase is also intended to provide a VTS Authority with a framework to proceed further with development against carefully established guidelines of requirement, cost, risk and time.  It should comprise some or all of the following:

- Role of VTS in meeting the various overall objectives (safety, security, efficiency, environmental protection and others) based on Cost-Benefit analysis and the identification of any trade-offs

- Description of the constraints and context in which the VTS will operate;

- Evaluation of the technology available and determination of  the standards to be used;

- Evaluation of the human resources needed for operation of the system and consideration of manning levels, training and skills required;

- Evaluation of the health and safety facilities needed to safeguard staff and other persons associated with the VTS system;

- Preparation of a management plan for the entire development;

- Assessment of the method, or methods, to be used for Quality Assurance;

- Assessment of the probability that the VTS system will be developed, installed, tested and ready for operational use within both the required time scale and the available financial resources;

- Development and evaluation of system design options.

This feasibility sub-phase should identify the range of activity that will need to be examined during the technical specification sub-phase, show the feasibility of any actions suggested and eliminate high-risk elements.

On satisfactory completion of the feasibility sub-phase the VTS Authority will be in possession of a highly detailed basis for proceeding to the technical specification sub-phase with confidence that its outcome will provide a viable solution for developing the system.

Attention is also drawn to IALA Recommendation V-119 on the Implementation of Vessel Traffic Services, which provides a comprehensive list of all the relevant functional requirements to be addressed within the feasibility sub-phase

The Technical specification sub-phase should produce the definitive statement of how the system, including equipment and buildings, is to be constructed, and how sub-systems and components should interact with each other to produce a viable VTS.  It may be necessary to investigate thoroughly any proposed solutions in areas of technical difficulty or high risk to ensure that they are likely to be effective.

This sub-phase should produce a comprehensive design for the overall VTS, including the performance criteria and technical specifications for the purchase of equipment and the definition of the interface standards that need to be met. The technical specifications should be prepared to such a level that they can be used in the procurement documentation and should include clearly defined acceptance and testing programmes.

A detailed project plan for the design, manufacture/construction, installation and commission stages should be prepared and any tasks or items having a long lead time or which may be difficult to obtaining should be identified.

### 2.1.4	Risk assessment phase

The Risk Assessment Phase is intended to confirm that the measures being designed and introduced will reduce the risk of dangerous situations, incidents and accidents in the area to a level considered by the Competent Authority to be satisfactory.

The risk level should be calculated by taking into account:

- The vessels (e.g. type, size, speed, manoeuvrability, (hazardous) cargoes/ passengers), including local craft;

- The area (e.g. geography, local conditions, other activities, population, environmental sensitivity, infrastructure, economic importance);

- The facilities (e.g. aids to navigation, traffic routeing schemes, pilots, tugs, emergency services) in the area.

The total risk equation comprises the Probability, or frequency, of an incident occurring, the Consequences of an incident and the Governmental or public Acceptability of such an incident.  The risk assessment should identify and quantify each of these aspects.

IALA Guidelines on Risk Management (June 2001) outline a general risk assessment and risk management methodology for Marine Aids to Navigation including Vessel Traffic Services (VTS) so that all types of risks can be effectively managed.  The Guidelines may be used when assessing the optimum mix of aids to navigation, including VTS, for mitigating risk.

### 2.1.5	 Cost/ benefit phase

After completion of the Design and Risk Assessment Phases, a cost benefit analysis could be carried out to determine whether the expected reduction in risk and other benefits justify the investment, operational costs and efforts required.

Both the additional direct and indirect benefits and prospects that a VTS might offer, including additional value added services for the traffic in the future as well as the benefits to shore based port operations, should be taken into consideration.

A direct benefit that could be taken into consideration, amongst others, is the probable reduction in other waterway infrastructure costs that may arise from implementation of the changes, such as replacing labour intensive processes using traditional equipment with more modern equipment and automated processes.

Indirect benefits should include an estimation of costs that would otherwise have been incurred in the event of an incident/accident, based on the projected difference between the frequency of occurence of such incidents/accidents before and after implementation of any changes.

Further guidance on cost/benefit analysis is given in Annex 2 of this Manual and in IMO MSC/Circ.829 & MEPC/Circ.335 of 17 November 1997 on “Interim Guidelines for the Application of Formal Safety Assessment (FSA) to the IMO Rule-making Process”.

### 2.1.6 	Implementation phase

The main tasks during the Implementation phase will be associated with the:

- preparation and publication of tenders to carry out the construction and installation of the equipment and any construction or alterations to buildings;

- evaluation of tenders and awarding of contracts;

- monitoring of progress of the contracted work; and,

- verification or conduct of tests on individual items of equipment of sub-systems

- consulting and informing relevant parties (e.g. allied services, public)

While it can be expected that individual items of equipment will have been tested before installation, during this phase the system should be tested as an entity.  This should include the integration of VTS Personnel and therefore implies that the recruitment and training programme should be conducted before final acceptance of the system is undertaken.

All documentation necessary for the proper operation of the VTS, including operational rules, procedures and contingency plans, should be developed during this phase. In addition, all national and international administrative and legal matters concerning the establishment and operation of the VTS should be resolved, including the availability of radio communication frequencies and, if necessary, the approval by IMO for any traffic routeing schemes or ship reporting systems.

This phase will be complete when the VTS has satisfactorily finished all necessary testing and trials and has full operational capability.

### 2.1.7 	Evaluation phase

The Evaluation phase is intended to ascertain the effectiveness of the VTS in meeting its objectives and design targets, in particular with respect to mitigating the risks of collisions or groundings in the area concerned.	Wherever practicable, the evaluation should take place after experience has been gained by the VTS personnel in the use of the equipment and practical operation of the service.

An overall evaluation of the VTS should be preceded by an assessment of the effectiveness of:

- the technical equipment and sub-systems integrated together to form the VTS technical system; and,

- the operational use of the VTS technical system as a traffic management tool in the area concerned.

If this phase indicates that the VTS is less effective than its design target, the reasons for this should be investigated and if necessary modifications made to the system to improve its operational effectiveness.

It is advisable to use independent consultant with experience in risk management techniques for the assessment of risk reduction potential prior to implementation of a VTS and for risk reduction achievements after implementation of the VTS.

## 2.2	OPERATINAL ASPECTS

### 2.2.1	Service options

Before establishing a VTS, and taking into account the geographical area and the traffic density and pattern, the Competent Authority will need to consider whether an information service, navigational assistance service or traffic organisation service is to be provided.

In practice these services and those concerning co-operation with allied services cannot be considered in isolation.  Although an information service may be independent, a navigation assistance service will include an information service and a traffic organisation service will normally include both information and navigation assistance services.  However depending upon the type of VTS the emphasis on the type of service provided may differ.

As one of the allied services Shore Based Pilotage (SBP) or a similar service can be – and in some cases – is given from the VTS. Special consideration must then be given to the interrelation between this service and the regular services of the VTS. Generally only a part of the vessels in the VTS-area receiving VTS will also SBP. As those vessels will be receiving both services from the same VTS, possibly from different operators, at the same time due care needs to be given to avoiding confusion and information overload of the mariner.

### Information Service

An information service is a service to ensure that essential information is available in good time to assist the shipboard navigational decision making process.  This information may concern:

- the positions, intentions and destinations of vessels;

- amendments and changes in promulgated information concerning the VTS area, such as boundaries, procedures, radio channels or frequencies, reporting points etc.

- the variables influencing the navigation of vessels, such as meteorological and hydrological conditions, notices to mariners, status of aids to navigation, traffic congestion and special vessels with limited manoeuvrability which may impose restrictions on the navigation of other vessels or any other potential hindrances.

As a new element, with the introduction of AIS, the VTS can and should also inform the vessel of omissions and errors in the AIS messages which are broadcast by the vessel whenever such inconsistencies can be and are observed by the VTS.

### Navigational Assistance Service

A navigational assistance service is a service to assist the navigational decision making process on board and to monitor the effects, especially in difficult navigational or meteorological circumstances or in case of defects or deficiencies.

A VTS can contribute to the decision making process by giving navigational information such as:

- course and speed made good by a vessel;

- position relative to fairway axis and way-points;

- positions, identities and intentions of surrounding traffic;

- warnings to individual vessels

A VTS can participate in the decision making process by giving navigational advice, but must refrain from giving advice on desired course and speed as part of the navigational assistance.

The Competent Authority should be aware of the distinction between navigational information and navigational advice and should determine whether navigational advice can and may be given from the shore and if so under what circumstances and to what extent.

Navigational assistance is given at the request of the vessel or if deemed necessary by the VTS and should only be given if positive identification has been established and can be maintained throughout the process. The beginning and the end of navigational assistance should be clearly stated by the vessel or the VTS and acknowledged by the other party.

### Traffic Organisation Service

A Traffic Organisation Service is a service to prevent the development of dangerous situations and to provide for the safe and efficient movement of traffic within the VTS area.  Traffic organisation concerns the forward planning of movements and is particularly relevant in times of congestion or when the movement of special transports may affect the flow of other traffic.  Monitoring the traffic and enforcing adherence to governing rules and regulations are integral parts of traffic organisation.

The service may include establishing and operating a system of traffic clearances in respect of the priority of movements, the allocation of space, the mandatory reporting of movements, establishing routes to be followed, speed limits to be observed and such other measures as may be considered necessary and appropriate by the VTS.

Where the VTS is authorised to issue instructions to vessels, the instructions should be “result orientated” only and leave the details of the execution to the vessel.

VTS sailing plans are an instrument of traffic organisation and are a major source of information to the VTS.  A VTS sailing plan normally includes the estimated time of arrival in the VTS area or the departure from a berth or an anchorage in the VTS area.  The VTS Authority may consider making a procedural distinction between sailing plans established for efficiency reasons, for the co-ordination with allied services, and VTS sailing plans established for safety reasons, which may affect the general flow of traffic.  Authorities should state the vessels or categories of vessels for which a VTS sailing plan is compulsory.  VTS sailing plans should be adhered to as far as practicable and where the execution of a VTS sailing plan could affect general traffic movements the VTS Centre should advise other shipping of the relevant details of the plan.

The VTS Authority should specify the information required in the VTS sailing plans for all vessels or for special vessels according to the local circumstances. In exceptional circumstances the VTS sailing plan may be amplified at the request of the VTS.  The VTS may advise changes to the VTS sailing plan to take account of the traffic situation or special circumstances.  After the VTS sailing plan has been agreed between the vessel and the VTS, the vessel is permitted to participate in the VTS and should as far as practicable try to maintain the plan.  If special circumstances, or the safety of the traffic so require, the VTS may request the vessel to follow an amended VTS sailing plan, indicating the reasons for its request.  In areas where automatic tracking facilities such as shore based AIS are not available, regular position reporting by radiotelephony may be required from the vessel.

### 2.2.2	Operating Rules and Regulations

The authority establishing a VTS should delineate its area of coverage, declare it to be a VTS area and disseminate to mariners full details concerning the area of operation, including the limits of the areas where participation of vessels is required or recommended, the services provided and the procedures.

A VTS area can be divided into sub-areas or sectors but these should be as few as possible.  Area, sub-area and sector boundaries should not be located where vessels normally alter course or manoeuvre or where they are approaching convergence areas, route junctions or where there is crossing traffic.

In establishing the operating framework, great care must be taken to impose the minimum of rules or regulations so that the mariner’s workload is not substantially increased when passing through a VTS area.

Effective and realistic regulations applying to the movement of ships within the VTS area, should be introduced.  These regulations should identify the VTS area, describe the sub-areas and sectors, if any, and the communication frequencies, procedures and times of scheduled routine broadcasts to shipping.

Apart from these regulations pertaining VTS there should be regulations defining the other navigational requirements such as traffic routes, speed limits, movement restriction areas, anchorage areas, and setting out any special requirements such as the compulsory escort of certain vessels carrying hazardous cargoes to and from their berths. Matters which need to be taken into consideration when preparing the regulations include all of the following which are within the VTS area:

Aids to Navigation	Anchorage Areas

Areas to be avoided	Compulsory Pilotage and Pilot Boarding Areas

Constrained Vessels	Movement Restrictions

One-way Navigation Channels	Patrol Craft

Precautionary Zones	Segregation of Small Craft

Ships’ routeing systems adopted by IMO	Ship reporting systems adopted by IMO

Speed Limits	Traffic Separation Schemes

Language	AIS

Security	Places of refuge

Shore based pilotage	Other regional and local needs and requirements

The regulations should also include details of all reporting points. The number of reporting points should be kept to a minimum and be separated as widely as possible.  They should be clearly identified, for example by number, name and geographical position or description.

A VTS authority should ensure that the local traffic movement rules and regulations in force, the services offered and details of the area concerned are promulgated in a form convenient for use by mariners.

The information may be published as a local publication or as an international publication, or in both forms.

A local publication has the advantages that detailed information can be presented, and that the document can be quickly amended when required. Other information that is relevant to the VTS area can also be included. The VTS Authority should ensure that the information is written and presented in a format that is easy to read and clearly describes the procedures and regulatory requirements a ship must follow. In this respect where possible, chartlets should be included showing area, sub-area and sector boundaries, general navigation information about the area, together with procedures, radio frequencies or channels, reporting lines and reporting points.  Where a VTS area extends beyond territorial waters, the limit of the territorial waters should be clearly indicated on the chartlets.

An international publication has the advantage of presenting a standard format for all zones, thus enhancing the ability of users to interpret the information and requirements correctly.  In addition, users would be able to obtaining information on many vessel traffic services, including relevant radio and ship movement reporting requirements from one source document.

In addition to promulgating details of the VTS through local and international publications, the information should be forwarded to IALA Headquarters at the earliest possible time for inclusion in the IALA, IAPH, IMPA World VTS Guide.  IALA Headquarters will, on request, advise VTS Authorities on the most suitable format to be used for information intended to be included in the World VTS Guide.  An example of an entry in the World VTS Guide is attached at Annex 4.

### 2.2.3	Co-operation with Allied Services

Allowing or even encouraging involvement of VTS in allied services, such as port operations, emergency services, security agencies and adjacent VTS, is a supporting activity of the VTS, usually intended to increase the safety and efficiency of the traffic, the protection of the environment and the effectiveness of the VTS and to reduce the reporting burden of the vessel.

Co-operation with allied services is both safety and efficiency orientated.  It should be a continuous process and is of particular importance in cases where a VTS sailing plan is to be established and action agreement between services is required.  Procedures for the co-operation between parties should be established. The co-operation with port operations is primarily efficiency orientated but may be an important factor in establishing a VTS sailing plan.

Incidental co-operation with security agencies and emergency services such as Search and Rescue and Pollution Control should be conducted in accordance with pre-established contingency plans in which the procedures for the co-operations are laid down and responsibilities established.

Co-operation between vessel traffic services can be of particular interest where two such services share a common border because they may need to coordinate jointly with the master of a ship when the VTS sailing plan is being agreed.  In other cases it should be recognised that the exchange of data between vessel traffic services could give advance notice of arrivals thus relieving the reporting burden of vessels.  It could also provide an Administration with valuable information on future traffic and cargo flows in its intermediate sea area.

## 2.3	TECHNICAL OPTIONS (needs more updating)

To perform an information, navigational assistance or traffic organisation service the VTS personnel should at all times have access to a VTS traffic image.  For this purpose it is essential that the VTS collects and evaluates data to develop the traffic image and disseminates the decisions resulting from an assessment of the traffic image.  Each of the crucial tasks have different technical options.

The options for developing and maintaining the VTS traffic image can be simple in their level of technology and rely substantially on human involvement for proper operation, or employ highly sophisticated techniques requiring a minimum of human assistance, or be of a level of technology somewhere between these two extremes.  An authority intending to establish a VTS scheme will need to take into account both the technical options and the standard of VTS operators expected to be available when assessing the most cost-effective mix of human activities and equipment sophistication to suit the particular operational requirements for the VTS under consideration.

The display of the VTS traffic image with its widely varied data will need careful consideration.  In general it will be required by a VTS operator in two easily interpreted forms; a plan display which may or may not be radar based and an associated table.  In most VTS centres these will be maintained electronically with the information being extracted from the systems main track table.

A vital feature of such displays is that they should be flexible to the operators task and requirements. Whereas each display should be capable of showing all the available data, the VTS Operator may wish to restrict this by the use of special purpose features.

### 2.3.1	Data Collection

In a VTS information needs to be collected which relates to a number of different topics, for example, meteorological conditions in the VTS area, the operational serviceability of equipment, the utilisation of resources such as tugs, pilots and berths and the movements of shipping.

Data collection with the main objective of compiling and maintaining the VTS traffic image includes obtaining information on:-

- the fairway situation, including meteorological and hydrological conditions and the operational status of aids to navigation;

- the traffic situation, including vessel movements and vessel conditions with particular regard to cargo carried and the state of hull and machinery.

However, it should be recognised that a VTS traffic image is only as good as its inputs.  If the latter are wrong, then so is the traffic image and the information deduced from it. In this respect the information derived from AIS still needs special attention.

Most of the strategic information can be acquired from hydrographic and meteorological sensors such as tide and current gauges, databases of allied services and adjacent vessel traffic services and shore authorities, such as meteorological offices, port authorities.  This information can normally be obtained through the public switched networks by telephone, telex, facsimile or automatic data transfer systems.

There are several methods which can assist in obtaining tactical information on the movement and identification of ships by a VTS centre, including:

Radar, which is probably the major current technical method of monitoring the movement of shipping because it can provide an overview of the complete area covered by the particular radar installation.  With most modern VTS radar equipment it is possible to delineate on the display traffic routes, fairways and other places or marks of importance.  It is also possible to prevent the display of land masses and other areas which are not part of the VTS scheme and which might cause unnecessary difficulties in interpretation of the display, or interference to it.

Radar on its own cannot identify the targets which it display.  Identification is currently made by the use of VHF by requesting ships to report at certain points.  However, this method can be slow and prone to error.  Where VHF Direction Finders are available, the bearing, or bearings, of the ship can be obtained when identification calls are made and these can be superimposed on the radar display of the area concerned.  The radar echo of the ship can thus be identified and the ships’ movements subsequently monitored either manually on the display or by means of a computer system operating in conjunction with the radar.  This method is limited to areas in which there is radar coverage.

The Automatic Identification System (AIS) has been introduced on ships progressively from June 2002. This system will be capable of transmitting messages containing the identity, position and details of a ship:

- in automatic mode, at intervals of time of between 2 seconds and 3 minutes, depending upon the movements of the ship; or,

- in manual mode, when it is properly interrogated.

Position information is derived from a radionavigation system, such as GPS or, differential GPS where it is available, and provided continuously to the AIS equipment.  The movements of ships fitted with AIS equipment could be superimposed on a radar display or on an electronic chart display.  However it should be noted that information on vessels not fitted with AIS equipment will not be displayed.   Although in the first instance these systems are being developed primarily for use with VHF, they could also be used in other frequency bands to provide a long range service.  Further information on AIS is given in Chapter 4.

Substantial developments are taking place in the use of low light level televisions and infra-red devices.  These will provide information on the shape of a ship during darkness and in inclement weather.  However, these methods of image intensification are limited to visibility distance.

In some circumstances the identification and movement of ships might be obtained visually, either directly by personnel in a VTS centre or by another participating vessel.  However, this method requires the information to be transferred to the VTS traffic image manually, and is limited by the location of the VTS centre and by the possibility that there may be no other vessels in the vicinity of the ship in question.

### 2.3.2	Data Evaluation

The most important information that needs to be analysed is that concerning the movements of shipping in the VTS area in order to determine whether action from the VTS is required and if so to decide upon the appropriate action.  The movement of a ship needs to be considered with regard to the movements of other shipping in its vicinity, the geographical and navigational arrangement of the area concerned, navigable waterways, traffic routeing schemes, and the local rules which regulate the traffic movements.

To achieve this, the VTS traffic image should at all times be a comprehensive overview of traffic in the VTS area and take into account all factors which influence the movement of the traffic.  The VTS traffic image should enable VTS personnel to obtain, either by plotting or by monitoring and labelling, full information about each participating vessel and its intentions.

In the simplest arrangement, the evaluation of ship movement information can be carried out by the use of a manual VTS traffic image, showing the layout of the VTS, the geographical and navigational arrangements and the present position of ships.   The VTS operator would need to be able to develop the traffic image from the information collected and to assess whether the movement of every ship is in accordance with local regulations and whether the movements are likely to cause the risk of a collision, grounding or pollution to increase to an unacceptable level.

This type of arrangement may be suitable in small VTS areas where the traffic density is low, waterways are wide and provide ships with plenty of manoeuvring space and where therefore the risks are minimal.

A higher level of equipment sophistication is provided by the use of radar where the VTS traffic image of ship movements can be maintained on the radar display.  The VTS operator would still need to be able to assess the movements of each ship in terms of local regulations and risk but would normally be able to handle an area of medium traffic density in restricted waterways.

To enable an area of high traffic density with restricted waterways and with high risks to be monitored and controlled adequately assistance in the evaluation of vessel movement information can be obtained by the use of computers which develop automatically a traffic image of the complete VTS from the inputs of the radar equipment and other sensors.

With the computer software  programmed to suit the particular VTS area, in terms of the geographical and navigational arrangements and local shipping movement rules and regulations VTS operators would be able to assess the situation very rapidly and identify potentially hazardous shipping movements or breaches of rules or regulations with a minimum of delay. Computer programmes also exist to alert VTS operators when potentially dangerous situations are developing.

### 2.3.3	Data Dissemination

Decisions resulting from the assessment of the traffic image will need to be communicated to users and possibly others. This task includes decisions on the address, composition, character and priority, of the message, the selection of the transmission mode and other matters aimed at obtaining the desired results.

Communications with vessels of a general nature should be broadcast at routine scheduled times by radiotelephony on VHF.  The interval between broadcasts should be adjusted to suit the degree of risk in the area concerned and the importance of the information being provided.  Vital navigational information should be broadcast as soon as it is received at the VTS Centre and subsequently repeated at appropriate intervals of time.

Communication with a particular vessel should be made either on request by the vessel or when deemed necessary by the VTS and should be conducted in accordance with the established operational procedures which should include fall-back procedures in case of disrupted communication.

Any VTS message directed to a vessel should make it clear whether it contains information, advice or instructions.

The use of an automatic identification system could improve considerably the effectiveness of communications including the transfer of data between a VTS Centre and vessels. It would ensure that routine information was transmitted rapidly and with the minimal interference, keeping the VHF voice channel relatively clear of communication traffic so that important vessel movement information could be transmitted with a high probability that it would be received by the ship to which it was addressed.

In addition to radio, data may in appropriate circumstances, be sent by visual methods, including traffic lights, loudhailers and data transfer systems.

## 2.4	LEGAL CONSIDERATIONS

### 2.4.1	General

The intent of this section is to evaluate the legal authority of the VTS and the exercise of that authority.  It is not intended to be used as a legal guide for the placement of liability for vessel casualties.

Once a Competent Authority recognises the need to establish and operate a VTS within a port, harbour or waterway, that authority must assume responsibility for the proper and professional operation of the Service.  The Competent Authority should define the area of jurisdiction of the VTS and identify the persons or vessels subject to its authority.  It should also describe clearly the responsibilities of both the VTS personnel and those over whom the VTS exercises authority.

There are three major legal issues arising from the implementation of a VTS.  First, the authority of central government, port or other maritime entity to impose control over vessels by means of VTS regulations; second, the relationship between VTS personnel and ship masters or pilots operating in VTS waters; and third, the determination of liability in the event of an accident involving a vessel participating in a VTS.

### 2.4.2	Authority

A statutory authority with the necessary powers to implement a service designated to improve safety, security and efficiency of traffic and the protection of the environment for vessels operating within the area of its jurisdiction is the foundation for a VTS.  Such powers are generally provided in law formulated by a central government and delegated to a maritime arm of that government, a local or regional port.  Examples of such laws are:-

- The Harbours Act of the United Kingdom (1964)

- The Ports and Waterways Safety Act of the United States (1973, 78, 90)

- The Shipping Act of the Netherlands (1988)

- The Canada Shipping Act (1989)

- The Code of Maritime Ports (1974) (France)

A summary of some international and national legislative measures relevant to VTS is given in Annex 5.

These laws were created to establish a means of increased supervision of vessel and port operations in the interest of safe and efficient navigation and to protect property and the environment.  They authorise the establishment and operation of a VTS and may specify the classes of vessels required to utilise or comply with the VTS.  Local instruments may also require vessels to install and use specified navigation, communications and other safety equipment.

In many cases, laws are established to authorise control of vessel traffic in hazardous areas, under unsafe circumstances, in reduced visibility, adverse weather, or vessel congestion.  Degrees to which this authority can be exercised should be clearly established in the law, and may include the authority to:

- specify times of entry into, movement within  or departure from the VTS area;

- establish traffic routing schemes;

- establish vessel operating conditions, and any limitations of size, speed or draft;

- restrict operations to vessels having particular operating characteristics and capabilities; and

- instruct vessels to act when justified in the interest of safety.

Statutory instruments establishing a VTS Authority should recognised that shipping is  inherently an international activity and that the rights of innocent passage and free navigation are paramount. IMO has adopted conventions and regulations to ensure the safety of navigation, reduce the risk of maritime accidents and for the prevention of marine pollution. State legislation should supplement or parallel this international legal framework.

As a result of the United Nations Conference on the Law of the Sea (UNCLOS) coastal States can now claim jurisdiction over internal waters, territorial seas, contiguous zones, archipelagic waters, exclusive economic zones (EEZs) and the continental shelf.  However, the extent of the jurisdiction which can be claimed is different for each of the waters, seas and zones.  When a VTS is being considered, care should be taken to establish the extent of jurisdiction which can be applied to the VTS area and its sub-areas or sectors.

With regard to the authority which may be given to a VTS, a State retains the right to control its internal waters and all vessels which are subject to the jurisdiction of the State.  Therefore, the authority to establish and operate vessel traffic services in this region is clearly established, including the right to mandate participation in a VTS scheme and to exercise control over a vessel’s movements.  Within territorial waters, a coastal state may exercise its fully authority subject to the right of innocent passage.  Beyond territorial waters, a state’s authority with regard to VTS is substantially reduced.

In straits used for international navigation, a VTS Authority cannot restrict or impede the innocent passage of vessels. In these instances a State should endeavour to enter into agreements with neighbouring states or other maritime nations to agree on standards of conduct for vessels operating in these waters. These standards may include provisions for voluntary participation in a VTS.

### 2.4.3	Relationship between VTS and vessels

Safe movement of marine traffic in a VTS area requires the VTS to have an understanding of the responsibility of the masters of vessels and vice-versa.

A VTS, with its specialised knowledge of the waterway, has responsibility for managing the traffic in the area. While the master of a vessel, with his knowledge of its behaviour and his professional skills, has responsibility for the safety of the vessel.  Taking into consideration these different, but related, responsibilities, any instructions from a VTS to a vessel should be “result orientated” only, leaving the details of execution to the master, officer of the watch or pilot on board the vessel.

Generally masters of vessels rely with confidence on the expertise and professionalism of VTS personnel and carry out instructions which are given.  However, it should be recognised that there may be occasions when an instruction by a VTS is disregarded because the master considers that its execution might jeopardise the vessel.

### 2.4.4	Liability

Having recruited, trained and certified qualified personnel to operate a VTS, the VTS Authority must assume further responsibility to ensure that the actions of these persons are performed with a high degree of competence.  If these responsibilities are met fully the question of assignment of liability reverts to a case by case determination of the circumstances surrounding each incident.

The question of assignment of liability in regard to vessel movement is one of constant controversy.  It is complicated by the legal relationships between masters, pilots, ship owners, and VTS. Although there are many cases setting precedence on this issue, it continues to be an area in maritime and civil law that seems to be developing on a case by case basis.

In the event of a VTS operator issuing instructions to a vessel and an accident occurring, many maritime laws of liability may then apply relating to whether or not VTS participation was mandatory or voluntary, keeping in mind that a ship’s master has the authority to override the instructions of a VTS where, in his view, the instructions given could result in an accident.