# CHAPTER 1:  INTRODUCTION

# 0101  Purpose of the Manual

This is the fourth edition of the IALA VTS Manual.  Like its predecessors it is intended to be a comprehensive guide to VTS and a point of reference for further detailed study.

The contents are aimed at a wide readership to encompass all who are in any way involved with the policy for, provision, development, operation and effectiveness of VTS, including those with management responsibility at board or corporate level and those who deliver services.

0102  Hierarchy of IALA Publications

IALA has established a hierarchy of documents that it publishes that is indicated in the table .  It will be noted that detailed policy guidance and advice is published in IALA Recommendations and it is not intended to provide a similar level of detail in this manual where such information is already available.  Reference is made in the text to the appropriate documents where they exist.

### IALA Recommendations  These documents represent the highest level of IALA documentation (would relate to a ‘standard’ in an intergovernmental organization).  Recommendations provide direction to IALA members on uniform procedures and processes that will facilitate IALA objectives.  IALA recommendations contain information on how the members should, plan operate and manage Aids to Navigation. Recommendations may reference relevant International Standards and IALA Guidelines

### IALA Guidelines  These documents provide detailed, information on an aspect of a specific subject, indicating options, best practices and suggestions for implementation.  IALA Guidelines relate to planning, operations and managing Aids to Navigation

IALA Manuals  These documents provide an overall view of a large subject area. Whilst aimed at introducing a subject to a widely varied readership, reference is also made to IALA Guidelines and IALA Recommendations, as well as other related international documents, as an indicator of further study.

.

Heirarchy of IALA Publications

## 0103  Organisations and Bodies concerned with VTS

IALA attaches great importance to its association with other maritime consultative bodies who participate in the work of the VTS Committee and who have played a key role in the development of guidance and the contents of this publication.  These consultative bodies include the following international organizations:

International Maritime Pilots Association (IMPA),

International Harbour Masters Association (IHMA)

International Federation of Ship Masters Association (IFSMA)

International Association of Ports and Harbours (IAPH) and

The Nautical Institute.

## 0104  Development of VTS – A Brief History

For centuries shipping has been a major means of transport to support world commerce.  There has always been a need for ships to navigate accurately, safely and expeditiously and to assist in this many authorities have provided aids to navigation in and around their coastal waters. The earliest aids to navigation were beacons and lights followed by the introduction of buoys. Over the years these earlier aids were developed to improve their conspicuity together with improvements to the ranges of lights and the introduction of audible fog warning devices.

Not long after World War II it became clear that the limitations of the means of traffic management then available prevented the full utilisation of port facilities in bad visibility conditions.

Figure 1: VTS test site Rotterdam 1948  Source: Photo, collection D. Zwijnenburg.

In particular the delays in vessel traffic movements caused by periods of dense fog resulted in serious disruption of port operations, the storage of goods and to other modes of transporting cargoes to and from ports.

The general opinion of experts was that shore based radar could provide traffic images in order to keep maritime traffic flows moving in port areas and their approaches. The first was established in Douglas, Isle of Man, in 1948. Some three months later, in the same year, the port of Liverpool established their radar site and trials took place in Rotterdam. ( Fig. 1)

In the nineteen fifties a number of shore based radar chains were established in other North West European major ports, for example the approaches to the port of Amsterdam (Ymuiden) in 1952 and the entire Rotterdam area was covered in 1956.

These early systems were intended primarily to avoid traffic delays and to increase efficiency of traffic flows in general. However, attention was also being given to the number of accidents and the way in which these might be reduced. This resulted in studies of the effect that shore-based radar was having on the number of accidents in port areas under radar surveillance. The studies concluded that, in addition to increasing the operational hours, thereby providing better utilisation of a port’s capacity, the number of accidents was reduced significantly.

In the seventies major oil tanker disasters (e.g. Torrey Canyon, Metula, Amoco Cadiz and many others) increased the public awareness of the damage being caused and substantial pressure came from environmental groups to protect the marine environment.  The concern that such disasters might happen in port approaches and port areas caused policy makers to reconsider the concepts of the port radar chains and the co-operation between pilots and radar chain operators. It was also widely felt that some form of international harmonisation was needed. However in those early days of traffic management aided by radar surveillance, the view on how to proceed further was debated at considerable length.

Slowly there was movement towards a co-ordinated approach that was to become Vessel Traffic Services (VTS). VTS was defined and the matter was discussed in IMO, known then as the Inter-Governmental Maritime Consultative Organisation (IMCO). An Assembly Resolution was adopted on the implementation of VTS that provided a framework for further harmonisation. The requirement was considered by IALA and a follow-up study was initiated. The development of modern technology was very important for the technical concept of VTS. In the mid 1990’s the resolution was reviewed, revised and superseded in 1997 by IMO Assembly Resolution (A.857(20)), the international source policy document for VTS.

The number of VTS systems worldwide increased rapidly during the last two decades. In a limited number of countries VTS has been established in inland waters with the same overall objectives that apply to the maritime VTS systems.

In summary, VTS has developed from a shore based radar system with the aim of enhancing navigation in bad visibility conditions to a modern system using multiple sensors with the objectives of enhancing safety, improving the efficiency of maritime traffic and protecting the marine environment. The realities of modern shipping, with larger and less manœuvrable ships, traffic congestion in ports, hazardous cargoes and the potential for environmental damage, demand that sophisticated measures be taken to reduce risks.  Establishing a Vessel Traffic Service is a significant response to that demand.  When established, implemented and operated within the context of international laws, conventions and maritime customs, and with the co-operation of vessel operators, a VTS can contribute substantially to the safety and efficiency of maritime traffic and protection of the environment.

As a result of the improvements in efficiency, safety and the reduction of potential environmental pollution experienced by authorities using a VTS, together with the rapid developments in computer technology, the number of Vessel Traffic Services has increased considerably and there are now about 500 of these services operational.

As Vessel Traffic Services have grown in number throughout the world, the operating concepts have led to two categories of VTS, coastal services and port or river services.  A coastal VTS is a service provided to assist the safe and expeditious passage of shipping through coastal waters, particularly where there is a high density of maritime traffic or an area of environmental sensitivity or through waters which may be difficult to navigate because of geographical constraints or offshore exploration.  A port or river VTS is a service provided to assist the efficiency and safe navigation of shipping when entering or leaving ports and harbours or when sailing along rivers or through waters which similarly restrict the manoeuvring of ships.

0105 Definitions and Abbreviations

A list of definitions and a glossary of abbreviations of the terms commonly used in connection with VTS are respectively at Annex A and B to this chapter.

ANNEX A:   DEFINITIONS

| Accident Accredited training institute Accredited training programme Allied Services Competent Authority Hazardous Cargoes On-the-job training Place of Refuge Vessel traffic service VTS authority VTS area VTS centre VTS certification log VTS operator VTS operator position VTS sailing plan VTS services VTS superviser VTS traffic image | An unintended event resulting in fatality, injury, ship loss or damage, property loss or damage, or environmental damage An establishment approved by a competent authority for training VTS personnel Allied Services are services actively involved in the safe and efficient passage of the vessel through the VTS area. Competent Authority is the authority made responsible, in whole or in part, by the Government for the safety, including environmental safety, and efficiency of vessel traffic and the protection of the environment in the area Hazardous Cargoes include: - Goods classified in the IMDG Code - Oils, noxious and harmful substances defined in MARPOL - Radioactive materials listed in the INF Code A structured course of training undertaken at a candidates prospective work-place to consolidate theoretical training by practical experience, the successful completion of which enables the individual to become a watch stander under supervision. A place where a ship in need of assistance can take action to enable it to stabilize its condition and reduce hazards to navigation, and to protect human life and the environment. (IMO Resolution A.949(23). A service implemented by a Competent Authority, designed to improve the safety and efficiency of vessel traffic and to protect the environment. The service should have the capability to interact with the traffic and respond to traffic situations developing in the VTS area. The authority with responsibility for the management, operation and co-ordination of the VTS, interaction with participating vessels, and the safe and effective provision of the service. The delineated, formally declared service area of the VTS. A VTS area may be subdivided in sub-areas or sectors. The centre from which the VTS is operated. Each sub-area of the VTS may have its own sub-centre. An individual personal record of certificates and endorsements awarded for VTS skills. An appropriately qualified person performing one or more tasks contributing to the services of the VTS. A VTS Operator Position is the place in a VTS Centre from which a VTS Operator carries out his/her duties. A plan which is mutually agreed between a VTS Authority and the master of a vessel concerning the movement of the vessel in a VTS area. VTS should at least comprise an information service and may include services such as navigational assistance service or a traffic organization service, or both of these services, as defined in Resolution A.857(20). A VTS traffic image is the surface picture of vessels and their movements in a VTS area. |
| --- | --- |

ANNEX B:   GLOSSARY OF ABBREVIATIONS

| AIS AISM (see IALA) ARPA AtoN COLREGS COG CPA DCPA DGNSS DGPS DP DR DSC EC ECDIS ECS EDI EEZ EMPA EPIRB ETA ETD FSA GLONASS GMDSS GPS HAZMAT HNS HSC HNS IACS IALA (see AISM) IAMSAR IAPH IELTS IFSMA IHMA IHO IMDG IMO IMPA INMARSAT INF INS IS ISPS ITU ITU-R MARPOL MAS MEDIVAC MEPC MMSI MRCC MSC NAS NAVGUIDE NUC OJT OPRC PIANC PSSA RACON RCC RCDS RSO SAR SBP SMCP SOG SOLAS SRS STCW STDMA TOS TSS UAIS UKC UNCLOS VDR VHF VTS VTSO WMO | Automatic Identification System (sometimes Universal AIS) Association Internationale de Signalisation Maritime (Title of IALA in the French language) Automatic Radar Plotting Aid Aid(s) to Navigation Collision Regulations (International Regulations for Preventing Collisions at Sea) (IMO Pub. 904E) Course over the ground Closest point of approach Distance (to the) closest point of approach Differential Global Navigation Satellite System Differential Global Positioning System Dynamic Positioning (A vessel control system for precise positioning) Dead Reckoning Digital Selective Calling European Community Electronic chart display and information system Electronic chart system Electronic data interchange Exclusive Economic Zone (Defined in UNCLOS) European Maritime Pilots’ Association Emergency Position Indicating Radio Beacon Estimated time of arrival Estimated time of departure Formal Safety Assessment Global Navigation Satellite System Global Maritime Distress and Safety System Global Positioning System Hazardous Material Hazardous and Noxious Substances (IMO Convention, not yet in force) International Code of Safety for High Speed Craft (SOLAS Ch. X) Hazardous and Noxious Substances (Protocol to OPRC Convention) International Association of Classification Societies International Association for Marine Aids to Navigation and Lighthouse Authorities International Aeronautical and Marine Search and Rescue Manual International Association of Ports and Harbours International English language testing system International Federation of Ship Masters’ Association International Harbour Masters’ Association International Hydrographic Organisation International Maritime Dangerous Goods Code International Maritime Organization International Maritime Pilots’ Association International Marine Satellite Organisation Irradiated Nuclear Fuel on board Ships Code (IMO Publication 270E) ) Information Service ) International Ship and Port Facility Security Code (IMO Pub.116) International Telecommunications Union International Telecommunications Union – Radiocommunications Bureau Regulations and provisions of the International Convention for the Prevention of Pollution from Ships 1973/1978 Maritime assistance service (IMO Resolution A….) Medical evacuation Marine Environment Protection Committee (Standing Committee of IMO) Maritime mobile satellite identification (number) Maritime Rescue Co-ordination Centre Maritime Safety Committee (Standing Committee of IMO) Navigational Assistance Service IALA Aids to Navigation Guide Not under command (see Colregs) On-the-job training International Convention on Oil Pollution Preparedness, Response and Co-operation (OPRC),1990 Permanent International Association of Navigation Congresses Particularly Sensitive Sea Area Radar Beacon Rescue co-ordination centre Raster chart display system Recognized Security Organisation (ISPS Code) Search and rescue Shore based pilotage (a term in use but not accepted in practice) Standard Marine Communication Phrases Speed over the ground Convention on the Safety of Life at Sea Ship reporting system (Resolution A.851(20)) (Convention on) Standards of Training, Certification & Watchkeeping for Seafarers Self-organising time division multiple access Traffic Organisation Service Traffic Separation Scheme (Colregs Rule 10 and IMO Publication 927) See AIS Under keel clearance United Nations Convention on the Law of the Sea Voyage Data Recorder Very high frequency (radio in the Mhz band) Vessel Traffic Services Vessel Traffic Services Operator World Meteorological Organization |
| --- | --- |