# Note on Exchange of real time data between VTS Systems

Note from Norcontrol IT AS (O. Taugl)

The discussions around this topic have been concentrating on the data structures and how to transfer data between the different systems. As an input to this I would like to propose a standard using the principles from AIS:

- Uses NMEA format

  - $??VTS as header instead of $??AIS

  - Use checksum as according to NMEA

- Make a set of predefined messages suitable for VTS

  - Position Report

  - Voyage Report

  - Interrogation messages

  - Etc..

- Serial or LAN communication is decided on a project basis (not necessary to define as part of the protocol)

Most of the AIS messages are encoded according to 6 bit ASCII to preserve space on air. It is doubtful that this is needed in a land based protocol, so to make a more readable format, there is no real need to use this kind of preservation. The messages for VTS should therefore be in readable format for easier decoding and debugging.

The protocol is very easy to expand as new messages can be added instead of amending the existing ones. I.e. making a binary message containing an XML schema would be possible if needed. Also messages containing radar video, met/hyd data and even cctv pictures might be of interest.

Another great advantage using this approach is that all(?) the VTS manufacturers have already interfaced the AIS protocol, so using a similar protocol for VTS would probably reduce development time considerably as opposed to other technologies.