REPORT

on

the 80th session of the IMO Maritime Safety Committee

(MSC)

Contents

# 1.	Introduction.

2.	Long-range identification and tracking of ships

3.	Voluntary IMO Audit Scheme

### 4.	Adoption of SOLAS amendments and other amendments

## 5.	Matters related to the Sub-Committee on Radiocommunication and Search and Rescue (COMSAR)

# 6.	Other item: goal-based construction standards for new ships

# 7.	Recommended actions by IALA

8.	Date of the next sessions

Annex  :

Agenda of the Sub-Committee on Radiocommunication and Search and Rescue 10th session (6 – 10 March 2006)

________

# 1.	Introduction.

The 80th session of the IMO Maritime Safety Committee was held at the headquarters of the Organisation in London during the period of 11 to 20 May 2005. It was attended by delegations from 104 countries and 42 observer organisations. The Secretary General and J-C Leclair represented the Association.

The items of the agenda for the session of most interest to IALA members were related to Long-Range Identification and Tracking of ships – for which no consensus  was yet reached due to different opinions on the right of coastal States – and the Voluntary IMO Audit Scheme, approved by the Committee for adoption by the next IMO Assembly (November 2005). The Committee also adopted several amendments to the SOLAS Convention and its subsidiary mandatory Codes, in particular on revised provisions for subdivision and stability (chapter II-1 Construction - Structure, subdivision and stability, machinery and electrical installations) and on the mandatory requirement for IMO unique company and registered owners identification number scheme. Another important issue on the MSC agenda was related to the goal-based construction standards for new ships for which the Committee took a first decision in adopting the goals themselves.

2.	Long-range identification and tracking of ships

The Committee had been discussing the issue of long-range identification and tracking of ships (LRIT) since February 2002. The matter had also been debated by the COMSAR Sub-Committee as well as working groups and correspondence groups. At this session again the Committee and its Working Group on Maritime Security held extensive discussions on the matter. The general mechanism of the system should be adopted next year based on the general design such as proposed by the last COMSAR meeting (ref. report of COMSAR 9). Two main points still remain to be solved.

The first one regards the choice on the SOLAS chapter into which the proposed SOLAS Regulation on LRIT would be included, chapter XI-2 as proposed by the United States and supported by two other delegations, or chapter V as requested by a large majority of delegations considering the use of LRIT not only for security purposes but also for safety and protection of environment matters. The Committee postponed its decision to the next session.

The second point is more contentious and regards the provision of LRIT information to a coastal State and to SOLAS contracting Governments. At this stage no consensus was reached and the Committee took note of the current position of the Working Group on Maritime Security summarized by its Chairman as follow:

“General: Nothing in the Regulation shall prejudice the rights or obligations of States under international law.

Purpose: The purpose of the Regulation is to establish a mechanism for the collection from ships of LRIT information for security, search and rescue and any other purpose as determined by the Organization and a scheme for the provision of LRIT information to Contracting Governments.

Application: The regulation shall apply to the following types of ships engaged on international voyages: passenger ships, including high-speed passenger craft; cargo ships, including high speed craft, of 500 gross tonnage and upwards, and mobile offshore drilling units as defined in regulation XI-2/1.1.5.

Flag State: The Administration shall be able to receive LRIT information from all ships entitled to fly its flag irrespective where such ships may be located. An Administration shall be able to communicate to the Organization the name(s) of the Contracting Government(s) which shall not receive, under the provisions of paragraph 6, LRIT information from some, all, or certain types of the ships entitled to fly its flag. [An Administration shall be able to decide and communicate to the Organization the distance that it consents to any Contracting Government receiving, as a Coastal State, LRIT information].

Port State: A Contracting Government shall be able to receive LRIT information from all ships, irrespective of the flag such ships are entitled to fly, which have communicated to that Contracting Government an intention to enter a port facility or a place under its jurisdiction. Contracting Governments shall specify, and shall communicate to the Organization, either the distance from their coast or the period of time prior to the expected time of arrival of the ship in a port facility or a place under their jurisdiction, during which they request the provision of LRIT information.

Coastal State: Subject to the previous provisions regarding flag State, a Contracting Government shall be able to receive LRIT information from all ships, irrespective of the flag such ships are entitled to fly and not intending to enter a port facility or a place under its jurisdiction, navigating within a distance not exceeding [2000] nautical miles from its coastline, commensurate to its security needs. Contracting Governments shall communicate to the Organization the distance from their coastline within which they request the provision of LRIT information. A Contracting Government shall not receive LRIT information until such time it communicates to the Organization the distance from its coastline, within which it requests the provision of LRIT information. A Contracting Government which has been prevented from receiving LRIT information under the previous provisions regarding flag State, shall be able to conclude with the Administration concerned an agreement which would enable the Contracting Government to receive LRIT information. Such agreements shall be communicated to the Organization.

Related matters: The Organization shall circulate salient elements of the communication received, to Contracting Governments, on a need to know basis, for their information. The Organization and the Contracting Governments are obliged to respect the confidential nature of the information provided and shall protect these against unauthorized access or disclosure. Information circulated by the Organization shall in no way adversely affect the security of Contracting Governments or of ships entitled to fly their flags. Communications shall be considered as being effective [30] days after the date on which they are notified to the Organization. A Contracting Government shall be able, at any time, to amend, modify or withdraw any communications it has made to the Organization. A Contracting Government, other than the flag State, shall not receive LRIT information from any ship whilst that ship is within the internal waters [or territorial sea] of another Contracting Government.

Development of LRIT: The LRIT system should be developed in such way so that, when it expands to include other applications as determined by the Organization, it will not require any substantial changes.”

The Committee noted that there were still a number of outstanding issues to be resolved

and agreed that:

- an intersessional working group should meet ahead of the 10th session of the COMSAR Sub-Committee in early 2006 (27 February – 3 March) so that COMSAR 10 (6 – 10 March) would be able to finalize the work;

- the COMSAR correspondence group on LRIT should consider a number of technical issues;

- an MSC intersessional working group on LRIT (17 – 19 October 2005) should also meet for the purpose of developing draft SOLAS amendment on LRIT with a view to consideration and adoption at MSC 81 (10 – 19 May 2006).

3.	Voluntary IMO Audit Scheme

The objectives of the proposed IMO Member State Audit Scheme are designated to help promote maritime safety and environmental protection by assessing how effectively Member States implement and enforce relevant IMO Convention standards, and by providing them with feedback and advice on their current performance. The Audit Scheme is based on the standards contained in the Code for the implementation of mandatory IMO instruments, developed by the FSI Sub-Committee, which stated that, under the provisions of the United Nations Convention on the Law of the Sea 1982 (UNCLOS) and of IMO conventions, Administrations are responsible for promulgating laws and regulations and for taking all other steps which may be necessary to give these instruments full and complete effect so as to ensure the safety of life at sea and the protection of the marine environment.

The mandatory instruments addressed in the Code and covered by the audits are limited to SOLAS, MARPOL, STCW, Load Lines, Tonnage 69 and COLREG. But all obligations contained in these conventions should be audited. It means that if a country is volunteer to be audited, all chapters of SOLAS, including chapter V, should be included in the audit program which should cover its obligations as flag State, port State and coastal State. Therefore the list of the obligations developed by the Joint Working Group includes subjects of particular interest for IALA such as the aids to navigation, the promulgation of information to mariners, the traffic routeing measures and the vessel traffic services.

The Maritime Safety Committee approved the draft Code for the implementation of mandatory IMO instruments and invited the next Assembly to adopt it. The Committee also approved the IMO Member State Audit Scheme, composed of the following draft documents, in view of their adoption by the Council then by the Assembly in November 2005:

Framework for Member State Audit

Procedures for Member State Audit, including a model Memorandum of Co-operation

Pre-Audit Questionnaire

Assembly Resolution.

As proposed by the IALA delegation during the works of the Joint Working Group on the Voluntary IMO Member States Audit Scheme, the Committee also endorsed the offer from the Association to develop guidance which will assist in the audit of vessel traffic services and aids to navigation services. The MSC also invited IHO to consider undertaking similar initiative for hydrography.

### 4.	Adoption of SOLAS amendments and other amendments

The so-called “expanded” Maritime Safety Committee met during the session and formally adopted the proposed amendments to the 1974 SOLAS Convention as follows:

- amendments to Chapter II-1, Construction – Structure, Subdivision and Stability, Machinery and Electrical installation (parts A, B, B-1 and C), to harmonize the provisions on subdivision and damage stability for passenger and cargo ships;

- amendments to Chapter XI-1, Special Measures to Enhance Maritime Safety, Regulation 3-1, Company and Registered Owner Identification Number;

- other consequential amendments in other chapters, to take into account the two previous amendments.

- Amendments to the ISM Code and to the ISPS Code were also adopted.

## 5.	Matters related to the Sub-Committee on Radiocommunication and Search and Rescue (COMSAR)

The Committee approved the report of the 9th session of the COMSAR Sub-Committee, in particular the works done on LRIT.

It endorsed the Sub-Committee’s action in issuing COMSAR/Circ.36 on broadcast of warnings for tsunami and other disasters in line with the IMO response to the disaster of 26 December 2004. The Secretary General of IMO reported to the Committee the actions taken by the Organization together with IALA and IHO to assist the countries affected by the tsunami.

# 6	Other item: goal-based construction standards for new ships

The Committee continued to give consideration to the development of goal-based construction standards for new ships, started at MSC 77, which should form the foundation for the future advance of international regulatory standards.

It was already agreed that the standards will be developed on the basis of a five-tier system: goals (Tier I), functional requirements (Tier II), verification of compliance criteria (Tier III), technical procedures and guidelines, classification rules and industry standards (Tier IV) and codes of practice and safety and quality systems for shipbuilding, ship operation, maintenance, training, manning, etc. (Tier V).

At this session, the Committee succeeded to reach a decision on tier 1 – goals – based on the premise that ships are to be designed and constructed for a specified design life to be safe and environmentally-friendly, when properly operated and maintained under the specified operating and environmental conditions, in intact and specified damage conditions, throughout their life.

# 7	Recommended actions by IALA

# It is recommended that:

# 7.1	The Council

- Note the discussions on long range identification and tracking (LRIT) of ships (item 2).

- Note the decision taken by the Committee on the Voluntary IMO Audit Scheme (item 3) and the IALA’s proposal to develop guidance which will assist in the audit of vessel traffic services and aids to navigation services.

## 7.2	The Committees

- Note the discussions on long range identification and tracking (LRIT) of ships (item 2).

- Note the decision taken by the Committee on the Voluntary IMO Audit Scheme (item 3) and the IALA’s proposal to develop guidance which will assist in the audit of vessel traffic services and aids to navigation services.

# 8	Date of the next session.

The 81st session had been scheduled to take place from 10 to 19 May 2006.

*  *  *

J.Ch. Leclair.

IALA Representative to IMO

18 July 2005.

# Annex

Sub-Committee on Radiocommunications and Search and Rescue (COMSAR)

10th session - Agenda proposed to the Maritime Safety Committee

Opening of the session

1	Adoption of the agenda

2	Decisions of other IMO bodies

3 	Global Maritime Distress and Safety System (GMDSS)

- Matters relating to the GMDSS Master Plan

- Operational and technical co-ordination provisions of maritime safety information (MSI) services, including review of the related documents

4	ITU maritime radiocommunication matters

- Radiocommunication ITU-R Study Group 8 matters

- ITU World Radiocommunication Conference matters

5	Satellite services (Inmarsat and COSPAS-SARSAT)

6	Emergency radiocommunications, including false alerts and interference

7	Matters concerning search and rescue, including those related to the 1979 SAR Conference and the implementation of the GMDSS

- Harmonization of aeronautical and maritime search and rescue procedures, including SAR training matters

- Plan for the provision of maritime SAR services, including procedures for routeing distress information in the GMDSS

- Medical assistance in SAR services

8	Developments in maritime radiocommunication systems and technology

9	Review of the IAMSAR Manual

10	Review of the SPS Code

11	Measures to enhance maritime security

12	Passenger ship safety

13	Revision of the performance standards for SART

14	Work program and agenda for COMSAR 11

15	Election of Chairman and Vice-Chairman for 2007

16	Any other business

17	Report to the Maritime Safety Committee

__________