# CHAPTER 7:   DETERMINING THE NEED FOR VTS

0701 Introduction

This chapter provides guidelines to aid the decision making process in judging the need for establishing a VTS, or review an existing VTS, by providing a framework to assist competent authorities to:

- Assess the risks associated with a waterway

- Assess the contribution that VTS can provide in mitigating risk and improve the safety and efficiency of navigation, safety of life and the protection of the environment

- Determine the level of sophistication of the vessel traffic management system required where it is decided that a VTS is the appropriate tool.

In deciding whether or not to implement a VTS there are essentially two fundamental questions to be addressed by a competent authority.  These are:

- What are the environmental, safety and economic consequences of having or not having a VTS given the currently implemented safety systems?

- What is the level of investment that can be justified by the improvement in the system safety?

## 0702	Mechanisms to Improve Maritime Safety and Efficiency of Navigation

Each harbour, port or coastal waterway is inherently different and the requirement to manage navigation varies considerably.  It should be recognized that a VTS may be essential in some waterways, however, other mechanisms may be more appropriate in others.

Determining whether a VTS is an appropriate mechanism to address concerns about the levels of safety is often difficult to determine.  In most, if not all, the need for a VTS only becomes readily apparent when all mitigating factors are considered in a formal assessment of navigational risk to identify what management of navigation is required and to what degree monitoring and traffic organisation needs to play a role in mitigating risk.

From the risk assessment some authorities may identify the need to provide a VTS as specified in the IMO and IALA documentation.  Other mechanisms, such as Port Information (see Chapter 5.x), will often provide a suitable level of service to mitigate risk where it has been assessed that a VTS, as described above, is excessive or inappropriate.

Identifying the threshold between Port Information and VTS is often difficult to determine.  It is likely to be port specific and will only become clear in the risk assessment process, when all mitigating factors are considered. Port Information is applicable where interaction is unnecessary to fulfil the statutory requirements of the harbour authority’s duties with regards to navigational safety.

### 0703	VTS

The purpose of a VTS is to improve the maritime safety and efficiency of navigation, safety of life at sea and the protection of the marine environment and/or the adjacent shore area, work sites and offshore installations from possible adverse effects of marine traffic in a given area. <insert IALA reference re role of VTS in security [paper went to council identifying VTS role in Security – session 20]>

The benefits of implementing a VTS are that it allows identification and monitoring of vessels, strategic planning of vessel movements and provision of navigational information and assistance. It can also assist in reducing the risk of pollution and coordinating pollution response.

Many authorities express difficulty in establishing justifiable criteria for identifying whether or not VTS is the most appropriate tool to improve the safety and efficiency of navigation, safety of life and the protection of the environment.

A VTS is generally appropriate in areas that may include any, or a combination, of the following:

- High traffic density;

- Traffic carrying hazardous cargoes;

- Conflicting and complex navigation patterns;

- Difficult hydrographical, hydrological and meteorological elements;

- Shifting shoals and other local hazards;

- Environmental considerations;

- Interference by vessel traffic with other waterborne activities;

- Number of casualties in an area during a specified period;

- Existing or planned vessel traffic services on adjacent waterways and the need for cooperation between neighbouring states, if appropriate;

- Narrow channels, port configuration, bridges, locks, bends and similar areas where the progress of vessels may be restricted;

- Existing or foreseeable changes in the traffic pattern in the area.

## 0704	Needs Analysis

A VTS is invariably a considerable investment. It is strongly recommended that before considering the establishment of a new VTS or the enhancement of an existing VTS, a Competent Authority should undertake a formal study to clearly define the need, the functional requirements and identify both the set-up and on-going costs associated with implemention.

Attention is drawn to IALA Recommendation V-119 on the Implementation of Vessel Traffic Services, which provides a comprehensive list of all the relevant items to be addressed.  These recommendations provide four key steps to undertaking a needs analysis to determine whether or not a VTS is an appropriate mechanism to maintain or improve maritime safety and that the competent authority has the requisite capability and resources to implement one.  These include:

| - Preliminary Assessment (Inception) | The inception phase is intended to answer the question of whether active traffic management is the appropriate means to address the local traffic problems. This phase includes a preliminary risk assessment and in some cases may be enough to identify whether to proceed further. |
| --- | --- |
| - Feasibility and Design | The Feasibility and Design Phase is intended to identify the functional requirements required to attain the desired level of safety and efficiency of the maritime traffic. |
| - Formal Risk Assessment | The Risk Assessment Phase is intended to confirm that the measures being designed and introduced will reduce the risk of collisions and groundings in the area to a level considered by the Competent Authority to be satisfactory. |
| - Cost / Benefit Analysis | The cost / benefit phase is intended to identify whether the expected reduction in risk would be justified in terms of the level of investment required. |

Fig. 1 steps for undertaking needs analysis

This chapter focuses on these four steps. The diagram below outlines the position of the steps within the entire VTS implementation process.

[Review and amend diagram to provide better format; note sections of diagram covered in this chapter and the sections that will need to be covered in other chapters]

INCEPTION

Problems

Constraints

Area

Traffic Management

Tools

Evolve options

Weightings

Importance

Risk ( Prelim. Assessment)

Organisation

Framework

Structure

Legal

Security

Funding

Operational objectives

Areas

Sub Area

Sub Area

Delineate areas

Measures to be taken

Determine level of service

FEASIBILITY AND

Functional Requirements

Functional Specification

Traffic Management Measures

Finalise Inception Proposals

Outline Procedures

Manning Training

Cost/Benefit analysis (preliminary)

…DESIGN STUDY

Geotechnical Data • Project Plan

Resource Allocation • Spend Profile

Timescale • Design Review/Audit

Equipment Technical Specification

Civil

Mechanical

Electrical

IT Hardware/Software

Training

FORMAL RISK ASSESSMENT

Navigational Risks:

• Environmental Risks

FSA Techniques

Estimation

Evaluation

Alternative mitigation measures

Estimated reduction

COST/BENEFIT

Analysis-Investment v risks reduction

Direct benefits

Indirect benefits

Prospects

Value Added Services

IMPLEMENTATION

Implement other traffic management measures

Preparation • Tender procedure

Purchasing           • Building

Legislation/organisation/personnel   • Trial operations

EVALUATION

Technical Performance

Operational Performance

Operational Objectives

Problems alleviated

Not acceptable               Acceptable

Figure 2  Main activities of the various phases in the development and procurement of a VTS

The Inception and the Feasibility and Design Phases should provide details of the VTS requirements to enable cost and performance estimations to be carried out under the Cost/Benefit Study Phase. The Cost Benefit Study should consider direct risk reduction (which may be vague), the less evident benefits that a future VTS might offer and the further value added services for shipping in the future. A realistic cost estimate for running a VTS is important. An estimate of possible future cost reduction to be achieved by slimming down the other waterway infrastructure costs should also be provided.

In the case where the Feasibility Study gives a positive result, the Competent Authority may proceed with the final design and planning work and launch a bid for tenders.

Sometimes the Inception, Feasibility and Design, Risk Assessment and Cost/Benefit Phases of the project are altogether classified as the Feasibility Study. This approach could be followed in the case where the Competent and/or VTS Authority has carried out a separate initial investigation to identify all the options available to address the risk and has subsequently determined that the preferred solution is to proceed with a Feasibility Study. Furthermore, the Feasibility and Design Phases may be incorporated within one phase, as opposed to comprising two separate phases. In this Recommendation, the Feasibility and Design has been treated as a single Phase.

IALA Recommendation V-119

Key Point

### 0705	Preliminary Assessment (Inception)

The purpose of the preliminary assessment phase is to decide the suitability of VTS as an appropriate traffic management option. Where this is confirmed, the information collected will provide the basis for undertaking the feasibility study. The preliminary assessment should identify as a minimum, the potential hazards, as well as the existing organisational infrastructure, operations and procedures.

The preliminary assessment should identify whether active traffic management is an appropriate means to address the local traffic problems. Active traffic management should only be used in those areas where other means are inadequate to provide the desired level of safety and protection of the environment.

The preliminary assessment phase should be an iterative process that involves the following key steps:

- A review of the organisational structure, including its culture, policies, procedures and priorities

- A review of the legal framework

- Define the area under consideration including its oceanographic characteristics

- Define and/or quantify:

- The inherent navigational and environmental attributes of the waterway

- The stakeholders

- The economic and environmental value of the waterway

- The public interest

- The maritime traffic utilising the waterway

- Available incident data, such as such as collisions and groundings

- Available data on traffic problems, delays

- Security considerations

- Identify the existing safety management structure, including it’s strengths and weaknesses

- Identify the key risks to navigation not being addressed by existing safety management structure

- Identify options to address the key risks

- Define operational objectives to alleviate the risks

- Identify the most appropriate traffic management tools, in terms of effectiveness and costs, to mitigate the defined problems. These traffic management tools may range from simple routeing measures through to the implementation of an advanced VTS system.

It should be recognised that one of the main difficulties faced in undertaking any form of risk assessment is that, in many cases, the full consequences of recorded casualties are not available. In such circumstances they must be estimated by expert judgement.

One must also take into consideration that the future is not a simple extension of history, so more refined methods must be applied to assess the estimated casualty costs and other consequences for the next ten years or so by taking into account all foreseeable trends. Risk estimation and evaluation form vital inputs to any risk assessment.

Future developments in the port infrastructure and the resulting changes in traffic volumes and composition, including dangerous cargoes and any other relevant future development in the area concerned should be considered in this phase.

In the specific case of a Coastal VTS, future trends in traffic volume and other activities in the coastal area, such as fishing, recreation and offshore activities should be taken into account.

Further developments in VTS technology and SOLAS requirements for navigational and communication equipment on board vessels should be considered as well.

Furthermore in the case of a decision to establish a VTS, the following aspects need to be addressed;

- Organisational framework of the national and local maritime authorities in relation to implementing new traffic management solutions, VTS in particular; and

- The present regulatory or legislative framework, including local by-laws, rules and recommendations. Special attention has to be devoted to ascertain any requirement for adjusting the framework to allow for effective implementation of a VTS.

In further deciding upon the establishment of a VTS, Authorities or Competent Authorities should also consider the responsibilities and the availability of the requisite technology and expertise.

### 0706	Feasibility and Design

The Feasibility and Design Phase is intended to identify the functional requirements required to achieve the desired level of safety and efficiency of the maritime traffic.

- The foundation for proceeding with the Feasibility and Design Study Phase is the information compiled in the preliminary assessment (Inception) Phase and the expected functions and benefits of a future VTS. This input may also give an indication of the desired typ of service to be provided by the VTS.

To adequately establish the functional requirements for the VTS the Competent Authority needs to assess the types of vessels using a particular area, the requirements to aid their safe and expeditious passage, the operational benefit of a VTS and the broad implications of providing the service.   These considerations should take into account the existing aids to navigation and traffic routeing schemes in the area concerned.

It is very important in this Feasibility and Design phase that the functional requirements to be developed do not lead to unnecessary expenses in the future VTS. Any consultants appointed by the Competent Authority should be independent from any VTS equipment manufacturers. Furthermore consideration should be given to the availability of the requisite technology and expertise. This is of particular importance for the required regular maintenance and to remedy defects and other trouble-shooting.

The feasibility and design study phase is also intended to provide a VTS Authority with a framework for proceeding further with development against carefully established guidelines of requirement, cost, risk and time. It should comprise some or all of the following:

- Description of the constraints and context in which the VTS will operate;

- Evaluation of the technology available and determination of  the standards to be used;

- Evaluation of the human resources needed for operation of the system and consideration of manning levels, training and skills required;

- Evaluation of the health and safety facilities needed to safeguard staff and other persons associated with the VTS system;

- Preparation of a management plan for the entire development;

- Assessment of the method, or methods, to be used for Quality Assurance;

- Assessment of the probability that the VTS system will be developed, installed, tested and ready for operational use within both the required time scale and the available financial resources;

- Development and evaluation of system design options

- Determination of the Integrated Logistic Support (ILS) requirements, including the identification of the through-life elements of the system and the means for achieving enhancement and upgrades;

- Evaluation of a Cost-Benefit analysis and the identification of any trade-offs.

The feasibility sub-phase should identify the range of activity that will need to be examined during the technical specification sub- phase, show the feasibility of any actions suggested and eliminate high-risk elements.

On satisfactory completion of the feasibility sub-phase the VTS Authority will be in possession of a highly detailed basis for proceeding to technical specification phase with confidence that its outcome will provide a viable solution for developing the system.

Attention is also drawn to IALA Recommendation V-119 on the Implementation of Vessel Traffic Services, which provides a comprehensive list of all the relevant functional requirements to be addressed within the feasibility sub-phase

In order to better facilitate the Cost Benefit Analysis it is important that a basic functional design is provided. Further, a system model, containing the key system attributes (sensors and other components) in broad outline, will be required.

The Technical requirements specifications, should produce the definitive statement of how the system, including buildings, is to be constructed, and how sub-systems and components should interact with each other to produce a viable VTS.

[Detailed information on technical specifications required/requested; t.b.d. where this should be placed – Cross Check as agreed]

### 0707	Formal Risk Assessment

The Risk Assessment Phase is intended to confirm that the measures being designed and introduced will reduce the risk of collisions and groundings in the area to a level considered by the Competent Authority to be satisfactory.

The risk level should be calculated by taking into account:

- The type, size, speed, manoeuvrability, routes and spatial distribution of ships using the area, including local craft;

- The types of aids to navigation provided in the area and their locations; and,

- The traffic routeing schemes in use in the area.

A total risk equation comprises the probability, or frequency, of an incident occurring, the consequences of an incident and the Governmental or public acceptability of such an incident.  The risk assessment should identify and quantify each of these aspects.

IALA Guideline 1018 on Risk Management (June 2001) outline a general risk assessment and risk management methodology for Marine Aids to Navigation including Vessel Traffic Services (VTS) so that all types of risks can be effectively managed.  The Guidelines may be used when assessing the optimum mix of aids to navigation, including VTS, for mitigating risk.

### Reference Documentation

Documentation which should be consulted includes, but not limited to:

- The IALA Guidelines on Risk Management –These Guidelines break down the Risk Management process into five clearly identifiable steps, namely, Risk Identification, Risk Assessment, Risk Control Options, Decision Making and Take Action.contain further details on risk assessment.

- IALA Risk Assessment model  – This document ,,,,<STILL waiting on a copy from IALA  - Should this be included as part of preliminary analysis or as part of the Risk Assessment –  suggest the latter>]

### 0708	Cost Benefit Analysis

After completion of the Design and Risk Assessment Phases, a cost benefit analysis should be carried out to determine whether the expected reduction in risk is justified in terms of the level of investment required.

Both the additional direct and indirect benefits and prospects that a VTS might offer, including additional value added services for the traffic in the future as well as the benefits to shore based port operations, should be taken into consideration.

A direct benefit that could be taken into consideration, amongst others, is the probable reduction in other waterway infrastructure costs that may arise from implementation of the changes, such as replacing labour intensive processes using traditional equipment with more modern equipment and automated processes.

Indirect benefits should include an estimation of costs that would otherwise have been incurred in the event of an incident/accident, based on the projected difference between the frequency of occurence of such incidents/accidents before and after implementation of any changes.