IALA Guideline

Maritime Service Portfolios:
digitising maritime services

Edition 1.0

Document date

Revisions to this IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

1.	INTRODUCTION	10

1.1.	General Description	10

1.2.	Purpose	10

1.3.	Implementation	10

1.3.1.	Change process	10

1.4.	Technical Issues	12

2.	Governing body, SERVICE PROVIDERS & STAKEHOLDERS	16

2.1.	Definitions	16

2.2.	Responsible service providers [to be decided later]	16

3.	Defined sea areas for MSP's	17

4.MARITIME SERVICES	18

4.1.	MS 1 VTS Information Service (INS)	18

4.1.1.	Submitting Organisation	18

4.1.2.	Description of the Digital Maritime Service	18

4.1.3.	Purpose	19

4.1.4.	Operational Approach	19

4.1.5.	User Needs	19

4.1.6.	Information to be Provided	20

4.1.7.	Associated Technical Services	20

4.1.8.	Relationship To Other DMSs	20

4.2.	MS 2 Navigational Assistance Service (NAS)	22

4.2.1.	Submitting Organisation	22

4.2.2.	Description of the Digital Maritime Service	22

4.2.3.	Purpose	22

4.2.4.	Operational Approach	23

4.2.5.	User Needs	23

4.2.6.	Information to be provided	24

4.2.7.	Associated Technical Services	25

4.2.8.	Relation to other DMSs	25

4.3.	MS 3 Traffic Organization Service (TOS)	25

4.3.1.	Submitting Organisation	25

4.3.2.	Description of the Digital Maritime Service	25

4.3.3.	Purpose	27

4.3.4.	Operational Approach	27

4.3.5.	User Needs	27

4.3.6.	Information to be provided	28

4.3.7.	Associated Technical Services	28

4.3.8.	Relation to other MSPs	28

4.4.	MS 4 Local Port Service (LPS)	29

4.4.1.	Submitting Organisation	29

4.4.2.	Description of Digital Maritime Service	29

4.4.3.	Purpose	29

4.4.4.	Operational Approach	29

4.4.5.	User Needs	29

4.4.6.	Information to be Provided	29

4.4.7.	Associated Technical Services	29

4.4.8.	Relation to other Digital Maritime Services	30

4.5.	MS 5 Maritime Safety Information service (MSI)	31

4.5.1.	Submitting Organisation	31

4.5.2.	Description of Digital Maritime Service	31

4.5.3.	Purpose	31

4.5.4.	Operational Approach	32

4.5.5.	User Needs	34

4.5.6.	Information to be Provided	34

4.5.7.	Associated Technical Services	34

4.5.8.	Relation to other Digital Maritime Services	34

4.6.	MS6 Pilotage service [IMPA]	35

4.6.1.	Submitting Organisation	35

4.6.2.	Description of Digital Maritime Service	35

4.6.3.	Purpose	35

4.6.4.	Operational Approach	35

4.6.5.	User Needs	35

4.6.6.	Information to be Provided	35

4.6.7.	Associated Technical Services	35

4.6.8.	Relation to other Digital Maritime Services	35

4.7.	MS 7 Tugs service	36

4.7.1.	Submitting Organisation	36

4.7.2.	Description of Digital Maritime Service	36

4.7.3.	Purpose	36

4.7.4.	Operational Approach	36

4.7.5.	User Needs	36

4.7.6.	Information to be Provided	36

4.7.7.	Associated Technical Services	36

4.7.8.	Relation to other Digital Maritime Services	36

4.8.	MS 8 Vessel shore reporting	37

4.8.1.	Submitting Organisation	37

4.8.2.	Description of Digital Maritime Service	37

4.8.3.	Purpose	37

4.8.4.	Operational Approach	37

4.8.5.	User Needs	37

4.8.6.	Information to be Provided	38

4.8.7.	Associated Technical Services	38

4.8.8.	Relation to other Digital Maritime Services	38

4.9.	MS 9 Telemedical Assistance Service (TMAS)h	38

4.9.1.	Submitting Organisation	38

4.9.2.	Description of Digital Maritime Service	38

4.9.3.	Purpose	38

4.9.4.	Operational Approach	38

4.9.5.	User Needs	39

4.9.6.	Information to be Provided	39

4.9.7.	Associated Technical Services	39

4.9.8.	Relation to other Digital Maritime Services	39

4.10.	MS 10 Maritime Assistance Service (MAS)	40

4.10.1.	Submitting Organisation	40

4.10.2.	Description of Digital Maritime Service	40

4.10.3.	Purpose	40

4.10.4.	Operational Approach	40

4.10.5.	User Needs	40

4.10.6.	Information to be Provided	40

4.10.7.	Associated Technical Services	40

4.10.8.	Relation to other Digital Maritime Services	40

4.11.	MS 11 Nautical Chart Service	41

4.11.1.	Submitting Organisation	41

4.11.2.	Description of Digital Maritime Service	41

4.11.3.	Purpose	41

4.11.4.	Operational Approach	42

4.11.5.	User Needs	42

4.11.6.	Information to be Provided	43

4.11.7.	Associated Technical Services	43

4.11.8.	Relation to other Digital Maritime Services	44

4.12.	MS 12 Nautical publications service	45

4.12.1.	Submitting Organisation	45

4.12.2.	Description of Digital Maritime Service	45

4.12.3.	Purpose	45

4.12.4.	Scope	46

4.12.5.	Objective	46

4.12.6.	Operational Approach	47

4.12.7.	User Needs	47

4.12.8.	Information to be Provided	48

4.12.9.	Associated Technical Services	48

4.12.10.	Relation to other Digital Maritime Services	48

4.13.	MS 13 Ice navigation service WMO	49

4.13.1.	Submitting Organisation	49

4.13.2.	Description of Digital Maritime Service	49

4.13.3.	Purpose	49

4.13.4.	Operational Approach	49

4.13.5.	User Needs	50

4.13.6.	Information to be Provided	54

4.13.7.	Associated Technical Services	54

4.13.8.	Relation to other Digital Maritime Services	54

4.14.	MS 14 Meteorological information service [WMO]	55

4.14.1.	Submitting Organisation	55

4.14.2.	Description of Digital Maritime Service	55

4.14.3.	Purpose	55

4.14.4.	Operational Approach	56

4.14.5.	User Needs	57

4.14.6.	Information to be Provided	59

4.14.7.	Associated Technical Services	59

4.14.8.	Relation to other Digital Maritime Services	59

4.15.	MS 15 Real-time hydrographic and environmental information services	60

4.15.1.	Submitting Organisation	60

4.15.2.	Description of Digital Maritime Service	60

4.15.3.	Purpose	60

4.15.4.	Operational Approach	61

4.15.5.	User Needs	61

4.15.6.	Information to be Provided	61

4.15.7.	Associated Technical Services	61

4.15.8.	Relation to other Digital Maritime Services	61

4.16.	MS 16 Search and Rescue (SAR) Service	62

4.16.1.	Submitting Organisation	62

4.16.2.	Description of Digital Maritime Service	62

4.16.3.	Purpose	62

4.16.4.	Operational Approach	62

4.16.5.	User Needs	62

4.16.6.	Information to be Provided	62

4.16.7.	Associated Technical Services	62

4.16.8.	Relation to other Digital Maritime Services	62

4.17.	MS 17 Aids to Navigation services (AtoN)	64

4.17.1.	Submitting Organisation	64

4.17.2.	Description of Digital Maritime Service	64

4.17.3.	Purpose	64

4.17.4.	Operational Approach	64

4.17.5.	User Needs	64

4.17.6.	Information to be Provided	64

4.17.7.	Associated Technical Services	64

4.17.8.	Relation to other Digital Maritime Services	64

4.18.	MS19 PNT and augmentation services	64

4.18.1.	Submitting Organisation	64

4.18.2.	Description of Digital Maritime Service	64

4.18.3.	Purpose	64

4.18.4.	Operational Approach	64

4.18.5.	User Needs	64

4.18.6.	Information to be Provided	64

4.18.7.	Associated Technical Services	65

4.18.8.	Relation to other Digital Maritime Services	65

6.	RELEVANT ASSOCIATED IMO GUIDELINES	72

6.1.	Guidelines on SQA and HCD	72

6.2.	Guidelines on Display of navigation information from communications	72

6.3.	Guidelines on test beds reporting	72

7.	LIST OF PUBLICATIONS THAT CAN BE DIGITAL	72

8.	ACRONYMS To be checked	72

ANNEX A	MS 1 INFORMATION SERVICE Template	74

8.1.	74

List of Tables

Table 1	Responsible Authorities	7

# INTRODUCTION

## General Description

When developing the International Maritime Organization (IMO) e-Navigation strategy to improve safety and efficiency of maritime transportation, it became clear that information services provided from coastal states to ships would be an essential part of this initiative. In order to adequately describe, structure and implement those services, IMO introduced the concept of “Marine Service Portfolios” (MSPs).

A “Maritime Service Portfolio (MSP)” defines and describes a set of operational and technical (digital) Maritime Services and the required level of service provided by a stakeholder in a given sea area, waterway or port, as appropriate. (NAV 57/6 para 23)

REMARK: Changed definition of MSP / MS need to be clarified.

IMO has identified a preliminary list of sixteen Maritime Services (MS). Essential services under the mandate of IALA such as Aids to Navigation (AtoNs) and Positioning, Navigation, and Timing (PNT) were not included in the IMO’s preliminary list but, have been added to these guidelines.

In parallel to the development of the MS content, the IMO is working on Guidelines for harmonized display of navigation information received via communication equipment. These Guidelines aim to develop standards and specifications to allow display of MS information received from a recognized and official means of communication on shipboard equipment. This interoperability will allow a national authority to select, among e-Navigation recognized means of communication, those adapted to its own situation and capacity.

## Purpose

These Guidelines describe the operational aspects regarding the IMO e-navigation solution on maritime information services provided from shore to ship and displayed on a graphical display onboard the ship.

International organizations have provided the information that belongs to their respective domains and this cooperation constitutes the basic foundation for further global technical harmonization.

These Guidelines are mainly for providers of Maritime Services as defined in MSPs and aim to clarify what is expected by the maritime community if a dedicated provider of such services is declaring the availability of an MS in their jurisdiction. It presents briefly the content of an MS, its purpose and operational requirements and the links to other MSs when applicable.

This overall approach will help maritime service providers to better understand the steps required to migrate from analogue communication systems to digital services. Technical specifications necessary for the harmonized implementation of those MSs are defined elsewhere through the respective competent bodies, and are only referenced in this document for easy access.

The maritime services described in these Guidelines are not limited to IALA’s domain, but also include other responsible international organizations. This coordination and common understanding will facilitate the implementation of maritime services.

## Implementation

Implementation of Maritime Services will be achieved according to a coastal state capacity. Therefore, one should expect a gradual transition towards digital communication services.

### Change process

To be successful, the transformation of existing services into digital services will require proper and realistic planning and will include several different processes (Figure 1).

The first step is an assessment of the current situation and communication capacities as well as a clear definition of what the future digital situation may require. This assessment includes a review of the organization processes, systems, operational procedures and outputs for both situations: Current and Future.

In order for this assessment to serve as the basis for further analysis, it would need to consider different categories of information:

- technical

- procedural

- existing experience, and

- guidelines.

The analysis phase will document all relevant information in accordance with the initial assessment of the expected outcomes. Both a Change Process Plan as well as a Change Process Timeline will have to be created. Once these two plans are available, the execution can start in accordance to the plans.

Figure 1

For example: If a certain Competent Authority decides to change or develop a service to digital information (implementing an MS), it should take into account the following steps:

Assess the services being delivered by traditional means and the actual delivery platforms (e.g. VHF network);

Identify the services under own responsibility and the eventual need for new delivery platforms.

Identify the services to be delivered in the new format (MS) and the timeline for the implementation

Identify the providers of information for the selected MS

Establish a common workable platform between all providers. This platform must allow interoperability between the different providers and systems. A working protocol must also be established to guarantee the responsibility levels and the flow of information.

Coordinate the intended work with MS users and organizations creating tools for using MS outputs, to ensure usability by intended end users.

Implement the onshore structures (infrastructures, hardware and software) necessary for the delivery of MS portfolio.

Guarantee a testing phase to assess the resilience of the system.  When the Competent Authority determines that the service is reliable, it can be made fully operational and used as primary means for the delivery of that information to mariners. However, due to the existence of different levels of equipment onboard vessels, both analogue and digital services must be maintained as long as required.

During the transformation phase, it is necessary to continuously evaluate each process  to improve and optimize the service, and also to identify gaps and implement the required changes. This continuous process must be complemented by a permanent monitoring of the system to detect any communication failures, as well as the capacity to collect feedback from mariners and other stakeholders.

Figure 2

## Technical Issues

According to the situation, information provided electronically (digital format) could complement and/or replace voice communication. However, critical and urgent situations would continue to be best addressed through radio and voice communication, which ensures that the distress call has been received and that a proper response is triggered. For non-critical situations, information could be provided in a digital format but care should be taken to ensure that all ships have the capacity to properly receive and display the information.

Given that neither all shore-based authorities nor all ships will have modernized their equipment to the digital format level at the same time, a transition period from the current provision of Information Services to the Digital Service format must be envisaged. This transition period will also allow both ship and shore authorities to become familiar with the new technology and have confidence in it.

It is also important to consider that the provision of Maritime Services in many areas is not limited to SOLAS ships but also includes other type of vessels (domestic, fishing, pleasure craft, etc.). These vessels will need a similar period of transition, which may be longer according to the financial investment that might be required. Thus, the transition period will require that both methods of communication, analogue and digital, be maintained in parallel to ensure safety of navigation, until the digital format proves to be efficient and reliable for all situations (figure x).

Figure 3 – Illustration of the provision of information services over time.

Each existing MS and future new MS should make use of the methodology in Annex A for mapping and identifying what would be digital information in the future. This form will also identify existing product specifications in the s-100 registry and the GAPs that has to developed in addition.

Also, many services are produced by different national authorities (see figure 4 below) and the conversion of information such as ice, meteorological and hydrographic data to the digital format will gradually change the role of communication centres, such as VTS centres, with respect to this data. In some cases, the digitalization process will allow transmission of the information directly to mariners without passing through a VTS centre. However, these situations will require that VTS centres have constant access to this data in order to monitor their status and provide the information to ships experiencing electronic problems.

Currently, there are situations where the exchange of information between shore authorities and ships is done electronically.

For example:

Provision of VTS services such as INS can be done electronically without radio voice communication;

Meteorological, hydrographical and other types of environmental data can be broadcasted directly to ships through AIS Application Specific Message (AIS ASM), without passing through a VTS centre; and

The content of VHF communications can be transmitted electronically and be displayed as a text in parallel to voice communications.

Figure 4: Illustration of the historical communication process between VTS onshore and ship.

Over the last several decades, the common practice has been to “push” information to ships. This allowed VTS to have a knowledge of the status of this information and to trigger a notification procedure whenever necessary. Digitalization of information introduces new ways to communicate with ships and although it will still be possible to push it through communication systems such as AIS and VDES, the capacity of ships  to “pull” it directly from the internet through web services will also exist. In both cases, the assistance of VTS may not be required. This may decrease the situational awareness  of VTS as they may not know whether or not information has been received by ships. In order to keep pace with the digital transformation of information service, VTS will need to have access and monitor the status of information transmitted directly to ships. Hence, if there is ever a technological failure and information services cannot be digitally communicated to ships, VTS will be in a position to notify mariners as well as the service producer, or to provide the information directly to the ship.

Even though information could be made accessible directly to ships in the near future, VTS will remain the primary contact with ships for urgent and important messages, including as a back-up for electronic failure. Figure 5 illustrates some of the current and future possibilities with respect to shore  ship communications.

Figure 5: Illustration of some current and future communication processes between shore and ship.

Implementation of MS1 by a coastal state will require  strong coordination with all other MS producers to clarify the means of communication that wil be used to transmit their own service information to mariners, and to determine how VTS will be kept informed. Development of national procedures detailing how shore  ship communications will proceed will be essential to maintain safety to navigation.

# Governing body, SERVICE PROVIDERS & STAKEHOLDERS

## Definitions

- MSP governing body, which defines and maintains the overall architecture of the MSPs, endorses the definition and scope of individual Maritime Services, ensures interoperability and consistency, etc. (the IMO/IHO HGDM could be the initial basis for defining further that structure);

- Service definition owner which proposes the definition to the governing body and then implements the agreed definition through technical specifications;

- Service provider responsible for delivering an operational service according to the relevant specifications;

- Service producer refers to a national authority responsible for the collection or creation of an information or data.

- Users refer to all type of mariners who will receive the information transmitted by the service provider. In some cases (i.e. MSI service), there may be a need to distinguish between the provider of the information content (i.e. a NAVAREA coordinator) and the provider of the communication infrastructure/service (i.e. SafetyNET).

- Technical service specification owner  refers to the body responsible for developing and maintaining the technical specification(s) of a service, based on the corresponding service definitions [by way of example: for VTS Information Service, technical service specification owners could be IALA and the IHO]

## Responsible service providers

In each country there will be authorities responsible for providing information services.  The table below offers examples of authorities responsible for each maritime service., This example may differ according to a country’s internal organisation .

Responsible authorities may require service providers to deliver the operational service.

Responsible Authorities

| MS No | Identified Services | Identified Responsible Service Provider |
| --- | --- | --- |
| 1 | VTS Information Service (INS) | VTS Authority |
| 2 | Navigational Assistance Service (NAS) | VTS Authority |
| 3 | Traffic Organisation Service (TOS) | VTS Authority |
| 4 | Local Port Service (LPS) | Local Port/Harbour Authority |
| 5 | Maritime Safety Information (MSI) Service | National Competent Authority |
| 6 | Pilotage service | Pilotage Authority/Pilot Organization |
| 7 | Tug Service | National Competent Authority; Local Port/Harbour Authority |
| 8 | Vessel Shore Reporting | National Competent Authority and appointed service providers |
| 9 | Telemedical Assistance Service (TMAS) | National health organization / dedicated health organization |
| 10 | Maritime Assistance Service (MAS) | Coastal/Port Authority / Organization |
| 11 | Nautical Chart Service | National Hydrographic Authority / Organization |
| 12 | Nautical Publications service | National Hydrographic Authority / Organization |
| 13 | Ice Navigation Service | National Competent Authority Organization |
| 14 | Meteorological Information service | National Meteorological Authority Public Institutions |
| 15 | Real time hydrographic and environmental information service | National Hydrographic and Meteorological Authorities |
| 16 | Search and Rescue Service | SAR Authorities |

# Defined areas for A PORTOFOLIO OF MS’s

Maritime Services will be provided depending on the service type and the mandate that a service provider has for its provision in different geographic areas.  Each service provider should ensure that the service area is clearly defined and publicised.

# Flow chart of developing a new Maritime Service

The schedule of the flow chart below reflects a recommended process of steps to be     followed, with the following clarification

These descriptions of maritime services, using the templates, should contain references to relevant international Standards, Recommendations and Guidelines, which may contain criteria for the eventual implementation of these maritime services as well as identified user requirements for data and information;

The domain coordinating bodies will submit the descriptions of maritime services, using the template in Annex xx  to the Organization as the recipient (3), using the IMO Secretariat as a mailbox for automated registration and forwarding to the right IMO body.

The assigned IMO forum (eg HGDM) will study and evaluate the submitted descriptions of maritime services, using the templates, and identify potential harmonized data models (5) and finally report to NCSR (6) with a request for consideration (7) to incorporate, where appropriate, an approved description of a maritime service, using the template, into a relevant IMO instrument (e.g. Resolution, Guideline or Circular) (8);

Once the IMO instrument has been finalized and consequently, in accordance with IMO procedures, be approved by MSC, the Organization can provide guidance to the member states and the domain organizers (relevant NGO's and IGO's) to their members (9) on how to implement maritime services.

# MARITIME SERVICES

## MS 1 VTS Information Service (INS)

### Submitting Organisation

IALA

### Description of the maritime service

IALA Guideline 1089 on Provision of Vessel Traffic Services (INS, TOS, NAS) gives guidance on the delivery of the three different types of services provided by a VTS: Information Service (INS), Traffic Organization Service (TOS) and Navigational Assistance Service (NAS).

An information service provided by a VTS is defined by IMO as “a service to ensure that essential information becomes available in time for on-board navigational decision-making.”

“The information service is provided by broadcasting information at fixed times and intervals or when deemed necessary by the VTS or at the request of a vessel, and may include for example reports on the position, identity and intentions of other traffic, waterway conditions, weather, hazards, or any other factors that may influence the vessel's transit.” (IMO Res A.857(20))

Examples of the types of information that may be provided by the VTS operating an Information Service (IALA Guideline 1089)

| Information related to: | Examples: |
| --- | --- |
| Navigational situations (including traffic and route information) | Position, identity, destination of vessels and the intention of other traffic Amendments and changes in promulgated information concerning the VTS area such as boundaries, procedures, radio frequencies, reporting points, the mandatory reporting of movements Limited maneuverability that may impose restrictions on the navigation of other vessels, or any other potential hindrances Suspension or change of routes, etc. |
| Navigational warnings | Dangerous wrecks, obstacles not otherwise promulgated, diving operations, vessels not under command, etc. |
| Meteorology | Information that will include the speed and direction of the prevailing wind, direction and height of the waves, visibility, atmospheric pressure, the formation of ice, etc. |
| Meteorological warnings | Gale, storm, tsunami, restricted visibility, etc. |
| Hydrography | Information that will include factors such as the stability of the seabed, sea depth, the accuracy of surveys, tidal ranges, tidal streams, prevailing currents and swell, etc. |
| Electronic navigational aids | The availability of electronic navigational aids such as: GNSS, Loran, DGPS, AIS, RACON, etc. |
| Other information | Port information, pilot or tug request, cargo information, health condition, PSC, ISPS, etc. |

### Purpose

The purpose of MS 1 is to provide data in a digital format to support VTS Information Service (INS) and to create the means to reduce administrative burden and information overload, reduce miscommunication due to external interference, simplify work procedures, promote sustainable shipping, and increase navigational safety.

Information provided in a digital format could complement and/or replace verbal/voice communication. The steps to achieve this transition to digital information exchange may vary in different areas and for different types of vessels. Details about digital information exchange should be published by the VTS authority.

### Operational approach

The digitalisation of information will diversify the communication means between shore authorities and vessels and will affect VTS procedures regarding information provision.

Not all vessels are capable of receiving information in digital format; provisions should therefore be made to ensure that less capable vessels are receiving the information they require.  VTS should remain the primary contact with vessels for urgent and important messages, and will ensure communications with mariners.

### User needs

IMO resolution A.857(20) contains examples of information that can be provided to vessels.

The use case below is based on the information from table 1.

#### Use Case - Vessel Arrival

[Before or upon arrival in the VTS area, a data collection system on board sends all details regarding the arrival via relevant infrastructure to the VTS. The VTS collects the vessel’s data directly into its system, and automatically updates the vessel’s pre-registered data. Both vessel and VTS use chart systems as a graphic interface to present details that are of interest to the voyage.

The example is generic and intended for description purposes only. Actions and template categories may differ for different countries. Information exchange can be in real time instead of at specific times as indicated in the table. Content in the column named “Template Info (technical)” is pending submissions from relevant stakeholders.

The categories of services and the associated details are listed in Appendix 1, MS1 Information Service template.]

| Time | Automated Vessel Action | Automated VTS Action | Info category in appendix |
| --- | --- | --- | --- |
| 01:00 | Provides pre-arrival info | Replying with information on weather | Environmental |
| 02:00 | Enters VTS Area, provides sailing route | Traffic information to vessel | Traffic and Route information |
| 02:30 | Passes reporting point line | Provides information on current, wave height, etc. | Hydrographical information |
| 03:00 | Requires port information | Provides quay details | Traffic and Route information |
| 03:30 | Passes second reporting point | Provides operational information on AtoNs | Navigation Hazards |
| 04:00 | Vessel along side | Gives information on wind speeds, visibility | Environmental |

### Information to be provided

[See Appendix 1, MS 1 VTS Information Service template.]

### Associated technical services

REMARK: Living list, the herein outlined technical services are according to the definition of IALA G1128

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |
| Voyage Information Service | urn:mrn:stm:service:specification:sma:vis | The service supports exchange of voyage plans, text messages and area messages. |  | IEC |
| Weather Service |  |  |  |  |
| ENSI Voyage Reporting Service | urn:mrn:mcp:service:specification:fta:ENSI-VRS | The Service provides route validation for ships and facilitate sharing of SRS reports and Voyage information to shore centres. |  |  |
| Routing information | urn:mrn:iho | S-127 |  | IHO |
| Currents Service | urn:mrn:iho | S-101 |  | IHO |
| Wave height service |  |  |  |  |
| Port information (harbour charting information) | urn:mrn:iho | S-101 |  | IHO |
| Port information (harbour services information) |  |  |  | IHMA |
| AtoN Information | urn:mrn:iala |  |  | IALA |

[To be filled later

NOTE! Appendix 1 could be complimented with required information regarding this table]

### Relationship to other MSs

MS 1 has a relationship with other MSs where it affects the VTS:

Examples may be different depending on the coastal state arrangements.

| Description | Examples of data that could be of interest for MS 1 |
| --- | --- |
| MS 1 VTS INS | [See Appendix 1, MS 1, Information Service Template] |
| MS 2 VTS NAS | [See Appendix 2, MS 2, Navigation Assistance Service Template] |
| MS 3 VTS TOS | [See Appendix 3, MS 3, Traffic Organization Service Template] |
| MS 4 Local Port Service | Delays, obstruction, cargo operations, port availability and anchorage area in the port, ISPS state, Marsec level |
| MS 5 Maritime Safety Information | All information depending on structure of MSI |
| MS 6 Pilotage Service | Pilot orders and updates |
| MS 7 Tug Service | Tug order and updates |
| MS 8 Vessel Shore Reporting | Notification of arrival, dangerous cargo etc. |
| MS 9 Telemedical | Delays |
| MS 10 Maritime Assistance Service | Notifications, routing, places of refuge |
| MS 11 Nautical Chart Service | Local Area updates, chart updates |
| MS 12 Nautical Publication Service | Updates to publication |
| MS 13 Ice Navigation Service | Ice routes, ice conditions, ice breaking assistance |
| MS 14 Meteorological Service |  |
| MS 15 Real Time Hydrographic and Environmental Information Service | Horizontal and vertical Tidal information in VTS area, available water column |
| MS 16 Search and Rescue service | Search pattern and vessel of opportunity |

## MS 2 Navigational Assistance Service (NAS)

### Submitting Organisation

IALA

### Description of the maritime service

IALA Guideline 1089 on Provision of Vessel Traffic Services (INS, TOS, NAS) gives guidance on the delivery of the three different types of services provided by a VTS Information Service (INS), Traffic Organization Service (TOS) and Navigational Assistance Service (NAS).

Navigational Assistance Service operated by VTS is defined by IMO as “a service to assist on-board navigational decision-making and to monitor its effects.”

“The navigational assistance service is especially important in difficult navigational or meteorological circumstances or in case of defects or deficiencies. This service is normally rendered at the request of a vessel or by the VTS when deemed necessary.” (IMO Res.A857(20))

Examples of the types of information that may be provided by a VTS operating a Navigational Assistance Service (IALA Guideline 1089)

| Information related to NAS | Examples |
| --- | --- |
| Request and identification | availability of NAS, start and end of NAS; request for vessel identification such as position, course made good and speed over the ground; status of vessel's equipment; etc. |
| Navigational information (including position and course information) | Examples provided to an individual vessel: provide range and bearing from fixed objects, fairway/channel or way‐points; proximity to navigational hazards, etc. provide information related to navigating into a channel/fairway/lane (i.e. track is parallel/diverging/converging with/from/to reference line); etc. |
| Advice (or instruction) | advise (or instruct) a vessel to alter the course, speed; advise (or instruct) to keep clear from area/position, close up/drop back on/from vessels; etc. |
| Warning | Diverging from the recommended track towards dangerous wrecks, obstacles not otherwise promulgated; diving operations; vessels not under command; etc. |

### Purpose

The purpose of MS 2 is to provide data in a digital format to support Navigational Assistance Service (NAS) and to create means to reduce administrative burden and information overload, reduce the risk for miscommunication due to external interference, simplify work procedures, promote sustainable shipping, and increase navigational safety.

Information provided in a digital format could complement and/or replace verbal/voice communication. The steps to achieve this transition to digital information exchange may vary in different areas and for different types of vessels. Details about digital information exchange should be published by the VTS authority.

### Operational approach

All information related to MS 2 Navigational Assistance Service should be delivered only by VTS authorities.

VTS should remain the primary contact with vessels for urgent and important messages necessary for the on board decision making.

Information provided digitally could complement voice communications in time critical situations and in addition partly replace voice communications in non-time critical situations.

Note: Example of time critical situation:

Risk of grounding/striking/collision. In addition to voice communications, the vessel can be provided with an electronic route recommendation.

Note: Example of non-time critical situation:

Assist a vessel to an anchoring position by providing the vessel with an electronic route recommendation without voice communications.

- The identity of the vessel receiving Navigational Assistance Service should be assured.  Other items listed in the IALA Guideline 1089 On Provision of Vessel Traffic Service (Appendix B) should also be taken into consideration for digital transmission of information.

- All information related to this service should be displayed in real time. Measures should be taken to ensure that the information is received and acknowledged.

### User needs

The use case below are based on the information from table 2.

The use cases are generic and intended for description purposes only. Actions and template categories may differ for different countries. Content in the column named “Template Info (technical)” is pending submissions from relevant stakeholders.

For example:

Recommended route can be send digitally to a vessel.

Pre-arrival reporting can be done digitally without voice communication for update of route of voyage plan in order to avoid collisions, groundings and strikings and assist in safe navigation.

The content of the voice communication can be provided digitally and be displayed as text in parallel / in addition to voice communication.

#### Use Case Vessel deviates from planned route

[Vessel approaches VTS area according to voyage plan sent to VTS. The route is displayed in the VTS application and vessels position is automatically compared to the planned route. The system alerts the VTS operator, who then confirms that the vessel has deviated from its route. The VTS operator informs, warns and if necessary instructs / advises the vessel to change course via voice communication. Navigational assistance information is also presented on the vessel’s own navigation system. The VTS operator ensures that the vessel has changed course according to the solution. The VTS application continues to monitor the vessel’s voyage. It will alert the VTS operator if new deviation occurs.

The categories of services and the associated details are listed in Appendix 2, MS2 Navigational Assistance Service template.

The example is generic and intended for description purposes only. Actions and template categories may differ for different countries. Content in the column named “Template Info (technical)” is pending submissions from relevant stakeholders. ]

| Time | Vessel Action | VTS Action | Info category in appendix |
| --- | --- | --- | --- |
| 01:00 | Approaches VTS area | Receive voyage plan and monitor vessels progress | Traffic and Route Information |
| 01:30 | Deviates from her route | Informs, warns and advises / instructs the vessel to change course/speed | Navigational advice |
| 01:35 | Changes course | Verifies that vessel has changed course and is no longer in danger |  |

#### Use Case – assistance to a vessel to an anchoring position

[Vessel is inside VTS area and needs to stop for engine repair. Vessel asks for a safe anchorage position. VTS provides anchoring position. Vessel takes direct course to the anchoring position, over a shallow area. VTS gives warning to the vessel and provides a safe route to anchorage position.

After the vessel has anchored, high winds develop causing the vessel to drag anchor. The VTS operator monitoring the anchorage receives an alarm and warns the vessel through automated digital alert and through voice communication.]

| Time | Vessel Action | VTS Action | Info category in appendix |
| --- | --- | --- | --- |
| 00:00 | Ask for anchorage position | Provides position | Navigation Information |
| 00:10 | Takes course to the position | Warning: You are running into danger - shallow waters... Provides vessel safe route | Navigational warning / Advice |
| 00:20 | Follows route provided by VTS to the anchorage | Monitors | Navigational Information |
| 05:00 | Drags anchor | Warning: You are dragging anchor | Navigational warning |
| 05:30 | Repositions | Monitors | Navigational Information |

### information to be provided

[See Appendix 2, MS 2 Navigational Assistance Service template]

### Associated technical services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |
| Voyage Information Service | urn:mrn:stm:service:specification:sma:vis | The service supports exchange of voyage plans, text messages and area messages. |  | IEC |
| Route information |  | S-421 |  | IEC |

### Relationship to other MSs

MS 2 has a relationship to other MSs where it affects VTS:

Examples may be different depending on the coastal state arrangements.

| Description | Examples of data that could be of interest for MS 2 |
| --- | --- |
| MS 1 VTS INS | See Appendix 1, MS 1, Information Service Template |
| MS 2 VTS NAS | See Appendix 2, MS 2, Navigation Assistance Service Template |
| MS 3 VTS TOS | See Appendix 3, MS 3, Traffic Organisation Service Template |
| MS 4 Local Port Service | Delays, obstruction, cargo operations, port availability and anchorage area in the port, ISPS state, Marsec level |
| MS 5 Maritime Safety Information | All information depending on structure of MSI |
| MS 6 Pilotage Service | Pilot orders and updates |
| MS 7 Tug Service | Tug order and updates |
| MS 8 Vessel Shore Reporting | Notification of arrival, dangerous cargo etc. |
| MS 9 Telemedical | Delays |
| MS 10 Maritime Assistance Service | Notifications, routing, places of refuge |
| MS 11 Nautical Chart Service | Local Area updates, chart updates |
| MS 12 Nautical Publication Service | Updates to publication |
| MS 13 Ice Navigation Service | Ice routes, ice conditions, ice breaking assistance |
| MS 14 Meteorological Service | Weather information |
| MS 15 Real Time Hydrographic and Environmental Information Service | Horizontal and vertical tidal information in VTS area, available water column |
| MS 16 Search and Rescue service | Search pattern and vessels of opportunity |

## MS 3 Traffic Organization Service (TOS)

### Submitting Organisation

IALA VTS Committee

### Description of the maritime service

IALA Guideline 1089 on Provision of Vessel Traffic Services (INS, TOS, NAS) gives guidance on the delivery of the three different types of services provided by a VTS, Information Service (INS), Traffic Organization Service (TOS) and Navigational Assistance Service (NAS).

Traffic Organization Service operated by VTS is defined by IMO as “a service to prevent the development of dangerous maritime traffic situations and to provide for the safe and efficient movement of vessel traffic within the VTS area.”

“The traffic organization service concerns the operational management of traffic and the forward planning of vessels movements to prevent congestion and dangerous situations, and is particularly relevant in times of high traffic density or when the movement of special transports may affect the flow of other traffic. The service may also include establishing and operating a system of traffic clearances or VTS sailing plans or both in relation to priority of movements, allocation of space, mandatory reporting of movements in the VTS area, routes to be followed, speed limits to be observed or other appropriate measures which are considered necessary by the VTS authority.” (IMO Res. A.857(20))

Examples of types of information that may be provided by the VTS within a Traffic Organization Service (IALA Guideline 1089)

| Information related to: | Examples |
| --- | --- |
| Traffic clearance | Give authorization under conditional circumstances to a vessel when: prior to or entering a VTS area; departing from a berth or an anchorage position within a VTS area; entering into a fairway within a VTS area; or prior to commencing a manoeuvre that may be detrimental to safe navigation. Examples of conditions: a VTS sailing plan before entering a VTS area; lock and bridge passage planning; report position at determined reporting point/line/pilot station; use a second fairway in case of bad visibility/weather; use a tug boat in case of strong wind; dredging or compass swing in confined waterway. |
| Anchorage | Examples of anchorage situations: organizing the movements to/from an anchorage position/area; assignment of an anchorage position; assisting vessels into anchorage position. |
| Enforcement | Examples of enforcement: speed limits; adherence to rules regarding traffic routeing measures; pilotage requirements; other traffic regulations and possibly local by‐laws |
| Waterway (sea, channels and fairway) management | Examples of management measures: the use of one‐way traffic as an alternative of two way traffic, depending on the dimensions of vessel or the weather conditions; organizing other traffic when a vessel has passed point of no return; slot management to allocate vessels in a time window; organizing the traffic concerning vessel dimensions in comparison to fairway restrictions; instruct vessels when overtaking is not permitted; establish and organise vessel safety zones in case of particular operations; establish and organise exclusion zones; instruct vessels to keep clear from special areas/positions; organizing the traffic with regards to meteorological, hydrographical or other restrictions such as visibility, wind speed, current, sea state and under keel clearance. |

### Purpose

The purpose of MS 3 is to provide data in a digital format to support Traffic Organization Service (TOS) and to create means to reduce administrative burden and information overload, reduce the risk for miscommunication due to external interference, simplify work procedures, promote sustainable shipping, and increase navigational safety.

Information provided in a digital format could complement and/or replace verbal/voice communication. The steps to achieve this transition to digital information exchange may vary in different areas and for different types of vessels. Details about digital information exchange should be published by the VTS authority.

### Operational approach

A Traffic Organization Service should be responsible for separating traffic in the interest of safety and efficiency. This separation could be defined in space, time and/or distance.

Enforcement may also be carried out within a Traffic Organization Service where the VTS should monitor adherence to applicable rules and regulations and to take appropriate action where required and within the authority of the VTS (IALA Guideline 1089 On Provision of Vessel Traffic Services).

Digital communication may apply to elements of the Traffic Organization Service that are not time critical situations.

Examples:

Slot management: provide vessels digitally with priority of arrival and distance between two vessels

Traffic clearance: provide vessels digitally with permission to proceed, impose conditions or deny entry

Route information: provide vessels digitally with recommended route information

Traffic information: vessel provide VTS digitally their intentions, such as overtaking of another vessel

Information regarding restricted or no go area: the content (draft, closed fairway/port/quay etc.) can be provided digitally to vessels without using voice communication

All information provided digitally can complement and/or replace verbal/voice communication.

### User needs

The use cases below are based on the information from table 3.

The use cases are generic and are intended for description purposes only. Actions and template categories may differ for different countries. Content in the column named “Template Info (technical)” is pending submissions from relevant stakeholders.

#### Use case – vessel leaves quay

[When the vessel is ready to sail it sends its planned time of departure digitally to VTS where it is presented in the VTS application. The application alerts operator on upcoming traffic conflicts and advises on a solution, which is assessed by VTS. The VTS operator takes action and instructs vessel digitally to delay planned departure by thirty minutes. The instructions are graphically displayed in applications, acknowledged by the vessel and VTS operator provides delayed departure information to other traffic via digital and/or verbal means for vessels not able to receive information digitally.

Detailed information can be found in the Appendix 3, MS 3, Traffic Organization Service Template.]

| Time | Vessel Action | VTS Action | Info category in appendix |
| --- | --- | --- | --- |
| 00:00 | Sends ETD to VTS and requests permission to leave quay (in some cases additional communication by voice could be required). | Deny clearance. Give permission to leave in thirty minutes | Waterway management |
| 00:01 | The vessel acknowledges revised ETD | VTS receives acknowledgement and informs other traffic of revised ETD | Waterway management |
| 00:30 | The vessel informs VTS of intended departure | VTS issues traffic clearance with any appropriate conditions attached |  |

#### Use case – vessel transiting protected area

[As a vessels is approaching a marine mammal protected area where a speed restriction may or may not be active depending on the presence of marine mammals. When marine mammals are present, vessels are advised digitally that a speed restriction is in effect. For example, the vessel receives a digital message and the extent of the area is displayed in the navigational systems.]

| Time | Vessel Action | VTS Action | Info category in appendix |
| --- | --- | --- | --- |
| 00:00 | Sailing in the vicinity of marine mammal protected area | Receives information confirming presence of marine mammals and activates the speed restriction area. | Enforcement |
| 00:30 | Approaches a marine mammal protected area | Sends automated digital message regarding active speed restriction and the area (text and visual) | Waterway management |
| 02:00 | Entering the marine mammal protected area | Confirm that speed restrictions are in force | Waterway management |
| 02:10 | Exceeding speed limit | Send automated digital alert message requesting vessel conform to speed limit | Enforcement |

### information to be provided

[See Appendix 3, MS 3, Traffic Organisation Service template.]

### Associated technical services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other MSs

MS3 has a relationship with other MSs where it affects the VTS:

Examples may be different depending on the coastal state arrangements.

| Description | Examples of data that could be of interest for MS 3 |
| --- | --- |
| MS 1 VTS INS | See Appendix 1, MS 1, Information Service Template |
| MS 2 VTS NAS | See Appendix 2, MS 2, Navigation Assistance Service Template |
| MS 3 VTS TOS | See Appendix 3, MS 3, Traffic Organisation Service Template |
| MS 4 Local Port Service | Delays, obstruction, cargo operations, port availability and anchorage area in the port, ISPS state, Marsec level |
| MS 5 Maritime Safety Information | All information depending on structure of MSI |
| MS 6 Pilotage Service | Pilot orders and updates |
| MS 7 Tug Service | Tug order and updates |
| MS 8 Vessel Shore Reporting | Notification of arrival, dangerous cargo etc. |
| MS 9 Telemedical | Delays |
| MS 10 Maritime Assistance Service | Notifications, routing, places of refuge |
| MS 11 Nautical Chart Service | Local Area updates, chart updates |
| MS 12 Nautical Publication Service | Updates to publication |
| MS 13 Ice Navigation Service | Ice routes, ice conditions, ice breaking assistance |
| MS 14 Meteorological Service | Weather information |
| MS 15 Real Time Hydrographic and Environmental Information Service | Horizontal and vertical Tidal information in VTS area, available water column |
| MS 16 Search and Rescue service | Search pattern and vessel of opportunity |

## MS 4 Local Port Service (LPS)

### Submitting Organisation

International Harbour Master Association (IHMA)

### Description of the Maritime Service

A port authority is a hub from which several types of maritime and terrestrial services are provided. A port is generally not the final destination of goods, and so it needs to link to other modes of transportation (rail, truck, smaller ships). This ensures the fluid movement of goods to market. Port infrastructure and facilities are important links in the chain of intermodal transportation. To be capable of offering timely services, a port needs information on a ship’s transit from departure until time of arrival.

The size of a port and its status (local, regional, national or international) will influence the type and amount of services provided. These services include both maritime and shore sectors.

Table X.X - Examples of port services

| Maritime Services | Shore Services |
| --- | --- |
| Notification/Documentation/Reporting | Berthage |
| Navigation (anchorage, pilotage, tug, etc.) | Cargo Operations |
| Safety & Security | Fresh Water/Electricity/Waste/Repairs/Medical facilities/Shipping Agent/etc. |
| Nautical information | Bunkering |
| Communication | Linesmen |
| Weather | Intermodal transportation |

The combined services contribute to safe navigation, protection of the environment and the efficient movement of goods and people.

Figure x. Brief summary of services that may be provided by a port.

### Purpose

The Local Port Service aims to facilitate the exchange of information and coordination among key stakeholders during each phase of a ship’s transit. To promote optimal coordination and efficient operations all along the transportation chain, the exchange of information must be done in a timely manner. This allows all actors involved in LPS to take specific actions at the right moment either to respond or to adapt to new situations, such as delay, premature departure/arrival, incident, etc.

All actors involved in a ship’s transit will benefit from a common information platform. Access to timely information at each phase of a transit will improve:

- Just in time arrival and departure

- Reduction in waiting time for berthing (anchorage)

- Reduction of fuel consumption and greenhouse gases

- Predictability of cargo delivery to market

- Work schedules of services

A common communication platform will facilitate the fluid movement of cargo and persons, and contribute to a better and more transparent decision-making process.

### Operational Approach

An efficient LPS must allow all key stakeholders to access information they need during a ship transit. Sharing a common operational picture will optimize the decision-making process of each actor and facilitate adjustment whenever a situation changes. A basic operational approach for LPS would be the development and utilization of a common electronic information platform.

This platform will require each actor to enter its own relevant data in a timely manner for the benefit of other actors (see Figure xx). For sensitive and/or confidential data, restrictions can be set to provide access only to actors that require the information.

Figure xx. Examples of actors and data exchange that can contribute to a common LPS platform. Status of actions taken by each actor is illustrated by the lights (green, yellow and red).

Apart from sensitive information, all other data should be accessible to all participants involved in the transportation chain to create a common operational picture. Given that input from one actor might be critical for others’ decisions and influence their inputs, a management process would need to keep the platform updated in real-time. This would inform actors of status, or give notification, if required data is omitted. The port authority, or another organization, could assume responsibility for monitoring the platform and the status of data entries.

Figure 6 Terms for Vertical Measurement of vessels, depths and elevations. Identified in the IHMA and UKHO port information project.

### User Needs

- Provision of LPS is designed to improve coordination and exchange of information among stakeholders involved in a ship’s transit. The nature of this information is variable and has several purposesincluding safety, efficiency, protection of environment, fluidity of goods to market, etc. As such, user needs cover both maritime and shore information from departure to the time of arrival.

- Table X.X – Examples of types of information that LPS can cover

| Information related to: | Examples |
| --- | --- |
| Arrival and Berthing | Examples of Arrival and Berthing information: - Port allocation - ETA / ATA - Time slot, with slot management to allocate ships in a time window; - Mooring information - Draft and limitations - Safety and limitations of traffic concerning vessel dimensions in comparison to fairway restrictions - Local warnings and restrictions |
| Port information | Examples of port information: - ISPS information - Water supply - Crane and other cargo handling arrangement - Waste handling - Fuel arrangement - Immigration - Custom clearance - Pilot information - VTS information - Reporting formalities |
| Anchorage | Examples of anchorage situations: - organizing the movements to/from an anchorage position/area; - assignment of an anchorage position; - assisting vessels into anchorage position. |
| Enforcement | Examples of enforcement: - speed limits; - adherence to rules regarding traffic routeing measures; - pilotage requirements; - other traffic regulations and possibly local by‐laws |

### Information to be Provided

See Annex 4, MS 4 Local Port Service template

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other Maritime Services

This may be different depending on the coastal state arrangements.

| Description | Examples of data that could be used in MS 4 |
| --- | --- |
| MS 1 VTS IS | VTS area, type of VTS service, contact to VTS, information from sensors, radar, AIS, met, CCTV |
| MS2 VTS NAS | Information about vessel undertaking NAS |
| MS3 VTS TOS | Information about timeslots / ETA, route advisories |
| MS 5 Maritime Safety Information | All safety-related information |
| MS 6 Pilotage Service | Local pilot boarding place and stations, pilot regulations, information from pilot booking system |
| MS 7 Tug Service | Tug operation, available tug capacity |
| MS 8 Vessel Shore reporting | ETA/ATA, cargo information, incidents reports, dangerous goods, waste, number of persons onboard |
| MS 9 Telemedical | Local address of medical centre, communication and capacity |
| MS 10 Maritime Assistance Service | Information about vessels requesting relevant MAS,incidents , notifications, places of refuge |
| MS 11 Nautical Chart Service | Chart update information, hinge resolution charts for the port, Notice to Mariners, accuracy coverage characteristics, contour lines |
| MS 12. Nautical Publications Service | Local descriptions and publications. |
| MS 13 Ice Navigation Service | Ice forecast, ice charts, icebreaker services |
| MS 14 Meteorological Service | All meteorological information |
| MS 15 Real Time Hydro and Information Service | Sea state, current, wave, moving sandbanks |
| MS 16 Search and Rescue Service | SAR situations and operations in local areas, search and rescue capacity, SAR communication capability |

## MS 5 Maritime Safety Information service (MSI)

### Submitting Organisation

IHO World-Wide Navigational Warning Service Sub Committee & WMO World-Wide Met-Ocean Information and Warning Service Committee

### Description of the maritime service

The MSI Service describes the provision of navigational and meteorological warnings, meteorological forecasts and other urgent safety-related messages broadcast to ships. The MSI Service is the internationally and nationally coordinated network of broadcasts containing vital urgent (?) information which is necessary for safe navigation, received in ships by equipment which automatically monitors the appropriate transmissions, displays information which is relevant to the ship and provides a print capability.

### Purpose

The purpose of the MSI Service is to provide the mariner with information related to navigational and meteorological warnings, meteorological forecasts and other urgent safety-related messages.

The provision of MSI makes available to mariners, prior to and during voyages, information that improves their situational awareness and assists with safety of navigation.

The promulgation of MSI is defined in IMO Resolution A.705(17) and it is further defined by Chapter IV in the International Convention for the Safety of Life at Sea, 1974 (SOLAS Convention), as amended, as part of the “The Global Maritime Distress and Safety System (GMDSS)”.

SOLAS Chapter V, regulations 4 through 7 governs the contracting government’s responsibilities with regards to providing MSI.

The Revised Joint IMO/IHO/WMO Manual on MSI, Publication S-53 (the Joint Manual on MSI) describes the provision of the service and the receiving methods in more detail.

The delivery methods have been described by the International SafetyNET Manual MSC.1/Circ. 1364.

The roles and responsibilities of a METAREA Coordinator are defined in the IMO Resolution A.1051(27), and the provision of marine meteorological services is guided by WMO No.558 (Manual on Marine Meteorological Services) and WMO No.471 (Guide to Marine Meteorological Services).

Services that constitute the digital maritime services (MSP5) are currently provided in a fully electronic format and as such there is no requirement to transition from analogue to digital information provision. Additional analogue (voice) services do exist but there is no intent to transition these to digital services.

### Operational approach

The MSI Service, as defined in Resolution A.705(17), is “the internationally and nationally coordinated network of broadcasts containing information which is necessary for safe navigation, received in ships by equipment which automatically monitors the appropriate transmissions, displays information which is relevant to the ship and provides a print capability. This concept is illustrated in figure below.

Figure -1 The maritime safety information service of the Global Maritime Distress and Safety System  (Source: S-53)

Within GMDSS, Maritime Safety Information is promulgated to defined areas that are managed by area coordinators as illustrated in the figures below

Figure 0-2 NAVAREAs for coordinating and promulgating navigational warnings under the World-Wide Navigational Warning Service (Source: S-53)

Figure 3 METAREAs for coordinating and promulgating meteorological warnings and forecasts under the World-Wide Met-Ocean Information and Warnings Service (Source: S-53)

## MS6 Pilotage service [IMPA]

### Submitting Organisation

International Maritime Pilots Association

### Description of Digital Maritime Service

Ships proceeding or leaving a port or a specific area, should have easy access to information regarding the pilotage service provided. Information such as local regulations, contact, notices, means of boarding, boarding point, limitations or pilot booking procedure, could be accessible by electronic means, where available.

The information provided through this service is not piloting information as pilotage is a service physically performed onboard ships by duly qualified and certificated or licensed maritime pilots.

### Purpose

This service is limited to information provided to ships regarding the pilotage service in a given geographic area. It does not address the act of piloting, which is provided by a pilot on the bridge of a ship. The purpose of MS6 is to provide information related to the pilotage service when planning an operation before the pilot boards the vessel, by using modern technology and common standards.

### Operational Approach

Pilot organizations providing pilotage service in an area could provide information to ships about the pilotage service in a digital and easy accessible way. The information could be, as an example, portrayed as a layer on the ECDIS or in a graphical display. This information could include, for inbound ships, the location of the pilot station(s) or boarding point(s) in latitude/longitude or distance and bearing from a location, or marked by an aid to navigation. In addition, the transmitted information could include the VHF channels to contact the pilot or pilot boat. Typically, the Pilotage Service information will not be provided by the pilot, but rather by the pilot organization, because the pilot must be engaged in the actual performance of his/her pilotage duties.

Examples of information can be:

| Information related to: | Examples |
| --- | --- |
| General information | Examples of information: - Pilot requirements in the area - Local regulations - limitations - Requirements and procedures for ordering the pilot - Requirements and procedures for pilot boarding - Contact information to pilot station - Mandatory needs for tug assistance - Pilot boarding point |
| Operational information | Examples of information: - Contact to pilot boat, launch, helicopter - Position of pilot station, pilot boat - Required arrangements for pilot boarding - Boarding speed - Communication - Set up of ship’s radar, ECDIS and other equipment as requested for the pilot’s use. - Any other actions requested of the ship for the pilot’s benefit |

### User Needs

Ships are concerned by this service and need to know the pilot boarding position, the pilot request procedures, local and special regulations and the compulsory use of tugs.

### Information to be Provided

See section 6.4.

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other Maritime Services

MS 6 has relationships with other MSs where it affects the Pilot boarding operation and contribute to safe and efficient operations.

## MS 7 Tugs service

### Submitting Organisation

### Description of the Maritime Service

Tug services range from small vessels with limited capacity and service in ports and rivers to oceangoing vessels built for complex operations and salvage. This service contributes to the safety of navigation, protection of the environment, and efficiency of marine transportation by conducting different type of operations such as:

- transportation (personnel and staff from port to anchorage);

- ship assistance (ex: mooring);

- salvage (grounded ships or structures);

- shore;

- towage (harbour/ocean);

- escort;

- oil spill response.

The need of tug services differ from port to port, type of vessel and cargo. In some cases, information about a tug service capacity and/or availability may be difficult to obtain due to communication deficiencies. This maritime service is dedicated to improve information availability of tug services.

Tug service would encompass all kinds of tugs, such as

- conventional;

- azimuth stern drive;

- tractor;

- rotor.

| Information related to: | Examples information shared in a Tug service |
| --- | --- |
| Deep sea information | Examples of information: - Contact information for tug vessel / operator - Safety procedures and regulations - Available resources - Working hours |
| Local port or river information | Examples of information: - Contact information for tug vessel / operator - Mooring and berthing information - Available resources - Working hours |
| Tug information | Examples of tug information: - Type of tug - Capacity - Size - Assistance services - Response time - Contact information - Working hours |

Table xx – Examples of information that can be shared in a Tug service

### Purpose

Tug service aims to facilitate access to all necessary information required by ships heading to port, in order to optimize transit times and promote efficient movement of goods and persons by using modern technology and common standards.

Effective communications and exchange of information between relevant stakeholders would contribute to efficient tug service. Exchange of information electronically would significantly contribute to the improvement of this service.  For example, notifying a ship officer in advance about tug availability in port could lead the ship to adapt its speed accordingly.  In some cases this may prevent a requirement to anchor the ship. The types of information which can be exchanged include:

- ETA (request);

- confirmation requests;

- updates on transit status and tug availability;

- updates among stakeholders;

- standardized messages to overcome language barriers.

### Operational Approach

Tug service is a key element of the marine transportation chain and well-coordinated procedures and communication means should be in place to ensure fluid movement of ships.

Like the Local Port Service, utilization of a common platform to exchange information electronically and keep users updated on a regular basis about the status of operations would significantly improve this service, for both the ships’ operator and the tug owners. The tug service aims mainly to improve the communications involved in a ship request, rather than altering current operational procedures. Some of these communications may include:

- Ship’s size

- Number of tugs required

- Time the service is required

- Time the tug may be on site

- Estimated duration of operations

- End of operations.

- Access to this information electronically would enhance the awareness of a ship’s time-stamp.

- Figure xx. Example of an electronic communication platform for all actors involved in the tug service.

- Increased connectivity, through sharing of harmonized digital information regarding tug services in ports, rivers or deep-sea, will enhance efficiency through just-in-time services. It will also reduce human factor errors, such as language barriers or outdated information in analog publications, enhancing efficiency and access to information in a fast and easy-to-use-manner.

### User Needs

### Easy and timely access to tug service information is critical to ensure fluidity in the transportation chain. The main information required from this service are:

- Capacity

- Availability

- Time of response

- Status of operations

- Duration of operations

### In return, the tug services should be regularly updated on the ship’s ETA/ATA to plan its operations accordingly. In the event of an unanticipated change, the tug service and ship officers must be able to communicate easily with each other to keep both parties informed about the evolving situation and allow for proper decision-making. An easy communication link must be part of the user needs and this communication link would also benefit all other actors.

### Information to be Provided

See Annex 7, MS 7 Tug Service

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other Maritime Services

| Description | Examples of data that could be used in MS 7 |
| --- | --- |
| MS 1 VTS IS | VTS area, types of VTS service, VTS contact information, places of refuge, local regulations, limitation, visibility, information regarding traffic in the area |
| MS2 VTS NAS | Not relevant |
| MS3 VTS TOS | Traffic clearance, time slots |
| MS4 Local Port Service | Berthing information, time slots, security, local regulations, supply, assistance, port contact information |
| MS 5 Maritime Safety Information | Navigational warnings, meteorological information and other urgent safety-related information. |
| MS 6 Pilotage Service | Pilot regulations, contact information, request procedures |
| MS 8 Vessel Shore reporting | Vessel information, cargo information, crew information |
| MS 9 Telemedical | Not relevant |
| MS 10 Maritime Assistance Service | Contact information, places of refuge |
| MS 11 Nautical Chart Service | Charting information, chart updates |
| MS 12. Nautical Publications Service | Digital information from nautical publications that is relevant for the operations at hand |
| MS 13 Ice Navigation Service | Ice chart, ice conditions, information regarding icebreaker service / assistance, ice routes |
| MS 14 Meteorological Service | Information regarding weather in the area |
| MS 15 Real Time Hydro and Information Service | Information from real time sensors |
| MS 16 Search and Rescue Service | Salvage information, drifting parts, SAR areas, and rescue capabilities in the area |

## MS 8 Vessel shore reporting

### Submitting Organisation

### Description of Maritime Service

This service provides information from shore to ship about two different reporting regimes.

The first regime is Vessel Shore Reporting (VSR), implemented for pre-arrival reporting. This service can cover information and guidelines related to reporting formalities and instructions (when, what and how) for reporting to a specific port. In addition, this service can be extended to the full exchange of information required in a Single Window ship reporting system. The Vessel Shore Reporting (VSR) regime may contain the following elements:

- Marine security regulations

- Vessel Traffic Services zones regulations

- Customs and immigration regulations

- Port state regulations

- Health and veterinary regulations

- Environmental regulations

Many of these elements will be addressed by the IMO Facilitation Committee, which is developing the specifications for the Single Window Reporting System. This is linked to e-Navigation Strategy Implementation Plan Solution 2 - Means for standardized and automated reporting -.

IMO e-navigation solution 2, regarding automated ship reporting, is one of the most important solutions to reduce the mariners’ workload (amount of time spent on preparing and submitting reports to shore-based authorities). To achieve this, reports should be automatically generated as much as possible from on-board systems.

The second regime is linked to a transit in an area where a Ship Reporting System (SRS) is established.

In SRS areas, shore authorities can provide more automated and efficient reporting systems by using technology such as AIS and VDES, in combination with common data structures and product specifications.

In a ship reporting system (SRS), MSC 43(64) recalls that communication between a shore-based authority and a participating ship should be limited to information essential to achieve the objectives of the ship reporting system (SRS). The initial report required from a ship entering the system should generally be limited to:

- ship's name

- call sign

- IMO identification number if applicable, and

- position.

The existing resolution is based on Regulation 11 of SOLAS Chapter V as follows:

“Ship reporting systems contribute to safety of life at sea, safety and efficiency of navigation and/or protection of the marine environment. A ship reporting system, when adopted and implemented in accordance with the guidelines and criteria developed by the Organization pursuant to this regulation, shall be used by all ships, or certain categories of ships or ships carrying certain cargoes in accordance with the provisions of each system so adopted.

The Organization is recognized as the only international body for developing guidelines, criteria and regulations on an international level for ship reporting systems. Contracting Governments shall refer proposals for the adoption of ship reporting systems to the Organization. The Organization will collate and disseminate to Contracting Governments all relevant information with regard to any adopted ship reporting system.”

Although the regulation refers to safety of life at sea, safety and efficiency of navigation and/or protection of the marine environment, resolution MSC.43(64) states that other supplementary information may also be requested in the initial report, if justified to ensure the effective operation of the ship (MSC.43(64) Annex para 2.2.1.4).

This information may include the intended movement of the ship through the area covered by the reporting system and any operational defects or difficulties affecting the ship , as well as the general categories of any hazardous cargoes on board (MSC.43(64) Annex para 2.2.1.4)

A Vessel Shore Reporting (VSR) system normally includes a web solution and an onshore representative, or agency, for the vessel operator or owner.  The agency acts on behalf of the master or owner and provides information to relevant authorities.

The VSR as envisaged by e-navigation solution 2 aims to reduce time-consuming paperwork onboard.

Information required to be transmitted as part of a mandatory SRS is generally transmitted to the VTS centre, or  the relevant authority, via VHF voice communication. Some SRSs also accept reports transmitted through AIS, internet-based reporting systems, e-mail, fax, SATcom, mobile phone, or a combination of these communication means.

Figure XX – Examples of reporting points from Bay of Fundy

### Purpose

The purpose of the vessel shore reporting service is to:

- Secure submission and distribution of reports required by shore-based authorities in the harmonized standard and in the required timeframe.

- Reduce the administrative burden on board the ship and ashore.

- Reduce the number of human errors and missing information by automating the reporting processes as much as possible.  .  And,

- Provide real-time access to information to relevant stakeholders in a secure manner.

MSC 43(64) states that the objectives of a ship reporting system (SRS) should be based upon:

“the improvement of the safety of life at sea, the safety and efficiency of navigation and/or to increase the protection of the marine environment. They may or may not be operated as part of a vessel traffic service.”

As such, the VSR service contributes to the traffic situational awareness of a coastal administration either by being informed of vessels heading into its waters or the ones already in transit. Based on the information collected, a coastal administration can initiate an intervention plan, if required.

### Operational Approach

Electronic systems for reporting should use the same protocols and product specifications, in a Single Window solution, to send digital pre-arrival information such as the FAL forms and other regional/national requirements, as defined in SOLAS V Regulation 11.2 for ship reporting systems. This will ensure a common harmonized platform for all ship reporting systems.

National Competent Authorities should provide information about reporting formalities and ensure that all information regarding reporting is easy to understand, accessible and even automated for the master or operator.

This service should provide ICT tools for shipboard and shore-based personnel to streamline the processes and procedures associated with the generation and distribution of required reports, including retrieval of information from other ship systems (Ballast Management, Waste Management System, Emission Control System, Navigation System, etc) and from shore-based sources (cargo and passenger booking offices, crewing agents, stevedores, etc).

Examples of information to provide can be:

| Information related to: | Examples |
| --- | --- |
| Reporting regulations | - What to report. The pre-arrival information may consist of ship particulars, arrival notice, crew and passenger lists, crew and passenger effects declarations, stores list, IMDG information, waste declaration, ship’s certificates, ports of call list, dangerous cargo declaration and manifests, vaccination list, narcotic list, ship’s money declaration, etc. - When / What to report (e.g. 24h, 48h, 72h, 96h before arrival) - To who (e.g. immigrations, police, port master, etc.) |
| Reporting tools | - Web, App, etc. - Communication - Guidelines |
| Shore receivers And support | - Contact information - Support information - Information about local reporting aid / support such as coastal radio stations and agencies |

The type of information required and reporting periods may differ from country to country and create some confusion on the ships’ side, if the information requested is not clearly stated. Also, the reporting periods in some situations start as far in advance as 96 h before a ship enters a coastal administration’s waters. This may pose challenges with respect to the communication means available to provide the required information all along the transit.

Currently, the following communication systems are normally used to report information:

- AIS

- internet-based reporting systems

- e-mail

- fax

- SATCOM

- mobile phone, or

- a combination of these systems

As the scope, transmission capacity, and data format of each of these systems differ, the possibility of developing standardized protocols with their product specifications to automate the collection of data on-board and communicate it to shore-based authorities might constitute a real issue. There is a possibility of packaging the reporting information with a route exchange format (e.g. S-412), but the security of the means of communication would need to be carefully evaluated.

Figure XX – Example of current communication systems used to report information

Some of these reporting systems are not using a digital format, which prevents provision of an automated service. The first step in an operational approach would be the use of a communication system capable of transmitting digital data. Secondly, and given reporting requirements differ among coastal states, a library containing information required by each administration should be developed which complies with the standardized product specifications principle. Specific requirements requested by an administration would be part of the form pertaining to this country.

Figure XX – Example of electronic communication systems using a standardized product specifications library

Basic information such as the ship name, call sign, IMO number and MMSI, etc., can be automatically collected from the ship’s AIS system and transmitted through terrestrial or satellite AIS.

### User Needs

Ships’ masters are facing considerable administrative burdens to comply with different mandatory Vessel Shore Reporting requirements and transmission of  information. Different procedures, data formats, documentary requirements and formalities are applied depending on the country, ship reporting system or port of destination. One of the main challenges is that the ship has to communicate in different manners through different systems.

This, in turn, requires building and maintaining a library of required reports that are uniquely identified and characterized by their requirements for format, deadline, content, etc.  The vessel reporting system also requires developing and maintaining an S-100 Product Specification for CMDS that can be used to generate all required reports in the library.  Lastly it requires that ships’ systems that generate reporting information be certified to be compliant with an international machine-to-machine interface standard or ship network standards such as IEC 61162 series.

Figure x.x.x BAREP SRS between the Russian federation and Norway

### Information to be Provided

See Annex 8, MS 8 Vessel Shore Reporting

### Associated Technical Services

To be capable of generating this information automatically and transmitting it automatically, a realistic operational implementation of the VSR service would require the involvement of both coastal administrations and shipowners. Coastal administrations would need to develop an international library of ship reports that are uniquely identified and characterized by their requirements for format, deadline, content, etc. This library would be part of the S 10x register and standardized product specifications (e.g. S-127 and S-421) would be developed and maintained in order to generate the required reports.

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |
| S-127 Maritime Traffic Management |  | Marine Traffic Management (MTM) datasets describe the availability and reliability of vessel traffic services, pilotage, routing measures, and ship reporting systems. This includes their service areas, services offered and instructions for contacting or utilizing these services. |  | IHO |
| S-421 Route Plan based on S-100 |  | S-421 specifies the content, structure, and metadata needed for creating a fully S-100 compliant Route Plan Information and its portrayal within an S-100-based Application. |  | IEC |

### Relation to other Maritime Services

Digital information from the ship reporting system using common standards is beneficial for several stakeholders and actors involved.

In addition to authorities (e.g. border police, immigration, defence, coast guard, customs, etc.) claiming the information, the list below gives example of services that can benefit from access to ship reporting information.

| Description | Examples of data that could be used from MS 10 |
| --- | --- |
| MS 1 VTS IS | Type of vessel, nationality, MMSI, IMO no, contact information |
| MS2 VTS NAS | Draft, cargo |
| MS3 VTS TOS | ETA / ATA, ISPS information, purpose for arrival |
| MS4 Local Port Service | The majority of the information in a ship reporting system is useful and can be re-used an effective and transparent port operation |
| MS 5 Maritime Safety Information | Inform about changes to reporting requirements |
| MS 6 Pilotage Service | Ship reporting systems can exchange information with the pilot system. Easy access to information can be important for the pilot. A digital pilot requesting / booking system connected to the ship reporting system will increase efficiency. |
| MS 10 Maritime Assistance Service | Information about cargo, dangerous goods and persons onboard can reduce time before assistance and contribute to the allocation of the appropriate resources for the actual situation. |
| MS 11 Nautical Chart Service | Contain charted information about areas where reporting is required |
| MS 12 Nautical Publications Service | Contain detail information about reporting requirements, such as who must report, when reports are due, and to whom the reports must be submitted. |
| MS 13 Ice Navigation Service | Information from the ship reporting system can contribute to a more tailor-made and effective icebreaker service. |
| MS 16 Search and Rescue Service | Salvage information, drifting patterns, SAR areas, rescue capabilities in the area. |

## MS 9 Telemedical Assistance Service (TMAS)

### Submitting Organisation

### Description of the Maritime Service

According to IMO/ILO resolution 164, the TMAS centre should be able to provide medical advice for seafarers 24 h/day, 365 days/year. TMAS should be permanently staffed by physicians qualified in conducting remote consultations and who are well-versed in the particular nature of treatment on board ship.

Within maritime medicine, the prevailing view has for a long time been that a standardization of the TMAS services is both necessary and wanted. This would enhance the quality of the medical practice. A standardization of reporting and registering of medical events will also make a much better basis for advancement (MSC.1/Circ.1218; MSC/Circ.960).

### Purpose

TMAS can be delivered in all sea areas but differences in bandwidth, depending on available types of transmission techniques, will result in different levels of service.

Seafarers have some basic, but limited, training and skills in medical treatment. Sharing of digital information for diagnosis will increase the rate of appropriate treatment.

It is essential to provide seafarers with easy and fast access to a single point of contact with the TMAS centre,   as well as to sharee information between the TMAS centre and medically responsible personal onboard during treatment.

The purpose of TMAS is to give decision support and advice to the seafarer on board responsible for medical care whenever the provision of treatment cannot wait. This is today mostly done by voice communication using VHF, MF or short wave radio, or mail. In a future e-navigation scenario, the quality of remote diagnostics and even treatment can be expected to improve.

In addition to prevailing TMAS provisions, real time monitoring of the patient’s current health status will be important. The parameters monitored may differ in different systems, but may include heart rate, blood oxygen saturation and pressure. As an example, see the telemetric medical information screen below.

An example of a typical telemetry screen: top (green) curve is heart rate (Beats Per Minute from two redundant connectors), below (in red) is Ambulatory Blood Pressure (ABP) in mmHg, the third curve is Peripheral Capillary Oxygen Saturation (SPO2), then oxygen saturation of the blood (normal is 95-100 %), at the bottom of the screen are Noninvasive Blood Pressure (NBP) and the patient’s temperature in degrees Celsius.

Further needs might involve real-time video chat between the remote specialist doctor and the patient, as well as examinations with remotely controlled cameras. In addition,  extensive, detailed, cardiovascular (heart) data for a period of time should be able to be collected and sent as compressed files for further monitoring by the specialist doctor.

### Operational Approach

One may envision that in the not too distant future, what today is very expensive medical equipment could be installed on (at least some) ships, e.g. large cruise vessels. Examples of such equipment could be[short version] ultrasonography, computer tomography (CT) and surgical operation robots. Such equipment will then be remotely operated from land given reliable radio connections.

Diagnostic ultrasonography
is used to see internal body structures such as tendons, muscles, joints, vessels and internal organs. Its aim is often to find a source of a disease or to exclude any pathology. The practice of examining pregnant women using ultrasound is called obstetric ultrasound, and is widely used.

The European Space Agency has already tested a long-distance robotic ultrasound system for remote operation by distant specialists. An assistant on board simply holds the device against the patient and the ultrasound expert can move the probe as if present in the examination room, rather than thousands of kilometres away. They control the device in real time using a joystick, based on ultrasound imagery relayed back.

CAT
Computed tomography, more commonly known as a CT or CAT scan, is a diagnostic medical test that, like traditional x-rays, produces multiple images or pictures of the inside of the body.

Remote surgery (also known as tele-surgery)
Remote surgery allows the surgeon to remotely perform surgery either using direct tele-manipulator or through computer control. The surgeon does not have to be present, but can be anywhere in the world, leading to the possibility for remote surgery.

### User Needs

Today

A digital health emergency monitoring system allowing logged text interchange between a ship and medical specialist at a shore hospital including:

- Voice communication not only with the medical officer onboard, but also with the patient in the medical bay

- Real-time video for visual examination by the remote specialists using video

- Real-time transmission at length of medical telemetry

- Transmission of compressed data packages of e.g. EEG data

- Up to date digital georeferenced lists of available telemedical assistance services that can be offered by a coastal state. Comparison of TMAS with ship capabilities would facilitate the request of the most adapted service to respond to the situation on board.

In the future (examples)

- Transmission of data and remote control of ultrasonography

- Transmission of data and remote control of computer tomography and the like imaging techniques

- Transmission of data and remote control of robotic surgery

REMARK: The two sentences about communication limitations

https://www.welchallyn.com/content/dam/welchallyn/documents/upload-docs/Product-Literature/Brochure/MC11587%20Telemetry%20MedSurg.pdf

http://www.esa.int/Our_Activities/Space_Engineering_Technology/Long-distance_ultrasound_exams_controlled_by_joystick

### Information to be Provided

See Annex 9, MS 9 Telemedical Assistance Service

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other Digital Maritime Services

| Description | Examples of data that could be used from MS 9 |
| --- | --- |
| MS 1 VTS IS | Communication with ship |
| MS 12. Nautical Publications Service | Provide up to date information on radio station and earth stations offering TMAS. |
| MS 16 Search and Rescue Service | Medical information and constraints for SAR procedures. |

## MS 10 Maritime Assistance Service (MAS)

### Submitting Organisation

### Description of the Maritime Service

IMO Resolution A.950(23) on Maritime Assistance Services (MAS) specifies that the circumstances of a ship’s operation that involve a MAS are not those requiring rescue of persons. Three situations can arise:

- the ship is involved in an incident (e.g., loss of cargo, accidental discharge of oil, etc.) that does not impair its seakeeping ability but nevertheless has to be reported;

- the ship, according to its master’s assessment, is in need of assistance but not in a distress situation (about to sink, fire developing, etc.) that requires the rescue of those on board; and

- the ship is found to be in a distress situation and those on board have already been rescued, with the possible exception of those who have remained aboard or have been placed on board to attempt to deal with the ship’s situation.

If, however, in an evolving situation, the persons on board find themselves in distress, the involvement of the MRCC and not the MAS will have priority. As such, the MAS is responsible only for receiving and transmitting communications and monitoring the situation.

The establishment of a MAS should not necessarily entail the setting up of a new organization. The functions of the MAS could, at the discretion of the Administration, be discharged by an existing organization, preferably an MRCC, or alternatively a harbour master’s office, a coast guard operations centre (if one exists) or another body.

Coastal States are requested to notify IMO of the existence and details (call numbers, call signs, etc.) of their MAS, in accordance with the predetermined format (A 23/Res.950, appendix to annex 2). The IMO Secretariat will periodically publish the collected particulars in a circular. National organizations that disseminate nautical information are invited to publish such particulars.

### Purpose

The primary mission of the MAS is to manage communications between the coastal state, ship's officers requiring assistance, and other responsible maritime organizations: fleet owners, salvage companies, port authorities, brokers, etc. As an intermediary, the main purposes of the MAS are:

•	 receiving the reports, consultations and notifications required by the IMO instruments referred to in Annex 1 to the present resolution;

•	monitoring the ship situation if a report, as referred to in paragraph (a) above, discloses an incident that may cause the ship to be in need of assistance;

•	serving as the point of contact between the master and the coastal state concerned, if the ship’s situation requires exchanges of information between the ship and the coastal state but is not a distress situation that could lead to a search and rescue operation;

•	serving as the point of contact between those involved in a marine salvage operation undertaken by private facilities at the request of parties having a legitimate interest in the ship and the coastal state, if the coastal state concerned decides that it should monitor all phases of the operation.

### Operational Approach

A 23/Res.950 indicates that a MAS should be operational on a 24-hour basis and that it should be possible for the English language to be used in exchanges between a ship in need of assistance and a MAS. Also, MASs should be authorized by their respective governments to exchange information concerning reports received and situations involving ships which may be in need of assistance with each other.

The most common events requiring MAS are (IMO A23/Res. 949):

- fire

- explosion

- damage to the ship, including mechanical and/or structural failure

- collision

- pollution

- impaired vessel stability

- grounding.

Development of scenarios including their potential consequences  with regard to safety of persons and pollution, fire, toxic and explosion risks.

The development of a database containing the key services contact information and availability of their respective resources would help move towards the digitalization of the MAS. Also, the use of templates listing the types of incidents that commonly occurring and their level of severity could facilitate ship – shore based communication.  The exchange of information on key services, their contact details, capabilities and geographic service areas could be developed as an S-100-based product specification. On the ship side, digital nautical publications in the S-12x-series of product specifications, such as S-123 (Marine Radio Service), could contain MAS information.

Figure xx illustrates briefly the different functions of a MAS.

Figure xx. Illustration of some functions provided by a MAS

### User needs

- Table X.X – Examples of types of information provision that MAS can cover

| Information related to: | Examples |
| --- | --- |
| General information | - competent MAS - roles and responsibilities of authorities and responders (fire fighting capability) - Responsible SAR coordination centre - Contact information to MRCC / operator - Ongoing operations in an area |
| MAS information | - MAS geographical coordinates, contact information, capacity, etc. - Information about available resources for lightering - Information about available resources for pollution combating and recovery - Information about available resources for towage - Information about available resources for stowage - Information about available resources for salvage - Information about available resources for storage. - Contingency planning - Evacuation facilities - Type of operation - What and when - Vessels involved - Communication |

In some of these situations, the ship’s assistance may require the designation of a place of refuge in order to limit the navigational impacts, protect the environment or facilitate repairing the ship. In that case, an assessment of risks related to the identified event should be done taking into account:

1.	Environmental and social factors:

- safety of those on board

- threat to public safety

- pollution caused by the ship

- sensitive habitats and species

- facilities available

- etc.

2.	Natural conditions:

- Prevailing winds and sea conditions

- Tides and tidal currents

- Bathymetry

- Etc.

### Information to be Provided

See Annex 10, MS 10 Maritime Assistance Service

### Associated Technical Services

Regarding communication facilities, IMO Res. 950 recommends the  use of COMSAR/Circ.18, Guidance on minimum communication needs of maritime rescue co-ordination centres (MRCCs), as a basis. This Circular identifies the following communication systems:

- Telephone links

- Fax links

- Telex link

- VHF and MF with DSC

- Ordinary VHF

- Special networks : examine the possibility of linking with existing networks (Administration, armed forces telephone networks), in particular the aeronautical fixed telecommunication network (AFTN), which provides a link with the aeronautical rescue organization

- Inmarsat-C ship earth station or other satellite service providers

- Etc.

S-123 (Marine Radio Service) data products contain contact details for MRCCs.

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other the Maritime Services

| Description | Examples of data that could be used in MS 10 |
| --- | --- |
| MS 1 VTS IS | Local sensor information such as CCTV, Radar, AIS. Regulations. Other traffic. |
| MS 2 VTS NAS | Exchange of routes, directions, navigation advices and assistance |
| MS3 VTS TOS | Information regarding regulations and other traffic |
| MS 4 Local Port Service | Port availability and anchorage are in the port, services related to the MAS situation |
| MS 5 Maritime Safety Information | All information depending on the structure of MSI |
| MS 6 Pilotage Service | Contact information for pilotage, pilot assistance, pilot request |
| MS 7 Tug Service | Tug capacity, contact information, tug order and updates |
| MS 8 Vessel Shore reporting | Information about the reporting formalities, local regulations, contact information, notification about dangerous cargo, number of persons onboard, arrival and departure information etc. |
| MS 9 Telemedical | Contact Information |
| MS 11 Nautical Chart Service | Local area updates, chart updates, Notice to Mariners |
| MS 12. Nautical Publications Service | Tidal tables, Notice to Mariners, List of Lights, information updates to publications |
| MS 13 Ice Navigation Service | Ice routes, ice conditions, ice breaking assistance |
| MS 14 Meteorological Service | Weather information, visibility, wave information |
| MS 15 Real Time Hydro and Information Service | Horizontal and vertical tidal information in an area, real time current, real time wave form sensors in an area |
| MS 16 Search and Rescue Service | Search and Rescue contact information, SAR capacity. SAR areas of operations. |

## MS 11 Nautical Chart Service

### Submitting Organisation

Hydrographic Office

### Description of the Maritime Service

The aim of the nautical chart service is to safeguard navigation at sea by providing information, such as the nature and form of the coast, water depth, tides table, obstructions and other dangers to navigation, location and type of aids to navigation.  Most often, a nautical chart service would be signatories of the IMO SOLAS Convention where the responsibilities of a hydrographic service is defined

### Purpose

Contracting Governments to SOLAS V undertake to arrange for the collection and compilation of hydrographic data and the publication, dissemination and keeping up to date of all nautical information necessary for safe navigation. In particular, Contracting Governments undertake to co-operate in carrying out, as far as possible, the following nautical and hydrographic services, in the manner most suitable for the purpose of aiding navigation:

- to ensure that hydrographic surveying is carried out and adequate, as far as possible, to the requirements of safe navigation;

- to prepare and issue nautical charts, sailing directions, lists of lights, tide tables and other nautical publications, where applicable, satisfying the needs of safe navigation;

- to promulgate notices to mariners in order that nautical charts and publications are kept, as far as possible, up to date; and

- to provide data management arrangements to support these services.

Contracting Governments undertake to ensure the greatest possible uniformity in charts and nautical publications and to take into account, whenever possible, relevant international resolutions and recommendations, which means the appropriate resolutions and recommendations adopted by the International Hydrographic Organization (IHO).

IHO facilitates co-ordination of the hydrographic office activities to the greatest possible degree in order to ensure that hydrographic and nautical information is made available on a world-wide scale as timely, reliably, and unambiguously as possible.

Raster charts services also continue to be available and there is expected to be a continued demand for this service  at least during the transition period.

Examples of Nautical Chart Service is listed in Table x.

Table x – Nautical Chart Service

| Information related to: | Examples: |
| --- | --- |
| Navigational charts | Paper charts Raster Navigational Charts (RNC) Electronic Navigation Charts (ENC) |
| Chart catalogue | Catalogue of available products |
| Bathymetric charts | Bathymetric Information with greater details than normally given in a navigational chart |
| Notice to Mariners | Regular updating service for products issued by the individual Nautical Chart Service. |
| Hydrography | Information that will include factors such as the stability of the seabed, sea depth, the accuracy of surveys, tidal ranges, tidal streams, prevailing currents and swell, etc. |

### Operational Approach

A hydrographic service produces and distributes paper charts, ENCs and other products to safeguard navigation at sea. These services are carried out as defined in the IHO publications M-3 and, for ENCs, the WEND (Worldwide Electronic Navigational Chart Database) principles (M-3, Resolutions of the IHO – Resolution 1/1997 (as amended)) which note that:

“The purpose of WEND is to ensure a world-wide consistent level of high-quality, updated official ENCs through integrated services that support chart carriage requirements of SOLAS Chapter V, and the requirements of the IMO Performance Standards for ECDIS.”

The IHO S-65 Standard notes that hydrographic offices are responsible for:

- The preparation and provision of digital data and its subsequent updating for waters of national jurisdiction.

- Ensuring that, mariners, anywhere in the world, can obtain fully updated ENCs for all shipping routes and ports across the world and that their ENC data are available to users through integrated services.

- Assuring the high quality of its ENC services through the use of a Quality Management System,certified by a relevant body as conforming to a suitable recognised standard.  Typically this will be ISO 9001.

- Ensuring compliance with all relevant IHO and IMO standards and criteria (including IHO S-57, IHO S-52, or their replacements).

- Providing timely updates to the ENC for the mariner. These should be at least as frequent and timely as those provided by the Contracting Government for the correction of paper charts.

Similar arrangements and principles apply to nautical charts and other nautical products (e.g. S-102 high definition bathymetry) made available by hydrographic offices.

### User Needs

Nautical chart services should offer both paper charts and digital charts in raster and vector formats for use in ECDIS or ECS, depending on vessel classification. Nautical charts should be made available as widely as possible, using various distribution channels.  This may include direct distribution, agents and other service providers. ENC may be distributed via a RENC, which distributes them onward with the distributor network.

Figure 4.11-1 - General principle of ENC distribution (Source: S-65)

A chart catalogue service should be included as part of a Nautical Chart service to give users a quick reference guide regarding the products offered.

IHO is developing the next version of ENC, called S-101. It should be expected that a new ECDIS or ECS system may be needed to utilize these next generation ENC. For future nautical chart services, the commercial model of distribution is expected to continue. Chart vendors provide for licensing and payment on a when-needed basis even now, and this will continue to be a requirement. The technical specifications of this service must support licensing, payment (both subscription-based and spot payments), and data protection.

Requirements for future services include faster turnaround to process mariner reports to agencies and the issuance of chart updates, making it easier to include more types of relevant information with reports such as information in mixed media, and reporting methods that are better integrated with ENCs. For example, initial generation of a hydrographic report on an ECDIS followed by automatic transfer of the initial report to a back-of-bridge system and thence to the appropriate hydrographic office.

Provision should be made for value-added data (VAD) and value-added services (VAS) supplied by vendors (this means that VAD is inherent to chart data and not an overlay ), including SENC distribution.

Raster chart services also continue to be available and there is expected to be a continued demand during the transition period at least.

### Information to be Provided

See Annex 11, MS 11 Nautical Chart Service.

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other Maritime Services

MS 12 and DMS 13 will supplement MS 11 data for navigation purposes. MS 5 will supplement it with up-to-date information on the status of navigation aids, obstructions to shipping and other events that impact navigational safety.

| Description | Examples of data that could be used in MS 11 |
| --- | --- |
| MS 5 Maritime Safety Information | Provides supplemental up-to-date information on the status of navigation aids, obstructions to shipping and other events that impact navigational safety. |
| MS 12 Nautical Publications Service | Provides supplemental navigational information |
| MS 13 Ice Navigation Service | Provides supplemental navigational information |
| MS 15 Real Time Hydro and Information Service | Provides supplemental navigational information |

## MS 12 Nautical publications service

### Submitting Organisation

International Hydrographic Organization

### Description of the maritime service

Nautical Publications Services deliver a set of nautical information available for a particular marine area.  The aim of the Nautical Publications Service is to provide information as a support to the navigation process.  This comprises information to complement nautical charts, such as information on ports and sea areas as defined in paragraph 23 of MSC.1/Circ. 1595, as well as the contact information of authorities and services for a sea area or port.  It further describes regulations, restrictions, recommendations and other nautical information applicable in these areas.

Nautical Publications Services include:

- the information traditionally provided within updated nautical publications such as sailing directions, lists of lights, notices to mariners, tide tables and all other nautical publications necessary for the intended voyage (SOLAS Chapter V Regulation 27).  The majority of the information can be delivered from shore to ship in a digital format.  This will enhance the usability, increase the quality and update rate and give the navigator an opportunity to tailor made the information needed.

- a discovery service to allow users to determine what is available in their area of interest (geographic and context);

- an ordering service to allow users to order the information required from the service providers identified;

- a delivery service to allow the user to receive the information required.

### Purpose

The purpose of the Nautical Publications Service is to electronically provide the mariner with information to complement ENCs/Nautical Charts for advance planning and to navigate a ship safely during the intended voyage.

The Nautical Publications Service provides information which is continuously updated and which is required for voyage planning and execution.  It improves the situational awareness during the voyage.

SOLAS Chapter V Regulation 2 allows the provision of nautical publication information in digital format as database and SOLAS Chapter V Regulation 27 requires the carriage of nautical publications suitable for the intended voyage.  The combination of both is a digital provision of nautical information requested for navigation according to SOLAS Chapter V.

The information covered in nautical publications is either provided as printed paper publications (NP1) or as digital publications based upon existing paper publications (NP2).  The next evolutionary step is the provision of information in digital datasets based on internationally harmonised and appropriate data models (NP3).  The datasets will be distributed by appropriate methods to electronic onboard equipment.

The anticipated steps in the transition to full digital delivery can be described only in general terms at this time:

- Development of product specifications (including data models) for digital data products.

- Conversion of appropriate parts of the content of existing NP1 and NP2 nautical publications to NP3 data products.

- Integration of appropriate new sources of nautical publications information into the supply and production chain for NP3 data products.

- Delivery infrastructure and methods – either the design and construction of new delivery infrastructure/methods, or the integration into existing or under-development delivery infrastructure/methods.

- Application upgrades or new application development to make best use of the digital products.

- Test-beds for the data products, delivery methods, and applications.

### Operational approach

The data model is based on the IHO S-100 Hydrographic Data Model and derived Product Specifications.  It enables the information provision in a harmonised way.  The products are designed for a display based on INTERIM GUIDELINES FOR THE HARMONIZED DISPLAY OF NAVIGATION INFORMATION RECEIVED VIA COMMUNICATION EQUIPMENT (MSC.1/Circ.1593) and the data provision should take into account a harmonised display of navigational information.  The used product specifications comprise rules for interoperation and harmonized graphical presentations of datasets that will be interacting one each other and with the ENC information when used by a navigation system such an ECDIS.

The provision of Nautical Publications Service should use distribution strategies, methods, and technologies which can adapt to serve vessels in locations or conditions that are highly challenging for information transfer.

The data provision follows the S-100 based data protection schema.

### User needs

The primary users are mariners responsible for maritime navigation.  Access to the information is required both onshore and at sea.

SOLAS Chapter V, Regulation 19 paragraph 2.1.4 describes the requirement for ships to carry ‘nautical charts and nautical publications to plan and display the ship's route for the intended voyage and to plot and monitor positions throughout the voyage.’  Timely and simple access to uniform up-to-date nautical information for a particular sea area is essential for the conduct of safe voyages.

The Nautical Publications Service provides navigational information for safe navigation on open sea, for making landfall, and for navigation in confined waters.

The Nautical Publications Service provides information on maritime services available and provides details to get access to responsible authorities and services provided by those authorities.

Secondary users such as Pilot Services, Defence, VTS Authorities or any individuals or organisations, onshore and at sea, require access to the information for reference.

### Information to be provided

The appropriate resolutions and recommendations adopted by the International Hydrographic Organization provide the recommended set of information to be covered by the Nautical Publication Service.

| Information related to: | Examples: |
| --- | --- |
| Transits and routeing | - Routes in constricted shipping lanes - Routeing measures, traffic separation schemes, and shipping lanes - Associated Vessel Traffic Service - The mandatory reporting of vessel traffic movements - Associated Ship Reporting System |
| Ports approaches and entry | - Hazards, directions, limiting conditions - Pilot service, outer anchorages - Traffic regulation, arrival procedure |
| Summary information about port facilities | - Function, port authority - Basins and berths - Depth alongside berths, and quay lengths - Cargo handling facilities at specified terminals and berths - Specific vessel parameters, such as length, draft, beam |
| Marine radio services | - Geographic availability of services - Frequencies and channels used and broadcast schedules - Purposes supported – Weather forecasts, MSI, telemedical assistance etc. |
| Protected area information | - Locations of marine protected areas - Restrictions and regulations applicable within specific areas |
| Prevailing natural conditions | - Seasonal hazardous conditions - Periodic (e.g., tide‐related) or irregular hazardous conditions |
| Regulatory information | - Laws and regulations applicable in specific locations. - Laws and regulations applying to vessels of specific dimensions or carrying specified cargo - Local rules regarding use of specific pilot boarding places by vessels exceeding specified dimensions or carrying hazardous cargo |
| Port Services | - Waste disposal, collection of ship pollutants such as oily wastes - Repair, bunkering, - Availability of potable water - Issuing of Ship Sanitation Certificates - Pilot services contact information and notice times |
| Navigation aids | - descriptions of Lights - descriptions of buoys |
| Climatic Information, predictions | - Tide surge prediction tables and tidal stream atlases - Weather routeing, solar radiation and precipitation - Cold/hot durations and warnings - Air temperature, wind speed and direction - Cloudiness and barometric pressure - Ephemerides and nautical almanacs for celestial navigation |
| Planning | - Mariners’ Routeing Guides |
| Controlled areas | - Vessel Traffic Service contact information - Ship Reporting System contact information - Exercise Area contact information |
| Chart catalogue | - Graphically display a chart catalogue |

The Nautical Publications Service provides up‐to‐date information pertaining to the area along the planned route.

Users should be enabled to report discrepancies between the real world and the information provided by the Nautical Publications Service with no or minimal human interference.

Corrections to Nautical Publications Service information should be provided as updates (either as updates of the whole dataset or as incremental updates) in a format which supports the automatic correction and the traceability of the corrections of the on‐board datasets.

### Associated technical services

| Name | ID (MRN) | Description | Architect(s) | Standardization body |
| --- | --- | --- | --- | --- |

The Service should be capable to work within multiple levels of bandwidth limitations.  The Service should provide the data in various data packages according to the bandwidth capability.

### Relation to other maritime services

Nautical Publications Service provides overviews of other Maritime Services.  It summarises content information which is covered by other Maritime Services in more detail.  Other Maritime Services may reuse information which is provided by Nautical Publications Service.

| Maritime Service No.. | Nautical Publications Services |  |  |  |
| --- | --- | --- | --- | --- |
|  | provides overview | summarizes information | supplements information | reuse of information |
| 1 VTS Information Service (INS)) | - Area of responsibility | - Contact information |  |  |
| 2 Navigational Assistance Service (NAS) | - Area of responsibility | - Contact information |  |  |
| 3 Traffic Organization Service (TOS) | - Area of responsibility | - Contact information |  |  |
| 4 Local Port Service (LPS) | - Area of responsibility | - Contact information |  |  |
| 5 Maritime Safety Information Service (MSI) | - Area of responsibility | - Contact information |  |  |
| 6 Pilotage service | - Area of responsibility | - Contact information |  |  |
| 7 Tug service | - availability | - Contact information |  |  |
| 8 Vessel Shore Reporting | - Area of responsibility | - Contact information |  |  |
| 9 Telemedical Assistance Service (TMAS) |  | - Contact information |  |  |
| 10 Maritime Assistance Service (MAS) |  | - Contact information |  |  |
| 11 Nautical Chart Service |  |  | - charted information |  |
| 13 Ice Navigation Service |  | - Climatic information |  | - radio services information |
| 14 Meteorological Information Service |  |  | - Climatic information |  |
| 15 Real-time hydrographic and environmental information Service |  |  |  | - radio services information |
| 16 Search and Rescue Service | - Area of responsibility - Conduction of service | - Contact information |  | - Tide information and forecasts (for SAR planners) |

Another way of providing the information above

| Description | Examples of data that could be used by MS 12 |
| --- | --- |
| MS 1 VTS IS | Area of the service, functions, contact information, communication, Local sensor information such as CCTV, Radar, AIS. Regulations. Other traffic. Information regarding regulations and special traffic |
| MS 2 VTS NAS | Recommended routes, directions, navigation advices |
| MS3 VTS TOS | Not relevant |
| MS 4 Local Port Service | Port security, facilitation and anchorage area, services related to the vessel, arrival procedure, contact information, communication |
| MS 5 Maritime Safety Information | Area of the service, contact information, communication, navigational warnings issued by the MSI service |
| MS 6 Pilotage Service | Applicability, cContact information for pilotage, Pilot assistance, Pilot request |
| MS 7 Tug Service | Availability, contact information, regulations |
| MS 8 Vessel Shore reporting | Applicability, information about the reporting formalities, local regulations, contact information |
| MS 9 Telemedical Assistance Service | Contact information |
| MS 10 MAS | Contact information |
| MS 11 Nautical Chart Service | Charted information, Notice to Mariners |
| MS 13 Ice Navigation Service | Ice routes, ice breaking assistance |
| MS 14 Meteorological Service | Local weather phenomena, climatic information, wave information |
| MS 15 Real Time Hydrographic and Environmental Information Service | Information about sensors in an area, radio services information |
| MS 16 Search and Rescue Service | Search and Rescue contact information, communication, SAR capacity, SAR areas of responsibility |

## MS 13 Ice navigation service

### Submitting Organisation

World Meteorological Organisation

### Description of the Maritime Service

To provide ice navigation information to ships

### Purpose

The World Meteorological Organization’s Manual on Marine Meteorology (WMO No. 558) defines the procedures for marine meteorological information text bulletins involving ice-related components in High Seas areas, Coastal, Offshore and Local Waters, for decision making whether or not proceed with ice navigation

SOLAS V Regulation 5 outlines obligations for the provision of weather information suitable for shipping with forecasts including ice conditions and hazards.

The IMO Polar Code (MSC, MEPC) outlines the environmental information requirements for ships operating in polar waters.  The IMO Polar Code reinforces operating guidelines for the hazard of Ice Accretion and Ice Waters (sea-ice conditions), and introduces new operating guidelines for hazards related to Low Air Temperature, and defines the Polar Service Temperature for equipment performance.

SOLAS V, regulation 6 governs the ice patrol service in the North Atlantic stating that “Ice Patrol contributes to safety of life at sea, safety and efficiency of navigation and protection of the marine environment in the North Atlantic. Ships transiting the region of icebergs guarded by the Ice Patrol during the ice season are required to make use of the services provided by the Ice Patrol.” This service is currently carried out jointly by USA and Canada.

The IMO Assembly Resolution A.1051(27) outlines the functions of the Worldwide Met-Ocean Information and Warning Service. The Worldwide Met-Ocean Information and Warning Service (WWMIWS) provides met-ocean Maritime Safety Information (MSI) including sea-ice conditions and hazards to mariners.

The standards for the ice terminology and symbology, including sea ice, ice of land origin (icebergs) and lake ice are set by the WMO publication 259 “WMO Sea-Ice Nomenclature” (vol.I – Terminology, vol.III – International System of Sea-Ice Symbols) with JCOMM publications TR-080 “Electronic Chart Systems Ice Objects Catalogue” and TR-081 “S-411 Ice Information Product Specification” documenting coding and portrayal of ice conditions on Electronic Navigational Chart systems.

WMO publication 574 “Sea-Ice Information Services in the World” has comprehensive up-to-date information on National Ice Services.

### Operational Approach

Examples of Ice Navigation Services are listed in Table x.

Table x – Ice Navigation Service

| Information related to: | Examples: |
| --- | --- |
| Ice conditions (as an ice chart) | Near real-time and forecasts of: - Sea ice concentration - Sea ice stage of development ( or thickness) - Form of sea ice - Ice dynamics (ridging, pressure, drift) - Location and orientation of leads, cracks - Icebergs location, concentration and drift - Limits of all known sea ice, iceberg risk and/or ice edge |
| Ice reports and bulletins | - Text summaries of ice conditions |
| Routeing aids | - Recommended routes - Ice pilotage - Icebreaker assistance |
| Navigation Planning | - Risk assessment - Application for navigation (from relevant bodies) |

The most important features of sea ice which affect marine operations are:

(a)	the amount of ice present, I.e. concentration usually measured as tenths of the sea surface covered by ice;

(b)	ice thickness, referred to as stage of development which is related to ice age;

(c)	form of ice, i.e. whether it is fast ice or  drifting ice, floe size;

(d)	ice dynamics including ridging, pressure, drift;

(e) 	location and orientation of leads and ctacks

The position of icebergs at specified times are required with information about their estimated size, concentration or number within certain area; and speed and direction of movement.

Warnings of ice accretion highlight areas where the accumulation of ice on the superstructure and deck equipment of vessels may potencially  (effect depends on true wind and waves angles, tonnage, hull shape) affect safety and operational efficiency.

Information about extremely low air temperatures is important for the safety of workers, while historical information about cold air temperatures enables planning and ship design based on the Polar Service Temperature guidelines.

#### Ice analysis and forecast

Some 20 nations around the world offer an ice information service. Services may provide analysis of ice conditions and numerical short-term ice forecasts in a form of ice charts once a day or less for a period of 24 to 144 hours. These are tactical forecasts and may provide advice on difficult ice conditions forming or dissipating, the general motion of the pack, opening and closing of leads, etc. They are strongly influenced by meteorological prediction and should always be used in concert with the weather forecast. Practically in all cases the ice charts are complemented by high and medium resolution satellite imagery – commonly optical and active microwave radar, with resolution and range dependant on the season, region, cloud conditions and type of support. Near coast operations may be complemented by shore-based ice radar imagery.

Other longer-range predictions – those covering periods from 7–10 days to 30 days and seasonal predictions – are  based on numerical, climatological,  analog or statistical methods.

#### Vessel Escort and Ice breaking

Icebreaking and support services may be available to ships transiting ice-covered waters. Coast Guards or other national agencies may operate Ice Operations Centres. These Centres generally provide up-to-date ice information, suggest routes for ships to follow through or around ice, and co-ordinate icebreaker assistance to shipping. Ice Operations Centres are generally in contact with icebreakers at all times and monitor progress of shipping within their area of responsibility. Ice Operations Centres may also provide Recommended Ice Routing services, such as routing maps.

#### Ice navigation planning

Voyage in the ice-covered waters is commonly preceded by planning and acquisition of permission for ice navigation in contact with regulatory bodies and icebreaking services. Planning and application process is dependent on the assigned ice class, region and season of navigation and is done using the acting in the region of operation risk assessment or other regulatory criteria.

### User Needs

Activity based information requirements

| Information related to | Examples |
| --- | --- |
| En-route or at sea | - Broad, area-based forecasts - Higher detail in complex waterways - Increased interest in synoptic features and movement - Longer forecast lead-time essential |
| Entering, transiting and exiting a port | - Point (small area) based forecasts, - High spatial and temporal detail - Real-time observations - Focus on short-term lead-times |
| At berth | - Forecasts of changes to ice conditions |
| Planning a trip | - Focus on short-term timeframes, as well as longer forecast lead-times - Forecasts and warnings - Specific details on timing of wind changes or hazardous weather leading to changes in ice conditions - Focus on forecast details for specific areas or routes |
| Vessel and equipment design | - Historical values of low air temperatures and water temperatures - Focus on ocean and sea routes |

Formats

Ice information may be provided in a number of formats to meet user requirements.  The following descriptions outline some of the benefits and constraints for each format:

- Map

  - Map or ice chart (as it formally calls for ice parameters), displays highly detailed information across defined spatial domains, and if provided as a time sequence, a user can study the evolution of the weather, ice and ocean elements during specific time periods.  Colours and hatching may be used to highlight hazards or important conditions to operations. Maps may be produced in a digital vector WMO SIGRID-3 or S-100 (S-411, S-412) formats, other WMO gridded formats, as an image, or provided as a web map service, for display by on-screen GIS or ENC software.

  - Similar to a map, remotely sensed coastal ice radar, UAV and satellite high or medium resolution imagery displays highly detailed information on ice conditions. Annotations can be used to highlight particular ice features and recommended routes. Imagery may be provided with georeference (e.g. GeoTIFF, GeoJPEG, GeoJPEG2000) or without it, with animation for a sequence of images from the coastal ice radar and usually in compressed way to comply with restricted traffic bandwidths in high seas and latitudes, for further display by on-screen GIS or ENC software similar to ice charts.

- Text

  - Text products provide short summaries and broader detail for a defined area and time period.  These text products may be simpler to interpret for most users, and can be used for GMDSS satellite and marine radio broadcasts.  Text products generally have a small file size for internet dissemination to mariners at sea.

- Voice

  - Voice products may be transmitted as audio or by video accompanied by other formats.

- Table

  - Information displayed as a table is usually for a specific location, so a user will get the benefit of detailed information over a period of time for that location but may lose context of what is happening over nearby areas.

- Grids

  - Gridded information may be integrated into decision support systems and situational awareness tools, or interrogated to output customized information for the particular marine activity or operating risk threshold.

#### Examples

Figure xx: Example of iceberg analysis.

Figure xx Example of a Recommended Ice Route in the Gulf of St. Lawrence (source ”Ice Navigation in Canadian Waters” – Canadian Coast Guard )

Figure XX – Example of a multi-source satellite annotated Imagery for the Antarctic peninsula area

Fig XX – Example of ice analysis and forecast ice chart for the Kara Sea in a ENS system

### Information to be Provided

See Appendix 13, MS 13 Ice Navigation Service Service

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |
| Ice Service |  | Service that provides sea ice and iceberg information to mariners and related support services. |  | JCOMM (WMO/IOC) |

### Relation to other Maritime Services

| Description | Examples of data that could be used in MS 13 |
| --- | --- |
| MS 5 Maritime Safety Information | Provides supplemental up-to-date information on the status of ice dangers. |
| MS 11 Nautical Chart Service | Provides supplemental navigational information |
| MS 12 Nautical Publication Service | Provides supplemental navigational information |
| MS 14 Meteorological Service | Provides supplemental navigational information |
| MS 15 Real Time Hydro and Information Service | Provides supplemental navigational information |

## MS 14 Meteorological information service

### Submitting Organisation

World Meteorological Organization

### Description of the Maritime Service

To provide meteorological information digitally to ships

### Purpose

The World Meteorological Organization’s Manual on Marine Meteorology (WMO No. 558) defines two types of marine meteorological information:

- Forecasts and warnings for the High Seas; and

- Forecasts and warnings for coastal, offshore and local areas (including ports and harbours).

SOLAS V Regulation 5 obligates contracting parties to produce and distribute to shipping warnings about severe weather such as gales, storms and tropical cyclones, and to produce and provide other weather information suitable for shipping consisting of data, analyses, warnings and forecasts of weather, waves and ice.

The IMO Assembly Resolution A.1051(27) outlines the functions of the Worldwide Met-Ocean Information and Warning Service. The Worldwide Met-Ocean Information and Warning Service (WWMIWS) provides meteorological Maritime Safety Information (MSI) to mariners in the form of marine forecast and warning products. The WWMIWS is coordinated across the worlds’ oceans through 21 defined areas, called METAREAs. Ships receive the MSI products via marine communication systems such as SafetyNet and NAVTEX, which form part of the Global Maritime Distress and Safety System (GMDSS).

The IMO Polar Code (MSC, MEPC) outlines the environmental information requirements for ships operating in polar waters.  The IMO Polar Code reinforces operating guidelines for the hazard of Ice Accretion and Ice Waters (sea-ice conditions), and introduces new operating guidelines for hazards related to Low Air Temperature, and defines the Polar Service Temperature for equipment performance.

Weather routing services are provided in accordance with SOLAS Chapter V, Regulation 34, and IMO Resolution A.893 and IMO MSC/Circular 1063 – Minimum Standards for Provision of Weather Routing Services, outlines the minimum characteristics for a service.  The SOLAS Chapter V, Regulation 5 states that met-ocean services shall be issued by the National Meteorological Service, and this would imply that WMO and its Members should oversee weather routing services and standards as well.

The standards for the portrayal of met-ocean conditions on Electronic Navigational Chart systems are documented within S-412.

Details of service availability, broadcast times and radio frequencies for services provided to vessels at sea are maintained in the WMO publication: WMO No. 9, Volume D, Information for Shipping.

Examples of Meteorological Information Service is listed in Table x.

Table x – Meteorological Information Service

| Information related to: | Examples: |
| --- | --- |
| Wind | - Wind speed, direction, gust information - Real-time values from instruments or satellite |
| Waves | - Forecast wave height, direction, period - Real-time values from buoys or satellite |
| Atmospheric conditions | - Forecast temperature, squalls, cloud, rainfall - Real-time values from instruments or satellite |
| Ocean | - Forecast surface temperature, currents, salinity - Forecast sub-surface temperature and currents - Real-time values from instruments and satellite |
| Weather systems | - Mean Sea Level Pressure contours - System features such as cold fronts, tropical cyclones, low pressure centres, high pressure centres - Satellite images |
| Dangerous weather | - Warnings about location, strength, and movement of storms - Warnings about fog or phenomena causing reduced visibility, ice accretion, cold air temperature, squalls |
| Bulletins and forecasts | - Surface weather analysis, synoptic features with barometric pressure - Forecasts of wind, waves, weather |
| Polar Service Temperature | - Historical values for ocean and port areas |
| Low Air Temperature | - Forecasts of hazard areas - Historical values for ocean and port areas |
| Ship observations | - Receipt of reports from ships in the Voluntary Observation System - Transmission of information extracted from received ship reports to shipping |

MS14 can be delivered in all sea areas (1-6).

### Operational Approach

In general, marine meteorological services have two functions:

- To serve international shipping, fishing and other marine activities on the high seas; and,

- To serve the various activities which take place in coastal and offshore areas, ports and on the coast.

SOLAS convention (Chapter V, Regulation 34, and within Annex A.24 Voyage Planning) describes how vessels should prepare for their trip and route and therefore their information requirements.  The Annex specifically outlines to small vessels the importance of:

- checking the weather forecast for the journey;

- knowing the tides;

- knowing the vessel limitations for the expected weather and wave conditions.

SOLAS V Regulation 5 describes the underlying obligations for weather services, i.e., conveying warnings about severe weather and other weather information useful for shipping, and facilitating weather reports by ships and their distribution as needed for the safety of navigation.

In general, the impact which could result from a meteorological condition depends on its severity and on the sensitivity of a particular activity or operation to that condition. Similarly, meteorological phenomena can make recreational activities and the work of fishing and shipping fleets much more difficult or hazardous.

Marine operations are sensitive to environmental conditions. Generally, extreme values of waves, wind and obstructions to visibility increase the risk to the safety of the vessel or sea structure and to the persons involved in the operation. Less extreme values, even if safety is not threatened, will affect the efficiency, effectiveness or comfort of the operation. The usefulness of a warning or a forecast depends on the accuracy of the prediction, the format and communication platform through which the information is delivered, its timeliness, i.e. the number of hours or days in advance of the event that the forecast can be provided, and the ability of the user to react to the information.

### User Needs

#### Activity based information requirements

| Information related to | Examples |
| --- | --- |
| En-route or at sea | - Broad, area-based forecasts - Higher detail in complex waterways - Increased interest in synoptic features and movement - Longer forecast lead-time essential |
| Entering, transiting and exiting a port | - Point (small area) based forecasts, - High spatial and temporal detail - Real-time observations - Warnings of reduced visibility, squalls - Focus on short-term lead-times |
| At berth | - Warnings of squalls, thunderstorms - Forecasts of general weather conditions |
| Planning a trip | - Focus on short-term timeframes, as well as longer forecast lead-times - Increased interest in synoptic features and movement - Forecasts and warnings - Specific details on timing of wind changes or hazardous weather - Focus on forecast details for specific areas or routes |

#### Formats

Weather information may be provided in a number of formats to meet user requirements.  The following descriptions outline some of the benefits and constraints for each format:

- Map

  - Map displays provide highly detailed information across defined spatial domains, and if provided as a time sequence, a user can study the evolution of the weather and ocean elements during specific time periods.  Maps may be produced in an image format, or provided as a web map service, gridded or S-100 compatible file for display by on-screen software.

- Text

  - Text products provide short summaries and broader detail for a defined area and time period.  These text products may be simpler to interpret for most users, and can be used for marine radio broadcasts.  Text products generally have a small file size for internet dissemination to mariners at sea.

- Voice

  - Voice products may be transmitted as audio or by video accompanied by other formats.  There may be time limits imposed for radio broadcasts, and consideration should also be given to the reception quality on board vessels and impact on a mariner’s ability to interpret the information whilst doing other duties if the broadcast is too long.  These constraints have an impact on the information provided in the text product that it is based on.

- Table

  - Information displayed as a table is usually for a specific location, so a user will get the benefit of detailed information over a period of time for that location but may lose context of what is happening over nearby areas.

- Grids

  - Gridded information may be integrated into decision support systems and situational awareness tools, or interrogated to output customized information for the particular marine activity or operating risk threshold.

#### Examples

Examples of products include surface analysis, wind and wave forecasts and analyses.

Figure 4.14-1. Surface analysis, West Atlantic. (NOAA)

Figure 4.14-2. Wind and wave forecast. (NOAA)

Figure 4.14-3. Wind and wave analysis (NOAA)

### Information to be Provided

See Annex 14, MS 14 Meteorological information service

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other Digital Maritime Services

| Description | Examples of data that could be used in MS 14 |
| --- | --- |
| MS 5 Maritime Safety Information | Provides supplemental up-to-date information on the status of extreme weather. |
| MS 11 Nautical Chart Service | Provides supplemental navigational information |
| MS 12 Nautical Publication Service | Provides supplemental navigational information |
| MS 13 Ice Navigation Service | Provides supplemental navigational information |
| MS 15 Real Time Hydro and Information Service | Provides supplemental navigational information |

## MS 15 Real-time hydrographic and environmental information services

### Submitting Organisation

TBA

### Description of the Maritime Service

The real time hydrographic and environmental information service is essential to safeguard navigation at sea and protect the environment.  The services provided include:

current speed and direction;

wave height;

marine habitat and bathymetry;

- Sensor networks such as ODAS, wave radar, water level gauges, anemometers, current gauges, etc., are widespread and utilized in providing data for these services. These data are often made available on web sites, web mapping services, and other appropriate internet locations such as major ports or VTS. However, the data format appears to be variable.

Examples of Real-time Hydrographic and Environmental Information Service are listed in Table x.

Table x – Real-time Hydrographic and Environmental Information Service

| Information related to: | Examples: |
| --- | --- |
| Current speed and direction | buoy information via AIS Surface current snapshots |
| Surface conditions | Wave heights via radar |
| Environmental conditions | Temperature, pressure, tendencies (rising/falling) Wind speed and direction Visibility |
| Water column | Depths, salinity, temperatures |

In general MSP 15 services can be delivered in all sea areas (1-6). Specific services may be limited to areas where the type of hydrographic and environmental information conveyed by the service is collected.

### Purpose

The main objective of this service is similar to MS 14, and consists of transmitting hydrographic and environmental information to ships wherever it is needed. IHO M-3 2/1962 as amended recommends that the results of oceanographic observations be communicated for the maximum utilization by all marine scientific and hydrographic users.

Another objective is the transmission of environmental conditions by ships and the distribution of information gathered from weather reports to shipping.

Local and regional services may transmit hydrographic and environmental information with AIS-ASM, specifically messages 6 and 8.

IHO is developing a product specification for surface currents named S-111, which is intended for use in ECDIS.

### Operational Approach

### User Needs

Real-time hydrographic and environmental information is distributed in a variety of ways, including radio, AIS-ASM, and internet.

For accurate on board display of AIS messages, receiving vessels must be equipped with a proper Class A AIS device.

Internet access and possibly a user account is required to access real-time hydrographic and environmental information via internet.

### Information to be Provided

See Annex 15, DMS 15 Real time hydrographic and environmental information services

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardization Body |
| --- | --- | --- | --- | --- |

### Relation to other Maritime Services

| Description | Examples of data that could be used in MS 15 |
| --- | --- |
| MS 1 VTS IS | MS1 for communication |
| MS 5 Maritime Safety Information | Communicates warnings about extreme cases |
| MS 11 Nautical Chart Service | Provides supplemental navigational information |
| MS 12 Nautical Publication Service | Provides climatic data to supplement real time data |
| MS 13 Ice Navigation Service | Provides supplemental navigational information |
| MS 14 Meteorological Service | Provides meteorological information that may impact real time information, especially when related to same event |

## MS 16 Search and Rescue (SAR) Service

### Submitting Organisation

### Description of the Maritime Service

The IMO’s 1979 SAR Convention was aimed at developing an international SAR plan to ensure that everywhere in the world, the rescue of persons in distress at sea would be co-ordinated by a responsible SAR organization or by co-operation between neighbouring SAR organizations.

Following the adoption of the 1979 SAR Convention, IMO's Maritime Safety Committee divided the world's oceans into thirteen search and rescue areas, with provisional SAR plans in place for each of these areas.  In each area, the countries concerned have search and rescue regions for which they are responsible.

Parties to the Convention must ensure that arrangements are made for the provision of adequate SAR services in their coastal waters. Parties are encouraged to enter into SAR agreements with neighbouring states involving the establishment of SAR regions, the pooling of facilities, establishment of common procedures, training and liaison visits. The Convention also states that Parties should take measures to expedite entry into its territorial waters of rescue units from other Parties.

The Convention then goes on to establish preparatory measures which should be taken, including the establishment of Rescue Co-ordination Centres (RCCs) and Sub-Centres. It establishes a common glossary to be used by all Parties and outlines operating procedures to be followed in the event of emergencies/alerts and during SAR operations. This includes the designation of an on-scene commander and their duties.

The IMO and International Civil Aviation Organization (ICAO) jointly publish the three-volume IAMSAR Manual. This manual provides international guidelines for a common aviation and maritime approach to organizing and providing SAR services. The three volumes are divided as follows:

- Volume 1, Organization and Management

- Volume 2, Mission Co-ordination

- Volume 3, Mobile Facilities

### Purpose

The RCC responsible for the Search and Rescue Region (SRR) is the primary contact in a SAR incident and ultimately responsible for incident management and decision-making.

In maintaining a state of full readiness, the service is responsible for a number of search and rescue functions, including:

detection and coordination of maritime incidents

control and conduct of maritime search and rescue operations.

These functions may include activities such as:

assisting  the crew and passengers of vessels in distress

assisting victims of maritime and aircraft accidents or incidents

coordinating the medical evaluation of seriously injured or ill persons from a vessel at sea, and their subsequent evacuation, if deemed necessary, to a suitable medical facility

monitoring towing operations;

monitoring and evaluating levels of risk from Maritime Safety Information (MSI) broadcasts to ensure an immediate response to potential life threatening situations;

monitoring vessels not under command

monitoring pollution reports and vessels aground

assisting other emergency response organizations when they require additional resources to prevent loss of life

liasing with other organizations assisting with search and rescue operations

information collection, distribution, and coordination.

### Operational Approach

Time, resources and effective communication will always be challenges when it comes to search and rescue cases involving human lives.  Effective use of time and allocation of rescue resources to the person(s) in distress are of primary importance.

Several actors are normally involved in a SAR operation. Manual exchange of information can be easily misunderstood in the form of wrong positions, SAR areas and other significant search information, and even a short delay can be the difference between life and death.  Lack of language skills has been identified as a major challenge in SAR communications.

Digital exchange and sharing of information could significantly reduce the human errors in SAR case prosecution and also result in huge time savings by allowing RCCs more timely access to accurate and relevant information.

Examples of information for SAR coordination communications that could be shared electronically to mitigate these challenges include:

- last known position (LKP)

- search areas

- search patterns

- search resources

- drift predictions

- last known position (LKP)

- datum

- radio frequencies

- NOTSHIPs

- known routing and planning information for search object (in the case of overdue aircraft or vessels)

- EPIRB hex codes and associated information

- SLDMB information

- satellite imaging

- live stream and images from search resources

- images of search objects

- images of search area

- SAR Briefing Report (with information for vessels engaged in search)

Credit photo: JRCC Halifax.

Figure xx. Simulation of drift prediction

Credit photo: JRCC Halifax.

Figure xx. Illustration of search areas and additional information that may be used during a SAR operation

Credit photo: JRCC Halifax.

Figure xx. Illustration of a SAR Briefing Report

The RCC could then deliver this important information about the SAR situation to a graphical display  shared with the on-scene commander, search resources and all partners involved, improving communications and increasing situational awareness for everyone. This would make it easier for all units to get current information on the search operation underway.

Information from several services assist the SAR Coordinators in making timely decisions and appropriate use of resources. Some examples would include:

- vessel information such as position, course and speed,

- medical and firefighting capabilities of nearby resources,

- other emergency organizations in the area and their rescue capabilities

- meteorological information (including tides and currents, ice information)

- Maritime Safety Information

- ship reporting information such as persons on board, capacity, cargo information,

### User Needs

Coordination, communication and effective exchange of information are important elements of the SAR service. The coordinating RCC must be able to share and receive information quickly and easily with a number of different resources:

- Other RCCs

- Vessels assisting in search and transiting search area

- VTS services

- Ground search and rescue

- Search target

- Other emergency organizations (for example, police, fire, ambulance)

- Port authorities

| Information related to: | Examples |
| --- | --- |
| General information | - Responsible RCC - Other emergency organizations assisting |
| SAR information | - Type of operation - What and when - Last Known Position - Drift plot - Number persons missing/in distress - Dangers such as dangerous substances - In raft, lifeboat, persons in water etc - Datum - SAR area - SAR resources in area - Vessels of opportunity within and near search area - Vessels and aircraft involved in search - Search areas assigned and completed - on-scene commander - Communication - Meteorological information |
| Assistance | - Position of vessels or stations assisting - Towing vessels available and positions - Capacity, Bollard pull (BP), - Other salvage and oil recovery capacities - Response time - Connection information - Intended route / towing plan - speed limits - maneuvering limits - Communication |

Figure x-x-x: Example of common sharing of digital search and rescue information

### Information to be Provided

See Annex 16, MS 16 Search and Rescue Service.

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other Digital Maritime Services

| Description | Examples of data that could be used in MS 16 |
| --- | --- |
| MS 1 VTS IS | Information on VTS areas, places of refuge, vessel traffic in the area, vessel identification information (type of vessel, nationality, MMSI, IMO number, contact information), information from sensors (meteorological, hydrographical, visibility, etc.), radar, AIS, CCTV, communication with ship |
| MS2 VTS NAS | Information about vessels undertaking NAS, exchange of routes, navigation advice and assistance |
| MS3 VTS TOS | Traffic clearance information, time slots, ETAs, route advisories |
| MS4 Local Port Service | Port availability and anchorage areas, berthing information, cargo operations, security, supply, available assistance, port contact information |
| MS 5 Maritime Safety Information | Warnings, no-go areas, diving operations, exercises |
| MS 6 Pilotage Service | Pilot boarding stations, Pilot vessel positions and orders, contact information, request procedures |
| MS 7 Tug service | Tug operations, available tug capacity |
| MS 8 Vessel Shore reporting | ETAs, Notification of arrival, vessel information, cargo information, number of persons on board |
| MS 9 Telemedical | Local address of medical centres and communication and capacity, medical advice |
| MS 10 Maritime Assistance Service | Contact information, places of refuge, routing, information about vessels requesting relevant MAS, incidents , notifications |
| MS 11 Nautical Chart Service | Local area updates, chart updates, notice to mariners |
| MS 12. Nautical Publications Service | Local descriptions and publications, digital information from nautical publications relevant for the operation |
| MS 13 Ice Navigation Service | Ice forecast, ice chart, ice conditions, information regarding icebreaker service / assistance, ice routes |
| MS 14 Meteorological Service | Weather in SAR area |
| MS 15 Real Time Hydro and Information Service | Tidal and current information in SAR area, all information from real time sensors |

## MS 17 Aids to Navigation services (AtoN)

### Submitting Organisation

### Description of the Maritime Service

### Purpose

### Operational Approach

### User Needs

### Information to be Provided

Annex 17, MS 17 Aids to Navigation Services (AtoN)

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other Maritime Services

## MS19 PNT and augmentation services [UK GLA]

### Submitting Organisation

### Description of Maritime Service

### Purpose

### Operational Approach

### User Needs

### Information to be Provided

Annex 18, MS 18 PNT and Augmentation Services

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |

### Relation to other Maritime Services

E-navigation Strategy Implementation Plan – Update 1; 25 May 2018

To fulfil IEC61174 "in order to identify the date and origin of the ENC in use, the ECDIS shall include a graphical index of ENC data available, presented upon the mariner’s request and providing access to the edition and date of each cell.",