Input paper: ENAV23-3.1.11

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	□  ENG		□  PAP					□  Input

X  ENAV	□  VTS							X  Information

Agenda item			n.n

Technical Domain / Task Number	Working Group 1(Harmonisation)

Author(s) / Submitter(s)			Gumjun SON(KR), Namsun Kang(Marine Works), Eivind Mong(MCG)

Hyejin KIM(Marine Works), Yujun JUNG(Marine Works),

Seongsang YU(KR), SMART-Navigation Projcet Office(SNPO)

# Report on the development of SMART-Navigation product specification

# Summary

SMART-Navigation is a Korean e-Navigation test bed project with an implementation period from 2016 to 2020, and aims customizing IMO maritime service to Korean sea by applying S-100 as a data exchange method between ship and shore. The SMART-Navigation product specification is developed as modular based product specifications in consider of the interoperability and maintenance of six services. And through the development of the message catalog, we have solved the interoperability problem for eight specialized product specifications and two common product specifications, because the regulations for compatibility are under development in the S-100 standard.

The SMART-Navigation Project has developed a total of 11 product specifications to be used to provide Maritime Service in republic of Korea. The SMART- Navigation services use each product specification to communicate to the target when exchanging data between ship and land, and the message catalog product specification is always delivered together. This document describes the development results of the first version of the SMART-Navigation product specification.

## Purpose of the document

This document introduces smart navigation product specifications as a result of Korean e-Navigation test bed. By doing this, we hope to have a chance for technical cooperation and exchange with other test-beds.

## Related documents

- ENAV21-12.7	Progress and plan on the MSI Service of SMART Navigation

- ENAV22-8.1.3	Research Progress on SMART-Navgation Product specification

# Background

The SMART-Navigation   project is organized and funded by Ministry of Oceans and Fisheries of the Republic of Korea to implement e-navigation services in its waters based on IMO’s e-Navigation concept adding special features: (1) services for non-SOLAS ships and (2) broadband communication using navigation-dedicated LTE networks (LTE-Maritime) for better connectivity between ship and shore.

This paper has been jointly produced by KR (Korean Register) and Marine Works whom have taken on the role of data standardization within the development group of SMART-Navigation for the establishment of specification for SMART-Navigation service made by service developer.

# Discussion

The SMART-Navigation project is organized and funded by Ministry of Oceans and Fisheries of the Republic of Korea to implement e-navigation services in its waters based on IMO’s e-Navigation concept adding additional Republic of Korea specific features for non-SOLAS ships services and broadband communication using navigation-dedicated LTE networks (LTE-Maritime) for improved connectivity between ship and shore.

The project aims to implement specific parts of Maritime Services 1-7, 10-12, 14 and 15. To establish these Maritime Services the project will implement six technical services: (1) Navigation Monitoring & Assistance Service (NAMAS), (2) Ship-borne System Monitoring Service (SBSMS), (3) Safe & Optimal Route Planning Service (SORPS), (4) Real-time Electronic Navigational Chart Distribution & Streaming Service (REDSS), (5) Pilot & Tub Assistance Service (PITAS) and (6) Maritime Environment and Safety Information Service (MESIS). Most of the services are designed for vulnerable ships which statistically have high-accident ratio or higher potential for huge accident impact socially, environmentally or economically.

The SMART-Navigation Project has developed a total of 11 product specifications to provide the in scope Maritime Services in Republic of Korea. The SMART- Navigation services use each product specification to communicate to the target when exchanging data between ship and land, and the message catalog product specification is always delivered together. The product specifications adopt the S-100 Edition 3.0.0 standard and may be upgraded to a later version in future revisions.

Details of the Maritime Services and related product specifications are found in annex 1.

# References

IHO S-100 Universal Hydrographic Data Model, Edition 4.0.0, December 2018.

# Action requested of the Committee

This paper is only for information and requires no action by the committee.

- Annex 1. SMART-Navigation product specification Ver.1

SMART-Navigation Product Specification Ver.1.0

CONTENTS

1	Introduction	6

1.1	General	6

1.2	Background	6

1.3	Structure of this paper	7

1.4	Terminology	7

1.5	Acronyms	7

2	SMART-NAVIGATION Product specification : System(overview)	9

2.1	Criteria	9

2.2	Data covered by SMART-Navigation  	10

2.3	Data package	12

2.4	Message product specification	13

3	SMART-NAVIGATION Product specification : service	14

3.1	Navigation Monitoring & Assistance Service(NAMAS)	14

3.2	Ship-borne System Monitoring Service (SBSMS)	15

3.3	Safe & Optimal Route Planning Service(SORPS)	17

3.4	Real-time Electronic Navigational Chart Distribution & Streaming Service (REDSS)	18

3.5	Pilot & Tug Assistance Service (PITAS)	20

3.6	Maritime Environment and Safety Information Service (MESIS).	23

4	SMART-NAVIGATION Product specification : Message	25

4.1	Message Catalogue	26

4.2   Ship Information	45

4.3	Ship Dynamic	75

4.4   Accident Warning	99

4.5	Ship Alarm & Emergency	123

4.6   ENC Service meta	146

4.7	Pilot service Request	166

4.8   Pilot Schedule	186

4.9	Port Guideline	210

4.10 Ship Density	241

4.11	MIO Service Meta	259

# 1. Introduction

## 1.1 General

The SMART-Navigation product specification is the product specification developed to standardize the data exchange within the SMART-Navigation system.

The SMART-Navigation product specification is designed to assure a high quality of data exchanged between ship and shore (SMART-Navigation center).

The scope of the SMART-Navigation product specification is to unify the data interchange format and to enhance information comprehension in for the end users.

The specification adopts S-100 standards, in which information is generated in encoding format such as 8211, GML, HDF-5, LUA. This means that it can be implemented on different, and often disparate, IT systems. It is this methodology, including the concept of modularization, which may make the specification of interest to the other S-100 based product specifications or e-Navigation services.

## 1.2 BackGround

The SMART-Navigation project is organized and funded by Ministry of Oceans and Fisheries of the Republic of Korea to implement e-navigation services in its waters based on IMO’s e-Navigation concept adding special features: (1) services for non-SOLAS ships and (2) broadband communication using navigation-dedicated LTE networks (LTE-Maritime) for better connectivity between ship and shore.

The project is aiming to implement parts of MS 1-7, MS10-12 and MS14-15. Within these MS’s the project will implement six technical services: (1) Navigation Monitoring & Assistance Service (NAMAS), (2) Ship-borne System Monitoring Service (SBSMS), (3) Safe & Optimal Route Planning Service (SORPS), (4) Real-time Electronic Navigational Chart Distribution & Streaming Service (REDSS), (5) Pilot & Tub Assistance Service (PITAS) and (6) Maritime Environment and Safety Information Service (MESIS). Most of the services are designed for vulnerable ships which are ships with high-accident ratio or huge accident impact socially, environmentally or economically.

- NAMAS monitors navigation of vulnerable ships and gives alarm for navigation assistance to prevent collision and grounding. It uses positional information of ships and their route information when available. LTE-Maritime and VDES are being considered as physical communication links for NAMAS.

- SBSMS monitors on-board systems of passenger ships with Korean flag and other ships requesting the service to detect hazardous events within the ships such as flooding, fire and engine failure. LTE-Maritime and VDES are being considered as physical communication links for SBSMS but not limited to these. Other communication links available on-board of service requesting ship can be used as well.

- SORPS provides safe and optimal routes plan when requested. It can be used for voyage planning by merchant ships or for emergency route guidance for small vessels without navigation-aid systems such as radar and AIS. LTE-Maritime and VDES are being considered as physical communication links for SORPS but not limited to these. Other communication links available on-board of service requesting ship can be used as well.

- REDSS provides ENC of Korean waters for SOLAS and Non-SOLAS ships when requested. It supports streaming as well for small ships without on-board electronic chart system (ECS). Only LTE-Maritime is being considered as physical communication links for REDSS.

- PITAS supports pilotage by providing pilots and tugs with information needed for pilotage. Only LTE-Maritime is being considered as physical communication links for PITAS.

- MESIS provides maritime safety information (MSI) including navigational warning, weather information, hydrographic information and maritime environment information. LTE-Maritime and VDES are being considered as physical communication links for MESIS but not limited to these. Other communication links available on-board of service requesting ship can be used as well

## 1.3 Structure of this Document

The remainder of this document is organized as follows:

- Section 2 lists the exchange data and introduce structure of data package.

- Section 3 presents the exchange scenario for SMART-Navigation   service such as list the using message model, introduction of service, data exchange cycle and situation.

- Section 4 drills down the details of the message model as service components

## 1.4 Terminology

| SMART-Navigation | Korean e-Navigation test-bed project with 6 service |
| --- | --- |
| NAMAS | As one of the smart navigation services, this service monitors the navigation of vulnerable vessels and provides navigational assistance alerts for collision and ground prevention. |
| SBSMS | As one of the smart navigation services, this service monitors on-board systems of passenger ships with Korean flag and other ships requesting the service to detect hazardous events within the ships such as flooding, fire and engine failure. |
| SORPS | As one of the smart navigation services, this service provides safe and optimal routes plan when requested |
| REDSS | As one of the smart navigation services, this service provides ENC of Korean waters for SOLAS and Non-SOLAS ships when requested and It supports streaming as well for small ships without on-board electronic chart system (ECS). |
| PITAS | As one of the smart navigation services, this service supports pilotage by providing pilots and tugs with information needed for pilotage. |
| MESIS | As one of the smart navigation services, this service provides maritime safety information (MSI) including navigational warning, weather information, hydrographic information and maritime environment information |
| Message specification | As a minimum unit module defined for smart navigation data exchange, which is an information unit exchanged when each event occurs. |

## 1.5 Acronyms

AIS	Automatic Identification System

DDS	Data Distribution Service

ECS	Electronic Chart System

ENC	Electronic Navigation Chart

GML	Geography Markup Language

HDF-5	Hierarchical Data Format version 5

IALA	International Association of Marine Aids to Navigation and lighthouse Authorities - AISM

JSON	JavaScript Object Notation

LTE	Long-Term Evolution (telecommunication)

MCC	Maritime Connectivity Platform Connector

MCP	The Maritime Connectivity Platform (formerly the Maritime Cloud)

MMS		Maritime Messaging Service

MRN	Maritime Resource Name

MS	Maritime Service

MSI	Maritime Safety Information

PPU		Pilot Portable Unit

S-100	Universal Hydrographic Data Model (IHO)

UML	Unified Modelling Language

VDES	VHF Data Exchange System

VHF	Very High Frequency

VTS	Vessel Traffic Services

WMTS	Web Map Tile Service

XML	Extensible Mark-up Language

# 2. SMART-NAVIGATION   Product Specification : System overview

## 2.1 Criteria

The SMART-Navigation product specification covers technical information and data structure for a service that wishes to adopt this specification.

The specification implements the S-100 standard, in which information is generated in formats such as GML, LUA, HDF-5, 8211. This means that it can be implemented on different, and often disparate, IT systems. This specification applies modularization concept to enhance development flexibility in consideration of interoperability and maintenance.

SMART-Navigation product specification have developed product specification focusing on the events that occur in each service rather than one service. So, we call the smart navigation product specification a module-based product specification.

As shown in Figure 1, the SMART-Navigation system is made of various entities, and has four main components: MCP Connector, DDS (Data Distribution Service), e-Navigation Service, and Database. Each system entity (a person or an organization) interface with the other services or manages data in SMART-Navigation system.

The data format for internal interface (interchanging data among components) is a bespoke format, while the external interface (produced from SMART-Navigation service and provided by external systems) format is one of the S-100 encoding formats and the other format.

The role of the MCP Connector is to bridge internal and external interfaces to improve service efficiency. All service data based on S-100 encoding formats, that are applicable to the product are gathered and managed in the MCP Connector as shown in figure 1.

Figure 1 The Architecture of SMART-Navigation system

## 2.1.1 Example Data Flow Scenario

Figure 2 below presents an example of the data flow in a simple scenario. Consumers request the optimal safety route with the current location and destination in the smart navigation system. The request message is S-100 GML format. The request message is sent to the MCC via MCP, and the MCC converts the request message in its own bespoke format. The converted message is delivered to SORPS and the optimal route based on request information is generated in the service. The optimal route is delivered to the MCC, and the format is an internal format. The optimal route is converted from MCC to S-100 GML format, and is provided to consumers through MCP.

Figure 2 Data Flow Scenario for SORPS

## 2.2 Data Covered by SMART-Navigation

The SMART-Navigation system covers the exchange of the main information listed in table 1.

Table 1 Ship Information List

| No. | Information | Type of data |
| --- | --- | --- |
| 1 | Vessel Name | Text |
| 2 | IMO No. | Integer |
| 3 | Source MRN of Vessel | URN |
| 4 | Call Sign | Text |
| 5 | MMSI Code | Integer |
| 6 | Ship Yard | Text |
| 7 | Delivery date | Date Time |
| 8 | Vessel Type | Code List |
| 9 | Flag | Text |
| 10 | Registered port | Text |
| 11 | Class Society | Text |
| 12 | Ship Dimensions | Real |
| 13 | Engine Specification | Text |
| 14 | Position | Real |
| 15 | Speed | Real |
| 16 | Ship Motion | Real |
| 17 | Navigation Status | Enumeration |
| 18 | Navigation Equipment Type | Enumeration |
| 19 | Navigation Equipment alarm status | Bool |
| 20 | Engine room alarm type | Enumeration |
| 21 | Engine room alarm Status | Bool |
| 22 | Fire Door position | Text |
| 23 | Fire Door status | Bool |
| 24 | Fire Alarm Count | Integer |
| 25 | Fire Detection Position | Text |
| 26 | Fire Alarm status | Bool |
| 27 | Tank Type | Enumeration |
| 28 | Tank Capacity | Real |
| 29 | Tank Alarm status | Bool |
| 30 | Tank Level Gauge | Real |

Table 2 Accident Information List

| No. | Information | Type of data |
| --- | --- | --- |
| 1 | Collision Risk | Real |
| 2 | Grounding Risk | Real |
| 3 | Closest approach distance | Real |
| 4 | Remaining time to risk | Real |
| 5 | Degree of Risk | Real |
| 6 | Type of Accident | Enumeration |

Table 3 Support Information List

| No. | Information | Type of data |
| --- | --- | --- |
| 1 | ENC Category | Enumeration |
| 2 | ENC Zone | Enumeration |
| 3 | ENC Download | Text |
| 4 | MSI Service Identification Information | Text |
| 5 | Korea Sea Area | Enumeration |

Table 4 Route Information List

| No. | Information | Type of data |
| --- | --- | --- |
| 1 | Number of way Points | Real |
| 2 | Way point latitude | Real |
| 3 | Way point longitude | Real |
| 4 | Distance from waypoint to next waypoint | Real |
| 5 | Time to final destination | Real |
| 6 | Estimated arrival time | Real |
| 7 | Route type | Enumeration |

Table 5 Port Information List

| No. | Information | Type of data |
| --- | --- | --- |
| 1 | Port Name | Text |
| 2 | Navigation Section | Text |
| 3 | Navigation Rule | Text |
| 4 | Environmental Limit | Text |
| 5 | Tug Boat | Text |
| 6 | Pilot | Text |
| 7 | Pilot Service Area | Text |
| 8 | Pilot Card Image | Image |
| 9 | Berthing | Text |
| 10 | Density Zone | Real |
| 11 | Density Value | Real |
| 12 | Ship Count | integer |

## 2.3 Data Package

Information produced in accordance with S-100 standard is achieved in a modular form, called "Message specification", which is defined as the smallest data unit within a SMART-Navigation product model.

SMART-Navigation defines data package as containers for data exchange.

The data package is the collection of all the message specifications related to a given service in a given configuration.

As shown in Figure 3, a data package contains the following kind of information:

- An identification data with message specification identification information such as message type, size, name etc.

- A content data, which is different, depending on the service such as ship information, ship alarm etc.

Figure 3 Configuration of Data Package

The message specifications that SMART-Navigation product specification model makes use of are the following:

- Message Catalogue,

- Ship Information,

- Ship Dynamic,

- Accident Warning

- Ship Alarm & Emergency,

- ENC Service Meta,

- Pilot Service Request,

- Pilot Schedule,

- Port Guideline,

- Ship Density,

- MIO Service Meta

## 2.4 Message specification

The message specification is the smallest data unit, and has related data classes, and purposes to reduce redundant information and to increase consistency. It has been developed to cater for several different types of services.

Each message specification involves purpose, scope, type of data, establishing the business rules for production and exchange. All message specifications based on the common basic structure which is defined in the S-100 standard.

Message standards can be divided into three categories according to usage characteristics;

- Message specification with identifying message purpose: Message catalogue

- Message specifications commonly used in multiple service: Ship information, Ship dynamic.

- Message specifications related to a specific service: Accident Warning, Ship Alarm & Emergency, ENC Service Meta, Pilot Service Request, Pilot Schedule, Port Guideline, Ship Density, MIO Service Meta.

# 3. SMART-NAVIGATION Product Specification : Service

## 3.1 Navigation Monitoring & Assistance Service (NAMAS)

NAMAS provides a vessel in danger with identified risk scenario information such as collision and grounding risks based on AIS Information. NAMAS share information from vessel at risk, in real time, with relevant organizations for early response purposes in case of maritime accident

### 3.1.1 Exchange Scenario

### 3.1.2 Ship-Shore Exchange Message

| No. | Message | Communication Method(Encoding) | Status | Description |
| --- | --- | --- | --- | --- |
| 1 | Accident Warning | Unicasting(GML) | Shore→ Vessel | - Provides vessel with possible collision scenarios (excluding wreck) between ship and object. - Provides ship accident information to vessels sailing nearby. |

### 3.1.3 Use Case of Ship-Shore Exchange Message

| Name | Accident Risk |
| --- | --- |
| Description | Provides vessel with possible collision or grounding scenarios (excluding wreck) between ship and object. |
| Data | Issue Date, Status, Accident Type, Ship MMSI, Ship Latitude, Ship Longitude, Ship Direction, Ship ID, Ship Speed, Navigation Situation, Collision Latitude (Prediction), Collision Longitude (Prediction), CPA Distance, CPA Time, Distance to the collision, Time to the collision, Collision Risk Level, Predicted Collision Area |
| Communication | LTE-M, VDES |
| Frequency | When Potential collision scenario identified in NAMAS |
| Trigger Rule | initial message and update message |
| Message Specification | Message Catalogue, Accident Warning |
| Name | Accident Notice |
| Description | Provide ship accident information to vessels sailing nearby. |
| Data | Issue date, Accident time, Accident Type, MMSI of the Accident ship, Accident Ship ID, Accident ship Latitude, Accident ship Longitude |
| Communication | LTE-M, VDES |
| Frequency | when accident occurs |
| Trigger Rule | Once |
| Message Specification | Message Catalogue, Accident Warning |

### 3.1.5 NAMAS Data Mode

## 3.2 Ship-borne System Monitoring Service (SBSMS)

This service assesses abnormal symptoms from ship's sensor and makes/supplies recommendations of response. The purpose of this service is to prevent marine accidents and prevent secondary accidents.

### 3.2.1 Exchange Scenario

### 3.2.2 Ship-Shore Exchange Message

| No. | Message | Communication Method(Encoding) | Status | Description |
| --- | --- | --- | --- | --- |
| 1 | Ship Alarm & Emergency | Unicasting (GML) | Vessel→Shore Shore→ Vessel | - Provides sensor information (fire alarm, equipment information) and tank loading status information of a vessel navigating a specific sea area - Provide guidance on risk situations based on vessel alert information |

### 3.2.3 Use Case of Ship-Shore Exchange Message

| Name | Sending ship alarm and tank data |
| --- | --- |
| Description | Define sensor-based alarm information installed on ship and Information on tank type, capacity and number |
| Data | Navigation device (list, alarm, operating), main engine, generator, steering gear, No. of generator, fire door, file alarm, Tank type, tank capacity, No. of tank, tank alarm type, tank alarm status, tank gauge |
| Communication | LTE-M, VDES |
| Frequency | When navigating to a specific sea area(Korean Coastal waters) |
| Trigger Rule | repeat at certain intervals |
| Message Specification | Message Catalogue, Ship Alarm & Emergency, Ship Dynamic |
| Name | Provide emergency guideline |
| Description | Emergency and guidance codes information |
| Data | Emergency occurrence time, emergence type, emergence level, emergency guidance code |
| Communication | LTE-M, VDES |
| Frequency | When navigate a specific sea area |
| Trigger Rule | initial message and update message |
| Message Specification | Message Catalogue, Ship Alarm & Emergency, Ship Dynamic |

### 3.2.5 SBSMS Data Model

## 3.3 Safe & Optimal Route Planning Service (SORPS)

This service provides vulnerable vessel with optimal route information based on ship traffic, sea area environment, marine meteorology.

### 3.3.1 Exchange Scenario

3.3.2 Ship-Shore Exchange Message

| No. | Message | Communication Method(Encoding) | Status | Description |
| --- | --- | --- | --- | --- |
| 1 | Route | Unicasting(GML) | Vessel → Shore, Shore → Vessel | - Provides a vessel with optimal safety routes from current location to destination. |

### 3.3.3 Use Case of Ship-Shore Exchange Message

| Name | Route |
| --- | --- |
| Description | Define information about the ship’s route |
| Data | ENC Version, Voyage No., No. of Waypoint, Waypoint of version, Position of Waypoint, Waypoint Area, safety isobathymetric line, Safety Water Level, Distance to next waypoint, Annotation of ETA/ETD, Annotation of resolution target, Annotation of Route, Expected ETD, Recommended ETD, Expected ETA, Recommended ETA, COG, Voyage Name, Writer, Status of Voyage, Type of Voyage, Vessel Name, Ship MMSI, Ship IMO No., Vessel Capacity, Cargo, Wind, Ship Maximum Speed |
| Communication | LTE-M, VDES |
| Frequency | On request of ship's route, on response with ship's route |
| Trigger Rule | initial message and update message |
| Message Specification | Message Catalogue, S-421 |

### 3.3.5 SORPS Data Model

## 3.4 Real-time Electronic Navigational Chart Distribution & Streaming Service (REDSS)

This service provide ENC download and update services to SOLAS vessels, and provide certified electronic streaming services to Non-SOLAS vessels through Mobil App.

3.4.1 Exchange Scenario

### 3.4.2 Ship-Shore Exchange Message

| No. | Message | Communication Method(Encoding) | Status | Description |
| --- | --- | --- | --- | --- |
| 1 | ENC Service Meta | Unicasting (GML) | Vessel→Shore Shore→Vessel | - Provides requirements and related attribute information of vessel's electronic chart system requests and responses. - When first time using REDSS, Provides authentication keys for electronic chart streaming services. |

### 3.4.3 Use Case of Ship-Shore Exchange Message

| Name | ENC Service Request & Response |
| --- | --- |
| Description | Define ENC property information for request/response |
| Data | ENC type, device information(license, maker, serial, model), ship’s location, ENC zone, download route, file size, file download URL |
| Communication | LTE-M, VDES |
| Frequency | when requesting ENC data, when response with ENC data |
| Trigger Rule | Once |
| Message Specification | Message Catalogue, ENC Service Meta |
| Name | Streaming certification |
| Description | Define authentication information for using streaming services. |
| Data | Certification Key, WMTS URL |
| Communication | LTE-M, VDES |
| Frequency | When the app is installed |
| Trigger Rule | Once |
| Message Specification | Message Catalogue, ENC Service Meta |

### 3.4.5 SBSMS Data Model

## 3.5 Pilot & Tug Assistance Service (PITAS)

This Service provides necessary information for vessel departure/arrival in pilot area with information such as meteorological information and maritime traffic information. Tugs receive necessary information for berthing according to pilot's order.

3.5.1 Exchange Scenario

### 3.5.2 Use Case of Ship-Shore Exchange Message

| No. | Message | Communication Method(Encoding) | Status | Description |
| --- | --- | --- | --- | --- |
| 1 | Pilot schedule | Unicasting (GML) | Vessel→Shore Shore→Vessel | - Provide information on the request for the pilot. |
| 2 | Ship density | Unicasting (GML) | Shore→Vessel | - Provides a ship density in specific areas of the port. |
| 3 | Port guideline | Unicasting (GML) | Shore→Vessel | - Provides ship in the pilot area with relevant safety information needed for departure and arrival. |

### 3.5.3 Ship-Shore Exchange Message Definition

| Name | Pilot schedule Request & Responce |
| --- | --- |
| Description | Define information to request a pilot |
| Data | Call sign, pilot request number, request status, request time, pilot information, pilot service area, target ship information pilot comment, towing information, pilot card information |
| Communication | LTE-M, VDES |
| Frequency | when requesting pilot schedule, when responding with pilot schedule |
| Trigger Rule | Once |
| Message Specification | Message Catalogue, Ship information, Ship dynamic, Pilot Schedule, Pilot Service Request |
| Name | Ship Density Request & Responce |
| Description | Gives ship density within 5 km radius of target vessel |
| Data | Ship information (MMSI, location), density measuring area, density |
| Communication | LTE-M, VDES |
| Frequency | On Request |
| Trigger Rule | once |
| Message Specification | Message Catalogue, Ship Density, Pilot Service Request |
| Name | Port Guideline Request & Responce |
| Description | Define general information in a port |
| Data | Port name, cargo & size restrict information, passage information(navigation area, passage time), tugboat information |
| Communication | LTE-M, VDES |
| Frequency | On Request |
| Trigger Rule | Once |
| Message Specification | Message Catalogue, Port Guideline, Pilot Service Request |

### 3.5.5 PITAS Data Model

## 3.6 Maritime Environment and Safety Information Service (MESIS)

This service provides maritime safety information service (MS 5), navigation publication service (MS 12), marine weather service (MS 14), and dynamic waterway information service (MS 15).

### 3.6.1 Exchange Scenario

### 3.6.2 Ship-Shore Exchange Message

| No. | Message | Communication Method(Encoding) | Status | Description |
| --- | --- | --- | --- | --- |
| 1 | Marine Information Object Service Support | Unicasting (GML) | Vessel→Shore Shore→Vessel | - Provides maritime safety information on the Korean coast. |

### 3.6.3 Use Case of Ship-Shore Exchange Message

| Name | Marine Information Object Service Support |
| --- | --- |
| Description | Define property information for requesting/responding maritime safety information |
| Data | S-10x identification number, S-10x version, spatial information type, request location, range(meter), service request time, service end time, catalogue list |
| Communication | LTE-M, VDES |
| Frequency | On Request |
| Trigger Rule | Once |
| Message Specification | Message Catalogue, MIO Service Meta |

### 3.6.5 MESIS Data Model

# 4. SMART-NAVIGATION Product SPECIFICATION: Message

Message Specification consist of the following 11 Product Models

- Message Catalogue : Information about message identification, such as sender and receiver

- Ship Information : Information on ship specifications, including ship name, MMSI

- Ship Dynamic : Ship dynamic information, including current location and status

- Accident Warning : Information that includes possible collision scenarios between ships and objects except for collision into subsurface shipwrecks, and location and basic information of the accident ship.

- Ship Alarm & Emergency : Information on the ship's alarm status and status-based guideline

- ENC Service Meta : Information that include meta information to request & reply ENC Streaming service and ENC downloading service.

- Pilot Service Request : Information including request & reply meta information to use pilot schedule, Port Guidline, ship density product specification.

- Pilot Schedule : Information that include pilot information containing tug.

- Port Guideline : Information provided to pilots in order to smoothly perform the pilotage.

- Ship Density : Information on the density level of ships in specific ports for safe pilotage & towage.

- MIO Service Meta : Information that include meta information to request & reply MSI information such as S-124, S-421.

## 4.1 Message Catalogue

### 4.1.1 Overview

### 4.1.1.1 Introduction

This document has been produced by the member of SMART-Navigation project team(Korea Register, Marine-Works) in response to a requirement(SMART-Navigation project manager) to more easily identify module product specifications at the SMART-Navigation devices(Electronic Chart Display & Information Systems and Electronic Chart System) or operating systems. It is based on the IHO S-100 framework specification and the ISO 19100 series of standards. It is a textTable product specification that is primarily intended for encoding the kind and size of exchanged product specification, for Identification purposes.

### 4.1.1.2 References

IHO S-100 IHO Universal Hydrographic Data Model Edition 4.0.0 (December 2017).

ISO 8601. 2004. Data elements and interchange formats - Information interchange - Representation of dates and times. 2004.

ISO 3166-1. 1997. Country Codes. 1997.

ISO 10646-1 Information technology - Universal Multiple-Octet Coded Character Set (UCS) - Part 1: Architecture and Basic Multilingual Plane

ISO 19101-2:2008 Geographic Information - Rules for Application Schema

ISO/TS 19103:2005 Geographic Information - Conceptual schema language

ISO 19106:2004 Geographic Information – Profiles

ISO 19107:2003 Geographic Information – Spatial schema

ISO 19109:2005 Geographic Information - Rules for Application Schema

ISO 19111:2003 Geographic Information - Spatial referencing by coordinates

ISO 19115:2003+Corr1 (2006) Geographic Information - Metadata

ISO 19115-2:2009 Geographic Information - Metadata: Extensions for imagery and gridded data

ISO 19123:2005 Geographic Information - Schema for coverage geometry and functions

ISO 19129:2009 Geographic Information - Imagery gridded and coverage data framework

ISO 19131:2007 Geographic Information - Data product specifications

ISO 19136:2007 Geographic Information – Geography Markup Language

ISO 19136-2:2015, Geographic Information – Geography Markup Language.

ISO/TS 19139, Geographic Information – Metadata – XML schema implementation.

### 4.1.1.3 Terms, definitions and abbreviations

Terms and Definitions

The S-100 framework is based on the ISO 19100 series of geographic standards. The terms and definitions provided here are used to standardize the nomenclature found within that framework, whenever possible. They are taken from the references cited in clause 2.1. Modifications have been made when necessary.

- Application

Manipulation and processing of data in support of user requirements (ISO 19101)

- Application schema

Conceptual schema for data required by one or more applications (ISO 19101)

- Conceptual model

Model that defines concepts of a universe of discourse (ISO 19101)

- Conceptual schema

Formal description of a conceptual model (ISO 19101)

- Coverage

Feature that acts as a function to return values from its range for any direct position within its spatial, temporal or spatiotemporal domain (ISO 19123)

- Data product

Dataset or dataset series that conforms to a data product specification

- Data product specification

Detailed description of a dataset or dataset series together with additional information that will enable it to be created, supplied to and used by another party

- Dataset

Identifiable collection of data (ISO 19115)

- Feature

Abstraction of real world phenomena (ISO 19101)

- Feature association

Relationship that links instances of one feature type with instances of the same or a different feature  type (ISO19110)

- Feature attribute

Characteristic of a feature (ISO 19101)

- Geographic data

Data with implicit or explicit reference to a location relative to the Earth (ISO 19109)

- Metadata

Data about data (ISO 19115)

- Model

Abstraction of some aspects of reality (ISO 19109)

- Portrayal

Presentation of information to humans (ISO 19117)

- Quality

Totality of characteristics of a product that bear on its ability to satisfy stated and implied needs (ISO 19101)

Abbreviations

This product specification adopts the following convention for symbols and abbreviated terms:

- ASCII 	American Standard Code for Information Interchange

- ECDIS	Electronic Chart Display and Information Systems

- ECS	Electronic Chart Systems

- ENC	Electronic Navigational Chart

- GML	Geography Markup Language

- IHO	International Hydrographic Organization

- IOC	International Oceanographic Commission

- ISO	International Organization for Standardization

- UML	Unified Modelling Language

- URI	Uniformed Resource Identifier

- URL 	Uniform Resource Locator

- WMS	Web Map Service

- WFS	Web Feature Service

- WWW 	World Wide Web

- XML	Extensible Markup Language

- XSLT	eXtensible Stylesheet Language Transformations

### 4.1.1.4 General Data Product Description

Informal Description of Data Product

This clause contains general information about the data product

- Title:  	Message Catalogue Product Specification.

- Abstract: 	 Message Catalogue product specification provides the means to identify of SMART-Navigation service in the maritime domain such as file name, file size, type of product specification.   This may include details of attribute of exchanged product specification, target information and information of exchanged file for utilizing SMART-Navigation service.

- Content:	Datasets conforming to this specification will contain relevant exchange message meta information of product specification for the SMART-Navigation services.

- Spatial Extent:	Korean Coast

- Specific Purpose: Describing SMART-Navigation service in the maritime domain for utilization in ECDIS & ECS, and to introduce method about the acquisition of data related to SMART-Navigation service to interested stakeholders.

### 4.1.1.5 Data product specification metadata

This information uniquely identifies this Product Specification and provides information about its creation and maintenance. For further information on dataset metadata see the metadata clause.

- Title:	Message Catalogue Information

- S-100 Version: 	4.0.0

- Message Catalogue Version:	1.0.0

- Date: 	2019-03-08

- Language: 	English

- Classification: 	Unclassified

- Contact: 	32, Yuseong-daero 1312beon-gil, Yuseong-gu, Daejeon, Korea 32(Jang Dong) inside KRISO

- URL: 	http://www.smart-navigation.org

- Identifier: 	Message Catalogue

- Maintenance: 	Amendments to this specification will be produced on a needs basis. For reporting issues with this specification which need correction, use the contact information.

### 4.1.2 Specification Scopes

This product specification describes one data product and therefore requires only one scope which is described below:

- Scope ID:  		Message Catalogue datasets.

- Hierarchical level: 		MD_ScopeCode 005

- Hierarchical level name: 	dataset.

- Level description:		information applies to the dataset

- Extent:	EX_Extent.description: Korean Coast

### 4.1.3 Dataset identification

This section describes how to identify data sets that conform to the specification. A dataset that conforms to this Product Specification may be identified by its discovery metadata as defined in clause 12 of this specification. The information identifying the data product may include the following items from S-100 3.0.0 clause 11-6 (adapted from ISO 19115).

| Title | Message Catalogue |
| --- | --- |
| Abstract | Message Catalogue dataset is a textTable dataset containing Identification information of exchanged product specification in SMART-Navigation |
| Acronym | MC |
| Content | Type of Message, Name of exchanged product specification, Size of exchanged file. |
| Topic Category | Structure |
| Purpose | Support for easy identification of product standards that are exchanged between ship and shore for Solution such as ECDIS |
| Language | EN Additional values if any use CharacterString values from ISO 639-2 |

### 4.1.4 Data Content and structure

### 4.1.4.1 Introduction

The Message Catalogue product is based on the S-100 General Feature Model (GFM), and is a feature-based vector product. All Message Catalogue features and information classes are derived from one of the abstract classes FeatureType and InformationType defined in the Message Catalogue application schema, which realize the GFM meta-classes S100_GF_FeatureType and S100_GF_InformationType respectively.

This clause contains the Application Schema expressed in UML and an associated Feature Catalogue.

### 4.1.4.2 Application Schema

The UML models in this clause are segments of the overall Message Catalogue application schema, and include overviews of the feature classes, information classes, meta features, spatial types, and the relationships between them.

This clause contains a general overview of the classes and relationships in the Message Catalogue application schema. Detailed information about how to use the feature types and information types to encode Message Catalogue information is provided in the SMART-Navigation Data Classification and Encoding Guide (DCEG).

The following conventions are used in the UML diagrams depicting the application schema:

- Standard UML conventions for classes, associations, inheritance, roles, and multiplicities apply. These conventions are described in Part 1 of S-100.

- Italic font for a class name indicates an abstract class.

- Feature classes are depicted with pink background; the dark shade for abstract feature classes and the light shade for ordinary (non-abstract) feature classes.

- Complex attributes are depicted with a pink background.

- Enumeration lists and codelists are depicted with a tan background. The numeric code corresponding to each listed value is shown to its right following an ‘=’ sign.

- No significance attaches to the colour of associations. (Complex diagrams may use different colours to distinguish associations that cross one another.)

- Subclasses inherit the attributes and associations of their superclasses at all levels, unless such inheritance is explicitly overridden in the subclass.

### 4.1.4.2.1 Domain model

Figure 4 Message Catalogue Overview

### 4.1.4.3 Feature Catalogue

### 4.1.4.3.1 Introduction

The Feature Catalogue describes the feature types, information types, attributes, attribute values, associations and roles which may be used in the product.  The Message Catalogue Feature Catalogue is available in an XML document which conforms to the S-100 XML Feature Catalogue Schema.  Simple attributes used in this specification are listed in below.

Name: 	Message Catalogue Feature Catalogue

Scope: 	Smart-Navigation

Version Number: 	1.0.0

Version Date:	2019-03-15

Producer:	SMART Navigation Project

Language: 	                 English

### 4.1.4.3.1.1 Feature Types

Feature types contain descriptive attributes that characterize real-world entities. The word ‘feature’ may be used in one of two senses – feature type and feature instance. A feature type is a class and is defined in a Feature Catalogue. A feature instance is a single occurrence of the feature type and represented as an object in a dataset. A feature instance is located by a relationship to one or more spatial instances. A feature instance may exist without referencing a spatial instance.

### 4.1.4.3.1.2 Geographic

A Geographic (Geo) feature type carries the descriptive characteristics of a real-world entity.

### 4.1.4.3.1.3 Meta

Meta features contain information about other features within a dataset. Information defined by meta features override the default metadata values defined by the dataset descriptive records. Meta attribution on individual features overrides attribution on meta features.

### 4.1.4.3.1.4 Feature Relationship

A feature relationship links instances of one feature type with instances of the same or a different feature type.

### 4.1.4.3.1.5 Information Types

Information types are identifiable pieces of information in a dataset that can be shared between other features. They have attributes but have no relationship to any geometry; information types may reference other information types.

### 4.1.4.3.1.6 Attributes

SMART-Navigation product defines attributes as either simple or complex.

### 4.1.4.3.1.6.1 Simple Attributes

SMART-Navigation product uses nine types of simple attributes; they are listed in the following table:

| Type | Definition |
| --- | --- |
| Enumeration | A fixed list of valid identifiers of named literal values |
| Boolean | A value representing binary logic. The value can be either True or False. The default state for Boolean type attributes (i.e. where the attribute is not populated for the feature) is False. |
| Real | A signed Real (floating point) number consisting of a mantissa and an exponent |
| Integer | A signed integer number. The representation of an integer is encapsulation and usage dependent. |
| Text | An arbitrary-length sequence of characters including accents and special characters from a repertoire of one of the adopted character sets |
| Date | A date provides values for year, month and day according to the Gregorian Calendar. Character encoding of a date is a string which must follow the calendar date format (complete representation, basic format) for date specified by ISO 8601:1988. EXAMPLE 19980918 (YYYY-MM-DD) |
| Time | A time is given by an hour, minute and second. Character encoding of a time is a string that follows the local time (complete representation, basic format) format defined in ISO 8601:1988. EXAMPLE 183059 or 183059+0100 or 183059Z |
| Date and Time | A DateTime is a combination of a date and a time type. Character encoding of a DateTime shall follow ISO 8601:1988 EXAMPLE 19850412T101530 |
| Codelist | A type of flexible enumeration. A code list type is a list of literals which may be extended only in conformance with specified rules. |

### 4.1.4.3.1.6.2 Complex Attributes

Complex attributes are aggregations of other attributes that are either simple or complex. The aggregation is defined by means of attribute bindings.

### 4.1.4.3.1.6.3 Geometric Representation

Geometric representation is the digital description of the spatial component of an object as described in S-100 and ISO 19107. This product specification uses no geometry attribute.

### 4.1.4.3.2 Feature Catalogue General

The Message Catalogue feature catalogue describes the feature types, information types, attributes, attribute values, associations and roles which may be used in a Message Catalogue data set.

### 4.1.4.3.2.1 Feature Types

- Geographic

- Meta

- Message Catalogue

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| exchangeFile | information related to data file exchanged between shore and ship for smart-navigation | complex attribute | 1..* | mandatory |  |
| messageType | The type of message defined to recognize the purpose of the exchanged message | enumeration | 1 | mandatory |  |
| information | Textual information about the feature. The information may be provided as a string of text or as a file name of a single external text file that contains the text. | complex attribute | 0..* | optional |  |

- Cartographic

- Theme

### 4.1.4.3.2.2 Feature Relationship

- N/A

### 4.1.4.3.2.3 Information Relationship

| Role Type | Association Name | Role | Feature | Multiplicity |
| --- | --- | --- | --- | --- |
| Association | Identification | service identification | MessageCatalogue | 1 |

### 4.1.4.3.2.4 Information Type

- Service Identification

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| sourceMRN | The MRN of the source producing the service data | URN | 1 | mandatory |  |
| destinationMRN | The MRN of the destination receiving the service data | URN | 1 | mandatory |  |
| timeOfIssue | The issued date and time of the data | dateTime | 1 | mandatory |  |

### 4.1.4.3.2.5 Simple Attribute

- Simple Attribute List

| Name | Description | Data Type | Remarks |
| --- | --- | --- | --- |
| language | The method of human communication, either spoken or written, consisting of the use of words in a structured and conventional way.(ISO 639-3) | text |  |
| text | A non-formatted digital text string. | text |  |
| nameOfFile | Name of file exchanged between shore and Ship for smart-navigation | enumeration |  |
| sizeOfFile | Volume of file exchanged between shore and Ship for smart-navigation | real |  |
| typeOfFile | Encoding format of exchangeable file | enumeration |  |
| codeOfStandard | Product Specification Code Number presented by IHO | text |  |
| nameOfModel | Name of data model exchanged between shore and Ship for smart-navigation | enumeration |  |
| versionOfModel | Version of data model exchanged between shore and Ship for smart-navigation. ( ex. 3.5.2 ) | text |  |
| sourceMRN | The MRN of the source producing the service data | URN |  |
| destinationMRN | The MRN of the destination receiving the service data | URN |  |
| timeOfIssue | The issued date and time of the data | dateTime |  |
| messageType | The type of message defined to recognize the purpose of the exchanged message | enumeration |  |

- type Of File

| Value | Label | Description |
| --- | --- | --- |
| 1 | GML | Geographic Markup Language |
| 2 | HDF5 | Hierarchical Data Format (HDF) is a set of file formats (HDF4, HDF5) designed to store and organize large amounts of data |
| 3 | ISO8211 | Digital Information Geographic Exchange Standard Format |
| 4 | LUA | Lua combines simple procedural syntax with powerful data description constructs based on associative arrays and extensible semantics. Lua is dynamically typed, runs by interpreting bytecode with a register-based virtual machine, and has automatic memory management with incremental garbage collection, making it ideal for configuration, scripting, and rapid prototyping. |
| 5 | XML | The Extensible Markup Language (XML) is a simple text-based format for representing structured information: documents, data, configuration, books, transactions, invoices, and much more. It was derived from an older standard format called SGML (ISO 8879), in order to be more suitable for Web use. |

- name Of Model

| Value | Label | Description |
| --- | --- | --- |
| 1 | Ship Information | A model that contains unique property information of the ship. |
| 2 | Ship Dynamic | Information provided to monitor the condition of the ship, such as the position and attitude of the ship, which changes according to the state of navigation of the ship. Ship equipment status / sensor information excluded. |
| 3 | Ship Alarm | Model including sensor information of ship during voyage such as alarm occurrence, machine temperature, machine pressure etc. |
| 4 | Pilot Service Request | Model for requesting Pilot Service (Pilot and tug schedule, Ship Information, Pilot Note, Tugboat Information, Pilot card Information, Ship Position Predict, Ship Density, Port Guideline) |
| 5 | Pilot Schedule | Models that include information on requests and response for departure/arrival, such as weather information, traffic information, dynamic information, and safety information |
| 6 | Port Guideline | Models that contain general information on ports, such as regulations and precautions |
| 7 | Ship Density | A model containing the density level of a ship operating in a specific zone based on the requested vessel location |
| 8 | Accident Warning | A model containing scenarios in which a ship accident such as collision or grounding |
| 9 | ENC Service Meta | Model with information needed for ENC request and respond |
| 10 | MIO Service Support | A model including authentication information request / response related property information for ENC streaming service |

- message Type

| Value | Label | Description |
| --- | --- | --- |
| 1 | alarm | Message is transmitted from the Smart Navigation Center to the ship or from the ship to the Smart Navigation Center without a request message |
| 2 | request | Message requesting service from the ship to the Smart Navigation Center |
| 3 | response | Messages delivered to the ship from the Smart Navigation Center in response to a request |

### 4.1.4.3.2.6 Complex Attribute

1) information

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| language | The method of human communication, either spoken or written, consisting of the use of words in a structured and conventional way.(ISO 639-3) | text | 0..1 | option |  |
| text | A non-formatted digital text string. | text | 1 | mandatory |  |

- exchangeFile

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| typeOfFile | Encoding format of exchangeable file | text | 0..1 | option |  |
| sizeOfFile | Volume of file exchanged between shore and Ship for smart-navigation | text | 1 | mandatory |  |
| nameOfFile | Name of data model exchanged between shore and Ship for smart-navigation | text | 1 | mandatory |  |
| exchangeModel | data model information exchanged between shore and Ship for smart-navigation | complex attribute | 1 | mandatory |  |

- exchangeModel

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| codeOfStandard | Product Specification Code Number presented by IHO | text | 0..1 | option |  |
| nameOfModel | Name of data model exchanged between shore and Ship for smart-navigation | enumeration | 1 | mandatory |  |
| versionOfModel | Version of data model exchanged between shore and Ship for smart-navigation. ( ex. 3.5.2 ) | text | 1 | mandatory |  |

### 4.1.5 Coordinate Reference System(CRS)

N/A

### 4.1.6 Data Quality

### 4.1.6.1 Introduction

Message Catalogue products must be tested with the Message Catalogue specific checks prior to release by the data producer. The data producer must review the check results and address any issues to ensure sufficient quality of the data products. The checks are a mix of data format validation checks, conformance to standard checks and logical consistency checks. But the checklist has not been written yet. It will be updated in the next version.

### 4.1.7 Data Capture and classification

See DCEG for this section.

### 4.1.8 maintenance

### 4.1.8.1 Introduction

Datasets are maintained as needed and must include mechanisms for Message Catalogue updating. Data updates will be made by new editions. The maintenance and update frequency of Message Catalogue datasets should be defined by the producers (official national authority) implementing this specification.

Data Producers must use applicable sources to maintain and update data and provide a brief description of the sources that were used to produce the dataset in the appropriate metadata field.

The data product shall provide information on how the data is maintained and should describe the principles and criteria applied in maintenance regime. This should specify the expected frequency of updates.

Table 6 Maintenance and update frequency

| Item Name | Description | Multiplicity | Type |
| --- | --- | --- | --- |
| maintenanceAndUpdateFrequency | Frequency with which changes and additions are made to the data product (per update scope) | 1..* | MD_MaintenanceInformation (ISO 19115) |
| dataSource | Identification of the kinds of data sources usable to produce datasets | 1..* | LI_Source (ISO 19115) |
| productionProcess | Textual description of the production process applicable to the datasets (per scope or data source) | 1..* | LI_ProcessStep (ISO 19115) |

### 4.1.8.2 Production process for base and update datasets

Data Producers should follow their established production processes for maintaining and updating datasets. Data is produced against the DCEG and checked against the appropriate set of validation rules in Appendix E. (Appendix E is not included in this release.)

### 4.1.8.3 Dataset updates

N/A (Datset Update is not relevant in this model)

### 4.1.8.4 Support file updates

N/A (Support file is not included in this model)

### 4.1.8.5 Feature and portrayal catalogues

For each new version of the Message Catalogue Product Specification a new feature catalogue will be released. The system must be able to manage datasets and their catalogues that are created on different versions of the Message Catalogue product specification (Portrayal Catalogue is not defined in this model.)

### 4.1.8.6 Feature history, versions, and change tracking

If applications or production systems require versioning of individual instances of feature or information types, maintenance of histories, or change tracking, the methods for versioning, history management, and change tracking and display are left to the application or production system.

### 4.1.8.7 Dataset encryption

Details about dataset encryption are still to be determined, and may mirror the method described in S-101.

### 4.1.9 Portrayal

Portrayal is not defined in this version of Message Catalogue Product Specifications. Users are free to choose the means and methodology of portrayal as they see best suited for their needs. It should be noted that future versions of Message Catalogue may include a portrayal catalogue, and any implementer should therefore anticipate this, and make sufficient provisions in any system supporting Message Catalogue.

### 4.1.10 data product format(encoding)

### 4.1.10.1 Introduction

The GML encoding of Message Catalogue datasets is based on the S-100 profile of GML 3.2.1.

Format Name: Message Catalogue GML

Version:	1.0

Character Set: UTF-8

Specification: SMART-Navigation Product Specification

### 4.1.10.2 Encoding of Latitude and Longitude

N/A (This model has no geometry attribute.)

### 4.1.10.2.1 Encoding of coordinates as decimals

N/A (This model has no geometry attribute.)

### 4.1.10.3 Text Attribute

Character strings must be encoded using the character set defined in ISO 10646-1, in Unicode Transformation Format-8 (UTF-8).

### 4.1.10.4 Mandatory Attribute Values

There are four reasons why attribute values may be considered mandatory:

- They determine whether a feature is in the display base,

- Certain features make no logical sense without specific attributes,

- Some attributes are necessary to determine which symbol is to be displayed,

- Some attributes are required for safety of navigation.

All mandatory attributes are identified in the Feature Catalogue

### 4.1.10.5 Unknow Attribute Values

When a mandatory attribute code or tag is present but the attribute value is missing, it means that the producer wishes to indicate that this attribute value is unknown.  Missing mandatory attributes must be “nilled” with a GML nilReason attribute giving the reason for omission.

Optional attributes must be omitted altogether if the value is unknown or missing. They must not be “nilled.”

### 4.1.10.6 Structure of dataset files

### 4.1.10.6.1 Sequence of dataset files

The order of data objects in each dataset file is described below:

Dataset Identification Information

Dataset structure information

Spatial records for by-reference geometries

Point

Multi point

Curve

Composite Curve

Surface

Information objects

Feature objects (Geometry may be encoded inline or by reference.)

Meta features

Geo features

### 4.1.10.7 Numeric Attribute Encoding

Floating point and integer attribute values must not contain leading zeros.  Floating point attribute values must not contain non-significant trailing zeros.

### 4.1.10.8 Object identifiers

The “name” of feature records must provide a unique world-wide identifier of feature records. The “name” of the record is the combination of the subfields agency, featureObjectIdentifier, and featureIdentificationSubdivision elements of the featureObjectIdentifier element of the object.

Features, information types, collection objects, meta features, and geometries (inline or external) are all required by the schema to have a gml:id attribute with a value that is unique within the dataset. The gml:id values must be used as the reference for the object from another object in the same dataset or another dataset.

### 4.1.10.9 Dataset validation

Fields may be repeated or omitted as permitted by the XML schemas and the validation tests. Since XML schema cannot encode rules for conditional presence or attributes, these rules are checked by Schematron rules or other validation code.

### 4.1.10.10 Location of Data Product Format schema Files

The schema file will provide a future location and is available from that location.

### 4.1.10.11 Detailed documentation of Message Catalogue XML Schema

See DCEG for this section.

### 4.1.11 data product delivery

This clause specifies the encoding and delivery mechanisms for a Message Catalogue dataset. Data which conforms to this product specification must be delivered by means of an exchange set.

### 4.1.11.1 Exchange set

Message Catalogue datasets are grouped into exchange sets. Each exchange set consists of one or more Message Catalogue datasets with an associated XML metadata file and a single Exchange Catalogue XML file containing metadata. It may also include one or more support files.

Units of Delivery:				Exchange Set

Transfer Size:					Unlimited

Medium Name:					Digital data delivery

Other Delivery Information:

Each dataset must be contained in a physically separate, uniquely identified file on the transfer medium.

Each exchange set has a single exchange catalogue which contains the discovery metadata for each dataset and references to any support files.

Support files are supplementary information which are linked to the features and information types by attributes. The attributes containing these links are described in the application schema and feature catalogue.

An exchange set may be encapsulated into a form suitable for transmission by a mapping called a transmission encoding. An encoding translates each of the elements of the exchange set into a logical form suitable for writing to media and for transmission online. An encoding may also define other elements in addition to the exchange set contents (i.e., media identification, data extents etc…) and also may define commercial constructs such as encryption and compression methods.

If the data is transformed (e.g., for encryption or compression purposes) its content must not be changed.

This product specification does not define the transmission encoding which must be used as a default for transmission of data between parties.

The exchange set elements are as follows:

Mandatory Elements

- Message Catalogue datasets – GML encoding of features/attributes and their associated geometry and metadata.

- Exchange Catalogue – The XML encoded representation of exchange set catalogue features [discovery metadata].

Optional Elements

- Supplementary files – These are contained within the exchange set as files and the map from the name included within the dataset and the physical location on the media is defined within the Exchange Catalogue.

- Feature Catalogue – If it is necessary to deliver the latest feature catalogue to the end user it may be done using Message Catalogue exchange set mechanism for datasets – i.e., include the updated feature catalogue in an exchange set.

### 4.1.11.2 Dataset

### 4.1.11.2.1 Datasets

Datasets are distributed as files which are part of exchange sets structured as described in this specification. The distribution media are left to the discretion of the producer and distributor.

The following types of dataset files may be produced and contained within an exchange set:

- New dataset and new edition of a dataset (base dataset):  Each new edition of a dataset must have thesame name as the dataset that it replaces.  A new edition can also be AtoN data that has previously been produced for this area.

### 4.1.11.2.2 Dataset size

Datasets must not exceed 1 MB.

### 4.1.11.2.3 Dataset file naming

Dataset files shall be named

CCXXXXXXXX_EEE.GML

The file name forms a unique identifier where:

- CC - The first two characters identify the issuing agency (mandatory).

- The third to tenth characters are optional and may be used in any way by the producer to provide the unique file name. The following characters are allowed in the dataset name, A to Z, 0 to 9 and the special character _ (underscore).

- EEE – new editions and re-issues use 000, and increment until a limit of 999 (mandatory).

- The maximum number of characters preceding _EEE is ten.

- GML – the character sequence “GML” or “gml”.

### 4.1.11.2.4 New Editions and re-issues

This section defines the sequencing of datasets for New Editions and Re-issues. In order to ensure that feature type updates are incorporated into an SYSTEM in the correct sequence without any omission, a number of parameters encoded in the data header and/or metadata are used in the following way:

| edition number | When a dataset is initially created, the edition number 1 is assigned to it. The edition number is increased by 1 at each new edition. |
| --- | --- |

### 4.1.11.2.5 Cancellations

Dataset cancellation is not considered I this product specification. In the SMART Navigation project, the system automatically cancels the dataset after 7 days from the date the dataset is received.

### 4.1.11.3 Support Files

N/A (Support file is not included in this model)

### 4.1.11.4 Exchange Catalogue

N/A (Exchange Catalogue is not used in MSART Navigation)

### 4.1.12 meta data

### 4.1.12.1 Introduction

The Message Catalogue metadata description is based on the S-100 metadata document section, which is a profile of the ISO 19115 standard. These documents provide a structure for describing digital geographic data and define metadata elements, a common set of metadata terminology, definitions and extension procedures.

Two metadata packages are described in this product specification: dataset metadata and exchange set metadata.

Figure 5 Metadata packages

Note 1: Types with CI_, EX_, and MD_ prefixes are from packages defined in ISO 19115 and adapted by S-100. Types with S100_ prefix are from packages defined in S-100.

Note 2: When a dataset is terminated, the purpose metadata field is set to 3 (terminated), and the editionNumber metadata field is set to 0. All other metadata fields must be blank.

### 4.1.12.2 Dataset metadata

Dataset metadata is intended to describe information about a dataset. It facilitates the management and exploitation of data and is an important requirement for understanding the characteristics of a dataset. Whereas dataset metadata is usually fairly comprehensive, there is also a requirement for a constrained subset of metadata elements that are usually required for discovery purposes. Discovery metadata are often used for building web catalogues, and can help users determine whether a product or service is fit for purpose and where they can be obtained.

Table 7 Dataset metadata

| Name | Cardinality | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| S100_DataSetDiscoveryMetadata |  |  |  |  |
| fileName | 1 |  | CharacterString | Dataset file name |
| filePath | 1 |  | CharacterString | Full path from the exchange set root directory |
| description | 1 |  | CharacterString | Short description of the area covered by dataset, e.g., area, harbour, or port name, between two named locations etc. |
| dataProtection | 0..1 | {1} or {2} | Boolean | True : Encrypted False : Unencrypted |
| protectionScheme | 0..1 |  | CharacterString |  |
| digitalSignature | 0..1 |  | CharacterString |  |
| digitalSignatureReference | 0..1 |  | CharacterString |  |
| digitalSignatureValue | 0..1 |  | CharacterString |  |
| copyright | 0..1 |  | MD_LegalConstraints>MD_RestrictionCode <copyright> (ISO 19115-1) |  |
| classification | 0..1 |  | Class MD_SecurityConstraints>MD_ClassificationCode (codelist) ISO 19115-1 | 1. unclassified 2. restricted 3. confidential 4. secret 5. top secret |
| purpose | 1 | {1}, {2} | CharacterString | - New dataset - New edition |
| specificUsage | 1 |  | MD_USAGE>specificUsage (character string) MD_USAGE>userContactInfo (CI_Responsibility) | Brief description of the resource and/or resource series usage |
| editionNumber | 1 | {1} | Integer | When a dataset is initially created, the edition number “1” is assigned to it. The edition number is increased by one with each new edition. |
| updateNumber | 1 |  | CharacterString | Update number 0 is assigned to a new dataset. |
| updateApplicationDate | 1 |  | Date | All updates dated on or before this date must have been applied by the producer. |
| issueDate | 1 |  | Date | Date on which the dataset was generated. |
| productSpecification | 1 |  | S100_ProductSpecification | See Notes below this table for constraints on values. |
| producingAgency | 1 |  | CI_Responsibility>CI_Organisation or CI_Responsibility>CI_Individual | Party responsible for generating the dataset. |
| optimumDisplayScale | 0..1 |  | Integer |  |
| maximumDisplayScale | 0..1 |  | Integer |  |
| minimumDisplayScale | 0..1 |  | Integer |  |
| horizontalDatumReference | 1 | EPSG | CharacterString |  |
| horizontalDatumValue | 1 | 4326 | Integer | WGS84 |
| verticalDatum | 0..1 |  | S100_VerticalAndSoundingDatum | Not included in Message Catalogue |
| soundingDatum | 0..1 |  | S100_VerticalAndSoundingDatum | Not included in Message Catalogue |
| dataType | 1 | GML | CharacterString | The only value allowed is “GML”. |
| otherDataTypeDescription | 0..1 |  | CharacterString |  |
| dataTypeVersion | 1 | 3.2.1 | CharacterString |  |
| dataCoverage | 1..* |  | S100_DataCoverage |  |
| comment | 0..1 |  | CharacterString | Any additional Information |
| layerID | 1..* | S-101 | CharacterString | Dataset must be used with ENC in an ECDIS. |

### 4.1.12.3 Update Dataset Metadata

N/A (Dataset update is not relevant in this model)

### 4.1.12.4 Support file metadata

N/A (Support file is not included in this model)

### 4.1.12.5 Exchange set metadata

N/A (Exchange Catalogue is not used in SMART Navigation)

### 4.1.12.6 Catalogue file Metadata

All Message Catalogue Exchange Catalogue files must contain at least the mandatory metadata elements.

Table 8 Catalogue file metadata

| Name | Multiplicity | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| S100_ExchangeCatalogue |  |  |  |  |
| identifier | 1 |  | S100_CatalogueIdentifier |  |
| contact | 1 |  | S100_CataloguePointOfContact |  |
| productSpecification | 0..1 |  | S100_ProductSpecification | Conditional on all the datasets using the same product specification.. |
| metadataLanguage | 1 | English | CharacterString | All datasets conforming to this PS must use English language. A catalogue in English must be provided. Discovery metadata elements within catalogues have their own locale attributes and may be repeated in languages other than English. |
| exchangeCatalogueName | 0..1 |  | CharacterString | Not used in SMART-Navigation |
| exchangeCatalogueDescription | 1 |  | CharacterString |  |
| exchangeCatalogueComment | 0..1 |  | CharacterString | Any additional Information |
| compressionFlag | 0..1 |  | Boolean | Yes or No |
| sourceMedia | 0..1 |  | CharacterString |  |
| replacedData | 0..1 |  | Boolean |  |
| dataReplacement | 0..1 |  | CharacterString |  |

## 4.2 Ship Information

### 4.2.1 Overview

### 4.2.1.1 Introduction

This document has been produced by the member of SMART-Navigation project team(Korean Register, Marine-Works) in response to a requirement to produce a data product that can be used as a ship’s specification information at the SMART-Navigation services. It is based on the IHO S-100 framework specification and the ISO 19100 series of standards. It is a textTable product specification that is primarily intended for encoding the information of ship’s specification, for sharing target information

### 4.2.1.2 References

IHO S-100 IHO Universal Hydrographic Data Model Edition 4.0.0 (December 2017).

ISO 8601. 2004. Data elements and interchange formats - Information interchange - Representation of dates and times. 2004.

ISO 3166-1. 1997. Country Codes. 1997.

ISO 10646-1 Information technology - Universal Multiple-Octet Coded Character Set (UCS) - Part 1: Architecture and Basic Multilingual Plane

ISO 19101-2:2008 Geographic Information - Rules for Application Schema

ISO/TS 19103:2005 Geographic Information - Conceptual schema language

ISO 19106:2004 Geographic Information – Profiles

ISO 19107:2003 Geographic Information – Spatial schema

ISO 19109:2005 Geographic Information - Rules for Application Schema

ISO 19111:2003 Geographic Information - Spatial referencing by coordinates

ISO 19115:2003+Corr1 (2006) Geographic Information - Metadata

ISO 19115-2:2009 Geographic Information - Metadata: Extensions for imagery and gridded data

ISO 19123:2005 Geographic Information - Schema for coverage geometry and functions

ISO 19129:2009 Geographic Information - Imagery gridded and coverage data framework

ISO 19131:2007 Geographic Information - Data product specifications

ISO 19136:2007 Geographic Information – Geography Markup Language

ISO 19136-2:2015, Geographic Information – Geography Markup Language.

ISO/TS 19139, Geographic Information – Metadata – XML schema implementation.

### 4.2.1.3 Terms, definitions and abbreviations

Terms and Definitions

The S-100 framework is based on the ISO 19100 series of geographic standards. The terms and definitions provided here are used to standardize the nomenclature found within that framework, whenever possible. They are taken from the references cited in clause 2.1. Modifications have been made when necessary.

- Application

Manipulation and processing of data in support of user requirements (ISO 19101)

- Application schema

Conceptual schema for data required by one or more applications (ISO 19101)

- Conceptual model

Model that defines concepts of a universe of discourse (ISO 19101)

- Conceptual schema

Formal description of a conceptual model (ISO 19101)

- Coverage

Feature that acts as a function to return values from its range for any direct position within its spatial, temporal or spatiotemporal domain (ISO 19123)

- Data product

Dataset or dataset series that conforms to a data product specification

- Data product specification

Detailed description of a dataset or dataset series together with additional information that will enable it to be created, supplied to and used by another party

- Dataset

Identifiable collection of data (ISO 19115)

- Feature

Abstraction of real world phenomena (ISO 19101)

- Feature association

Relationship that links instances of one feature type with instances of the same or a different feature type (ISO19110)

- Feature attribute

Characteristic of a feature (ISO 19101)

- Geographic data

Data with implicit or explicit reference to a location relative to the Earth (ISO 19109)

- Metadata

Data about data (ISO 19115)

- Model

Abstraction of some aspects of reality (ISO 19109)

- Portrayal

Presentation of information to humans (ISO 19117)

- Quality

Totality of characteristics of a product that bear on its ability to satisfy stated and implied needs (ISO 19101)

Abbreviations

This product specification adopts the following convention for symbols and abbreviated terms:

- ASCII 	American Standard Code for Information Interchange

- ECDIS	Electronic Chart Display and Information Systems

- ECS	Electronic Chart Systems

- ENC	Electronic Navigational Chart

- GML	Geography Markup Language

- IHO	International Hydrographic Organization

- IOC	International Oceanographic Commission

- ISO	International Organization for Standardization

- UML	Unified Modelling Language

- URI	Uniformed Resource Identifier

- URL 	Uniform Resource Locator

- WMS	Web Map Service

- WFS	Web Feature Service

- www 	World Wide Web

- XML	Extensible Markup Language

- XSLT	eXtensible Stylesheet Language Transformations

### 4.2.1.4 General Data Product Description

Informal Description of Data Product

This clause contains general information about the data product

- Title:  	Ship Information

- Abstract: 	Ship Information product specification is a common product specification for SMART-Navigation that standardizes the ship specification information such as ship name, MMSI, contact information to utilize from a common point of view.

- Content:	Datasets conforming to this specification will contain all relevant ship particular information for the SMART-Navigation.

- Spatial Extent:	Korean Coast

- Specific Purpose:	Describing ship particular information for utilization of SMART-Navigation service.

### 4.2.1.5 Data product specification metadata

This information uniquely identifies this Product Specification and provides information about its creation and maintenance. For further information on dataset metadata see the metadata clause.

- Title: 	SMART-Navigation Ship Information

- S-100 Version: 	4.0.0

- Ship Information Version: 	1.0.0

- Date: 	2018-03-08

- Language: 	English

- Classification: 	Unclassified

- Contact: 	32, Yuseong-daero 1312beon-gil, Yuseong-gu, Daejeon, Korea 32(Jang Dong) inside KRISO

- URL: 	http://www.smart-navigation.org

- Identifier: 	Ship Information

- Maintenance: 	Amendments to this specification will be produced on a needs basis. For reporting issues with this specification which need correction, use the contact information.

### 4.2.2 Specification Scopes

This product specification describes one data product and therefore requires only one scope which is described below:

- Scope ID:  		Ship Information datasets.

- Hierarchical level: 		MD_ScopeCode - 005

- Hierarchical level name: 	dataset.

- Level description:		information applies to the dataset

- Extent:	EX_Extent.description: Korean Coast

### 4.2.3 Dataset identification

This section describes how to identify data sets that conform to the specification. A dataset that conforms to this Product Specification may be identified by its discovery metadata as defined in clause 12 of this specification. The information identifying the data product may include the following items from S-100 4.0.0 clause 11-6 (adapted from ISO 19115).

| Title | Ship Information |
| --- | --- |
| Abstract | Ship Information dataset is a textTable dataset containing details of the ship |
| Acronym | MTI |
| Content | Ship name, MMSI, Callsign, ShipMRN, MainEngine, Propellers etc. |
| Topic Category | Structure |
| Purpose | Standardization of specification information of ship |
| Language | EN Additional values if any use CharacterString values from ISO 639-2 |

### 4.2.4 Data Content and structure

### 4.2.4.1 Introduction

The Ship Information product is based on the S-100 General Feature Model (GFM), and is a feature-based vector product. All Ship Information features and information classes are derived from one of the abstract classes FeatureType and InformationType defined in the Ship Information application schema, which realize the GFM meta-classes S100_GF_FeatureType and S100_GF_InformationType respectively.

This clause contains the Application Schema expressed in UML and an associated Feature Catalogue.

### 4.2.4.2 Application Schema

The UML models in this clause are segments of the overall Ship Information application schema, and include overviews of the feature classes, information classes, meta features, spatial types, and the relationships between them.

This clause contains a general overview of the classes and relationships in the Ship Information application schema. Detailed information about how to use the feature types and information types to encode Ship Information is provided in the SMART-Navigation Data Classification and Encoding Guide (DCEG).

The following conventions are used in the UML diagrams depicting the application schema:

- Standard UML conventions for classes, associations, inheritance, roles, and multiplicities apply. These conventions are described in Part 1 of S-100.

- Italic font for a class name indicates an abstract class.

- Feature classes are depicted with pink background; the dark shade for abstract feature classes and the light shade for ordinary (non-abstract) feature classes.

- Information type classes are depicted with pink background; the dark shade for abstract information type classes and the light shade for ordinary information types.

- Association classes are depicted with a pink background.

- Complex attributes are depicted with a light yellow background.

- Enumeration lists and codelists are depicted with a green background. The numeric code corresponding to each listed value is shown to its right following an ‘=’ sign.

- No significance attaches to the colour of associations. (Complex diagrams may use different colours to distinguish associations that cross one another.)

- Subclasses inherit the attributes and associations of their superclasses at all levels, unless such inheritance is explicitly overridden in the subclass.

### 4.2.4.2.1 Domain model

Figure 6 Ship Information Overview

### 4.2.4.3 Feature Catalogue

### 4.2.4.3.1 Introduction

The Feature Catalogue describes the feature types, information types, attributes, attribute values, associations and roles which may be used in the product.  The Ship Information Feature Catalogue is available in an XML document which conforms to the S-100 XML Feature Catalogue Schema.  Simple attributes used in this specification are listed in below.

Name: 	Ship Information Feature Catalogue

Scope: 	Smart-Navigation

Version Number: 	1.0.0

Version Date:	2019-03-15

Producer:	SMART Navigation Project

Language:	English

### 4.2.4.3.1.1 Feature Types

Feature types contain descriptive attributes that characterize real-world entities. The word ‘feature’ may be used in one of two senses – feature type and feature instance. A feature type is a class and is defined in a Feature Catalogue. A feature instance is a single occurrence of the feature type and represented as an object in a dataset. A feature instance is located by a relationship to one or more spatial instances. A feature instance may exist without referencing a spatial instance.

### 4.2.4.3.1.2 Geographic

A Geographic (Geo) feature type carries the descriptive characteristics of a real-world entity.

### 4.2.4.3.1.3 Meta

Meta features contain information about other features within a dataset. Information defined by meta features override the default metadata values defined by the dataset descriptive records. Meta attribution on individual features overrides attribution on meta features.

### 4.2.4.3.1.4 Feature Relationship

A feature relationship links instances of one feature type with instances of the same or a different feature type.

### 4.2.4.3.1.5 Information Types

Information types are identifiable pieces of information in a dataset that can be shared between other features. They have attributes but have no relationship to any geometry; information types may reference other information types.

### 4.2.4.3.1.6 Attributes

SMART-Navigation product defines attributes as either simple or complex.

### 4.2.4.3.1.6.1 Simple Attributes

SMART-Navigation product uses nine types of simple attributes; they are listed in the following table:

| Type | Definition |
| --- | --- |
| Enumeration | A fixed list of valid identifiers of named literal values |
| Boolean | A value representing binary logic. The value can be either True or False. The default state for Boolean type attributes (i.e. where the attribute is not populated for the feature) is False. |
| Real | A signed Real (floating point) number consisting of a mantissa and an exponent |
| Integer | A signed integer number. The representation of an integer is encapsulation and usage dependent. |
| Text | An arbitrary-length sequence of characters including accents and special characters from a repertoire of one of the adopted character sets |
| Date | A date provides values for year, month and day according to the Gregorian Calendar. Character encoding of a date is a string which must follow the calendar date format (complete representation, basic format) for date specified by ISO 8601:1988. EXAMPLE 19980918 (YYYY-MM-DD) |
| Time | A time is given by an hour, minute and second. Character encoding of a time is a string that follows the local time (complete representation, basic format) format defined in ISO 8601:1988. EXAMPLE 183059 or 183059+0100 or 183059Z |
| Date and Time | A DateTime is a combination of a date and a time type. Character encoding of a DateTime shall follow ISO 8601:1988 EXAMPLE 19850412T101530 |
| Codelist | A type of flexible enumeration. A code list type is a list of literals which may be extended only in conformance with specified rules. |

### 4.2.4.3.1.6.2 Complex Attributes

Complex attributes are aggregations of other attributes that are either simple or complex. The aggregation is defined by means of attribute bindings.

### 4.2.4.3.1.6.3 Geometric Representation

Geometric representation is the digital description of the spatial component of an object as described in S-100 and ISO 19107. This product specification uses three types of geometries: GM_Point, GM_OrientableCurve, and GM_OrientableSurface. This product specification uses no geometry attribute.

### 4.2.4.3.2 Feature Catalogue General

The Ship Information feature catalogue describes the feature types, information types, attributes, attribute values, associations and roles which may be used in a Ship Information data set.

### 4.2.4.3.2.1 Feature Types

- Geographic

- Meta

- PrincipalShipCharacteristics

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| vesselName | - | complex attribute | 1..* | mandatory |  |
| IMO No | The seven-digit number assigned to the vessel, the vessel management company, and the ship documents company under the SOLAS rules. | integer | 1 | mandatory |  |
| shipMRN | MRN is a naming scheme that can uniquely identify any maritime resource on a global scale. shipMRN is the unique value given to the ship. | URN | 1 | mandatory |  |
| callSign | The designated call-sign of a radio station | text | 0..1 | option |  |
| MMSICode | The Maritime Mobile Service Identity (MMSI) Code is formed of a series of nine digits which are transmitted over the radio path in order to uniquely identify ship stations, ship earth stations, coast stations, coast earth stations, and group calls. These identities are formed in such a way that the identity or part thereof can be used by telephone and telex subscribers connected to the general telecommunications network principally to call ships automatically. (Adapted from Appendix 43 of the International Telecommunications Union Radio Regulations). | integer | 0..1 | option |  |
| vesselType | - | complex attribute | 0..1 | option |  |
| registration | - | complex attribute | 0..1 | option |  |
| vesselMeasurement | - | complex attribute | 0..* | option |  |
| bulbousBow | - | boolean | 0..1 | option |  |
| sideThruster | - | complex attribute | 0..* | option |  |
| steeringGear | - | complex attribute | 0..* | option |  |
| rudder | - | complex attribute | 0..* | option |  |
| propellers | - | complex attribute | 0..* | option |  |
| mainEngine | - | complex attribute | 1..* | mandatory |  |

- AdditionalInformation

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| builderShipyard | The shipyard that built this ship | text | 0..1 | option |  |
| dateOfBuilt | - | dateTime | 0..1 | option |  |
| dateOfDelivery | Ship's delivery date | dateTime | 0..1 | option |  |
| sailCode | - | enumeration | 0..1 | option |  |
| beforeCallsign | - | text | 0..1 | option |  |
| bareBoatCharter | - | enumeration | 0..1 | option |  |
| internationalCertificate | - | boolean | 0..1 | option |  |
| nationalityOfShipManagers | - | complex attribute | 0..* | option |  |

- Cartographic

- Theme

### 4.2.4.3.2.2 Feature Relationship

| Role Type | Association Name | Role | Feature | Multiplicity |
| --- | --- | --- | --- | --- |
| Association | Ship Additional Information | providedFor | AdditionalInformation | 1 |
| Association | Ship Additional Information | provides | PrincipalShipCharacteristics | 0..1 |

### 4.2.4.3.2.3 Information Relationship

| Role Type | Association Name | Role | Feature | Multiplicity |
| --- | --- | --- | --- | --- |
| Association | Ship Additional Information | provides | ShipInformation | 0..* |

### 4.2.4.3.2.4 Information Type

- ContactDetail

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| categoryOfCommunicationPreference | Classification of frequencies for communication based on preference. | eumeration | 0..1 | option |  |
| communicationChannel | - | text | 0..* | option |  |
| contactAddress | - | complex attribute | 0..* | option |  |
| contactInstructions | supplemental instructions on how or when to contact the individual, organisation, or service | text | 0..1 | option |  |
| information | - | complex attribute | 1..* | option |  |
| relecommunications | Information for contact by means of a telecommunications service. Distinctions: email Address, internet Address, callName, callSign | complex attribute | 0..* | option |  |

### 4.2.4.3.2.5 Simple Attribute

- Simple Attribute List

| Name | Description | Data Type | Remarks |
| --- | --- | --- | --- |
| IMONo | The seven-digit number assigned to the vessel, the vessel management company, and the ship documents company under the SOLAS rules. | integer |  |
| shipMRN | MRN is a naming scheme that can uniquely identify any maritime resource on a global scale. shipMRN is the unique value given to the ship. | URN |  |
| callSign | The designated call-sign of a radio station | text |  |
| MMSICode | The Maritime Mobile Service Identity (MMSI) Code is formed of a series of nine digits which are transmitted over the radio path in order to uniquely identify ship stations, ship earth stations, coast stations, coast earth stations, and group calls. These identities are formed in such a way that the identity or part thereof can be used by telephone and telex subscribers connected to the general telecommunications network principally to call ships automatically. (Adapted from Appendix 43 of the International Telecommunications Union Radio Regulations). | integer |  |
| builderShipyard | The shipyard that built this ship | text |  |
| yearBuilt | - | text |  |
| dateOfDelivery | Ship's delivery date | dateTime |  |
| categoryOfCommPref | Classification of frequencies for communication based on preference. | enumeration |  |
| communicationChannel | - | text |  |
| contactInstructions | Supplemental instructions on how or when to contact the individual, organisation, or service | text |  |
| displayName | A statement expressing if a feature name is to be displayed in certain display settings or not. Indication: Boolean. A True value is an indication that the name is intended to be displayed. | boolean |  |
| language | The language is encoded by a character code following ISO 639-3 | ISO 639-3 |  |
| name | The individual name of a feature. | text |  |
| categoryOfVessel | Classification of vessels by function or use. | S_100 Codelist |  |
| typeOfHull | ship hull design and construction method where the bottom and sides of the ship | enumeration |  |
| flagState | The flag state of a merchant vessel is the jurisdiction under whose laws the vessel is registered or licensed. | ISO 3166-2 |  |
| registeredPort | the port at which it is based, which may not be the same as its port of registry shown on its registration documents | text |  |
| categoryOfVesselRegistry | The locality of vessel registration or enrolment relative to the nationality of a port, territorial sea, administrative area, exclusive zone or other location. | enumeration |  |
| classificationSociety | The name of the Classification to be verified to be designed, constructed and operated to operate the ship in accordance with internationally recognized safety standards. | text |  |
| vesselsCharacteristics | Characteristics of the vessels | enumeration |  |
| vesselsCharacteristicsValue | The value of a particular characteristic such as a dimension or tonnage of a vessel. | real |  |
| vesselsCharacteristicsUnits | The unit used for vessel characteristics attribute | enumeration |  |
| mainEngineID | - | text |  |
| typeOfMainEngine | Type of main engine installed on a ship | text |  |
| powerValue | - | Real |  |
| powerUnit | - | enumeration |  |
| loadingCondition | - | enumeration |  |
| shipSpeedCorrespondingTo EngineTelegraph | - | enumeration |  |
| correspondingSpeedValue | - | real |  |
| correspondingPropellerPitch | - | real |  |
| correspondingEngineRPM | - | real |  |
| proportionOfFullAheadPower | - | real |  |
| engineCriticalRPM | - | real |  |
| maximumNumberOfConsecutiveStarts | - | real |  |
| timeFullAheadToFullAstern | - | real |  |
| timeLimitAstern | - | real |  |
| ruddersID |  | text |  |
| typeOfRudder | - | enumeration |  |
| areaOfRudder | Area of rudder blades | real |  |
| ruddersMaximumAngle | - | real |  |
| timeHardOverToHardOver | - | Real |  |
| rudderAngleForNeutralEffect | - | real |  |
| propellerID | - | text |  |
| propellerType | - | enumeration |  |
| directionOfTurn | - | boolean |  |
| noOfBlades | Number of propeller blades installed on the ship | integer |  |
| diameterOfPropeller | Diameter of Propeller in meters. | real |  |
| steeringGearID | - | text |  |
| steeringIdiosyncrasies | - | text |  |
| thrusterID | - | text |  |
| thrusterLocation | - | enumeration |  |
| bulbousBow | - | boolean |  |
| dateOfBuilt | - | dateTime |  |
| beforeCallsign | - | text |  |
| bareBoatCharter | - | enumeration |  |
| internationalCertificate | - | boolean |  |
| deliveryPoint | Details of where post can be delivered such as the apartment, name and/or number of a street, building or PO Box. | text |  |
| cityName | The name of a town or city. | text |  |
| administrativeDivision | Administrative division is a generic term for an administrative region within a country at a level below that of the sovereign state. | text |  |
| country | The name of a nation. (Adapted from The American Heritage Dictionaries) | text |  |
| postalCode | Known in various countries as a postcode, or ZIP code, the postal code is a series of letters and/or digits that identifies each postal delivery area. | text |  |
| fileLocator | The string encodes the location of a fragment of text or other information in a support file. | text |  |
| fileReference | The string encodes the file name of a single external text file that contains the text. | text |  |
| headline | Words set at the head of a passage or page to introduce or categorize | text |  |
| language | ISO 639-3 value | ISO639-3 |  |
| text | A non-formatted digital text string. | text |  |
| categoryOfCommPref | - | enumeration |  |
| contactInstructions | instructions on how and when to contact an individual or organisation | text |  |
| telcomCarrier | The name of provider or type of carrier for a telecommunications service | text |  |
| telecommunicationIdentifier | Identifier used for contact by means of a telecommunications service, such as a telephone number | text |  |
| telecommunicationService | Methods to communicate between involved parties over a distance. | S_100 Codelist |  |
| categoryOfManagement | Classification of groups with ownership in relation to vessels | enumeration |  |
| nationality | - | ISO 3166-2 |  |

- vesselsCharacteristicsUnits

| Value | Label | Description |
| --- | --- | --- |
| 1 | metre | - |
| 2 | foot | - |
| 3 | metric ton | The tonne or metric ton (U.S.), often redundantly referred to as a metric tonne, is a unit of mass equal to 1,000 kg (2,205 lb) or approximately the mass of one cubic metre of water at four degrees Celsius. It is sometimes abbreviated as mt in the United States, but this conflicts with other SI symbols. The tonne is not a unit in the International System of Units (SI), but is accepted for use with the SI. In SI units and prefixes, the tonne is a megagram (Mg). The Imperial and US customary units comparable to the tonne are both spelled ton in English, though they differ in mass. Pronunciation of tonne (the word used in the UK) and ton is usually identical, but is not too confusing unless accuracy is important as the tonne and UK long ton differ by only 1.6%. |
| 4 | ton | Long ton (weight ton or imperial ton) is the name for the unit called the "ton" in the avoirdupois or Imperial system of measurements, as used in the United Kingdom and several other Commonwealth countries. It has been mostly replaced by the tonne, and in the United States by the short ton. One long ton is equal to 2,240 pounds (1,016 kg) or 35 cubic feet (0.9911 m3) of salt water with a density of 64 lb/ft?(1.025 g/ml). It has some limited use in the United States, most commonly in measuring the displacement of ships, and was the unit prescribed for warships by the Washington Naval Treaty for example battleships were limited to a mass of 35,000 long tons (36,000 t; 39,000 ST). |
| 5 | short ton | The short ton is a unit of weight equal to 2,000 pounds (907.18474 kg). In the United States it is often called simply ton without distinguishing it from the metric ton (tonne, 1,000 kilograms) or the long ton (2,240 pounds / 1,016.0469088 kilograms); rather, the other two are specifically noted. There are, however, some U.S. applications for which unspecified tons normally means long tons (for example, Navy ships) or metric tons (world grain production figures). Both the long and short ton are defined as 20 hundredweights, but a hundredweight is 100 pounds (45.359237 kg) in the U.S. system (short or net hundredweight) and 112 6pounds (50.80234544 kg) in the Imperial system (long or gross h7undredweight). |
| 6 | gross ton | Gr8oss tonnage (GT) is a function of the volume of all ship's enclosed spaces (from keel to funnel) measured to the outside of the hull framing. There is a sliding scale factor. So GT is a kind of capacity-derived index that is used to rank a ship for purposes of determining manning, safety and other statutory requirements and is expressed simply as GT, which is a unitless entity, even though its derivation is tied to the cubic meter unit of volumetric capacity. Tonnage measurements are now governed by an IMO Convention (International Convention on Tonnage Measurement of Ships, 1969 (London-Rules)), which applies to all ships built after July 1982. In accordance with the Convention, the correct term to use now is GT, which is a function of the moulded volume of all enclosed spaces of the ship. |
| 7 | net ton | Net tonnage (NT) is based on a calculation of the volume of all cargo spaces of the ship. It indicates a vessel’s earning space and is a function of the moulded volume of all cargo spaces of the ship. |
| 8 | panama canal/universal measurement system net tonnage | - |
| 9 | suez canal net tonnage | The Suez Canal Net Tonnage (SCNT) is derived with a number of modifications from the former net register tonnage of the Moorsom System and was established by the International Commission of Constantinople in its Protocol of 18 December 1873. It is still in use, as amended by the Rules of Navigation of the Suez Canal Authority, and is registered in the Suez Canal Tonnage Certificate. |
| 10 | none | - |
| 11 | cubic metres | - |
| 12 | suez canal gross tonnage | - |

- typeOfRudder

| Value | Label | Description |
| --- | --- | --- |
| 1 | balanced rudder | - |
| 2 | semi-balanced rudder | - |
| 3 | unbalanced rudder | - |
| 4 | flaps rudder | - |
| 5 | pleuger rudder | - |

- categoryOfManagement

| Value | Label | Description |
| --- | --- | --- |
| 1 | owner | ship owners |
| 2 | manager | ship manager or ship management company |
| 3 | operator | ship operator or shipping company |

- categoryOfVessel

| Value | Label | Description |
| --- | --- | --- |
| 1 | general cargo vessel | A vessel designed to carry general cargo |
| 2 | container carrier | A vessel designed to carry ISO containers |
| 3 | tanker | A vessel designed to carry bulk liquid or gas, including LPG and LNG |
| 4 | bulk carrier | A vessel designed to carry bulk solid material |
| 5 | passenger vessel | A vessel designed to carry passengers; often a cruise ship |
| 6 | roll-on roll-off | A vessel designed to allow road vehicles to be driven on and off; often a ferry |
| 7 | refrigerated cargo vessel | A vessel designed to carry refrigerated cargo |
| 8 | fishing vessel | A vessel designed to catch or hunt fish |
| 9 | service | A vessel which provides a service such as a tug, anchor handler, survey or supply vessel |
| 10 | warship | A vessel designed for the conduct of military operations |
| 11 | towed or pushed composite unit | Either a tug and tow, or any combination of a tug providing propulsion to barges or vessels secured ahead or alongside |
| 12 | tug and tow | A combination of tug(s) and non-powered tow(s) |
| 13 | light recreational | A pleasure boat or watercraft, or an excursion vessel used for short cruises such as whale watching |
| 14 | semi-submersible offshore installation | An installation which is designed to float at all times and which is normally anchored in position when deployed in the offshore gas and oil industry. |
| 15 | jack-up exploration or project installation | An exploration or project installation with legs which can be raised and lowered. The legs are raised when the installation is repositioned. When stationary the legs are lowered to the sea floor and the working platform is raised clear of the sea surface |
| 16 | livestock carrier | A vessel designed to carry large quantities of live animals. |
| 17 | sport fishing | A vessel used in fishing for pleasure or competition. |

- telecommunicationService

| Value | Label | Description |
| --- | --- | --- |
| 1 | voice | The transfer or exchange of information by using sounds that are being made by mouth and throat when speaking |
| 2 | facsimile | A system of transmitting and reproducing graphic matter (as printing or still pictures) by means of signals sent over telephone lines |
| 3 | sms | Short Message Service, a form of text messaging communication on phones and mobile phones |
| 4 | data | Facts or information used usually to calculate, analyze, or plan something |
| 5 | streamedData | Streamed data is data that that is constantly received by and presented to an end-user while being delivered by a provider. |
| 6 | telex | A system of communication in which messages are sent over long distances by using a telephone system and are printed by using a special machine (called a teletypewriter) |
| 7 | telegraph | An apparatus, system, or process for communication at a distance by electric transmission over wire |
| 8 | email | Messages and other data exchanged between individuals using computers in a network. |

- typeOfHull

| Value | Label | Description |
| --- | --- | --- |
| 1 | single structure | - |
| 2 | double structure | - |
| 3 | triple structure | - |

- categoryOfVesselRegistry

| Value | Label | Description |
| --- | --- | --- |
| 1 | domestic | - |
| 2 | foreign | - |

- vesselsCharacteristics

| Value | Label | Description |
| --- | --- | --- |
| 1 | length overall | - |
| 2 | length at waterline | - |
| 3 | breadth | - |
| 4 | draught | - |
| 6 | displacement tonnage | - |
| 7 | displacement tonnage, light | - |
| 8 | displacement tonnage, loaded | - |
| 9 | deadweight tonnage | - |
| 10 | gross tonnage | - |
| 11 | net tonnage | - |
| 12 | panama canal/universal measurement system net tonnage | - |
| 13 | suez canal net tonnage | - |
| 14 | suez canal gross tonnage | - |
| 15 | registered length | - |
| 16 | length between perpendicular | - |
| 17 | moulded beam | - |
| 18 | moulded depth | - |
| 19 | moulded | - |
| 20 | air draft | - |
| 21 | height | - |
| 22 | thames measurement tonnage | - |
| 23 | ballastwater tank | - |
| 24 | freshwater tank | - |
| 25 | feul oil tank | - |
| 26 | disel oil tank | - |
| 27 | parallel W/L loaded | - |
| 28 | parallel W/L ballast | - |
| 29 | bridge to bow length | - |
| 30 | bridge to bulbous bow length | - |
| 31 | bridge to stern length | - |
| 32 | manifold to bow length | - |

- powerUnit

| Value | Label | Description |
| --- | --- | --- |
| 1 | hp | - |
| 2 | kw | - |

- propellerType

| Value | Label | Description |
| --- | --- | --- |
| 1 | FPP | Fixed pitch propeller |
| 2 | CPP | Controllable pitch propeller |

- thrusterLocation

| Value | Label | Description |
| --- | --- | --- |
| 1 | bow | - |
| 2 | stern | - |

- sailCode

| Value | Label | Description |
| --- | --- | --- |
| 1 | liner shipping | - |
| 2 | tramp shipping | - |

- bareBoatCharter

| Value | Label | Description |
| --- | --- | --- |
| 1 | bare boat charter of hire purchase | - |
| 2 | bare boat charter | - |
| 3 | time charter | - |
| 4 | others charter | - |

- categoryOfCommPref

| Value | Label | Description |
| --- | --- | --- |
| 1 | preferred calling | - |
| 2 | alternate calling | - |
| 3 | preferred working | - |
| 4 | alternate working | - |

### 4.2.4.3.2.6 Complex Attribute

- FeatureName

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| displayName | A statement expressing if a feature name is to be displayed in certain display settings or not. Indication: Boolean. A True value is an indication that the name is intended to be displayed. | boolean | 0..1 | option |  |
| language | The language is encoded by a character code following ISO 639-3 | ISO 639-3 | 0..1 | mandatory |  |
| name | The individual name of a feature. | text | 1 | mandatory |  |

- vesselType

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| categoryOfVessel | Classification of vessels by function or use. | S_100 Codelist | 1 | mandatory |  |
| typeOfHull | ship hull design and construction method where the bottom and sides of the ship | enumeration | 0..1 | option |  |

- registration

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| flagState | The flag state of a merchant vessel is the jurisdiction under whose laws the vessel is registered or licensed. | ISO 3166-2 | 0..1 | option |  |
| registeredPort | The port at which it is based, which may not be the same as its port of registry shown on its registration documents | text | 0..1 | option |  |
| categoryOfVesselRegistry | The locality of vessel registration or enrolment relative to the nationality of a port, territorial sea, administrative area, exclusive zone or other location. | enumeration | 0..1 | option |  |
| classificationSociety | The name of the Classification to be verified to be designed, constructed and operated to operate the ship in accordance with internationally recognized safety standards. | text | 0..1 | option |  |

- contactAddress

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| deliveryPoint | Details of where post can be delivered such as the apartment, name and/or number of a street, building or PO Box. | text | 0..* | option |  |
| cityName | The name of a town or city. | text | 0..1 | option |  |
| administrativeDivision | Administrative division is a generic term for an administrative region within a country at a level below that of the sovereign state. | text | 0..1 | option |  |
| country | The name of a nation. (Adapted from The American Heritage Dictionaries) | text | 0..1 | option |  |
| postalCode | Known in various countries as a postcode, or ZIP code, the postal code is a series of letters and/or digits that identifies each postal delivery area. | text | 0..1 | option |  |

- information

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| fileLocator | The string encodes the location of a fragment of text or other information in a support file. | text | 0..1 | option |  |
| fileReference | The string encodes the file name of a single external text file that contains the text. | text | 0..1 | option |  |
| headline | Words set at the head of a passage or page to introduce or categorize | text | 0..1 | option |  |
| language | ISO 639-3 value | ISO 639-3 | 0..1 | option |  |
| text | A non-formatted digital text string. | text | 0..1 | option |  |

- telecommunications

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| categoryOfCommunicationPreference | - | enumeration | 0..1 | option |  |
| contactInstructions | Instructions on how and when to contact an individual or organisation | text | 0..1 | option |  |
| telcomCarrier | The name of provider or type of carrier for a telecommunications service | text | 0..1 | option |  |
| telecommunicationIdentifier | Identifier used for contact by means of a telecommunications service, such as a telephone number | text | 1 | mandatory |  |
| telecommunicationService | Methods to communicate between involved parties over a distance. | S_100 Codelist | 0..* | option |  |

- vesselMeasurement

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| vesselsCharacteristics | Characteristics of the vessels | enumeration | 0..1 | option |  |
| vesselsCharacteristicsValue | The value of a particular characteristic such as a dimension or tonnage of a vessel. | real | 0..1 | option |  |
| vesselsCharacteristicsUnits | the unit used for vessel characteristics attribute | enumeration | 0..1 | option |  |

- mainEngine

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| mainEngineID | - | text | 1 | mandatory |  |
| typeOfMainEngine | Type of main engine installed on a ship | text | 0..1 | option |  |
| mainEngineMaximumPower | Maximum power that an engine can put out. | complex attribute | 1..2 | mandatory |  |
| correspondingSpeed | - | complex attribute | 0..* | option |  |
| engineCriticalRPM | - | real | 0..* | option |  |
| maximumNumberOfConsecutiveStarts | - | real | 0..1 | option |  |
| timeFullAheadTo FullAstern | - | real | 0..1 | option |  |
| timeLimitAstern | - | real | 0..1 | option |  |

- rudder

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| ruddersID | - | text | 0..1 | option |  |
| typeOfRudder | - | enumeration | 0..1 | option |  |
| areaOfRudder | Area of rudder blades | real | 0..1 | option |  |
| ruddersMaximumAngle | - | real | 0..1 | option |  |
| timeHardOverTo HardOver | - | real | 0..1 | option |  |
| rudderAngleFor NeutralEffect | - | real | 0..1 | option |  |

- propeller

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| propellerID | - | text | 0..1 | option |  |
| propellerType | - | enumeration | 1 | mandatory |  |
| directionOfTurn | - | boolean | 0..1 | option |  |
| noOfBlades | Number of propeller blades installed on the ship | integer | 0..1 | option |  |
| diameterOfPropeller | Diameter of Propeller in meters. | real | 0..1 | option |  |

- steeringGear

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| steeringGearID | - | text | 0..1 | option |  |
| steeringIdiosyncrasies | - | text | 0..1 | option |  |

- sideThruster

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| thrusterID | - | text | 0..1 | option |  |
| thrusterLocation | - | enumeration | 1..1 | mandatory |  |
| thrusterPowerCapacity | - | complex attribute | 0..2 | option |  |

- thrusterPowerCapacity

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| powerValue | - | Real | 1 | mandatory |  |
| powerUnit | - | enumeration | 1 | mandatory |  |

- NationalityOfShipManagers

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| categoryOfManagement | - | enumeration | 1 | mandatory |  |
| nationality | - | ISO 3166-2 | 1 | mandatory |  |

### 4.2.5 Coordinate Reference System(CRS)

N/A

### 4.2.6 Data Quality

### 4.2.6.1 Introduction

Ship Information products must be tested with the Ship Information specific checks prior to release by the data producer. The data producer must review the check results and address any issues to ensure sufficient quality of the data products. The checks are a mix of data format validation checks, conformance to standard checks and logical consistency checks. But the checklist has not been written yet. It will be updated in the next version.

### 4.2.7 Data Capture and classification

See DCEG for this section.

### 4.2.8 maintenance

### 4.2.8.1 Introduction

Datasets are maintained as needed and must include mechanisms for Ship Informatrion updating. Data updates will be made by new editions. The maintenance and update frequency of Ship Information datasets should be defined by the producers (official national authority) implementing this specification.

Data Producers must use applicable sources to maintain and update data and provide a brief description of the sources that were used to produce the dataset in the appropriate metadata field.

The data product shall provide information on how the data is maintained and should describe the principles and criteria applied in maintenance regime. This should specify the expected frequency of updates.

Table 9 Maintenance and update frequency

| Item Name | Description | Multiplicity | Type |
| --- | --- | --- | --- |
| maintenanceAndUpdateFrequency | Frequency with which changes and additions are made to the data product (per update scope) | 1..* | MD_MaintenanceInformation (ISO 19115) |
| dataSource | Identification of the kinds of data sources usable to produce datasets | 1..* | LI_Source (ISO 19115) |
| productionProcess | Textual description of the production process applicable to the datasets (per scope or data source) | 1..* | LI_ProcessStep (ISO 19115) |

### 4.2.8.2 Production process for base and update datasets

Data Producers should follow their established production processes for maintaining and updating datasets. Data is produced against the DCEG and checked against the appropriate set of validation rules in Appendix E. (Appendix E is not included in this release.)

### 4.2.8.3 Dataset updates

N/A (Dataset Update is not relevant in this model)

### 4.2.8.4 Support file updates

N/A (Support file is not included in this model)

### 4.2.8.5 Feature and portrayal catalogues

For each new version of the Message Catalogue Product Specification a new feature catalogue will be released. The system must be able to manage datasets and their catalogues that are created on different versions of the Message Catalogue product specification. (Portrayal Catalogue is not defined in this model.)

### 4.2.8.6 Feature history, versions, and change tracking

If applications or production systems require versioning of individual instances of feature or information types, maintenance of histories, or change tracking, the methods for versioning, history management, and change tracking and display are left to the application or production system.

### 4.2.8.7 Dataset encryption

Details about dataset encryption are still to be determined, and may mirror the method described in S-101.

### 4.2.9 Portrayal

Portrayal is not defined in this version of Ship Information Product Specifications. Users are free to choose the means and methodology of portrayal as they see best suited for their needs. It should be noted that future versions of Ship Information may include a portrayal catalogue, and any implementer should therefore anticipate this, and make sufficient provisions in any system supporting Ship Information.

### 4.2.10 data product format(encoding)

### 4.2.10.1 Introduction

The GML encoding of Ship Information datasets is based on the S-100 profile of GML 3.2.1.

Format Name: Ship Information GML

Version:	1.0

Character Set: UTF-8

Specification: SMART-Navigation Product Specification

### 4.2.10.2 Encoding of Latitude and Longitude

N/A(This Model has no geometry attribute.).

### 4.2.10.3 Text Attribute

Character strings must be encoded using the character set defined in ISO 10646-1, in Unicode Transformation Format-8 (UTF-8).

### 4.2.10.4 Mandatory Attribute Values

There are four reasons why attribute values may be considered mandatory:

- They determine whether a feature is in the display base,

- Certain features make no logical sense without specific attributes,

- Some attributes are necessary to determine which symbol is to be displayed,

- Some attributes are required for safety of navigation.

All mandatory attributes are identified in the Feature Catalogue

### 4.2.10.5 Unknown Attribute Values

When a mandatory attribute code or tag is present but the attribute value is missing, it means that the producer wishes to indicate that this attribute value is unknown.  Missing mandatory attributes must be “nilled” with a GML nilReason attribute giving the reason for omission.

Optional attributes must be omitted altogether if the value is unknown or missing. They must not be “nilled.”

### 4.2.10.6 Structure of dataset files

### 4.2.10.6.1 Sequence of dataset files

The order of data objects in each dataset file is described below:

Dataset Identification Information

Dataset structure information

Spatial records for by-reference geometries

Point

Multi point

Curve

Composite Curve

Surface

Information objects

Feature objects (Geometry may be encoded inline or by reference.)

Meta features

Geo features

### 4.2.10.7 Numeric Attribute Encoding

Floating point and integer attribute values must not contain leading zeros.  Floating point attribute values must not contain non-significant trailing zeros.

### 4.2.10.8 Object identifiers

The “name” of feature records must provide a unique world-wide identifier of feature records. The “name” of the record is the combination of the subfields agency, featureObjectIdentifier, and featureIdentificationSubdivision elements of the featureObjectIdentifier element of the object.

Features, information types, collection objects, meta features, and geometries (inline or external) are all required by the schema to have a gml:id attribute with a value that is unique within the dataset. The gml:id values must be used as the reference for the object from another object in the same dataset or another dataset.

### 4.2.10.9 Dataset validation

Fields may be repeated or omitted as permitted by the XML schemas and the validation tests. Since XML schema cannot encode rules for conditional presence or attributes, these rules are checked by Schematron rules or other validation code.

### 4.2.10.10 Location of Data Product Format schema Files

The schema file will provide a future location and is available from that location.

### 4.2.10.11 Detailed documentation of Ship Information XML Schema

See DCEG for this section.

### 4.2.11 data product delivery

This clause specifies the encoding and delivery mechanisms for a Ship Information dataset.  Data which conforms to this product specification must be delivered by means of an exchange set.

### 4.2.11.1 Exchange set

Ship Information datasets are grouped into exchange sets. Each exchange set consists of one or more Ship Information datasets with an associated XML metadata file and a single Exchange Catalogue XML file containing metadata. It may also include one or more support files.

Units of Delivery:				Exchange Set

Transfer Size:					Unlimited

Medium Name:					Digital data delivery

Other Delivery Information:

Each dataset must be contained in a physically separate, uniquely identified file on the transfer medium.

Each exchange set has a single exchange catalogue which contains the discovery metadata for each dataset and references to any support files.

Support files are supplementary information which are linked to the features and information types by attributes. The attributes containing these links are described in the application schema and feature catalogue.

An exchange set may be encapsulated into a form suitable for transmission by a mapping called a transmission encoding. An encoding translates each of the elements of the exchange set into a logical form suitable for writing to media and for transmission online. An encoding may also define other elements in addition to the exchange set contents (i.e., media identification, data extents etc.) and also may define commercial constructs such as encryption and compression methods.

If the data is transformed (e.g., for encryption or compression purposes) its content must not be changed.

This product specification does not define the transmission encoding which must be used as a default for transmission of data between parties.

The exchange set elements are as follows:

Mandatory Elements

- Ship Information datasets – GML encoding of features/attributes and their associated geometry and metadata.

- Exchange Catalogue – the XML encoded representation of exchange set catalogue features [discovery metadata].

Optional Elements

- Supplementary files – These are contained within the exchange set as files and the map from the name included within the dataset and the physical location on the media is defined within the Exchange Catalogue.

- Feature Catalogue – If it is necessary to deliver the latest feature catalogue to the end user it may be done using the Ship Information exchange set mechanism for datasets – i.e., include the updated feature catalogue in an exchange set.

### 4.2.11.2 Dataset

### 4.2.11.2.1 Datasets

Datasets are distributed as files which are part of exchange sets structured as described in this specification. The distribution media are left to the discretion of the producer and distributor.

The following types of dataset files may be produced and contained within an exchange set:

- New dataset and new edition of a dataset (base dataset):  Each new edition of a dataset must have the same name as the dataset that it replaces.  A new edition can also be AtoN data that has previously been produced for this area.

- Re-issue of a dataset: including all the updates applied to the original dataset up to the date of the reissue. A re-issue does not contain any new information additional to that previously issued by updates.

- Cancellation:  The dataset is cancelled and is deleted from the SYSTEM.  The structure for a cancellation is described in clause 4.2.11.2.5.

### 4.2.11.2.2 Dataset size

Datasets must not exceed 1 MB.

### 4.2.11.2.3 Dataset file naming

Dataset files shall be named

CCXXXXXXXX_EEE.GML

The file name forms a unique identifier where:

- CC - the first two characters identify the issuing agency (mandatory).

- The third to tenth characters are optional and may be used in any way by the producer to provide the unique file name. The following characters are allowed in the dataset name, A to Z, 0 to 9 and the special character _ (underscore).

- EEE – new editions and re-issues use 000, and increment until a limit of 999 (mandatory).

- The maximum number of characters preceding _EEE is ten.

- GML – the character sequence “GML” or “gml”.

### 4.2.11.2.4 New Editions and re-issues

This section defines the sequencing of datasets for New Edition. In order to ensure that feature type updates are incorporated into an SYSTEM in the correct sequence without any omission, a number of parameters encoded in the data header and/or metadata are used in the following way:

| edition number | when a dataset is initially created, the edition number 1 is assigned to it. The edition number is increased by 1 at each new edition. |
| --- | --- |

### 4.2.11.2.5 Cancellations

Dataset cancellation is not considered I this product specification. In the SMART Navigation project, the system automatically cancels the dataset after 7 days from the date the dataset is received.

### 4.2.11.3 Support Files

N/A (Support file is not included in this model)

### 4.2.11.4 Exchange Catalogue

N/A (Exchnage Catalogue is not used in SMART Navigation)

### 4.2.12 meta data

### 4.2.12.1 Introduction

The Ship Information metadata description is based on the S-100 metadata document section, which is a profile of the ISO 19115 standard. These documents provide a structure for describing digital geographic data and define metadata elements, a common set of metadata terminology, definitions and extension procedures.

Two metadata packages are described in this product specification: dataset metadata and exchange set metadata.

Figure 7 Metadata packages

Note 1: Types with CI_, EX_, and MD_ prefixes are from packages defined in ISO 19115 and adapted by S-100. Types with S100_ prefix are from packages defined in S-100.

Note 2: When a dataset is terminated, the purpose metadata field is set to 3 (terminated), and the editionNumber metadata field is set to 0. All other metadata fields must be blank.

### 4.2.12.2 Dataset metadata

Dataset metadata is intended to describe information about a dataset. It facilitates the management and exploitation of data and is an important requirement for understanding the characteristics of a dataset. Whereas dataset metadata is usually fairly comprehensive, there is also a requirement for a constrained subset of metadata elements that are usually required for discovery purposes. Discovery metadata are often used for building web catalogues, and can help users determine whether a product or service is fit for purpose and where they can be obtained.

Table 10 Dataset metadata

| Name | Cardinality | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| S100_DataSetDiscoveryMetadata |  |  |  |  |
| fileName | 1 |  | CharacterString | Dataset file name |
| filePath | 1 |  | CharacterString | Full path from the exchange set root directory |
| description | 1 |  | CharacterString | Short description of the area covered by dataset, e.g., area, harbour, or port name, between two named locations etc. |
| dataProtection | 0..1 | {1} or {2} | Boolean | True : Encrypted False : Unencrypted |
| protectionScheme | 0..1 |  | CharacterString |  |
| digitalSignature | 0..1 |  | CharacterString |  |
| digitalSignatureReference | 0..1 |  | CharacterString |  |
| digitalSignatureValue | 0..1 |  | CharacterString |  |
| copyright | 0..1 |  | MD_LegalConstraints>MD_RestrictionCode <copyright> (ISO 19115-1) |  |
| classification | 0..1 |  | Class MD_SecurityConstraints>MD_ClassificationCode (codelist) ISO 19115-1 | 1. unclassified 2. restricted 3. confidential 4. secret 5. top secret |
| purpose | 1 | {1}, {2} | CharacterString | - New dataset - New edition |
| specificUsage | 1 |  | MD_USAGE>specificUsage (character string) MD_USAGE>userContactInfo (CI_Responsibility) | brief description of the resource and/or resource series usage |
| editionNumber | 1 | {1} | Integer | When a dataset is initially created, the edition number “1” is assigned to it. The edition number is increased by one with each new edition. |
| updateNumber | 1 |  | CharacterString | Update number 0 is assigned to a new dataset. |
| updateApplicationDate | 1 |  | Date | All updates dated on or before this date must have been applied by the producer. |
| issueDate | 1 |  | Date | Date on which the dataset was generated. |
| productSpecification | 1 |  | S100_ProductSpecification | See Notes below this table for constraints on values. |
| producingAgency | 1 |  | CI_Responsibility>CI_Organisation or CI_Responsibility>CI_Individual | Party responsible for generating the dataset. |
| optimumDisplayScale | 0..1 |  | Integer |  |
| maximumDisplayScale | 0..1 |  | Integer |  |
| minimumDisplayScale | 0..1 |  | Integer |  |
| horizontalDatumReference | 1 | EPSG | CharacterString |  |
| horizontalDatumValue | 1 | 4326 | Integer | WGS84 |
| verticalDatum | 0..1 |  | S100_VerticalAndSoundingDatum | Not included in Ship information |
| soundingDatum | 0..1 |  | S100_VerticalAndSoundingDatum | Not included in Ship information |
| dataType | 1 | GML | CharacterString | The only value allowed is “GML”. |
| otherDataTypeDescription | 0..1 |  | CharacterString |  |
| dataTypeVersion | 1 | 3.2.1 | CharacterString |  |
| dataCoverage | 1..* |  | S100_DataCoverage |  |
| comment | 0..1 |  | CharacterString | Any additional Information |
| layerID | 1..* | S-101 | CharacterString | Dataset must be used with ENC in an ECDIS. |

### 4.2.12.3 Update Dataset Metadata

N/A (Dataset Update is not relevant in this model)

### 4.2.12.4 Support file metadata

N/A (Support file is not included in this model)

### 4.2.12.5 Exchange set metadata

N/A (Exchange Catalogue is not used in SMART-Navigation)

### 4.2.12.6 Catalogue file Metadata

All Ship Information Exchange Catalogue files must contain at least the mandatory metadata elements.

Table 11 Catalogue file metadata

| Name | Multiplicity | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| S100_ExchangeCatalogue |  |  |  |  |
| identifier | 1 |  | S100_CatalogueIdentifier |  |
| contact | 1 |  | S100_CataloguePointOfContact |  |
| productSpecification | 0..1 |  | S100_ProductSpecification | Conditional on all the datasets using the same product specification.. |
| metadataLanguage | 1 | English | CharacterString | All datasets conforming to this PS must use English language. A catalogue in English must be provided. Discovery metadata elements within catalogues have their own locale attributes and may be repeated in languages other than English. |
| exchangeCatalogueName | 0..1 |  | CharacterString | Not used in SMART-Navigation |
| exchangeCatalogueDescription | 1 |  | CharacterString |  |
| exchangeCatalogueComment | 0..1 |  | CharacterString | Any additional Information |
| compressionFlag | 0..1 |  | Boolean | Yes or No |
| sourceMedia | 0..1 |  | CharacterString |  |
| replacedData | 0..1 |  | Boolean |  |
| dataReplacement | 0..1 |  | CharacterString |  |

## 4.3 Ship dynamic

### 4.3.1 Overview

### 4.3.1.1 Introduction

This document has been produced by the member of SMART-Navigation project team(Korean Register, Marine-Works) in response to a requirement to produce a data product about ship moves under control of the maneuvering equipment. It is based on the IHO S-100 framework specification and the ISO 19100 series of standards. It is a vector product specification that is primarily intended for encoding the information of ship’s movement such as location, motion, speed etc.

### 4.3.1.2 References

IHO S-100 IHO Universal Hydrographic Data Model Edition 4.0.0 (December 2017).

ISO 8601. 2004. Data elements and interchange formats - Information interchange - Representation of dates and times. 2004.

ISO 3166-1. 1997. Country Codes. 1997.

ISO 10646-1 Information technology - Universal Multiple-Octet Coded Character Set (UCS) - Part 1: Architecture and Basic Multilingual Plane

ISO 19101-2:2008 Geographic Information - Rules for Application Schema

ISO/TS 19103:2005 Geographic Information - Conceptual schema language

ISO 19106:2004 Geographic Information – Profiles

ISO 19107:2003 Geographic Information – Spatial schema

ISO 19109:2005 Geographic Information - Rules for Application Schema

ISO 19111:2003 Geographic Information - Spatial referencing by coordinates

ISO 19115:2003+Corr1 (2006) Geographic Information - Metadata

ISO 19115-2:2009 Geographic Information - Metadata: Extensions for imagery and gridded data

ISO 19123:2005 Geographic Information - Schema for coverage geometry and functions

ISO 19129:2009 Geographic Information - Imagery gridded and coverage data framework

ISO 19131:2007 Geographic Information - Data product specifications

ISO 19136:2007 Geographic Information – Geography Markup Language

ISO 19136-2:2015, Geographic Information – Geography Markup Language.

ISO/TS 19139, Geographic Information – Metadata – XML schema implementation.

### 4.3.1.3 Terms, definitions and abbreviations

Terms and Definitions

The S-100 framework is based on the ISO 19100 series of geographic standards. The terms and definitions provided here are used to standardize the nomenclature found within that framework, whenever possible. They are taken from the references cited in clause 2.1. Modifications have been made when necessary.

- Application

Manipulation and processing of data in support of user requirements (ISO 19101)

- Application schema

Conceptual schema for data required by one or more applications (ISO 19101)

- Conceptual model

Model that defines concepts of a universe of discourse (ISO 19101)

- Conceptual schema

Formal description of a conceptual model (ISO 19101)

- Coverage

Feature that acts as a function to return values from its range for any direct position within its spatial, temporal or spatiotemporal domain (ISO 19123)

- Data product

Dataset or dataset series that conforms to a data product specification

- Data product specification

Detailed description of a dataset or dataset series together with additional information that will enable it to be created, supplied to and used by another party

- Dataset

Identifiable collection of data (ISO 19115)

- Feature

Abstraction of real world phenomena (ISO 19101)

- Feature association

Relationship that links instances of one feature type with instances of the same or a different feature type (ISO19110)

- Feature attribute

Characteristic of a feature (ISO 19101)

- Geographic data

Data with implicit or explicit reference to a location relative to the Earth (ISO 19109)

- Metadata

Data about data (ISO 19115)

- Model

Abstraction of some aspects of reality (ISO 19109)

- Portrayal

Presentation of information to humans (ISO 19117)

- Quality

Totality of characteristics of a product that bear on its ability to satisfy stated and implied needs (ISO 19101)

Abbreviations

This product specification adopts the following convention for symbols and abbreviated terms:

- ASCII 	American Standard Code for Information Interchange

- ECDIS	Electronic Chart Display and Information Systems

- ECS	Electronic Chart Systems

- ENC	Electronic Navigational Chart

- GML	Geography Markup Language

- IHO	International Hydrographic Organization

- IOC	International Oceanographic Commission

- ISO	International Organization for Standardization

- UML	Unified Modelling Language

- URI	Uniformed Resource Identifier

- URL 	Uniform Resource Locator

- WMS	Web Map Service

- WFS	Web Feature Service

- www 	World Wide Web

- XML	Extensible Markup Language

- XSLT	eXtensible Stylesheet Language Transformations

### 4.3.1.4 General Data Product Description

Informal Description of Data Product

This clause contains general information about the data product

- Title:  	Ship dynamic Product Specification.

- Abstract: 	 Ship dynamic product specification describe the means to express ship status such as the position, condition of the ship that the ship changes as the ship moves under control of the maneuvering equipment. This may include details on the propulsion system condition, ship motions and navigation status for ship, excluding ship equipment status and sensor information.

- Content:	Datasets conforming to this specification will contain all ship movement status information for the area of coverage, additionally, there will be relevant metadata data quality, production authority, data sources and publication date.

- Spatial Extent:	Korean Coast

- Specific Purpose:	Describing ship movement information for utilization of SMART-Navigation service

### 4.3.1.5 Data product specification metadata

This information uniquely identifies this Product Specification and provides information about its creation and maintenance. For further information on dataset metadata see the metadata clause.

- Title: 	Ship Dynamic

- S-100 Version: 	4.0.0

- Ship Dynamic Version: 	1.0.0

- Date: 	2019-03-08

- Language: 	English

- Classification: 	Unclassified

- Contact: 	32, Yuseong-daero 1312beon-gil, Yuseong-gu, Daejeon, Korea 32(Jang Dong) inside KRISO

- URL: 	http://www.smart-navigation.org

- Identifier: 	Ship Dynamic

- Maintenance: 	Amendments to this specification will be produced on a needs basis. For reporting issues with this specification which need correction, use the contact information.

### 4.3.2 Specification Scopes

This product specification describes one data product and therefore requires only one scope which is described below:

- Scope ID:  		Ship Information datasets.

- Hierarchical level: 		MD_ScopeCode - 005

- Hierarchical level name: 	dataset.

- Level description:		information applies to the dataset

- Extent:	EX_Extent.description: Korean Coast

### 4.3.3 Dataset identification

This section describes how to identify data sets that conform to the specification. A dataset that conforms to this Product Specification may be identified by its discovery metadata as defined in clause 12 of this specification. The information identifying the data product may include the following items from S-100 4.0.0 clause 11-6 (adapted from ISO 19115).

| Title | Ship Dynamic |
| --- | --- |
| Abstract | Ship Dynamic dataset is a vector dataset containing all ship movement information regarding the SMART-Navigation services within a defined geographical area. |
| Acronym | SD |
| Content | Draft, Engine telegraph order, roll,ship speed, navigation status |
| Topic category | Transportation(vehicle or vessel location) |
| geographicDescription | Area specific to marine navigation within korean sea area |
| Purpose | Standardization of ship movement status information |
| Language | EN Additional values if any use CharacterString values from ISO 639-2 |

4.3.4 Data Content and structure

### 4.3.4.1 Introduction

The Ship Dynamic product is based on the S-100 General Feature Model (GFM), and is a feature-based vector product. All Ship Dynamic features and information classes are derived from one of the abstract classes FeatureType and InformationType defined in the Ship Dynamic application schema, which realize the GFM meta-classes S100_GF_FeatureType and S100_GF_InformationType respectively.

This clause contains the Application Schema expressed in UML and an associated Feature Catalogue.

### 4.3.4.2 Application Schema

The UML models in this clause are segments of the overall Ship Dynamic application schema, and include overviews of the feature classes, information classes, meta features, spatial types, and the relationships between them.

This clause contains a general overview of the classes and relationships in the Ship Dynamic application schema. Detailed information about how to use the feature types and information types to encode Ship Dynamic information is provided in the SMART-Navigation Data Classification and Encoding Guide (DCEG).

The following conventions are used in the UML diagrams depicting the application schema:

- Standard UML conventions for classes, associations, inheritance, roles, and multiplicities apply. These conventions are described in Part 1 of S-100.

- Italic font for a class name indicates an abstract class.

- Feature classes are depicted with pink background; the dark shade for abstract feature classes and the light shade for ordinary (non-abstract) feature classes.

- Complex attributes are depicted with a light yellow background.

- Enumeration lists and codelists are depicted with a green background. The numeric code corresponding to each listed value is shown to its right following an ‘=’ sign.

- No significance attaches to the colour of associations. (Complex diagrams may use different colours to distinguish associations that cross one another.)

- Where the association role or name is not explicitly shown, the default rules for roles and names apply:

- The role name is ‘the<CLASSNAME>’ where <CLASSNAME> is the name of the class to which that association end is linked.

- The association name is ‘<CLASSNAME1>_<CLASSNAME2>’ where <CLASSNAME1> is the source and <CLASSNAME2> the target. In case of a feature/information association the feature is the source. For feature/feature or information/information associations without explicit names the source/target are indicated by an arrowhead.

- Subclasses inherit the attributes and associations of their superclasses at all levels, unless such inheritance is explicitly overridden in the subclass.

### 4.3.4.2.1 Domain model

Figure 8 Ship Dynamic Overview

### 4.3.4.3 Feature Catalogue

### 4.3.4.3.1 Introduction

The Feature Catalogue describes the feature types, information types, attributes, attribute values, associations and roles which may be used in the product.  The Ship Dynamic Feature Catalogue is available in an XML document which conforms to the S-100 XML Feature Catalogue Schema.  Simple attributes used in this specification are listed in below.

Name: 	Ship Dynamic Feature Catalogue

Scope: 	Smart-Navigation

Version Number: 	1.0.0

Version Date:	2019-03-15

Producer:	SMART Navigation Project

Language: 	                  English

### 4.3.4.3.1.1 Feature Types

Feature types contain descriptive attributes that characterize real-world entities. The word ‘feature’ may be used in one of two senses – feature type and feature instance. A feature type is a class and is defined in a Feature Catalogue. A feature instance is a single occurrence of the feature type and represented as an object in a dataset. A feature instance is located by a relationship to one or more spatial instances. A feature instance may exist without referencing a spatial instance.

### 4.3.4.3.1.2 Geographic

A Geographic (Geo) feature type carries the descriptive characteristics of a real-world entity.

### 4.3.4.3.1.3 Meta

Meta features contain information about other features within a dataset. Information defined by meta features override the default metadata values defined by the dataset descriptive records. Meta attribution on individual features overrides attribution on meta features.

### 4.3.4.3.1.4 Feature Relationship

A feature relationship links instances of one feature type with instances of the same or a different feature type.

### 4.3.4.3.1.5 Information Types

Information types are identifiable pieces of information in a dataset that can be shared between other features. They have attributes but have no relationship to any geometry; information types may reference other information types.

### 4.3.4.3.1.6 Attributes

SMART-Navigation product defines attributes as either simple or complex.

### 4.3.4.3.1.6.1 Simple Attributes

SMART-Navigation product uses nine types of simple attributes; they are listed in the following table:

| Type | Definition |
| --- | --- |
| Enumeration | A fixed list of valid identifiers of named literal values |
| Boolean | A value representing binary logic. The value can be either True or False. The default state for Boolean type attributes (i.e. where the attribute is not populated for the feature) is False. |
| Real | A signed Real (floating point) number consisting of a mantissa and an exponent |
| Integer | A signed integer number. The representation of an integer is encapsulation and usage dependent. |
| Text | An arbitrary-length sequence of characters including accents and special characters from a repertoire of one of the adopted character sets |
| Date | A date provides values for year, month and day according to the Gregorian Calendar. Character encoding of a date is a string which must follow the calendar date format (complete representation, basic format) for date specified by ISO 8601:1988. EXAMPLE 19980918 (YYYY-MM-DD) |
| Time | A time is given by an hour, minute and second. Character encoding of a time is a string that follows the local time (complete representation, basic format) format defined in ISO 8601:1988. EXAMPLE 183059 or 183059+0100 or 183059Z |
| Date and Time | A DateTime is a combination of a date and a time type. Character encoding of a DateTime shall follow ISO 8601:1988 EXAMPLE 19850412T101530 |
| Codelist | A type of flexible enumeration. A code list type is a list of literals which may be extended only in conformance with specified rules. |

### 4.3.4.3.1.6.2 Complex Attributes

Complex attributes are aggregations of other attributes that are either simple or complex. The aggregation is defined by means of attribute bindings.

### 4.3.4.3.1.6.3 Geometric Representation

Geometric representation is the digital description of the spatial component of an object as described in S-100 and ISO 19107. This product specification uses three types of geometries: GM_Point.

### 4.3.4.3.2 Feature Catalogue General

The Ship Dynamic feature catalogue describes the feature types, information types, attributes, attribute values, associations and roles which may be used in a Ship Dynamic data set.

### 4.3.4.3.2.1 Feature Types

- Geographic

- ShipIdentification

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| shipMRN | MRN is a naming scheme that can uniquely identify any maritime resource on a global scale. shipMRN is the unique value given to the ship. | URN | 1 | mandatory |  |
| MMSICode | The Maritime Mobile Service Identity (MMSI) Code is formed of a series of nine digits which are transmitted over the radio path in order to uniquely identify ship stations, ship earth stations, coast stations, coast earth stations, and group calls. These identities are formed in such a way that the identity or part thereof can be used by telephone and telex subscribers connected to the general telecommunications network principally to call ships automatically. (Adapted from Appendix 43 of the International Telecommunications Union Radio Regulations). | integer | 0..1 | option |  |

- ShipDynamicInformation

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| shipSpeed | Ship's speed information | complex attribute | 0..1 | option |  |
| shipMotion | Information related to the movement of the ship, such as heading, draft of ship, rotational motions. | complex attribute | 0..1 | option |  |
| measurementMethod | Indicates whether the dynamic information of the ship represents the current position or the information that predicts the future position. | enumeration | 1 | mandatory |  |
| shipDynamicInformationMeasurementDate | - | dateTime | 1 | mandatory |  |
| navigationStatus | Ship's navigation status | enum | 0..1 | option |  |
| shipLocation | Ship's location (longitude, latitude) | GM_Point | 1 | mandatory |  |

- ShipManoeuvringInformation

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| shipManoeuvringInformationMeasurementDate | - | dateTime | 1 | mandatory |  |
| mainEngine | - | complex attribute | 0..* | option |  |
| propellers | - | complex attribute | 0..* | option |  |
| rudder | - | complex attribute | 0..* | option |  |
| sideThruster | - | complex attribute | 0..* | option |  |

- Meta

- Cartographic

- Theme

### 4.3.4.3.2.2 Feature Relationship

- N/A

### 4.3.4.3.2.3 Information Relationship

- N/A

### 4.3.4.3.2.4 Information Type

- N/A

### 4.3.4.3.2.5 Simple Attribute

- Simple Attribute List

| Name | Description | Data Type | Remarks |
| --- | --- | --- | --- |
| shipMRN | MRN is a naming scheme that can uniquely identify any maritime resource on a global scale. shipMRN is the unique value given to the ship. | URN |  |
| MMSICode | The Maritime Mobile Service Identity (MMSI) Code is formed of a series of nine digits which are transmitted over the radio path in order to uniquely identify ship stations, ship earth stations, coast stations, coast earth stations, and group calls. These identities are formed in such a way that the identity or part thereof can be used by telephone and telex subscribers connected to the general telecommunications network principally to call ships automatically. (Adapted from Appendix 43 of the International Telecommunications Union Radio Regulations). | integer |  |
| speedReference | Indicates the type of speed measurement | enumeration |  |
| speedValue | speed value | real |  |
| speedUnits | The units for description of speed. | enumeration |  |
| navigationStatus | Ship's navigation status | enumeration |  |
| shipDynamicInformation MeasurementDate | Date & Time at which ship dynamic information was measure. | dateTime |  |
| measurementMethod | Indicates whether the dynamic information of the ship represents the current position or the information that predicts the future position. | enumeration |  |
| heading | Heading of the target in degrees. | real |  |
| courseOverGround | Course over ground in degrees. | real |  |
| forwardDraft | Draft at the forward of the ship, Average value of right-to-left values | real |  |
| forwardDraftPort | Draft at the forward of the ship, at the port | real |  |
| forwardDraftStarboard | Draft at the forward of the ship, at the starboard | real |  |
| midshipDraft | Draft at the mid of the ship, Average value of right-to-left values | real |  |
| midshipDraftPort | Draft at the mid of the ship, at the port | real |  |
| midshipDraftStarboard | Draft at the mid of the ship, at the starboard | real |  |
| afterDraft | Draft at the after of the ship, Average value of right-to-left values | real |  |
| afterDraftPort | Draft at the after of the ship, at the port | real |  |
| afterDraftStarboard | Draft at the after of the ship, at the starboard | real |  |
| heelAngle | The amount a vessel is heeled from the upright | real |  |
| trimAngle | The longitudinal draft at which the ship is subjected to external forces | real |  |
| ROT | Rate of turn in degrees per minute. | real |  |
| roll | The tilting rotation of a vessel about its longitudinal/X (front-back or bow-stern) axis | real |  |
| pitch | The up/down rotation of a vessel about its lateral/Y (side-to-side or port-starboard) axis | real |  |
| verticalCenterOfGravity | - | real |  |
| metacentricHeight | - | real |  |
| mainEngineID | - | text |  |
| engineTelegraphOrder | Engine Command Value - Engine Telegraph Lever Input Value | enumeration |  |
| settingRPM | The Main Engine RPM value, intentionally set by officer of the vessel for navigation. ( + : Ahead / - : Astern ) | real |  |
| propellerID | - | text |  |
| engineShaftRPM | Propeller RPM | real |  |
| propellerPitch | For variable pitch, the currently set Pitch value. | real |  |
| ruddersID | - | text |  |
| rudderAngleOrder | The Rudder Angle value, intentionally set by officer of the vessel for maneuvering. ( + : Starboard / - : Port ) | real |  |
| rudderAngleFeedback | The current rudder value displayed in the rudder indicator. | real |  |
| thrusterID | - | text |  |
| thrusterTelegraphOrder | Telegraph command value (for each thrust) | real |  |
| thrusterPitch | The value currently displayed in the Thrust Pitch Indicator. | real |  |
| thrusterLoad | The value currently displayed in the Thrust Load indicator. | real |  |
| shipManoeuvringInformation MeasurementDate | Date & Time at which ship manoeuvring information was measure. | dateTime |  |

- speedUnits

| Value | Label | Description |
| --- | --- | --- |
| 1 | meters per second | - |
| 2 | kilometers per hour | - |
| 3 | miles per hour | - |
| 4 | nautical miles per hour | - |

- speedReference

| Value | Label | Description |
| --- | --- | --- |
| 1 | other | - |
| 2 | speed over ground | The vessel's actual speed, determined by dividing the distance between successive fixes by the time between the fixes |
| 3 | speed through water | The vessel's actual speed, determined by substracting the speed over ground by the current speed |

- navigationStatus

| Value | Label | Description |
| --- | --- | --- |
| 1 | under way using engine | - |
| 2 | at anchor | - |
| 3 | not under command | - |
| 4 | restricted manoeuverability | - |
| 5 | constrained by her draught | - |
| 6 | moored | - |
| 7 | aground | - |
| 8 | engaged in fishing | - |
| 9 | under way sailing | - |
| 10 | engaged in fishing other than trawling | - |
| 11 | aircushion vessel in non displacement mode or WIG craft taking off, landing or in flight | - |
| 12 | power driven vessel towing astern | - |
| 13 | power driven vessel pushing ahead or towing alongside | - |
| 14 | in distress or requiring assistance | - |
| 15 | AIS SART, seeking to attract attention | - |
| 16 | undefined default | - |

- measurementMethod

| Value | Label | Description |
| --- | --- | --- |
| 1 | real time | - |
| 2 | prediction | - |

### 4.3.4.3.2.6 Complex Attribute

- shipSpeed

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| speedReference | Indicates the type of speed measurement | enumeration | 0..* | option |  |
| speedValue | speed value | real | 0..* | option |  |
| speedUnits | The units for description of speed. | enumeration | 0..* | option |  |

- shipMotion

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| heading | Heading of the target in degrees. | real | 0..1 | option |  |
| courseOverGround | Course over ground in degrees. | real | 0..1 | option |  |
| forwardDraft | Draft at the forward of the ship, Average value of right-to-left values | real | 0..1 | option |  |
| forwardDraftPort | Draft at the forward of the ship, Average value of right-to-left values | real | 0..1 | option |  |
| forwardDraftStarboard | Draft at the forward of the ship, at the starboard | real | 0..1 | option |  |
| midshipDraft | Draft at the mid of the ship, Average value of right-to-left values | real | 0..1 | option |  |
| midshipDraftPort | Draft at the mid of the ship, at the port | real | 0..1 | option |  |
| midshipDraftStarboard | Draft at the mid of the ship, at the starboard | real | 0..1 | option |  |
| afterDraft | Draft at the after of the ship, Average value of right-to-left values | real | 0..1 | option |  |
| afterDraftPort | Draft at the after of the ship, at the port | real | 0..1 | option |  |
| afterDraftStarboard | Draft at the after of the ship, at the starboard | real | 0..1 | option |  |
| heelAngle | The amount a vessel is heeled from the upright | real | 0..1 | option |  |
| trimAngle | The longitudinal draft at which the ship is subjected to external forces | real | 0..1 | option |  |
| ROT | Rate of turn in degrees per minute. | real | 0..1 | option |  |
| roll | The tilting rotation of a vessel about its longitudinal/X (front-back or bow-stern) axis | real | 0..1 | option |  |
| pitch | The up/down rotation of a vessel about its lateral/Y (side-to-side or port-starboard) axis | real | 0..1 | option |  |
| verticalCenterOf Gravity | - | real | 0..1 | option |  |
| metacentricHeight | - | real | 0..1 | option |  |

- mainEngine

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| mainEngineID | - | text | 0..1 | option |  |
| engineTelegraphOrder | Engine Command Value - Engine Telegraph Lever Input Value | enumeration | 0..1 | option |  |
| settingRPM | The Main Engine RPM value, intentionally set by officer of the vessel for navigation. ( + : Ahead / - : Astern ) | real | 1 | mandatory |  |

- propellers

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| propellerID | - | text | 0..1 | option |  |
| engineShaftRPM | Propeller RPM | real | 1..1 | mandatory |  |
| propellerPitch | For variable pitch, the currently set Pitch value. | real | 0..1 | option |  |

- rudder

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| ruddersID | - | text | 0..1 | option |  |
| rudderAngleOrder | The Rudder Angle value, intentionally set by officer of the vessel for maneuvering. ( + : Starboard / - : Port ) | real | 1 | mandatory |  |
| rudderAngleFeedback | The current rudder value displayed in the rudder indicator. | real | 0..1 | option |  |

- sideThruster

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| thrusterID | - | text | 0..1 | option |  |
| thrusterTelegraphOrder | Telegraph command value (for each thrust) | real | 0..1 | option |  |
| thrusterPitch | The value currently displayed in the Thrust Pitch Indicator. | real | 0..1 | option |  |
| thrusterLoad | The value currently displayed in the Thrust Load indicator. | real | 0..1 | option |  |

### 4.3.5 Coordinate Reference System(CRS)

### 4.3.5.1 Introduction

The location of an object in the S-100 standard is defined by means of coordinates which relate a feature to a position. The coordinate reference system used for this product specification is World Geodetic System 1984 (WGS 84) which is defined by the European Petroleum Survey Group (EPSG) code 4326, (or similar - North American Datum 1983 / Canadian Spatial Reference System).

Spatial data are expressed as latitude (φ) and longitude (λ) geographic coordinates. Latitude values are stored as a negative number to represent a position south of the Equator. Longitude values are stored as a negative number to represent a position west of the Prime Meridian. Coordinates are expressed as real value, degree / degree decimal format. Datasets conforming to this product specification are not projected.

Horizontal coordinate reference system:	WGS 84

Projection: 					None

Vertical coordinate reference system: 	              None

Temporal reference system: 		Gregorian calendar

Coordinate reference system registry: 	EPSG Geodetic Parameter Registry

Date type (according to ISO 19115-1):  	002 - publication

### 4.3.5.2 Horizontal Reference System

Positional data is expressed in latitude and longitude geographic coordinates to one of the reference horizontal reference systems defined in the HORDAT attribute.  Unless otherwise defined, the World Geodetic System 84 (WGS 84) will be used for Ship Dynamic data products.

### 4.3.5.3 Projection

Ship Dynamic data products are un-projected.

### 4.3.5.4 Vertical Coordinate Reference System

Ship Dynamic data products do not provide detailed vertical information.

### 4.3.5.5 Temporal Reference System

Time is measured by reference to Calendar dates and Clock time in accordance with ISO 19108:2002 Temporal Schema clause 5.4.4.

### 4.3.5.6 Ship Dynamic Data and Scale

Ship Dynamic data must be compiled in the best applicable scale. The use of the data itself is "scale independent". That means that the data can be used at any scale. S-100 allows the association of multiple spatial attributes to a single feature instance. In principle, each of these spatial attributes can be qualified by maximum and minimum scales.

For example, it is possible, within one dataset, to have a single instance of a feature that has more than one area geometry. Each of these geometries has different scale max/min attributes. Moreover, due to cluttering in smaller scales, the scale minimum attribute may be used to turn off portrayal of some features at smaller scales.

### 4.3.6 Data Quality

### 4.3.6.1 Introduction

Ship Dynamic products must be tested with the Ship Dynamic specific checks prior to release by the data producer. The data producer must review the check results and address any issues to ensure sufficient quality of the data products. The checks are a mix of data format validation checks, conformance to standard checks and logical consistency checks. But the checklist has not been written yet. It will be updated in the next version.

### Data Capture and classification

See DCEG for this section.

### 4.3.8 maintenance

### 4.3.8.1 Introduction

Datasets are maintained as needed and must include mechanisms for Ship Dynamic updating. Data updates will be made by new editions. The maintenance and update frequency of Ship Dynamic datasets should be defined by the producers (official national authority) implementing this specification.

Data Producers must use applicable sources to maintain and update data and provide a brief description of the sources that were used to produce the dataset in the appropriate metadata field.

The data product shall provide information on how the data is maintained and should describe the principles and criteria applied in maintenance regime. This should specify the expected frequency of updates.

Table 12 Maintenance and update frequency

| Item Name | Description | Multiplicity | Type |
| --- | --- | --- | --- |
| maintenanceAndUpdateFrequency | Frequency with which changes and additions are made to the data product (per update scope) | 1..* | MD_MaintenanceInformation (ISO 19115) |
| dataSource | Identification of the kinds of data sources usable to produce datasets | 1..* | LI_Source (ISO 19115) |
| productionProcess | Textual description of the production process applicable to the datasets (per scope or data source) | 1..* | LI_ProcessStep (ISO 19115) |

### 4.3.8.2 Production process for base and update datasets

Data Producers should follow their established production processes for maintaining and updating datasets. Data is produced against the DCEG and checked against the appropriate set of validation rules in Appendix E. (Appendix E is not included in this release.)

### 4.3.8.3 Dataset updates

N/A (Datset Update is not relevant in this model)

### 4.3.8.4 Support file updates

N/A (Support file is not included in this model)

### 4.3.8.5 Feature and portrayal catalogues

For each new version of the Ship Dynamic Product Specification a new feature and portrayal catalogue will be released. The system must be able to manage datasets and their catalogues that are created on different versions of the Ship Dynamic product specification.

### 4.3.8.6 Feature history, versions, and change tracking

If applications or production systems require versioning of individual instances of feature or information types, maintenance of histories, or change tracking, the methods for versioning, history management, and change tracking and display are left to the application or production system.

### 4.3.8.7 Dataset encryption

Details about dataset encryption are still to be determined, and may mirror the method described in S-101.

### 4.3.9 Portrayal

The portrayal catalogue model is defined in S-100 Part 9.

The “Ship Dynamic” Portrayal Catalogue will be available in an XML document which conforms to the S-100 XML Portrayal Catalogue Schema and is structured as follows:

Root ---- (contains the catalogue named “portrayal_catalogue.xml”)

|

|-- Pixmaps (contains XML files describing pixmaps)

|

|-- ColorProfiles (contains XML files with colour profiles and CSS2 style sheets)

|

|-- Symbols (contains SVG files with symbols)

|

|-- LineStyles (contains XML files with line styles)

|

|-- AreaFills (contains XML files area fills)

|

|-- Fonts (contains TrueType font files)

|

|-- Rules (contains XSLT files with templates)

### 4.3.9.1 Pixmaps

N/A

### 4.3.9.2 ColorProfiles

colorProfile.xml  (This is the same as that used for S-101.)

### 4.3.9.3 Symbols

| No. | Name | Description | Symbol |
| --- | --- | --- | --- |
| 1 | HeadingVessel | The shape of the symbol representing the vessel having a heading value. |  |
| 2 | NonHeadingVessel | The shape of the symbol representing the vessel having no any heading reference value. |  |
| 3 | QUESMRK1 | Object which is not sufficiently described to be symbolized, or for which no symbol exists in the symbol library |  |
| 4 | EMQUESM1 | Embedded symbol |  |

### 4.3.9.4 Line Styles

| No. | Name | Contents |
| --- | --- | --- |
| 1 | QUESMRK1 | <intervalLength>17.5</intervalLength> <pen width="0.64"><color>CHMGD</color></pen> <dash><start>2</start><length>3</length></dash> <dash><start>7</start><length>3</length></dash> <dash><start>14.6</start><length>3</length></dash> <symbol reference="EMQUESM1"><position>11.4</position></symbol> |

### 4.3.9.5 AreaFills

N/A

### 4.3.9.6 Fonts

N/A

### 4.3.9.7 Rules

| No | Name | Match ( Priority = 9 ) |  |  |
| --- | --- | --- | --- | --- |
| 1 | ShipDynamic | [@primitive='point'] |  |  |
|  | ViewingGroup | DisplalyPlane | DrawingPriority | Symbol |
|  | 50001 | OVERRADAR | 27 | NonHeadingVessel |

| No | Name | Match ( Priority = 2 ) |  |  |
| --- | --- | --- | --- | --- |
| 2 | ShipDynamic | [@primitive='point' and shipMotion\heading!=''] |  |  |
|  | ViewingGroup | DisplalyPlane | DrawingPriority | Symbol |
|  | 50001 | OVERRADAR | 27 | HeadingVessel |

| No | Name | Match |  |  |
| --- | --- | --- | --- | --- |
| 3 | default | [@primitive='Curve'] |  |  |
|  | ViewingGroup | DisplalyPlane | DrawingPriority | LineStyle |
|  | 50099 | OVERRADAR | 15 | QUESMRK1 |

| No | Name | Match |  |  |
| --- | --- | --- | --- | --- |
| 4 | default | [@primitive='Point'] |  |  |
|  | ViewingGroup | DisplalyPlane | DrawingPriority | Symbol |
|  | 50099 | OVERRADAR | 15 | QUESMRK1 |

| No | Name | Match |  |  |
| --- | --- | --- | --- | --- |
| 5 | default | [@primitive='Surface'] |  |  |
|  | ViewingGroup | DisplalyPlane | DrawingPriority | LineStyle |
|  | 50099 | OVERRADAR | 15 | Dash , 0.32 , CHMGD |

- default.xsl		(This is the same as that used for S-101.)

- attributeRules.xsl  	(This is the same as that used for S-101.)

- simpleLineStyle.xsl  	(This is the same as that used for S-101.)

- textStyle.xsl 		(This is the same as that used for S-101.)

### 4.3.10 data product format(encoding)

### 4.3.10.1 Introduction

The GML encoding of Ship Dynamic datasets is based on the S-100 profile of GML 3.2.1.

Format Name: Ship Dynamic GML

Version:	1.0

Character Set: UTF-8

Specification: SMART-Navigation Product Specification

### 4.3.10.2 Encoding of Latitude and Longitude

Values of latitude and longitude must be accurate to 7 decimal places. Coordinates must be encoded as decimals in the format described below. The encoding is indicated by multiplication factor fields defined in the dataset identification record by the S-100 GML schemas.

### 4.3.10.2.1 Encoding of coordinates as decimals

Values should be coded as decimal numbers with 7 or fewer digits after the decimal. The normative encoding is in degrees, with an accuracy of 10-7 degrees, i.e., 7 digits after the decimal point.

The decimal point must be indicated by the “.” character.

Trailing zeroes after the decimal point (and the decimal point itself if appropriate) may be omitted at producer discretion, but the accuracy must still be as indicated (e.g., 10-7 degrees for coordinates of default accuracy).

Latitude and longitude multiplication factors held in the Dataset Structure Information field under [coordMultFactorX] and [coordMultFactorY] must be set to a value corresponding to the encoding, i.e., {1} for coordinates encoded in decimal degrees.

EXAMPLE 1 A longitude = 42.0000 is converted into X = longitude * coordMultFactorX = 42.0000 * 1 = 42.0000000.

### 4.3.10.3 Text Attribute

Character strings must be encoded using the character set defined in ISO 10646-1, in Unicode Transformation Format-8 (UTF-8).

### 4.3.10.4 Mandatory Attribute Values

There are four reasons why attribute values may be considered mandatory:

- They determine whether a feature is in the display base,

- Certain features make no logical sense without specific attributes,

- Some attributes are necessary to determine which symbol is to be displayed,

- Some attributes are required for safety of navigation.

All mandatory attributes are identified in the Feature Catalogue.

### 4.3.10.5 Unknown Attribute Values

When a mandatory attribute code or tag is present but the attribute value is missing, it means that the producer wishes to indicate that this attribute value is unknown.  Missing mandatory attributes must be “nilled” with a GML nilReason attribute giving the reason for omission.

Optional attributes must be omitted altogether if the value is unknown or missing. They must not be “nilled.”

### 4.3.10.6 Structure of dataset files

### 4.3.10.6.1 Sequence of dataset files

The order of data objects in each dataset file is described below:

Dataset Identification Information

Dataset structure information

Spatial records for by-reference geometries

Point

Multi point

Curve

Composite Curve

Surface

Information objects

Feature objects (Geometry may be encoded inline or by reference.)

Meta features

Geo features

### 4.3.10.7 Numeric Attribute Encoding

Floating point and integer attribute values must not contain leading zeros.  Floating point attribute values must not contain non-significant trailing zeros.

### 4.3.10.8 Object identifiers

The “name” of feature records must provide a unique world-wide identifier of feature records. The “name” of the record is the combination of the subfields agency, featureObjectIdentifier, and featureIdentificationSubdivision elements of the featureObjectIdentifier element of the object.

Features, information types, collection objects, meta features, and geometries (inline or external) are all required by the schema to have a gml:id attribute with a value that is unique within the dataset. The gml:id values must be used as the reference for the object from another object in the same dataset or another dataset.

### 4.3.10.9 Dataset validation

Fields may be repeated or omitted as permitted by the XML schemas and the validation tests. Since XML schema cannot encode rules for conditional presence or attributes, these rules are checked by Schematron rules or other validation code.

### 4.3.10.10 Location of Data Product Format schema Files

The schema file will provide a future location and is available from that location.

### 4.3.10.11 Detailed documentation of Ship Dynamic XML Schema

See DCEG for this section.

### 4.3.11 data product delivery

This clause specifies the encoding and delivery mechanisms for a Ship Dynamic dataset.  Data which conforms to this product specification must be delivered by means of an exchange set.

### 4.3.11.1 Exchange set

Ship Dynamic datasets are grouped into exchange sets. Each exchange set consists of one or more Ship Dynamic datasets with an associated XML metadata file and a single Exchange Catalogue XML file containing metadata. It may also include one or more support files.

Units of Delivery:				Exchange Set

Transfer Size:					Unlimited

Medium Name:					Digital data delivery

Other Delivery Information:

Each dataset must be contained in a physically separate, uniquely identified file on the transfer medium.

Each exchange set has a single exchange catalogue which contains the discovery metadata for each dataset and references to any support files.

Support files are supplementary information which are linked to the features and information types by attributes. The attributes containing these links are described in the application schema and feature catalogue.

An exchange set may be encapsulated into a form suitable for transmission by a mapping called a transmission encoding. An encoding translates each of the elements of the exchange set into a logical form suitable for writing to media and for transmission online. An encoding may also define other elements in addition to the exchange set contents (i.e., media identification, data extents etc…) and also may define commercial constructs such as encryption and compression methods.

If the data is transformed (e.g., for encryption or compression purposes) its content must not be changed.

This product specification does not define the transmission encoding which must be used as a default for transmission of data between parties.

The exchange set elements are as follows:

Mandatory Elements

- Ship Dynamic datasets – GML encoding of features/attributes and their associated geometry and metadata.

- Exchange Catalogue – the XML encoded representation of exchange set catalogue features [discovery metadata].

Optional Elements

- Supplementary files – These are contained within the exchange set as files and the map from the name included within the dataset and the physical location on the media is defined within the Exchange Catalogue.

- Feature Catalogue – If it is necessary to deliver the latest feature catalogue to the end user it may be done using the Ship Dynamic exchange set mechanism for datasets – i.e., include the updated feature catalogue in an exchange set.

- Portrayal Catalogue - If it is necessary to deliver the latest portrayal catalogue to the end user it may be done using the Ship Dynamic exchange set mechanism for datasets datasets – i.e., include the updated feature catalogue in an exchange set.

### 4.3.11.2 Dataset

### 4.3.11.2.1 Datasets

Datasets are distributed as files which are part of exchange sets structured as described in this specification. The distribution media are left to the discretion of the producer and distributor.

The following types of dataset files may be produced and contained within an exchange set:

- New dataset and new edition of a dataset (base dataset):  Each new edition of a dataset must have the same name as the dataset that it replaces. A new edition can also be AtoN data that has previously been produced for this area.

### 4.3.11.2.2 Dataset size

Datasets must not exceed 1 MB.

### 4.3.11.2.3 Dataset file naming

Dataset files shall be named

CCXXXXXXXX_EEE.GML

The file name forms a unique identifier where:

- CC - the first two characters identify the issuing agency (mandatory).

- The third to tenth characters are optional and may be used in any way by the producer to provide the unique file name. The following characters are allowed in the dataset name, A to Z, 0 to 9 and the special character _ (underscore).

- EEE – new editions and re-issues use 000, and increment until a limit of 999 (mandatory).

- The maximum number of characters preceding _EEE is ten.

- GML – the character sequence “GML” or “gml”.

### 4.3.11.2.4 New Editions and re-issues

This section defines the sequencing of datasets for New Editions. In order to ensure that feature type updates are incorporated into an SYSTEM in the correct sequence without any omission, a number of parameters encoded in the data header and/or metadata are used in the following way:

| edition number | When a dataset is initially created, the edition number 1 is assigned to it. The edition number is increased by 1 at each new edition. |
| --- | --- |

### 4.3.11.2.5 Cancellations

Dataset cancellation is not considered I this product specification. In the SMART Navigation project, the system automatically cancels the dataset after 7 days from the date the dataset is received.

### 4.3.11.3 Support Files

N/A (Suport file is not included in this model)

### 4.3.11.4 Exchange Catalogue

N/A (Exchnage Catalogue is not used in SMART Navigation)

### 4.3.12 meta data

### 4.3.12.1 Introduction

The Ship Dynamic metadata description is based on the S-100 metadata document section, which is a profile of the ISO 19115 standard. These documents provide a structure for describing digital geographic data and define metadata elements, a common set of metadata terminology, definitions and extension procedures.

Two metadata packages are described in this product specification: dataset metadata and exchange set metadata.

Figure 9 Metadata packages

Note 1: Types with CI_, EX_, and MD_ prefixes are from packages defined in ISO 19115 and adapted by S-100. Types with S100_ prefix are from packages defined in S-100.

Note 2: When a dataset is terminated, the purpose metadata field is set to 3 (terminated), and the editionNumber metadata field is set to 0. All other metadata fields must be blank.

### 4.3.12.2 Dataset metadata

Dataset metadata is intended to describe information about a dataset. It facilitates the management and exploitation of data and is an important requirement for understanding the characteristics of a dataset. Whereas dataset metadata is usually fairly comprehensive, there is also a requirement for a constrained subset of metadata elements that are usually required for discovery purposes. Discovery metadata are often used for building web catalogues, and can help users determine whether a product or service is fit for purpose and where they can be obtained.

Table 13 Dataset metadata

| Name | Cardinality | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| S100_DataSetDiscoveryMetadata |  |  |  |  |
| fileName | 1 |  | CharacterString | Dataset file name |
| filePath | 1 |  | CharacterString | Full path from the exchange set root directory |
| description | 1 |  | CharacterString | Short description of the area covered by dataset, e.g., area, harbour, or port name, between two named locations etc. |
| dataProtection | 0..1 | {1} or {2} | Boolean | True : Encrypted False : Unencrypted |
| protectionScheme | 0..1 |  | CharacterString |  |
| digitalSignature | 0..1 |  | CharacterString |  |
| digitalSignatureReference | 0..1 |  | CharacterString |  |
| digitalSignatureValue | 0..1 |  | CharacterString |  |
| copyright | 0..1 |  | MD_LegalConstraints>MD_RestrictionCode <copyright> (ISO 19115-1) |  |
| classification | 0..1 |  | Class MD_SecurityConstraints>MD_ClassificationCode (codelist) ISO 19115-1 | 1. unclassified 2. restricted 3. confidential 4. secret 5. top secret |
| purpose | 1 | {1}, {2} | CharacterString | - New dataset - New edition |
| specificUsage | 1 |  | MD_USAGE>specificUsage (character string) MD_USAGE>userContactInfo (CI_Responsibility) | brief description of the resource and/or resource series usage |
| editionNumber | 1 | {1} | Integer | When a dataset is initially created, the edition number “1” is assigned to it. The edition number is increased by one with each new edition. |
| updateNumber | 1 |  | CharacterString | Update number 0 is assigned to a new dataset. |
| updateApplicationDate | 1 |  | Date | All updates dated on or before this date must have been applied by the producer. |
| issueDate | 1 |  | Date | Date on which the dataset was generated. |
| productSpecification | 1 |  | S100_ProductSpecification | See Notes below this table for constraints on values. |
| producingAgency | 1 |  | CI_Responsibility>CI_Organisation or CI_Responsibility>CI_Individual | Party responsible for generating the dataset. |
| optimumDisplayScale | 0..1 |  | Integer |  |
| maximumDisplayScale | 0..1 |  | Integer |  |
| minimumDisplayScale | 0..1 |  | Integer |  |
| horizontalDatumReference | 1 | EPSG | CharacterString |  |
| horizontalDatumValue | 1 | 4326 | Integer | WGS84 |
| verticalDatum | 0..1 |  | S100_VerticalAndSoundingDatum |  |
| soundingDatum | 0..1 |  | S100_VerticalAndSoundingDatum |  |
| dataType | 1 | GML | CharacterString | The only value allowed is “GML”. |
| otherDataTypeDescription | 0..1 |  | CharacterString |  |
| dataTypeVersion | 1 | 3.2.1 | CharacterString |  |
| dataCoverage | 1..* |  | S100_DataCoverage |  |
| comment | 0..1 |  | CharacterString | Any additional Information |
| layerID | 1..* | S-101 | CharacterString | Dataset must be used with ENC in an ECDIS. |

### 4.3.12.3 Update Dataset Metadata

N/A (Dataset Update is not relevant in this model)

### 4.3.12.4 Support file metadata

N/A (Support file is not included in this model)

### 4.3.12.5 Exchange set metadata

N/A (Exchange Catalogue is not used in SMART-Navigation)

### 4.3.12.6 Catalogue file Metadata

All Ship Dynamic Exchange Catalogue files must contain at least the mandatory metadata elements.

Table 14 Catalogue file metadata

| Name | Multiplicity | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| S100_ExchangeCatalogue |  |  |  |  |
| identifier | 1 |  | S100_CatalogueIdentifier |  |
| contact | 1 |  | S100_CataloguePointOfContact |  |
| productSpecification | 0..1 |  | S100_ProductSpecification | Conditional on all the datasets using the same product specification.. |
| metadataLanguage | 1 | English | CharacterString | All datasets conforming to this PS must use English language. A catalogue in English must be provided. Discovery metadata elements within catalogues have their own locale attributes and may be repeated in languages other than English. |
| exchangeCatalogueName | 1 |  | CharacterString | Catalogue filename |
| exchangeCatalogueDescription | 1 |  | CharacterString |  |
| exchangeCatalogueComment | 0..1 |  | CharacterString | Any additional Information |
| compressionFlag | 0..1 |  | Boolean | Yes or No |
| sourceMedia | 0..1 |  | CharacterString |  |
| replacedData | 0..1 |  | Boolean |  |
| dataReplacement | 0..1 |  | CharacterString |  |

## 4.4 Accident Warning

### 4.4.1 Overview

### 4.4.1.1 Introduction

This document has been produced by the member of SMART-Navigation project teams(Korean Register, Marine-Works) in response to a requirement to produce a data product that can offer ship's accident warning information related to Navigation Monitoring & Assistance service. It is based on the IHO S-100 framework specification and the ISO 19100 series of standards. It is a vector product specification that is primarily intended for encoding the information related to accident status in surrounding area and accident risk such as collision, grounding etc.

### 4.4.1.2 References

IHO S-100 IHO Universal Hydrographic Data Model Edition 4.0.0 (December 2017).

ISO 8601. 2004. Data elements and interchange formats - Information interchange - Representation of dates and times. 2004.

ISO 3166-1. 1997. Country Codes. 1997.

ISO 10646-1 Information technology - Universal Multiple-Octet Coded Character Set (UCS) -- Part 1: Architecture and Basic Multilingual Plane

ISO 19101-2:2008 Geographic Information - Rules for Application Schema

ISO/TS 19103:2005 Geographic Information - Conceptual schema language

ISO 19106:2004 Geographic Information – Profiles

ISO 19107:2003 Geographic Information – Spatial schema

ISO 19109:2005 Geographic Information - Rules for Application Schema

ISO 19111:2003 Geographic Information - Spatial referencing by coordinates

ISO 19115:2003+Corr1 (2006) Geographic Information - Metadata

ISO 19115-2:2009 Geographic Information - Metadata: Extensions for imagery and gridded data

ISO 19123:2005 Geographic Information - Schema for coverage geometry and functions

ISO 19129:2009 Geographic Information - Imagery gridded and coverage data framework

ISO 19131:2007 Geographic Information - Data product specifications

ISO 19136:2007 Geographic Information – Geography Markup Language

ISO 19136-2:2015, Geographic Information – Geography Markup Language.

ISO/TS 19139, Geographic Information – Metadata – XML schema implementation.

### 4.4.1.3 Terms, definitions and abbreviations

Terms and Definitions

The S-100 framework is based on the ISO 19100 series of geographic standards. The terms and definitions provided here are used to standardize the nomenclature found within that framework, whenever possible. They are taken from the references cited in clause 2.1. Modifications have been made when necessary.

- Application

Manipulation and processing of data in support of user requirements (ISO 19101)

- Application schema

Conceptual schema for data required by one or more applications (ISO 19101)

- Conceptual model

Model that defines concepts of a universe of discourse (ISO 19101)

- Conceptual schema

Formal description of a conceptual model (ISO 19101)

- Coverage

Feature that acts as a function to return values from its range for any direct position within its spatial, temporal or spatiotemporal domain (ISO 19123)

- Data product

Dataset or dataset series that conforms to a data product specification

- Data product specification

Detailed description of a dataset or dataset series together with additional information that will enable it to be created, supplied to and used by another party

- Dataset

Identifiable collection of data (ISO 19115)

- Feature

Abstraction of real world phenomena (ISO 19101)

- Feature association

Relationship that links instances of one feature type with instances of the same or a different feature  type (ISO19110)

- Feature attribute

Characteristic of a feature (ISO 19101)

- Geographic data

Data with implicit or explicit reference to a location relative to the Earth (ISO 19109)

- Metadata

Data about data (ISO 19115)

- Model

Abstraction of some aspects of reality (ISO 19109)

- Portrayal

Presentation of information to humans (ISO 19117)

- Quality

Totality of characteristics of a product that bear on its ability to satisfy stated and implied needs (ISO 19101)

Abbreviations

This product specification adopts the following convention for symbols and abbreviated terms:

- ASCII 	American Standard Code for Information Interchange

- ECDIS	Electronic Chart Display and Information Systems

- ECS	Electronic Chart Systems

- ENC	Electronic Navigational Chart

- GML	Geography Markup Language

- IHO	International Hydrographic Organization

- IOC	International Oceanographic Commission

- ISO	International Organization for Standardization

- UML	Unified Modelling Language

- URI	Uniformed Resource Identifier

- URL 	Uniform Resource Locator

- WMS	Web Map Service

- WFS	Web Feature Service

- WWW 	World Wide Web

- XML	Extensible Markup Language

- XSLT	eXtensible Stylesheet Language Transformations

### 4.4.1.4 General Data Product Description

Informal Description of Data Product

This clause contains general information about the data product

- Title:  	Accident Warning Product Specification.

- Abstract:	Accident Warning product specification describes the means to express ship risk alarm information such as ship accident status of surrounding area and accident(collision, grounding) risk, this may include details on risk of degree, expected accident time, accident alarm time, time to closet point of approach.

- Content:	Datasets conforming to this specification will contain relevant vector information of product specification for the Navigation Monitoring & Assistance service.

- Spatial Extent:	Korean Coast

- Specific Purpose: Describing ship accident status and ship accident risk information for utilization for Navigation monitoring & Assistance service.

### 4.4.1.5 Data product specification metadata

This information uniquely identifies this Product Specification and provides information about its creation and maintenance. For further information on dataset metadata see the metadata clause.

- Title: 	Accident Warning

- S-100 Version: 	4.0.0

- Accident Warning Version:	1.0.0

- Date: 	2019-03-08

- Language: 	English

- Classification: 	Unclassified

- Contact: 	32, Yuseong-daero 1312beon-gil, Yuseong-gu, Daejeon, Korea 32(Jang Dong) inside KRISO

- URL: 	http://www.smart-navigation.org

- Identifier: 	Accident Warning

- Maintenance: 	Amendments to this specification will be produced on a need’s basis. For reporting issues with this specification which correction, use the contact information.

### 4.4.2 Specification Scopes

This product specification describes one data product and therefore requires only one scope which is described below:

- Scope ID:  		Accident Warning datasets.

- Hierarchical level: 		MD_ScopeCode 005

- Hierarchical level name: 	dataset.

- Level description:		information applies to the dataset

- Extent:	EX_Extent.description: Korean Coast

### 4.4.3 Dataset identification

This section describes how to identify data sets that conform to the specification. A dataset that conforms to this Product Specification may be identified by its discovery metadata as defined in clause 12 of this specification. The information identifying the data product may include the following items from S-100 3.0.0 clause 11-6 (adapted from ISO 19115).

| Title | Accident Warning |
| --- | --- |
| Abstract | Accident Warning dataset is a vector dataset containing ship accident status and risk information regarding the Navigation Monitoring & Assistance service within a defined geographical area. |
| Acronym | AW |
| Content | Accident category, Accident risk, accident vessel, accident notice etc. |
| Topic Category | Location(domain code 013) |
| geographicDescription | Area specific to marine navigation within korean sea area |
| Purpose | Describing Navigation Monitoring & Assistance service except ship particular & dynamic information in the SMART-Navigation domain for utilization in SMART-Navigation devices. |
| Language | EN Additional values if any use CharacterString values from ISO 639-2 |

### 4.4.4 Data Content and structure

### 4.4.4.1 Introduction

The Accident Warning product is based on the S-100 General Feature Model (GFM), and is a feature-based vector product. All Accident Warning features and information classes are derived from one of the abstract classes FeatureType and InformationType defined in the Accident Warning application schema, which realize the GFM meta-classes S100_GF_FeatureType and S100_GF_InformationType respectively.

This clause contains the Application Schema expressed in UML and an associated Feature Catalogue.

### 4.4.4.2 Application Schema

The UML models in this clause are segments of the overall Accident Warning application schema, and include overviews of the feature classes, information classes, meta features, spatial types, and the relationships between them.

This clause contains a general overview of the classes and relationships in the Accident Warning application schema. Detailed information about how to use the feature types and information types to encode Accident Warning information is provided in the SMART-Navigation Data Classification and Encoding Guide (DCEG).

The following conventions are used in the UML diagrams depicting the application schema:

- Standard UML conventions for classes, associations, inheritance, roles, and multiplicities apply. These conventions are described in Part 1 of S-100.

- Italic font for a class name indicates an abstract class.

- Feature classes are depicted with pink background; the dark shade for abstract feature classes and the light shade for ordinary (non-abstract) feature classes.

- Complex attributes are depicted with a light yellow background.

- Enumeration lists and codelists are depicted with a green background. The numeric code corresponding to each listed value is shown to its right following an ‘=’ sign.

- No significance attaches to the colour of associations. (Complex diagrams may use different colours to distinguish associations that cross one another.)

- Where the association role or name is not explicitly shown, the default rules for roles and names apply:

- The role name is ‘the<CLASSNAME>’ where <CLASSNAME> is the name of the class to which that association end is linked.

- The association name is ‘<CLASSNAME1>_<CLASSNAME2>’ where <CLASSNAME1> is the source and <CLASSNAME2> the target. In case of a feature/information association the feature is the source. For feature/feature or information/information associations without explicit names the source/target are indicated by an arrowhead.

- Subclasses inherit the attributes and associations of their superclasses at all levels, unless such inheritance is explicitly overridden in the subclass.

### 4.4.4.2.1 Domain model

Figure 10 Accident Warning Overview

### 4.4.4.3 Feature Catalogue

### 4.4.4.3.1 Introduction

The Feature Catalogue describes the feature types, information types, attributes, attribute values, associations and roles which may be used in the product.  The Accident Warning Feature Catalogue is available in an XML document which conforms to the S-100 XML Feature Catalogue Schema.  Simple attributes used in this specification are listed in below.

Name: 	Accident Warning Feature Catalogue

Scope: 	Smart-Navigation

Version Number: 	1.0.0

Version Date:	2019-03-15

Producer:	SMART Navigation Project

Language:	English

### 4.4.4.3.1.1 Feature Types

Feature types contain descriptive attributes that characterize real-world entities. The word ‘feature’ may be used in one of two senses – feature type and feature instance. A feature type is a class and is defined in a Feature Catalogue. A feature instance is a single occurrence of the feature type and represented as an object in a dataset. A feature instance is located by a relationship to one or more spatial instances. A feature instance may exist without referencing a spatial instance.

### 4.4.4.3.1.2 Geographic

A Geographic (Geo) feature type carries the descriptive characteristics of a real-world entity.

### 4.4.4.3.1.3 Meta

Meta features contain information about other features within a dataset. Information defined by meta features override the default metadata values defined by the dataset descriptive records. Meta attribution on individual features overrides attribution on meta features.

### 4.4.4.3.1.4 Feature Relationship

A feature relationship links instances of one feature type with instances of the same or a different feature type.

### 4.4.4.3.1.5 Information Types

Information types are identifiable pieces of information in a dataset that can be shared between other features. They have attributes but have no relationship to any geometry; information types may reference other information types.

### 4.4.4.3.1.6 Attributes

SMART-Navigation product defines attributes as either simple or complex.

### 4.4.4.3.1.6.1 Simple Attributes

SMART-Navigation product uses nine types of simple attributes; they are listed in the following table:

| Type | Definition |
| --- | --- |
| Enumeration | A fixed list of valid identifiers of named literal values |
| Boolean | A value representing binary logic. The value can be either True or False. The default state for Boolean type attributes (i.e. where the attribute is not populated for the feature) is False. |
| Real | A signed Real (floating point) number consisting of a mantissa and an exponent |
| Integer | A signed integer number. The representation of an integer is encapsulation and usage dependent. |
| Text | An arbitrary-length sequence of characters including accents and special characters from a repertoire of one of the adopted character sets |
| Date | A date provides values for year, month and day according to the Gregorian Calendar. Character encoding of a date is a string which must follow the calendar date format (complete representation, basic format) for date specified by ISO 8601:1988. EXAMPLE 19980918 (YYYY-MM-DD) |
| Time | A time is given by an hour, minute and second. Character encoding of a time is a string that follows the local time (complete representation, basic format) format defined in ISO 8601:1988. EXAMPLE 183059 or 183059+0100 or 183059Z |
| Date and Time | A DateTime is a combination of a date and a time type. Character encoding of a DateTime shall follow ISO 8601:1988 EXAMPLE 19850412T101530 |
| Codelist | A type of flexible enumeration. A code list type is a list of literals which may be extended only in conformance with specified rules. |

### 4.4.4.3.1.6.2 Complex Attributes

Complex attributes are aggregations of other attributes that are either simple or complex. The aggregation is defined by means of attribute bindings.

### 4.4.4.3.1.6.3 Geometric Representation

Geometric representation is the digital description of the spatial component of an object as described in S-100 and ISO 19107. This product specification uses three types of geometries: GM_Point, GM_OrientableCurve, and GM_OrientableSurface.

### 4.4.4.3.2 Feature Catalogue General

The Accident Warning feature catalogue describes the feature types, information types, attributes, attribute values, associations and roles which may be used in an Accident Risk data set.

### 4.4.4.3.2.1 Feature Types

- Geographic

- AccidentBasicIdentification

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| shipMRN | MRN of the ship receiving accident risk information or accident information. | URN | 1 | mandatory |  |
| serviceMRN | MRN service that generated accident risk information or accident information. | URN | 0..1 | option |  |
| accidentCategory | Classification of accident information. | enumeration | 1 | mandatory |  |
| MMSI Code | MMSI Code of the ship at which the accident / accident occurred. | integer | 1..* | mandatory |  |
| idOfAccidentRiskObject | In case of the accident type is grounded or enters a hazardous area, Object ID that is expected to cause a ship accident. | text | 0..* | option |  |

- Accident Risk

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| alarmStatus | It is the state of the accident risk prediction alarm. | enumeration | 1 | mandatory |  |
| degreeOfRisk | The value calculated as a percentage of accident risk | real | 1 | mandatory |  |
| issuedDateTime | The time when the accident risk information was generated. | dateTime | 1 | mandatory |  |
| validTime | The time in which the accident risk information is valid and the remaining time in minutes. | dateTime | 1 | mandatory |  |
| distanceToClosestPointOfApproach | An estimated point in which the distance between two objects, of which at least one is in motion, will reach its minimum value; abbreviated CPA. The estimate is used to evaluate the risk of a collision of e.g. two ships. | complex attribute | 1 | mandatory |  |
| timeToClosestPointOfApproach | Time remaining till the Closest Point of Approach. | complex attribute | 1 | mandatory |  |
| pointOfCollisionExpected | In case of the accident type is collision, The expected collision point. | GM_Point | 0..1 | option |  |
| expectedAccidentArea | - | PointOrLineOrSurface | 1 | mandatory |  |

- AccidentVessel

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| MMSI Code | The Maritime Mobile Service Identity (MMSI) Code is formed of a series of nine digits which are transmitted over the radio path in order to uniquely identify ship stations, ship earth stations, coast stations, coast earth stations, and group calls. These identities are formed in such a way that the identity or part thereof can be used by telephone and telex subscribers connected to the general telecommunications network principally to call ships automatically. (Adapted from Appendix 43 of the International Telecommunications Union Radio Regulations). | integer | 1 | mandatory |  |
| courseOverGround |  | real | 1 | mandatory |  |
| position | The current position of the ship with the risk of accident. | GM_Point | 1 | mandatory |  |
| shipSpeed | Ship's speed information | complex attribute | 0..1 | option |  |
| remainingDistanceToRisk | - | complex attribute | 1 | mandatory |  |
| remainingTimeToRisk | - | complex attribute | 1 | mandatory |  |

- AccidentNotice

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| issuedDateTime | The time when the accident information was generated. | dateTime | 1 | mandatory |  |
| expectedDateTimeOfAccident | The estimated time of the accident predicted by the SV10 and may be different from the actual occurrence time of the accident. | dateTime | 1 | mandatory |  |
| accidentMessage | General information about the accident. | text | 1 | mandatory |  |
| accidentLocation | - | GM_Point | 1 | mandatory |  |

- Response

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| receiveDate | The time when the accident risk information was received. | dateTime | 1 | mandatory |  |
| responseDate | The time when the accident risk information was received and the response message was sent. | dateTime | 1 | mandatory |  |
| responseMessage | In the response, if the user acknowledged the accident risk information, write down 1. | enumeration | 1 | mandatory |  |

- Meta

- Cartographic

- Theme

### 4.4.4.3.2.2 Feature Relationship

| Role Type | Association Name | Role | Feature | Multiplicity |
| --- | --- | --- | --- | --- |
| Composition | vesselRisk | accidentInformation | AccidentRisk | 1 |
| Composition | vesselRisk | vesselInformation | AccidentVessel | 1..* |

### 4.4.4.3.2.3 Information Relationship

- N/A

### 4.4.4.3.2.4 Information Type

- N/A

### 4.4.4.3.2.5 Simple Attribute

- Simple Attribute List

| Name | Description | Data Type | Remarks |
| --- | --- | --- | --- |
| shipMRN | Maritime Identifier (MRN) based identification information developed by IALA | URN |  |
| serviceMRN | MRN service that generated accident risk information or accident information. | URN |  |
| MMSI Code | In case of the accident type is collision, The MMSI Code is provided with own ship and target ship, and otherwise provides only the own ship MMSI Code. | integer |  |
| idOfAccidentRisk Object | In case of the accident type is grounded or enters a hazardous area, Object ID that is expected to cause a ship accident | text |  |
| accidentCategory | Classification of accident information. | enumeration |  |
| issuedDateTime | The time when the accident risk information was generated. | dateTime |  |
| validTime | The time in which the accident risk information is valid and the remaining time in minutes. | dateTime |  |
| alarmStatus | It is the state of the accident risk prediction alarm. This value is used to confirm that the accident risk information has been generated, provided to the ship and received and acknowledged by the ship. | enumeration |  |
| distance | - | real |  |
| distanceUnit | - | enumeration |  |
| timeCount | - | real |  |
| timeUnit | - | enumeration |  |
| degreeOfRisk | The value calculated as a percentage of accident risk | real |  |
| speedReference | Indicates the type of speed measurement | enumeration |  |
| speedValue | - | real |  |
| speedUnits | The units for description of speed. | enumeration |  |
| courseOverGround | - | real |  |
| expectedDateAnd TimeOfAccident | The estimated time of the accident predicted by the SV10 and may be different from the actual occurrence time of the accident. | dateTime |  |
| accidentMessage | General information about the accident. | text |  |
| receiveDate | The time when the accident risk information was received. | dateTime |  |
| responseDate | The time when the accident risk information was received and the response message was sent. | dateTime |  |
| responseMessage | In the response, if the user acknowledged the accident risk information, write down 1. | integer |  |

- time Unit

| Value | Label | Description |
| --- | --- | --- |
| 1 | second | second as a unit of time |
| 2 | minute | minute as a unit of time |
| 3 | hour | hour as a unit of time |

- distance Unit

| Value | Label | Description |
| --- | --- | --- |
| 1 | meter | meter as a unit of distance |
| 2 | kilometer | kilometer as a unit of distance |
| 3 | nautical mile | nautical mile as a unit of distance |

- accidentCategory

| Value | Label | Description |
| --- | --- | --- |
| 1 | head on | - |
| 2 | crossing | - |
| 3 | over taking | - |
| 4 | shore line | - |
| 5 | object | - |
| 6 | rock | - |
| 7 | get in | - |
| 8 | get out | - |
| 9 | in danger | - |

- speed Reference

| Value | Label | Description |
| --- | --- | --- |
| 1 | other | Other speed reference value unit |
| 2 | speed over ground | Vessel speed based on land |
| 3 | speed through water | Vessel speed under the influence of wind, current, etc. |

- alarmStatus

| Value | Label | Description |
| --- | --- | --- |
| 1 | first occurrence of accident | - |
| 2 | only A ship accident perception | - |
| 3 | only B ship accident perception | - |
| 4 | all ship accident perception | - |

### 4.4.4.3.2.6 Complex Attribute

- distanceToClosestPointOfApproach

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| distance | The value to present the distance between two specific points. | real | 1 | mandatory |  |
| distanceUnit | The unit of distance value | enumeration | 0..1 | option |  |

- remainingDistanceToRisk

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| distance | The value to present the distance between two specific points. | real | 1 | mandatory |  |
| distanceUnit | The value to present the distance between two specific points. | enumeration | 0..1 | option |  |

- timeToClosestPointOfApproach

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| timeUnit | The unit of time value | real | 0..1 | mandatory |  |
| timeCount | The value to present the difference between two specific times. | enumeration | 1 | mandatory |  |

- remainingTimeToRisk

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| timeUnit | The unit of time value | real | 0..1 | mandatory |  |
| timeCount | The value to present the difference between two specific times. | enumeration | 1 | mandatory |  |

- Ship Speed

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| speedReference | Indicates the type of speed measurement | enumeration | 1 | mandatory |  |
| speedValue | speed value | real | 1 | mandatory |  |
| speedUnits | The units for description of speed. | enumeration | 0..1 | option |  |

### 4.4.5 Coordinate Reference System(CRS)

### 4.4.5.1 Introduction

The location of an object in the S-100 standard is defined by means of coordinates which relate a feature to a position. The coordinate reference system used for this product specification is World Geodetic System 1984 (WGS 84) which is defined by the European Petroleum Survey Group (EPSG) code 4326, (or similar - North American Datum 1983 / Canadian Spatial Reference System).

Spatial data are expressed as latitude (φ) and longitude (λ) geographic coordinates. Latitude values are stored as a negative number to represent a position south of the Equator. Longitude values are stored as a negative number to represent a position west of the Prime Meridian. Coordinates are expressed as real value, degree / degree decimal format. Datasets conforming to this product specification are not projected.

Horizontal coordinate reference system:	WGS 84

Projection: 					None

Vertical coordinate reference system: 	              None

Temporal reference system: 		              Gregorian calendar

Coordinate reference system registry: 	              EPSG Geodetic Parameter Registry

Date type (according to ISO 19115-1):  	              002 - publication

### 4.4.5.2 Horizontal Reference System

Positional data is expressed in latitude and longitude geographic coordinates to one of the reference horizontal reference systems defined in the HORDAT attribute.  Unless otherwise defined, the World Geodetic System 84 (WGS 84) will be used for Accident Warning data products.

### 4.4.5.3 Projection

Accident Warning data products are un-projected.

### 4.4.5.4 Vertical Coordinate Reference System

Accident Warning data products do not provide detailed vertical information.

### 4.4.5.5 Temporal Reference System

Time is measured by reference to Calendar dates and Clock time in accordance with ISO 19108:2002 Temporal Schema clause 5.4.4.

### 4.4.5.6 Accident Warning Data and Scale

Accident Warning data must be compiled in the best applicable scale. The use of the data itself is "scale independent". That means that the data can be used at any scale. S-100 allows the association of multiple spatial attributes to a single feature instance. In principle, each of these spatial attributes can be qualified by maximum and minimum scales.

For example, it is possible, within one dataset, to have a single instance of a feature that has more than one area geometry. Each of these geometries has different scale max/min attributes. Moreover, due to cluttering in smaller scales, the scale minimum attribute may be used to turn off portrayal of some features at smaller scales.

### 4.4.6 Data Quality

### 4.4.6.1 Introduction

Accident Warning products must be tested with the Accident Warning specific checks prior to release by the data producer. The data producer must review the check results and address any issues to ensure sufficient quality of the data products. The checks are a mix of data format validation checks, conformance to standard checks and logical consistency checks. But the checklist has not been written yet. It will be updated in the next version.

### 4.4.7 Data Capture and classification

See DCEG for this section.

### 4.4.8 maintenance

### 4.4.8.1 Introduction

Datasets are maintained as needed and must include mechanisms for Accident Warning updating. Data updates will be made by new editions. The maintenance and update frequency of Accident Warning datasets should be defined by the producers (official national authority) implementing this specification.

Data Producers must use applicable sources to maintain and update data and provide a brief description of the sources that were used to produce the dataset in the appropriate metadata field.

The data product shall provide information on how the data is maintained and should describe the principles and criteria applied in maintenance regime. This should specify the expected frequency of updates.

Table 15 Maintenance and update frequency

| Item Name | Description | Multiplicity | Type |
| --- | --- | --- | --- |
| maintenanceAndUpdateFrequency | Frequency with which changes and additions are made to the data product (per update scope) | 1..* | MD_MaintenanceInformation (ISO 19115) |
| dataSource | Identification of the kinds of data sources usable to produce datasets | 1..* | LI_Source (ISO 19115) |
| productionProcess | Textual description of the production process applicable to the datasets (per scope or data source) | 1..* | LI_ProcessStep (ISO 19115) |

### 4.4.8.2 Production process for base and update datasets

Data Producers should follow their established production processes for maintaining and updating datasets. Data is produced against the DCEG and checked against the appropriate set of validation rules in Appendix E. (Appendix E is not included in this release.)

### 4.4.8.3 Dataset updates

N/A (Dataset Update is not relevant in this model.)

### 4.4.8.4 Support file updates

N/A (Supprot file is not included in this model.)

### 4.4.8.5 Feature and portrayal catalogues

For each new version of the Accident Warning Product Specification a new feature and portrayal catalogue will be released. The system must be able to manage datasets and their catalogues that are created on different versions of the Accident Warning product specification.

### 4.4.8.6 Feature history, versions, and change tracking

If applications or production systems require versioning of individual instances of feature or information types, maintenance of histories, or change tracking, the methods for versioning, history management, and change tracking and display are left to the application or production system.

### 4.4.8.7 Dataset encryption

Details about dataset encryption are still to be determined, and may mirror the method described in S-101.

### 4.4.9 Portrayal

The portrayal catalogue model is defined in S-100 Part 9.

The “Accident Warning” Portrayal Catalogue will be available in an XML document which conforms to the S-100 XML Portrayal Catalogue Schema and is structured as follows:

Root ---- (contains the catalogue named “portrayal_catalogue.xml”)

|

|-- Pixmaps (contains XML files describing pixmaps)

|

|-- ColorProfiles (contains XML files with colour profiles and CSS2 style sheets)

|

|-- Symbols (contains SVG files with symbols)

|

|-- LineStyles (contains XML files with line styles)

|

|-- AreaFills (contains XML files area fills)

|

|-- Fonts (contains TrueType font files)

|

|-- Rules (contains XSLT files with templates)

### 4.4.9.1 Pixmaps

N/A

### 4.4.9.2 ColorProfiles

colorProfile.xml  (This is the same as that used for S-101.)

### 4.4.9.3 Symbols

| No. | Name | Description | Symbol |
| --- | --- | --- | --- |
| 1 | AccidentNotice | Object that represents the location of the accident provided by SMART Navigation. |  |
| 2 | CollisionRiskPoint | Objects whose location is predicted to have a collision risk provided by the SMART Navigation service. |  |
| 3 | GroundingRiskPoint | Objects whose location is predicted to have a grounding risk provided by the SMART Navigation service. |  |
| 4 | RiskVessel | Object representing the vessel subject to accident risk information provided by the SMART Navigation service. |  |
| 5 | QUESMRK1 | Object which is not sufficiently described to be symbolized, or for which no symbol exists in the symbol library |  |
| 6 | EMQUESM1 | Embedded symbol |  |

### 4.4.9.4 Line Styles

| No. | Name | Contents |
| --- | --- | --- |
| 1 | QUESMRK1 | <intervalLength>17.5</intervalLength> <pen width="0.64"><color>CHMGD</color></pen> <dash><start>2</start><length>3</length></dash> <dash><start>7</start><length>3</length></dash> <dash><start>14.6</start><length>3</length></dash> <symbol reference="EMQUESM1"><position>11.4</position></symbol> |

### 4.4.9.5 AreaFills

N/A

### 4.4.9.6 Fonts

N/A

### 4.4.9.7 Rules

| No | Name | Match ( Priority = 1 ) |  |  |
| --- | --- | --- | --- | --- |
| 1 | AccidentNotice | [@primitive='point'] |  |  |
|  | ViewingGroup | DisplalyPlane | DrawingPriority | Symbol |
|  | 50101 | OVERRADAR | 28 | AccidentNotice |

| No | Name | Match ( Priority = 1 ) |  |  |
| --- | --- | --- | --- | --- |
| 2 | AccidentRisk | [@primitive='point' and accidentCategory=1] [@primitive='point' and accidentCategory=2] [@primitive='point' and accidentCategory=3] |  |  |
|  | ViewingGroup | DisplalyPlane | DrawingPriority | Symbol |
|  | 50102 | OVERRADAR | 28 | CollisionRiskPoint |

| No | Name | Match ( Priority = 2 ) |  |  |
| --- | --- | --- | --- | --- |
| 3 | AccidentRisk | [@primitive='surface' and accidentCategory=1] [@primitive='surface ' and accidentCategory=2] [@primitive='surface ' and accidentCategory=3] |  |  |
|  | ViewingGroup | DisplalyPlane | DrawingPriority | LineStyle |
|  | 50102 | OVERRADAR | 28 | Solid , 0.64 , SCLBR |

| No | Name | Match |  |  |
| --- | --- | --- | --- | --- |
| 4 | AccidentRisk | [@primitive='point' and accidentCategory=4] [@primitive='point ' and accidentCategory=5] [@primitive='point ' and accidentCategory=6] |  |  |
|  | ViewingGroup | DisplalyPlane | DrawingPriority | Symbol |
|  | 50103 | OVERRADAR | 28 | GroundingRiskPoint |
| No | Name | Match |  |  |
| 5 | RiskVessel | [@primitive='point'] |  |  |
|  | ViewingGroup | DisplalyPlane | DrawingPriority | Symbol |
|  | 50104 | OVERRADAR | 28 | RiskVessel |

| No | Name | Match |  |  |
| --- | --- | --- | --- | --- |
| 6 | default | [@primitive='Curve'] |  |  |
|  | ViewingGroup | DisplalyPlane | DrawingPriority | LineStyle |
|  | 50099 | OVERRADAR | 15 | QUESMRK1 |

| No | Name | Match |  |  |
| --- | --- | --- | --- | --- |
| 7 | default | [@primitive='Point'] |  |  |
|  | ViewingGroup | DisplalyPlane | DrawingPriority | Symbol |
|  | 50099 | OVERRADAR | 15 | QUESMRK1 |

| No | Name | Match |  |  |
| --- | --- | --- | --- | --- |
| 8 | default | [@primitive='Surface'] |  |  |
|  | ViewingGroup | DisplalyPlane | DrawingPriority | LineStyle |
|  | 50099 | OVERRADAR | 15 | Dash , 0.32 , CHMGD |

- default.xsl		(This is the same as that used for S-101.)

- attributeRules.xsl  	(This is the same as that used for S-101.)

- simpleLineStyle.xsl  	(This is the same as that used for S-101.)

- textStyle.xsl 		(This is the same as that used for S-101.)

### 4.4.10 data product format(encoding)

### 4.4.10.1 Introduction

The GML encoding of Accident Warning datasets is based on the S-100 profile of GML 3.2.1.

Format Name: Accident Warning GML

Version:	1.0

Character Set: UTF-8

Specification: SMART-Navigation Product Specification

### 4.4.10.2 Encoding of Latitude and Longitude

Values of latitude and longitude must be accurate to 7 decimal places. Coordinates must be encoded as decimals in the format described below. The encoding is indicated by multiplication factor fields defined in the dataset identification record by the S-100 GML schemas.

### 4.4.10.2.1 Encoding of coordinates as decimals

Values should be coded as decimal numbers with 7 or fewer digits after the decimal. The normative encoding is in degrees, with an accuracy of 10-7 degrees, i.e., 7 digits after the decimal point.

The decimal point must be indicated by the “.” character.

Trailing zeroes after the decimal point (and the decimal point itself if appropriate) may be omitted at producer discretion, but the accuracy must still be as indicated (e.g., 10-7 degrees for coordinates of default accuracy).

Latitude and longitude multiplication factors held in the Dataset Structure Information field under [coordMultFactorX] and [coordMultFactorY] must be set to a value corresponding to the encoding, i.e., {1} for coordinates encoded in decimal degrees.

EXAMPLE 1 A longitude = 42.0000 is converted into X = longitude * coordMultFactorX = 42.0000 * 1 = 42.0000000.

### 4.4.10.3 Text Attribute

Character strings must be encoded using the character set defined in ISO 10646-1, in Unicode Transformation Format-8 (UTF-8).

### 4.4.10.4 Mandatory Attribute Values

There are four reasons why attribute values may be considered mandatory:

- They determine whether a feature is in the display base,

- Certain features make no logical sense without specific attributes,

- Some attributes are necessary to determine which symbol is to be displayed,

- Some attributes are required for safety of navigation.

All mandatory attributes are identified in the Feature Catalogue.

### 4.4.10.5 Unknow Attribute Values

When a mandatory attribute code or tag is present but the attribute value is missing, it means that the producer wishes to indicate that this attribute value is unknown.  Missing mandatory attributes must be “nilled” with a GML nilReason attribute giving the reason for omission.

Optional attributes must be omitted altogether if the value is unknown or missing. They must not be “nilled.”

### 4.4.10.6 Structure of dataset files

### 4.4.10.6.1 Sequence of dataset files

The order of data objects in each dataset file is described below:

Dataset Identification Information

Dataset structure information

Spatial records for by-reference geometries

Point

Multi point

Curve

Composite Curve

Surface

Information objects

Feature objects (Geometry may be encoded inline or by reference.)

Meta features

Geo features

### 4.4.10.7 Numeric Attribute Encoding

Floating point and integer attribute values must not contain leading zeros.  Floating point attribute values must not contain non-significant trailing zeros.

### 4.4.10.8 Object identifiers

The “name” of feature records must provide a unique world-wide identifier of feature records. The “name” of the record is the combination of the subfields agency, featureObjectIdentifier, and featureIdentificationSubdivision elements of the featureObjectIdentifier element of the object.

Features, information types, collection objects, meta features, and geometries (inline or external) are all required by the schema to have a gml:id attribute with a value that is unique within the dataset. The gml:id values must be used as the reference for the object from another object in the same dataset or another dataset.

### 4.4.10.9 Dataset validation

Fields may be repeated or omitted as permitted by the XML schemas and the validation tests. Since XML schema cannot encode rules for conditional presence or attributes, these rules are checked by Schematron rules or other validation code.

### 4.4.10.10 Location of Data Product Format schema Files

The schema file will provide a future location and is available from that location.

### 4.4.10.11 Detailed documentation of Accident Waring XML Schema

See DCEG for this section.

### 4.4.11 data product delivery

This clause specifies the encoding and delivery mechanisms for an Accident Warning dataset.  Data which conforms to this product specification must be delivered by means of an exchange set.

### 4.4.11.1 Exchange set

Accident Warning datasets are grouped into exchange sets. Each exchange set consists of one or more Accident Warning datasets with an associated XML metadata file and a single Exchange Catalogue XML file containing metadata. It may also include one or more support files.

Units of Delivery:				Exchange Set

Transfer Size:					Unlimited

Medium Name:					Digital data delivery

Other Delivery Information:

Each dataset must be contained in a physically separate, uniquely identified file on the transfer medium.

Each exchange set has a single exchange catalogue which contains the discovery metadata for each dataset and references to any support files.

Support files are supplementary information which are linked to the features and information types by attributes. The attributes containing these links are described in the application schema and feature catalogue.

An exchange set may be encapsulated into a form suitable for transmission by a mapping called a transmission encoding. An encoding translates each of the elements of the exchange set into a logical form suitable for writing to media and for transmission online. An encoding may also define other elements in addition to the exchange set contents (i.e., media identification, data extents etc…) and also may define commercial constructs such as encryption and compression methods.

If the data is transformed (e.g., for encryption or compression purposes) its content must not be changed.

This product specification does not define the transmission encoding which must be used as a default for transmission of data between parties.

The exchange set elements are as follows:

Mandatory Elements

- Accident Warning datasets – GML encoding of features/attributes and their associated geometry and metadata.

- Exchange Catalogue – the XML encoded representation of exchange set catalogue features [discovery metadata].

Optional Elements

- Supplementary files – These are contained within the exchange set as files and the map from the name included within the dataset and the physical location on the media is defined within the Exchange Catalogue.

- Feature Catalogue – If it is necessary to deliver the latest feature catalogue to the end user it may be done using the Accident Warning exchange set mechanism for datasets – i.e., include the updated feature catalogue in an exchange set.

- Portrayal Catalogue - If it is necessary to deliver the latest portrayal catalogue to the end user it may be done using the Accident Warning exchange set mechanism for datasets datasets – i.e., include the updated feature catalogue in an exchange set.

### 4.4.11.2 Dataset

### 4.4.11.2.1 Datasets

Datasets are distributed as files which are part of exchange sets structured as described in this specification. The distribution media are left to the discretion of the producer and distributor.

The following types of dataset files may be produced and contained within an exchange set:

- New dataset and new edition of a dataset (base dataset):  Each new edition of a dataset must have the same name as the dataset that it replaces.  A new edition can also be AtoN data that has previously been produced for this area.

### 4.4.11.2.2 Dataset size

Datasets must not exceed 1 MB.

### 4.4.11.2.3 Dataset file naming

Dataset files shall be named

CCXXXXXXXX_EEE.GML

The file name forms a unique identifier where:

- CC - the first two characters identify the issuing agency (mandatory).

- The third to tenth characters are optional and may be used in any way by the producer to provide the unique file name. The following characters are allowed in the dataset name, A to Z, 0 to 9 and the special character _ (underscore).

- EEE – new editions and re-issues use 000, and increment until a limit of 999 (mandatory).

- The maximum number of characters preceding _EEE is ten.

- GML – the character sequence “GML” or “gml”.

### 4.4.11.2.4 New Editions and re-issues

This section defines the sequencing of datasets for New Editions. In order to ensure that feature type updates are incorporated into an SYSTEM in the correct sequence without any omission, a number of parameters encoded in the data header and/or metadata are used in the following way:

| edition number | when a dataset is initially created, the edition number 1 is assigned to it. The edition number is increased by 1 at each new edition. |
| --- | --- |

### 4.4.11.2.5 Cancellations

Dataset cancellation is not considered I this product specification. In the SMART Navigation project, the system automatically cancels the dataset after 7 days from the date the dataset is received.

### 4.4.11.3 Support Files

N/A (Support file is not included in this model)

### 4.4.11.4 Exchange Catalogue

N/A (Exchange catalogue is not used in Mart Navigation)

### 4.4.12 meta data

### 4.4.12.1 Introduction

The Accident Warning metadata description is based on the S-100 metadata document section, which is a profile of the ISO 19115 standard. These documents provide a structure for describing digital geographic data and define metadata elements, a common set of metadata terminology, definitions and extension procedures.

Two metadata packages are described in this product specification: dataset metadata and exchange set metadata.

Figure 11 Metadata packages

Note 1: Types with CI_, EX_, and MD_ prefixes are from packages defined in ISO 19115 and adapted by S-100. Types with S100_ prefix are from packages defined in S-100.

Note 2: When a dataset is terminated, the purpose metadata field is set to 3 (terminated), and the editionNumber metadata field is set to 0. All other metadata fields must be blank.

### 4.4.12.2 Dataset metadata

Dataset metadata is intended to describe information about a dataset. It facilitates the management and exploitation of data and is an important requirement for understanding the characteristics of a dataset. Whereas dataset metadata is usually fairly comprehensive, there is also a requirement for a constrained subset of metadata elements that are usually required for discovery purposes. Discovery metadata are often used for building web catalogues, and can help users determine whether a product or service is fit for purpose and where they can be obtained.

Table 16 Dataset metadata

| Name | Cardinality | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| S100_DataSetDiscoveryMetadata |  |  |  |  |
| fileName | 1 |  | CharacterString | Dataset file name |
| filePath | 1 |  | CharacterString | Full path from the exchange set root directory |
| description | 1 |  | CharacterString | Short description of the area covered by dataset, e.g., area, harbour, or port name, between two named locations etc. |
| dataProtection | 0..1 | {1} or {2} | Boolean | True: Encrypted False: Unencrypted |
| protectionScheme | 0..1 |  | CharacterString |  |
| digitalSignature | 0..1 |  | CharacterString |  |
| digitalSignatureReference | 0..1 |  | CharacterString |  |
| digitalSignatureValue | 0..1 |  | CharacterString |  |
| copyright | 0..1 |  | MD_LegalConstraints>MD_RestrictionCode <copyright> (ISO 19115-1) |  |
| classification | 0..1 |  | Class MD_SecurityConstraints>MD_ClassificationCode (codelist) ISO 19115-1 | 1. unclassified 2. restricted 3. confidential 4. secret 5. top secret |
| purpose | 1 | {1}, {2} | CharacterString | - New dataset - New edition |
| specificUsage | 1 |  | MD_USAGE>specificUsage (character string) MD_USAGE>userContactInfo (CI_Responsibility) | brief description of the resource and/or resource series usage |
| editionNumber | 1 | {1} | Integer | When a dataset is initially created, the edition number “1” is assigned to it. The edition number is increased by one with each new edition. |
| updateNumber | 1 |  | CharacterString | Update number 0 is assigned to a new dataset. |
| updateApplicationDate | 1 |  | Date | All updates dated on or before this date must have been applied by the producer. |
| issueDate | 1 |  | Date | Date on which the dataset was generated. |
| productSpecification | 1 |  | S100_ProductSpecification | See Notes below this table for constraints on values. |
| producingAgency | 1 |  | CI_Responsibility>CI_Organisation or CI_Responsibility>CI_Individual | Party responsible for generating the dataset. |
| optimumDisplayScale | 0..1 |  | Integer |  |
| maximumDisplayScale | 0..1 |  | Integer |  |
| minimumDisplayScale | 0..1 |  | Integer |  |
| horizontalDatumReference | 1 | EPSG | CharacterString |  |
| horizontalDatumValue | 1 | 4326 | Integer | WGS84 |
| verticalDatum | 0..1 |  | S100_VerticalAndSoundingDatum |  |
| soundingDatum | 0..1 |  | S100_VerticalAndSoundingDatum |  |
| dataType | 1 | GML | CharacterString | The only value allowed is “GML”. |
| otherDataTypeDescription | 0..1 |  | CharacterString |  |
| dataTypeVersion | 1 | 3.2.1 | CharacterString |  |
| dataCoverage | 1..* |  | S100_DataCoverage |  |
| comment | 0..1 |  | CharacterString | Any additional Information |
| layerID | 1..* | S-101 | CharacterString | Dataset must be used with ENC in an ECDIS. |

### 4.4.12.3 Update Dataset Metadata

N/A (Dataset Update is not relevant in this model)

### 4.4.12.4 Support file metadata

N/A (Support file is not included in this model)

### 4.4.12.5 Exchange set metadata

N/A (Exchange Catalogue is not used in SMART-Navigation)

### 4.4.12.6 Catalogue file Metadata

All Accident Warning Exchange Catalogue files must contain at least the mandatory metadata elements.

Table 17 Catalogue file metadata

| Name | Multiplicity | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| S100_ExchangeCatalogue |  |  |  |  |
| identifier | 1 |  | S100_CatalogueIdentifier |  |
| contact | 1 |  | S100_CataloguePointOfContact |  |
| productSpecification | 0..1 |  | S100_ProductSpecification | Conditional on all the datasets using the same product specification.. |
| metadataLanguage | 1 | English | CharacterString | All datasets conforming to this PS must use English language. A catalogue in English must be provided. Discovery metadata elements within catalogues have their own locale attributes and may be repeated in languages other than English. |
| exchangeCatalogueName | 1 |  | CharacterString | Catalogue filename |
| exchangeCatalogueDescription | 1 |  | CharacterString |  |
| exchangeCatalogueComment | 0..1 |  | CharacterString | Any additional Information |
| compressionFlag | 0..1 |  | Boolean | Yes or No |
| sourceMedia | 0..1 |  | CharacterString |  |
| replacedData | 0..1 |  | Boolean |  |
| dataReplacement | 0..1 |  | CharacterString |  |

## 4.5 SHip alarm & emergency

### 4.5.1 Overview

### 4.5.1.1 Introduction

This document has been produced by the member of SMART-Navigation project team(Korean Register, Marine-Works) in response to a requirement to produce a data product about emergency guidance based on the alarm information of the collected ship. It is based on the IHO S-100 framework specification and the ISO 19100 series of standards. It is a textTable product specification that is primarily intended for encoding the information of navigation equipment alarm information, engine room alarm information, and fire door and fire alarm information.

### 4.5.1.2 References

IHO S-100 IHO Universal Hydrographic Data Model Edition 4.0.0 (December 2017).

ISO 8601. 2004. Data elements and interchange formats - Information interchange - Representation of dates and times. 2004.

ISO 3166-1. 1997. Country Codes. 1997.

ISO 10646-1 Information technology - Universal Multiple-Octet Coded Character Set (UCS) - Part 1: Architecture and Basic Multilingual Plane.

ISO 19101-2:2008 Geographic Information - Rules for Application Schema

ISO/TS 19103:2005 Geographic Information - Conceptual schema language

ISO 19106:2004 Geographic Information – Profiles

ISO 19107:2003 Geographic Information – Spatial schema

ISO 19109:2005 Geographic Information - Rules for Application Schema

ISO 19111:2003 Geographic Information - Spatial referencing by coordinates

ISO 19115:2003+Corr1 (2006) Geographic Information - Metadata

ISO 19115-2:2009 Geographic Information - Metadata: Extensions for imagery and gridded data

ISO 19123:2005 Geographic Information - Schema for coverage geometry and functions

ISO 19129:2009 Geographic Information - Imagery gridded and coverage data framework

ISO 19131:2007 Geographic Information - Data product specifications

ISO 19136:2007 Geographic Information – Geography Markup Language

ISO 19136-2:2015, Geographic Information – Geography Markup Language.

ISO/TS 19139, Geographic Information – Metadata – XML schema implementation.

### 4.5.1.3 Terms, definitions and abbreviations

Terms and Definitions

The S-100 framework is based on the ISO 19100 series of geographic standards. The terms and definitions provided here are used to standardize the nomenclature found within that framework, whenever possible. They are taken from the references cited in clause 2.1. Modifications have been made when necessary.

- Application

Manipulation and processing of data in support of user requirements (ISO 19101)

- Application schema

Conceptual schema for data required by one or more applications (ISO 19101)

- Conceptual model

Model that defines concepts of a universe of discourse (ISO 19101)

- Conceptual schema

Formal description of a conceptual model (ISO 19101)

- Coverage

Feature that acts as a function to return values from its range for any direct position within its spatial, temporal or spatiotemporal domain (ISO 19123)

- Data product

Dataset or dataset series that conforms to a data product specification

- Data product specification

Detailed description of a dataset or dataset series together with additional information that will enable it to be created, supplied to and used by another party

- Dataset

Identifiable collection of data (ISO 19115)

- Feature

Abstraction of real world phenomena (ISO 19101)

- Feature association

Relationship that links instances of one feature type with instances of the same or a different feature type (ISO19110)

- Feature attribute

Characteristic of a feature (ISO 19101)

- Geographic data

Data with implicit or explicit reference to a location relative to the Earth (ISO 19109)

- Metadata

Data about data (ISO 19115)

- Model

Abstraction of some aspects of reality (ISO 19109)

- Portrayal

Presentation of information to humans (ISO 19117)

- Quality

Totality of characteristics of a product that bear on its ability to satisfy stated and implied needs (ISO 19101)

Abbreviations

This product specification adopts the following convention for symbols and abbreviated terms:

- ASCII 	American Standard Code for Information Interchange

- ECDIS	Electronic Chart Display and Information Systems

- ECS	Electronic Chart Systems

- ENC	Electronic Navigational Chart

- GML	Geography Markup Language

- IHO	International Hydrographic Organization

- IOC	International Oceanographic Commission

- ISO	International Organization for Standardization

- UML	Unified Modelling Language

- URI	Uniformed Resource Identifier

- URL 	Uniform Resource Locator

- WMS	Web Map Service

- WFS	Web Feature Service

- www 	World Wide Web

- XML	Extensible Markup Language

- XSLT	eXtensible Stylesheet Language Transformations

### 4.5.1.4 General Data Product Description

Informal Description of Data Product

This clause contains general information about the data product

- Title:  	Ship Alarm & Emergency

- Abstract: 	 Ship Alarm & Emergency product specification describe the means to exchange emergencyguideline relevant information and alarm state of on-board equipment. This may include details on the navigation equipment alarm information, engine room alarm information, and fire door fire alarm information.

- Content:	Datasets conforming to this specification will contain all ship alarm state information and emergency guideline relevant information for the area of coverage, additionally, there will be relevant metadata quality, production authority, data sources and publication date.

- Spatial Extent:	Korean Coast

- Specific Purpose:	Describing alarm state information and emergency guideline relevant information for utilization of Ship-Borne System Monitoring service

### 4.5.1.5 Data product specification metadata

This information uniquely identifies this Product Specification and provides information about its creation and maintenance. For further information on dataset metadata see the metadata clause.

- Title: 	Ship Alarm & Emergency

- S-100 Version: 	4.0.0

- Ship Alarm & Emergency Version:	1.0.0

- Date: 	2019-03-08

- Language: 	English

- Classification: 	Unclassified

- Contact: 	32, Yuseong-daero 1312beon-gil, Yuseong-gu, Daejeon, Korea 32(Jang Dong) inside KRISO

- URL: 	http://www.smart-navigation.org

- Identifier: 	Ship Alarm & Emergency

- Maintenance: 	Amendments to this specification will be produced on a needs basis. For reporting issues with this specification which need correction, use the contact information.

### 4.5.2 Specification Scopes

This product specification describes one data product and therefore requires only one scope which is described below:

- Scope ID:  		Ship Alarm & Emergency datasets.

- Hierarchical level: 		MD_ScopeCode - 005

- Hierarchical level name: 	dataset.

- Level description:		information applies to the dataset

- Extent:	EX_Extent.description: Korean Coast

### 4.5.3 Dataset identification

This section describes how to identify data sets that conform to the specification. A dataset that conforms to this Product Specification may be identified by its discovery metadata as defined in clause 12 of this specification. The information identifying the data product may include the following items from S-100 4.0.0 clause 11-6 (adapted from ISO 19115).

| Title | Ship Alarm & Emergency |
| --- | --- |
| Abstract | Ship Alarm & Emergency dataset is a textTable dataset containing ship alarm state and emergency guideline information regarding Ship-Borne System Monitoring service the within a defined geographical area. |
| Acronym | SAE |
| Content | Ship Emergency information, Wind Wave information, Equipment Alarm, Tank information |
| Topic category | Structure |
| geographicDescription | Area specific to marine navigation within korean sea area |
| Purpose | Standardization of on-board equipment alarm state and emergency guideline relevant information |
| Language | EN Additional values if any use CharacterString values from ISO 639-2 |

### 4.5.4 Data Content and structure

### 4.5.4.1 Introduction

The Ship Alarm & Emergency product is based on the S-100 General Feature Model (GFM), and is a feature-based vector product. All Ship Alarm & Emergency features and information classes are derived from one of the abstract classes FeatureType and InformationType defined in the Ship Alarm & Emergency respectively.

This clause contains the Application Schema expressed in UML and an associated Feature Catalogue.

### 4.5.4.2 Application Schema

The UML models in this clause are segments of the overall Ship Alarm & Emergency application schema, and include overviews of the feature classes, information classes, meta features, spatial types, and the relationships between them.

This clause contains a general overview of the classes and relationships in the Ship Alarm & Emergency application schema. Detailed information about how to use the feature types and information types to encode Ship Alarm & Emergency information is provided in the SMART-Navigation Data Classification and Encoding Guide (DCEG).

The following conventions are used in the UML diagrams depicting the application schema:

- Standard UML conventions for classes, associations, inheritance, roles, and multiplicities apply. These conventions are described in Part 1 of S-100.

- Italic font for a class name indicates an abstract class.

- Feature classes are depicted with pink background; the dark shade for abstract feature classes and the light shade for ordinary (non-abstract) feature classes.

- Complex attributes are depicted with a light yellow background.

- Enumeration lists and codelists are depicted with a green background. The numeric code corresponding to each listed value is shown to its right following an ‘=’ sign.

- No significance attaches to the colour of associations. (Complex diagrams may use different colours to distinguish associations that cross one another.)

- Subclasses inherit the attributes and associations of their superclasses at all levels, unless such inheritance is explicitly overridden in the subclass.

### 4.5.4.2.1 Domain model

Figure 12 Ship Alarm information Overview

### 4.5.4.3 Feature Catalogue

### 4.5.4.3.1 Introduction

The Feature Catalogue describes the feature types, information types, attributes, attribute values, associations and roles which may be used in the product.  The Ship Alarm & Emergency Feature Catalogue is available in an XML document which conforms to the S-100 XML Feature Catalogue Schema.  Simple attributes used in this specification are listed in below.

Name: 	Ship Alarm & Emergency Feature Catalogue

Scope: 	Smart-Navigation

Version Number: 	1.0.0

Version Date:	2019-03-15

Producer:	SMART Navigation Project

Language: 	                  English

### 4.5.4.3.1.1 Feature Types

Feature types contain descriptive attributes that characterize real-world entities. The word ‘feature’ may be used in one of two senses – feature type and feature instance. A feature type is a class and is defined in a Feature Catalogue. A feature instance is a single occurrence of the feature type and represented as an object in a dataset. A feature instance is located by a relationship to one or more spatial instances. A feature instance may exist without referencing a spatial instance.

### 4.5.4.3.1.2 Geographic

A Geographic (Geo) feature type carries the descriptive characteristics of a real-world entity.

### 4.5.4.3.1.3 Meta

Meta features contain information about other features within a dataset. Information defined by meta features override the default metadata values defined by the dataset descriptive records. Meta attribution on individual features overrides attribution on meta features.

### 4.5.4.3.1.4 Feature Relationship

A feature relationship links instances of one feature type with instances of the same or a different feature type.

### 4.5.4.3.1.5 Information Types

Information types are identifiable pieces of information in a dataset that can be shared between other features. They have attributes but have no relationship to any geometry; information types may reference other information types.

### 4.5.4.3.1.6 Attributes

SMART-Navigation product defines attributes as either simple or complex.

### 4.5.4.3.1.6.1 Simple Attributes

SMART-Navigation product uses nine types of simple attributes; they are listed in the following table:

| Type | Definition |
| --- | --- |
| Enumeration | A fixed list of valid identifiers of named literal values |
| Boolean | A value representing binary logic. The value can be either True or False. The default state for Boolean type attributes (i.e. where the attribute is not populated for the feature) is False. |
| Real | A signed Real (floating point) number consisting of a mantissa and an exponent |
| Integer | A signed integer number. The representation of an integer is encapsulation and usage dependent. |
| Text | An arbitrary-length sequence of characters including accents and special characters from a repertoire of one of the adopted character sets |
| Date | A date provides values for year, month and day according to the Gregorian Calendar. Character encoding of a date is a string which must follow the calendar date format (complete representation, basic format) for date specified by ISO 8601:1988. EXAMPLE 19980918 (YYYY-MM-DD) |
| Time | A time is given by an hour, minute and second. Character encoding of a time is a string that follows the local time (complete representation, basic format) format defined in ISO 8601:1988. EXAMPLE 183059 or 183059+0100 or 183059Z |
| Date and Time | A DateTime is a combination of a date and a time type. Character encoding of a DateTime shall follow ISO 8601:1988 EXAMPLE 19850412T101530 |
| Codelist | A type of flexible enumeration. A code list type is a list of literals which may be extended only in conformance with specified rules. |

### 4.5.4.3.1.6.2 Complex Attributes

Complex attributes are aggregations of other attributes that are either simple or complex. The aggregation is defined by means of attribute bindings.

### 4.5.4.3.1.6.3 Geometric Representation

Geometric representation is the digital description of the spatial component of an object as described in S-100 and ISO 19107. This product specification uses no geometry attribute.

### 4.5.4.3.2 Feature Catalogue General

The Ship Alarm & Emergency feature catalogue describes the feature types, information types, attributes, attribute values, associations and roles which may be used in a Ship Alarm & Emergency data set.

### 4.5.4.3.2.1 Feature Types

- Geographic

- Meta

- Ship

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| shipMRN | MRN is a naming scheme that can uniquely identify any maritime resource on a global scale. shipMRN is the unique value given to the ship. | URN | 1 | mandatory |  |
| vesselId | It is an identifier to recognize the ship, and it is the value that is given to the ship only in SMART-Nav SV20. | text | 0..1 | option |  |
| generatorCount | Number of generators installed on the ship. | integer | 0..1 | option |  |
| steeringGearCount | Number of Steering Gears installed on the ship. | integer | 0..1 | option |  |
| fireDoorCount | Number of Fire Doors installed on the ship. | integer | 0..1 | option |  |
| rampDoorCount | Number of Ramp Door Installed on ship | integer | 0..1 | option |  |
| telegraphCount | Number of telegraphs installed on the ship. | integer | 0..1 | option |  |
| fireAlarmCount | Number of fire alarms installed on the ship. | integer | 0..1 | option |  |
| oldTypeFireAlarmCount | Number of old fire alarms installed on the ship. | integer | 0..1 | option |  |
| measurementDate | Date & Time at which ship alarm was measure. | dateTime | 1 | mandatory |  |
| useEquipmentInformation | - | complex attribute | 0..* | option |  |

- ShipEmergencyInformation

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| emergencyLevelAndGuidanceGenerationTimes | Emergency Level and Guidance Generation Times | dateTime | 1 | mandatory |  |
| emergencyGuidance | - | complex Attribute | 1..* | mandatory |  |

- WindWaveInformation

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| useWind | Installation of wind direction anemometer. | boolean | 1 | mandatory |  |
| azimuthDegreesOfWindDirection | The wind direction in degrees, from 000 to 360, from which the wind blows | integer | 0..1 | option |  |
| valueOfWindSpeed | The ratio value of the distance covered by the air to the time taken to cover it. | real | 0..1 | option |  |
| waveHeight | The average height of wind waves, significant waves, or swell waves. | integer | 0..1 | option |  |
| waveLengthValue | The distance between two successive peaks (or other points of identicalphase) on an electromagnetic wave. | real | 0..1 | option |  |
| wavePeriod | - | real | 0..1 | option |  |

- EquipmentAlarm

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| equipmentInstallation | Whether the equipment or sensor is installed. | boolean | 1 | mandatory |  |
| alarmStatus | - | boolean | 0..1 | option |  |
| equipmentId | ID of the equipment or sensor installed on the ship. | text | 0..1 | option |  |
| positionId | The location where the equipment or sensor is installed. | enumeration | 0..1 | option |  |
| navigationEquipmentType | Types of navigation equipment | enumeration | 0..1 | option |  |
| generatorAlarmType | Generator Alarm Type | enumeration | 0..1 | option |  |
| mainEngineAlarmType | Types of alarm devices installed on the main engine. | enumeration | 0..1 | option |  |
| steeringGearAlarmType | Types of steering gear alarm devices | enumeration | 0..1 | option |  |
| telegraphStatus | - | enumeration | 0..1 | option |  |
| typesOfFireAlarms | - | enumeration | 0..1 | option |  |
| doorType | Attribute to classify fire door and ramp door installed on ship. | enumeration | 0..1 | option |  |
| alarmTime | The time when a fire occurred and an alarm occurred. | dateTime | 0..1 | option |  |

- TankInformation

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| tankType | Types of tanks installed on ships | enumeration | 1 | mandatory |  |
| tankCapacity | The total volume of the tank or the total amount that can be put into the tank. | real | 0..1 | option |  |
| tankTotalCount | Total number of ships installed by tank type | integer | 1 | mandatory |  |
| tankDetail | Specifications and loading status information for individual tanks | complex attribute | 0..* | option |  |

- Cartographic

- Theme

### 4.5.4.3.2.2 Feature Relationship

| Role Type | Association Name | Role | Feature | Multiplicity |
| --- | --- | --- | --- | --- |
| Association | emergencyGuidance | cause | ShipEmergencyInformation | 1 |
| Association | emergencyGuidance | effect | ShipAlarm | 1 |
| composition | shipEquipmentAlarm | componentOf | Ship | 1 |
| composition | shipEquipmentAlarm | consistOf | WindWaveInformation | 0..1 |
| composition | shipEquipmentAlarm | componentOf | Ship | 1 |
| composition | shipEquipmentAlarm | consistOf | EquipmentAlarm | 0..* |
| composition | shipEquipmentAlarm | componentOf | Ship | 1 |
| composition | shipEquipmentAlarm | consistOf | TankInformation | 0..* |

### 4.5.4.3.2.3 Information Relationship

- N/A

### 4.5.4.3.2.4 Information Type

- N/A

### 4.5.4.3.2.5 Simple Attribute

- Simple Attribute List

| Name | Description | Data Type | Remarks |
| --- | --- | --- | --- |
| shipMRN | MRN is a naming scheme that can uniquely identify any maritime resource on a global scale. shipMRN is the unique value given to the ship. | URN |  |
| vesselId | It is an identifier to recognize the ship, and it is the value that is given to the ship only in SMART-Nav SV20. | text |  |
| generatorCount | Number of generators installed on the ship. | integer |  |
| steeringGearCount | Number of Steering Gears installed on the ship. | integer |  |
| fireDoorCount | Number of Fire Doors installed on the ship. | integer |  |
| rampDoorCount | Number of Ramp Door Installed on ship | integer |  |
| telegraphCount | Number of telegraphs installed on the ship. | integer |  |
| fireAlarmCount | Number of fire alarms installed on the ship. | integer |  |
| oldTypeFireAlarmCount | Number of old fire alarms installed on the ship. | integer |  |
| measurementDate | Date & Time at which ship alarm was measure. | dateTime |  |
| emergencyLevelAndGuidance GenerationTimes | Emergency Level and Guidance Generation Times | dateTime |  |
| useWind | Installation of wind direction anemometer. | boolean |  |
| azimuthDegreesOfWindDirection | The wind direction in degrees, from 000 to 360, from which the wind blows | integer |  |
| valueOfWindSpeed | The ratio value of the distance covered by the air to the time taken to cover it. | real |  |
| waveHeight | The average height of wind waves, significant waves, or swell waves. | integer |  |
| waveLengthValue | The distance between two successive peaks (or other points of identicalphase) on an electromagnetic wave. | real |  |
| wavePeriod | - | real |  |
| equipmentInstallation | Whether the equipment or sensor is installed. | boolean |  |
| alarmStatus | - | boolean |  |
| equipmentId | ID of the equipment or sensor installed on the ship. | text |  |
| positionId | The location where the equipment or sensor is installed. | enumeration |  |
| navigationEquipmentType | Types of navigation equipment | enumeration |  |
| generatorAlarmType | Generator Alarm Type | enumeration |  |
| mainEngineAlarmType | Types of alarm devices installed on the main engine. | enumeration |  |
| steeringGearAlarmType | Types of steering gear alarm devices | enumeration |  |
| telegraphStatus | - | enumeration |  |
| typesOfFireAlarms | - | enumeration |  |
| doorType | Attribute to classify fire door and ramp door installed on ship. | enumeration |  |
| alarmTime | The time when a fire occurred and an alarm occurred. | dateTime |  |
| tankType | Types of tanks installed on ships | enumeration |  |
| tankCapacity | The total volume of the tank or the total amount that can be put into the tank. | real |  |
| tankTotalCount | Total number of ships installed by tank type | integer |  |
| emergencyAlarmType | Types of risk situations | enumeration |  |
| emergencyLevel | Level by type of risk. | enumeration |  |
| guidanceCode | Provided as a code value as a guideline on the ship in case of danger. | text |  |
| levelAlarmType | - | enumeration |  |
| alarmStatus | - | enumeration |  |
| tankID | ID for identifying individual tanks | text |  |
| tankMoldedHeight | Maximum height of individual tank(molded height) | real |  |
| tankLevelGauge | A gauge sensor for continuously measuring the amount of oil / water stored in the tank | real |  |
| typeOfEquipmentInformation | - | enumeration |  |
| idOfEquipmentInformation | - | text |  |
| useInformation | - | boolean |  |

- navigationEquipmentType

| Value | Label | Description |
| --- | --- | --- |
| 1 | speedLog | - |
| 2 | ais | - |
| 3 | main gps | - |
| 4 | secondary gps | - |
| 5 | ecdis | - |
| 6 | main radar | - |
| 7 | secondary radar | - |
| 8 | echo sounder | - |
| 9 | gyro | - |
| 10 | autopilot | - |

- mainEngineAlarmType

| Value | Label | Description |
| --- | --- | --- |
| 1 | shut down | - |
| 2 | slow down | - |

- generatorAlarmType

| Value | Label | Description |
| --- | --- | --- |
| 1 | jacket cooling fresh water high temp trip | - |
| 2 | system lubricating oil low press trip | - |
| 3 | over speed trip | - |
| 4 | common trip | - |
| 5 | speed sensor fail trip | - |
| 6 | start fail | - |
| 7 | common shutdown | - |

- steeringGearAlarmType

| Value | Label | Description |
| --- | --- | --- |
| 1 | steeringGearAlarmType | - |
| 2 | hydro lubricating oil tank Low Level | - |
| 3 | hyd locak alarm | - |
| 4 | motor run | -- |
| 5 | no voltage | - |
| 6 | overload | - |
| 7 | phase fail | - |

- doorType

| Value | Label | Description |
| --- | --- | --- |
| 1 | fire door | - |
| 2 | Ramp door | - |

- fireAlarmType

| Value | Label | Description |
| --- | --- | --- |
| 1 | smoke | - |
| 2 | heat | - |
| 3 | flame | - |
| 4 | old type fire alarm | - |

- emergencyAlarmType

| Value | Label | Description |
| --- | --- | --- |
| 1 | fire emergency | - |
| 2 | seakeeping emergency | - |
| 3 | navigation emergency | - |

- emergencyLevel

| Value | Label | Description |
| --- | --- | --- |
| 1 | normal | - |
| 2 | attention | - |
| 3 | warning | - |
| 4 | alert | - |
| 5 | critical | - |

- position

| Value | Label | Description |
| --- | --- | --- |
| 1 | starboard | - |
| 2 | portside | - |
| 3 | forward | - |
| 4 | after |  |
| 5 | c-deck |  |
| 6 | d-deck |  |
| 7 | after-mid |  |

- telegraphStatus

| Value | Label | Description |
| --- | --- | --- |
| 1 | no information | - |
| 2 | ahead navigation full | - |
| 3 | ahead full | - |
| 4 | ahead half |  |
| 5 | ahead slow |  |
| 6 | ahead dead slow |  |
| 7 | astern full |  |
| 8 | astern half |  |
| 9 | astern slow |  |
| 10 | astern dead slow |  |

- tankType

| Value | Label | Description |
| --- | --- | --- |
| 1 | cargo tank | - |
| 2 | slop tank | - |
| 3 | heavy fuel oil tank | - |
| 4 | marine diesel oil tank |  |
| 5 | lubricating oil tank |  |
| 6 | fresh water tank |  |
| 7 | ballast water tank |  |

- levelAlarmType

| Value | Label | Description |
| --- | --- | --- |
| 1 | high level alarm sensor | A sensor that gives an alarm when the tank is loaded with more than 95% liquid. |
| 2 | overfill alarm sensor | A sensor that gives an alarm when the tank is loaded with more than 98% liquid. |

- typeOfEquipmentInformation

| Value | Label | Description |
| --- | --- | --- |
| 1 | engine | - |
| 2 | rudder | - |
| 3 | gps | - |
| 4 | roll |  |
| 5 | pitch |  |
| 6 | fire alarm |  |

### 4.5.4.3.2.6 Complex Attribute

- emergencyGuidance

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| emergencyAlarmType | Types of risk situations | enumeration | 1 | mandatory |  |
| emergencyLevel | Level by type of risk. | enumeration | 1 | mandatory |  |
| guidanceCode | Provided as a code value as a guideline on the ship in case of danger. | text | 1 | mandatory |  |

- levelAlertSystem

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| levelAlarmType | - | enumeration | 0..1 | option |  |
| alarmStatus | - | enumeration | 0..1 | option |  |

- tankDetail

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| tankID | ID for identifying individual tanks | text | 0..1 | option |  |
| tankMoldedHeight | Maximum height of individual tank(molded height) | real | 0..1 | option |  |
| tankLevelGauge | A gauge sensor for continuously measuring the amount of oil / water stored in the tank | real | 0..1 | option |  |
| levelAlertSystem | Alarm system to check dangerous level of tank installed on vessel | complex attribute | 0..1 | option |  |

- useEquipmentInformation

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| typeOfEquipmentInformation | - | enumeration | 1 | mandatory |  |
| idOfEquipmentInformation | - | text | 0..1 | option |  |
| useInformation | - | boolean | 0..1 | option |  |

### 4.5.5 Coordinate Reference System(CRS)

.

### 4.5.5.5 Temporal Reference System

N/A (This model has no geometry attribute)

### 4.5.6 Data Quality

### 4.5.6.1 Introduction

Ship Alarm & Emergency products must be tested with the Ship Alarm & Emergency specific checks prior to release by the data producer. The data producer must review the check results and address any issues to ensure sufficient quality of the data products. The checks are a mix of data format validation checks, conformance to standard checks and logical consistency checks. But the checklist has not been written yet. It will be updated in the next version.

### 4.5.7 Data Capture and classification

See DCEG for this section.

### 4.5.8 maintenance

### 4.5.8.1 Introduction

Datasets are maintained as needed and must include mechanisms for Ship Alarm & Emergency updating. Data updates will be made by new editions. The maintenance and update frequency of Ship Alarm & Emergency datasets should be defined by the producers (official national authority) implementing this specification.

Data Producers must use applicable sources to maintain and update data and provide a brief description of the sources that were used to produce the dataset in the appropriate metadata field.

The data product shall provide information on how the data is maintained and should describe the principles and criteria applied in maintenance regime. This should specify the expected frequency of updates.

Table 18 Maintenance and update frequency

| Item Name | Description | Multiplicity | Type |
| --- | --- | --- | --- |
| maintenanceAndUpdateFrequency | Frequency with which changes and additions are made to the data product (per update scope) | 1..* | MD_MaintenanceInformation (ISO 19115) |
| dataSource | Identification of the kinds of data sources usable to produce datasets | 1..* | LI_Source (ISO 19115) |
| productionProcess | Textual description of the production process applicable to the datasets (per scope or data source) | 1..* | LI_ProcessStep (ISO 19115) |

### 4.5.8.2 Production process for base and update datasets

Data Producers should follow their established production processes for maintaining and updating datasets. Data is produced against the DCEG and checked against the appropriate set of validation rules in Appendix E. (Appendix E is not included in this release.)

### 4.5.8.3 Dataset updates

N/A (Dataset update is not relevant in this model)

### 4.5.8.4 Support file updates

N/A (Support file is not included in this model)

### 4.5.8.5 Feature and portrayal catalogues

For each new version of the Ship Alarm & Emergency Product Specification a new feature

### 4.5.8.6 Feature history, versions, and change tracking

If applications or production systems require versioning of individual instances of feature or information types, maintenance of histories, or change tracking, the methods for versioning, history management, and change tracking and display are left to the application or production system.

### 4.5.8.7 Dataset encryption

Details about dataset encryption are still to be determined, and may mirror the method described in S-101.

### 4.5.9 Portrayal

Portrayal is not defined in this version of Ship Alarm & Emergency Product Specifications. Users are free to choose the means and methodology of portrayal as they see best suited for their needs. It should be noted that future versions of Ship Alarm & Emergency may include a portrayal catalogue, and any implementer should therefore anticipate this, and make sufficient provisions in any system supporting Ship Alarm & Emergency.

### 4.5.10 data product format(encoding)

### 4.5.10.1 Introduction

The GML encoding of Ship Alarm & Emergency datasets is based on the S-100 profile of GML 3.2.1.

Format Name: Ship Alarm & Emergency GML

Version:	1.0

Character Set: UTF-8

Specification: SMART-Navigation Product Specification

### 4.5.10.2 Encoding of Latitude and Longitude

N/A (This model has no geometry attribute)

### 4.5.10.3 Text Attribute

Character strings must be encoded using the character set defined in ISO 10646-1, in Unicode Transformation Format-8 (UTF-8).

### 4.5.10.4 Mandatory Attribute Values

There are four reasons why attribute values may be considered mandatory:

- They determine whether a feature is in the display base,

- Certain features make no logical sense without specific attributes,

- Some attributes are necessary to determine which symbol is to be displayed,

- Some attributes are required for safety of navigation.

All mandatory attributes are identified in the Feature Catalogue.

### 4.5.10.5 Unknown Attribute Values

When a mandatory attribute code or tag is present but the attribute value is missing, it means that the producer wishes to indicate that this attribute value is unknown.  Missing mandatory attributes must be “nilled” with a GML nilReason attribute giving the reason for omission.

Optional attributes must be omitted altogether if the value is unknown or missing. They must not be “nilled.”

### 4.5.10.6 Structure of dataset files

### 4.5.10.6.1 Sequence of dataset files

The order of data objects in each dataset file is described below:

Dataset Identification Information

Dataset structure information

Spatial records for by-reference geometries

Point

Multi point

Curve

Composite Curve

Surface

Information objects

Feature objects (Geometry may be encoded inline or by reference.)

Meta features

Geo features

### 4.5.10.7 Numeric Attribute Encoding

Floating point and integer attribute values must not contain leading zeros.  Floating point attribute values must not contain non-significant trailing zeros.

### 4.5.10.8 Object identifiers

The “name” of feature records must provide a unique world-wide identifier of feature records. The “name” of the record is the combination of the subfields agency, featureObjectIdentifier, and featureIdentificationSubdivision elements of the featureObjectIdentifier element of the object.

Features, information types, collection objects, meta features, and geometries (inline or external) are all required by the schema to have a gml:id attribute with a value that is unique within the dataset. The gml:id values must be used as the reference for the object from another object in the same dataset or another dataset.

### 4.5.10.9 Dataset validation

Fields may be repeated or omitted as permitted by the XML schemas and the validation tests. Since XML schema cannot encode rules for conditional presence or attributes, these rules are checked by Schematron rules or other validation code.

### 4.5.10.10 Location of Data Product Format schema Files

The schema file will provide a future location and is available from that location.

### 4.5.10.11 Detailed documentation of Ship Alarm & Emergency XML Schema

See DCEG for this section.

### 4.5.11 data product delivery

This clause specifies the encoding and delivery mechanisms for a Ship Alarm & Emergency dataset.  Data which conforms to this product specification must be delivered by means of an exchange set.

### 4.5.11.1 Exchange set

Ship Alarm & Emergency datasets are grouped into exchange sets. Each exchange set consists of one or more Ship Alarm & Emergency datasets with an associated XML metadata file and a single Exchange Catalogue XML file containing metadata. It may also include one or more support files.

Units of Delivery:				Exchange Set

Transfer Size:					Unlimited

Medium Name:					Digital data delivery

Other Delivery Information:

Each dataset must be contained in a physically separate, uniquely identified file on the transfer medium.

Each exchange set has a single exchange catalogue which contains the discovery metadata for each dataset and references to any support files.

Support files are supplementary information which are linked to the features and information types by attributes. The attributes containing these links are described in the application schema and feature catalogue.

An exchange set may be encapsulated into a form suitable for transmission by a mapping called a transmission encoding. An encoding translates each of the elements of the exchange set into a logical form suitable for writing to media and for transmission online. An encoding may also define other elements in addition to the exchange set contents (i.e., media identification, data extents etc…) and also may define commercial constructs such as encryption and compression methods.

If the data is transformed (e.g., for encryption or compression purposes) its content must not be changed.

This product specification does not define the transmission encoding which must be used as a default for transmission of data between parties.

The exchange set elements are as follows:

Mandatory Elements

- Ship Alarm & Emergency datasets – GML encoding of features/attributes and their associated geometry and metadata.

- Exchange Catalogue – the XML encoded representation of exchange set catalogue features [discovery metadata].

Optional Elements

- Supplementary files – These are contained within the exchange set as files and the map from the name included within the dataset and the physical location on the media is defined within the Exchange Catalogue.

- Feature Catalogue – If it is necessary to deliver the latest feature catalogue to the end user it may be done using the Ship Alarm & Emergency exchange set mechanism for datasets – i.e., include the updated feature catalogue in an exchange set.

### 4.5.11.2 Dataset

### 4.5.11.2.1 Datasets

Datasets are distributed as files which are part of exchange sets structured as described in this specification. The distribution media are left to the discretion of the producer and distributor.

The following types of dataset files may be produced and contained within an exchange set:

- New dataset and new edition of a dataset (base dataset):  Each new edition of a dataset must have the same name as the dataset that it replaces.  A new edition can also be AtoN data that has previously been produced for this area.

### 4.5.11.2.2 Dataset size

Datasets must not exceed 1 MB.

### 4.5.11.2.3 Dataset file naming

Dataset files shall be named

CCXXXXXXXX_EEE.GML

The file name forms a unique identifier where:

- CC - the first two characters identify the issuing agency (mandatory).

- The third to tenth characters are optional and may be used in any way by the producer to provide theunique file name. The following characters are allowed in the dataset name, A to Z, 0 to 9 and the special character _ (underscore).

- EEE – new editions and re-issues use 000, and increment until a limit of 999 (mandatory).

- The maximum number of characters preceding _EEE is ten.

- GML – the character sequence “GML” or “gml”.

### 4.5.11.2.4 New Editions and re-issues

This section defines the sequencing of datasets for New Editions. In order to ensure that feature type updates are incorporated into an SYSTEM in the correct sequence without any omission, a number of parameters encoded in the data header and/or metadata are used in the following way:

| edition number | when a dataset is initially created, the edition number 1 is assigned to it. The edition number is increased by 1 at each new edition. |
| --- | --- |

### 4.5.11.2.5 Cancellations

Dataset cancellation is not considered I this product specification. In the SMART Navigation project, the system automatically cancels the dataset after 7 days from the date the dataset is received.

### 4.5.11.3 Support Files

N/A (Support file is not included in this model)

### 4.5.11.4 Exchange Catalogue

N/A (Exchange Catalogue is not used in SMART Navigation)

### 4.5.12 meta data

### 4.5.12.1 Introduction

The Ship Alarm & Emergency metadata description is based on the S-100 metadata document section, which is a profile of the ISO 19115 standard. These documents provide a structure for describing digital geographic data and define metadata elements, a common set of metadata terminology, definitions and extension procedures.

Two metadata packages are described in this product specification: dataset metadata and exchange set metadata.

Figure 13 Metadata packages

Note 1: Types with CI_, EX_, and MD_ prefixes are from packages defined in ISO 19115 and adapted by S-100. Types with S100_ prefix are from packages defined in S-100.

Note 2: When a dataset is terminated, the purpose metadata field is set to 3 (terminated), and the editionNumber metadata field is set to 0. All other metadata fields must be blank.

### 4.5.12.2 Dataset metadata

Dataset metadata is intended to describe information about a dataset. It facilitates the management and exploitation of data and is an important requirement for understanding the characteristics of a dataset. Whereas dataset metadata is usually fairly comprehensive, there is also a requirement for a constrained subset of metadata elements that are usually required for discovery purposes. Discovery metadata are often used for building web catalogues, and can help users determine whether a product or service is fit for purpose and where they can be obtained.

Table 19 Dataset metadata

| Name | Cardinality | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| S100_DataSetDiscoveryMetadata |  |  |  |  |
| fileName | 1 |  | CharacterString | Dataset file name |
| filePath | 1 |  | CharacterString | Full path from the exchange set root directory |
| description | 1 |  | CharacterString | Short description of the area covered by dataset, e.g., area, harbour, or port name, between two named locations etc. |
| dataProtection | 0..1 | {1} or {2} | Boolean | True : Encrypted False : Unencrypted |
| protectionScheme | 0..1 |  | CharacterString |  |
| digitalSignature | 0..1 |  | CharacterString |  |
| digitalSignatureReference | 0..1 |  | CharacterString |  |
| digitalSignatureValue | 0..1 |  | CharacterString |  |
| copyright | 0..1 |  | MD_LegalConstraints>MD_RestrictionCode <copyright> (ISO 19115-1) |  |
| classification | 0..1 |  | Class MD_SecurityConstraints>MD_ClassificationCode (codelist) ISO 19115-1 | 1. unclassified 2. restricted 3. confidential 4. secret 5. top secret |
| purpose | 1 | {1}, {2} | CharacterString | - New dataset - New edition |
| specificUsage | 1 |  | MD_USAGE>specificUsage (character string) MD_USAGE>userContactInfo (CI_Responsibility) | brief description of the resource and/or resource series usage |
| editionNumber | 1 | {1} | Integer | When a dataset is initially created, the edition number “1” is assigned to it. The edition number is increased by one with each new edition. |
| updateNumber | 1 |  | CharacterString | Update number 0 is assigned to a new dataset. |
| updateApplicationDate | 1 |  | Date | All updates dated on or before this date must have been applied by the producer. |
| issueDate | 1 |  | Date | Date on which the dataset was generated. |
| productSpecification | 1 |  | S100_ProductSpecification | See Notes below this table for constraints on values. |
| producingAgency | 1 |  | CI_Responsibility>CI_Organisation or CI_Responsibility>CI_Individual | Party responsible for generating the dataset. |
| optimumDisplayScale | 0..1 |  | Integer |  |
| maximumDisplayScale | 0..1 |  | Integer |  |
| minimumDisplayScale | 0..1 |  | Integer |  |
| horizontalDatumReference | 1 | EPSG | CharacterString |  |
| horizontalDatumValue | 1 | 4326 | Integer | WGS84 |
| verticalDatum | 0..1 |  | S100_VerticalAndSoundingDatum | Not included in Ship Alarm & Emergency |
| soundingDatum | 0..1 |  | S100_VerticalAndSoundingDatum | Not included in Ship Alarm & Emergency |
| dataType | 1 | GML | CharacterString | The only value allowed is “GML”. |
| otherDataTypeDescription | 0..1 |  | CharacterString |  |
| dataTypeVersion | 1 | 3.2.1 | CharacterString |  |
| dataCoverage | 1..* |  | S100_DataCoverage |  |
| comment | 0..1 |  | CharacterString | Any additional Information |
| layerID | 1..* | S-101 | CharacterString | Dataset must be used with ENC in an ECDIS. |

### 4.5.12.3 Update Dataset Metadata

N/A (Dataset update is not relevant in this model)

### 4.5.12.4 Support file metadata

N/A (Support file is not included in this model)

### 4.5.12.5 Exchange set metadata

N/A (Exchange Catalogue is not used in SMART Navigation)

### 4.5.12.6 Catalogue file Metadata

All Ship Alarm & Emergency Exchange Catalogue files must contain at least the mandatory metadata elements.

Table 20 Catalogue file metadata

| Name | Multiplicity | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| S100_ExchangeCatalogue |  |  |  |  |
| identifier | 1 |  | S100_CatalogueIdentifier |  |
| contact | 1 |  | S100_CataloguePointOfContact |  |
| productSpecification | 0..1 |  | S100_ProductSpecification | Conditional on all the datasets using the same product specification.. |
| metadataLanguage | 1 | English | CharacterString | All datasets conforming to this PS must use English language. A catalogue in English must be provided. Discovery metadata elements within catalogues have their own locale attributes and may be repeated in languages other than English. |
| exchangeCatalogueName | 1 |  | CharacterString | Catalogue filename |
| exchangeCatalogueDescription | 1 |  | CharacterString |  |
| exchangeCatalogueComment | 0..1 |  | CharacterString | Any additional Information |
| compressionFlag | 0..1 |  | Boolean | Yes or No |
| sourceMedia | 0..1 |  | CharacterString |  |
| replacedData | 0..1 |  | Boolean |  |
| dataReplacement | 0..1 |  | CharacterString |  |

## 4.6 ENC Service meta

### 4.6.1 Overview

### 4.6.1.1 Introduction

This document has been produced by the member of SMART-Navigation project team(Korean Register, Marine-Works) in response to a requirement to produce a data product that can be used as a request/response message to exchange ENC. It is based on the IHO S-100 framework specification and the ISO 19100 series of standards. It is a textTable product specification that is primarily intended for encoding the download information, ENC Streaming address, zone information, device property

### 4.6.1.2 References

IHO S-100 IHO Universal Hydrographic Data Model Edition 4.0.0 (December 2017).

ISO 8601. 2004. Data elements and interchange formats - Information interchange - Representation of dates and times. 2004.

ISO 3166-1. 1997. Country Codes. 1997.

ISO 10646-1 Information technology - Universal Multiple-Octet Coded Character Set (UCS) - Part 1: Architecture and Basic Multilingual Plane

ISO 19101-2:2008 Geographic Information - Rules for Application Schema

ISO/TS 19103:2005 Geographic Information - Conceptual schema language

ISO 19106:2004 Geographic Information – Profiles

ISO 19107:2003 Geographic Information – Spatial schema

ISO 19109:2005 Geographic Information - Rules for Application Schema

ISO 19111:2003 Geographic Information - Spatial referencing by coordinates

ISO 19115:2003+Corr1 (2006) Geographic Information - Metadata

ISO 19115-2:2009 Geographic Information - Metadata: Extensions for imagery and gridded data

ISO 19123:2005 Geographic Information - Schema for coverage geometry and functions

ISO 19129:2009 Geographic Information - Imagery gridded and coverage data framework

ISO 19131:2007 Geographic Information - Data product specifications

ISO 19136:2007 Geographic Information – Geography Markup Language

ISO 19136-2:2015, Geographic Information – Geography Markup Language.

ISO/TS 19139, Geographic Information – Metadata – XML schema implementation.

### 4.6.1.3 Terms, definitions and abbreviations

Terms and Definitions

The S-100 framework is based on the ISO 19100 series of geographic standards. The terms and definitions provided here are used to standardize the nomenclature found within that framework, whenever possible. They are taken from the references cited in clause 2.1. Modifications have been made when necessary.

- Application

Manipulation and processing of data in support of user requirements (ISO 19101)

- Application schema

Conceptual schema for data required by one or more applications (ISO 19101)

- Conceptual model

Model that defines concepts of a universe of discourse (ISO 19101)

- Conceptual schema

Formal description of a conceptual model (ISO 19101)

- Coverage

Feature that acts as a function to return values from its range for any direct position within its spatial, temporal or spatiotemporal domain (ISO 19123)

- Data product

Dataset or dataset series that conforms to a data product specification

- Data product specification

Detailed description of a dataset or dataset series together with additional information that will enable it to be created, supplied to and used by another party

- Dataset

Identifiable collection of data (ISO 19115)

- Feature

Abstraction of real world phenomena (ISO 19101)

- Feature association

Relationship that links instances of one feature type with instances of the same or a different feature type (ISO19110)

- Feature attribute

Characteristic of a feature (ISO 19101)

- Geographic data

Data with implicit or explicit reference to a location relative to the Earth (ISO 19109)

- Metadata

Data about data (ISO 19115)

- Model

Abstraction of some aspects of reality (ISO 19109)

- Portrayal

Presentation of information to humans (ISO 19117)

- Quality

Totality of characteristics of a product that bear on its ability to satisfy stated and implied needs (ISO 19101)

Abbreviations

This product specification adopts the following convention for symbols and abbreviated terms:

- ASCII 	American Standard Code for Information Interchange

- ECDIS	Electronic Chart Display and Information Systems

- ECS	Electronic Chart Systems

- ENC	Electronic Navigational Chart

- GML	Geography Markup Language

- IHO	International Hydrographic Organization

- IOC	International Oceanographic Commission

- ISO	International Organization for Standardization

- UML	Unified Modelling Language

- URI	Uniformed Resource Identifier

- URL 	Uniform Resource Locator

- WMS	Web Map Service

- WFS	Web Feature Service

- www 	World Wide Web

- XML	Extensible Markup Language

- XSLT	eXtensible Stylesheet Language Transformations

### 4.6.1.4 General Data Product Description

Informal Description of Data Product

This clause contains general information about the data product

- Title:  	ENC Service Meta Product Specification.

- Abstract: 	 ENC Service Meta product specification provides the means to exchange ENC file for Real-time Electronic Navigational Chart Distribution & Streaming Service in the maritime domain such as ENC service request, ENC download, ENC streaming, zone information. This may include details on category of ENC, download information, zone property, device property, type of ENC.

- Content:	Datasets conforming to this specification will contain all message meta information to exchange ENC file for Real-time Electronic Navigational Chart Distribution & Streaming Service. Additionally, there will be relevant metadata data quality, production authority, data sources and publication date.

- Spatial Extent:	Korean Coast

- Specific Purpose: Describing Real-time Electronic Navigational Chart Distribution & Streaming Service in the maritime domain for utilization in ECDIS & ECS.

### 4.6.1.5 Data product specification metadata

This information uniquely identifies this Product Specification and provides information about its creation and maintenance. For further information on dataset metadata see the metadata clause.

- Title: 	ENC Service Meta

- S-100 Version: 	4.0.0

- ENC Service Meta Version:	1.0.0

- Date: 	2019-03-08

- Language: 	English

- Classification: 	Unclassified

- Contact: 	32, Yuseong-daero 1312beon-gil, Yuseong-gu, Daejeon, Korea 32(Jang Dong) inside KRISO

- URL: 	http://www.smart-navigation.org

- Identifier: 	ENC Service Meta

- Maintenance: 	Amendments to this specification will be produced on a needs basis. For reporting issues with this specification which need correction, use the contact information.

### 4.6.2 Specification Scopes

This product specification describes one data product and therefore requires only one scope which is described below:

- Scope ID:  		ENC Service Meta datasets.

- Hierarchical level: 		MD_ScopeCode 005

- Hierarchical level name: 	dataset.

- Level description:		information applies to the dataset

- Extent:	EX_Extent.description: Korean Coast

### 4.6.3 Dataset identification

This section describes how to identify data sets that conform to the specification. A dataset that conforms to this Product Specification may be identified by its discovery metadata as defined in clause 12 of this specification. The information identifying the data product may include the following items from S-100 3.0.0 clause 11-6 (adapted from ISO 19115).

| Title | ENC Service Meta |
| --- | --- |
| Abstract | ENC Service Meta dataset is a textTable dataset containing message meta information to exchange ENC for Real-time Electronic Navigational Chart Distribution & Streaming Service such as request and response message. |
| Acronym | ENCSM |
| Content | Device property, download address, zone information, category of ENC, Type of ENC, Category of Service, type of zone. |
| Topic Category | Utilities Communication |
| Purpose | Describing Real-time Electronic Navigational Chart Distribution & Streaming Service in the maritime domain for utilization in ECDIS. |
| Language | EN Additional values if any use CharacterString values from ISO 639-2 |

### 4.6.4 Data Content and structure

### 4.6.4.1 Introduction

The ENC Servcie Meta product is based on the S-100 General Feature Model (GFM), and is a feature-based vector product. All ENC Servcie Meta features and information classes are derived from one of the abstract classes FeatureType and InformationType defined in the ENC Servcie Meta respectively.

This clause contains the Application Schema expressed in UML and an associated Feature Catalogue.

### 4.6.4.2 Application Schema

The UML models in this clause are segments of the overall ENC Servcie Meta application schema, and include overviews of the feature classes, information classes, meta features, spatial types, and the relationships between them.

This clause contains a general overview of the classes and relationships in the ENC Servcie Meta application schema. Detailed information about how to use the feature types and information types to encode ENC Servcie Meta information is provided in the SMART-Navigation Data Classification and Encoding Guide (DCEG).

The following conventions are used in the UML diagrams depicting the application schema:

- Standard UML conventions for classes, associations, inheritance, roles, and multiplicities apply. These conventions are described in Part 1 of S-100.

- Italic font for a class name indicates an abstract class.

- Feature classes are depicted with pink background; the dark shade for abstract feature classes and the light shade for ordinary (non-abstract) feature classes.

- Complex attributes are depicted with a light yellow background.

- Enumeration lists and codelists are depicted with a green background. The numeric code corresponding to each listed value is shown to its right following an ‘=’ sign.

- No significance attaches to the colour of associations. (Complex diagrams may use different colours to distinguish associations that cross one another.)

- Subclasses inherit the attributes and associations of their superclasses at all levels, unless such inheritance is explicitly overridden in the subclass.

### 4.6.4.2.1 Domain model

Figure 14 ENC Service Meta Overview

### 4.6.4.3 Feature Catalogue

### 4.6.4.3.1 Introduction

The Feature Catalogue describes the feature types, information types, attributes, attribute values, associations and roles which may be used in the product.  The ENC Service Meta Feature Catalogue is available in an XML document which conforms to the S-100 XML Feature Catalogue Schema.  Simple attributes used in this specification are listed in below.

Name: 	ENC Service Meta Feature Catalogue

Scope: 	Smart-Navigation

Version Number: 	1.0.0

Version Date:	2019-03-15

Producer:	SMART Navigation Project

Language: 	                  English

### 4.6.4.3.1.1 Feature Types

Feature types contain descriptive attributes that characterize real-world entities. The word ‘feature’ may be used in one of two senses – feature type and feature instance. A feature type is a class and is defined in a Feature Catalogue. A feature instance is a single occurrence of the feature type and represented as an object in a dataset. A feature instance is located by a relationship to one or more spatial instances. A feature instance may exist without referencing a spatial instance.

### 4.6.4.3.1.2 Geographic

A Geographic (Geo) feature type carries the descriptive characteristics of a real-world entity.

### 4.6.4.3.1.3 Meta

Meta features contain information about other features within a dataset. Information defined by meta features override the default metadata values defined by the dataset descriptive records. Meta attribution on individual features overrides attribution on meta features.

### 4.6.4.3.1.4 Feature Relationship

A feature relationship links instances of one feature type with instances of the same or a different feature type.

### 4.6.4.3.1.5 Information Types

Information types are identifiable pieces of information in a dataset that can be shared between other features. They have attributes but have no relationship to any geometry; information types may reference other information types.

### 4.6.4.3.1.6 Attributes

SMART-Navigation product defines attributes as either simple or complex.

### 4.6.4.3.1.6.1 Simple Attributes

SMART-Navigation product uses nine types of simple attributes; they are listed in the following table:

| Type | Definition |
| --- | --- |
| Enumeration | A fixed list of valid identifiers of named literal values |
| Boolean | A value representing binary logic. The value can be either True or False. The default state for Boolean type attributes (i.e. where the attribute is not populated for the feature) is False. |
| Real | A signed Real (floating point) number consisting of a mantissa and an exponent |
| Integer | A signed integer number. The representation of an integer is encapsulation and usage dependent. |
| Text | An arbitrary-length sequence of characters including accents and special characters from a repertoire of one of the adopted character sets |
| Date | A date provides values for year, month and day according to the Gregorian Calendar. Character encoding of a date is a string which must follow the calendar date format (complete representation, basic format) for date specified by ISO 8601:1988. EXAMPLE 19980918 (YYYY-MM-DD) |
| Time | A time is given by an hour, minute and second. Character encoding of a time is a string that follows the local time (complete representation, basic format) format defined in ISO 8601:1988. EXAMPLE 183059 or 183059+0100 or 183059Z |
| Date and Time | A DateTime is a combination of a date and a time type. Character encoding of a DateTime shall follow ISO 8601:1988 EXAMPLE 19850412T101530 |
| Codelist | A type of flexible enumeration. A code list type is a list of literals which may be extended only in conformance with specified rules. |

### 4.6.4.3.1.6.2 Complex Attributes

Complex attributes are aggregations of other attributes that are either simple or complex. The aggregation is defined by means of attribute bindings.

### 4.6.4.3.1.6.3 Geometric Representation

Geometric representation is the digital description of the spatial component of an object as described in S-100 and ISO 19107. This product specification uses no geometry attribute.

### 4.6.4.3.2 Feature Catalogue General

The ENC Service Meta feature catalogue describes the feature types, information types, attributes, attribute values, associations and roles which may be used in an ENC servide Meta data set.

### 4.6.4.3.2.1 Feature Types

- Geographic

- Meta

- ENCServiceMeta

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| shipMRN | MRN of the ship using the ENC service | URN | 1 | mandatory |  |
| serviceMRN | MRN of the service providing the ENC service | URN | 0..1 | option |  |

- ENCServiceRequest

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| categoryOfService | List of service type requested | enumeration | 1 | mandatory |  |
| vesselPosition | Geometry position of the ship requesting the ENC service | GM_Point | 0..1 | option |  |
| deviceProperty | Detail information related to the ship device using the ENC service | complex attribute | 0..1 | option |  |
| downloadRequestDetail | Detailed information to be delivered for the ENC download service request | complex attribute | 0..1 | option |  |

- ENCDownload

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| result | Response result of requested contents | boolean | 1 | mandatory |  |
| information | If the response to the request is un-successful, its detailed information | complex attribute | 0..1 | option |  |
| downloadInformation | Information to be delivered to the terminal for download service | complex attribute | 1..* | mandatory |  |

- ENCStreaming

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| deviceCertificateKey | Authentication key for providing streaming service | text | 1 | mandatory |  |
| WMSAddress | WMS address for providing streaming service | text | 0..1 | option |  |
| WMTSAddress | WMTS address for providing streaming service | text | 0..1 | option |  |

- ZoneInformation

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| result | Response result of requested contents | boolean | 1 | mandatory |  |
| information | If the response to the request is un-successful, its detailed information | complex attribute | 0..1 | option |  |
| zoneProperty | Detailed information of the zone delivered to the ship device | complex attribute | 1..* | mandatory |  |

### 4.6.4.3.2.2 Feature Relationship

- N/A

### 4.6.4.3.2.3 Information Relationship

- N/A

### 4.6.4.3.2.4 Information Type

- N/A

### 4.6.4.3.2.5 Simple Attribute

- Simple Attribute List

| Name | Description | Data Type | Remarks |
| --- | --- | --- | --- |
| shipMRN | MRN of the ship using the ENC service | URN |  |
| serviceMRN | MRN of the service providing the ENC service | URN |  |
| categoryOfService | List of service type requested | enumeration |  |
| result | Response result of requested contents | boolean |  |
| deviceCertificateKey | Authentication key for providing streaming service | text |  |
| WMSAddress | WMS address for providing streaming service | text |  |
| WMTSAddress | WMTS address for providing streaming service | text |  |
| categoryOfENC | There are two types of ENCs that request EN (Base) and ER (Update). | enumeration |  |
| hashFunctionValue | Hash Function Value | text |  |
| zoneName | The name of ENC Zone | text |  |
| zoneVersion | The version of ENC Zone | text |  |
| deviceLicense | The license number assigned to the ship device using the service | text |  |
| deviceMaker | The manufacturer information of the ship device using the service | text |  |
| deviceSerialNumber | The serial number of the ship device using the service | text |  |
| deviceModel | The model name of the ship device using the service | text |  |
| downloadURL | URL address for downloading ENC | URL |  |
| fileName | Name of the ENC file to be downloaded | text |  |
| destinationPath | ENC download path | text |  |
| fileSize | size of the file to be exchanged (kilobyte) | real |  |
| language | The method of human communication, either spoken or written, consisting of the use of words in a structured and conventional way. | ISO 3166-3 |  |
| text | A non-formatted digital text string. | text |  |
| northLatitude | Coordinates of northern boundary | real |  |
| southLatitude | Coordinates of southern boundary | real |  |
| eastLongitude | Coordinates of eastern boundary | real |  |
| westLongitude | Coordinates of western boundary | real |  |
| typeOfENC | - | enumeration |  |
| ENCVersion | - | text |  |
| ENCReleaseDate | - | date |  |
| typeOfZone | - | enumeration |  |
| installedENCVersion | - | text |  |
| installedENCReleaseDate | - | date |  |

- categoryOfENC

| Value | Label | Description |
| --- | --- | --- |
| 1 | Base collection | Collection of all ENC base and revision datasets at particular point in time. |
| 2 | Update collection | Update collection of all new ENC base and revision datasets since last request. |

- typeOfENC

| Value | Label | Description |
| --- | --- | --- |
| 1 | S-57 | Electronic navigation chart |
| 2 | S-63 | - |
| 3 | S-101 | Next generation Electronic navigation chart |

- categoryOfService

| Value | Label | Description |
| --- | --- | --- |
| 1 | ENC Download | - |
| 2 | ENC Streaming | - |
| 3 | Zone Information | zone information |

- typeOfZone

| Value | Label | Description |
| --- | --- | --- |
| 1 | BaseZone | - |
| 2 | UpdateZone | - |

### 4.6.4.3.2.6 Complex Attribute

- downloadInformation

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| categoryOfENC | There are two types of ENCs that request EN (Base) and ER (Update). | enumeration | 1 | mandatory |  |
| hashFunctionValue | Hash Function Value | text | 1 | mandatory |  |
| downloadExchangeset | Information provided to enable the download feature on your device | complex attribute | 1 | mandatory |  |
| responseZone | Information about the zone of the download service and details of the ENC to be installed in the zone | complex attribute | 1 | mandatory |  |

- zoneProperty

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| zoneName | The name of ENC Zone | text | 1 | mandatory |  |
| zoneVersion | The version of ENC Zone | text | 1 | mandatory |  |
| zoneBoundary | Boundary information of the zone | complex attribute | 1 | mandatory |  |

- deviceProperty

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| deviceLicense | The license number assigned to the ship device using the service | text | 1 | mandatory |  |
| deviceMaker | The manufacturer information of the ship device using the service | text | 0..1 | option |  |
| deviceSerialNumber | The serial number of the ship device using the service | text | 0..1 | option |  |
| deviceModel | The model name of the ship device using the service | text | 0..1 | option |  |

- downloadRequestDetail

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| categoryOfENC | There are two types of ENCs that request EN (Base) and ER (Update). | enumeration | 1 | mandatory |  |
| zoneProperty | ENC Information about the zone requesting the download service and details of the ENC installed in the zone for BASE ENC | complex attribute | 1..* | mandatory |  |

- downloadExchangeset

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| downloadURL | URL address for downloading ENC | URL | 1 | mandatory |  |
| fileName | Name of the ENC file to be downloaded | text | 1 | mandatory |  |
| destinationPath | ENC download path | text | 1 | mandatory |  |
| fileSize | Size of the file to be exchanged (kilobyte) | real | 1 | mandatory |  |

- information

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| language | The method of human communication, either spoken or written, consisting of the use of words in a structured and conventional way. | ISO 3166-3 | 0..1 | option |  |
| text | A non-formatted digital text string. | text | 1 | mandatory |  |

- zoneBoundary

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| northLatitude | Coordinates of northern boundary | real | 1 | mandatory |  |
| southLatitude | Coordinates of southern boundary | real | 1 | mandatory |  |
| eastLongitude | Coordinates of eastern boundary | real | 1 | mandatory |  |
| westLongitude | Coordinates of western boundary | real | 1 | mandatory |  |

- responseZone

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| zoneName | The name of ENC Zone | text | 1 | mandatory |  |
| zoneVersion | The version of ENC Zone | text | 1 | mandatory |  |
| typeOfENC | - | enumeration | 0..1 | option |  |
| ENCVersion | - | text | 0..1 | option |  |
| ENCReleaseDate | - | date | 0..1 | option |  |

- requestZoneProperty

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| zoneName | The name of ENC Zone | text | 1 | mandatory |  |
| zoneVersion | The version of ENC Zone | text | 1 | mandatory |  |
| typeOfZone | - | enumeration | 1 | mandatory |  |
| installedENCVersion | - | text | 0..1 | option |  |
| installedENCReleaseDate | - | date | 0..1 | option |  |

### 4.6.5 Coordinate Reference System(CRS)

N/A (This model has no geometry attribute)

### 4.6.6 Data Quality

### 4.6.6.1 Introduction

ENC Service Meta products must be tested with the ENC Service Meta specific checks prior to release by the data producer. The data producer must review the check results and address any issues to ensure sufficient quality of the data products. The checks are a mix of data format validation checks, conformance to standard checks and logical consistency checks. But the checklist has not been written yet. It will be updated in the next version.

### 4.6.7 Data Capture and classification

See DCEG for this section.

### 4.6.8 maintenance

### 4.6.8.1 Introduction

Datasets are maintained as needed and must include mechanisms for ENC Service Meta updating. Data updates will be made by new editions. The maintenance and update frequency of ENC Service Meta datasets should be defined by the producers (official national authority) implementing this specification.

Data Producers must use applicable sources to maintain and update data and provide a brief description of the sources that were used to produce the dataset in the appropriate metadata field.

The data product shall provide information on how the data is maintained and should describe the principles and criteria applied in maintenance regime. This should specify the expected frequency of updates.

Table 21 Maintenance and update frequency

| Item Name | Description | Multiplicity | Type |
| --- | --- | --- | --- |
| maintenanceAndUpdateFrequency | Frequency with which changes and additions are made to the data product (per update scope) | 1..* | MD_MaintenanceInformation (ISO 19115) |
| dataSource | Identification of the kinds of data sources usable to produce datasets | 1..* | LI_Source (ISO 19115) |
| productionProcess | Textual description of the production process applicable to the datasets (per scope or data source) | 1..* | LI_ProcessStep (ISO 19115) |

### 4.6.8.2 Production process for base and update datasets

Data Producers should follow their established production processes for maintaining and updating datasets. Data is produced against the DCEG and checked against the appropriate set of validation rules in Appendix E. (Appendix E is not included in this release.)

### 4.6.8.3 Dataset updates

N/A (Dataset update is not relevant in this model)

### 4.6.8.4 Support file updates

N/A ( Support file is not included in this model)

### 4.6.8.5 Feature and portrayal catalogues

For each new version of the ENC Service Meta Product Specification a new feature catalogue will be released. The system must be able to manage datasets and their catalogues that are created on different versions of the ENC Service Meta product specification (Portrayal Catalogue is not defined in this model).

### 4.6.8.6 Feature history, versions, and change tracking

If applications or production systems require versioning of individual instances of feature or information types, maintenance of histories, or change tracking, the methods for versioning, history management, and change tracking and display are left to the application or production system.

### 4.6.8.7 Dataset encryption

Details about dataset encryption are still to be determined, and may mirror the method described in S-101.

### 4.6.9 Portrayal

Portrayal is not defined in this version of ENC Service Meta Product Specifications. Users are free to choose the means and methodology of portrayal as they see best suited for their needs. It should be noted that future versions of ENC Service Meta may include a portrayal catalogue, and any implementer should therefore anticipate this, and make sufficient provisions in any system supporting ENC Service Meta.

### 4.6.10 data product format(encoding)

### 4.6.10.1 Introduction

The GML encoding of ENC Service Meta datasets is based on the S-100 profile of GML 3.2.1.

Format Name: ENC Service Meta GML

Version:	1.0

Character Set: UTF-8

Specification: SMART-Navigation Product Specification

### 4.6.10.2 Encoding of Latitude and Longitude

N/A (This model has no geometry attribute).

### 4.6.10.3 Text Attribute

Character strings must be encoded using the character set defined in ISO 10646-1, in Unicode Transformation Format-8 (UTF-8).

### 4.6.10.4 Mandatory Attribute Values

There are four reasons why attribute values may be considered mandatory:

- They determine whether a feature is in the display base,

- Certain features make no logical sense without specific attributes,

- Some attributes are necessary to determine which symbol is to be displayed,

- Some attributes are required for safety of navigation.

All mandatory attributes are identified in the Feature Catalogue.

### 4.6.10.5 Unknow Attribute Values

When a mandatory attribute code or tag is present but the attribute value is missing, it means that the producer wishes to indicate that this attribute value is unknown.  Missing mandatory attributes must be “nilled” with a GML nilReason attribute giving the reason for omission.

Optional attributes must be omitted altogether if the value is unknown or missing. They must not be “nilled.”

### 4.6.10.6 Structure of dataset files

### 4.6.10.6.1 Sequence of dataset files

The order of data objects in each dataset file is described below:

Dataset Identification Information

Dataset structure information

Spatial records for by-reference geometries

Point

Multi point

Curve

Composite Curve

Surface

Information objects

Feature objects (Geometry may be encoded inline or by reference.)

Meta features

Geo features

### 4.6.10.7 Numeric Attribute Encoding

Floating point and integer attribute values must not contain leading zeros.  Floating point attribute values must not contain non-significant trailing zeros.

### 4.6.10.8 Object identifiers

The “name” of feature records must provide a unique world-wide identifier of feature records. The “name” of the record is the combination of the subfields agency, featureObjectIdentifier, and featureIdentificationSubdivision elements of the featureObjectIdentifier element of the object.

Features, information types, collection objects, meta features, and geometries (inline or external) are all required by the schema to have a gml:id attribute with a value that is unique within the dataset. The gml:id values must be used as the reference for the object from another object in the same dataset or another dataset.

### 4.6.10.9 Dataset validation

Fields may be repeated or omitted as permitted by the XML schemas and the validation tests. Since XML schema cannot encode rules for conditional presence or attributes, these rules are checked by Schematron rules or other validation code.

### 4.6.10.10 Location of Data Product Format schema Files

The schema file will provide a future location and is available from that location.

### 4.6.10.11 Detailed documentation of ENC Service Meta XML Schema

See DCEG for this section.

### 4.6.11 data product delivery

This clause specifies the encoding and delivery mechanisms for an ENC Service Meta dataset.  Data which conforms to this product specification must be delivered by means of an exchange set.

### 4.6.11.1 Exchange set

ENC Service Meta datasets are grouped into exchange sets. Each exchange set consists of one or more ENC Service Meta datasets with an associated XML metadata file and a single Exchange Catalogue XML file containing metadata. It may also include one or more support files.

Units of Delivery:				Exchange Set

Transfer Size:					Unlimited

Medium Name:					Digital data delivery

Other Delivery Information:

Each dataset must be contained in a physically separate, uniquely identified file on the transfer medium.

Each exchange set has a single exchange catalogue which contains the discovery metadata for each dataset and references to any support files.

Support files are supplementary information which are linked to the features and information types by attributes. The attributes containing these links are described in the application schema and feature catalogue.

An exchange set may be encapsulated into a form suitable for transmission by a mapping called a transmission encoding. An encoding translates each of the elements of the exchange set into a logical form suitable for writing to media and for transmission online. An encoding may also define other elements in addition to the exchange set contents (i.e., media identification, data extents etc…) and also may define commercial constructs such as encryption and compression methods.

If the data is transformed (e.g., for encryption or compression purposes) its content must not be changed.

This product specification does not define the transmission encoding which must be used as a default for transmission of data between parties.

The exchange set elements are as follows:

Mandatory Elements

- ENC Service Meta datasets – GML encoding of features/attributes and their associated geometry and metadata.

- Exchange Catalogue – the XML encoded representation of exchange set catalogue features [discovery metadata].

Optional Elements

- Supplementary files – These are contained within the exchange set as files and the map from the name included within the dataset and the physical location on the media is defined within the Exchange Catalogue.

- Feature Catalogue – If it is necessary to deliver the latest feature catalogue to the end user it may be done using the ENC Service Meta exchange set mechanism for datasets – i.e., include the updated feature catalogue in an exchange set.

### 4.6.11.2 Dataset

### 4.6.11.2.1 Datasets

Datasets are distributed as files which are part of exchange sets structured as described in this specification. The distribution media are left to the discretion of the producer and distributor.

The following types of dataset files may be produced and contained within an exchange set:

- New dataset and new edition of a dataset (base dataset):  Each new edition of a dataset must have the same name as the dataset that it replaces.  A new edition can also be AtoN data that has previously been produced for this area.

### 4.6.11.2.2 Dataset size

Datasets must not exceed 1 MB.

### 4.6.11.2.3 Dataset file naming

Dataset files shall be named

CCXXXXXXXX_EEE.GML

The file name forms a unique identifier where:

- CC - the first two characters identify the issuing agency (mandatory).

- The third to tenth characters are optional and may be used in any way by the producer to provide the unique file name. The following characters are allowed in the dataset name, A to Z, 0 to 9 and the special character _ (underscore).

- EEE – new editions and re-issues use 000, and increment until a limit of 999 (mandatory).

- The maximum number of characters preceding _EEE is ten.

- GML – the character sequence “GML” or “gml”.

### 4.6.11.2.4 New Editions and re-issues

This section defines the sequencing of datasets for New Editions. In order to ensure that feature type updates are incorporated into an SYSTEM in the correct sequence without any omission, a number of parameters encoded in the data header and/or metadata are used in the following way:

| edition number | When a dataset is initially created, the edition number 1 is assigned to it. The edition number is increased by 1 at each new edition. |
| --- | --- |

### 4.6.11.2.5 Cancellations

Dataset cancellation is not considered I this product specification. In the SMART Navigation project, the system automatically cancels the dataset after 7 days from the date the dataset is received.

### 4.6.11.3 Support Files

N/A (Support file is not included in this model)

### 4.6.11.4 Exchange Catalogue

N/A (Exchange Catalogue is not used in SMART-Navigation)

### 4.6.12 meta data

### 4.6.12.1 Introduction

The metadata description of ENC Service Meta is based on the S-100 metadata document section, which is a profile of the ISO 19115 standard. These documents provide a structure for describing digital geographic data and define metadata elements, a common set of metadata terminology, definitions and extension procedures.

Two metadata packages are described in this product specification: dataset metadata and exchange set metadata.

Figure 15 Metadata packages

Note 1: Types with CI_, EX_, and MD_ prefixes are from packages defined in ISO 19115 and adapted by S-100. Types with S100_ prefix are from packages defined in S-100.

Note 2: When a dataset is terminated, the purpose metadata field is set to 3 (terminated), and the editionNumber metadata field is set to 0. All other metadata fields must be blank.

### 4.6.12.2 Dataset metadata

Dataset metadata is intended to describe information about a dataset. It facilitates the management and exploitation of data and is an important requirement for understanding the characteristics of a dataset. Whereas dataset metadata is usually fairly comprehensive, there is also a requirement for a constrained subset of metadata elements that are usually required for discovery purposes. Discovery metadata are often used for building web catalogues, and can help users determine whether a product or service is fit for purpose and where they can be obtained.

Table 22 Dataset metadata

| Name | Cardinality | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| S100_DataSetDiscoveryMetadata |  |  |  |  |
| fileName | 1 |  | CharacterString | Dataset file name |
| filePath | 1 |  | CharacterString | Full path from the exchange set root directory |
| description | 1 |  | CharacterString | Short description of the area covered by dataset, e.g., area, harbour, or port name, between two named locations etc. |
| dataProtection | 0..1 | {1} or {2} | Boolean | True : Encrypted False : Unencrypted |
| protectionScheme | 0..1 |  | CharacterString |  |
| digitalSignature | 0..1 |  | CharacterString |  |
| digitalSignatureReference | 0..1 |  | CharacterString |  |
| digitalSignatureValue | 0..1 |  | CharacterString |  |
| copyright | 0..1 |  | MD_LegalConstraints>MD_RestrictionCode <copyright> (ISO 19115-1) |  |
| classification | 0..1 |  | Class MD_SecurityConstraints>MD_ClassificationCode (codelist) ISO 19115-1 | 1. unclassified 2. restricted 3. confidential 4. secret 5. top secret |
| purpose | 1 | {1}, {2} | CharacterString | - New dataset - New edition |
| specificUsage | 1 |  | MD_USAGE>specificUsage (character string) MD_USAGE>userContactInfo (CI_Responsibility) | Brief description of the resource and/or resource series usage |
| editionNumber | 1 | {1} | Integer | When a dataset is initially created, the edition number “1” is assigned to it. The edition number is increased by one with each new edition. |
| updateNumber | 1 |  | CharacterString | Update number 0 is assigned to a new dataset. |
| updateApplicationDate | 1 |  | Date | All updates dated on or before this date must have been applied by the producer. |
| issueDate | 1 |  | Date | Date on which the dataset was generated. |
| productSpecification | 1 |  | S100_ProductSpecification | See Notes below this table for constraints on values. |
| producingAgency | 1 |  | CI_Responsibility>CI_Organisation or CI_Responsibility>CI_Individual | Party responsible for generating the dataset. |
| optimumDisplayScale | 0..1 |  | Integer |  |
| maximumDisplayScale | 0..1 |  | Integer |  |
| minimumDisplayScale | 0..1 |  | Integer |  |
| horizontalDatumReference | 1 | EPSG | CharacterString |  |
| horizontalDatumValue | 1 | 4326 | Integer | WGS84 |
| verticalDatum | 0..1 |  | S100_VerticalAndSoundingDatum | Not included in ENC Service Meta |
| soundingDatum | 0..1 |  | S100_VerticalAndSoundingDatum | Not included in ENC Service Meta |
| dataType | 1 | GML | CharacterString | The only value allowed is “GML”. |
| otherDataTypeDescription | 0..1 |  | CharacterString |  |
| dataTypeVersion | 1 | 3.2.1 | CharacterString |  |
| dataCoverage | 1..* |  | S100_DataCoverage |  |
| comment | 0..1 |  | CharacterString | Any additional Information |
| layerID | 1..* | S-101 | CharacterString | Dataset must be used with ENC in an ECDIS. |

### 4.6.12.3 Update Dataset Metadata

N/A (Dataset update is not relevant in this model)

### 4.6.12.4 Support file metadata

N/A (Support file is not included in this model)

### 4.6.12.5 Exchange set metadata

N/A (Exchnage Catalogue is not used in SMART-Navigation).

### 4.6.12.6 Catalogue file Metadata

All ENC Service Meta Exchange Catalogue files must contain at least the mandatory metadata elements.

Table 23 Catalogue file metadata

| Name | Multiplicity | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| S100_ExchangeCatalogue |  |  |  |  |
| identifier | 1 |  | S100_CatalogueIdentifier |  |
| contact | 1 |  | S100_CataloguePointOfContact |  |
| productSpecification | 0..1 |  | S100_ProductSpecification | Conditional on all the datasets using the same product specification.. |
| metadataLanguage | 1 | English | CharacterString | All datasets conforming to this PS must use English language. A catalogue in English must be provided. Discovery metadata elements within catalogues have their own locale attributes and may be repeated in languages other than English. |
| exchangeCatalogueName | 1 |  | CharacterString | Catalogue filename |
| exchangeCatalogueDescription | 1 |  | CharacterString |  |
| exchangeCatalogueComment | 0..1 |  | CharacterString | Any additional Information |
| compressionFlag | 0..1 |  | Boolean | Yes or No |
| sourceMedia | 0..1 |  | CharacterString |  |
| replacedData | 0..1 |  | Boolean |  |
| dataReplacement | 0..1 |  | CharacterString |  |

## 4.7 pilot service request

### 4.7.1 Overview

### 4.7.1.1 Introduction

This document has been produced by the member of SMART-Navigation project team(Korean Register, Marine-Works) in response to a requirement to produce a data product for requesting Port Guideline, Ship Density, Pilot Schedule product specification at the Pilot & Tug Assistance Service. It is based on the IHO S-100 framework specification and the ISO 19100 series of standards. It is a textTable product specification that is primarily intended for encoding the information of ship position predict request, ship density request, port guideline request, marine ship request.

### 4.7.1.2 References

IHO S-100 IHO Universal Hydrographic Data Model Edition 4.0.0 (December 2017).

ISO 8601. 2004. Data elements and interchange formats - Information interchange - Representation of dates and times. 2004.

ISO 3166-1. 1997. Country Codes. 1997.

ISO 10646-1 Information technology - Universal Multiple-Octet Coded Character Set (UCS) - Part 1: Architecture and Basic Multilingual Plane

ISO 19101-2:2008 Geographic Information - Rules for Application Schema

ISO/TS 19103:2005 Geographic Information - Conceptual schema language

ISO 19106:2004 Geographic Information – Profiles

ISO 19107:2003 Geographic Information – Spatial schema

ISO 19109:2005 Geographic Information - Rules for Application Schema

ISO 19111:2003 Geographic Information - Spatial referencing by coordinates

ISO 19115:2003+Corr1 (2006) Geographic Information - Metadata

ISO 19115-2:2009 Geographic Information - Metadata: Extensions for imagery and gridded data

ISO 19123:2005 Geographic Information - Schema for coverage geometry and functions

ISO 19129:2009 Geographic Information - Imagery gridded and coverage data framework

ISO 19131:2007 Geographic Information - Data product specifications

ISO 19136:2007 Geographic Information – Geography Markup Language

ISO 19136-2:2015, Geographic Information – Geography Markup Language.

ISO/TS 19139, Geographic Information – Metadata – XML schema implementation.

### 4.7.1.3 Terms, definitions and abbreviations

Terms and Definitions

The S-100 framework is based on the ISO 19100 series of geographic standards. The terms and definitions provided here are used to standardize the nomenclature found within that framework, whenever possible. They are taken from the references cited in clause 2.1. Modifications have been made when necessary.

- Application

Manipulation and processing of data in support of user requirements (ISO 19101)

- Application schema

Conceptual schema for data required by one or more applications (ISO 19101)

- Conceptual model

Model that defines concepts of a universe of discourse (ISO 19101)

- Conceptual schema

Formal description of a conceptual model (ISO 19101)

- Coverage

Feature that acts as a function to return values from its range for any direct position within its spatial, temporal or spatiotemporal domain (ISO 19123)

- Data product

Dataset or dataset series that conforms to a data product specification

- Data product specification

Detailed description of a dataset or dataset series together with additional information that will enable it to be created, supplied to and used by another party

- Dataset

Identifiable collection of data (ISO 19115)

- Feature

Abstraction of real world phenomena (ISO 19101)

- Feature association

Relationship that links instances of one feature type with instances of the same or a different feature  type (ISO19110)

- Feature attribute

Characteristic of a feature (ISO 19101)

- Geographic data

Data with implicit or explicit reference to a location relative to the Earth (ISO 19109)

- Metadata

Data about data (ISO 19115)

- Model

Abstraction of some aspects of reality (ISO 19109)

- Portrayal

Presentation of information to humans (ISO 19117)

- Quality

Totality of characteristics of a product that bear on its ability to satisfy stated and implied needs (ISO 19101)

Abbreviations

This product specification adopts the following convention for symbols and abbreviated terms:

- ASCII 	American Standard Code for Information Interchange

- ECDIS	Electronic Chart Display and Information Systems

- ECS	Electronic Chart Systems

- ENC	Electronic Navigational Chart

- GML	Geography Markup Language

- IHO	International Hydrographic Organization

- IOC	International Oceanographic Commission

- ISO	International Organization for Standardization

- UML	Unified Modelling Language

- URI	Uniformed Resource Identifier

- URL 	Uniform Resource Locator

- WMS	Web Map Service

- WFS	Web Feature Service

- www 	World Wide Web

- XML	Extensible Markup Language

- XSLT	eXtensible Stylesheet Language Transformations

### 4.7.1.4 General Data Product Description

Informal Description of Data Product

This clause contains general information about the data product

- Title:  	Pilot Service Request Product Specification.

- Abstract: 	 Pilot Service Request product specification provides the means to request relevant product specification(Ship Density, Port Guideline Pilot Schedule) for the pilot & Tug Assistance Service. This may include details on marine ship request, ship position predict request, ship density request, port guideline request.

- Content:	Datasets conforming to this specification will contain message information to request for Pilot & Tug Assistance Service. Additionally, there will be relevant metadata data quality, production authority, data sources and publication date.

- Spatial Extent:	Korean Coast

- Specific Purpose: Describing method utilizing pilot & Tug assistance service.

### 4.7.1.5 Data product specification metadata

This information uniquely identifies this Product Specification and provides information about its creation and maintenance. For further information on dataset metadata see the metadata clause.

- Title: 	Pilot Service Request

- S-100 Version: 	4.0.0

- Pilot Service Request Version:	1.0.0

- Date: 	2019-03-08

- Language: 	English

- Classification: 	Unclassified

- Contact: 	32, Yuseong-daero 1312beon-gil, Yuseong-gu, Daejeon, Korea 32(Jang Dong) inside KRISO

- URL: 	http://www.smart-navigation.org

- Identifier: 	Pilot Service Request

- Maintenance: 	Amendments to this specification will be produced on a needs basis. For reporting issues with this specification which need correction, use the contact information.

### 4.7.2 Specification Scopes

This product specification describes one data product and therefore requires only one scope which is described below:

- Scope ID:  		Pilot Service Request datasets.

- Hierarchical level: 		MD_ScopeCode 005

- Hierarchical level name: 	dataset.

- Level description:		information applies to the dataset

- Extent:	EX_Extent.description: Korean Coast

### 4.7.3 Dataset identification

This section describes how to identify data sets that conform to the specification. A dataset that conforms to this Product Specification may be identified by its discovery metadata as defined in clause 12 of this specification. The information identifying the data product may include the following items from S-100 3.0.0 clause 11-6 (adapted from ISO 19115).

| Title | Pilot Service Request |
| --- | --- |
| Abstract | Pilot Service Request dataset is a textTable dataset containing message information for Pilot & Tug assistance service |
| Acronym | PSR |
| Content | Pilot certification no., Type of marine ship request, Port Name, Request range |
| Topic Category | Utilities Communication |
| Purpose | Describing method utilizing for Pilot & Tug assistance service in the maritime domain |
| Language | EN Additional values if any use CharacterString values from ISO 639-2 |

### 4.7.4 Data Content and structure

### 4.7.4.1 Introduction

The Pilot Service Request product is based on the S-100 General Feature Model (GFM), and is a feature-based vector product. All Pilot Service Request features and information classes are derived from one of the abstract classes FeatureType and InformationType defined in the Pilot Service Request respectively.

This clause contains the Application Schema expressed in UML and an associated Feature Catalogue.

### 4.7.4.2 Application Schema

The UML models in this clause are segments of the overall Pilot Service Request application schema, and include overviews of the feature classes, information classes, meta features, spatial types, and the relationships between them.

This clause contains a general overview of the classes and relationships in the Pilot Service Request application schema. Detailed information about how to use the feature types and information types to encode Pilot Service Request information is provided in the SMART-Navigation Data Classification and Encoding Guide (DCEG).

The following conventions are used in the UML diagrams depicting the application schema:

- Standard UML conventions for classes, associations, inheritance, roles, and multiplicities apply. These conventions are described in Part 1 of S-100.

- Italic font for a class name indicates an abstract class.

- Feature classes are depicted with pink background; the dark shade for abstract feature classes and the light shade for ordinary (non-abstract) feature classes.

- Complex attributes are depicted with a light yellow background.

- Enumeration lists and codelists are depicted with a green background. The numeric code corresponding to each listed value is shown to its right following an ‘=’ sign.

- No significance attaches to the colour of associations. (Complex diagrams may use different colours to distinguish associations that cross one another.)

- Subclasses inherit the attributes and associations of their superclasses at all levels, unless such inheritance is explicitly overridden in the subclass.

### 4.7.4.2.1 Domain model

Figure 16 Pilot Service Request Overview

### 4.7.4.3 Feature Catalogue

### 4.7.4.3.1 Introduction

The Feature Catalogue describes the feature types, information types, attributes, attribute values, associations and roles which may be used in the product.  The Pilot Servide Request Feature Catalogue is available in an XML document which conforms to the S-100 XML Feature Catalogue Schema.  Simple attributes used in this specification are listed in below.

Name: 	Pilot Service Request Feature Catalogue

Scope: 	Smart-Navigation

Version Number: 	1.0.0

Version Date:	2019-03-15

Producer:	SMART Navigation Project

Language: 	                  English

### 4.7.4.3.1.1 Feature Types

Feature types contain descriptive attributes that characterize real-world entities. The word ‘feature’ may be used in one of two senses – feature type and feature instance. A feature type is a class and is defined in a Feature Catalogue. A feature instance is a single occurrence of the feature type and represented as an object in a dataset. A feature instance is located by a relationship to one or more spatial instances. A feature instance may exist without referencing a spatial instance.

### 4.7.4.3.1.2 Geographic

A Geographic (Geo) feature type carries the descriptive characteristics of a real-world entity.

### 4.7.4.3.1.3 Meta

Meta features contain information about other features within a dataset. Information defined by meta features override the default metadata values defined by the dataset descriptive records. Meta attribution on individual features overrides attribution on meta features.

### 4.7.4.3.1.4 Feature Relationship

A feature relationship links instances of one feature type with instances of the same or a different feature type.

### 4.7.4.3.1.5 Information Types

Information types are identifiable pieces of information in a dataset that can be shared between other features. They have attributes but have no relationship to any geometry; information types may reference other information types.

### 4.7.4.3.1.6 Attributes

SMART-Navigation product defines attributes as either simple or complex.

### 4.7.4.3.1.6.1 Simple Attributes

SMART-Navigation product uses nine types of simple attributes; they are listed in the following table:

| Type | Definition |
| --- | --- |
| Enumeration | A fixed list of valid identifiers of named literal values |
| Boolean | A value representing binary logic. The value can be either True or False. The default state for Boolean type attributes (i.e. where the attribute is not populated for the feature) is False. |
| Real | A signed Real (floating point) number consisting of a mantissa and an exponent |
| Integer | A signed integer number. The representation of an integer is encapsulation and usage dependent. |
| Text | An arbitrary-length sequence of characters including accents and special characters from a repertoire of one of the adopted character sets |
| Date | A date provides values for year, month and day according to the Gregorian Calendar. Character encoding of a date is a string which must follow the calendar date format (complete representation, basic format) for date specified by ISO 8601:1988. EXAMPLE 19980918 (YYYY-MM-DD) |
| Time | A time is given by an hour, minute and second. Character encoding of a time is a string that follows the local time (complete representation, basic format) format defined in ISO 8601:1988. EXAMPLE 183059 or 183059+0100 or 183059Z |
| Date and Time | A DateTime is a combination of a date and a time type. Character encoding of a DateTime shall follow ISO 8601:1988 EXAMPLE 19850412T101530 |
| Codelist | A type of flexible enumeration. A code list type is a list of literals which may be extended only in conformance with specified rules. |

### 4.7.4.3.1.6.2 Complex Attributes

Complex attributes are aggregations of other attributes that are either simple or complex. The aggregation is defined by means of attribute bindings.

### 4.7.4.3.1.6.3 Geometric Representation

Geometric representation is the digital description of the spatial component of an object as described in S-100 and ISO 19107. This product specification uses no geometry attribute.

### 4.7.4.3.2 Feature Catalogue General

The Pilot Service Request feature catalogue describes the feature types, information types, attributes, attribute values, associations and roles which may be used in a Pilot Service Request data set.

### 4.7.4.3.2.1 Feature Types

- Geographic

- Meta

- PilotServiceRequest

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| shipMRN | MRN is a naming scheme that can uniquely identify any maritime resource on a global scale. shipMRN is the unique value given to the ship. | URN | 1 | mandatory |  |
| MMSICode | The Maritime Mobile Service Identity (MMSI) Code is formed of a series of nine digits which are transmitted over the radio path in order to uniquely identify ship stations, ship earth stations, coast stations, coast earth stations, and group calls. These identities are formed in such a way that the identity or part thereof can be used by telephone and telex subscribers connected to the general telecommunications network principally to call ships automatically. (Adapted from Appendix 43 of the International Telecommunications Union Radio Regulations). | integer | 0..1 | option |  |
| IMONo | The seven-digit number assigned to the vessel, the vessel management company, and the ship documents company under the SOLAS rules. | integer | 0..1 | option |  |
| callSign | The designated call-sign of a radio station. | text | 0..1 | option |  |
| vesselName | - | text | 0..1 | option |  |
| requestTime | Time to request Pilot Service | dateTime | 0..1 | option |  |
| userMRN | - | URN | 0..1 | option |  |
| vesselName | - | complex attribute | 0..1 | option |  |
| shipLocation | - | GM_Point | 0..1 | option |  |

- MarineShipRequest

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| pilotCertificateNo | Pilot Certificate No | text | 0..1 | option |  |
| typeOfMarineShipRequest | - | enumeration | 1 | mandatory |  |

- ShipPositionPredictRequest

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| dataCount | Number of data requests. | integer | 1 | mandatory |  |
| dataInteval | Predicted data interval. unit: sec | integer | 1 | mandatory |  |

- ShipDensityRequest

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| requestRange | The radius of the area requesting information. unit: km | real | 1 | mandatory |  |

- PortGuidelineRequest

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| portName | Name of the port requesting information | complex attribute | 1..1 | mandatory |  |

- Cartographic

- Theme

### 4.7.4.3.2.2 Feature Relationship

- N/A

### 4.7.4.3.2.3 Information Relationship

- N/A

### 4.7.4.3.2.4 Information Type

- N/A

### 4.7.4.3.2.5 Simple Attribute

- Simple Attribute List

| Name | Description | Data Type | Remarks |
| --- | --- | --- | --- |
| shipMRN | MRN is a naming scheme that can uniquely identify any maritime resource on a global scale. shipMRN is the unique value given to the ship. | URN |  |
| MMSICode | The Maritime Mobile Service Identity (MMSI) Code is formed of a series of nine digits which are transmitted over the radio path in order to uniquely identify ship stations, ship earth stations, coast stations, coast earth stations, and group calls. These identities are formed in such a way that the identity or part thereof can be used by telephone and telex subscribers connected to the general telecommunications network principally to call ships automatically. (Adapted from Appendix 43 of the International Telecommunications Union Radio Regulations). | integer |  |
| IMONo | The seven-digit number assigned to the vessel, the vessel management company, and the ship documents company under the SOLAS rules. | integer |  |
| callSign | The designated call-sign of a radio station. | text |  |
| requestTime | Time to request Pilot Service | dateTime |  |
| userMRN | - | URN |  |
| pilotCertificateNo | Pilot Certificate No | text |  |
| typeOfMarineShipRequest | - | enumeration |  |
| dataCount | Number of data requests. | integer |  |
| dataInterval | Predicted data interval. | integer |  |
| requestRange | The radius of the area requesting information. | real |  |
| displayName | A statement expressing if a feature name is to be displayed in certain display settings or not. Indication: Boolean. A True value is an indication that the name is intended to be displayed. Remarks: Where it is allowable to encode multiple instances of feature name for a single feature instance, only one feature name instance can indicate that the name is to be displayed (display name set to True). | boolean |  |
| language | The language is encoded by a character code following ISO 639-3 | ISO639-3 |  |
| name | The individual name of a feature. | text |  |

- typeOfMarineShipRequest

| Value | Label | Description |
| --- | --- | --- |
| 1 | pilot and tug work schedule | - |
| 2 | ship additional information | - |
| 3 | pilot note | - |
| 4 | tugboat information | - |
| 5 | pilot card information | - |

### 4.7.4.3.2.6 Complex Attribute

- portName

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| displayName | A statement expressing if a feature name is to be displayed in certain display settings or not. Indication: Boolean. A True value is an indication that the name is intended to be displayed. Remarks: Where it is allowable to encode multiple instances of feature name for a single feature instance, only one feature name instance can indicate that the name is to be displayed (display name set to True). | boolean | 0..1 | option |  |
| language | The language is encoded by a character code following ISO 639-3 | ISO639-3 | 0..1 | option |  |
| name | The individual name of a feature. | text | 1 | mandatory |  |

- vesselName

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| displayName | A statement expressing if a feature name is to be displayed in certain display settings or not. Indication: Boolean. A True value is an indication that the name is intended to be displayed. Remarks: Where it is allowable to encode multiple instances of feature name for a single feature instance, only one feature name instance can indicate that the name is to be displayed (display name set to True). | boolean | 0..1 | option |  |
| language | The language is encoded by a character code following ISO 639-3 | ISO639-3 | 0..1 | option |  |
| name | The individual name of a feature. | text | 1 | mandatory |  |

### 4.7.5 Coordinate Reference System(CRS)

N/A (This model has no geometry attribute).

### 4.7.6 Data Quality

### 4.7.6.1 Introduction

Pilot Service Request products must be tested with the Pilot Service Request specific checks prior to release by the data producer. The data producer must review the check results and address any issues to ensure sufficient quality of the data products. The checks are a mix of data format validation checks, conformance to standard checks and logical consistency checks. But the checklist has not been written yet. It will be updated in the next version.

### 4.7.7 Data Capture and classification

See DCEG for this section.

### 4.7.8 maintenance

### 4.7.8.1 Introduction

Datasets are maintained as needed and must include mechanisms for Pilot Service Request updating. Data updates will be made by new editions. The maintenance and update frequency of Pilot Service Request datasets should be defined by the producers (official national authority) implementing this specification.

Data Producers must use applicable sources to maintain and update data and provide a brief description of the sources that were used to produce the dataset in the appropriate metadata field.

The data product shall provide information on how the data is maintained and should describe the principles and criteria applied in maintenance regime. This should specify the expected frequency of updates.

Table 24 Maintenance and update frequency

| Item Name | Description | Multiplicity | Type |
| --- | --- | --- | --- |
| maintenanceAndUpdateFrequency | Frequency with which changes and additions are made to the data product (per update scope) | 1..* | MD_MaintenanceInformation (ISO 19115) |
| dataSource | Identification of the kinds of data sources usable to produce datasets | 1..* | LI_Source (ISO 19115) |
| productionProcess | Textual description of the production process applicable to the datasets (per scope or data source) | 1..* | LI_ProcessStep (ISO 19115) |

### 4.7.8.2 Production process for base and update datasets

Data Producers should follow their established production processes for maintaining and updating datasets. Data is produced against the DCEG and checked against the appropriate set of validation rules in Appendix E. (Appendix E is not included in this release.)

### 4.7.8.3 Datasetr updates

N/A (Dataset update is not relevant in this model)

### 4.7.8.4 Support file updates

N/A (Support file is not included in this model)

### 4.7.8.5 Feature and portrayal catalogues

For each new version of the Pilot Service Request Product Specification a new feature catalogue will be released. The system must be able to manage datasets and their catalogues that are created on different versions of the Pilot Service Request product specification (Portrayal Catalogue is not defined in this model).

### 4.7.8.6 Feature history, versions, and change tracking

If applications or production systems require versioning of individual instances of feature or information types, maintenance of histories, or change tracking, the methods for versioning, history management, and change tracking and display are left to the application or production system.

### 4.7.8.7 Dataset encryption

Details about dataset encryption are still to be determined, and may mirror the method described in S-101.

### 4.7.9 Portrayal

Portrayal is not defined in this version of Pilot Service Request Product Specifications. Users are free to choose the means and methodology of portrayal as they see best suited for their needs. It should be noted that future versions of Pilot Service Request may include a portrayal catalogue, and any implementer should therefore anticipate this, and make sufficient provisions in any system supporting Pilot Service Request.

### 4.7.10 data product format(encoding)

### 4.7.10.1 Introduction

The GML encoding of Pilot Service Request datasets is based on the S-100 profile of GML 3.2.1.

Format Name: Pilot Service Request GML

Version:	1.0

Character Set: UTF-8

Specification: SMART-Navigation Product Specification

### 4.7.10.2 Encoding of Latitude and Longitude

N/A (This model has no geometry attribute)

### 4.7.10.3 Text Attribute

Character strings must be encoded using the character set defined in ISO 10646-1, in Unicode Transformation Format-8 (UTF-8).

### 4.7.10.4 Mandatory Attribute Values

There are four reasons why attribute values may be considered mandatory:

- They determine whether a feature is in the display base,

- Certain features make no logical sense without specific attributes,

- Some attributes are necessary to determine which symbol is to be displayed,

- Some attributes are required for safety of navigation.

All mandatory attributes are identified in the Feature Catalogue.

### 4.7.10.5 Unknow Attribute Values

When a mandatory attribute code or tag is present but the attribute value is missing, it means that the producer wishes to indicate that this attribute value is unknown.  Missing mandatory attributes must be “nilled” with a GML nilReason attribute giving the reason for omission.

Optional attributes must be omitted altogether if the value is unknown or missing. They must not be “nilled.”

### 4.7.10.6 Structure of dataset files

### 4.7.10.6.1 Sequence of dataset files

The order of data objects in each dataset file is described below:

Dataset Identification Information

Dataset structure information

Spatial records for by-reference geometries

Point

Multi point

Curve

Composite Curve

Surface

Information objects

Feature objects (Geometry may be encoded inline or by reference.)

Meta features

Geo features

### 4.7.10.7 Numeric Attribute Encoding

Floating point and integer attribute values must not contain leading zeros.  Floating point attribute values must not contain non-significant trailing zeros.

### 4.7.10.8 Object identifiers

The “name” of feature records must provide a unique world-wide identifier of feature records. The “name” of the record is the combination of the subfields agency, featureObjectIdentifier, and featureIdentificationSubdivision elements of the featureObjectIdentifier element of the object.

Features, information types, collection objects, meta features, and geometries (inline or external) are all required by the schema to have a gml:id attribute with a value that is unique within the dataset. The gml:id values must be used as the reference for the object from another object in the same dataset or another dataset.

### 4.7.10.9 Dataset validation

Fields may be repeated or omitted as permitted by the XML schemas and the validation tests. Since XML schema cannot encode rules for conditional presence or attributes, these rules are checked by Schematron rules or other validation code.

### 4.7.10.10 Location of Data Product Format schema Files

The schema file will provide a future location and is available from that location.

### 4.7.10.11 Detailed documentation of Pilot Service Request XML Schema

See DCEG for this section.

### 4.7.11 data product delivery

This clause specifies the encoding and delivery mechanisms for a Pilot Service Request dataset.  Data which conforms to this product specification must be delivered by means of an exchange set.

### 4.7.11.1 Exchange set

Pilot Service Request datasets are grouped into exchange sets. Each exchange set consists of one or more Pilot Service Request datasets with an associated XML metadata file and a single Exchange Catalogue XML file containing metadata. It may also include one or more support files.

Units of Delivery:				Exchange Set

Transfer Size:					Unlimited

Medium Name:					Digital data delivery

Other Delivery Information:

Each dataset must be contained in a physically separate, uniquely identified file on the transfer medium.

Each exchange set has a single exchange catalogue which contains the discovery metadata for each dataset and references to any support files.

Support files are supplementary information which are linked to the features and information types by attributes. The attributes containing these links are described in the application schema and feature catalogue.

An exchange set may be encapsulated into a form suitable for transmission by a mapping called a transmission encoding. An encoding translates each of the elements of the exchange set into a logical form suitable for writing to media and for transmission online. An encoding may also define other elements in addition to the exchange set contents (i.e., media identification, data extents etc…) and also may define commercial constructs such as encryption and compression methods.

If the data is transformed (e.g., for encryption or compression purposes) its content must not be changed.

This product specification does not define the transmission encoding which must be used as a default for transmission of data between parties.

The exchange set elements are as follows:

Mandatory Elements

- Pilot Service Request datasets – GML encoding of features/attributes and their associated geometry and metadata.

- Exchange Catalogue – the XML encoded representation of exchange set catalogue features [discovery metadata].

Optional Elements

- Supplementary files – These are contained within the exchange set as files and the map from the name included within the dataset and the physical location on the media is defined within the Exchange Catalogue.

- Feature Catalogue – If it is necessary to deliver the latest feature catalogue to the end user it may be done using the Pilot Service Request exchange set mechanism for datasets – i.e., include the updated feature catalogue in an exchange set.

### 4.7.11.2 Dataset

### 4.7.11.2.1 Datasets

Datasets are distributed as files which are part of exchange sets structured as described in this specification. The distribution media are left to the discretion of the producer and distributor.

The following types of dataset files may be produced and contained within an exchange set:

- New dataset and new edition of a dataset (base dataset):  Each new edition of a dataset must have the same name as the dataset that it replaces.  A new edition can also be AtoN data that has previously been produced for this area.

### 4.7.11.2.2 Dataset size

Datasets must not exceed 1 MB.

### 4.7.11.2.3 Dataset file naming

Dataset files shall be named

CCXXXXXXXX_EEE.GML

The file name forms a unique identifier where:

- CC - the first two characters identify the issuing agency (mandatory).

- The third to tenth characters are optional and may be used in any way by the producer to provide the unique file name. The following characters are allowed in the dataset name, A to Z, 0 to 9 and the special character _ (underscore).

- EEE – new editions and re-issues use 000, and increment until a limit of 999 (mandatory).

- The maximum number of characters preceding _EEE is ten.

- GML – the character sequence “GML” or “gml”.

### 4.7.11.2.4 New Editions and re-issues

This section defines the sequencing of datasets for New Editions. In order to ensure that feature type updates are incorporated into an SYSTEM in the correct sequence without any omission, a number of parameters encoded in the data header and/or metadata are used in the following way:

| edition number | when a dataset is initially created, the edition number 1 is assigned to it. The edition number is increased by 1 at each new edition. |
| --- | --- |

### 4.7.11.2.5 Cancellations

Dataset cancellation is not considered I this product specification. In the SMART Navigation project, the system automatically cancels the dataset after 7 days from the date the dataset is received.

### 4.7.11.3 Support Files

N/A (Support ifle is not included in this model)

### 4.7.11.4 Exchange Catalogue

N/A (Exchange Catalogue is not used in SMART Navigation)

### 4.7.12 meta data

### 4.7.12.1 Introduction

The Pilot Service Request metadata description is based on the S-100 metadata document section, which is a profile of the ISO 19115 standard. These documents provide a structure for describing digital geographic data and define metadata elements, a common set of metadata terminology, definitions and extension procedures.

Two metadata packages are described in this product specification: dataset metadata and exchange set metadata.

Figure 17 Metadata packages

Note 1: Types with CI_, EX_, and MD_ prefixes are from packages defined in ISO 19115 and adapted by S-100. Types with S100_ prefix are from packages defined in S-100.

Note 2: When a dataset is terminated, the purpose metadata field is set to 3 (terminated), and the editionNumber metadata field is set to 0. All other metadata fields must be blank.

### 4.7.12.2 Dataset metadata

Dataset metadata is intended to describe information about a dataset. It facilitates the management and exploitation of data and is an important requirement for understanding the characteristics of a dataset. Whereas dataset metadata is usually fairly comprehensive, there is also a requirement for a constrained subset of metadata elements that are usually required for discovery purposes. Discovery metadata are often used for building web catalogues, and can help users determine whether a product or service is fit for purpose and where they can be obtained.

Table 25 Dataset metadata

| Name | Cardinality | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| S100_DataSetDiscoveryMetadata |  |  |  |  |
| fileName | 1 |  | CharacterString | Dataset file name |
| filePath | 1 |  | CharacterString | Full path from the exchange set root directory |
| description | 1 |  | CharacterString | Short description of the area covered by dataset, e.g., area, harbour, or port name, between two named locations etc. |
| dataProtection | 0..1 | {1} or {2} | Boolean | True : Encrypted False : Unencrypted |
| protectionScheme | 0..1 |  | CharacterString |  |
| digitalSignature | 0..1 |  | CharacterString |  |
| digitalSignatureReference | 0..1 |  | CharacterString |  |
| digitalSignatureValue | 0..1 |  | CharacterString |  |
| copyright | 0..1 |  | MD_LegalConstraints>MD_RestrictionCode <copyright> (ISO 19115-1) |  |
| classification | 0..1 |  | Class MD_SecurityConstraints>MD_ClassificationCode (codelist) ISO 19115-1 | 1. unclassified 2. restricted 3. confidential 4. secret 5. top secret |
| purpose | 1 | {1}, {2} | CharacterString | - New dataset - New edition |
| specificUsage | 1 |  | MD_USAGE>specificUsage (character string) MD_USAGE>userContactInfo (CI_Responsibility) | brief description of the resource and/or resource series usage |
| editionNumber | 1 | {1} | Integer | When a dataset is initially created, the edition number “1” is assigned to it. The edition number is increased by one with each new edition. |
| updateNumber | 1 |  | CharacterString | Update number 0 is assigned to a new dataset. |
| updateApplicationDate | 1 |  | Date | All updates dated on or before this date must have been applied by the producer. |
| issueDate | 1 |  | Date | Date on which the dataset was generated. |
| productSpecification | 1 |  | S100_ProductSpecification | See Notes below this table for constraints on values. |
| producingAgency | 1 |  | CI_Responsibility>CI_Organisation or CI_Responsibility>CI_Individual | Party responsible for generating the dataset. |
| optimumDisplayScale | 0..1 |  | Integer |  |
| maximumDisplayScale | 0..1 |  | Integer |  |
| minimumDisplayScale | 0..1 |  | Integer |  |
| horizontalDatumReference | 1 | EPSG | CharacterString |  |
| horizontalDatumValue | 1 | 4326 | Integer | WGS84 |
| verticalDatum | 0..1 |  | S100_VerticalAndSoundingDatum | Not included in Pilot Service Request |
| soundingDatum | 0..1 |  | S100_VerticalAndSoundingDatum | Not included in Pilot Service Request |
| dataType | 1 | GML | CharacterString | The only value allowed is “GML”. |
| otherDataTypeDescription | 0..1 |  | CharacterString |  |
| dataTypeVersion | 1 | 3.2.1 | CharacterString |  |
| dataCoverage | 1..* |  | S100_DataCoverage |  |
| comment | 0..1 |  | CharacterString | Any additional Information |
| layerID | 1..* | S-101 | CharacterString | Dataset must be used with ENC in an ECDIS. |

### 4.7.12.3 Update Dataset Metadata

N/A (Dataset update is not relevant in this model)

### 4.7.12.4 Support file metadata

N/A (Support file is not included in this model)

### 4.7.12.5 Exchange set metadata

N/A (Exchange Catalogue is not used in SMART Navigation)

### 4.7.12.6 Catalogue file Metadata

All Pilot Service Request Exchange Catalogue files must contain at least the mandatory metadata elements.

Table 26 Catalogue file metadata

| Name | Multiplicity | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| S100_ExchangeCatalogue |  |  |  |  |
| identifier | 1 |  | S100_CatalogueIdentifier |  |
| contact | 1 |  | S100_CataloguePointOfContact |  |
| productSpecification | 0..1 |  | S100_ProductSpecification | Conditional on all the datasets using the same product specification.. |
| metadataLanguage | 1 | English | CharacterString | All datasets conforming to this PS must use English language. A catalogue in English must be provided. Discovery metadata elements within catalogues have their own locale attributes and may be repeated in languages other than English. |
| exchangeCatalogueName | 1 |  | CharacterString | Catalogue filename |
| exchangeCatalogueDescription | 1 |  | CharacterString |  |
| exchangeCatalogueComment | 0..1 |  | CharacterString | Any additional Information |
| compressionFlag | 0..1 |  | Boolean | Yes or No |
| sourceMedia | 0..1 |  | CharacterString |  |
| replacedData | 0..1 |  | Boolean |  |
| dataReplacement | 0..1 |  | CharacterString |  |

## 4.8 pilot shcedule

### 4.8.1 Overview

### 4.8.1.1 Introduction

This document has been produced by the member of SMART-Navigation project team(Korean Register, Marine-Works) in response to a requirement to produce a data product that can be used as a pilot information including tug vessel within an Portable Pilot Unit(PPU). It is based on the IHO S-100 framework specification and the ISO 19100 series of standards. It is a textTable product specification that is primarily intended for encoding the pilot & tug work schedule, tugboat information, pilot note, pilot card information.

### 4.8.1.2 References

IHO S-100 IHO Universal Hydrographic Data Model Edition 4.0.0 (December 2017).

ISO 8601. 2004. Data elements and interchange formats - Information interchange - Representation of dates and times. 2004.

ISO 3166-1. 1997. Country Codes. 1997.

ISO 10646-1 Information technology - Universal Multiple-Octet Coded Character Set (UCS) - Part 1: Architecture and Basic Multilingual Plane

ISO 19101-2:2008 Geographic Information - Rules for Application Schema

ISO/TS 19103:2005 Geographic Information - Conceptual schema language

ISO 19106:2004 Geographic Information – Profiles

ISO 19107:2003 Geographic Information – Spatial schema

ISO 19109:2005 Geographic Information - Rules for Application Schema

ISO 19111:2003 Geographic Information - Spatial referencing by coordinates

ISO 19115:2003+Corr1 (2006) Geographic Information - Metadata

ISO 19115-2:2009 Geographic Information - Metadata: Extensions for imagery and gridded data

ISO 19123:2005 Geographic Information - Schema for coverage geometry and functions

ISO 19129:2009 Geographic Information - Imagery gridded and coverage data framework

ISO 19131:2007 Geographic Information - Data product specifications

ISO 19136:2007 Geographic Information – Geography Markup Language

ISO 19136-2:2015, Geographic Information – Geography Markup Language.

ISO/TS 19139, Geographic Information – Metadata – XML schema implementation.

### 4.8.1.3 Terms, definitions and abbreviations

Terms and Definitions

The S-100 framework is based on the ISO 19100 series of geographic standards. The terms and definitions provided here are used to standardize the nomenclature found within that framework, whenever possible. They are taken from the references cited in clause 2.1. Modifications have been made when necessary.

- Application

Manipulation and processing of data in support of user requirements (ISO 19101)

- Application schema

Conceptual schema for data required by one or more applications (ISO 19101)

- Conceptual model

Model that defines concepts of a universe of discourse (ISO 19101)

- Conceptual schema

Formal description of a conceptual model (ISO 19101)

- Coverage

Feature that acts as a function to return values from its range for any direct position within its spatial, temporal or spatiotemporal domain (ISO 19123)

- Data product

Dataset or dataset series that conforms to a data product specification

- Data product specification

Detailed description of a dataset or dataset series together with additional information that will enable it to be created, supplied to and used by another party

- Dataset

Identifiable collection of data (ISO 19115)

- Feature

Abstraction of real world phenomena (ISO 19101)

- Feature association

Relationship that links instances of one feature type with instances of the same or a different feature type (ISO19110)

- Feature attribute

Characteristic of a feature (ISO 19101)

- Geographic data

Data with implicit or explicit reference to a location relative to the Earth (ISO 19109)

- Metadata

Data about data (ISO 19115)

- Model

Abstraction of some aspects of reality (ISO 19109)

- Portrayal

Presentation of information to humans (ISO 19117)

- Quality

Totality of characteristics of a product that bear on its ability to satisfy stated and implied needs (ISO 19101)

Abbreviations

This product specification adopts the following convention for symbols and abbreviated terms:

- ASCII 	American Standard Code for Information Interchange

- ECDIS	Electronic Chart Display and Information Systems

- ECS	Electronic Chart Systems

- ENC	Electronic Navigational Chart

- GML	Geography Markup Language

- IHO	International Hydrographic Organization

- IOC	International Oceanographic Commission

- ISO	International Organization for Standardization

- UML	Unified Modelling Language

- URI	Uniformed Resource Identifier

- URL 	Uniform Resource Locator

- WMS	Web Map Service

- WFS	Web Feature Service

- www 	World Wide Web

- XML	Extensible Markup Language

- XSLT	eXtensible Stylesheet Language Transformations

- PPU	Portable Pilot Unit

### 4.8.1.4 General Data Product Description

Informal Description of Data Product

This clause contains general information about the data product

- Title:  	Pilot Schedule

- Abstract: 	Pilot schedule product specification describe the means to express pilot information including tug work schedule for towing such as pilot & tug work schedule, tug boat information, pilot note, pilot card information. This may include details on pilotage date, pilot opinion, port authority code, pilot service area, pilotage request time.

- Content:	Datasets conforming to this specification will contain response information for pilot request in the Pilot & Tug Assistance service. Additionally, there will be relevant metadata quality, production authority, data sources and publication date.

- Spatial Extent:	Korean Coast

- Specific Purpose:	Describing pilot information including tug vessel for utilizing the Portable Pilot Unit(PPU)

### 4.8.1.5 Data product specification metadata

This information uniquely identifies this Product Specification and provides information about its creation and maintenance. For further information on dataset metadata see the metadata clause.

- Title: 	Pilot Schedule

- S-100 Version: 	4.0.0

- Pilot Schedule Version:	1.0.0

- Date: 	2019-03-08

- Language: 	English

- Classification: 	Unclassified

- Contact: 	32, Yuseong-daero 1312beon-gil, Yuseong-gu, Daejeon, Korea 32(Jang Dong) inside KRISO

- URL: 	http://www.smart-navigation.org

- Identifier: 	Pilot Schedule

- Maintenance: 	Amendments to this specification will be produced on a needs basis. For reporting issues with this specification which need correction, use the contact information.

### 4.8.2 Specification Scopes

This product specification describes one data product and therefore requires only one scope which is described below:

- Scope ID:  		Pilot Schedule datasets.

- Hierarchical level: 		MD_ScopeCode - 005

- Hierarchical level name: 	dataset.

- Level description:		information applies to the dataset

- Extent:	EX_Extent.description: Korean Coast

### 4.8.3 Dataset identification

This section describes how to identify data sets that conform to the specification. A dataset that conforms to this Product Specification may be identified by its discovery metadata as defined in clause 12 of this specification. The information identifying the data product may include the following items from S-100 4.0.0 clause 11-6 (adapted from ISO 19115).

| Title | Pilot Schedule |
| --- | --- |
| Abstract | Pilot Schedule dataset is a textTable dataset containing pilot schedule & comment and tug particular regarding Pilot & Tug assistance service the within a defined geographical area. |
| Acronym | PS |
| Content | Berthing direction, Contact information, Pilot information, Pilot service area, Port authority code, Pilotage Date, Pilot Opinion |
| Topic category | Transportation |
| geographicDescription | Area specific to marine navigation within korean sea area |
| Purpose | Standardization of pilot relevant information and tugboat information |
| Language | EN Additional values if any use CharacterString values from ISO 639-2 |

### 4.8.4 Data Content and structure

### 4.8.4.1 Introduction

The Pilot Schedule product is based on the S-100 General Feature Model (GFM), and is a feature-based vector product. All Pilot Schedule features and information classes are derived from one of the abstract classes FeatureType and InformationType defined in the Pilot Schedule respectively.

This clause contains the Application Schema expressed in UML and an associated Feature Catalogue.

### 4.8.4.2 Application Schema

The UML models in this clause are segments of the overall Pilot Schedule application schema, and include overviews of the feature classes, information classes, meta features, spatial types, and the relationships between them.

This clause contains a general overview of the classes and relationships in the Pilot Schedule application schema. Detailed information about how to use the feature types and information types to encode Pilot Schedule information is provided in the SMART-Navigation Data Classification and Encoding Guide (DCEG).

The following conventions are used in the UML diagrams depicting the application schema:

- Feature classes are depicted with pink background; the dark shade for abstract feature classes and the light shade for ordinary (non-abstract) feature classes.

- Complex attributes are depicted with a light yellow background.

- Enumeration lists and codelists are depicted with a green background. The numeric code corresponding to each listed value is shown to its right following an ‘=’ sign.

- No significance attaches to the colour of associations. (Complex diagrams may use different colours to distinguish associations that cross one another.)

- Subclasses inherit the attributes and associations of their superclasses at all levels, unless such inheritance is explicitly overridden in the subclass.

### 4.8.4.2.1 Domain model

Figure 18 Pilot Schedule Overview

### 4.8.4.3 Feature Catalogue

### 4.8.4.3.1 Introduction

The Feature Catalogue describes the feature types, information types, attributes, attribute values, associations and roles which may be used in the product.  The Pilot Schedule Feature Catalogue is available in an XML document which conforms to the S-100 XML Feature Catalogue Schema.  Simple attributes used in this specification are listed in below.

Name: 	Pilot Schedule Feature Catalogue

Scope: 	Smart-Navigation

Version Number: 	1.0.0

Version Date:	2019-03-15

Producer:	SMART Navigation Project

Language: 	                  English

### 4.8.4.3.1.1 Feature Types

Feature types contain descriptive attributes that characterize real-world entities. The word ‘feature’ may be used in one of two senses – feature type and feature instance. A feature type is a class and is defined in a Feature Catalogue. A feature instance is a single occurrence of the feature type and represented as an object in a dataset. A feature instance is located by a relationship to one or more spatial instances. A feature instance may exist without referencing a spatial instance.

### 4.8.4.3.1.2 Geographic

A Geographic (Geo) feature type carries the descriptive characteristics of a real-world entity.

### 4.8.4.3.1.3 Meta

Meta features contain information about other features within a dataset. Information defined by meta features override the default metadata values defined by the dataset descriptive records. Meta attribution on individual features overrides attribution on meta features.

### 4.8.4.3.1.4 Feature Relationship

A feature relationship links instances of one feature type with instances of the same or a different feature type.

### 4.8.4.3.1.5 Information Types

Information types are identifiable pieces of information in a dataset that can be shared between other features. They have attributes but have no relationship to any geometry; information types may reference other information types.

### 4.8.4.3.1.6 Attributes

SMART-Navigation product defines attributes as either simple or complex.

### 4.8.4.3.1.6.1 Simple Attributes

SMART-Navigation product uses nine types of simple attributes; they are listed in the following table:

| Type | Definition |
| --- | --- |
| Enumeration | A fixed list of valid identifiers of named literal values |
| Boolean | A value representing binary logic. The value can be either True or False. The default state for Boolean type attributes (i.e. where the attribute is not populated for the feature) is False. |
| Real | A signed Real (floating point) number consisting of a mantissa and an exponent |
| Integer | A signed integer number. The representation of an integer is encapsulation and usage dependent. |
| Text | An arbitrary-length sequence of characters including accents and special characters from a repertoire of one of the adopted character sets |
| Date | A date provides values for year, month and day according to the Gregorian Calendar. Character encoding of a date is a string which must follow the calendar date format (complete representation, basic format) for date specified by ISO 8601:1988. EXAMPLE 19980918 (YYYY-MM-DD) |
| Time | A time is given by an hour, minute and second. Character encoding of a time is a string that follows the local time (complete representation, basic format) format defined in ISO 8601:1988. EXAMPLE 183059 or 183059+0100 or 183059Z |
| Date and Time | A DateTime is a combination of a date and a time type. Character encoding of a DateTime shall follow ISO 8601:1988 EXAMPLE 19850412T101530 |
| Codelist | A type of flexible enumeration. A code list type is a list of literals which may be extended only in conformance with specified rules. |

### 4.8.4.3.1.6.2 Complex Attributes

Complex attributes are aggregations of other attributes that are either simple or complex. The aggregation is defined by means of attribute bindings.

### 4.8.4.3.1.6.3 Geometric Representation

Geometric representation is the digital description of the spatial component of an object as described in S-100 and ISO 19107. This product specification uses no geometry attribute.

### 4.8.4.3.2 Feature Catalogue General

The Pilot Schedule feature catalogue describes the feature types, information types, attributes, attribute values, associations and roles which may be used in a Pilot Schedule data set.

### 4.8.4.3.2.1 Feature Types

- Geographic

- Meta

- PilotScheduleIdentification

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| shipMRN | MRN is a naming scheme that can uniquely identify any maritime resource on a global scale. shipMRN is the unique value given to the ship. | URN | 1 | mandatory |  |
| userMRN | userMRN is a unique value assigned to the user who is logged in to use the SV51 Pilot Schedule service. | URN | 0..1 | option |  |
| IMONo | The seven-digit number assigned to the vessel, the vessel management company, and the ship documents company under the SOLAS rules. | integer | 0..1 | option |  |
| callSign | The designated call-sign of a radio station. | text | 0..1 | option |  |
| MMSICode | The Maritime Mobile Service Identity (MMSI) Code is formed of a series of nine digits which are transmitted over the radio path in order to uniquely identify ship stations, ship earth stations, coast stations, coast earth stations, and group calls. These identities are formed in such a way that the identity or part thereof can be used by telephone and telex subscribers connected to the general telecommunications network principally to call ships automatically. (Adapted from Appendix 43 of the International Telecommunications Union Radio Regulations). | text | 0..1 | option |  |

- PilotWork

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| pilot | - | complex attribute | 1 | mandatory |  |
| pilotServiceArea | - | complex attribute | 0..* | option |  |

- PilotAndTugWorkSchedule

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| pilotageRequestNo | Pilotage Request No | text | 1 | mandatory |  |
| requestStatus | Pilotage Request Status | enumeration | 1 | mandatory |  |
| pilotageRequestTime | Pilotage Request Time | dateTime | 1 | mandatory |  |
| berthingDirection | Direction of ship 's berthing. | enumeration | 1 | mandatory |  |
| lineHanding | Name of the line Handing company | text | 0..1 | option |  |
| pilotRequestRemark | - | text | 0..* | option |  |

- TugBoatInformation

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| callSignOfTugBoat | Call sign of tugBoat | text | 0..1 | option |  |
| MMSICodeOfTugBoat | MMSI Code of tugBoat | integer | 0..1 | option |  |
| maximumTowingForce | The maximum force that can be generated through the line between the Tug and Vessel, when the Tug is towing. | real | 0..1 | option |  |
| theCountryOf Manufacture | Producer of tugboat | ISO 3166-2 | 0..1 | option |  |
| tugBoatName | - | complex attribute | 1..* | mandatory |  |
| vesselMeasurement | Vessel Measurement of TugBoat | complex attribute | 0..* | option |  |
| mainEngine | Main Engine Characteristics of TugBoat | complex attribute | 1 | mandatory |  |

- PilotNote

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| portAuthorityCode | Port Authority Code | text | 1 | mandatory |  |
| pilotageDate | - | dateTime | 1 | mandatory |  |
| pilotOpinion | - | text | 0..* | option |  |

- PilotCardInformation

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| pilotCardOtherInformation | - | complex attribute | 1 | mandatory |  |
| anchor | - | complex attribute | 0..* | option |  |
| navigationEquipmentCondition | - | complex attribute | 0..1 | option |  |
| pilotCardImageURL | - | URL | 0..1 | option |  |

- Cartographic

- Theme

### 4.8.4.3.2.2 Feature Relationship

| Role Type | Association Name | Role | Feature | Multiplicity |
| --- | --- | --- | --- | --- |
| Association | additionalInformation | informationProvideFor | TugBoatInformation | 1 |
| Association | additionalInformation | provideInformation | PilotAndTugWorkSchedule | 0..* |

### 4.8.4.3.2.3 Information Relationship

- N/A

### 4.8.4.3.2.4 Information Type

- N/A

### 4.8.4.3.2.5 Simple Attribute

- Simple Attribute List

| Name | Description | Data Type | Remarks |
| --- | --- | --- | --- |
| pilotName | Pilot Name | text |  |
| pilotCertificateNo | Pilot Certificate No | text |  |
| pilotDistrict | - | text |  |
| pilotDistrictCategory | - | enumeration |  |
| powerValue | - | real |  |
| powerUnit | - | enumeration |  |
| typeOfMainEngine | - | text |  |
| berthingDirection | Direction of ship 's berthing. | enumeration |  |
| anchorChainCount | Anchor Chain Count | integer |  |
| shacklesDefinition | Shackles Definition | real |  |
| abnormalAnchors | - | text |  |
| anchorsClearedAway | - | boolean |  |
| radarType | Radar Type | enumeration |  |
| abnormalRadar | Abnormal Radar | text |  |
| radarARPA | Radar ARPA | boolean |  |
| vesselsCharacteristics | - | enumeration |  |
| vesselsCharacteristicsValue | - | real |  |
| vesselsCharacteristicsUnits | - | enumeration |  |
| equipmentOperationalDefects | - | text |  |
| numberOfThruster | - | integer |  |
| numberOfRudder | - | integer |  |
| numberOfPropellers | - | integer |  |
| otherImportantDetails | Other Important Details | text |  |
| mastersName | Masters Name | text |  |
| issueDate | Issue Date | date |  |
| whistles | Whistles | text |  |
| speedLog | Speed Log | text |  |
| electronicPositionFixing | Electronic Position Fixing | text |  |
| electronicPositionFixingType | Electronic Position Fixing Type | text |  |
| compassSystem | Compass System | text |  |
| compassSystemError | Compass System Error | real |  |
| numberOfPowerUnitsInUse | Number Of Power Units In Use | integer |  |
| engineTelegraphs | Engine Telegraphs | text |  |
| VHFEquipment | - | text |  |
| mooringWinchesAndLines | - | text |  |
| steeringGear | - | text |  |
| rudderIndicators | - | text |  |
| RPMPitchIndicatos | - | text |  |
| rateOfTurnIndicator | - | text |  |
| displayName | A statement expressing if a feature name is to be displayed in certain display settings or not. Indication: Boolean. A True value is an indication that the name is intended to be displayed. Remarks: Where it is allowable to encode multiple instances of feature name for a single feature instance, only one feature name instance can indicate that the name is to be displayed (display name set to True). | boolean |  |
| language | The language is encoded by a character code following ISO 639-3 | ISO639-3 |  |
| name | The individual name of a feature. | text |  |
| shipMRN | MRN is a naming scheme that can uniquely identify any maritime resource on a global scale. shipMRN is the unique value given to the ship. | URN |  |
| userMRN | userMRN is a unique value assigned to the user who is logged in to use the SV51 Pilot Schedule service. | URN |  |
| IMONo | The seven-digit number assigned to the vessel, the vessel management company, and the ship documents company under the SOLAS rules. | integer |  |
| callSign | The designated call-sign of a radio station. | text |  |
| MMSICode | The Maritime Mobile Service Identity (MMSI) Code is formed of a series of nine digits which are transmitted over the radio path in order to uniquely identify ship stations, ship earth stations, coast stations, coast earth stations, and group calls. These identities are formed in such a way that the identity or part thereof can be used by telephone and telex subscribers connected to the general telecommunications network principally to call ships automatically. (Adapted from Appendix 43 of the International Telecommunications Union Radio Regulations). | text |  |
| pilotageRequestNo | Pilotage Request No | text |  |
| requestStatus | Pilotage Request Status | enumeration |  |
| pilotageRequestTime | Pilotage Request Time | dateTime |  |
| berthingDirection | Direction of ship 's berthing. | enumeration |  |
| lineHanding | Name of the line Handing company | text |  |
| pilotRequestRemark | - | text |  |
| callSignOfTugBoat | Call sign of tugBoat | text |  |
| MMSICodeOfTugBoat | MMSI Code of tugBoat | integer |  |
| maximumTowingForce | The maximum force that can be generated through the line between the Tug and Vessel, when the Tug is towing. | real |  |
| theCountryOf Manufacture | Producer of tugboat | ISO 3166-2 |  |
| portAuthorityCode | Port Authority Code | text |  |
| pilotageDate | - | dateTime |  |
| pilotOpinion | - | text |  |
| pilotCardImageURL | - | URL |  |

- requestStatus

| Value | Label | Description |
| --- | --- | --- |
| 1 | application | - |
| 2 | acknowledgement | - |
| 3 | emergency | - |
| 4 | cancellation | - |
| 5 | pilotonBoard | - |
| 6 | completion | - |

- berthingDirection

| Value | Label | Description |
| --- | --- | --- |
| 1 | unknown | - |
| 2 | starboard | - |
| 3 | portside | - |
| 4 | stern | - |
| 5 | double | - |

- pilotDistrictCategory

| Value | Label | Description |
| --- | --- | --- |
| 1 | start point | - |
| 2 | end point | - |

- powerUnit

| Value | Label | Description |
| --- | --- | --- |
| 1 | kw | - |
| 2 | hp | - |

- radarType

| Value | Label | Description |
| --- | --- | --- |
| 1 | x-Band | - |
| 2 | s-Band | - |

- vesselsCharacteristics

| Value | Label | Description |
| --- | --- | --- |
| 1 | gross tonnage | - |

- vesselsCharacteristicsUnits

| Value | Label | Description |
| --- | --- | --- |
| 1 | ton | - |

### 4.8.4.3.2.6 Complex Attribute

- pilot

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| pilotName | Pilot Name | text | 1 | mandatory |  |
| pilotCertificateNo | Pilot Certificate No | text | 0..1 | option |  |

- pilotServiceArea

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| pilotDistrict | - | text | 1 | mandatory |  |
| pilotDistrictCategory | - | enumeration | 1 | mandatory |  |

- mainEnginePower

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| powerValue | - | real | 1 | mandatory |  |
| powerUnit | - | enumeration | 0..1 | option |  |

- mainEngine

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| typeOfMainEngine | - | text | 0..1 | option |  |
| mainEnginePower | - | complex attribute | 1..* | mandatory |  |

- anchorChain

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| berthingDirection | Direction of ship 's berthing. | enumeration | 1 | mandatory |  |
| anchorChainCount | Anchor Chain Count | integer | 1 | mandatory |  |
| shacklesDefinition | Shackles Definition | real | 0..1 | option |  |

- anchor

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| anchorChain | - | complex attribute | 1..* | mandatory |  |
| abnormalAnchors | - | text | 0..1 | option |  |
| anchorsClearedAway | - | boolean | 0..1 | option |  |

- radarEquipmet

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| radarType | Radar Type | enumeration | 1 | mandatory |  |
| abnormalRadar | Abnormal Radar | text | 1 | mandatory |  |
| radarARPA | Radar ARPA | boolean | 0..1 | option |  |

- vesselMeasurement

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| vesselsCharacteristics | - | enumeration | 0..1 | option |  |
| vesselsCharacteristicsValue | - | real | 1 | mandatory |  |
| vesselsCharacteristicsUnits | - | enumeration | 0..1 | option |  |

- pilotCardOtherInformation

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| equipmentOperationalDefects | - | text | 0..1 | option |  |
| numberOfThruster | - | integer | 0..1 | option |  |
| numberOfRudder | - | integer | 0..1 | option |  |
| numberOfPropellers | - | integer | 0..1 | option |  |
| otherImportantDetails | Other Important Details | text | 0..1 | option |  |
| mastersName | Masters Name | text | 1 | mandatory |  |
| issueDate | Issue Date | date | 1 | mandatory |  |

- navigationEquipmentCondition

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| whistles | Whistles | text | 0..1 | option |  |
| speedLog | Speed Log | text | 0..1 | option |  |
| electronicPositionFixing | Electronic Position Fixing | text | 0..1 | option |  |
| electronicPositionFixingType | Electronic Position Fixing Type | text | 0..1 | option |  |
| compassSystem | Compass System | text | 0..1 | option |  |
| compass systemError | Compass System Error | real | 0..1 | option |  |
| numberOfPowerUnitsInUse | Number Of Power Units In Use | integer | 0..1 | option |  |
| engineTelegraphs | Engine Telegraphs | text | 0..1 | option |  |
| VHFEquipment | - | text | 0..1 | option |  |
| mooringWinchesAndLines | - | text | 0..1 | option |  |
| steeringGear | - | text | 0..1 | option |  |
| rudderIndicators | - | text | 0..1 | option |  |
| RPMPitchIndicatos | - | text | 0..1 | option |  |
| rateOfTurnIndicator | - | text | 0..1 | option |  |
| radarEquipmet | - | complex attribute | 0..2 | option |  |

- tugBoatName

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| displayName | A statement expressing if a feature name is to be displayed in certain display settings or not. Indication: Boolean. A True value is an indication that the name is intended to be displayed. | boolean | 0..1 | option |  |
| language | The language is encoded by a character code following ISO 639-3 | ISO639-3 | 0..1 | option |  |
| name | The individual name of a feature. | text | 1 | mandatory |  |

### 4.8.5 Coordinate Reference System(CRS)

N/A (This model has no geometry attribute)

### 4.8.6 Data Quality

### 4.8.6.1 Introduction

Pilot Schedule products must be tested with the Pilot Schedule specific checks prior to release by the data producer. The data producer must review the check results and address any issues to ensure sufficient quality of the data products. The checks are a mix of data format validation checks, conformance to standard checks and logical consistency checks. But the checklist has not been written yet. It will be updated in the next version.

### 4.8.7 Data Capture and classification

See DCEG for this section..

### 4.8.8 maintenance

### 4.8.8.1 Introduction

Datasets are maintained as needed and must include mechanisms for Pilot Schedule updating. Data updates will be made by new editions. The maintenance and update frequency of Pilot Schedule datasets should be defined by the producers (official national authority) implementing this specification.

Data Producers must use applicable sources to maintain and update data and provide a brief description of the sources that were used to produce the dataset in the appropriate metadata field.

The data product shall provide information on how the data is maintained and should describe the principles and criteria applied in maintenance regime. This should specify the expected frequency of updates.

Table 27 Maintenance and update frequency

| Item Name | Description | Multiplicity | Type |
| --- | --- | --- | --- |
| maintenanceAndUpdateFrequency | Frequency with which changes and additions are made to the data product (per update scope) | 1..* | MD_MaintenanceInformation (ISO 19115) |
| dataSource | Identification of the kinds of data sources usable to produce datasets | 1..* | LI_Source (ISO 19115) |
| productionProcess | Textual description of the production process applicable to the datasets (per scope or data source) | 1..* | LI_ProcessStep (ISO 19115) |

### 4.8.8.2 Production process for base and update datasets

Data Producers should follow their established production processes for maintaining and updating datasets. Data is produced against the DCEG and checked against the appropriate set of validation rules in Appendix E. (Appendix E is not included in this release.)

### 4.8.8.3 Datasetr updates

N/A (Dataset update is not relevant in this model)

### 4.8.8.4 Support file updates

N/A (Support file is not included in this model)

### 4.8.8.5 Feature and portrayal catalogues

For each new version of the Pilot Schedule Product Specification a new feature catalogue will be released. The system must be able to manage datasets and their catalogues that are created on different versions of the Pilot Schedule product specification (Portrayal Catalogue is not defined in this model).

### 4.8.8.6 Feature history, versions, and change tracking

If applications or production systems require versioning of individual instances of feature or information types, maintenance of histories, or change tracking, the methods for versioning, history management, and change tracking and display are left to the application or production system.

### 4.8.8.7 Dataset encryption

Details about dataset encryption are still to be determined, and may mirror the method described in S-101.

### 4.8.9 Portrayal

Portrayal is not defined in this version of Pilot Schedule Product Specifications. Users are free to choose the means and methodology of portrayal as they see best suited for their needs. It should be noted that future versions of Pilot Schedule may include a portrayal catalogue, and any implementer should therefore anticipate this, and make sufficient provisions in any system supporting Pilot Schedule.

### 4.8.10 data product format(encoding)

### 4.8.10.1 Introduction

The GML encoding of Pilot Schedule datasets is based on the S-100 profile of GML 3.2.1.

Format Name: Pilot Schedule GML

Version:	1.0

Character Set: UTF-8

Specification: SMART-Navigation Product Specification

### 4.8.10.2 Encoding of Latitude and Longitude

N/A (This model has no geometry attribute)

### 4.8.10.3 Text Attribute

Character strings must be encoded using the character set defined in ISO 10646-1, in Unicode Transformation Format-8 (UTF-8).

### 4.8.10.4 Mandatory Attribute Values

There are four reasons why attribute values may be considered mandatory:

- They determine whether a feature is in the display base,

- Certain features make no logical sense without specific attributes,

- Some attributes are necessary to determine which symbol is to be displayed,

- Some attributes are required for safety of navigation.

All mandatory attributes are identified in the Feature Catalogue.

### 4.8.10.5 Unknown Attribute Values

When a mandatory attribute code or tag is present but the attribute value is missing, it means that the producer wishes to indicate that this attribute value is unknown.  Missing mandatory attributes must be “nilled” with a GML nilReason attribute giving the reason for omission.

Optional attributes must be omitted altogether if the value is unknown or missing. They must not be “nilled.”

### 4.8.10.6 Structure of dataset files

### 4.8.10.6.1 Sequence of dataset files

The order of data objects in each dataset file is described below:

Dataset Identification Information

Dataset structure information

Spatial records for by-reference geometries

Point

Multi point

Curve

Composite Curve

Surface

Information objects

Feature objects (Geometry may be encoded inline or by reference.)

Meta features

Geo features

### 4.8.10.7 Numeric Attribute Encoding

Floating point and integer attribute values must not contain leading zeros.  Floating point attribute values must not contain non-significant trailing zeros.

### 4.8.10.8 Object identifiers

The “name” of feature records must provide a unique world-wide identifier of feature records. The “name” of the record is the combination of the subfields agency, featureObjectIdentifier, and featureIdentificationSubdivision elements of the featureObjectIdentifier element of the object.

Features, information types, collection objects, meta features, and geometries (inline or external) are all required by the schema to have a gml:id attribute with a value that is unique within the dataset. The gml:id values must be used as the reference for the object from another object in the same dataset or another dataset.

### 4.8.10.9 Dataset validation

Fields may be repeated or omitted as permitted by the XML schemas and the validation tests. Since XML schema cannot encode rules for conditional presence or attributes, these rules are checked by Schematron rules or other validation code.

### 4.8.10.10 Location of Data Product Format schema Files

The schema file will provide a future location and is available from that location.

### 4.8.10.11 Detailed documentation of Pilot Schedule XML Schema

See DCEG for this section.

### 4.8.11 data product delivery

This clause specifies the encoding and delivery mechanisms for a Pilot Schedule dataset.  Data which conforms to this product specification must be delivered by means of an exchange set.

### 4.8.11.1 Exchange set

Pilot Schedule datasets are grouped into exchange sets. Each exchange set consists of one or more Pilot Schedule datasets with an associated XML metadata file and a single Exchange Catalogue XML file containing metadata. It may also include one or more support files.

Units of Delivery:				Exchange Set

Transfer Size:					Unlimited

Medium Name:					Digital data delivery

Other Delivery Information:

Each dataset must be contained in a physically separate, uniquely identified file on the transfer medium.

Each exchange set has a single exchange catalogue which contains the discovery metadata for each dataset and references to any support files.

Support files are supplementary information which are linked to the features and information types by attributes. The attributes containing these links are described in the application schema and feature catalogue.

An exchange set may be encapsulated into a form suitable for transmission by a mapping called a transmission encoding. An encoding translates each of the elements of the exchange set into a logical form suitable for writing to media and for transmission online. An encoding may also define other elements in addition to the exchange set contents (i.e., media identification, data extents etc…) and also may define commercial constructs such as encryption and compression methods.

If the data is transformed (e.g., for encryption or compression purposes) its content must not be changed.

This product specification does not define the transmission encoding which must be used as a default for transmission of data between parties.

The exchange set elements are as follows:

Mandatory Elements

- Pilot Schedule datasets – GML encoding of features/attributes and their associated geometry and metadata.

- Exchange Catalogue – the XML encoded representation of exchange set catalogue features [discovery metadata].

Optional Elements

- Supplementary files – These are contained within the exchange set as files and the map from the name included within the dataset and the physical location on the media is defined within the Exchange Catalogue.

- Feature Catalogue – If it is necessary to deliver the latest feature catalogue to the end user it may be done using the Pilot Sechedule exchange set mechanism for datasets – i.e., include the updated feature catalogue in an exchange set.

### 4.8.11.2 Dataset

### 4.8.11.2.1 Datasets

Datasets are distributed as files which are part of exchange sets structured as described in this specification. The distribution media are left to the discretion of the producer and distributor.

The following types of dataset files may be produced and contained within an exchange set:

- New dataset and new edition of a dataset (base dataset):  Each new edition of a dataset must have the same name as the dataset that it replaces.  A new edition can also be AtoN data that has previously been produced for this area.

### 4.8.11.2.2 Dataset size

Datasets must not exceed 1 MB.

### 4.8.11.2.3 Dataset file naming

Dataset files shall be named

CCXXXXXXXX_EEE.GML

The file name forms a unique identifier where:

- CC - the first two characters identify the issuing agency (mandatory).

- The third to tenth characters are optional and may be used in any way by the producer to provide the unique file name. The following characters are allowed in the dataset name, A to Z, 0 to 9 and the special character _ (underscore).

- EEE – new editions and re-issues use 000, and increment until a limit of 999 (mandatory).

- The maximum number of characters preceding _EEE is ten.

- GML – the character sequence “GML” or “gml”.

### 4.8.11.2.4 New Editions and re-issues

This section defines the sequencing of datasets for New Editions. In order to ensure that feature type updates are incorporated into an SYSTEM in the correct sequence without any omission, a number of parameters encoded in the data header and/or metadata are used in the following way:

| edition number | when a dataset is initially created, the edition number 1 is assigned to it. The edition number is increased by 1 at each new edition. |
| --- | --- |

### 4.8.11.2.5 Cancellations

Dataset cancellation is not considered I this product specification. In the SMART Navigation project, the system automatically cancels the dataset after 7 days from the date the dataset is received.

### 4.8.11.3 Support Files

N/A (Support file is not included in this model)

### 4.8.11.4 Exchange Catalogue

N/A (Exchange Catalogue is not used in SMART Navigation)

### 4.8.12 meta data

### 4.8.12.1 Introduction

The Pilot Schedule metadata description is based on the S-100 metadata document section, which is a profile of the ISO 19115 standard. These documents provide a structure for describing digital geographic data and define metadata elements, a common set of metadata terminology, definitions and extension procedures.

Two metadata packages are described in this product specification: dataset metadata and exchange set metadata.

Figure 19 Metadata packages

Note 1: Types with CI_, EX_, and MD_ prefixes are from packages defined in ISO 19115 and adapted by S-100. Types with S100_ prefix are from packages defined in S-100.

Note 2: When a dataset is terminated, the purpose metadata field is set to 3 (terminated), and the editionNumber metadata field is set to 0. All other metadata fields must be blank.

### 4.8.12.2 Dataset metadata

Dataset metadata is intended to describe information about a dataset. It facilitates the management and exploitation of data and is an important requirement for understanding the characteristics of a dataset. Whereas dataset metadata is usually fairly comprehensive, there is also a requirement for a constrained subset of metadata elements that are usually required for discovery purposes. Discovery metadata are often used for building web catalogues, and can help users determine whether a product or service is fit for purpose and where they can be obtained.

Table 28 Dataset metadata

| Name | Cardinality | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| S100_DataSetDiscoveryMetadata |  |  |  |  |
| fileName | 1 |  | CharacterString | Dataset file name |
| filePath | 1 |  | CharacterString | Full path from the exchange set root directory |
| description | 1 |  | CharacterString | Short description of the area covered by dataset, e.g., area, harbour, or port name, between two named locations etc. |
| dataProtection | 0..1 | {1} or {2} | Boolean | True : Encrypted False : Unencrypted |
| protectionScheme | 0..1 |  | CharacterString |  |
| digitalSignature | 0..1 |  | CharacterString |  |
| digitalSignatureReference | 0..1 |  | CharacterString |  |
| digitalSignatureValue | 0..1 |  | CharacterString |  |
| copyright | 0..1 |  | MD_LegalConstraints>MD_RestrictionCode <copyright> (ISO 19115-1) |  |
| classification | 0..1 |  | Class MD_SecurityConstraints>MD_ClassificationCode (codelist) ISO 19115-1 | 1. unclassified 2. restricted 3. confidential 4. secret 5. top secret |
| purpose | 1 | {1}, {2} | CharacterString | - New dataset - New edition |
| specificUsage | 1 |  | MD_USAGE>specificUsage (character string) MD_USAGE>userContactInfo (CI_Responsibility) | brief description of the resource and/or resource series usage |
| editionNumber | 1 | {1} | Integer | When a dataset is initially created, the edition number “1” is assigned to it. The edition number is increased by one with each new edition. |
| updateNumber | 1 |  | CharacterString | Update number 0 is assigned to a new dataset. |
| updateApplicationDate | 1 |  | Date | All updates dated on or before this date must have been applied by the producer. |
| issueDate | 1 |  | Date | Date on which the dataset was generated. |
| productSpecification | 1 |  | S100_ProductSpecification | See Notes below this table for constraints on values. |
| producingAgency | 1 |  | CI_Responsibility>CI_Organisation or CI_Responsibility>CI_Individual | Party responsible for generating the dataset. |
| optimumDisplayScale | 0..1 |  | Integer |  |
| maximumDisplayScale | 0..1 |  | Integer |  |
| minimumDisplayScale | 0..1 |  | Integer |  |
| horizontalDatumReference | 1 | EPSG | CharacterString |  |
| horizontalDatumValue | 1 | 4326 | Integer | WGS84 |
| verticalDatum | 0..1 |  | S100_VerticalAndSoundingDatum | Not included in Pilot Schedule |
| soundingDatum | 0..1 |  | S100_VerticalAndSoundingDatum | Not included in Pilot Schedule |
| dataType | 1 | GML | CharacterString | The only value allowed is “GML”. |
| otherDataTypeDescription | 0..1 |  | CharacterString |  |
| dataTypeVersion | 1 | 3.2.1 | CharacterString |  |
| dataCoverage | 1..* |  | S100_DataCoverage |  |
| comment | 0..1 |  | CharacterString | Any additional Information |
| layerID | 1..* | S-101 | CharacterString | Dataset must be used with ENC in an ECDIS. |

### 4.8.12.3 Update Dataset Metadata

N/A (Datset update is not relevant in this model)

### 4.8.12.4 Support file metadata

N/A (Support file is not included in this model)

### 4.8.12.5 Exchange set metadata

N/A (Exchange Catalogue is not used in SMART Navigation)

### 4.8.12.6 Catalogue file Metadata

All Pilot Schedule Exchange Catalogue files must contain at least the mandatory metadata elements.

Table 29 Catalogue file metadata

| Name | Multiplicity | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| S100_ExchangeCatalogue |  |  |  |  |
| identifier | 1 |  | S100_CatalogueIdentifier |  |
| contact | 1 |  | S100_CataloguePointOfContact |  |
| productSpecification | 0..1 |  | S100_ProductSpecification | Conditional on all the datasets using the same product specification. |
| metadataLanguage | 1 | English | CharacterString | All datasets conforming to this PS must use English language. A catalogue in English must be provided. Discovery metadata elements within catalogues have their own locale attributes and may be repeated in languages other than English. |
| exchangeCatalogueName | 1 |  | CharacterString | Catalogue filename |
| exchangeCatalogueDescription | 1 |  | CharacterString |  |
| exchangeCatalogueComment | 0..1 |  | CharacterString | Any additional Information |
| compressionFlag | 0..1 |  | Boolean | Yes or No |
| sourceMedia | 0..1 |  | CharacterString |  |
| replacedData | 0..1 |  | Boolean |  |
| dataReplacement | 0..1 |  | CharacterString |  |

## 4.9 port guideline

### 4.9.1 Overview

### 4.9.1.1 Introduction

This document has been produced by the member of SMART-Navigation project team(Korean Register, Marine-Works) in response to a requirement to produce a data product that can be used as a specific port information within an Portable Pilot Unit(PPU).  It is based on the IHO S-100 framework specification and the ISO 19100 series of standards. It is a vector product specification that is primarily intended for encoding wharf information, anchorage information, VTS information, safety pilotage guideline, attention information, tug boat information, fairway information, pilot station information.

### 4.9.1.2 References

IHO S-100 IHO Universal Hydrographic Data Model Edition 4.0.0 (December 2017).

ISO 8601. 2004. Data elements and interchange formats - Information interchange - Representation of dates and times. 2004.

ISO 3166-1. 1997. Country Codes. 1997.

ISO 10646-1 Information technology - Universal Multiple-Octet Coded Character Set (UCS) - Part 1: Architecture and Basic Multilingual Plane

ISO 19101-2:2008 Geographic Information - Rules for Application Schema

ISO/TS 19103:2005 Geographic Information - Conceptual schema language

ISO 19106:2004 Geographic Information – Profiles

ISO 19107:2003 Geographic Information – Spatial schema

ISO 19109:2005 Geographic Information - Rules for Application Schema

ISO 19111:2003 Geographic Information - Spatial referencing by coordinates

ISO 19115:2003+Corr1 (2006) Geographic Information - Metadata

ISO 19115-2:2009 Geographic Information - Metadata: Extensions for imagery and gridded data

ISO 19123:2005 Geographic Information - Schema for coverage geometry and functions

ISO 19129:2009 Geographic Information - Imagery gridded and coverage data framework

ISO 19131:2007 Geographic Information - Data product specifications

ISO 19136:2007 Geographic Information – Geography Markup Language

ISO 19136-2:2015, Geographic Information – Geography Markup Language.

ISO/TS 19139, Geographic Information – Metadata – XML schema implementation.

### 4.9.1.3 Terms, definitions and abbreviations

Terms and Definitions

The S-100 framework is based on the ISO 19100 series of geographic standards. The terms and definitions provided here are used to standardize the nomenclature found within that framework, whenever possible. They are taken from the references cited in clause 2.1. Modifications have been made when necessary.

- Application

Manipulation and processing of data in support of user requirements (ISO 19101)

- Application schema

Conceptual schema for data required by one or more applications (ISO 19101)

- Conceptual model

Model that defines concepts of a universe of discourse (ISO 19101)

- Conceptual schema

Formal description of a conceptual model (ISO 19101)

- Coverage

Feature that acts as a function to return values from its range for any direct position within its spatial, temporal or spatiotemporal domain (ISO 19123)

- Data product

Dataset or dataset series that conforms to a data product specification

- Data product specification

Detailed description of a dataset or dataset series together with additional information that will enable it to be created, supplied to and used by another party

- Dataset

Identifiable collection of data (ISO 19115)

- Feature

Abstraction of real world phenomena (ISO 19101)

- Feature association

Relationship that links instances of one feature type with instances of the same or a different feature type (ISO19110)

- Feature attribute

Characteristic of a feature (ISO 19101)

- Geographic data

Data with implicit or explicit reference to a location relative to the Earth (ISO 19109)

- Metadata

Data about data (ISO 19115)

- Model

Abstraction of some aspects of reality (ISO 19109)

- Portrayal

Presentation of information to humans (ISO 19117)

- Quality

Totality of characteristics of a product that bear on its ability to satisfy stated and implied needs (ISO 19101)

Abbreviations

This product specification adopts the following convention for symbols and abbreviated terms:

- ASCII 	American Standard Code for Information Interchange

- ECDIS	Electronic Chart Display and Information Systems

- ECS	Electronic Chart Systems

- ENC	Electronic Navigational Chart

- GML	Geography Markup Language

- IHO	International Hydrographic Organization

- IOC	International Oceanographic Commission

- ISO	International Organization for Standardization

- UML	Unified Modelling Language

- URI	Uniformed Resource Identifier

- URL 	Uniform Resource Locator

- WMS	Web Map Service

- WFS	Web Feature Service

- www 	World Wide Web

- XML	Extensible Markup Language

- XSLT	eXtensible Stylesheet Language Transformations

### 4.9.1.4 General Data Product Description

Informal Description of Data Product

This clause contains general information about the data product

- Title:  	Port Guideline Product Specification.

- Abstract: 	 port guideline product specification describes the means to deliver unique information of specific port such as safety pilotage guideline, attention information, fairway information, pilot station information. This may include details on the VHF channel, fairway name, fairway location, fairway dimension, pilot station location, accident location, speed information, safety standard information.

- Content:	Datasets conforming to this specification will contain all port information for the area of coverage, additionally, there will be relevant metadata data quality, production authority, data sources and publication date.

- Spatial Extent:	Korean Coast

- Specific Purpose:	Describing specific port information for utilization of Pilot & Tug Assistance service.

### 4.9.1.5 Data product specification metadata

This information uniquely identifies this Product Specification and provides information about its creation and maintenance. For further information on dataset metadata see the metadata clause.

- Title: 	Port Guideline

- S-100 Version: 	4.0.0

- Port Guideline Version: 	1.0.0

- Date: 	2019-03-08

- Language: 	English

- Classification: 	Unclassified

- Contact: 	32, Yuseong-daero 1312beon-gil, Yuseong-gu, Daejeon, Korea 32(Jang Dong) inside KRISO

- URL: 	http://www.smart-navigation.org

- Identifier: 	Port Guideline

- Maintenance: 	Amendments to this specification will be produced on a needs basis. For reporting issues with this specification which need correction, use the contact information.

### 4.9.2 Specification Scopes

This product specification describes one data product and therefore requires only one scope which is described below:

- Scope ID:  		Port Guideline datasets.

- Hierarchical level: 		MD_ScopeCode - 005

- Hierarchical level name: 	dataset.

- Level description:		information applies to the dataset

- Extent:	EX_Extent.description: Korean Coast

### 4.9.3 Dataset identification

This section describes how to identify data sets that conform to the specification. A dataset that conforms to this Product Specification may be identified by its discovery metadata as defined in clause 12 of this specification. The information identifying the data product may include the following items from S-100 4.0.0 clause 11-6 (adapted from ISO 19115).

| Title | Port Guideline |
| --- | --- |
| Abstract | Port Guideline dataset is a vector dataset containing specific port information regarding the Pilot & Tug Assistance service services within a defined geographical area. |
| Acronym | PG |
| Content | Couse location information, Tug boat information, Limited speed, restricted water |
| Topic category | Transportation |
| geographicDescription | Area specific to marine navigation within korean sea area |
| Purpose | Standardization of port information |
| Language | EN Additional values if any use CharacterString values from ISO 639-2 |

### 4.9.4 Data Content and structure

### 4.9.4.1 Introduction

The Port Guideline product is based on the S-100 General Feature Model (GFM), and is a feature-based vector product. All Port Guideline features and information classes are derived from one of the abstract classes FeatureType and InformationType defined in the Port Guideline respectively.

This clause contains the Application Schema expressed in UML and an associated Feature Catalogue.

### 4.9.4.2 Application Schema

The UML models in this clause are segments of the overall Port Guideline application schema, and include overviews of the feature classes, information classes, meta features, spatial types, and the relationships between them.

This clause contains a general overview of the classes and relationships in the Port Guideline application schema. Detailed information about how to use the feature types and information types to encode Port Guideline information is provided in the SMART-Navigation Data Classification and Encoding Guide (DCEG).

The following conventions are used in the UML diagrams depicting the application schema:

- Standard UML conventions for classes, associations, inheritance, roles, and multiplicities apply. These conventions are described in Part 1 of S-100.

- Italic font for a class name indicates an abstract class.

- Feature classes are depicted with pink background; the dark shade for abstract feature classes and the light shade for ordinary (non-abstract) feature classes.

- Complex attributes are depicted with a light yellow background.

- Enumeration lists and codelists are depicted with a green background. The numeric code corresponding to each listed value is shown to its right following an ‘=’ sign.

- No significance attaches to the colour of associations. (Complex diagrams may use different colours to distinguish associations that cross one another.)

### 4.9.4.2.1 Domain model

Figure 20 Port Guideline Overview

### 4.9.4.3 Feature Catalogue

### 4.9.4.3.1 Introduction

The Feature Catalogue describes the feature types, information types, attributes, attribute values, associations and roles which may be used in the product.  The Port Guideline Feature Catalogue is available in an XML document which conforms to the S-100 XML Feature Catalogue Schema.  Simple attributes used in this specification are listed in below.

Name: 	Port Guideline Feature Catalogue

Scope: 	Smart-Navigation

Version Number: 	1.0.0

Version Date:	2019-03-15

Producer:	SMART Navigation Project

Language: 	English

### 4.9.4.3.1.1 Feature Types

Feature types contain descriptive attributes that characterize real-world entities. The word ‘feature’ may be used in one of two senses – feature type and feature instance. A feature type is a class and is defined in a Feature Catalogue. A feature instance is a single occurrence of the feature type and represented as an object in a dataset. A feature instance is located by a relationship to one or more spatial instances. A feature instance may exist without referencing a spatial instance.

### 4.9.4.3.1.2 Geographic

A Geographic (Geo) feature type carries the descriptive characteristics of a real-world entity.

### 4.9.4.3.1.3 Meta

Meta features contain information about other features within a dataset. Information defined by meta features override the default metadata values defined by the dataset descriptive records. Meta attribution on individual features overrides attribution on meta features.

### 4.9.4.3.1.4 Feature Relationship

A feature relationship links instances of one feature type with instances of the same or a different feature type.

### 4.9.4.3.1.5 Information Types

Information types are identifiable pieces of information in a dataset that can be shared between other features. They have attributes but have no relationship to any geometry; information types may reference other information types.

### 4.9.4.3.1.6 Attributes

SMART-Navigation product defines attributes as either simple or complex.

### 4.9.4.3.1.6.1 Simple Attributes

SMART-Navigation product uses nine types of simple attributes; they are listed in the following table:

| Type | Definition |
| --- | --- |
| Enumeration | A fixed list of valid identifiers of named literal values |
| Boolean | A value representing binary logic. The value can be either True or False. The default state for Boolean type attributes (i.e. where the attribute is not populated for the feature) is False. |
| Real | A signed Real (floating point) number consisting of a mantissa and an exponent |
| Integer | A signed integer number. The representation of an integer is encapsulation and usage dependent. |
| Text | An arbitrary-length sequence of characters including accents and special characters from a repertoire of one of the adopted character sets |
| Date | A date provides values for year, month and day according to the Gregorian Calendar. Character encoding of a date is a string which must follow the calendar date format (complete representation, basic format) for date specified by ISO 8601:1988. EXAMPLE 19980918 (YYYY-MM-DD) |
| Time | A time is given by an hour, minute and second. Character encoding of a time is a string that follows the local time (complete representation, basic format) format defined in ISO 8601:1988. EXAMPLE 183059 or 183059+0100 or 183059Z |
| Date and Time | A DateTime is a combination of a date and a time type. Character encoding of a DateTime shall follow ISO 8601:1988 EXAMPLE 19850412T101530 |
| Codelist | A type of flexible enumeration. A code list type is a list of literals which may be extended only in conformance with specified rules. |

### 4.9.4.3.1.6.2 Complex Attributes

Complex attributes are aggregations of other attributes that are either simple or complex. The aggregation is defined by means of attribute bindings.

### 4.9.4.3.1.6.3 Geometric Representation

Geometric representation is the digital description of the spatial component of an object as described in S-100 and ISO 19107. This product specification uses three types of geometries: GM_Point, GM_OrientableCurve, and GM_OrientableSurface.

### 4.9.4.3.2 Feature Catalogue General

The Port Guideline feature catalogue describes the feature types, information types, attributes, attribute values, associations and roles which may be used in a Port Guideline data set.

### 4.9.4.3.2.1 Feature Types

- PortGuideline

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| shipMRN | MRN is a naming scheme that can uniquely identify any maritime resource on a global scale. shipMRN is the unique value given to the ship. | URN | 1 | mandatory |  |
| MMSICode | The Maritime Mobile Service Identity (MMSI) Code is formed of a series of nine digits which are transmitted over the radio path in order to uniquely identify ship stations, ship earth stations, coast stations, coast earth stations, and group calls. These identities are formed in such a way that the identity or part thereof can be used by telephone and telex subscribers connected to the general telecommunications network principally to call ships automatically. (Adapted from Appendix 43 of the International Telecommunications Union Radio Regulations). | integer | 1 | mandatory |  |
| portName | The complex attribute provides the name of an entity, defines the national language of the name, and provides the option to display the name at various system display settings. | complex attribute | 0..* | option |  |

- WharfInformation

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| wharfName | - | complex attribute | 1 | mandatory |  |
| detailedWharfName | - | complex attribute | 1 | mandatory |  |
| wharfDimension | - | complex attribute | 0..1 | option |  |
| wharfLocation | - | GM_Point | 0..1 | option |  |
| wharfDivision | - | text | 0..1 | option |  |
| length | - | real | 0..1 | option |  |
| loadingCapacity | - | text | 0..1 | option |  |

- AnchorageInformation

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| anchorageName | - | complex attribute | 1 | mandatory |  |
| courseName | - | complex attribute | 1 | mandatory |  |
| anchorBerthAreaRestriction | - | complex attribute | 0..* | option |  |
| anchorageLocation | - | GM_Point | 0..1 | option |  |
| division | - | text | 0..1 | option |  |

- VTSInformation

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| callSign | The designated call-sign of a radio station. | text | 1 | mandatory |  |
| informationTransmissionMethod | - | enumeration | 0..1 | option |  |
| VHFChannel | - | text | 0..1 | option |  |
| reportingPoint | - | text | 0..1 | option |  |
| reportingTime | - | text | 0..1 | option |  |
| reportingContent | - | text | 0..1 | option |  |

- SafetyPilotageStandardInformation

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| state | - | text | 1 | mandatory |  |
| content | - | text | 1 | mandatory |  |

- SafetyPilotageSpeedStandardInformation

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| restrictedWater | - | GM_Surface | 0..1 | option |  |
| restrictedWaterName | - | complex attribute | 1 | mandatory |  |
| targetVessel | - | complex attribute | 0..1 | option |  |
| limitedSpeed | - | real | 0..1 | option |  |

- TugBoatInformation

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| requiredTugBoatInformation | - | complex attribute | 0..* | option |  |

- SpeedInformation

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| featureName | - | complex attribute | 1 | mandatory |  |
| location | - | GM_Point | 0..* | option |  |
| speedLimit | - | real | 0..1 | option |  |

- FairwayInformation

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| fairwayName | - | complex attribute | 1 | mandatory |  |

- FairwayCharacteristic

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| seaDimension | - | complex attribute | 0..* | option |  |
| information | Provides textual information that cannot be provided using other allowable attributes for the feature, in a defined language. The information may be provided as a string in sub-attribute text, or by encoding the file name of a single external text file that contains the text in sub-attribute file reference. | complex attribute | 0..* | option |  |
| fairwayLocation | - | GM_Surface | 0..1 | option |  |

- PilotStationInformation

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| typeOfPilotStation | - | enumeration | 0..1 | option |  |
| pilotStationLocation | - | GM_Surface | 0..1 | option |  |

- AttentionInformation

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| accidentlocation | - | GM_Point | 0..1 | option |  |
| typeOfGuideline | - | enumeration | 0..1 | option |  |
| content | - | text | 0..1 | option |  |

- CourseInformation

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| typeOfCourse | - | enumeration | 0..1 | option |  |

- CourseLocationInformation

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| courseLocation | - | GM_Point | 1 | mandatory |  |
| courseHeading | - | real | 1 | mandatory |  |

- Meta

- Cartographic

- Theme

### 4.9.4.3.2.2 Feature Relationship

| Role Type | Association Name | Role | Feature | Multiplicity |
| --- | --- | --- | --- | --- |
| Composition | portGuidelineInformation | componentOf | WharfInformation | 1 |
| Composition | portGuidelineInformation | componentOf | AnchorageInformation | 1 |
| Composition | portGuidelineInformation | componentOf | VTS Information | 1 |
| Composition | portGuidelineInformation | componentOf | SafetyPilotageStandardInformation | 1 |
| Composition | portGuidelineInformation | componentOf | SafetyPilotageSpeedStandardInformation | 1 |
| Composition | portGuidelineInformation | componentOf | TugBoatInformation | 1 |
| Composition | portGuidelineInformation | componentOf | FairwayInformation | 1 |
| Composition | portGuidelineInformation | componentOf | SpeedInformation | 1 |
| Composition | portGuidelineInformation | consistOf | PortGuideline | 0..* |
| Composition | locationInformation | componentOf | CourseLocationInformation | 1 |
| Composition | locationInformation | consistOf | CourseInformation | 1..* |

### 4.9.4.3.2.3 Information Relationship

- N/A

### 4.9.4.3.2.4 Information Type

- N/A

### 4.9.4.3.2.5 Simple Attribute

- Simple Attribute List

| Name | Description | Data Type | Remarks |
| --- | --- | --- | --- |
| shipMRN | Ship MRN | URN |  |
| MMSICode | MMSI Code | text |  |
| wharfDivision | wharfDivision | text |  |
| length | Length | real |  |
| loadingCapacity | Loading Capacity | text |  |
| division | - | text |  |
| callSign | Call Sign | text |  |
| informationTransmissionMethod | - | enumeration |  |
| VHFChannel | VHF Channel | text |  |
| reportingPoint | Reporting Point | text |  |
| reportingTime | Reporting Time | text |  |
| reportingContent | Reporting Content | text |  |
| state | State | text |  |
| content | Content | text |  |
| limitedSpeed | Limited Speed | real |  |
| speedLimit | Speed Limit | real |  |
| typeOfPilotStation | Type Of Pilot Station | enumeration |  |
| typeOfGuideline | Type Of Guideline | enumeration |  |
| content | content | text |  |
| typeOfCourse | Type Of Course | enumeration |  |
| courseHeading | Course Heading | real |  |
| vesselsCharacteristics | Vessels Characteristics | enumeration |  |
| vesselsCharacteristicsValue | Vessels Characteristics Value | real |  |
| vesselsCharacteristicsUnit | Vessels Characteristics Unit | enumeration |  |
| comparisonOperator | Comparison Operator | enumeration |  |
| displayName | DisplayName | boolean |  |
| language | Language | text |  |
| name | Name | text |  |
| text | Text | text |  |
| vesselCount | Vessel Count | integer |  |
| categoryOfVessel | Category Of Vessel | enumeration |  |
| mainEnginePower | Main Engine Power | real |  |
| tugBoatCount | Tug Boat Count | integer |  |
| typeOfDimension | Type Of Dimension | enumeration |  |
| typeOfDimensionValue | - | enumeration |  |
| dimensionValue | Dimension Value | real |  |

- informationTransmissionMethod

| Value | Label | Description |
| --- | --- | --- |
| 1 | VHF | VHF |
| 2 | reporting | reporting |

- typeOfPilotStation

| Value | Label | Description |
| --- | --- | --- |
| 1 | when the pilot is on board | when the pilot is on board |
| 2 | when the pilot is disembarked | when the pilot is disembarked |

- typeOfGuideline

| Value | Label | Description |
| --- | --- | --- |
| 1 | accident | accident |
| 2 | safety | safety |
| 3 | attention | attention |

- typeOfCourse

| Value | Label | Description |
| --- | --- | --- |
| 1 | inbound | inbound |
| 2 | outbound | outbound |

- vesselsCharacteristics

| Value | Label | Description |
| --- | --- | --- |
| 1 | length overall | length overall |
| 10 | none | none |

- vesselsCharacteristicsUnits

| Value | Label | Description |
| --- | --- | --- |
| 1 | metre | - |
| 2 | foot | - |
| 3 | metric ton | The tonne or metric ton (U.S.), often redundantly referred to as a metric tonne, is a unit of mass equal to 1,000 kg (2,205 lb) or approximately the mass of one cubic metre of water at four degrees Celsius. It is sometimes abbreviated as mt in the United States, but this conflicts with other SI symbols. The tonne is not a unit in the International System of Units (SI), but is accepted for use with the SI. In SI units and prefixes, the tonne is a megagram (Mg). The Imperial and US customary units comparable to the tonne are both spelled ton in English, though they differ in mass. Pronunciation of tonne (the word used in the UK) and ton is usually identical, but is not too confusing unless accuracy is important as the tonne and UK long ton differ by only 1.6%. |
| 4 | ton | Long ton (weight ton or imperial ton) is the name for the unit called the "ton" in the avoirdupois or Imperial system of measurements, as used in the United Kingdom and several other Commonwealth countries. It has been mostly replaced by the tonne, and in the United States by the short ton. One long ton is equal to 2,240 pounds (1,016 kg) or 35 cubic feet (0.9911 m3) of salt water with a density of 64 lb/ft?(1.025 g/ml). It has some limited use in the United States, most commonly in measuring the displacement of ships, and was the unit prescribed for warships by the Washington Naval Treaty for example battleships were limited to a mass of 35,000 long tons (36,000 t; 39,000 ST). |
| 5 | short ton | The short ton is a unit of weight equal to 2,000 pounds (907.18474 kg). In the United States it is often called simply ton without distinguishing it from the metric ton (tonne, 1,000 kilograms) or the long ton (2,240 pounds / 1,016.0469088 kilograms); rather, the other two are specifically noted. There are, however, some U.S. applications for which unspecified tons normally means long tons (for example, Navy ships) or metric tons (world grain production figures). Both the long and short ton are defined as 20 hundredweights, but a hundredweight is 100 pounds (45.359237 kg) in the U.S. system (short or net hundredweight) and 112 6pounds (50.80234544 kg) in the Imperial system (long or gross h7undredweight). |
| 6 | gross ton | Gr8oss tonnage (GT) is a function of the volume of all ship's enclosed spaces (from keel to funnel) measured to the outside of the hull framing. There is a sliding scale factor. So GT is a kind of capacity-derived index that is used to rank a ship for purposes of determining manning, safety and other statutory requirements and is expressed simply as GT, which is a unitless entity, even though its derivation is tied to the cubic meter unit of volumetric capacity. Tonnage measurements are now governed by an IMO Convention (International Convention on Tonnage Measurement of Ships, 1969 (London-Rules)), which applies to all ships built after July 1982. In accordance with the Convention, the correct term to use now is GT, which is a function of the moulded volume of all enclosed spaces of the ship. |
| 7 | net ton | Net tonnage (NT) is based on a calculation of the volume of all cargo spaces of the ship. It indicates a vessel’s earning space and is a function of the moulded volume of all cargo spaces of the ship. |
| 8 | panama canal/universal measurement system net tonnage | - |
| 9 | suez canal net tonnage | The Suez Canal Net Tonnage (SCNT) is derived with a number of modifications from the former net register tonnage of the Moorsom System and was established by the International Commission of Constantinople in its Protocol of 18 December 1873. It is still in use, as amended by the Rules of Navigation of the Suez Canal Authority, and is registered in the Suez Canal Tonnage Certificate. |
| 10 | none | - |
| 11 | cubic metres | - |

- comparisonOperator

| Value | Label | Description |
| --- | --- | --- |
| 1 | greather than | greather than |
| 2 | greather than or equal to | greather than or equal to |
| 3 | less than | less than |
| 4 | less than or equal to | less than or equal to |
| 5 | equal to | equal to |
| 6 | not equal to | not equal to |

- categoryOfVessel

| Value | Label | Description |
| --- | --- | --- |
| 1 | cargo vessel | cargo vessel |
| 2 | container carrier | container carrier |
| 3 | tanker | tanker |
| 4 | bulk carrier | bulk carrier |
| 5 | passenger vessel | passenger vessel |
| 6 | roll-on roll-off | roll-on roll-off |
| 7 | refrigerated cargo vessel | refrigerated cargo vessel |
| 8 | fishing vessel | fishing vessel |
| 9 | service | service |
| 10 | warship | warship |
| 11 | towed or pushed composite unit | towed or pushed composite unit |
| 12 | tug and tow | tug and tow |
| 13 | light recreational | light recreational |
| 14 | semi-submersible offshore installation | semi-submersible offshore installation |
| 15 | jack-up exploration or project installation | jack-up exploration or project installation |
| 16 | livestock carrier | livestock carrier |
| 17 | sport fishing | sport fishing |
| 18 | all vessel | all vessel |

- typeOfDimension

| Value | Label | Description |
| --- | --- | --- |
| 1 | front depth | front depth |
| 2 | depth | depth |
| 3 | width | width |
| 4 | length | length |

- typeOfDimensionValue

| Value | Label | Description |
| --- | --- | --- |
| 1 | min | min |
| 2 | max | max |

### 4.9.4.3.2.6 Complex Attribute

- featureName

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| displayName | DisplayName | boolean | 0..1 | option |  |
| language | Language | text | 0..1 | option |  |
| name | Name | text | 1 | mandatory |  |

- portName

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| displayName | DisplayName | boolean | 0..1 | option |  |
| language | Language | text | 0..1 | option |  |
| name | Name | text | 1 | mandatory |  |

- wharfName

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| displayName | DisplayName | boolean | 0..1 | option |  |
| language | Language | text | 0..1 | option |  |
| name | Name | text | 1 | mandatory |  |

- detailedWharfName

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| displayName | DisplayName | boolean | 0..1 | option |  |
| language | Language | text | 0..1 | option |  |
| name | Name | text | 1 | mandatory |  |

- anchorageName

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| displayName | DisplayName | boolean | 0..1 | option |  |
| language | Language | text | 0..1 | option |  |
| name | Name | text | 1 | mandatory |  |

- restrictedWaterName

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| displayName | DisplayName | boolean | 0..1 | option |  |
| language | Language | text | 0..1 | option |  |
| name | Name | text | 1 | mandatory |  |

- fairwayName

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| displayName | DisplayName | boolean | 0..1 | option |  |
| language | Language | text | 0..1 | option |  |
| name | Name | text | 1 | mandatory |  |

- courseName

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| displayName | DisplayName | boolean | 0..1 | option |  |
| language | Language | text | 0..1 | option |  |
| name | Name | text | 1 | mandatory |  |

- seaDimension

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| typeOfDimension | Type Of Dimension | enumeration | 1 | mandatory |  |
| typeOfDimensionValue | - | enumeration | 0..1 | option |  |
| dimensionValue | Dimension Value | real | 1 | mandatory |  |

- wharfDimension

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| typeOfDimension | Type Of Dimension | enumeration | 1 | mandatory |  |
| typeOfDimensionValue | - | enumeration | 0..1 | option |  |
| dimensionValue | Dimension Value | real | 1 | mandatory |  |

- vesselMeasurement

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| vesselsCharacteristics | Vessels Characteristics | enumeration | 0..* | option |  |
| vesselsCharacteristicsValue | Vessels Characteristics Value | real | 0..1 | option |  |
| vesselsCharacteristicsUnits | Vessels Characteristics Unit | enumeration | 0..1 | option |  |
| comparisonOperator | Comparison Operator | enumeration | 0..1 | option |  |

- tugBoatMeasurement

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| vesselsCharacteristics | Vessels Characteristics | enumeration | 0..* | option |  |
| vesselsCharacteristicsValue | Vessels Characteristics Value | real | 0..1 | option |  |
| vesselsCharacteristicsUnits | Vessels Characteristics Unit | enumeration | 0..1 | option |  |
| comparisonOperator | Comparison Operator | enumeration | 0..1 | option |  |

- information

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| language | Language | text | 0..1 | option |  |
| text | Text | text | 0..1 | option |  |

- anchorBerthAreaRestriction

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| vesselMeasurement | Vessel Measurement | complex attribute | 0..* | option |  |
| vesselCount | Vessel Count | integer | 1 | mandatory |  |

- targetVessel

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| categoryOfVessel | Category Of Vessel | enumeration | 1 | mandatory |  |
| vesselMeasurement | Vessel Measurement | complex attribute | 0..* | option |  |

- availableTugboat

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| mainEnginePower | Main Engine Power | real | 1 | mandatory |  |
| tugBoatCount | Tug Boat Count | integer | 1 | mandatory |  |

- requiredTugBoatInformation

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| tugBoatMesurement | - | complex attribute | 0..* | option |  |
| availableTugBoat | - | complex attribute | 0..* | option |  |

### 4.9.5 Coordinate Reference System(CRS)

### 4.9.5.1 Introduction

The location of an object in the S-100 standard is defined by means of coordinates which relate a feature to a position. The coordinate reference system used for this product specification is World Geodetic System 1984 (WGS 84) which is defined by the European Petroleum Survey Group (EPSG) code 4326, (or similar - North American Datum 1983 / Canadian Spatial Reference System).

Spatial data are expressed as latitude (φ) and longitude (λ) geographic coordinates. Latitude values are stored as a negative number to represent a position south of the Equator. Longitude values are stored as a negative number to represent a position west of the Prime Meridian. Coordinates are expressed as real value, degree / degree decimal format. Datasets conforming to this product specification are not projected.

Horizontal coordinate reference system:	WGS 84

Projection: 					None

Vertical coordinate reference system: 	None

Temporal reference system: 		Gregorian calendar

Coordinate reference system registry: 	EPSG Geodetic Parameter Registry

Date type (according to ISO 19115-1):  	002 - publication

### 4.9.5.2 Horizontal Reference System

Positional data is expressed in latitude and longitude geographic coordinates to one of the reference horizontal reference systems defined in the HORDAT attribute.  Unless otherwise defined, the World Geodetic System 84 (WGS 84) will be used for Port Guideline data products.

### 4.9.5.3 Projection

Port Guideline data products are un-projected.

### 4.9.5.4 Vertical Coordinate Reference System

Port Guideline data products do not provide detailed vertical information.

### 4.9.5.5 Temporal Reference System

Time is measured by reference to Calendar dates and Clock time in accordance with ISO 19108:2002 Temporal Schema clause 5.4.4.

### 4.9.5.6 Port Guideline Data and Scale

Port Guideline data must be compiled in the best applicable scale. The use of the data itself is "scale independent". That means that the data can be used at any scale. S-100 allows the association of multiple spatial attributes to a single feature instance. In principle, each of these spatial attributes can be qualified by maximum and minimum scales.

For example, it is possible, within one dataset, to have a single instance of a feature that has more than one area geometry. Each of these geometries has different scale max/min attributes. Moreover, due to cluttering in smaller scales, the scale minimum attribute may be used to turn off portrayal of some features at smaller scales.

### 4.9.6 Data Quality

### 4.9.6.1 Introduction

Port Guideline products must be tested with the Port Guideline specific checks prior to release by the data producer. The data producer must review the check results and address any issues to ensure sufficient quality of the data products. The checks are a mix of data format validation checks, conformance to standard checks and logical consistency checks. But the checklist has not been written yet. It will be updated in the next version.

### 4.9.7 Data Capture and classification

See DCEG for this section.

### 4.9.8 maintenance

### 4.9.8.1 Introduction

Datasets are maintained as needed and must include mechanisms for Port Guideline updating. Data updates will be made by new editions. The maintenance and update frequency of Port Guideline datasets should be defined by the producers (official national authority) implementing this specification.

Data Producers must use applicable sources to maintain and update data and provide a brief description of the sources that were used to produce the dataset in the appropriate metadata field.

The data product shall provide information on how the data is maintained and should describe the principles and criteria applied in maintenance regime. This should specify the expected frequency of updates.

Table 30 Maintenance and update frequency

| Item Name | Description | Multiplicity | Type |
| --- | --- | --- | --- |
| maintenanceAndUpdateFrequency | Frequency with which changes and additions are made to the data product (per update scope) | 1..* | MD_MaintenanceInformation (ISO 19115) |
| dataSource | Identification of the kinds of data sources usable to produce datasets | 1..* | LI_Source (ISO 19115) |
| productionProcess | Textual description of the production process applicable to the datasets (per scope or data source) | 1..* | LI_ProcessStep (ISO 19115) |

### 4.9.8.2 Production process for base and update datasets

Data Producers should follow their established production processes for maintaining and updating datasets. Data is produced against the DCEG and checked against the appropriate set of validation rules in Appendix E. (Appendix E is not included in this release.)

### 4.9.8.3 Datasetr updates

N/A (Dataset update is not relevant in this model)

### 4.9.8.4 Support file updates

N/A (Support file is not included in this model)

### 4.9.8.5 Feature and portrayal catalogues

For each new version of the Port Guideline Product Specification a new feature and portrayal catalogue will be released. The system must be able to manage datasets and their catalogues that are created on different versions of the Port Guideline product specification.

### 4.9.8.6 Feature history, versions, and change tracking

If applications or production systems require versioning of individual instances of feature or information types, maintenance of histories, or change tracking, the methods for versioning, history management, and change tracking and display are left to the application or production system.

### 4.9.8.7 Dataset encryption

Details about dataset encryption are still to be determined, and may mirror the method described in S-101.

### 4.9.9 Portrayal

The portrayal catalogue model is defined in S-100 Part 9.

The “Port Guideline” Portrayal Catalogue will be available in an XML document which conforms to the S-100 XML Portrayal Catalogue Schema and is structured as follows:

Root ---- (contains the catalogue named “portrayal_catalogue.xml”)

|

|-- Pixmaps (contains XML files describing pixmaps)

|

|-- ColorProfiles (contains XML files with colour profiles and CSS2 style sheets)

|

|-- Symbols (contains SVG files with symbols)

|

|-- LineStyles (contains XML files with line styles)

|

|-- AreaFills (contains XML files area fills)

|

|-- Fonts (contains TrueType font files)

|

|-- Rules (contains XSLT files with templates)

### 4.9.9.1 Pixmaps

- N/A

### 4.9.9.2 ColorProfiles

colorProfile.xml  (This is the same as that used for S-101.)

### 4.9.9.3 Symbols

| No. | Name | Description | Symbol |
| --- | --- | --- | --- |
| 1 | PilotStation | Object that display the pilot embarkation / dis-embarkation point, in the port guideline information provided by SMART Navigation. |  |
| 2 | DisEmbarkation | An object that is displayed along with the PilotStation symbol and indicates that the PilotStation is for dis-embarkation. |  |
| 3 | Embarkation | An object that is displayed along with the PilotStation symbol and indicates that the PilotStation is for embarkation. |  |
| 4 | FairWay | Object that display the fiarway area of the port, in the port guideline information provided by SMART Navigation. |  |
| 5 | SpeedMark | A symbol designed to indicate a zone with information related to speed, in the port guideline information provided by SMART Navigation. |  |
| 6 | LimitMark | A symbol designed to indicate a zone with information related to any limit, in the port guideline information provided by SMART Navigation. |  |
| 7 | VesselGuide | Objects to guide the direction of the ship along the ship's position |  |
| 8 | QUESMRK1 | Object which is not sufficiently described to be symbolized, or for which no symbol exists in the symbol library |  |
| 9 | EMQUESM1 | Embedded symbol |  |

### 4.9.9.4 Line Styles

| No. | Name | Contents |
| --- | --- | --- |
| 1 | QUESMRK1 | <intervalLength>17.5</intervalLength> <pen width="0.64"><color>CHMGD</color></pen> <dash><start>2</start><length>3</length></dash> <dash><start>7</start><length>3</length></dash> <dash><start>14.6</start><length>3</length></dash> <symbol reference="EMQUESM1"><position>11.4</position></symbol> |
| 2 | FairWayLine | <intervalLength>24.8</intervalLength> <pen width="0.32"><color>RESBL</color></pen> <dash><start>17.0</start><length>2.9</length></dash> <dash><start>5.1</start><length>-3.1</length></dash> <dash><start>21.9</start><length>2.9</length></dash> <symbol reference="FairWay"><position>11.1</position></symbol> |
| 3 | SpeedLimitLine | <intervalLength>43.9</intervalLength> <pen width="0.32"><color>SCLBR</color></pen> <dash><start>26.3</start><length>2.9</length></dash> <dash><start>5.1</start><length>-3.1</length></dash> <dash><start>31.2</start><length>2.9</length></dash> <dash><start>36.1</start><length>2.9</length></dash> <dash><start>41.0</start><length>2.9</length></dash> <symbol reference="SpeedMark"><position>11.3</position></symbol> <symbol reference="LimitMark"><position>20.3</position></symbol> |

### 4.9.9.5 AreaFills

- N/A

### 4.9.9.6 Fonts

- N/A

### 4.9.9.7 Rules

| No | Name | Match ( Priority = 9 ) |  |  |
| --- | --- | --- | --- | --- |
| 1 | PilotStation | [@primitive='Surface'] |  |  |
|  | ViewingGroup | DisplalyPlane | DrawingPriority | Symbol |
|  | 50503 | UNDERRADAR | 25 | PilotStation (VisibleParts) |
|  | ViewingGroup | DisplalyPlane | DrawingPriority | LineStyle |
|  | 50503 | UNDERRADAR | 25 | Dash , 0.64 , RESBL |

| No | Name | Match ( Priority = 1 ) |  |  |
| --- | --- | --- | --- | --- |
| 2 | PilotStation | [@primitive='Surface' and typeOfPilotStation=1] |  |  |
|  | ViewingGroup | DisplalyPlane | DrawingPriority | Symbol |
|  | 50503 | UNDERRADAR | 25 | PilotStation (VisibleParts) |
|  | ViewingGroup | DisplalyPlane | DrawingPriority | Symbol |
|  | 50503 | UNDERRADAR | 25 | Embarkation (VisibleParts) |
|  | ViewingGroup | DisplalyPlane | DrawingPriority | LineStyle |
|  | 50503 | UNDERRADAR | 25 | Dash , 0.64 , RESBL |

| No | Name | Match ( Priority = 2 ) |  |  |
| --- | --- | --- | --- | --- |
| 3 | PilotStation | [@primitive='Surface' and typeOfPilotStation=2] |  |  |
|  | ViewingGroup | DisplalyPlane | DrawingPriority | Symbol |
|  | 50503 | UNDERRADAR | 25 | PilotStation (VisibleParts) |
|  | ViewingGroup | DisplalyPlane | DrawingPriority | Symbol |
|  | 50503 | UNDERRADAR | 25 | DisEmbarkation (VisibleParts) |
|  | ViewingGroup | DisplalyPlane | DrawingPriority | LineStyle |
|  | 50503 | UNDERRADAR | 25 | Dash , 0.64 , RESBL |

| No | Name | Match ( Priority = 1 ) |  |  |
| --- | --- | --- | --- | --- |
| 4 | FairWay | [@primitive='Surface'] |  |  |
|  | ViewingGroup | DisplalyPlane | DrawingPriority | LineStyle |
|  | 50501 | OVERRADAR | 25 | FairWayLine |

| No | Name | Match ( Priority = 1 ) |  |  |
| --- | --- | --- | --- | --- |
| 5 | Speed | [@primitive='Surface'] |  |  |
|  | ViewingGroup | DisplalyPlane | DrawingPriority | LineStyle |
|  | 50504 | OVERRADAR | 25 | SpeedLimitLine |

| No | Name | Match ( Priority = 1 ) |  |  |
| --- | --- | --- | --- | --- |
| 6 | CourseLocation | [@primitive='point'] |  |  |
|  | ViewingGroup | DisplalyPlane | DrawingPriority | Symbol |
|  | 50502 | OVERRADAR | 25 | VesselGuide |
|  | ViewingGroup | DisplalyPlane | DrawingPriority | Text |
|  | 50502 | OVERRADAR | 25 | “courseHeading” |

| No | Name | Match |  |  |
| --- | --- | --- | --- | --- |
| 7 | Anchorage | [@primitive='Point'] |  |  |
|  | ViewingGroup | DisplalyPlane | DrawingPriority | Symbol |
|  | 50099 | OVERRADAR | 15 | QUESMRK1 |

| No | Name | Match |  |  |
| --- | --- | --- | --- | --- |
| 8 | Attention | [@primitive='Point'] |  |  |
|  | ViewingGroup | DisplalyPlane | DrawingPriority | Symbol |
|  | 50099 | OVERRADAR | 15 | QUESMRK1 |

| No | Name | Match |  |  |
| --- | --- | --- | --- | --- |
| 9 | PilotStandard | [@primitive='Surface'] |  |  |
|  | ViewingGroup | DisplalyPlane | DrawingPriority | Symbol |
|  | 50099 | OVERRADAR | 15 | QUESMRK1 |

| No | Name | Match |  |  |
| --- | --- | --- | --- | --- |
| 10 | Wharf | [@primitive='Point'] |  |  |
|  | ViewingGroup | DisplalyPlane | DrawingPriority | Symbol |
|  | 50099 | OVERRADAR | 15 | QUESMRK1 |

| No | Name | Match |  |  |
| --- | --- | --- | --- | --- |
| 11 | default | [@primitive='Curve'] |  |  |
|  | ViewingGroup | DisplalyPlane | DrawingPriority | LineStyle |
|  | 50099 | OVERRADAR | 15 | QUESMRK1 |

| No | Name | Match |  |  |
| --- | --- | --- | --- | --- |
| 12 | default | [@primitive='Point'] |  |  |
|  | ViewingGroup | DisplalyPlane | DrawingPriority | Symbol |
|  | 50099 | OVERRADAR | 15 | QUESMRK1 |

| No | Name | Match |  |  |
| --- | --- | --- | --- | --- |
| 13 | default | [@primitive='Surface'] |  |  |
|  | ViewingGroup | DisplalyPlane | DrawingPriority | LineStyle |
|  | 50099 | OVERRADAR | 15 | Dash , 0.32 , CHMGD |

- default.xsl		(This is the same as that used for S-101.)

- attributeRules.xsl  	(This is the same as that used for S-101.)

- simpleLineStyle.xsl  	(This is the same as that used for S-101.)

- textStyle.xsl 		(This is the same as that used for S-101.)

### 4.9.10 data product format(encoding)

### 4.9.10.1 Introduction

The GML encoding of Port Guideline datasets is based on the S-100 profile of GML 3.2.1.

Format Name: Port Guideline GML

Version:	1.0

Character Set: UTF-8

Specification: SMART-Navigation Product Specification

### 4.9.10.2 Encoding of Latitude and Longitude

Values of latitude and longitude must be accurate to 7 decimal places. Coordinates must be encoded as decimals in the format described below. The encoding is indicated by multiplication factor fields defined in the dataset identification record by the S-100 GML schemas.

### 4.9.10.2.1 Encoding of coordinates as decimals

Values should be coded as decimal numbers with 7 or fewer digits after the decimal. The normative encoding is in degrees, with an accuracy of 10-7 degrees, i.e., 7 digits after the decimal point.

The decimal point must be indicated by the “.” character.

Trailing zeroes after the decimal point (and the decimal point itself if appropriate) may be omitted at producer discretion, but the accuracy must still be as indicated (e.g., 10-7 degrees for coordinates of default accuracy).

Latitude and longitude multiplication factors held in the Dataset Structure Information field under [coordMultFactorX] and [coordMultFactorY] must be set to a value corresponding to the encoding, i.e., {1} for coordinates encoded in decimal degrees.

EXAMPLE 1 A longitude = 42.0000 is converted into X = longitude * coordMultFactorX = 42.0000 * 1 = 42.0000000.

### 4.9.10.3 Text Attribute

Character strings must be encoded using the character set defined in ISO 10646-1, in Unicode Transformation Format-8 (UTF-8).

### 4.9.10.4 Mandatory Attribute Values

There are four reasons why attribute values may be considered mandatory:

- They determine whether a feature is in the display base,

- Certain features make no logical sense without specific attributes,

- Some attributes are necessary to determine which symbol is to be displayed,

- Some attributes are required for safety of navigation.

All mandatory attributes are identified in the Feature Catalogue.

### 4.9.10.5 Unknow Attribute Values

When a mandatory attribute code or tag is present but the attribute value is missing, it means that the producer wishes to indicate that this attribute value is unknown.  Missing mandatory attributes must be “nilled” with a GML nilReason attribute giving the reason for omission.

Optional attributes must be omitted altogether if the value is unknown or missing. They must not be “nilled.”

### 4.9.10.6 Structure of dataset files

### 4.9.10.6.1 Sequence of dataset files

The order of data objects in each dataset file is described below:

Dataset Identification Information

Dataset structure information

Spatial records for by-reference geometries

Point

Multi point

Curve

Composite Curve

Surface

Information objects

Feature objects (Geometry may be encoded inline or by reference.)

Meta features

Geo features

### 4.9.10.7 Numeric Attribute Encoding

Floating point and integer attribute values must not contain leading zeros.  Floating point attribute values must not contain non-significant trailing zeros.

### 4.9.10.8 Object identifiers

The “name” of feature records must provide a unique world-wide identifier of feature records. The “name” of the record is the combination of the subfields agency, featureObjectIdentifier, and featureIdentificationSubdivision elements of the featureObjectIdentifier element of the object.

Features, information types, collection objects, meta features, and geometries (inline or external) are all required by the schema to have a gml:id attribute with a value that is unique within the dataset. The gml:id values must be used as the reference for the object from another object in the same dataset or another dataset.

### 4.9.10.9 Dataset validation

Fields may be repeated or omitted as permitted by the XML schemas and the validation tests. Since XML schema cannot encode rules for conditional presence or attributes, these rules are checked by Schematron rules or other validation code.

### 4.9.10.10 Location of Data Product Format schema Files

The schema file will provide a future location and is available from that location.

### 4.9.10.11 Detailed documentation of Port Guideline XML Schema

See DCEG for this section.

### 4.9.11 data product delivery

This clause specifies the encoding and delivery mechanisms for a Port Guideline dataset.  Data which conforms to this product specification must be delivered by means of an exchange set.

### 4.9.11.1 Exchange set

Port Guideline datasets are grouped into exchange sets. Each exchange set consists of one or more Port Guideline datasets with an associated XML metadata file and a single Exchange Catalogue XML file containing metadata. It may also include one or more support files.

Units of Delivery:				Exchange Set

Transfer Size:					Unlimited

Medium Name:					Digital data delivery

Other Delivery Information:

Each dataset must be contained in a physically separate, uniquely identified file on the transfer medium.

Each exchange set has a single exchange catalogue which contains the discovery metadata for each dataset and references to any support files.

Support files are supplementary information which are linked to the features and information types by attributes. The attributes containing these links are described in the application schema and feature catalogue.

An exchange set may be encapsulated into a form suitable for transmission by a mapping called a transmission encoding. An encoding translates each of the elements of the exchange set into a logical form suitable for writing to media and for transmission online. An encoding may also define other elements in addition to the exchange set contents (i.e., media identification, data extents etc…) and also may define commercial constructs such as encryption and compression methods.

If the data is transformed (e.g., for encryption or compression purposes) its content must not be changed.

This product specification does not define the transmission encoding which must be used as a default for transmission of data between parties.

The exchange set elements are as follows:

Mandatory Elements

- Port Guideline datasets – GML encoding of features/attributes and their associated geometry and metadata.

- Exchange Catalogue – the XML encoded representation of exchange set catalogue features [discovery metadata].

Optional Elements

- Supplementary files – These are contained within the exchange set as files and the map from the name included within the dataset and the physical location on the media is defined within the Exchange Catalogue.

- Feature Catalogue – If it is necessary to deliver the latest feature catalogue to the end user it may be done using the Port Guideline exchange set mechanism for datasets – i.e., include the updated feature catalogue in an exchange set.

- Portrayal Catalogue - If it is necessary to deliver the latest portrayal catalogue to the end user it may be done using the Port Guideline exchange set mechanism for datasets datasets – i.e., include the updated feature catalogue in an exchange set.

### 4.9.11.2 Dataset

### 4.9.11.2.1 Datasets

Datasets are distributed as files which are part of exchange sets structured as described in this specification. The distribution media are left to the discretion of the producer and distributor.

The following types of dataset files may be produced and contained within an exchange set:

- New dataset and new edition of a dataset (base dataset):  Each new edition of a dataset must have the same name as the dataset that it replaces.  A new edition can also be AtoN data that has previously been produced for this area.

### 4.9.11.2.2 Dataset size

Datasets must not exceed 1 MB.

### 4.9.11.2.3 Dataset file naming

Dataset files shall be named

CCXXXXXXXX_EEE.GML

The file name forms a unique identifier where:

- CC - the first two characters identify the issuing agency (mandatory).

- The third to tenth characters are optional and may be used in any way by the producer to provide the unique file name. The following characters are allowed in the dataset name, A to Z, 0 to 9 and the special character _ (underscore).

- EEE – new editions and re-issues use 000, and increment until a limit of 999 (mandatory).

- The maximum number of characters preceding _EEE is ten.

- GML – the character sequence “GML” or “gml”.

### 4.9.11.2.4 New Editions and re-issues

This section defines the sequencing of datasets for New Editions. In order to ensure that feature type updates are incorporated into an SYSTEM in the correct sequence without any omission, a number of parameters encoded in the data header and/or metadata are used in the following way:

| edition number | When a dataset is initially created, the edition number 1 is assigned to it. The edition number is increased by 1 at each new edition. |
| --- | --- |

### 4.9.11.2.5 Cancellations

Dataset cancellation is not considered I this product specification. In the SMART Navigation project, the system automatically cancels the dataset after 7 days from the date the dataset is received.

### 4.9.11.3 Support Files

N/A (Support file is not included in this model)

### 4.9.11.4 Exchange Catalogue

N/A (Exchange Catalogue is not used in SMART Navigation)

### 4.9.12 meta data

### 4.9.12.1 Introduction

The Port Guideline metadata description is based on the S-100 metadata document section, which is a profile of the ISO 19115 standard. These documents provide a structure for describing digital geographic data and define metadata elements, a common set of metadata terminology, definitions and extension procedures.

Two metadata packages are described in this product specification: dataset metadata and exchange set metadata.

Figure 21 Metadata packages

Note 1: Types with CI_, EX_, and MD_ prefixes are from packages defined in ISO 19115 and adapted by S-100. Types with S100_ prefix are from packages defined in S-100.

Note 2: When a dataset is terminated, the purpose metadata field is set to 3 (terminated), and the editionNumber metadata field is set to 0. All other metadata fields must be blank.

### 4.9.12.2 Dataset metadata

Dataset metadata is intended to describe information about a dataset. It facilitates the management and exploitation of data and is an important requirement for understanding the characteristics of a dataset. Whereas dataset metadata is usually fairly comprehensive, there is also a requirement for a constrained subset of metadata elements that are usually required for discovery purposes. Discovery metadata are often used for building web catalogues, and can help users determine whether a product or service is fit for purpose and where they can be obtained.

Table 31 Dataset metadata

| Name | Cardinality | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| S100_DataSetDiscoveryMetadata |  |  |  |  |
| fileName | 1 |  | CharacterString | Dataset file name |
| filePath | 1 |  | CharacterString | Full path from the exchange set root directory |
| description | 1 |  | CharacterString | Short description of the area covered by dataset, e.g., area, harbour, or port name, between two named locations etc. |
| dataProtection | 0..1 | {1} or {2} | Boolean | True : Encrypted False : Unencrypted |
| protectionScheme | 0..1 |  | CharacterString |  |
| digitalSignature | 0..1 |  | CharacterString |  |
| digitalSignatureReference | 0..1 |  | CharacterString |  |
| digitalSignatureValue | 0..1 |  | CharacterString |  |
| copyright | 0..1 |  | MD_LegalConstraints>MD_RestrictionCode <copyright> (ISO 19115-1) |  |
| classification | 0..1 |  | Class MD_SecurityConstraints>MD_ClassificationCode (codelist) ISO 19115-1 | 1. unclassified 2. restricted 3. confidential 4. secret 5. top secret |
| purpose | 1 | {1}, {2} | CharacterString | - New dataset - New edition |
| specificUsage | 1 |  | MD_USAGE>specificUsage (character string) MD_USAGE>userContactInfo (CI_Responsibility) | Brief description of the resource and/or resource series usage |
| editionNumber | 1 | {1} | Integer | When a dataset is initially created, the edition number “1” is assigned to it. The edition number is increased by one with each new edition. |
| updateNumber | 1 |  | CharacterString | Update number 0 is assigned to a new dataset. |
| updateApplicationDate | 1 |  | Date | All updates dated on or before this date must have been applied by the producer. |
| issueDate | 1 |  | Date | Date on which the dataset was generated. |
| productSpecification | 1 |  | S100_ProductSpecification | See Notes below this table for constraints on values. |
| producingAgency | 1 |  | CI_Responsibility>CI_Organisation or CI_Responsibility>CI_Individual | Party responsible for generating the dataset. |
| optimumDisplayScale | 0..1 |  | Integer |  |
| maximumDisplayScale | 0..1 |  | Integer |  |
| minimumDisplayScale | 0..1 |  | Integer |  |
| horizontalDatumReference | 1 | EPSG | CharacterString |  |
| horizontalDatumValue | 1 | 4326 | Integer | WGS84 |
| verticalDatum | 0..1 |  | S100_VerticalAndSoundingDatum |  |
| soundingDatum | 0..1 |  | S100_VerticalAndSoundingDatum |  |
| dataType | 1 | GML | CharacterString | The only value allowed is “GML”. |
| otherDataTypeDescription | 0..1 |  | CharacterString |  |
| dataTypeVersion | 1 | 3.2.1 | CharacterString |  |
| dataCoverage | 1..* |  | S100_DataCoverage |  |
| comment | 0..1 |  | CharacterString | Any additional Information |
| layerID | 1..* | S-101 | CharacterString | Dataset must be used with ENC in an ECDIS. |

### 4.9.12.3 Update Dataset Metadata

N/A (Datset update is not relevant in this model)

### 4.9.12.4 Support file metadata

N/A (Support file is not included in this model)

### 4.9.12.5 Exchange set metadata

N/A (Exchange Catalgoue is not used in SMART-Navigation)

### 4.9.12.6 Catalogue file Metadata

All Port Guideline Exchange Catalogue files must contain at least the mandatory metadata elements.

Table 32 Catalogue file metadata

| Name | Multiplicity | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| S100_ExchangeCatalogue |  |  |  |  |
| identifier | 1 |  | S100_CatalogueIdentifier |  |
| contact | 1 |  | S100_CataloguePointOfContact |  |
| productSpecification | 0..1 |  | S100_ProductSpecification | Conditional on all the datasets using the same product specification. |
| metadataLanguage | 1 | English | CharacterString | All datasets conforming to this PS must use English language. A catalogue in English must be provided. Discovery metadata elements within catalogues have their own locale attributes and may be repeated in languages other than English. |
| exchangeCatalogueName | 1 |  | CharacterString | Catalogue filename |
| exchangeCatalogueDescription | 1 |  | CharacterString |  |
| exchangeCatalogueComment | 0..1 |  | CharacterString | Any additional Information |
| compressionFlag | 0..1 |  | Boolean | Yes or No |
| sourceMedia | 0..1 |  | CharacterString |  |
| replacedData | 0..1 |  | Boolean |  |
| dataReplacement | 0..1 |  | CharacterString |  |

## 4.10 ship density

### 4.10.1 Overview

### 4.10.1.1 Introduction

This document has been produced by the member of SMART-Navigation project team(Korean Register, Marine-Works) in response to a requirement to produce a data product that can be used as ship density level in specific port within the Portable Pilot Unit(PPU). .  It is based on the IHO S-100 framework specification and the ISO 19100 series of standards. It is a textTable product specification that is primarily intended for encoding ship density, ship density information.

### 4.10.1.2 References

IHO S-100 IHO Universal Hydrographic Data Model Edition 4.0.0 (December 2017).

ISO 8601. 2004. Data elements and interchange formats - Information interchange - Representation of dates and times. 2004.

ISO 3166-1. 1997. Country Codes. 1997.

ISO 10646-1 Information technology - Universal Multiple-Octet Coded Character Set (UCS) - Part 1: Architecture and Basic Multilingual Plane

ISO 19101-2:2008 Geographic Information - Rules for Application Schema

ISO/TS 19103:2005 Geographic Information - Conceptual schema language

ISO 19106:2004 Geographic Information – Profiles

ISO 19107:2003 Geographic Information – Spatial schema

ISO 19109:2005 Geographic Information - Rules for Application Schema

ISO 19111:2003 Geographic Information - Spatial referencing by coordinates

ISO 19115:2003+Corr1 (2006) Geographic Information - Metadata

ISO 19115-2:2009 Geographic Information - Metadata: Extensions for imagery and gridded data

ISO 19123:2005 Geographic Information - Schema for coverage geometry and functions

ISO 19129:2009 Geographic Information - Imagery gridded and coverage data framework

ISO 19131:2007 Geographic Information - Data product specifications

ISO 19136:2007 Geographic Information – Geography Markup Language

ISO 19136-2:2015, Geographic Information – Geography Markup Language.

ISO/TS 19139, Geographic Information – Metadata – XML schema implementation.

### 4.10.1.3 Terms, definitions and abbreviations

Terms and Definitions

The S-100 framework is based on the ISO 19100 series of geographic standards. The terms and definitions provided here are used to standardize the nomenclature found within that framework, whenever possible. They are taken from the references cited in clause 2.1. Modifications have been made when necessary.

- Application

Manipulation and processing of data in support of user requirements (ISO 19101)

- Application schema

Conceptual schema for data required by one or more applications (ISO 19101)

- Conceptual model

Model that defines concepts of a universe of discourse (ISO 19101)

- Conceptual schema

Formal description of a conceptual model (ISO 19101)

- Coverage

Feature that acts as a function to return values from its range for any direct position within its spatial, temporal or spatiotemporal domain (ISO 19123)

- Data product

Dataset or dataset series that conforms to a data product specification

- Data product specification

Detailed description of a dataset or dataset series together with additional information that will enable it to be created, supplied to and used by another party

- Dataset

Identifiable collection of data (ISO 19115)

- Feature

Abstraction of real world phenomena (ISO 19101)

- Feature association

Relationship that links instances of one feature type with instances of the same or a different feature type (ISO19110)

- Feature attribute

Characteristic of a feature (ISO 19101)

- Geographic data

Data with implicit or explicit reference to a location relative to the Earth (ISO 19109)

- Metadata

Data about data (ISO 19115)

- Model

Abstraction of some aspects of reality (ISO 19109)

- Portrayal

Presentation of information to humans (ISO 19117)

- Quality

Totality of characteristics of a product that bear on its ability to satisfy stated and implied needs (ISO 19101)

Abbreviations

This product specification adopts the following convention for symbols and abbreviated terms:

- ASCII 	American Standard Code for Information Interchange

- ECDIS	Electronic Chart Display and Information Systems

- ECS	Electronic Chart Systems

- ENC	Electronic Navigational Chart

- GML	Geography Markup Language

- IHO	International Hydrographic Organization

- IOC	International Oceanographic Commission

- ISO	International Organization for Standardization

- UML	Unified Modelling Language

- URI	Uniformed Resource Identifier

- URL 	Uniform Resource Locator

- WMS	Web Map Service

- WFS	Web Feature Service

- www 	World Wide Web

- XML	Extensible Markup Language

- XSLT	eXtensible Stylesheet Language Transformations

### 4.10.1.4 General Data Product Description

Informal Description of Data Product

This clause contains general information about the data product

- Title:  	Ship density Product Specification.

- Abstract: 	 Ship density product specification describe the means to express density level by ship in specific port within an Portable Pilot Unit(PPU). This may include details on grid increment value latitude, grid increment value longitude, density value, density zone name, ship count.

- Content:	Datasets conforming to this specification will contain ship density level information for the area of coverage, additionally, there will be relevant metadata data quality, production authority, data sources and publication date.

- Spatial Extent:	Korean Coast

- Specific Purpose:	Describing ship density level information for utilization of Pilot & Tug Assistance service.

### 4.10.1.5 Data product specification metadata

This information uniquely identifies this Product Specification and provides information about its creation and maintenance. For further information on dataset metadata see the metadata clause.

- Title: 	Ship Density

- S-100 Version: 	4.0.0

- Ship Density Version: 	1.0.0

- Date: 	2019-03-08

- Language: 	English

- Classification: 	Unclassified

- Contact: 	32, Yuseong-daero 1312beon-gil, Yuseong-gu, Daejeon, Korea 32(Jang Dong) inside KRISO

- URL: 	http://www.smart-navigation.org

- Identifier: 	Ship Density

- Maintenance: 	Amendments to this specification will be produced on a needs basis. For reporting issues with this specification which need correction, use the contact information.

### 4.10.2 Specification Scopes

This product specification describes one data product and therefore requires only one scope which is described below:

- Scope ID:  		Ship Density datasets.

- Hierarchical level: 		MD_ScopeCode - 005

- Hierarchical level name: 	dataset.

- Level description:		information applies to the dataset

- Extent:	EX_Extent.description: Korean Coast

### 4.10.3 Dataset identification

This section describes how to identify data sets that conform to the specification. A dataset that conforms to this Product Specification may be identified by its discovery metadata as defined in clause 12 of this specification. The information identifying the data product may include the following items from S-100 4.0.0 clause 11-6 (adapted from ISO 19115).

| Title | Ship Density |
| --- | --- |
| Abstract | Ship Density dataset is a textTable dataset containing ship density level regarding the Pilot & Tug Assistance service within a defined geographical area. |
| Acronym | SDY |
| Content | Grid increment value latitude, Grid increment value longitude, Density value, Density zone Name, Ship count |
| Topic category | Transportation(vehicle or vessel location) |
| geographicDescription | Area specific to marine navigation within korean sea area |
| Purpose | Standardization of sea-area status information in port |
| Language | EN Additional values if any use CharacterString values from ISO 639-2 |

### 4.10.4 Data Content and structure

### 4.10.4.1 Introduction

The Ship Density product is based on the S-100 General Feature Model (GFM), and is a feature-based vector product. All Ship Density features and information classes are derived from one of the abstract classes FeatureType and InformationType defined in the Ship Density respectively.

This clause contains the Application Schema expressed in UML and an associated Feature Catalogue.

### 4.10.4.2 Application Schema

The UML models in this clause are segments of the overall Ship Density application schema, and include overviews of the feature classes, information classes, meta features, spatial types, and the relationships between them.

This clause contains a general overview of the classes and relationships in the Ship Density application schema. Detailed information about how to use the feature types and information types to encode Ship Density information is provided in the SMART-Navigation Data Classification and Encoding Guide (DCEG).

The following conventions are used in the UML diagrams depicting the application schema:

- Standard UML conventions for classes, associations, inheritance, roles, and multiplicities apply. These conventions are described in Part 1 of S-100.

- Feature classes are depicted with pink background; the dark shade for abstract feature classes and the light shade for ordinary (non-abstract) feature classes.

- No significance attaches to the colour of associations. (Complex diagrams may use different colours to distinguish associations that cross one another.)

### 4.10.4.2.1 Domain model

Figure 22 Ship Density Overview

### 4.10.4.3 Feature Catalogue

### 4.10.4.3.1 Introduction

The Feature Catalogue describes the feature types, information types, attributes, attribute values, associations and roles which may be used in the product.  The Ship Density Feature Catalogue is available in an XML document which conforms to the S-100 XML Feature Catalogue Schema.  Simple attributes used in this specification are listed in below.

Name: 	Ship Density Feature Catalogue

Scope: 	Smart-Navigation

Version Number: 	1.0.0

Version Date:	2019-03-15

Producer:	SMART Navigation Project

Language: 	                 English

### 4.10.4.3.1.1 Feature Types

Feature types contain descriptive attributes that characterize real-world entities. The word ‘feature’ may be used in one of two senses – feature type and feature instance. A feature type is a class and is defined in a Feature Catalogue. A feature instance is a single occurrence of the feature type and represented as an object in a dataset. A feature instance is located by a relationship to one or more spatial instances. A feature instance may exist without referencing a spatial instance.

### 4.10.4.3.1.2 Geographic

A Geographic (Geo) feature type carries the descriptive characteristics of a real-world entity.

### 4.10.4.3.1.3 Meta

Meta features contain information about other features within a dataset. Information defined by meta features override the default metadata values defined by the dataset descriptive records. Meta attribution on individual features overrides attribution on meta features.

### 4.10.4.3.1.4 Feature Relationship

A feature relationship links instances of one feature type with instances of the same or a different feature type.

### 4.10.4.3.1.5 Information Types

Information types are identifiable pieces of information in a dataset that can be shared between other features. They have attributes but have no relationship to any geometry; information types may reference other information types.

### 4.10.4.3.1.6 Attributes

SMART-Navigation product defines attributes as either simple or complex.

### 4.10.4.3.1.6.1 Simple Attributes

SMART-Navigation product uses nine types of simple attributes; they are listed in the following table:

| Type | Definition |
| --- | --- |
| Enumeration | A fixed list of valid identifiers of named literal values |
| Boolean | A value representing binary logic. The value can be either True or False. The default state for Boolean type attributes (i.e. where the attribute is not populated for the feature) is False. |
| Real | A signed Real (floating point) number consisting of a mantissa and an exponent |
| Integer | A signed integer number. The representation of an integer is encapsulation and usage dependent. |
| Text | An arbitrary-length sequence of characters including accents and special characters from a repertoire of one of the adopted character sets |
| Date | A date provides values for year, month and day according to the Gregorian Calendar. Character encoding of a date is a string which must follow the calendar date format (complete representation, basic format) for date specified by ISO 8601:1988. EXAMPLE 19980918 (YYYY-MM-DD) |
| Time | A time is given by an hour, minute and second. Character encoding of a time is a string that follows the local time (complete representation, basic format) format defined in ISO 8601:1988. EXAMPLE 183059 or 183059+0100 or 183059Z |
| Date and Time | A DateTime is a combination of a date and a time type. Character encoding of a DateTime shall follow ISO 8601:1988 EXAMPLE 19850412T101530 |
| Codelist | A type of flexible enumeration. A code list type is a list of literals which may be extended only in conformance with specified rules. |

### 4.10.4.3.1.6.2 Complex Attributes

Complex attributes are aggregations of other attributes that are either simple or complex. The aggregation is defined by means of attribute bindings.

### 4.10.4.3.1.6.3 Geometric Representation

Geometric representation is the digital description of the spatial component of an object as described in S-100 and ISO 19107. This product specification uses three types of geometries.

### 4.10.4.3.2 Feature Catalogue General

The Ship Density feature catalogue describes the feature types, information types, attributes, attribute values, associations and roles which may be used in a Ship Density data set.

### 4.10.4.3.2.1 Feature Types

- ShipDensity

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| shipMRN | MRN is a naming scheme that can uniquely identify any maritime resource on a global scale. shipMRN is the unique value given to the ship. | URN | 1 | mandatory |  |
| MMSICode | The Maritime Mobile Service Identity (MMSI) Code is formed of a series of nine digits which are transmitted over the radio path in order to uniquely identify ship stations, ship earth stations, coast stations, coast earth stations, and group calls. These identities are formed in such a way that the identity or part thereof can be used by telephone and telex subscribers connected to the general telecommunications network principally to call ships automatically. (Adapted from Appendix 43 of the International Telecommunications Union Radio Regulations) | integer | 1 | mandatory |  |
| IMONo | The seven-digit number assigned to the vessel, the vessel management company, and the ship documents company under the SOLAS rules. | integer | 0..1 | option |  |
| issueDate | The production date & time of the source, e.g., the date of measurement. | dateTime | 0..1 | option |  |
| gridIncrementValueLatitude | - | real | 0..1 | option |  |
| gridIncrementValueLongitude | - | real | 0..1 | option |  |
| startZone | - | GM_Point | 1..1 | mandatory |  |

- ShipDensityInformation

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| densityZoneName | - | text | 1 | mandatory |  |
| densityValue | Vessel density within a specific area. | real | 1 | mandatory |  |
| shipCount | Number of ships in the zone. In this case, the vessel counting the number refers to the vessel registered on the e-Navi operation DB (the vessel recognized by the GICOMS system (V-Pass, AIS)). | integer | 1 | mandatory |  |

- Meta

- Cartographic

- Theme

### 4.10.4.3.2.2 Feature Relationship

| Role Type | Association Name | Role | Feature | Multiplicity |
| --- | --- | --- | --- | --- |
| Composition | densityOfGrid | densityInformationProvideFor | ShipDensityInformation | 1 |
| Composition | densityOfGrid | providesDensityInformation | ShipDensity | 1..* |

### 4.10.4.3.2.3 Information Relationship

- N/A

### 4.10.4.3.2.4 Information Type

- N/A

### 4.10.4.3.2.5 Simple Attribute

- Simple Attribute List

| Name | Description | Data Type | Remarks |
| --- | --- | --- | --- |
| shipMRN | MRN is a naming scheme that can uniquely identify any maritime resource on a global scale. shipMRN is the unique value given to the ship. | URN |  |
| MMSICode | The Maritime Mobile Service Identity (MMSI) Code is formed of a series of nine digits which are transmitted over the radio path in order to uniquely identify ship stations, ship earth stations, coast stations, coast earth stations, and group calls. These identities are formed in such a way that the identity or part thereof can be used by telephone and telex subscribers connected to the general telecommunications network principally to call ships automatically. (Adapted from Appendix 43 of the International Telecommunications Union Radio Regulations). | integer |  |
| IMONo | The seven-digit number assigned to the vessel, the vessel management company, and the ship documents company under the SOLAS rules. | integer |  |
| issueDate | The production date & time of the source, e.g., the date of measurement. | dateTime |  |
| gridIncrementValueLatitude | - | real |  |
| gridIncrementValueLongitude | - | real |  |
| densityZoneName | Density Zone Name | text |  |
| shipCount | Number of ships in the zone. | integer |  |
| densityValue | Vessel density within a specific area. | real |  |

### 4.10.4.3.2.6 Complex Attribute

- N/A

### 4.10.5 Coordinate Reference System(CRS)

### 4.10.5.1 Introduction

The location of an object in the S-100 standard is defined by means of coordinates which relate a feature to a position. The coordinate reference system used for this product specification is World Geodetic System 1984 (WGS 84) which is defined by the European Petroleum Survey Group (EPSG) code 4326, (or similar - North American Datum 1983 / Canadian Spatial Reference System).

Spatial data are expressed as latitude (φ) and longitude (λ) geographic coordinates. Latitude values are stored as a negative number to represent a position south of the Equator. Longitude values are stored as a negative number to represent a position west of the Prime Meridian. Coordinates are expressed as real value, degree / degree decimal format. Datasets conforming to this product specification are not projected.

Horizontal coordinate reference system:	WGS 84

Projection: 					None

Vertical coordinate reference system: 	None

Temporal reference system: 		Gregorian calendar

Coordinate reference system registry: 	EPSG Geodetic Parameter Registry

Date type (according to ISO 19115-1):  	002 - publication

### 4.10.5.2 Horizontal Reference System

Positional data is expressed in latitude and longitude geographic coordinates to one of the reference horizontal reference systems defined in the HORDAT attribute.  Unless otherwise defined, the World Geodetic System 84 (WGS 84) will be used for Ship Density data products.

### 4.10.5.3 Projection

Ship Density data products are un-projected.

### 4.10.5.4 Vertical Coordinate Reference System

Ship Density data products do not provide detailed vertical information.

### 4.10.5.5 Temporal Reference System

Time is measured by reference to Calendar dates and Clock time in accordance with ISO 19108:2002 Temporal Schema clause 5.4.4.

### 4.10.5.6 Ship Density Data and Scale

Ship Density data must be compiled in the best applicable scale. The use of the data itself is "scale independent". That means that the data can be used at any scale. S-100 allows the association of multiple spatial attributes to a single feature instance. In principle, each of these spatial attributes can be qualified by maximum and minimum scales.

For example, it is possible, within one dataset, to have a single instance of a feature that has more than one area geometry. Each of these geometries has different scale max/min attributes. Moreover, due to cluttering in smaller scales, the scale minimum attribute may be used to turn off portrayal of some features at smaller scales.

### 4.10.6 Data Quality

### 4.10.6.1 Introduction

Ship Density products must be tested with the Ship Density specific checks prior to release by the data producer. The data producer must review the check results and address any issues to ensure sufficient quality of the data products. The checks are a mix of data format validation checks, conformance to standard checks and logical consistency checks. But the checklist has not been written yet. It will be updated in the next version.

### 4.10.7 Data Capture and classification

See DCEG for this section.

### 4.10.8 maintenance

### 4.10.8.1 Introduction

Datasets are maintained as needed and must include mechanisms for Ship Density updating. Data updates will be made by new editions. The maintenance and update frequency of Ship Density datasets should be defined by the producers (official national authority) implementing this specification.

Data Producers must use applicable sources to maintain and update data and provide a brief description of the sources that were used to produce the dataset in the appropriate metadata field.

The data product shall provide information on how the data is maintained and should describe the principles and criteria applied in maintenance regime. This should specify the expected frequency of updates.

Table 33 Maintenance and update frequency

| Item Name | Description | Multiplicity | Type |
| --- | --- | --- | --- |
| maintenanceAndUpdateFrequency | Frequency with which changes and additions are made to the data product (per update scope) | 1..* | MD_MaintenanceInformation (ISO 19115) |
| dataSource | Identification of the kinds of data sources usable to produce datasets | 1..* | LI_Source (ISO 19115) |
| productionProcess | Textual description of the production process applicable to the datasets (per scope or data source) | 1..* | LI_ProcessStep (ISO 19115) |

### 4.10.8.2 Production process for base and update datasets

Data Producers should follow their established production processes for maintaining and updating datasets. Data is produced against the DCEG and checked against the appropriate set of validation rules in Appendix E. (Appendix E is not included in this release.)

### 4.10.8.3 Datasetr updates

N/A (Dataset update is not relevant in this model)

### 4.10.8.4 Support file updates

N/A (Support file is not included in this model)

### 4.10.8.5 Feature and portrayal catalogues

For each new version of the Ship Density Product Specification a new feature catalogue will be released. The system must be able to manage datasets and their catalogues that are created on different versions of the Ship Ensity product specification (Portrayal catalogue is not defined in this model)

### 4.10.8.6 Feature history, versions, and change tracking

If applications or production systems require versioning of individual instances of feature or information types, maintenance of histories, or change tracking, the methods for versioning, history management, and change tracking and display are left to the application or production system.

### 4.10.8.7 Dataset encryption

Details about dataset encryption are still to be determined, and may mirror the method described in S-101.

### 4.10.9 Portrayal

Portrayal is not defined in this version of Ship Density Product Specifications. Users are free to choose the means and methodology of portrayal as they see best suited for their needs. It should be noted that future versions of Ship Density may include a portrayal catalogue, and any implementer should therefore anticipate this, and make sufficient provisions in any system supporting Ship Density.

### 4.10.10 data product format(encoding)

### 4.10.10.1 Introduction

The GML encoding of Ship Density datasets is based on the S-100 profile of GML 3.2.1.

Format Name: Ship Density GML

Version:	1.0

Character Set: UTF-8

Specification: SMART-Navigation Product Specification

### 4.10.10.2 Encoding of Latitude and Longitude

Values of latitude and longitude must be accurate to 7 decimal places. Coordinates must be encoded as decimals in the format described below. The encoding is indicated by multiplication factor fields defined in the dataset identification record by the S-100 GML schemas.

### 4.10.10.2.1 Encoding of coordinates as decimals

Values should be coded as decimal numbers with 7 or fewer digits after the decimal. The normative encoding is in degrees, with an accuracy of 10-7 degrees, i.e., 7 digits after the decimal point.

The decimal point must be indicated by the “.” character.

Trailing zeroes after the decimal point (and the decimal point itself if appropriate) may be omitted at producer discretion, but the accuracy must still be as indicated (e.g., 10-7 degrees for coordinates of default accuracy).

Latitude and longitude multiplication factors held in the Dataset Structure Information field under [coordMultFactorX] and [coordMultFactorY] must be set to a value corresponding to the encoding, i.e., {1} for coordinates encoded in decimal degrees.

EXAMPLE 1 A longitude = 42.0000 is converted into X = longitude * coordMultFactorX = 42.0000 * 1 = 42.0000000.

### 4.10.10.3 Text Attribute

Character strings must be encoded using the character set defined in ISO 10646-1, in Unicode Transformation Format-8 (UTF-8).

### 4.10.10.4 Mandatory Attribute Values

There are four reasons why attribute values may be considered mandatory:

- They determine whether a feature is in the display base,

- Certain features make no logical sense without specific attributes,

- Some attributes are necessary to determine which symbol is to be displayed,

- Some attributes are required for safety of navigation.

All mandatory attributes are identified in the Feature Catalogue.

### 4.10.10.5 Unknow Attribute Values

When a mandatory attribute code or tag is present but the attribute value is missing, it means that the producer wishes to indicate that this attribute value is unknown.  Missing mandatory attributes must be “nilled” with a GML nilReason attribute giving the reason for omission.

Optional attributes must be omitted altogether if the value is unknown or missing. They must not be “nilled.”

### 4.10.10.6 Structure of dataset files

### 4.10.10.6.1 Sequence of dataset files

The order of data objects in each dataset file is described below:

Dataset Identification Information

Dataset structure information

Spatial records for by-reference geometries

Point

Multi point

Curve

Composite Curve

Surface

Information objects

Feature objects (Geometry may be encoded inline or by reference.)

Meta features

Geo features

### 4.10.10.7 Numeric Attribute Encoding

Floating point and integer attribute values must not contain leading zeros.  Floating point attribute values must not contain non-significant trailing zeros.

### 4.10.10.8 Object identifiers

The “name” of feature records must provide a unique world-wide identifier of feature records. The “name” of the record is the combination of the subfields agency, featureObjectIdentifier, and featureIdentificationSubdivision elements of the featureObjectIdentifier element of the object.

Features, information types, collection objects, meta features, and geometries (inline or external) are all required by the schema to have a gml:id attribute with a value that is unique within the dataset. The gml:id values must be used as the reference for the object from another object in the same dataset or another dataset.

### 4.10.10.9 Dataset validation

Fields may be repeated or omitted as permitted by the XML schemas and the validation tests. Since XML schema cannot encode rules for conditional presence or attributes, these rules are checked by Schematron rules or other validation code.

### 4.10.10.10 Location of Data Product Format schema Files

The schema file will provide a future location and is available from that location.

### 4.10.10.11 Detailed documentation of Ship Density XML Schema

See DCEG for this section.

### 4.10.11 data product delivery

This clause specifies the encoding and delivery mechanisms for a Ship Density dataset.  Data which conforms to this product specification must be delivered by means of an exchange set.

### 4.10.11.1 Exchange set

Ship Density datasets are grouped into exchange sets. Each exchange set consists of one or more Ship Density datasets with an associated XML metadata file and a single Exchange Catalogue XML file containing metadata. It may also include one or more support files.

Units of Delivery:				Exchange Set

Transfer Size:					Unlimited

Medium Name:					Digital data delivery

Other Delivery Information:

Each dataset must be contained in a physically separate, uniquely identified file on the transfer medium.

Each exchange set has a single exchange catalogue which contains the discovery metadata for each dataset and references to any support files.

Support files are supplementary information which are linked to the features and information types by attributes. The attributes containing these links are described in the application schema and feature catalogue.

An exchange set may be encapsulated into a form suitable for transmission by a mapping called a transmission encoding. An encoding translates each of the elements of the exchange set into a logical form suitable for writing to media and for transmission online. An encoding may also define other elements in addition to the exchange set contents (i.e., media identification, data extents etc…) and also may define commercial constructs such as encryption and compression methods.

If the data is transformed (e.g., for encryption or compression purposes) its content must not be changed.

This product specification does not define the transmission encoding which must be used as a default for transmission of data between parties.

The exchange set elements are as follows:

Mandatory Elements

- Ship Density datasets – GML encoding of features/attributes and their associated geometry and metadata.

- Exchange Catalogue – the XML encoded representation of exchange set catalogue features [discovery metadata].

Optional Elements

- Supplementary files – These are contained within the exchange set as files and the map from the name included within the dataset and the physical location on the media is defined within the Exchange Catalogue.

- Feature Catalogue – If it is necessary to deliver the latest feature catalogue to the end user it may be done using the Ship Density exchange set mechanism for datasets – i.e., include the updated feature catalogue in an exchange set.

### 4.10.11.2 Dataset

### 4.10.11.2.1 Datasets

Datasets are distributed as files which are part of exchange sets structured as described in this specification. The distribution media are left to the discretion of the producer and distributor.

The following types of dataset files may be produced and contained within an exchange set:

- New dataset and new edition of a dataset (base dataset):  Each new edition of a dataset must have the same name as the dataset that it replaces.  A new edition can also be AtoN data that has previously been produced for this area.

### 4.10.11.2.2 Dataset size

Datasets must not exceed 1 MB.

### 4.10.11.2.3 Dataset file naming

Dataset files shall be named

CCXXXXXXXX_EEE.GML

The file name forms a unique identifier where:

- CC - the first two characters identify the issuing agency (mandatory).

- The third to tenth characters are optional and may be used in any way by the producer to provide the unique file name. The following characters are allowed in the dataset name, A to Z, 0 to 9 and the special character _ (underscore).

- EEE – new editions and re-issues use 000, and increment until a limit of 999 (mandatory).

- The maximum number of characters preceding _EEE is ten.

- GML – the character sequence “GML” or “gml”.

### 4.10.11.2.4 New Editions and re-issues

This section defines the sequencing of datasets for New Editions and Re-issues. In order to ensure that feature type updates are incorporated into an SYSTEM in the correct sequence without any omission, a number of parameters encoded in the data header and/or metadata are used in the following way:

| edition number | When a dataset is initially created, the edition number 1 is assigned to it. The edition number is increased by 1 at each new edition. |
| --- | --- |

### 4.10.11.2.5 Cancellations

Dataset cancellation is not considered I this product specification. In the SMART Navigation project, the system automatically cancels the dataset after 7 days from the date the dataset is received.

### 4.10.11.3 Support Files

N/A (Support file is not included in this model)

### 4.10.11.4 Exchange Catalogue

N/A (Exchange Catalogue is not used in SMART Navigation)

### 4.10.12 meta data

### 4.10.12.1 Introduction

The Ship Density metadata description is based on the S-100 metadata document section, which is a profile of the ISO 19115 standard. These documents provide a structure for describing digital geographic data and define metadata elements, a common set of metadata terminology, definitions and extension procedures.

Two metadata packages are described in this product specification: dataset metadata and exchange set metadata.

Figure 23 Metadata packages

Note 1: Types with CI_, EX_, and MD_ prefixes are from packages defined in ISO 19115 and adapted by S-100. Types with S100_ prefix are from packages defined in S-100.

Note 2: When a dataset is terminated, the purpose metadata field is set to 3 (terminated), and the editionNumber metadata field is set to 0. All other metadata fields must be blank.

### 4.10.12.2 Dataset metadata

Dataset metadata is intended to describe information about a dataset. It facilitates the management and exploitation of data and is an important requirement for understanding the characteristics of a dataset. Whereas dataset metadata is usually fairly comprehensive, there is also a requirement for a constrained subset of metadata elements that are usually required for discovery purposes. Discovery metadata are often used for building web catalogues, and can help users determine whether a product or service is fit for purpose and where they can be obtained.

Table 34 Dataset metadata

| Name | Cardinality | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| S100_DataSetDiscoveryMetadata |  |  |  |  |
| fileName | 1 |  | CharacterString | Dataset file name |
| filePath | 1 |  | CharacterString | Full path from the exchange set root directory |
| description | 1 |  | CharacterString | Short description of the area covered by dataset, e.g., area, harbour, or port name, between two named locations etc. |
| dataProtection | 0..1 | {1} or {2} | Boolean | True : Encrypted False : Unencrypted |
| protectionScheme | 0..1 |  | CharacterString |  |
| digitalSignature | 0..1 |  | CharacterString |  |
| digitalSignatureReference | 0..1 |  | CharacterString |  |
| digitalSignatureValue | 0..1 |  | CharacterString |  |
| copyright | 0..1 |  | MD_LegalConstraints>MD_RestrictionCode <copyright> (ISO 19115-1) |  |
| classification | 0..1 |  | Class MD_SecurityConstraints>MD_ClassificationCode (codelist) ISO 19115-1 | 1. unclassified 2. restricted 3. confidential 4. secret 5. top secret |
| purpose | 1 | {1}, {2} | CharacterString | - New dataset - New edition |
| specificUsage | 1 |  | MD_USAGE>specificUsage (character string) MD_USAGE>userContactInfo (CI_Responsibility) | brief description of the resource and/or resource series usage |
| editionNumber | 1 | {1} | Integer | When a dataset is initially created, the edition number “1” is assigned to it. The edition number is increased by one with each new edition. |
| updateNumber | 1 |  | CharacterString | Update number 0 is assigned to a new dataset. |
| updateApplicationDate | 1 |  | Date | All updates dated on or before this date must have been applied by the producer. |
| issueDate | 1 |  | Date | Date on which the dataset was generated. |
| productSpecification | 1 |  | S100_ProductSpecification | See Notes below this table for constraints on values. |
| producingAgency | 1 |  | CI_Responsibility>CI_Organisation or CI_Responsibility>CI_Individual | Party responsible for generating the dataset. |
| optimumDisplayScale | 0..1 |  | Integer |  |
| maximumDisplayScale | 0..1 |  | Integer |  |
| minimumDisplayScale | 0..1 |  | Integer |  |
| horizontalDatumReference | 1 | EPSG | CharacterString |  |
| horizontalDatumValue | 1 | 4326 | Integer | WGS84 |
| verticalDatum | 0..1 |  | S100_VerticalAndSoundingDatum |  |
| soundingDatum | 0..1 |  | S100_VerticalAndSoundingDatum |  |
| dataType | 1 | GML | CharacterString | The only value allowed is “GML”. |
| otherDataTypeDescription | 0..1 |  | CharacterString |  |
| dataTypeVersion | 1 | 3.2.1 | CharacterString |  |
| dataCoverage | 1..* |  | S100_DataCoverage |  |
| comment | 0..1 |  | CharacterString | Any additional Information |
| layerID | 1..* | S-101 | CharacterString | Dataset must be used with ENC in an ECDIS. |

### 4.10.12.3 Update Dataset Metadata

N/A (Dataset update is not relevant in this model)

### 4.10.12.4 Support file metadata

N/A (Support file is not included in this model)

### 4.10.12.5 Exchange set metadata

N/A (Exchange Catalogue is not used in SMART Navigation)

### 4.10.12.6 Catalogue file Metadata

All Ship Density Exchange Catalogue files must contain at least the mandatory metadata elements.

Table 35 Catalogue file metadata

| Name | Multiplicity | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| S100_ExchangeCatalogue |  |  |  |  |
| identifier | 1 |  | S100_CatalogueIdentifier |  |
| contact | 1 |  | S100_CataloguePointOfContact |  |
| productSpecification | 0..1 |  | S100_ProductSpecification | Conditional on all the datasets using the same product specification.. |
| metadataLanguage | 1 | English | CharacterString | All datasets conforming to this PS must use English language. A catalogue in English must be provided. Discovery metadata elements within catalogues have their own locale attributes and may be repeated in languages other than English. |
| exchangeCatalogueName | 1 |  | CharacterString | Catalogue filename |
| exchangeCatalogueDescription | 1 |  | CharacterString |  |
| exchangeCatalogueComment | 0..1 |  | CharacterString | Any additional Information |
| compressionFlag | 0..1 |  | Boolean | Yes or No |
| sourceMedia | 0..1 |  | CharacterString |  |
| replacedData | 0..1 |  | Boolean |  |
| dataReplacement | 0..1 |  | CharacterString |  |

## 4.11 MIO Service MEta

### 4.11.1 Overview

### 4.11.1.1 Introduction

This document has been produced by the member of SMART-Navigation project team(Korean Register, Marine-Works) in response to a requirement to produce a data product that can be used as request/response message to exchange Marine Safety Information(MSI). It is based on the IHO S-100 framework specification and the ISO 19100 series of standards. It is a textTable product specification that is primarily intended for encoding the download information, version information, push, request.

### 4.11.1.2 References

IHO S-100 IHO Universal Hydrographic Data Model Edition 4.0.0 (December 2017).

ISO 8601. 2004. Data elements and interchange formats - Information interchange - Representation of dates and times. 2004.

ISO 3166-1. 1997. Country Codes. 1997.

ISO 10646-1 Information technology - Universal Multiple-Octet Coded Character Set (UCS) - Part 1: Architecture and Basic Multilingual Plane

ISO 19101-2:2008 Geographic Information - Rules for Application Schema

ISO/TS 19103:2005 Geographic Information - Conceptual schema language

ISO 19106:2004 Geographic Information – Profiles

ISO 19107:2003 Geographic Information – Spatial schema

ISO 19109:2005 Geographic Information - Rules for Application Schema

ISO 19111:2003 Geographic Information - Spatial referencing by coordinates

ISO 19115:2003+Corr1 (2006) Geographic Information - Metadata

ISO 19115-2:2009 Geographic Information - Metadata: Extensions for imagery and gridded data

ISO 19123:2005 Geographic Information - Schema for coverage geometry and functions

ISO 19129:2009 Geographic Information - Imagery gridded and coverage data framework

ISO 19131:2007 Geographic Information - Data product specifications

ISO 19136:2007 Geographic Information – Geography Markup Language

ISO 19136-2:2015, Geographic Information – Geography Markup Language.

ISO/TS 19139, Geographic Information – Metadata – XML schema implementation.

### 4.11.1.3 Terms, definitions and abbreviations

Terms and Definitions

The S-100 framework is based on the ISO 19100 series of geographic standards. The terms and definitions provided here are used to standardize the nomenclature found within that framework, whenever possible. They are taken from the references cited in clause 2.1. Modifications have been made when necessary.

- Application

Manipulation and processing of data in support of user requirements (ISO 19101)

- Application schema

Conceptual schema for data required by one or more applications (ISO 19101)

- Conceptual model

Model that defines concepts of a universe of discourse (ISO 19101)

- Conceptual schema

Formal description of a conceptual model (ISO 19101)

- Coverage

Feature that acts as a function to return values from its range for any direct position within its spatial, temporal or spatiotemporal domain (ISO 19123)

- Data product

Dataset or dataset series that conforms to a data product specification

- Data product specification

Detailed description of a dataset or dataset series together with additional information that will enable it to be created, supplied to and used by another party

- Dataset

Identifiable collection of data (ISO 19115)

- Feature

Abstraction of real world phenomena (ISO 19101)

- Feature association

Relationship that links instances of one feature type with instances of the same or a different feature  type (ISO19110)

- Feature attribute

Characteristic of a feature (ISO 19101)

- Geographic data

Data with implicit or explicit reference to a location relative to the Earth (ISO 19109)

- Metadata

Data about data (ISO 19115)

- Model

Abstraction of some aspects of reality (ISO 19109)

- Portrayal

Presentation of information to humans (ISO 19117)

- Quality

Totality of characteristics of a product that bear on its ability to satisfy stated and implied needs (ISO 19101)

Abbreviations

This product specification adopts the following convention for symbols and abbreviated terms:

- ASCII 	American Standard Code for Information Interchange

- ECDIS	Electronic Chart Display and Information Systems

- ECS	Electronic Chart Systems

- ENC	Electronic Navigational Chart

- GML	Geography Markup Language

- IHO	International Hydrographic Organization

- IOC	International Oceanographic Commission

- ISO	International Organization for Standardization

- UML	Unified Modelling Language

- URI	Uniformed Resource Identifier

- URL 	Uniform Resource Locator

- WMS	Web Map Service

- WFS	Web Feature Service

- www 	World Wide Web

- XML	Extensible Markup Language

- XSLT	eXtensible Stylesheet Language Transformations

- MIO	Maritime Information Service

- MSI	Marine Safety Information

### 4.11.1.4 General Data Product Description

Informal Description of Data Product

This clause contains general information about the data product

- Title:  	MIO Service Meta Product Specification.

- Abstract: 	 MIO Service Meta product specification provides the means to exchange MSI information for Maritime Environment and Safety Information Service in the maritime domain such as MIO service request, MIO service response, MIO download, MIO version, MIO Push. This may include details on category of request, periodic time range, request location, MIO service product, result code, bounded area.

- Content:	Datasets conforming to this specification will contain all message meta information to exchange Marine Safety Information (MSI) for Maritime Environment and Safety Information Service. Additionally, there will be relevant metadata data quality, production authority, data sources and publication date.

- Spatial Extent:	Korean Coast

- Specific Purpose: Describing Maritime Environment and Safety Information Service in the maritime domain for utilization in ECDIS & ECS.

### 4.11.1.5 Data product specification metadata

This information uniquely identifies this Product Specification and provides information about its creation and maintenance. For further information on dataset metadata see the metadata clause.

- Title: 	MIO Service Meta

- S-100 Version: 	4.0.0

- MIO Service Meta Version:	1.0.0

- Date: 	2019-03-08

- Language: 	English

- Classification: 	Unclassified

- Contact: 	32, Yuseong-daero 1312beon-gil, Yuseong-gu, Daejeon, Korea 32(Jang Dong) inside KRISO

- URL: 	http://www.smart-navigation.org

- Identifier: 	MIO Service Meta

- Maintenance: 	Amendments to this specification will be produced on a needs basis. For reporting issues with this specification which need correction, use the contact information.

### 4.11.2 Specification Scopes

This product specification describes one data product and therefore requires only one scope which is described below:

- Scope ID:  		MIO Service Meta datasets.

- Hierarchical level: 		MD_ScopeCode 005

- Hierarchical level name: 	dataset.

- Level description:		information applies to the dataset

- Extent:	EX_Extent.description: Korean Coast

### 4.11.3 Dataset identification

This section describes how to identify data sets that conform to the specification. A dataset that conforms to this Product Specification may be identified by its discovery metadata as defined in clause 12 of this specification. The information identifying the data product may include the following items from S-100 3.0.0 clause 11-6 (adapted from ISO 19115).

| Title | MIO Service Meta |
| --- | --- |
| Abstract | MIO Service Meta dataset is a textTable dataset containing message meta information to exchange ENC for Maritime Environment and Safety Information Service such as request and response message. |
| Acronym | MIOSM |
| Content | Code of standard, MIO Name, category of request, count of coordinates, coverage distance, periodic time range, request parameter, download URL, bounded area. |
| Topic Category | Utilities Communication |
| Purpose | Describing Maritime Environment and Safety Information Service in the maritime domain for utilization in ECDIS & ECS |
| Language | EN Additional values if any use CharacterString values from ISO 639-2 |

### 4.11.4 Data Content and structure

### 4.11.4.1 Introduction

The MIO Servcie Meta product is based on the S-100 General Feature Model (GFM), and is a feature-based vector product. All MIO Servcie Meta features and information classes are derived from one of the abstract classes FeatureType and InformationType defined in the MIO Servcie Meta respectively.

This clause contains the Application Schema expressed in UML and an associated Feature Catalogue.

### 4.11.4.2 Application Schema

The UML models in this clause are segments of the overall MIO Servcie Meta application schema, and include overviews of the feature classes, information classes, meta features, spatial types, and the relationships between them.

This clause contains a general overview of the classes and relationships in the MIO Servcie Meta application schema. Detailed information about how to use the feature types and information types to encode MIO Servcie Meta information is provided in the SMART-Navigation Data Classification and Encoding Guide (DCEG).

The following conventions are used in the UML diagrams depicting the application schema:

- Standard UML conventions for classes, associations, inheritance, roles, and multiplicities apply. These conventions are described in Part 1 of S-100.

- Italic font for a class name indicates an abstract class.

- Feature classes are depicted with pink background; the dark shade for abstract feature classes and the light shade for ordinary (non-abstract) feature classes.

- Complex attributes are depicted with a light yellow background.

- Enumeration lists and codelists are depicted with a green background. The numeric code corresponding to each listed value is shown to its right following an ‘=’ sign.

- No significance attaches to the colour of associations. (Complex diagrams may use different colours to distinguish associations that cross one another.)

- Subclasses inherit the attributes and associations of their superclasses at all levels, unless such inheritance is explicitly overridden in the subclass

### 4.11.4.2.1 Domain model

Figure 24 MIO Service Meta Overview

### 4.11.4.3 Feature Catalogue

### 4.11.4.3.1 Introduction

The Feature Catalogue describes the feature types, information types, attributes, attribute values, associations and roles which may be used in the product.  The MIO Service Meta Feature Catalogue is available in an XML document which conforms to the S-100 XML Feature Catalogue Schema.  Simple attributes used in this specification are listed in below.

Name: 	MIO Service Meta Feature Catalogue

Scope: 	Smart-Navigation

Version Number: 	1.0.0

Version Date:	2019-03-15

Producer:	SMART Navigation Project

Language: 	                  English

### 4.11.4.3.1.1 Feature Types

Feature types contain descriptive attributes that characterize real-world entities. The word ‘feature’ may be used in one of two senses – feature type and feature instance. A feature type is a class and is defined in a Feature Catalogue. A feature instance is a single occurrence of the feature type and represented as an object in a dataset. A feature instance is located by a relationship to one or more spatial instances. A feature instance may exist without referencing a spatial instance.

### 4.11.4.3.1.2 Geographic

A Geographic (Geo) feature type carries the descriptive characteristics of a real-world entity.

### 4.11.4.3.1.3 Meta

Meta features contain information about other features within a dataset. Information defined by meta features override the default metadata values defined by the dataset descriptive records. Meta attribution on individual features overrides attribution on meta features.

### 4.11.4.3.1.4 Feature Relationship

A feature relationship links instances of one feature type with instances of the same or a different feature type.

### 4.11.4.3.1.5 Information Types

Information types are identifiable pieces of information in a dataset that can be shared between other features. They have attributes but have no relationship to any geometry; information types may reference other information types.

### 4.11.4.3.1.6 Attributes

SMART-Navigation product defines attributes as either simple or complex.

### 4.11.4.3.1.6.1 Simple Attributes

SMART-Navigation product uses nine types of simple attributes; they are listed in the following table:

| Type | Definition |
| --- | --- |
| Enumeration | A fixed list of valid identifiers of named literal values |
| Boolean | A value representing binary logic. The value can be either True or False. The default state for Boolean type attributes (i.e. where the attribute is not populated for the feature) is False. |
| Real | A signed Real (floating point) number consisting of a mantissa and an exponent |
| Integer | A signed integer number. The representation of an integer is encapsulation and usage dependent. |
| Text | An arbitrary-length sequence of characters including accents and special characters from a repertoire of one of the adopted character sets |
| Date | A date provides values for year, month and day according to the Gregorian Calendar. Character encoding of a date is a string which must follow the calendar date format (complete representation, basic format) for date specified by ISO 8601:1988. EXAMPLE 19980918 (YYYY-MM-DD) |
| Time | A time is given by an hour, minute and second. Character encoding of a time is a string that follows the local time (complete representation, basic format) format defined in ISO 8601:1988. EXAMPLE 183059 or 183059+0100 or 183059Z |
| Date and Time | A DateTime is a combination of a date and a time type. Character encoding of a DateTime shall follow ISO 8601:1988 EXAMPLE 19850412T101530 |
| Codelist | A type of flexible enumeration. A code list type is a list of literals which may be extended only in conformance with specified rules. |

### 4.11.4.3.1.6.2 Complex Attributes

Complex attributes are aggregations of other attributes that are either simple or complex. The aggregation is defined by means of attribute bindings.

### 4.11.4.3.1.6.3 Geometric Representation

Geometric representation is the digital description of the spatial component of an object as described in S-100 and ISO 19107. This product specification uses three types of geometries: GM_Point, GM_OrientableCurve, and GM_OrientableSurface.

### 4.11.4.3.2 Feature Catalogue General

The MIO Service Support feature catalogue describes the feature types, information types, attributes, attribute values, associations and roles which may be used in an MIO Service Support data set.

### 4.11.4.3.2.1 Feature Types

- Geographic

- Meta

- MIOServiceMeta

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| shipMRN | MRN of the ship using the ENC service | URN | 1 | mandatory |  |
| serviceMRN | MRN of the service providing the ENC service | URN | 0..1 | option |  |

- MIOServiceResponse

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| MIOServiceProduct | Detail information of product provided from MIO service. | complex attribute | 1 | mandatory |  |
| information | If the response to the request is un-successful, its detailed information | complex attribute | 0..1 | option |  |
| resultCode | Response result code for requested contents | enumeration | 0..1 | option |  |

- MIOServiceRequest

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| requestParameter | Detailed parameter values delivered for MIO service requests | complex attribute | 1..* | mandatory |  |
| periodicTimeRange | The complex attribute describes the active period for MIO Service request, as the times between its sub-attributes. | complex attribute | 0..1 | option |  |
| requestTime | The time of service request start | dateTime | 0..1 | option |  |
| categoryOfRequest | The category of MIO service request | enumeration | 1 | mandatory |  |
| countOfCoordinates | The number of coordinates of the zone used for the MIO service request, this Attribute used to distinguish the MIO service request area within the system to identify whether it is point / line / zone. | integer | 0..1 | option |  |
| coverageDistance | The margin distance of the point or line made for the MIO service request. | integer | 0..1 | option |  |
| requestLocation | Geometry information made for MIO service request | Point Or Curve Or Surfcae | 0..1 | option |  |

- MIODownload

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| downloadURL | - | URL | 1..* | mandatory |  |

- MIOVersion

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| availableVersion | - | text | 1..* | mandatory |  |

- MIOPush

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| contents | Actual contents of the push service provided on board | text | 1 | mandatory |  |
| boundedArea | Contents area of the push service provided on board | GM_Surface | 1 | mandatory |  |

### 4.11.4.3.2.2 Feature Relationship

- N/A

### 4.11.4.3.2.3 Information Relationship

- N/A

### 4.11.4.3.2.4 Information Type

- N/A

### 4.11.4.3.2.5 Simple Attribute

- Simple Attribute List

| Name | Description | Data Type | Remarks |
| --- | --- | --- | --- |
| shipMRN | MRN of the ship using the ENC service | URN |  |
| serviceMRN | MRN of the service providing the ENC service | URN |  |
| resultCode | Response result code for requested contents | enumeration |  |
| Request Time | The time of service request start | dateTime |  |
| categoryOfRequest | The category of MIO service request | enumeration |  |
| countOfCoordinates | The number of coordinates of the zone used for the MIO service request, this Attribute used to distinguish the MIO service request area within the system to identify whether it is point / line / zone. | integer |  |
| coverageDistance | The margin distance of the point or line made for the MIO service request. | integer |  |
| downloadURL | - | URL |  |
| availableVersion | - | text |  |
| contents | Actual contents of the push service provided on board | text |  |
| timeStart | The start of an active period | dateTime |  |
| timeEnd | The end of an active period | dateTime |  |
| codeOfStandard | Product Specification Code Number presented by IHO | text |  |
| version | version of product specification. (ex. 3.5.1) | text |  |
| MIOName | Product Specification Name presented by IHO | enumeration |  |
| searchKey | A search key that can reflect the user's requirement | text |  |
| koreaSeaArea | Segmentation of Korea Sea Area designated by service. | enumeration |  |
| generalCategory | Request parameter for MIO service (S124) | text |  |
| catalog | Request parameter for MIO service (S412) | text |  |
| language | The method of human communication, either spoken or written, consisting of the use of words in a structured and conventional way. | ISO639-3 |  |
| text | A non-formatted digital text string. | text |  |

- MIOName

| Value | Label | Description |
| --- | --- | --- |
| 1 | Marine Protected Areas | Marine Service Product defined by IHO |
| 2 | Maritime Radio Services | Marine Service Product defined by IHO |
| 3 | Navigational Warnings | Marine Service Product defined by IHO |
| 4 | Marine Traffic Management | Marine Service Product defined by IHO |
| 5 | Catalogues of Nautical Products | Marine Service Product defined by IHO |
| 6 | Aid to Navigation Information | Marine Service Product defined by IALA |
| 7 | Weather Overlay | Marine Service Product defined by WMO |

- korea Sea Area

| Value | Label | Description |
| --- | --- | --- |
| 1 | SEA001 | offing of Jeju |
| 2 | SEA002 | The west offing of the south |
| 3 | SEA003 | The south outsea of the west |
| 4 | SEA004 | The middle outsea of the west |
| 5 | SEA005 | The middle offing of the east |
| 6 | SEA006 | The south offing of the east |
| 7 | SEA007 | The east offing of the south |
| 8 | SEA008 | The middle offing of the west |
| 9 | SEA009 | The south offing of the west |
| 10 | SEA010 | off the east outsea of the south |
| 11 | SEA011 | off the west outsea of the south |
| 12 | SEA012 | The south outsea of Jeju |
| 13 | SEA013 | The east outsea of the south |
| 14 | SEA014 | The south outsea of the east |
| 15 | SEA015 | The middle outsea of the east |

- result Code

| Value | Label | Description |
| --- | --- | --- |
| 1 | 0000 | No error issue |
| 2 | 5000 | specs is null. |
| 3 | 5001 | Unsupport spec. : (request spec) |
| 4 | 5100 | sourceMrn is null. |
| 5 | 5200 | version is null. |
| 6 | 5201 | version is not support. Please check specs version. |
| 7 | 5300 | type is null. |
| 8 | 5301 | search type is not available. please, select this values of one(point,line,polygon). |
| 9 | 5400 | coordinate x range is (-180.0 ~ 180.0)., coordinate y range is (-90.0 ~ 90.0). |
| 10 | 5401 | valid coordinate is null. |
| 11 | 5402 | point coordinate must valid coordinate pair 1. |
| 12 | 5403 | line coordinate minimum valid coordinate pair 2. |
| 13 | 5404 | polygon coordinate must valid coordinate pair 2. |
| 14 | 5500 | buffer range from 1 to 2147483647 |
| 15 | 5999 | different specs's count and version's count. |

- categoryOfRequest

| Value | Label | Description |
| --- | --- | --- |
| 1 | MIO Download | MIO Download service |
| 2 | MIO Version | MIO product version provided |

### 4.11.4.3.2.6 Complex Attribute

- periodicTimeRange

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| timeStart | The start of an active period | dateTime | 1 | mandatory |  |
| timeEnd | The end of an active period | dateTime | 1 | mandatory |  |

- MIO Service Product

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| codeOfStandard | Product Specification Code Number presented by IHO | text | 1 | mandatory |  |
| version | Version of product specification. (ex. 3.5.1) | text | 0..1 | option |  |
| MIOName | Product Specification Name presented by IHO | enumeration | 0..1 | option |  |

- requestParameter

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| searchKey | A search key that can reflect the user's requirement | text | 0..1 | option |  |
| koreaSeaArea | Segmentation of Korea Sea Area designated by service. | enumeration | 0..1 | option |  |
| generalCategory | Request parameter for MIO service (S124) | text | 0..1 | option |  |
| catalog | Request parameter for MIO service (S412) | text | 0..* | option |  |
| MIOServiceProduct | Detail information of product provided from MIO service. | complex attribute | 1 | mandatory |  |

- information

| Attribute Name | Description | Data Type | Multiplicity | Implementation | Remarks |
| --- | --- | --- | --- | --- | --- |
| language | The method of human communication, either spoken or written, consisting of the use of words in a structured and conventional way. | ISO639-3 | 0..1 | option |  |
| text | A non-formatted digital text string. | text | 1 | mandatory |  |

### 4.11.5 Coordinate Reference System(CRS)

### 4.11.5.1 Introduction

The location of an object in the S-100 standard is defined by means of coordinates which relate a feature to a position. The coordinate reference system used for this product specification is World Geodetic System 1984 (WGS 84) which is defined by the European Petroleum Survey Group (EPSG) code 4326, (or similar - North American Datum 1983 / Canadian Spatial Reference System).

Spatial data are expressed as latitude (φ) and longitude (λ) geographic coordinates. Latitude values are stored as a negative number to represent a position south of the Equator. Longitude values are stored as a negative number to represent a position west of the Prime Meridian. Coordinates are expressed as real value, degree / degree decimal format. Datasets conforming to this product specification are not projected.

Horizontal coordinate reference system:	WGS 84

Projection: 					None

Vertical coordinate reference system: 	None

Temporal reference system: 		Gregorian calendar

Coordinate reference system registry: 	EPSG Geodetic Parameter Registry

Date type (according to ISO 19115-1):  	002 - publication

### 4.11.5.2 Horizontal Reference System

Positional data is expressed in latitude and longitude geographic coordinates to one of the reference horizontal reference systems defined in the HORDAT attribute.  Unless otherwise defined, the World Geodetic System 84 (WGS 84) will be used for MIO Service Support data products.

### 4.11.5.3 Projection

MIO Service Support data products are un-projected.

### 4.11.5.4 Vertical Coordinate Reference System

MIO Service Support data products do not provide detailed vertical information.

### 4.11.5.5 Temporal Reference System

Time is measured by reference to Calendar dates and Clock time in accordance with ISO 19108:2002 Temporal Schema clause 5.4.4.

### 4.11.5.6 MIO Service Support Data and Scale

MIO Service Support data must be compiled in the best applicable scale. The use of the data itself is "scale independent". That means that the data can be used at any scale. S-100 allows the association of multiple spatial attributes to a single feature instance. In principle, each of these spatial attributes can be qualified by maximum and minimum scales.

For example, it is possible, within one dataset, to have a single instance of a feature that has more than one area geometry. Each of these geometries has different scale max/min attributes. Moreover, due to cluttering in smaller scales, the scale minimum attribute may be used to turn off portrayal of some features at smaller scales.

### 4.11.6 Data Quality

### 4.11.6.1 Introduction

MIO Service Support products must be tested with the MIO Service Support specific checks prior to release by the data producer. The data producer must review the check results and address any issues to ensure sufficient quality of the data products. The checks are a mix of data format validation checks, conformance to standard checks and logical consistency checks. But the checklist has not been written yet. It will be updated in the next version.

### 4.11.7 Data Capture and classification

See DCEG for this section.

### 4.11.8 maintenance

### 4.11.8.1 Introduction

Datasets are maintained as needed and must include mechanisms for MIO Support Service updating. Data updates will be made by new editions. The maintenance and update frequency of MIO Support Service datasets should be defined by the producers (official national authority) implementing this specification.

Data Producers must use applicable sources to maintain and update data and provide a brief description of the sources that were used to produce the dataset in the appropriate metadata field.

The data product shall provide information on how the data is maintained and should describe the principles and criteria applied in maintenance regime. This should specify the expected frequency of updates.

Table 36 Maintenance and update frequency

| Item Name | Description | Multiplicity | Type |
| --- | --- | --- | --- |
| maintenanceAndUpdateFrequency | Frequency with which changes and additions are made to the data product (per update scope) | 1..* | MD_MaintenanceInformation (ISO 19115) |
| dataSource | Identification of the kinds of data sources usable to produce datasets | 1..* | LI_Source (ISO 19115) |
| productionProcess | Textual description of the production process applicable to the datasets (per scope or data source) | 1..* | LI_ProcessStep (ISO 19115) |

### 4.11.8.2 Production process for base and update datasets

Data Producers should follow their established production processes for maintaining and updating datasets. Data is produced against the DCEG and checked against the appropriate set of validation rules in Appendix E. (Appendix E is not included in this release.)

### 4.11.8.3 Datasetr updates

N/A (Dataset update is not relevant in this model)

### 4.11.8.4 Support file updates

N/A (Support file is not included in this model)

### 4.11.8.5 Feature and portrayal catalogues

For each new version of the MIO Support Service Product Specification a new feature catalogue will be released. The system must be able to manage datasets and their catalogues that are created on different versions of the MIO Support Service product specification (Portrayal Catalogue is not defined in this model)

### 4.11.8.6 Feature history, versions, and change tracking

If applications or production systems require versioning of individual instances of feature or information types, maintenance of histories, or change tracking, the methods for versioning, history management, and change tracking and display are left to the application or production system.

### 4.11.8.7 Dataset encryption

Details about dataset encryption are still to be determined, and may mirror the method described in S-101.

### 4.11.9 Portrayal

Portrayal is not defined in this version of MIO Service Support Product Specifications. Users are free to choose the means and methodology of portrayal as they see best suited for their needs. It should be noted that future versions of MIO Service Support may include a portrayal catalogue, and any implementer should therefore anticipate this, and make sufficient provisions in any system supporting MIO Service Support.

### 4.11.10 data product format(encoding)

### 4.11.10.1 Introduction

The GML encoding of MIO Service Meta datasets is based on the S-100 profile of GML 3.2.1.

Format Name: MIO Service Meta GML

Version:	1.0

Character Set: UTF-8

Specification: SMART-Navigation Product Specification

### 4.11.10.2 Encoding of Latitude and Longitude

Values of latitude and longitude must be accurate to 7 decimal places. Coordinates must be encoded as decimals in the format described below. The encoding is indicated by multiplication factor fields defined in the dataset identification record by the S-100 GML schemas.

### 4.11.10.2.1 Encoding of coordinates as decimals

Values should be coded as decimal numbers with 7 or fewer digits after the decimal. The normative encoding is in degrees, with an accuracy of 10-7 degrees, i.e., 7 digits after the decimal point.

The decimal point must be indicated by the “.” character.

Trailing zeroes after the decimal point (and the decimal point itself if appropriate) may be omitted at producer discretion, but the accuracy must still be as indicated (e.g., 10-7 degrees for coordinates of default accuracy).

Latitude and longitude multiplication factors held in the Dataset Structure Information field under [coordMultFactorX] and [coordMultFactorY] must be set to a value corresponding to the encoding, i.e., {1} for coordinates encoded in decimal degrees.

EXAMPLE 1 A longitude = 42.0000 is converted into X = longitude * coordMultFactorX = 42.0000 * 1 = 42.0000000.

### 4.11.10.3 Text Attribute

Character strings must be encoded using the character set defined in ISO 10646-1, in Unicode Transformation Format-8 (UTF-8).

### 4.11.10.4 Mandatory Attribute Values

There are four reasons why attribute values may be considered mandatory:

- They determine whether a feature is in the display base,

- Certain features make no logical sense without specific attributes,

- Some attributes are necessary to determine which symbol is to be displayed,

- Some attributes are required for safety of navigation.

All mandatory attributes are identified in the Feature Catalogue.

### 4.11.10.5 Unknow Attribute Values

When a mandatory attribute code or tag is present but the attribute value is missing, it means that the producer wishes to indicate that this attribute value is unknown.  Missing mandatory attributes must be “nilled” with a GML nilReason attribute giving the reason for omission.

Optional attributes must be omitted altogether if the value is unknown or missing. They must not be “nilled.”

### 4.11.10.6 Structure of dataset files

### 4.11.10.6.1 Sequence of dataset files

The order of data objects in each dataset file is described below:

Dataset Identification Information

Dataset structure information

Spatial records for by-reference geometries

Point

Multi point

Curve

Composite Curve

Surface

Information objects

Feature objects (Geometry may be encoded inline or by reference.)

Meta features

Geo features

### 4.11.10.7 Numeric Attribute Encoding

Floating point and integer attribute values must not contain leading zeros.  Floating point attribute values must not contain non-significant trailing zeros.

### 4.11.10.8 Object identifiers

The “name” of feature records must provide a unique world-wide identifier of feature records. The “name” of the record is the combination of the subfields agency, featureObjectIdentifier, and featureIdentificationSubdivision elements of the featureObjectIdentifier element of the object.

Features, information types, collection objects, meta features, and geometries (inline or external) are all required by the schema to have a gml:id attribute with a value that is unique within the dataset. The gml:id values must be used as the reference for the object from another object in the same dataset or another dataset.

### 4.11.10.9 Dataset validation

Fields may be repeated or omitted as permitted by the XML schemas and the validation tests. Since XML schema cannot encode rules for conditional presence or attributes, these rules are checked by Schematron rules or other validation code.

### 4.11.10.10 Location of Data Product Format schema Files

The schema file will provide a future location and is available from that location.

### 4.11.10.11 Detailed documentation of MIO Service Meta XML Schema

See DCEG for this section.

### 4.11.11 data product delivery

This clause specifies the encoding and delivery mechanisms for an MIO Service Meta dataset.  Data which conforms to this product specification must be delivered by means of an exchange set.

### 4.11.11.1 Exchange set

MIO Service Meta datasets are grouped into exchange sets. Each exchange set consists of one or more MIO Service Meta datasets with an associated XML metadata file and a single Exchange Catalogue XML file containing metadata. It may also include one or more support files.

Units of Delivery:				Exchange Set

Transfer Size:					Unlimited

Medium Name:					Digital data delivery

Other Delivery Information:

Each dataset must be contained in a physically separate, uniquely identified file on the transfer medium.

Each exchange set has a single exchange catalogue which contains the discovery metadata for each dataset and references to any support files.

Support files are supplementary information which are linked to the features and information types by attributes. The attributes containing these links are described in the application schema and feature catalogue.

An exchange set may be encapsulated into a form suitable for transmission by a mapping called a transmission encoding. An encoding translates each of the elements of the exchange set into a logical form suitable for writing to media and for transmission online. An encoding may also define other elements in addition to the exchange set contents (i.e., media identification, data extents etc…) and also may define commercial constructs such as encryption and compression methods.

If the data is transformed (e.g., for encryption or compression purposes) its content must not be changed.

This product specification does not define the transmission encoding which must be used as a default for transmission of data between parties.

The exchange set elements are as follows:

Mandatory Elements

- MIO Service Meta datasets – GML encoding of features/attributes and their associated geometry and metadata.

- Exchange Catalogue – the XML encoded representation of exchange set catalogue features [discovery metadata].

Optional Elements

- Supplementary files – These are contained within the exchange set as files and the map from the name included within the dataset and the physical location on the media is defined within the Exchange Catalogue.

- Feature Catalogue – If it is necessary to deliver the latest feature catalogue to the end user it may be done using the MIO Service Meta exchange set mechanism for datasets – i.e., include the updated feature catalogue in an exchange set.

### 4.11.11.2 Dataset

### 4.11.11.2.1 Datasets

Datasets are distributed as files which are part of exchange sets structured as described in this specification. The distribution media are left to the discretion of the producer and distributor.

The following types of dataset files may be produced and contained within an exchange set:

- New dataset and new edition of a dataset (base dataset):  Each new edition of a dataset must have the same name as the dataset that it replaces.  A new edition can also be AtoN data that has previously been produced for this area.

### 4.11.11.2.2 Dataset size

Datasets must not exceed 20 MB.

### 4.11.11.2.3 Dataset file naming

Dataset files shall be named

CCXXXXXXXX_EEE.GML

The file name forms a unique identifier where:

- CC - the first two characters identify the issuing agency (mandatory).

- The third to tenth characters are optional and may be used in any way by the producer to provide the unique file name. The following characters are allowed in the dataset name, A to Z, 0 to 9 and the special character _ (underscore).

- EEE – new editions and re-issues use 000, and increment until a limit of 999 (mandatory).

- The maximum number of characters preceding _EEE is ten.

- GML – the character sequence “GML” or “gml”.

### 4.11.11.2.4 New Editions and re-issues

This section defines the sequencing of datasets for New Editions and Re-issues. In order to ensure that feature type updates are incorporated into an SYSTEM in the correct sequence without any omission, a number of parameters encoded in the data header and/or metadata are used in the following way:

| edition number | When a dataset is initially created, the edition number 1 is assigned to it. The edition number is increased by 1 at each new edition. |
| --- | --- |

### 4.11.11.2.5 Cancellations

Dataset cancellation is not considered I this product specification. In the SMART Navigation project, the system automatically cancels the dataset after 7 days from the date the dataset is received.

### 4.11.11.3 Support Files

N/A (Support file is not included in this model)

### 4.11.11.4 Exchange Catalogue

N/A (Exchange Catalogue is not used in SMART Navigation)

### 4.11.12 meta data

### 4.11.12.1 Introduction

The MIO Service Support metadata description is based on the S-100 metadata document section, which is a profile of the ISO 19115 standard. These documents provide a structure for describing digital geographic data and define metadata elements, a common set of metadata terminology, definitions and extension procedures.

Two metadata packages are described in this product specification: dataset metadata and exchange set metadata.

Figure 25 Metadata packages

Note 1: Types with CI_, EX_, and MD_ prefixes are from packages defined in ISO 19115 and adapted by S-100. Types with S100_ prefix are from packages defined in S-100.

Note 2: When a dataset is terminated, the purpose metadata field is set to 3 (terminated), and the editionNumber metadata field is set to 0. All other metadata fields must be blank.

### 4.11.12.2 Dataset metadata

Dataset metadata is intended to describe information about a dataset. It facilitates the management and exploitation of data and is an important requirement for understanding the characteristics of a dataset. Whereas dataset metadata is usually fairly comprehensive, there is also a requirement for a constrained subset of metadata elements that are usually required for discovery purposes. Discovery metadata are often used for building web catalogues, and can help users determine whether a product or service is fit for purpose and where they can be obtained.

Table 37 Dataset metadata

| Name | Cardinality | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| S100_DataSetDiscoveryMetadata |  |  |  |  |
| fileName | 1 |  | CharacterString | Dataset file name |
| filePath | 1 |  | CharacterString | Full path from the exchange set root directory |
| description | 1 |  | CharacterString | Short description of the area covered by dataset, e.g., area, harbour, or port name, between two named locations etc. |
| dataProtection | 0..1 | {1} or {2} | Boolean | True : Encrypted False : Unencrypted |
| protectionScheme | 0..1 |  | CharacterString |  |
| digitalSignature | 0..1 |  | CharacterString |  |
| digitalSignatureReference | 0..1 |  | CharacterString |  |
| digitalSignatureValue | 0..1 |  | CharacterString |  |
| copyright | 0..1 |  | MD_LegalConstraints>MD_RestrictionCode <copyright> (ISO 19115-1) |  |
| classification | 0..1 |  | Class MD_SecurityConstraints>MD_ClassificationCode (codelist) ISO 19115-1 | 1. unclassified 2. restricted 3. confidential 4. secret 5. top secret |
| purpose | 1 | {1}, {2} | CharacterString | - New dataset - New edition |
| specificUsage | 1 |  | MD_USAGE>specificUsage (character string) MD_USAGE>userContactInfo (CI_Responsibility) | brief description of the resource and/or resource series usage |
| editionNumber | 1 | {1} | Integer | When a dataset is initially created, the edition number “1” is assigned to it. The edition number is increased by one with each new edition. |
| updateNumber | 1 |  | CharacterString | Update number 0 is assigned to a new dataset. |
| updateApplicationDate | 1 |  | Date | All updates dated on or before this date must have been applied by the producer. |
| issueDate | 1 |  | Date | Date on which the dataset was generated. |
| productSpecification | 1 |  | S100_ProductSpecification | See Notes below this table for constraints on values. |
| producingAgency | 1 |  | CI_Responsibility>CI_Organisation or CI_Responsibility>CI_Individual | Party responsible for generating the dataset. |
| optimumDisplayScale | 0..1 |  | Integer |  |
| maximumDisplayScale | 0..1 |  | Integer |  |
| minimumDisplayScale | 0..1 |  | Integer |  |
| horizontalDatumReference | 1 | EPSG | CharacterString |  |
| horizontalDatumValue | 1 | 4326 | Integer | WGS84 |
| verticalDatum | 0..1 |  | S100_VerticalAndSoundingDatum |  |
| soundingDatum | 0..1 |  | S100_VerticalAndSoundingDatum |  |
| dataType | 1 | GML | CharacterString | The only value allowed is “GML”. |
| otherDataTypeDescription | 0..1 |  | CharacterString |  |
| dataTypeVersion | 1 | 3.2.1 | CharacterString |  |
| dataCoverage | 1..* |  | S100_DataCoverage |  |
| comment | 0..1 |  | CharacterString | Any additional Information |
| layerID | 1..* | S-101 | CharacterString | Dataset must be used with ENC in an ECDIS. |

### 4.11.12.3 Update Dataset Metadata

N/A (Dataset update is not relevant in this model)

### 4.11.12.4 Support file metadata

N/A (Support file is not included in this model)

### 4.11.12.5 Exchange set metadata

N/A (Exchange Catalogue is not used in SMART Navigation)

### 4.11.12.6 Catalogue file Metadata

All MIO Service Support Exchange Catalogue files must contain at least the mandatory metadata elements.

Table 38 Catalogue file metadata

| Name | ultiplicity | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| S100_ExchangeCatalogue |  |  |  |  |
| identifier | 1 |  | S100_CatalogueIdentifier |  |
| contact | 1 |  | S100_CataloguePointOfContact |  |
| productSpecification | 0..1 |  | S100_ProductSpecification | Conditional on all the datasets using the same product specification.. |
| metadataLanguage | 1 | English | CharacterString | All datasets conforming to this PS must use English language. A catalogue in English must be provided. Discovery metadata elements within catalogues have their own locale attributes and may be repeated in languages other than English. |
| exchangeCatalogueName | 1 |  | CharacterString | Catalogue filename |
| exchangeCatalogueDescription | 1 |  | CharacterString |  |
| exchangeCatalogueComment | 0..1 |  | CharacterString | Any additional Information |
| compressionFlag | 0..1 |  | Boolean | Yes or No |
| sourceMedia | 0..1 |  | CharacterString |  |
| replacedData | 0..1 |  | Boolean |  |
| dataReplacement | 0..1 |  | CharacterString |  |