# CHAPTER 10:   VTS EQUIPMENT

1001	Introduction

Traffic density, navigation hazards, local climate, topography and extent of a VTS area sets the requirements to VTS equipment and these factors will have substantial impact on life-cycle costs of a VTS and acquisition of VTS equipment e.g. including:

- Communications

- VTS Radar System

- Automatic Identification System (AIS)

- Closed Circuit TV Cameras (CCTV)

- Hydrometeo Equipment

- VTS Data System

The required features and in particular the need for coverage by sensors, e.g. radar, should be determined by an assessment of the service to be provided, the safety level to be reached and the user requirements of the VTS system.  Subsequently suitable positions for the equipment should to be determined by site survey, analysis, simulations and/or site-tests to ensure, as far as practicable, that the required functions and coverage will be provided.

Detailed guidance is given by IALA recommendation V-119 on the Implementation of Vessel Traffic Services and V-128, Operational and Technical Performance Requirements for VTS Equipment.  V-128 is divided into three different levels of capabilities:

- Basic - applicable to VTS information service and, where applicable, navigational assistance service.

- Standard - applicable to all types of VTS as identified by IMO – information service, navigational assistance service and traffic organizational service – for areas with medium traffic density and/or without major navigational hazards.

- Advanced - applicable to VTS areas with high traffic density and/or specific major navigational hazards.

The individual VTS authority should decide on appropriate levels. Different levels may be used as applicable within a VTS.  E.g. part of a VTS area may call for a Basic capability and another part may call for a Standard capability.

# 1002	Communication

Communications in a VTS area are presently heavily biased towards voice systems, mostly using VHF and data.  This requires a complex combination of procedural and language skills, which must be thoroughly understood by all VTS Personnel.

A VTS Operator will need to communicate with shipping for which he has responsibility and may also need to communicate with other operators or allied services.  The ideal situation would be for the control of all communication equipment to be integrated into each VTS Operators console.

#### Communication with Shipping in the VTS Area

Normally the communication coverage of the VTS area will be achieved by VHF using telephony or data transmissions and will involve the following.

Communication between shore and participating vessels, using appropriate international VHF channels can take place on simplex, as well as duplex channels.  In the case of duplex channels, re-transmission from the shore may need to be carried out if the information received is of interest to other vessels to enable them to better comprehend the traffic situation.

Private channels may also be available for use between VTS Centres and local service craft. However, in this case local service craft should always be able to communicate with shipping on the appropriate international VHF channel.

It is of the utmost importance that the VHF communication network of a VTS is set up properly, with high quality and reliable equipment and interconnection.  The number of VHF channels required for specific VTS usage should be assessed during the Project Definition phase and arranged with their National Telecommunication Authorities well in advance of the Commissioning and Acceptance phase.  Often a VTS has, apart from the sub-area or sector frequencies, one or more general reporting or emergency frequencies.  The number of channels required is dependent upon the number of the various sub-areas and sectors that will be used and the overall traffic density. The VTS Authority should also seek permission from their National Telecommunication Authority to use VHF Channel 16 for emergencies and for calling ships, which are not participating in the VTS.

Shipborne VHF equipment is normally capable of transmitting and receiving on the international channels 1-29 and 60-89 at full power (25W) and reduced power (1W).  Where practicable it is advisable to request low power transmission from ships.  Communications with vessels should be recorded, for accident investigation, disputes, etc.

Very High Frequency (VHF) Radio Communication

It is common for the VTS to have its own independent VHF network, for the use within specifically designated VHF Channels. This network may comprise of one or more VHF Channels in different sectors of the VTS Area. The VTS Authority may require specific VHF Channels to be designated by the National Telecommunications Authority for specific types of operations. The VHF equipment must comply with national and international regulations as those issued by the International Telecommunications Union (ITU).

Long Range Communication

In the case where a VTS Authority requires pre-arrival information, the normal maritime communication systems should be used and therefore an independent network is not required.

Communication with Allied Services

VTS Centres should be equipped with the ability to communicate with allied services by the use of reliable and secure communication networks. It was usual for a VTS Centre to be equipped with a point-to-point line with MRCCs for this purpose. However, point-to-point lines have been found to be inadequate and unnecessarily expensive to establish and maintain. Therefore it is recommended that VTS Centres should be equipped with a digital switched network, with caller identification.

Other Communication

The internal communication within a VTS is of the utmost importance.  If there is more than one VTS Centre it may be necessary for the operators in one VTS Centre to communicate easily with operators in other VTS Centres so that specific traffic information can be passed from one operator to another with the minimum of delay.

The following general types of communication may be required by a VTS:

- between VTSOs in the same VTS centre

- where appropriate, between VTSOs in different VTS centres belonging to the same VTS;

- between VTSOs of any adjacent VTS;

- with vessels about to enter a VTS area

- with Pilotage, tugboats, SAR, Port Authorities and other Authorities.

These types of communication can be performed by using the following equipment:

- public telephone lines

- private telephone lines

- telex

- facsimile

- radio telephony and microwave links

- automated data transfer systems.

Where there is a need for communication between a VTS and a shore authority to have a high degree of operational importance, consideration should be given to providing direct telephone lines to increase the reliability and decrease the time of the connection.

# 1003	VTS RADAR SYSTEM

Depending on the services that a VTS is to carry out the radar coverage can be:

- nil (automatic identification systems, voice communication and reporting only)

- partly (covered areas chosen intentionally with some blind sectors)

- totally by one radar sensor (without any blind sectors)

- totally by two or more radar sensors (for large VTS areas and to cover for shadow effects of other vessels). Stereographic processing of images from 2 or more radars may also be utilised for elimination of false (ghost) echoes.

In principle VTS radars typically function like ships radars, but they will in most cases need to operate simultaneously on short and long range, preferably without the need for operator adjustments. Weather related phenomena such as sea clutter and ducting will furthermore influence shore-based radars more than ships’ radars.

In addition, new challenges have developed over recent years, including:

- Introduction of new technologies, especially AIS, require the presentation of radar information to be sufficiently accurate to avoid ambiguity.

- Antenna side lobes and ghost targets (multiple reflections) may lead to false and dangerous results when radar returns and AIS plots are associated.  High precision, low sidelobe antennas and careful location of VTS radars is therefore required to allow for unambiguous correlation of position obtained from the two information sources.

- Offshore Renewable Energy developments, such as wind farms. VTS radars will normally not be dependent on Doppler shift and they are therefore not affected by the rotation of wind turbines, but the large towers may reflect radar signals resulting in false echoes. From a VTS Operator’s perspective however, these false echoes are normally easy to distinguish. Shadowing may also present difficulties but the extent of this potential problem is currently not understood. Competent/VTS Authorities are therefore encouraged to enter into early discussions with Offshore Renewable Energy Developers in order to minimise any potential effects on VTS operations.

- Increasing demands to see small targets in rough weather, if objectives include detection of targets for security purposes.

- Requirement to reduce spurious / out of band emissions.

These challenges require dynamic and resolution requirements that far exceed those required onboard ships.

Radar Functions

When radar is required, it should be able to detect and track, for subsequent display, all specified moving or stationary targets which satisfy the detection criteria within the specified coverage areas and during all specified operating circumstances.

Each radar should be equipped to reduce the adverse effects of rain and sea clutter and enhance the probability of target detection. The radar should also be designed and installed so as to eliminate, to the maximum extent possible, false echoes caused by side lobes or reflections from nearby structures.

The VTS Radar System should furthermore assist in the development of the traffic image, by performing the functions shown in Table 1 below.

Table 1 Radar Functions

| Parameters / Capability | Basic | Standard | Advanced |
| --- | --- | --- | --- |
| Path, time and track prediction, |  |  | X |
| CPA, | X | X | X |
| TCPA, | X | X | X |
| Anchor watch, |  |  | X |
| Vessels vector, | X | X | X |
| Course, speed and label/identity, | X | X | X |
| Collision alerts. | X | X | X |

Radar frequencies selected for VTS lie typically within the S-band and X-band frequencies, however, higher frequencies such as Ku band may also be utilised. The majority of VTS services use X-band radars as a best compromise, especially since technologies for rain clutter suppression have matured.  Also, as a result of production volume, X-band radars are the least expensive.

The second most used frequency is S-band, due to better weather penetration in heavy rainfall. A radar operating in the S-band is typically needed if precipitation rates frequently are greater than 25mm/h and required detection distance exceeds a few nautical miles.

The International Telecommunications Union (ITU) grants frequency band allocation, whereas permissions to transmit on given frequencies are granted on a national basis.

#### Characteristics of Radar Targets.

The characteristics of VTS radar targets are defined by its height above sea level, its radar cross-section (RCS) and its fluctuations in RCS. Formally, the RCS is defined as the ratio between the power [in W] scattered by the target back towards the radar receiver and the power density [in W/m2] hitting the target.  Thus RCS is measured in m2 and has the dimension of an area.  There is, however, no simple relation between the physical area of the target and the RCS, as the reflected power depends on the angle of incidence as well as target properties such as material and physical shape. Targets may fluctuate severely in RCS and the returned energy is highly dependant on propagation conditions. Table 2 below provides recommended data to be used when defining requirements for a VTS.

Table 2 Radar target characteristics

| TARGET |  | Type of Capability |  |  | Design Requirements |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Basic | Standard | Advanced | Radar cross section |  | Height of Target |
|  |  |  |  |  | S-band | X-band |  |
| 1 | Aids to Navigation etc. –without radar reflector. Small open boats, fibreglass, wood or rubber with outboard motor and at least 4 meters long, small speedboats, small fishing vessels, small sailing boats and the like. |  |  | X |  | 1 m2 | 1 m ASL |
| 2 | Inshore fishing vessels, sailing boats, speedboats and the like. |  |  | X |  | 3 m2 | 2 m ASL |
| 3 | Aids to Navigation with radar reflector. |  | X | X | 4 m2 | 10 m2 | 3 m ASL |
| 4 | Small metal ships, fishing vessels, patrol vessels and the like. | X | X | X | 40 m2 | 100 m2 | 5 m ASL |
| 5 | Coasters and the like. | X | X | X | 400 m2 | 1,000 m2 | 8 m ASL |
| 6 | Large coasters, bulk carriers, cargo ships and the like. | X | X | X | 4,000 m2 | 10,000 m2 | 12 m ASL |
| 7 | Container carriers, tankers etc. | X | X | X | 40,000 m2 | 100,000 m2 | 18 m ASL |

#### Detection Performance

Table 3 and Table 4 below provide examples of calculated range performance typical for radar equipment suitable for the three recommendation levels. The calculations in this table were performed using the software programme CARPET.

Table 3  Typical range performance, X-band

| Antenna elevation | Target type | Modelled as fluctuating point target |  | Detection and tracking ranges for standard atmosphere and rain/sea state as indicated |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RCS | Height | Basic recommendation |  |  | Standard recommendation |  | Advanced recommendation |  |
|  |  |  |  | Clear | 2 mm/h rain |  | Clear | 4 mm/h rain | Clear | 10 mm/h rain |
| 20 m ASL | 1 | 1 m2 | 1 m ASL | N/A |  |  | N/A |  | 5 NM | NIL |
|  |  |  |  |  |  |  |  |  | Up to sea state 4 |  |
|  | 2 | 3 m2 | 2 m ASL | N/A |  |  | 7 NM | 4NM | 7 NM | 6 NM |
|  |  |  |  |  |  |  | Up to sea state 3 |  | Up to sea state 5 |  |
|  | 3 | 10 m2 | 3 m ASL | 7 NM |  | 4 NM | 8 NM | 5NM | 9 NM | 7 NM |
|  |  |  |  | Up to sea state 3 |  |  | Up to sea state 4 |  | Up to sea state 6 |  |
|  | 4 | 100 m2 | 5 m ASL | 9 NM |  | 8 NM | 11 NM | 9NM | 12 NM | 10 NM |
|  |  |  |  | Up to sea state 4 |  |  | Up to sea state 5 |  | Up to sea state 7 |  |
|  | 5 | 1000 m2 | 8 m ASL | 12 NM |  | 10 NM | 13 NM | 11 NM | 14 NM | 13 NM |
|  |  |  |  | Up to sea state 5 |  |  | Up to sea state 6 |  | Up to sea state 8 |  |
| 50 m ASL | 1 | 1 m2 | 1 m ASL | N/A |  |  | N/A |  | 10 NM | NIL |
|  |  |  |  |  |  |  |  |  | Up to sea state 4 |  |
|  | 2 | 3 m2 | 2 m ASL | N/A |  |  | 10 NM | 7 NM | 12 NM | 9 NM |
|  |  |  |  |  |  |  | Up to sea state 3 |  | Up to sea state 5 |  |
|  | 3 | 10 m2 | 3 m ASL | 10 NM |  | 6 NM | 12 NM | 8 NM | 14 NM | 12 NM |
|  |  |  |  | Up to sea state 3 |  |  | Up to sea state 4 |  | Up to sea state 7 |  |
|  | 4 | 100 m2 | 5 m ASL | 13 NM |  | 12 NM | 15 NM | 13 NM | 17 NM | 15 NM |
|  |  |  |  | Up to sea state 4 |  |  | Up to sea state 5 |  | Up to sea state 7 |  |
|  | 5 | 1000 m2 | 8 m ASL | 16 NM |  | 15 NM | 18 NM | 17 NM | 20 NM | 18 NM |
|  |  |  |  | Up to sea state 5 |  |  | Up to sea state 6 |  | Up to sea state 8 |  |
| 100 m ASL | 1 | 1 m2 | 1 m ASL | N/A |  |  | N/A |  | 12 NM | NIL |
|  |  |  |  |  |  |  |  |  | Up to sea state 4 |  |
|  | 2 | 3 m2 | 2 m ASL |  |  |  | 13 NM | 5 NM | 16 NM | 10 NM |
|  |  |  |  |  |  |  | Up to sea state 3 |  | Up to sea state 5 |  |
|  | 3 | 10 m2 | 3 m ASL |  |  |  | 17 NM | 10 NM | 18 NM | 16 NM |
|  |  |  |  |  |  |  | Up to sea state 4 |  | Up to sea state 7 |  |
|  | 4 | 100 m2 | 5 m ASL |  |  |  | 20 NM | 19 NM | 22 NM | 20 NM |
|  |  |  |  |  |  |  | Up to sea state 5 |  | Up to sea state 7 |  |
|  | 5 | 1000 m2 | 8 m ASL |  |  |  | 23 NM | 22 NM | 25 NM | 23 NM |
|  |  |  |  |  |  |  | Up to sea state 6 |  | Up to sea state 8 |  |

Table 4  Typical range performance, S-band

| Antenna elevation | Target type | Modelled as fluctuating point target |  | Detection and tracking ranges for standard atmosphere and rain/sea state as indicated |  |
| --- | --- | --- | --- | --- | --- |
|  |  | RCS | Height | Standard recommendation |  |
|  |  |  |  | Clear | 16 mm/h rain |
| 20 m ASL | 3 | 4 m2 | 3 m ASL | 4 NM | 3 NM |
|  |  |  |  | Up to sea state 4 |  |
|  | 4 | 40 m2 | 5 m ASL | 7 NM | 5 NM |
|  |  |  |  | Up to sea state 5 |  |
|  | 5 | 400 m2 | 8 m ASL | 10 NM | 8 NM |
|  |  |  |  | Up to sea state 6 |  |
| 50 m ASL | 3 | 4 m2 | 3 m ASL | 7 NM | 4 NM |
|  |  |  |  | Up to sea state 4 |  |
|  | 4 | 40 m2 | 5 m ASL | 11 NM | 8 NM |
|  |  |  |  | Up to sea state 5 |  |
|  | 5 | 400 m2 | 8 m ASL | 14 NM | 13 NM |
|  |  |  |  | Up to sea state 6 |  |
| 100 m ASL | 3 | 4 m2 | 3 m ASL | 10 NM | NIL |
|  |  |  |  | Up to sea state 4 |  |
|  | 4 | 40 m2 | 5 m ASL | 14 NM | 12 NM |
|  |  |  |  | Up to sea state 5 |  |
|  | 5 | 400 m2 | 8 m ASL | 18 NM | 19 NM |
|  |  |  |  | Up to sea state 6 |  |

For detailed analysis, the recommended method for determination of radar coverage and range performance is a combination of site inspections and radar system performance calculations, made by experts with a sound operational and technical knowledge about the subject.

Calculation of performance should be focused on the smallest targets of interest in poor weather conditions. All applicable losses should be included in the calculations. The probability of detection and false alarm rates used should comply with that required to meet the performance required for the individual VTS.

#### Propagation Conditions

Performance should, in all cases, be evaluated assuming standard atmospheric conditions. In addition, for each individual VTS the influence from adverse propagation effects should be analysed in detail for areas of the world having tropical climate and dry and hot climate.

Ducting may occur almost anywhere, and all systems should be designed to eliminate adverse effects from this. For most parts of the world evaporation ducting tends to persist most of the time, giving extended range, especially for low mounted antennas.  The effect will give average improvement in detection performance and may therefore be very useful in respect to security applications, if required.  The effect is usually not stable enough to be calculated as a benefit in safety applications.

#### Accuracy and target discrimination

Accuracy as well as range and bearing resolution/precision is necessary in order to have a clear and distinct appreciation of the movement of vessels, including those which are at anchor. Consult IALA Recommendation V128 for further guidance.

#### Tracking

Provision of target tracking, where computers automatically follow radar plots and provide information in synthetic form, is done by a plot extraction process followed by an automatic tracking process.

Plot extraction should be automatic in the entire VTS area covered by radar. Track initiation should be automatic, except in selected areas or manual depending on the concept of operations.

In automatic track initiation modes, all plots in a scan should be considered potential targets.  Some of the plots will be associated with previously established tracks, while the remaining plots should be considered as candidates for new tracks, tentative tracks.

Tentative tracks will become confirmed tracks if plots from consecutive scans “fit into the picture” within reasonable physical manoeuvrability limits, otherwise the tentative tracks are discarded.

The tracking system should be able to handle at least a certain number of tentative tracks and to initiate tracks and eventually to confirm tracks under certain conditions of Pd (probability of detection) and Pfa (probability of false alarm).

It should also be possible to initiate a track manually.  In manual track initiation, the operator using a graphical tool selects a plot on the radar display.  When selected, this plot should form the starting point for a tentative track, which eventually should be confirmed or discarded, as in the case of automatic initiation.

If automatically or manually created tentative tracks persist over a certain length of time the tracks should be promoted to confirmed tracks.  Confirmed tracks should be shown on the display.  The tracking system should be able to handle at least a certain number of confirmed tracks as recommended in IALA Recommendation V-128.

If a confirmed track either moves outside a user defined maximum range, into a user defined non-tracking area and the quality of the track falls below a predefined minimum, or if the track cannot be updated with new plots over a certain length of time, then the track should be terminated.  In certain cases the operator should receive a warning as defined by the VTS Authority.

False tracks may appear as a result of noise, clutter (including wakes) and ghost echoes.  The maximum number of false tracks allowed is dependant on role of the VTS, however, false tracks should be avoided in safety critical areas.

There is a trade-off between the time for confirmation of tentative track and the number of false tracks.  A longer confirmation time implies less false tracks and it should be possible to balance this trade-off in the setup of the VTS.

Track loss may occur as a result of Pd < 1 in combination with targets manoeuvring, especially in the vicinity of obstructions such as bridges. A level generally accepted is that each operator should correct up to one track loss per hour.

Swapping of track identity may occur as a result of targets moving close together or even merging for a period of time, especially if targets are overtaking with small difference in speed and course. A simple method of manual correction should be employed. In the case of AIS information being available for the radar track(s) in question, automatic correction should be performed. The problem may also be addressed by implementing operational procedures to separate targets or to prevent overtaking in critical areas.

The VTS authority should analyze critical areas, such as those in the vicinity of bridges, and provided a detailed explanation of their requirements with regard to tracking to VTS equipment suppliers to allow them to offer appropriate solutions.

1004	AUTOMATIC IDENTIFICATION SYSTEM (AIS)

Most VTS Authorities require vessels to report to the VTS Centre on approaching or entering the VTS area.  Achieving vessel identity relies on such vessels reporting both identity and location to the VTS Centre, and the VTSO then correlating this information with an unassigned radar track.

The process is time consuming and wholly reliant on the co-operation of participating vessels.  It is not uncommon for some vessels to fail to comply with this requirement, thereby creating a potentially dangerous situation, and creating further distraction for the VTSO.  Even where VHF direction finding equipment is fitted, the VTS traffic image is still reliant on vessels reporting identity via VHF thereby permitting the correlation of identity with the radar track identified by DF.

AIS has brought many benefits to VTS Authorities/Centres.  Principal amongst these is the automatic and immediate provision of vessel identity (MMSI, call sign etc), thereby facilitating rapid radio communication where necessary, thereby overcoming the safety weaknesses and time consuming procedures, inherent in the previous arrangements.

AIS should be integrated into the VTS. However, the VTS Operator should have the ability to choose whether to display the information and whether targets are to be displayed using the AIS derived data only or from the radar display only or from both, in accordance with the requirements of the VTS Authority.

Please refer to “Guidelines for use of AIS as a VTS Tool” put in new refer (AIS Ops Guidelines0 for further advice.

1005	Radio Direction Finder (RDF)

A number of VTS Authorities require RDF receivers to receive information on the position and assist in the identification of vessels, using their radio emission.

In order to ensure accurate localization, the use of two or more separate RDF stations is required.

All bearings should be automatically displayed on the chosen screen when the signal has been received after a delay of no more than 3 seconds. The bearings should remain visible on the chosen screen as long as the vessel is transmitting a signal.

.

Consideration should be given to the requirements for availability of the bearings after the vessel has ceased transmitting, such as through recording or instant replay capability.

RDF may be regarded as being complementary to AIS.  However, it may aid in the localisation of vessels not equipped with AIS.

RDF is not suitable of being used for continuous tracking.

# 1006	HYDROLOGICAL/METEOLOGICAL EQUIPMENT

It is essential that a VTS Centre has access to hydrological /meteorological (hydro/meteo) systems, which will provide local Hydro/meteo information relevant to the VTS Area(s) and can, if required by the VTS Authority, disseminate this to their users and allied services.

Where a VTS Authority determines a need to establish their own monitoring stations, the individual VTS Authorities should determine the accuracy and availability requirements for each VTS Centre. IALA Recommendation V-128 provides an indication of typical minimum accuracy and availability requirements.

Typical meteo variables are those provided by weather stations and include air temperature and humidity, wind velocity and direction, and visibility. In certain locations, hydro variables such as tidal level, tidal stream/current direction and velocity may be required. This data may be obtained through sensors or available in tables/databases from national authorities. Sensors

providing this data, usually located at remote sites, communicate the variables to a VTS Centre via a telecommunications link. At the VTS Centre, graphical and/or numeric information is presented for use by the VTS Operators.

Hydrological and meteorological information may be integrated into VTS applications to provide the VTS Operator a real-time assessment of the environmental situation in the VTS area of responsibility. Information collected from this equipment can be provided to ships to assist in assessing the waterway conditions.

A number of countries operate tide gauges and current meters to assist the prediction of tidal heights and streams or for the broadcast of real-time information to shipping. The Intergovernmental Oceanographic Commission (IOC) is responsible for coordinating the Global Sea Level Observing System (GLOSS) program to establish global and regional networks of sea level stations for providing essential information for international oceanographic research programmes.

# Closed circuit TV (CCTV) CAMERAS

The performance requirements placed on the CCTV service varies depending on traffic density, levels of VTS, special regional features and the coverage of the VTS area.

CCTV information may be integrated into VTS applications to provide the operator a real-time assessment of the situation in the VTS area of responsibility. Information collected from this equipment can be provided to ships to assist in assessing the waterway conditions.

IALA Recommendation V-128 provides an indication of minimum performance requirements.

- INFORMATION MANAGEMENT

It is the task of the information management within a VTS, to collect, process and correlate information from different sources in order to present an integrated image of the traffic, its environment and maintain situational awareness.

This information may include:

- Communications, internal and external;

- Sensor data, i.e. data used to generate the traffic image such as radar, CCTV, AIS;

- Shipping information data, i.e. vessel and cargo data, including vessel movement information;

- Meteorological and hydrological data; and

- Data from other sources if relevant.

In parallel to presentation of information to the operators, the information or part hereof, may be recorded and stored for later use.

- OPERATOR INTERFACE

The Operator Interface should include the display of the traffic image, including the traffic situation and corresponding traffic information.

#### Traffic Situation Display

A VTS display mapping graphics, analytical graphics and overlay information should be provided to enable a VTS Operator to have a concise picture of the geographical features, waterways and navigational lanes. When this information is being displayed, in many cases the radar video echoes of the coastline should be suppressed beyond the coastline, making this area of the display available for other synthetic information. Definitions of the requirements and accuracy of all the graphic possibilities need to be developed during the Project Definition phase.

If VHF direction finders are included in the system, the bearing lines should be shown on the VTS display, with the possibility for the operator to switch them off.

IALA has developed a Recommendation on the use and presentation of symbology at a VTS Centre (including AIS). VTS Authorities should refer to Recommendation V-125 for information on this matter.

#### Traffic Information Display

In addition to the traffic situation display a VTS should include display of textual information in the form of tables etc.

A list of participating vessels should include static information and dynamic information concerning the vessel, for example Vessels Name, Call Sign, IMO Number, MMSI, ETA, ETD, Course, Speed and Position, if required.

#### WORK ENVIRONMENT

#### VTS Operator positions

A console should be provided at every VTSO position with the equipment integrated in the most ergonomic arrangement.  The illumination of the operator position should be such that all relevant equipment such as the VTS monitor, communication equipment, target analyzer etc. can be monitored effectively and administrative tasks can be carried out at the same time.  Daylight displays for radar equipment have the advantage that they can be operated efficiently in normal room light levels.

Care should be taken when choosing the site for the consoles that VTSO positions do not interfere with each other especially regarding sound.

In addition to the sensors and the communication equipment, access to data base information is necessary at a VTSO’s position for reference purposes and for providing information to shipping.

#### Operational conditions, Redundancy and emergency precautions

#### The operational conditions for VTS equipment will vary from one place to another.  In some countries the availability and reliability requirements need to take into account extremes of temperature or precipitation, in others special attention may need to be paid to wind force (including gusts) or earthquake resistance. The maintenance and availability of remote sensors should be given special consideration and adequate redundancy should be provided.

To ensure adequate availability and reliability, vital parts of the VTS, e.g. all operational VTS communication services should have back-up systems with a power source independent from the normal power supply. The need for redundant sensors and possibly an alternative site for the VTS Centre should be considered in accordance with the guidelines provided by IALA Recommendation. V-119. In the case of an alternative site for the VTS Centre the operation should be capable of being easily transferred to the alternative location, in the event of an emergency or maintenance situation resulting in temporary closure of the primary VTS Centre.

To optimize the operations of the VTS Centre, the data information should be easily transferable to another location capable of maintaining the VTS service in the event of an emergency situation resulting in temporary closure of the VTS Centre.

#### When determining the position and range of radar equipment, the possibility of malfunction of a radar should be taken into account, and where practicable the arrangements should enable another radar, or AIS where appropriate, to provide cover for the sub-area or sector affected.

#### One or more spare VTSO consoles should also be considered to:

- Substitute for any console that is unserviceable;

- Be used for On-the-Job training;

- Where a need exists, be used by a VTS Supervisor;

- Where appropriate, be used in the co-ordination of emergencies.

Furthermore the VTS should not be entirely dependent on synthetic images of the traffic.  Instead a combination of radar video and extracted information is recommended. This will provide the VTSO with a means to verify track positions and the experienced VTSO will often be able to classify the individual target on the basis of the radar video.

A VTS System should also be flexible and easily updateable alongside of the routine operations of the VTS Centre, without the need for interrupting these VTS operations.

#### Availability and reliability of equipment

The equipment performance parameters are strongly dependant upon the services to be provided which influences the Mean Time Between Failures (MTBF) and the availability of the service.  Information on availability and reliability methods is given in the IALA Guide to the Availability and Reliability of Aids to Navigation.