# CHAPTER 5:   AREAS, TYPES AND CATEGORIES OF VTS

[Drafting group to provide definitions for Assistance and [Instruction – Direction] and review entire in light of the clarification; Table to be an Annex to the chapter]

# 0501  Introduction

In many waterways vessels can operate independently under all conditions of traffic structure and frequency and weather likely to be encountered.  However, there are many waterways in which vessels must rely on some level of interaction with persons ashore to safely and efficiently conduct their movements due to traffic congestion, physical limitations, adverse weather or vulnerable environments.  In the extreme case, there are areas in which vessels must rely entirely on information, advice or directions from ashore in order to complete a safe transit. It is therefore necessary to take all these factors into account, in order to properly assess how they impact with each other in contributing to the level of risk. For instance, in some circumstances the need for a VTS may be deemed inappropriate or excessive, based on the Competent/VTS Authority’s risk assessment.

# 0502	Areas of Operation

Waterways are divided into non- VTS areas and VTS areas. The distinction between the two being based on the following key attributes of a VTS:

  - Ability to interact

  - Maintain a traffic image

  - Respond to developing situations

  - Forward Movement planning

  - Authority to direct

Type I. Unaided

Non

VTSS

Type II. Port/Local Information InformationPort/Local Information

Type V. Navigational Assistance Service

A special relationship with a single vessel

VTS

Type III. Information Service

Type IV. Traffic Organisation Service

Fig1: Types of Non VTS and VTS

0503	VTS Areas of Operation

Non VTS Areas: Areas without a VTS Service.

Key point

The Area in which a VTS is operated can be divided into one of the following:

- Port or Harbour and Approaches;

- Inland Waterways;

- Coastal Waters;

- Offshore; and

- Regional.

A VTS Centre/Authority may be authorised to operate in a combination of one or more of the above areas.

# 0504	VTS Area Definitions

Port or Harbour and Approaches - A VTS operating within the area of jurisdiction of a port or harbour, and its seaward approaches.

Inland Waterways - A VTS operating exclusively within the confines of an inland waterway and may therefore encompass a inland port, as well as rivers and canals.

Coastal Waters - A VTS operating within the costal waters of a Competent Authority. This may include International Straits and waters governed by Treaty. A Coastal VTS is mainly concerned with vessel traffic passing through the area.

Offshore – A VTS operating in an area not covered by any of the above, which may or may not be outside the jurisdiction of a Competent Authority, such as around an Offshore Production Platform or Renewable Energy Farm/Field.

Regional – A VTS operated by or on behalf of a Competent Authority or several Competent Authorities operating under a formal agreement.

# 0505  Types of Service

The Type of services provided by a non- Vessel Traffic Service and a Vessel Traffic Service should be determined by the waterway, traffic and navigation conditions in any given area.  Because these conditions can vary considerably, there are also varying types of services among non- VTSs and VTSs, as is illustrated in the figure No. xx above.  A systematic means is needed to classify these according to the services they provide and consequently the waterway conditions they address.

For instance, a Vessel Traffic Service, if appropriately equipped, staffed and operated, can mitigate certain risks inherent in navigation.  However, a VTS can be established and operated with varying levels of capability and authority.  Determining the appropriate type of service and capability to apply to certain conditions is important from a cost benefit standpoint.

Taking the two extremes of vessels operating completely independent from shore-side influences and the situation in which navigation cannot proceed without intervention from ashore, gives two anchor points from which a continuum of VTS capability types can be established.  Within this continuum are defining intervals that describe various types of VTS according to their respective capability to mitigate risk.  These are the key attributes described earlier.

These types of non-VTS and VTS can be used in the course of a risk assessment/mitigation exercise to determine in conjunction with other risk mitigation mechanisms, whether a VTS is necessary and at what level of capability and competence.  The types are defined as:

Type I:	Unaided (Non VTS)  Vessels navigating in these areas can safely proceed in all likely conditions of traffic and weather without any assistance from ashore.  In this scenario, the vessel can obtain all the information it needs to complete a passage by using its own on board resources, including pilots.

Type II:	Port/Local Information (Non VTS)  This case introduces a resource ashore that provides information to the bridge team.  Provision of Port Information is designed to improve port safety and co-ordination of port services within the port community by dissemination of port information with vessels and berth or terminal operators. It is mainly concerned with the management of the port, by the supply of information on berth and port conditions. Provision of Port Information can also act as a medium for liaison between vessels and stevedores or allied services, as well as providing a basis for implementing Port Emergency Plans.

Examples of Port Information may include:

- Details of shipping movements;

- Visibility in the area;

- Wind speed and direction;

- Tidal height;

- Berthing information;

- Preferred anchorages.

The main difference arising from provision of Port Information is that it does not provide VTS.  It is not required to have the ability and/or the resources to respond to developing traffic situations. Neither is there a requirement for a vessel traffic image to be maintained (see Type III below).  In addition, the training requirement for its operators is less comprehensive and the operators are unlikely to be certified to the V-103 standard. In summary, services from such a resource are limited to information only; the organization does not maintain a traffic image; does not interact with traffic or respond to developing traffic situations.  The organization does not participate in on board decision-making and lacks the authority to direct the movement of a vessel.

Type III: 	Information Service (VTS)  This type introduces a Competent/VTS Authority with resources to maintain a traffic image, interact with traffic and respond to developing traffic situations.  An information service ensures that essential information is available in good time to assist the shipboard navigational decision making process.  This information may concern:-

- the positions, intentions and destinations of vessels.

- amendments and changes in promulgated information concerning the VTS area such as boundaries, procedures, radio channels or frequencies, reporting points etc.

the variables influencing the navigation of vessels such as meteorological and hydrological conditions, notices to mariners, status of aids to navigation, traffic congestion and special vessels with limited manoeuvrability which may impose restrictions on the navigation of other vessels or any other potential hindrances.

An Information Service does not participate in on board decision-making and lacks the authority to direct the movement of a vessel.  All or certain classes of vessels may be required to provide information to this organization and to establish and maintain communication during their transit.

Key point

Type IV: 	Traffic Organisation Service (VTS)  This service possesses the capability of an Information Service (Type III) and additionally exercises the authority to direct the movements.

A Traffic Organisation Service is a service to identify and manage potentially dangerous traffic situations and to provide for the safe and efficient movement of traffic within the VTS area*.  Traffic Organisation concerns the forward planning of movements and is particularly relevant in times of congestion or when the movement of special transports may affect the flow of other traffic.  Monitoring the traffic and enforcing adherence to governing rules and regulations are integral parts of traffic organisation.

*This definition differs marginally from that contained in IMO Resolution A.857(20) but does not conflict with its intent. It is designed to ensure that the VTS exercises a proper duty of care.

The service may include establishing and operating a system of traffic clearances in respect of the priority of movements, the allocation of space, the mandatory reporting of movements, establishing routes to be followed, speed limits to be observed and such other measures as may be considered necessary and appropriate by the VTS.

When the VTS is authorised to issue instructions to vessels, these instructions should be result- oriented only, leaving the details of the execution to the master or pilot ….

IMO Resolution A.857(20)

All or certain classes of vessels will be required to participate in this service and shall not proceed without clearance.

VTS sailing plans are an instrument of traffic organisation and are a major source of information to the VTS.  A VTS sailing plan normally includes the estimated time of arrival in the VTS area or the departure from a berth or an anchorage in the VTS area.  The VTS Authority may consider making a procedural distinction between sailing plans established for efficiency reasons, for the co-ordination with allied services, and VTS sailing plans established for safety reasons, which may affect the general flow of traffic.

The VTS Authority should state the vessels or categories of vessels for which a VTS sailing plan is compulsory.  VTS sailing plans should be adhered to as far as practicable and where the execution of a VTS sailing plan could affect general traffic movements the VTS Centre should advise other shipping of the relevant details of the plan.

The VTS Authority should specify the information required in the VTS sailing plans for all vessels or for special vessels according to the local circumstances.  In exceptional circumstances the VTS sailing plan may be amplified at the request of the VTS.  The VTS may advise changes to the VTS sailing plan to take account of the traffic situation or special circumstances.  After the VTS sailing plan has been agreed between the vessel and the VTS, the vessel is permitted to participate in the VTS and should as far as practicable try to maintain the plan.  If special circumstances, or the safety of the traffic so require, the VTS may request the vessel to follow an amended VTS sailing plan, indicating the reasons for its request.  In areas where automatic tracking facilities are not available, regular position reporting by radiotelephony may be required from the vessel.

Annex 1/ Page xxx depicts an example of the Risk Mitigation Levels that can be attributed to both non-VTSs and VTSs.

### Type V: 	Navigational Assistance Service (VTS)

A navigational assistance service is a service to assist the navigational decision making process on board and to monitor the effects, especially in difficult navigational or meteorological circumstances or in case of defects or deficiencies.

A VTS can contribute to the decision making process by giving navigational information such as:

- course and speed made good by a vessel;

- position relative to fairway axis and way-points;

- positions, identities and intentions of surrounding traffic;

warnings to individual vessels

A VTS can participate in the decision making process by giving navigational advice.

Key point

The Competent/VTS Authority should be aware of the distinction between navigational information and navigational advice and should determine whether navigational advice can and may be given from the shore and if so under what circumstances and to what extent.

Navigational assistance is given at the request of the vessel or if deemed necessary by the VTS and should only be given if positive identification has been established and continuous communication can be maintained throughout the process. The beginning and the end of navigational assistance should be clearly stated by the vessel or the VTS and acknowledged by the other party.

A Navigational Assistance Service will include an information service.

A Traffic Organisation Service may also include both information and navigation assistance services.

Key point

There are two classes of Navigational Assistance Service.  These are Contributory and Participatory, depending on whether navigational information or advice is given.

Contributory Navigational Assistance Service - A Contributory Navigational Assistance Service is solely the provision of factual navigational information to assist the on-board decision making process.

The information is provided either in response to a specific request from a vessel or when the VTS Authority perceives that the information would be of use to the vessel.  A Contributory Navigational Assistance Service may include information on :

  - Courses and speeds made good;

  - Positions relative to fairway axis and waypoints;

  - Positions, identities and intentions of surrounding traffic;

  - Warnings of dangers.

Under specified conditions a VTS Centre delivering a Navigational Assistance Service may provide Passage Assistance to facilitate the safe transit of a vessel where a pilot onboard the vessel is not a requirement, or not practically achievable or is not considered warranted by the level of risk.

Note: A VTS that provides a Traffic Orgaisation Service may also offer Passage Assistance. Passage Assistance does not involve the giving of instructions.

Participatory Navigational Assistance Service - In a Participatory Navigational Assistance Service, the VTS can become involved in the on-board decision making process by providing navigational advice. Through the exchange of information between vessel and VTS, an agreed outcome should emerge. However, any recommendations from the VTS must be result oriented and must not include specific instructions [directions] on courses to steer and speed through the water. As with the Contributory service, it is provided on specific request or when perceived necessary by the VTS Authority, in the interests of safety.

Dependent on the complexity of the situation and the level of risk mitigation required, consideration should be given to the following :

  - [Authorisations of operators providing the service and recording of such authorisations;

  - The need to reflect this category of service in the On the Job Training of VTS Operators;

  - Operator work load during Participatory Navigational Assistance Service, including other responsibilities and activities, and the number of vessels being monitored or advised;]

  - Courses and speeds made good;

  - Positions relative to fairway axis and waypoints;

  - Positions, identities and intentions of surrounding traffic;

  - Warnings of dangers.

  - Use of a dedicated frequency;

  - Increased traffic restrictions;

- In addition, VTS Authorities should give careful consideration to staffing levels and their qualifications when giving consideration to implementing this type of service.

Where a VTS Centre also has the authority to direct the movement of a vessel and delivers a Traffic Organisation Service it may also provide Passage Instruction [Direction] to facilitate the safe transit of a vessel where a pilot onboard the vessel is not a requirement, or not practically achievable or is not considered warranted by the level of risk.

[Include Message marker use sentence – link to further explanation in Chapter 14]

# 0506  Categories Of Service

Introduction

A Formal Risk Assessment provides the basis for establishing the need for a VTS and is the fundamental process whereby hazards to the safety of navigation and need for protection of the environment may be identified, the risks estimated and then evaluated (Chapter 7).

Formal Risk Assessment increasingly forms the basis upon which a Safety Management System is derived. As it is upon this Safety Management System that the scope and functionality of the VTS is determined, it follows that it is fundamental in defining the category of the VTS.

Categorisation

When considering the development and implementation of VTS, the VTS Authority will need to decide on the Type of service that they provide, the availability of the service and the training standards of the staff that provide the service. Type and availability of service go to make up the Categorisation of the VTS.

There are two potential beneficiaries of “categorising” VTS, namely:

- The mariner, who needs to know what services and activities a specific VTS is able to provide and perform and under what circumstances;

- The VTS Authority and/or the Competent Authority, in deciding what VTS may be required to mitigate identified risks, and thereby what Type and availability of VTS should be procured.

The categorisation of a VTS, designed to benefit the mariner, is expressed in terms of a coding mechanism (see Chapter 13), which is based on similar regimes already in place in the aviation and maritime worlds. The code is designed to offer essential information, on the Type and availability of the service, to the mariner in a simple, easy to read format. It is a compilation of VTS elements that are the core requirements for categorisation, as identified by IALA. The use of the code enables sub-areas or sectors within a VTS area to be categorised differently, if necessary.

Promulgation of Information

When developing a VTS and having determined the type and availability of service, consideration should be given to the means of promulgation of this information. In order to assist users of the IALA/IAPH/IMPA/IFSMA/IHMA World VTS Guide(review in light of possible changes), VTS authorities are able to declare the details of the services they provide in a format compatible with the coding mechanism. On submission of the requisite information it is entered into and made available to mariners through the Guide. In order to ease identification of the VTS entry, within the Guide, it is prefixed by VTS. Further information about promulgation of information, including the coding mechanism can be found in Chapter 13.

# 0507	CERTIFICATION OF VTS

[Ensure consistency with chapter 11 and 12]

The responsibility for determining the type and standard of VTS required to mitigate identified hazards lies with the VTS authority and/or the Competent Authority. They are accountable for the standards they set, the resources, including staffing levels and their qualifications that they allocate and for the effectiveness of VTS they choose to adopt. It follows that appropriate and adequate operational and administrative procedures should be developed in order to provide the service effectively.  The Certification of a VTS is concerned with assuring that the operational and administration procedures used by a VTS Authority provide the services declared in its Categorisation.  Certification can be achieved by an appropriate auditing and accreditation process.

It is a national responsibility to determine whether the Competent Authority has taken action to ensure that the VTS authority/centre has appropriate procedures and practices in place to meet the declared standards.

[Annex 1]

VTS Risk Mitigation Levels - VTS as a tool to assist in the Reduction of Risk

|  | ## Type | Name |
| --- | --- | --- |
| Non-VTS | I | Unaided |
|  | II | Port / Local Information |
| VTS | III | Information Service (INS) |
|  | IV | Traffic Organisation Service3 (TOS) |

V Navigational Assistance Service (NAS)

- Contributory Assistance - Participatory Assistance - Passage Assistance

- Passage Instruction

|  | Type | Name | TYPE OF SUPPORT |  |  | FUNCTION |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Information | ###### Advice | Instruction1 2 | ###### Interaction | Maintain a Traffic Image | Respond to developing situation | Participation in On-Board Decision Making | Movement Planning |
| Non-VTS | ### I | Unaided | N | N | N | N | N | N | N | N |
|  | II | Port Information | Y | N | N | N | N | N | N | N |
| VTS | III | INS | Y | N | N | Y | Y | Y | N | N |
|  | III (a) | INS and NAS (Contributory) | Y | Y | N | Y | Y | Y | N | N |
|  | III(b) | INS and NAS (Participatory) | Y | Y | N | Y | Y | Y | Y | N |
|  | III(c) | INS and NAS (Passage Assistance) | Y | Y | N | Y | Y | Y | Y | Y |
|  | IV | TOS | Y | N | Y | Y | Y | Y | N | Y |
|  | IV(a) | TOS and NAS (Contributory) | Y | Y | Y | Y | Y | Y | N | Y |
|  | IV(b) | TOS and NAS (Participatory) | Y | Y | Y | Y | Y | Y | Y | Y |
|  | IV(c) | TOS and NAS (Passage Assistance) | Y | Y | N | Y | Y | Y | Y | Y |
|  | IV(d) | TOS and NAS (Passage Instruction) | Y | Y | Y | Y | Y | Y | Y | Y |

1 Instructions should be result oriented

2 Where the VTS has the regulatory power to do so

3 A TOS includes an IS