Chapter 6 Principles of Traffic Management

<Taken From VTS21/6/1 – Production of new edition for Publication in 2008>

Introduction

Development of safe standard operating procedures

Principles of safe waterspace and traffic management

Two-way and one-way traffic occupancy; channel width, length, points-of-no-return, ship size and manœuvrability.

High-risk ships and cargoes

Risk induced by geography and hydrography

Other factors that can increase risk.

Allocation of waterspace. Slot management

Authorisation of ship movements, allocation of priority

Provision of navigation assistance

Traffic organisation service

Passage Assistance – Geoff – suggest we use this term to be consistent with the draft for Chapter 5 Types of VTS and Categories of Service)

# CHAPTER 6:   PRINCIPLES OF TRAFFIC MANAGEMENT

0601  Introduction

This chapter discusses the principles that need to be taken into account to ensure sound traffic management and the establishment of a safe VTS environment.   Such study will enable a VTS authority to determine, and subsequently keep under review, those factors that are vital to the maintenance of safe waterways and to identify vulnerabilities which, unless properly mitigated, could lead to increased or inadequately defined risks.

All VTS are unique due to different circumstances..  It is the purpose of VTS to reduce the affects of these differences by creating operating environments that reduce or eliminate the difficulties of navigation and therefore limit the risk to which vessels are exposed.

<where is the charter/mandate to standardize between VTSs, TOR for VTS committee, IALA constitution>

### 0602  Development of safe standard operating procedures

<refer to 2.8 of Chapter 2  of VTS Manual>

VTS authorities are responsible for ensuring that the objectives of a VTS are met. This includes ensuring that the standards set by the competent authority for levels of service and operator qualifications are adhered to.

Levels of delegated authority applying to VTS staff should be clearly identified in the documented internal operational procedures of a VTS. Such procedures should be an integral part of a verifiable safety management system for the VTS. A properly implemented quality control programme approved by the competent authority can ensure that the standards set for the type and level of service are consistently maintained, and that the service is delivered safely and effectively.

- Refer to IALA Recommendation V-127 – Operational Procedures for VTS Edition 1.0 - June 2004

Operational procedures should be defined before the Commissioning and Acceptance phase for all the routine operations of a VTS, and the general performance of personnel in any given situation.  The procedures should set out the actions to be taken in respect of interaction with ships, interaction with other parties and authorities and contingency situations both internal and external.

The VTS Authority should decide the level of decision which can be taken within the VTS centre before reference should be made to an outside organisation, to the authority itself or to the national government.  This may be considered as deciding the boundary between routine and emergency actions and will assist in the preparation of emergency procedures within the VTS area.

## 0603

# 0604    Fairways and Channels– Safety Criteria

An authority will need to determine the safety criteria for safe shipping movements and to establish procedures to ensure that the correct conditions are met before movements are permitted. It is not the purpose of this chapter to provide a definitive explanation of all the factors involved but rather to outline the general procedures and to provide advice where more detailed information may be obtained.  The Permanent International Association of Navigation Congresses (PIANC) has published guidance on these topics that should be consulted.  The following is a synopsis of some of the available information; the details of the calculations are to be found in PIANC documents.

Definitions

Channel Width	The width of the dredged channel, designed for deep draught vessels.

Fairway Width	The width of the buoyed channel, which may be greater than channel width, permitting shallower draught vessels to navigate safely outside of the deep draught channel, whilst remaining within the buoyed fairway.

Outer Channel	One which is exposed to wave action that may cause a reduction of underkeel clearance by a significant amount.

Inner Channel	Generally sheltered and where vessels are not subject to wave action.

Safety Criteria The general safety criteria that need to be established in relation to a vessel, or vessels, passage of a channel are:

- Water Reference Level

- Vessel Draught

- Underkeel Clearance (UKC)

- Channel Width

# Water Reference Level  The calculated water level of the sea in an approach channel for a selected vessels maneuver taking into account the lack of precision in tidal predictions, anomalies due to adverse meteorological conditions and the change to water depth that may occur during the planned movement of a vessel, including its possible delay.

# Height of Tide predictions, in the form of local Tide Tables, together with Tidal profile curves, should be used to calculate the water depth that will be available throughout a vessels movement.  This calculation should include an allowance for any unforeseen adverse meteorological condition and any tidal change in the event that the vessels movement is delayed during its transit.  Tidal gauge information should either, confirm that the tidal movement conforms to the prediction made, and that the calculation is accurate or, that the tide is not conforming to that predicted and that an additional correction may need to be made.

# Vessel Draught   A vessels draught should be measured forward and aft, and in the case of a large vessel (ie VLCC) that could be subject to sag, an amidships reading should also be taken.  This will provide the vessels deepest physical draught for the movement operation to be undertaken.  It is recommended that a safety factor of 0.5 metres is added to this figure; the result being used for subsequent calculations.

To ascertain that an adequate depth of water exists requires several factors need to be taken into account and, in some cases monitored during a vessels movement.  These include Water Reference Level, a vessel’s Draught and a suitable allowance for Underkeel clearance (UKC).

# Underkeel Clearance

UKC is a complex topic and a key risk management and safety feature.  Its calculation involves making allowance for additional margins to compensate for some or all of the following conditions :  Vessel construction, Water Density, Squat, Wave & Swell allowance and Bottom type, whether the seabed comprises mud, gravel or rock.  An additional safety margin should also be considered.

Channel Width   The calculation of safe channel width depends on a variety of factors. Principal among these are vessel beam width and whether single or two-way traffic is envisaged, but allowance needs also to be made for a vessel’s maneuverability and the affects of channel banks, cross currents, aids-to-navigation, cargo hazards and other related factors that can impact on the safe width required. It also assumes that the safe water depth for a given vessel, as calculated above, extends across the entire width of the channel required.

## PIANC has derived a number of methods of calculating channel width from a rule of thumb to a more detailed analysis depending on the conditions anticipated.

0605

# Management of Ship Movements

- Legal Basis (Refer Chapter 2 for intro)

The jurisdiction that a VTS has outside of its territorial waters is limited by the provisions of the United Nations Conference on the Law of the Sea (UNCLOS).  However, it often occurs that a VTS is sited in close proximity to an IMO approved traffic separation scheme, and transgressions of the scheme may be reported to the offending vessel as information, and to the flag state of the vessel concerned for action, under the Regulations for the Prevention of Collision at Sea (COLREGS).  Therefore operational procedures should be very detailed regarding the exact arrangements to be followed in observing the incident, recording the radar pictures, informing the ship in a precise form of words, making the log entry and producing the report to be forwarded to the appropriate authority

- Authorisation of ship movements

- Allocation of priority (Link to Operational procedures)

- Allocation of waterspace.

- Time Slots – management

- Interaction with other services (including adjacent Services)

- Selection and Use of VTS Services (Refer Chapter 5)

  - INF

  - NAS

  - TOS