CH 15 of VTS Manual

Operational Records, Archives and Replay

Introduction

The nature of VTS operations is such that there may be a requirement to access, analyse and review previous events.  There is a requirement, therefore, for the capture, secure storage, retrieval and presentation of VTS related information.

Advances in data storage techniques now make possible archiving and retrieval options that may have appeared unachievable only a short period ago.  For example, storage and retrieval of basic raw data may be enhanced by the added capability of recording operator actions, the Human Machine Interface (HMI), which may prove invaluable in justifying the actions of VTS Staff in post incident analysis as well as improving the efficiency of VTS operations.

This chapter provides guidance on recording, archiving and replay techniques that a VTS Authority may wish to consider in selecting systems and procedures that are appropriate to their needs.

Purpose of Recording and Replay

Recordings may be required for the following purposes:

- Review of an accident or incident investigation;

- Use as evidence following an accident or incident;

- Technical evaluation and to check the function and performance of sensors etc;

- Quality monitoring of the operation of VTS as a whole and to allow for continuous improvement.

- Statistical analysis of traffic patterns etc within the VTS area;

- Training purposes.

Types of Data to be Recorded

The following areas should be considered for data capture (IALA Recommendation V-127 art 2.1.1), as appropriate:

- Radio Communications

- Telephone Communications

- Sensor data used to generate the traffic image such as:

  - Radar

  - AIS

  - CCTV

  - VHF DF

  - Long-range sensor data.

- Fused sensor data (track data, traffic image, etc.)

- Port Management Information Systems which may include:

  - Shipping information i.e. vessel and cargo data, including vessel movement information.

  - Pilotage management

  - Allied service provisions i.e. tug and line handing allocations.

- Meteorological and hydrological data;

- Logs and textual records;

- Operator actions (HMI).

Recording frequency and sampling rates

The frequency of sampling for recorded data sets should be appropriate for each specific type of data (e.g. continuously for audio, but not so for Met or Hydro data).

The relevant authority should define the period of time and temporal resolution of sensor data and other tracking performance parameters depending on traffic density and types of tracks.

While the frequency of individual data items may differ from item to item, the recording of all data sets should be continuous and time stamped to a common time frame.  A VTS system should have a master time reference to which all components and recordings are aligned.

Proper care should be taken by the relevant authority when considering the recording process and data storage with regard to failure to record or unwanted loss of recordings.

Storage of Recordings

IMO recommends a minimum of 30 days for other shore side activities (such as SAR) as the time-period to allow for the full retrieval of data post-incident/accident.  It can be assumed that this requirement is appropriate for VTS and applies to all data sets that may be used for incident replay.  As this data will be recorded in a rolling loop of, for example the most recent 30 days data, there is a requirement to store recordings for a period of time to safeguard recorded data in case of an incident.  It should be easy for a VTS operator or supervisor to archive a period of recorded data to other media (e.g. DVD-ROM, tape storage or similar).

Certain data should be considered for longer term storage in support of such benefits as analysis of traffic patterns and their changes over time, waterway usage changes, input for analysis of changes to buoyage and other aids to navigation and other such strategic vessel traffic management uses.  It is possible that such long-term archival of data is beyond the capability or responsibility of the VTS; the capabilities of other entities should be considered for this purpose (e.g., archival or statistical administrations).

A capability should be provided to store recordings of specific incident data beyond the minimum storage time or to produce a permanent record for legal, regulatory or analysis purposes.

Consideration should be given to securing recordings from unauthorized access or tampering, particularly those to be used in accident investigation or legal action.

The large file size of some data items such as audio or CCTV images, may necessitate moving of the data to another media (DVD-ROM or similar) for longer term storage.  This may also be a consideration in deciding whether to record and store original (raw) video, or digital (extracted) images as presented to an operator.

To ensure that records are consistent and complete, the data recording process should normally be automated and consideration should be given to the provision of a stand-alone replay system that does not interfere with the VTS function.  Consideration should be given to allow the retrieval of VTS information in standard formats (e.g., delimited text files or extensible markup language (XML)).

Replay System

Any replay system must ensure that times are accurately identified so that the traffic situation can be rebuilt during replay.  VTS Authorities may consider the integrated and synchronised replay of different data sources to aid incident review.  Replay may be required for the following purposes:

- Technical replay – using previously recorded data to fine-tune the system;

- Operator replay – replay of data sets as seen by VTSO for internal analysis;

- External replay – ‘standalone’ replay functionality, for example when replaying to a court or official inquiry.