Annex 1- 4 may j pedresen says make table nbr format same as other tables

Recommendation on the core operational requirements for VTS

1.	INTRODUCTION	2

1.1	Abbreviations	2

1.2	Supporting Documents	3

2.	VTS RADAR Service	4

2.1	General	4

2.2	Characteristics of the Radar Target	4

2.3	Radar Detection Performance and Disturbances	5

2.4	Radar Accuracy and Discrimination	6

2.5	Availability	7

2.6	Calculation	8

3.	AUTOMATIC IDENTIFICATION SYSTEM (AIS)	9

3.1	Availability	9

4.	COMMUNICATIONS	9

4.1	Very High Frequency (VHF)	9

4.2	Long Distance Communication	9

4.3	Radio Direction Finder (RDF)	9

4.4	Communication with Allied Services	10

4.5	Availability	10

5.	HYDROMETEO EQUIPMENT	11

6.	CLOSED CIRCUIT TV CAMERAS AS A VTS SENSOR (CCTV)	12

6.1	General	12

6.2	Detection Performance of CCTV Cameras	12

6.3	Characteristics of the CCTV	12

6.4	Availability	12

7.	VTS DATA SYSTEM	13

7.1	General	13

7.2	Control of Displayed Information	13

7.3	Long Range Sensor Data	13

7.3.1	Availability	13

7.4	Emergency Situations	13

7.5	List of Participating Vessels	15

8.	GATHERING AND RECORDING OF INFORMATION	Error! Bookmark not defined.

Recommendation on the core operational requirements for VTS

# INTRODUCTION

The purpose of this Annex is to give an overview of the core operational performance requirements for VTS including:

- Radar

- Automatic Identification System (AIS)

- Communications

- Closed Circuit TV (CCTV)

- Hydrometeo

- VTS Data System

Further details are given in Annexes 2 and onwards

Requirements of the VTS equipment may have a high impact on acquisition and life-cycle costs of a VTS and therefore the performance recommendations are divided into three different capabilities:

- Basic - applicable to VTS information service and, where applicable, navigational assistance service.

- Standard - applicable to all types of VTS as identified by IMO – information service, navigational assistance service and traffic organizational service – for areas with medium traffic density and/or without major navigational hazards.

- Advanced - applicable to VTS areas with high traffic density and/or specific major navigational hazards.

These capabilities may be used as applicable within a VTS, e.g. part of a VTS area may call for a Basic and another part may call for a Standard capability.

## Abbreviations

| AIS | Automatic Identification System |
| --- | --- |
| ASL | Above Sea Level |
| CCTV | Closed Circuit Television |
| CPA | Closest Point of Approach |
| EIA | Electronics Industry Association |
| ETA | Estimated Time of Arrival |
| ETD | Estimated Time of Departure |
| IALA | International Association of Marine Aids to Navigation and Lighthouse Authorities |
| IEEE | The Institute of Electrical and Electronic Engineers |
| IMO | International Maritime Organization |
| ITU | International Telecommunication Union |
| MMSI | Maritime Mobile Service Identity |
| MRCC | Maritime Rescue Co-ordination Centre |
| nm | Nautical Mile |
| RDF | Radio Direction Finder |
| RMP | Recognized Maritime Picture |
| S-band | 2.0 – 4.0 GHz |
| TCPA | Time to Closest Point of Approach |
| VHF | Very High Frequency |
| VTS | Vessel Traffic Services |
| VTSO | Vessel Traffic Services Operator |
| X-band | 8.0 – 12.0 GHz |
| º | Degree |
|  | Less than or equal to |
|  | Plus or minus |

## Supporting Documents

|  | IEEE Std 686-1997 | IEEE Standard Radar Definitions |
| --- | --- | --- |
|  | The International Organisation for Standardisation (ISO) | ISO 8729 Ships and marine technology – Marine radar reflectors |
|  | EIA Standard RS-170 | Electronics Industry Association Recommended Standard RS-170 |
|  | IALA guideline XXXX | Guideline for VTS radar service |

# RADAR Service

## General

The performance requirements placed on the VTS radar service vary a great deal depending on traffic density, type of VTS, regional features and the VTS coverage area.  Basic functions will be provided by the VTS Radar, enhanced functions may be provided by the VTS processing system. The purpose of this document is to describe the general performance requirements of the radar service. By meeting these general requirements, it is possible to provide the service required of the VTS radar equipment in the VTS coverage area.

The VTS Radar System should be capable of performing the functions shown in Table 2.1 These parameters will assist in the development of the traffic image.

:

Table 2.1 Performance Functions

| Parameters / Capability | Basic | Standard | Advanced |
| --- | --- | --- | --- |
| Path, time and track prediction, |  |  | X |
| CPA, | X | X | X |
| TCPA, | X | X | X |
| Anchor watch, |  |  | X |
| Vessels vector, | X | X | X |
| Course, speed and label/identity, | X | X | X |
| Collision alerts. | X | X | X |

## Characteristics of the Radar Target

The radar system’s detection and measurement capacity depends on the characteristics of the radar and the target. These include the target’s average reflective area and its oscillation in relation to the time and the measuring frequency used. The distribution of the target’s reflective parts, the incidence angle and turning speed have a major impact on the power reflected back from the target.

The detection capacity required of the system is determined according to the type of service, traffic density and potential navigational hazards. Table 2.2 below describes the typical reflection features and type of capability requirements of VTS targets.  The table illustrates typical target parameters, including reflection characteristics, and identifies the detection capabilities required for differing types of targets.

Following assessment of the type of targets required to be detected, which may differ within the VTS area, the table identifies the type of capability required and defines typical characteristics for use in setting performance standards.

When selecting a radar system the system should be designed in such a way that the defined target types can be detected and tracked reliably in the required area covered by the VTS service in visibility conditions, at precipitation rates at sea states and in propagation conditions relevant for the individual radar site.

Note that Radar Cross Section and height of the radar cross section varies substantial with aspect ratio and physical details for the individual target within each category. The figures in table 2 are conservative, recommended values for the type of targets listed,.

Table 2.2  Target Reflection Features and Type of Capability recommended

| TARGET |  | Type of Capability |  |  | Design Requirements |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Basic | Standard | Advanced | Radar cross section |  | Height of Target |
|  |  |  |  |  | S-band | X-band |  |
| 1 | Aids to Navigation etc. –without radar reflector. Small open boats, fibreglass, wood or rubber with outboard motor and at least 4 meters long, small speedboats, small fishing vessels, small sailing boats and the like. |  |  | X |  | 1 m2 | 1 m ASL |
| 2 | Inshore fishing vessels, sailing boats, speedboats and the like. |  |  | X |  | 3 m2 | 2 m ASL |
| 3 | Aids to Navigation with radar reflector. |  | X | X | 4 m2 | 10 m2 | 3 m ASL |
| 4 | Small metal ships, fishing vessels, patrol vessels and the like. | X | X | X | 40 m2 | 100 m2 | 5 m ASL |
| 5 | Coasters and the like. | X | X | X | 400 m2 | 1,000 m2 | 8 m ASL |
| 6 | Large coasters, bulk carriers, cargo ships and the like. | X | X | X | 4,000 m2 | 10,000 m2 | 12 m ASL |
| 7 | Container carriers, tankers etc. | X | X | X | 40,000 m2 | 100,000 m2 | 18 m ASL |

## Radar Detection Performance and Disturbances

Factors disturbing and restricting the performance of radar systems include noise as well as interference and clutter signals from various sources. Each radar site should be designed and equipped with devices to reduce the adverse effects of rain and sea clutter and enhance the probability of target detection per scan. The radar should also be designed and installed so as to eliminate to the maximum extent possible, false echoes caused by side lobes or reflections from nearby structures. When selecting a radar system and the measuring frequency, regional special conditions such as heavy rainfall should be taken into account.

There are several technological solutions available to raise the performance of the radar system to the required level. Most typical solutions include higher average transmission power, larger antennas, circular polarization, reducing the receiver noise, sector blanking and more effective processing and filtering of the transmitted and received signal.

## Radar Accuracy and Discrimination

The measurement of accuracy and discrimination of the system is determined by the VTS authority based on Type of Capability. The recommendations for target separation for different types of capability are listed in Table 2.3.

Table 2.3 Target Separation and Accuracy

| Radar accuracy and physical separation between small point targets for discrimination in display and tracking |  |  | Type of Capability |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Basic |  | Standard |  | Advanced |  |
|  |  |  | Display | Tracking | Display | Tracking | Display | Tracking |
| In range |  | Short range applications (<5 nm coverage – include waterways, harbours etc) | 25 m | 40 m | 20 m | 30 m | 15 m | 25 m |
|  |  | Long range applications (up to 20 nm coverage – littoral waters, offshore etc) | 75 m | 100m | 60 m | 75 m | 50 m | 60 m |
|  |  | Very long range applications (>20 nm coverage) | N/A |  | 100 m | 125 m | 80 m | 100 m |
| In azimuth | X-band | Angle between targets as seen from the radar | 1.2o | 1.3o | 0.7o | 0.8o | 0.55o | 0.6o |
|  |  | Or distance in meters, whichever is the greater | 25 m | 40 m | 20 m | 30 m | 15 m | 25 m |
|  |  | Corresponding –3 dB antenna horizontal beam width | ≤0.7o |  | ≤0.45o |  | ≤0.45o |  |
|  | S-band | Angle between targets as seen from the radar | N/A |  | 3.5o | 4o | 1.8o | 2o |
|  |  | Or distance in meters, whichever is the greater | N/A |  | 20 m | 30 m | 15 m | 25 m |
|  |  | Corresponding –3 dB antenna horizontal beam width | N/A |  | ≤2o |  | ≤1.25o |  |

The system should be designed in such a way that the defined radar accuracy and discrimination can be achieved in the entire area(s) covered by the VTS service. In long measuring distances, the impact of the height and type of antenna on the measuring accuracy and resolution should be taken into account.  The system should also be capable of displaying and tracking all targets of interest simultaneously in normal conditions without the need for manual adjustments by the operator.

## Availability

The VTS authority should define the requirements for the availability of the radar service. The recommendations for availability are listed in Table 2.4 below.

Table 2.4  Availability

| Availability for radar service |  |  |  |
| --- | --- | --- | --- |
|  | Type of Capability |  |  |
|  | Basic | Standard | Advanced |
| Recommended availability for the radar service | 99 % | 99.6 % | 99.9 % |

Availability is defined in IMO Resolution A.915 (220 Ref.40) as:

“The percentage of time that an aid, or system of aids, is performing a required function under stated conditions.  The non-availability can be caused by scheduled and/or unscheduled interruptions”.

Where the coverage from two or more radar sites provides overlapping coverage, as long as one of the radars is available and provides similar capabilities in the area of waterway normally covered, the requirements for radar service are met.  Thus from a service perspective, unusable time for a waterway can be calculated as the time when any portion of the waterway is without a usable radar.

The radar coverage provided by adjacent countries, may, if agreed, be taken into account when calculating the performance of the VTS radar.  In addition, in areas of high traffic where two waterways meet with converging traffic the location of radar sites should be determined in a manner that minimises shadowing.

Single coverage: Unless dual coverage is provided throughout the whole coverage area, some VTS sites provide a mix of overlapping and single radar coverage. VTS sites that provide single coverage to at least some portion of a waterway are considered to be single coverage sites,

## Calculation

Administrations may choose to calculate service availability using one of two methods:

- by waterway model, or

- by radar site/ radar site combination model

Waterway availability model: In this model administrations need to define which waterways are high risk and which waterways are low risk. Separate calculations for high and low risk are required, providing both exist within the coverage area. Individual waterway availability calculations are then averaged to produce one figure for each waterway risk category. If desired, a figure for each waterway may be reported.

Radar site availability model: In this model, administrations must define which radar sites serve low risk waterways and which serve high risk waterways. The overall availability is calculated by averaging the availability of the associated individual radar sites as illustrated in the examples below.

Example 1: An Advanced site receives complete radar coverage. For example, a VTS area having two radar sites where at Radar site A the availability was 99% and Radar site B’s availability was 99,7%, but both radar sites suffered coincidental outages during 0,1% of the period. Although neither radar site individually met the availability target the combined availability was 99.9% and met the performance goal achieved.

Example 2: A Basic site receives coverage from radar site A (as above) covering a low risk waterway. Radar site C, adjacent to radar site A, provides completely overlapping coverage to the low risk waterway portion served by radar site A. Radar site C’s performance was 99%, but neither unusable period (of radar sites A and C) coincided. The low risk waterway covered by radar site A received usable signals for 100% of the time.

While these two distinctly different situations have different availability requirements based upon the level of risk, the concept of availability remains consistent – the minimum requirement is met for the coverage area.

Note: Higher availability percentage targets may be applicable to more critical parts of the VTS area.  Risk assessment may support lower availability percentage targets in less critical areas.

# AUTOMATIC IDENTIFICATION SYSTEM (AIS)

Where both AIS and radar data are available they should be fused and presented to the VTSO as one unambiguous target using best available target data.  However, the VTS Operator should have the ability to choose whether to display the information on a sensor basis.  That is, whether individual targets are to be displayed using the AIS derived data only or the radar derived data only or from both subject to track fusion in accordance with the requirements of the VTS Authority.

If AIS information shows differences when compared to other sources, VTSOs should use appropriate procedures and evaluate the obtained information and where applicable advise the vessel immediately.

## Availability

The requirements for AIS availability is a matter for the VTS Authority to determine.

# COMMUNICATIONS

Reliable communications are essential to deliver VTS services and coverage should be available throughout the VTS Area from one or communication technologies as listed below.

## Very High Frequency (VHF)

The VTS Authority should utilise dedicated working VHF Channels designated by the National Radio Authority for specific types of operations.  In addition, one or more VHF Channels may be utilised in different sectors of the VTS Area.

It is common for the VTS to have its own independent VHF network, for the use within specifically designated VHF Channels.

The VHF equipment must comply with national and international regulations.

## Long Distance Communication

In the case where a VTS Authority requires long distance communication such as pre-arrival information, any available communication systems should be used and therefore an independent network is not required.

## Radio Direction Finder (RDF)

A number of VTS Authorities require RDF receivers to identify the target of a transmitting vessel on VTS display.  This may be used to correlate with the radar target.

In order to ensure accurate identification on the VTS display the use of two or more separate RDF bearing stations are required. Bearing angles on the target should be as close to 90º as possible.  The recommended bearing accuracy for the types of capability are provided in Table 4.5

Table 4.5  Bearing Accuracy

|  | Type of Capability |  |  |
| --- | --- | --- | --- |
|  | Basic | Standard | Advanced |
| Recommended Bearing Accuracy |   2.5º |   1.5º |   1.0º |

All bearings should be automatically displayed on the VTS display when the signal has been received after a delay of no more than 3 seconds. The bearings should remain visible on the VTS display as long as the vessel is transmitting a signal. The VTS operator should have the ability to suppress RDF information on the VTS display.

It is foreseen that this requirement may decrease as AIS usage by VTS Authorities increases, but will not become obsolete as, depending on the circumstances, AIS information may not always be available.

RDF is not suitable of being used for continuous tracking.

## Communication with Allied Services

VTS Centres should be equipped with the ability to communicate with relevant allied services by the use of reliable communication networks. It is recommended that VTS Centres should be equipped with a digital switched network, with caller identification.

## Availability

The requirements for the availability of Communications is a matter for the VTS Authority to determine.

# HYDROMETEO EQUIPMENT

It is essential that a VTS Centre has access to local Hydrometeo information relevant to the VTS Area(s) and can, if required by the VTS Authority, disseminate this to their users and allied services.

Where a VTS Authority determines a need to establish their own monitoring stations, it should be noted that the individual VTS Authorities should determine the accuracy and availability requirements for each VTS Centre, as these will be based on individual circumstances. Table 5.6 gives an indication of typical minimum accuracy requirements.

Note: The target availability should be as prescribed by IMO A.915(22).

Table 5.6 Indication of typical minimum accuracy

| Parameter | Minimum Accuracy |
| --- | --- |
| Height of Tide |   0.10 m |
| Rate of Tidal Stream/ Current |   0.5 knots |
| Direction of Tidal Stream/Current |   10 Degrees |
| Wave height |   10% of the height |
| Wave Direction |   20 Degrees |
| Wind speed |   1m per sec |
| Wind Direction |   10 Degrees |
| Visibility |   10% of the distance |
| Air Temperature |   10C |
| Air Humidity |   5 % |
| Air Pressure |   2 hPa |
| Sea Surface Temperature |   10C |

The VTS Authority should specify the time periods over which the various data parameters should be updated and may be averaged, if required, as these factors will depend upon the local circumstances pertaining to the VTS Centre.

# CLOSED CIRCUIT TV AS A VTS SENSOR (CCTV)

## General

The performance requirements of a CCTV service vary depending on traffic density, type of VTS service, special regional features, coverage of the VTS area and the intended use.

VTS Authorities should consider the need for low-light level, colour, intensified and laser-gated low-light level, as well as digital image processing and video compression of CCTV installations.

## Detection Performance of CCTV Cameras

Depending upon the circumstances, cameras should be capable of identifying the type and possibly the name of the vessels concerned.

The CCTV should enable identification of the type of vessel at a minimum range of 3 nm from the camera location, but this will depend on the individual circumstances, including local topography. The identity of a vessel by shape, colour and other features should be capable of being determined normally at a minimum range of 1 nm, but again this will depend on the individual circumstances. These ranges are based on where the nominal visibility is in excess of 10 nm. Image quality and update/refresh rates should meet these requirements.

## Characteristics of the CCTV

Cameras may be used either in low traffic density areas where this is a more cost effective solution to radar or in conjunction with radar as an additional sensor, depending on the level of risk.

The camera should be capable of automatically tracking a vessel manually selected by the VTS Operator via the VTS Screen, if required by the VTS Authority. In addition, there should be the possibility for the VTS Operator to manually de-select the automatically acquired target and manually select another target and/ or area of the acquired target, in order for the VTS Operator to perform specified monitoring tasks, such as pilot embarkation/disembarkation.

Where more than one camera is installed to cover a VTS area it is desirable for the output from both to be fused together to provide one composite picture.

## Availability

The requirements for the availability of CCTV is a matter for the VTS Authority to determine.

# VTS DATA SYSTEM

## General

A VTS Data System should have the capability to be flexible and easily upgraded and maintained alongside the routine operations of the VTS Centre without the need for interrupting the service.

VTS Centres should operate within a dual server environment to minimise disruption to normal operations.

## Control of Displayed Information

To ensure the best possible traffic image is maintained the duty VTSO should have the capability to select the primary target source being displayed for individual vessels or all vessels.  For example, selecting the radar target for a vessel with a faulty AIS unit or the AIS network is unreliable.

## Long Range Sensor Data

Where AIS, radar and Long Range Sensor Data (e.g. LRIT positional data) are utilised they should be fused and presented to the VTSO as one unambiguous target as shown in Figure 7-1. However, the VTS Operator should have the ability to choose whether to display the information on a sensor basis.  That is, whether individual targets are to be displayed using the AIS derived data, the radar derived data, or the LRIT derived data only, or from all three subject to track fusion in accordance with the requirements of the VTS Authority.

If Long Range Sensor Data information shows differences when compared to other sources, VTSOs should use appropriate procedures and evaluate the obtained information and where applicable advise the vessel immediately.

### Availability

The requirements for Long Range Sensor data availability is a matter for the VTS Authority to determine.

## Emergency Situations

To optimize the operations of the VTS Centre the data information should be easily transferred to another location capable of maintaining the VTS service in the event of an emergency situation resulting in temporary closure of the VTS Centre.

Figure 7-1 Long range image

## List of Participating Vessels

The list of participating vessels should include static information and dynamic information concerning the vessel, for example Vessels Name, Call Sign, IMO Number, MMSI, ETA, ETD, Draft, Course, Speed and Position, if required.

An example of a typical list is shown in Figure 7-2. This example is taken from the Baltic AIS Network which is integrated to the VTS system

Figure 7-2 Example of List of Participating Vessels –Source: Archepelago VTS, FMA

# RECORDING, ARCHIVING AND REPLAY

Provision should be made for the storage, security, retrieval and presentation of this information.

The data type, resolution and period of time for which information gathered by a VTS is required to be stored should be identified in internal procedures. This time period should be such that it allows for the full retrieval of data post-incident/accident, in compliance with national requirements and those of the incident/accident investigation procedures of the VTS authority and other authorised parties. This type of information should include:

- Communications, internal and external as defined in IALA Recommendation V-127

- Sensor data, i.e. data used to generate the traffic image such as radar, CCTV, AIS and long-range sensor data.

- Shipping information data, i.e. vessel and cargo data, including vessel movement information.

- Meteorological and hydrological data; and

- Data from other sources if relevant.

- Synchronization of voice / track data

The IMO recommends a minimum of 30 days for the time-period to allow for the full retrieval of data post-incident/accident.  The VTS authority should define the period of time and temporal resolution of sensor data and other tracking performance parameters depending on traffic density and types of tracks.

If required by the VTS Authority, the data should be recorded automatically and capable of being replayed onto a separate replay system.

Recording, Archiving and Replay

The period of time for which information gathered by a VTS is required to be stored should be identified in internal procedures. This time period should be such that it allows for the full retrieval of data post-incident/accident, in compliance with national requirements and those of the incident/accident investigation procedures of the VTS authority and other interested parties. This type of information may include:

- Communications (internal and/or external);

- Sensor data (i.e. data used to generate the traffic image such as radar, CCTV, AIS);

- Shipping information data ( e.g. vessel and cargo data, including vessel movement information);

- Meteorological and hydrological data; and

- Data from other sources.

Provision should be made for the storage, security, retrieval and presentation of this information.

IALA Recommendation V-127 On Operational Procedures for Vessel Traffic Services Edition 1 June 2004

Reference IALA Guideline 1035 on Availability and Reliability of Aids to Navigation, December 2004