Guideline for VTS Radar

Table of Contents

1.	Introduction	2

1.1	Scope	2

2.	References	3

3.	Definitions and Clarifications	4

3.1	Definitions	4

3.2	Software tools	4

3.3	Abbreviations	5

4.	Radar services	7

4.1	Radar Parameters	7

4.1.1	Radar frequency	8

4.1.2	Antenna parameters	8

4.1.3	Transmitter characteristics	9

4.1.4	Receiver characteristics	9

4.2	Pulse compression radars	10

4.3	Continuous wave radars	10

4.4	Solid State Radars	10

4.5	System losses	11

4.6	Radar Target Characteristics	11

4.6.1	Target properties	11

4.6.2	Radar cross-section	12

4.7	Influence from wind farms	13

4.8	Radar Range performance	14

4.8.1	Antenna height	15

4.8.2	Propagation conditions	15

4.8.3	Lobing	16

4.8.4	Shadow effect	17

4.8.5	Tide	17

4.9	Noise and clutter affecting radar performance	17

4.9.1	Thermal noise	18

4.9.2	Surface and volume clutter	18

4.9.3	Precipitation	19

4.9.4	Man-made noise	19

4.9.5	Multiple reflections (ghost images)	19

4.10	Ice	20

4.11	Target discrimination and position accuracy	20

4.11.1	Range resolution	20

4.11.2	Azimuth resolution	21

4.11.3	Range accuracy	21

4.11.4	Azimuth accuracy	21

4.12	Signal processing, extraction and tracking	21

4.13	Security implications	22

5.	Performance verification methods	24

# Introduction

Performance requirements for VTS radars are generally different to the requirements for marine navigational radars.  VTS radars normally need to operate simultaneously on short and long range and this leads to dynamic requirements that far exceed those required onboard a ship.

Furthermore, weather related phenomena such as ducting will influence VTS radars more than ships’ radars.  This can have a significant influence on the performance – either positive or negative.

Good clutter suppression is needed for sea clutter and, in most parts of the world, for rain clutter as well.  In addition, the need to see small targets in rough weather conditions is essential, especially if objectives include detection of targets for security purposes.

The introduction of AIS further develops VTS into a modern information system, and the presentation of radar information needs to follow the trend, putting new demands on the radar performance.  Antenna side lobes and ghost targets (multiple reflections) may lead to false and dangerous results when radar returns and AIS plots are associated.  High precision is therefore required to allow for unambiguous correlation of position obtained from two information sources.

## Scope

The aim of this document is to provide comprehensive guidance on the Radar subjects relevant for the VTS applications as a supplement to IALA  V-128.

# References

| [1] | IEEE Std 686-1997 | IEEE Standard Radar Definitions |
| --- | --- | --- |
| [2] | Merrill I Skolnik | Introduction to Radar Systems, McGraw-HILL Higher Education, ISBN 0-07-290980-3 |
| [3] | P.D.L. Williams, H.D, Cramp and Kay Curtis, | Experimental study of the radar cross section of maritime targets, ELECTRONIC CIRCUITS AND SYSTEMS, July 1978. Vol 2. No 4. |
| [4] | Ingo Harre | RCS in Radar Range Calculations for Maritime Targets. http://www.mar-it.de/Radar/RCS/RCS_18.pdf |
| [5] | IMO | Performance Standards for radar reflectors (latest edition) |
| [6] | International Telecommunications Union (ITU) | ITU-R SM.1541 Unwanted emissions in the out-of-band domain |
| [7] | International Telecommunications Union (ITU) | ITU-R SM.329-9 Spurious emissions |
| [8] | The International Organisation for Standardisation ( ISO) | ISO 8729 Ships and marine technology – Marine radar reflectors |
| [9] | IALA | IALA Recommendation V-128 on Operational and Technical Performance Requirements for VTS Equipment |

# Definitions and Clarifications

## Definitions

For general terms used throughout this annex refer to:

- IEEE Std 686-1997	IEEE Standard Radar Definitions.

Specific terms are defined as follows:

- Availability is the probability that a system will perform its specified function when required.

- Normal weather and propagation conditions are the conditions persisting 99% - 99.9 % of the time as defined by the individual VTS authority.  The rest of the time is considered having adverse weather and propagation conditions.

- Polarisation of a radar signal is determined by the orientation of the electrical field. In the case of circular polarisation the field rotates left or right.

- Radar as referred to in this document relates to all aspects of the radar from sensor through to the presentation of radar to the VTS operator.

- Radar PD is the probability of detection at the output of a radar, subsequent to signal processing and plot extraction, but prior to tracking, and presentation.

- Reliability is the probability that a system, when it is available performs a specified function without failure under given conditions for a given period of time.

- Sea characteristics include wave/swell height, direction and speed of waves/swell and distance between waves/swell.

- Squint are the angular difference between the axis of antenna rotation and a selected geometrical axis.

- Track swap is the transfer of a track identity (track label) to another track.

## Software tools

| CARPET | Computer Aided Radar Performance Tool TNO (Toegepast Natuurkundig Onderzoek) Physics and Electronics Laboratory, P.O.Box 96864, 2509 JG The Hague, Netherlands, http://www.tno.nl |
| --- | --- |
| AREPS | Advanced Refractive Effects Prediction System Space and Naval Warfare Systems Center, San Diego, http://sunspot.spawar.navy.mil. |

## Abbreviations

AIS	Automatic Identification System

AREPS	Advanced Refractive Effects Prediction System

ASL	Above Sea Level

CARPET	Computer Aided Radar Performance Evaluation Tool

CW	Continuous Wave

dB	Decibel

dBi	Decibel isotrope

dBm	Decibel milliwatt

DF	Direction Finder

FTC	Fast Time Constant

GHz	GigaHertz

GIT	Georgia Institute of Technology

GPS	Global Positioning System

IALA	International Association of Marine Aids to Navigation and Lighthouse Authorities

ICAO	International Civil Aviation Organization

IEC	International Electro-Technical Commission

IEEE	The Institute of Electrical and Electronic Engineers

IMO	International Maritime Organisation

ITU	International Telecommunication Union

kHz	kiloHertz

Ku-band	12.0 – 18.0 GHz

kW	kiloWatt

LNFE	Low Noise Front End

m	metre

m2	square metre

MDS	Minimum Detectable Signal

MHz	MegaHertz

mm/h	millimetre per hour

m/s	metre/second

MSC	Maritime Safety Committee

MTBF	Mean Time Between Failure

MTTR	Mean Time to Repair

N/A	Not applicable

nm	Nautical Mile

PD	Probability of Detection

PFA	Probability of False Alarm

PRF	Pulse Repetition Frequency

PW	Pulse Width

R	Range

RCS 	Radar Cross Section

RF 	Radio Frequency

S-band	2.0 – 4.0 GHz

UTC	Universal Time Co-ordinated

UTM	Universal Transverse Mercator

VTS	Vessel Traffic Services

X-band	8.0 – 12.0 GHz

µs	microsecond

º	Degree

>	Greater than

≥	Greater than or equal to

≤	Less than or equal to

# Radar services

Implementation of radar services for VTS is a complex issue and requirements of the VTS radar may have a high impact on acquisition and life-cycle costs of a VTS. The performance recommendations given by IALA V-128 are therefore divided into three different capabilities:

- Basic - applicable to VTS information service and, where applicable, navigational assistance service.

- Standard - applicable to all types of VTS as identified by IMO – information service, navigational assistance service and traffic organizational service – for areas with medium traffic density and/or without major navigational hazards.

- Advanced - applicable to VTS areas with high traffic density and/or specific major navigational hazards.

These capabilities may be used as applicable within a VTS, e.g. part of a VTS area may call for a Basic and another part may call for a Standard capability

The individual VTS authority should decide on the level of specification and define requirements related to climate and topography as identified in this document.  Consideration of other local conditions may dictate individual performance requirements that are independent of the level of recommendation.

Requirements to detect very small vessels for safety or security reasons may further extend requirements, beyond those identified by IALA V-128 for ‘Advanced recommendations’.

## Radar Parameters

Important Radar Parameters influencing radar coverage are:

- Radar frequency band;

- Radar location;

- Antenna height;

- Local weather;

- Transmitted power;

- Antenna characteristics;

- Receiver sensitivity;

- Receiver dynamic characteristics;

- Processing capabilities;

- System losses.

The parameters are often dependent on each other, and different vendors may choose different methods to solve the same issue.  Therefore, it is recommended that overall radar performance requirements are specified, taking the local weather into consideration, rather than specifying radar parameters.

However, in order to understand how the parameters affect radar performance, the influence from some of the parameters are described as follows.

### Radar frequency

Radar frequencies selected for VTS lie typically within the S-band and X-band frequencies, however, higher frequencies such as Ku band may also be utilised.

The major operational effects of different frequencies are:

- For a given physical antenna size, azimuth resolution will increase with higher frequency;

- Antenna gain and, thereby, overall system sensitivity, increases with higher frequency for a given physical antenna size. However, system losses also increase with frequency and may be severe at very high frequency, such as Ku band;

- Radar cross section of targets in general will increase with frequency, however, specific types of targets may give particularly strong returns in a given frequency band;

- Disturbance by precipitation increases with frequency.

The majority of VTS services use X-band radars as a best compromise, especially since technologies for rain clutter suppression have matured.  Also, as a result of production volume, X-band radars are the least expensive.

The second most used frequency is S-band, due to better weather penetration in heavy rainfall.

Frequency band allocation is granted by the ITU, whereas permissions to transmit on given frequencies are granted on a national basis.

Modern transmitter and processing technologies allow several radars in close vicinity to transmit on the same frequency without running the risk of disturbing each other.  The same techniques provide protection against interference from other radars, e.g. radars onboard ships.

### Antenna parameters

The antenna gain is one of the most important factors determining the radar range.  The gain is mainly determined by the operating frequency and the horizontal and vertical beam widths of the antenna array.

Narrowing the horizontal as well as vertical beam widths will result in reduction of the volume clutter and thereby improved performance in precipitation.  The horizontal beam width is also the governing parameter in respect to azimuth resolution and it has significant influence on the amount of surface clutter received.  The vertical beam characteristics are the governing factor in respect to coverage at short range.

Use of circular polarization will reduce unwanted returns from precipitation while maintaining good returns from targets of interest to the VTS operator.  However, some radar reflectors are specifically designed for linear polarisation and their visibility may therefore be reduced by the use of circular polarisation.

If small non-metallic targets, such as fibre glass, wooden, rubber or rigid hull inflatable boats, are identified by the VTS as a target of concern, then the selection of antenna polarization is even more complex and should be studied carefully.

Use of high antenna rotation speed increases the scan data rate, but the hits per scan (echo returns per scan from a particular target) is decreased correspondingly.  In some cases this may mask a weak target that would otherwise be detected by using a lower antenna speed.

#### Side lobes

A small part of the radio frequency energy from each transmitted pulse is radiated outside the main antenna beam, producing side lobe patterns whose axis in the horizontal plane have various angles with respect to the main beam.

Side lobes have no effect in case of distant or small surface targets, but the echo from a large target at short range may produce an arc similar to a section of a range ring, or they may appear as a series of echoes forming a dotted arc.

Side lobes may also occur far from the main beam, producing targets that are very difficult to discriminate from real targets.

The side lobe levels are entirely dependent on the antenna design. Far side lobe characteristics are very important, as side lobes far from the main beam may result in false targets, appearing to the operator with the same characteristics as a valid target.

### Transmitter characteristics

For traditional pulsed radars the average power and peak pulse power has an influence on the detection range.  However, its importance is often overestimated.  High peak power often results in higher losses in the transmission lines of the radar and it will always result in a larger amount of unwanted information (e.g. sea clutter).

Shorter pulse lengths provide better range resolution, a reduction of returns from rain and sea clutter and better overall performance.  The use of relatively short pulses (50 to 300 nanoseconds) is often beneficial for VTS applications.

### Receiver characteristics

The receiver sensitivity (often expressed by the receiver noise factor) is one of the main factors with respect to detection of small distant targets in clear weather.  The noise figure may be expressed in different terms such as ‘Low Noise Front End (LNFE) noise figure’ and ‘overall noise figure’, making it difficult to compare radars from different vendors.

A better expression for receiver sensitivity is the Minimum Detectable Signal (MDS); however, this is not ideal either as it may be difficult to measure this figure in a reliable manner.

The dynamic characteristics are very important, especially if operational conditions require:

- Simultaneous detection of targets on short and long range;

- Supply of signals for more than one display with different range settings;

- Simultaneous supply of signals for different operational roles;

- Supply of signals for automatic video processing and automatic tracking.

Minimum detectable signals at the receiver input and processed onwards lie typically at -90 to -105 dBm in VTS applications, or may be as low as -115 dBm for security applications.

Very large targets at close range are, however, not fully within the beam at short range, limiting the input at the receiver to about + 10 dBm for the typical case.

Consequently, the overall dynamic range needed for the individual radar for a VTS application is typically between 100 and 125 dB.

## Pulse compression radars

For pulse compression radars the pulse compression circuit in the receiver will generate so called ‘time side lobes’ which may appear as weaker, false signals in front of and behind large targets.

Special attention should be paid to this effect.

## Continuous wave radars

For continuous wave radars similar effects as for pulse compression radars can be observed due to the receiver processing.

In addition, spill-over of energy between transmitting and receiving antennas may limit the useful dynamic range.

The use of small antennas allowed by the technology may lead to reduction of azimuth resolution and result in higher susceptibility to rain and sea clutter.

Special attention should be paid to those effects

## Solid State Radars

Solid State Radars will typically employ Pulse Compression or Continuous Wave techniques. See paragraph 4.2 and 4.3

## System losses

System losses reduce performance, especially the ability to detect small distant targets.  Common reasons for system loss are:

- Long waveguide runs;

- Moisture in the waveguide runs;

- Flexible and especially flexible/twistable waveguide parts. (Not to be confused with elliptical waveguides which, in general, have small losses);

- Teflon or plastic gaskets used instead of proper pressure windows, especially if mounted between plain and choke flanges;

- Excessive number of joints in the waveguide system;

- Poor rotary joints;

- Poor workmanship in the manufacture of waveguide parts;

- Matching losses in receivers and distribution of signals, both with inadequate bandwidth / speed;

- Digitalisation of signals using too few bits;

- Losses in detection and tracking.

Furthermore, discontinuities in waveguide installations can result in reflections causing the centre of the radar picture to be totally saturated.  In extreme cases this can even disturb the transmitted pulse, giving reduced angular resolution as well as reduced performance with respect to discrimination of targets from clutter.

## Radar Target Characteristics

To understand the behaviour of radar targets it is essential to know that the strength of the echo returned from a target is largely governed by the size of the target, its aspect and its height above the radar horizon. However, these are not the only factors and a small target of good reflecting properties may well give a stronger echo than a much larger target of poor reflecting properties.

### Target properties

Aspect angle (i.e. the angle at which the radar pulse “hits” the target) is an important factor affecting echo strength. Certain types of objects that might be expected to give a good echo prove to be very poor radar targets because they reflect most of the energy that strikes them in a direction other than straight back to the radar.  The worst of these are gently curved smooth surfaces, such as sandy beaches, sandbanks, mud banks, and gently sloping hills that are regular in shape and not covered with trees or thick undergrowth. For the same reason lighthouses that are conical in shape do not give as good a response as might be expected.

Vertical, comparatively smooth surfaces, such as cliffs and isolated buildings, give strong reflection provided they face the radar.

Complex structures such as metal ships with several corners also give strong reflections, whereas ships made of other materials typically give weaker reflections.

### Radar cross-section

The characteristics of a target are defined by its radar cross-section (RCS) and the fluctuations in RCS.

Formally, the RCS is defined as the ratio between the power [in W] scattered by the target back towards the radar receiver and the power density [in W/m2] hitting the target. Thus RCS is measured in m2 and has the dimension of an area. There is, however, no simple relation between the physical area of the target and the RCS, as the reflected power depends on the angle of incidence as well as target properties such as material (conducting/non-conducting), coating, physical shape (smooth/sharp corners) etc.

The total reflection from a target is made up from the ‘elementary’ reflections from different parts of the target.  These elementary reflections will interfere constructively or destructively to form the total reflection.  Therefore, small changes in attitude of the target may result in large changes of the amount of reflected power.  The consequence of this is that, in radar performance evaluations, the RCS of a target is best described by a statistical distribution function. Different kinds of wanted targets (ships, buoys, etc.) and unwanted targets (sea clutter) have different distribution functions.  These differences can be exploited in the radar signal processing to enhance target visibility.

Scientific references giving information about the RCS of objects relevant for VTS can be found in 'Introduction to Radar Systems' [2] and in 'The experimental study of the radar cross-section of maritime targets' [3].  However, since the date of the study, ship design has resulted in larger variation in RCS and experience is that the modified values in Table 4-1 give appropriate values of RCS for various targets.

Comprehensive RCS data for other frequency bands were not available to be included in this document; however, different tests of radar reflectors have yielded differences between X-band and S-band of up to 11 dB. [5]

From evaluation of figure 2.17 in [2], [4] and in the absence of more detailed investigations, a given X-band RCS should be reduced to 40% for S-band range calculations.

Excluded from this are returns from birds and flocks of birds where the RCS of the individual species often results in a higher RCS on S-band than on X-band.

The RCS is also polarisation dependant. Especially; non-metallic targets may not be seen if using circular polarisation.

Furthermore it should be noted that RCS is a figure with highly statistical properties.  Facing this, the lower values in should be used for range calculations. The upper values are relevant for determination of dynamic requirements to the radar services.

## Influence from Wind farms, Power cables and other like obstructions

The main influence from wind farms etc will typically be reflections.

The symmetrical lay out of wind farms may result in unwanted ghost echoes.

VTS radars will normally not be dependent on Doppler shift and they are therefore not affected by the rotation of wind turbines.

Table 4-1  Typical target characteristics.

| Target types |  | Typical characteristics at X-band |  |  |
| --- | --- | --- | --- | --- |
|  |  | RCS | Height | Fluctuations etc. |
| 1 | Aids to Navigation without radar reflector. | Up to 1 m2 | 1 - 4 m ASL | Rapidly fluctuating, highly dependent on sea characteristics. |
| 2 | Aids to Navigation with radar reflector. | 10 – 100 m2 |  | Rapidly fluctuating, wind and currents may tilt to blind angles and lobing may cause reflectors to be in blind spots. |
| 3 | Small open boat, fibreglass, wood or rubber with outboard motor and at least 2 persons onboard, small speedboat, small fishing vessels or small sailing boats. | 0.5 – 5 m2 | 0.5 to 1 m ASL | Rapidly fluctuating may be hidden behind waves up to 50 % of the time. Slow moving targets tend to lie lower in the water than fast moving ones and therefore RCS visible to the radar tends to increase with speed. |
| 4 | Inshore fishing vessels, sailing boats and speedboats, equipped with radar reflector of good quality. | 3 – 10 m2 | 1 – 2 m ASL | Rapidly fluctuating. |
| 5 | Small metal ships, fishing vessels, patrol vessels and other similar vessels. | 10 – 100 m2 | 2 - 4 m ASL |  |
| 6 | Coasters and other similar vessels. | 100 – 1000 m2 | 6 - 10 m ASL | RCS is highly dependent on aspect angle of the individual vessel. Rate of fluctuations is typically moderate. |
| 7 | Large coasters, Bulk carriers, cargo ships and other similar vessels. | 1000 – 10000 m2 | 10 – 25 m ASL |  |
| 8 | Container carriers, tankers and other similar vessels. | 10000 – 1000000 m2 | 15 - 40 m ASL |  |
| 9 | Buildings, cranes. Stacks of containers and other large structures. | Up to 1000000 m2 | Depend on site | Insignificant. |
| 10 | Floating items, oil drums and other similar items. Birds, floating or flying. | Up to 1 m2 | 0 to 0.5 m ASL | Rapidly fluctuating, highly dependent on sea characteristics. |
| 11 | Flocks of birds. | Up to 3 m2 | Sea level and up | Rapidly fluctuating, flight paths tend to be characteristic of given species in given areas of interest. |

## Radar Range performance

The range performance of a radar system is the result of many effects.  In calm weather and under normal atmospheric conditions it depends mainly on the radar specifications and the antenna height, however sea and weather conditions affect the radar range as described in the following sections.

### Antenna height

The figures below illustrate how the height of the antenna above the water line affects the maximum and minimum detection range performance.

Figure 4.1  Target range and visibility.

### Propagation conditions

In some areas of the world, especially those with hot, dry climates, propagation effects cause radar performance to vary drastically over time – sometimes from hour to hour.

The density of the atmosphere is not uniform with altitude.  The gradient of density results in a bending of the radar waves in a manner similar to the bending of light waves by an optical prism.  Water vapour is the atmospheric component chiefly responsible for the bending of radar waves in the lower atmosphere as illustrated in Table 2.3.  Temperature also has an influence.

The temperature and humidity gradients in the air above the sea surface cause the radar beam to be bent up or down - in most cases down - and to follow the earth’s curvature to some extent, thereby elongating the range for potential target detection. This condition is called super-refraction, or ducting.

The air in contact with the ocean’s surface is saturated with water vapour.  A few metres above the surface the air is not usually saturated, so there is a decrease of water vapour pressure from the surface to some value well above the surface.  Such a change in the moisture distribution, without an accompanying temperature change, forms a so-called evaporation duct, always leading to extended radar range if present.

The atmosphere may also form a trapping layer and generate a wave-guide like duct, a surface-based duct or an elevated duct.

Under trapping layer conditions a radar antenna situated within the duct may receive extremely long-range returns.  The result includes a considerable increase in the amount of clutter and false echoes generated by objects beyond the radar ambiguity range (2nd and multiple-time around echoes).

Above a strong duct (a trapping layer) there is often a layer of low refraction (sub refraction).  An antenna situated above a trapping layer or, even worse, above more than one trapping layer, will radiate most of its energy into the sub-refractive layer, which will not direct the energy down towards targets on the sea or on the ground.  Therefore, the radar will detect only very short-range targets.  This may limit usable antenna height in dry hot regions of the world, especially in Middle Eastern countries.

Severe atmospheric conditions, such as thunderstorms or the acute build-up of static electricity during pre-storm situations also have the affect of distorting the radar owing to the contorting effect of the electrical disturbances in the atmosphere.

Figure 4.2  Effects of propagation conditions.

The radar range is limited by the earth’s curvature and is a function of antenna elevation and target height.  A first estimation of its value, in nautical miles (NM), can be derived from the equation:

R (NM) = 2.2 (hA + hT)

This assumes standard atmospheric conditions as defined by the International Civil Aviation Organization, (ICAO).  Antenna height hA and target height hT are measured in metres.

However, it should be noted that detection of large targets can only be expected to the theoretical radar horizon.  Complex calculations, such as those illustrated in Figure 4.14, give a better impression of what may be expected.  A software tool such as can be used for complex calculations (for example, the CARPET).

### Lobing

The surface of the earth or the sea not only restricts the line of sight, but it may also have an effect on the radar coverage of targets within the line of sight.  Two waves arrive at the target via two separate paths:  One travels along the direct path from radar to target; the other path includes a reflection from the water surface.  The two waves can interfere destructively or constructively according to their difference in phase.  This effect is known as lobing.

One of the effects of lobing is that point targets, such as buoys, may become invisible to a radar at certain ranges and in certain conditions as illustrated by Figure 4.3.

Figure 4.3  Coverage diagram for a typical VTS radar.  Detection probability PD is given as a percentage as shown in the lower bar in the figure.

### Shadow effect

For determination of shadow effect, the propagation of radio waves at radar frequencies can be considered as for optical visibility in clear weather.

### Tide

Tide may have a substantial influence on coastlines, riverbanks etc. as well as the antenna height above sea level.

## Noise and clutter affecting radar performance

Noise is the part of the radar signal which is uncorrelated to the transmitter pulse, whereas clutter is unwanted radar return signals from e.g. sea waves, precipitation or land areas.

Targets, as well as various sources of noise or clutter, behave statistically in different ways.  Knowledge of how to limit the effect of noise is essential to obtain adequate performance of the radar system and the ability to detect target echoes depends not only on the signal power, but also on the competing noise and clutter power present in the receiver.

The sources of this noise/clutter lie both within the radar and in the outside environment as illustrated in Figure 4.4.

Figure 4.4  Noise sources.

### Thermal noise

Historically, the sensitivity (or MDS) of radar systems has been limited by the thermal noise of the receiver, expressed by the noise figure.  However, processing techniques that became common during the 1990’s can remove the white noise and improve the radar system sensitivity (or MDS).

### Surface and volume clutter

A radar set that is sensitive enough to detect small targets, like buoys and small boats, will also be sensitive enough to detect returns (clutter) from land, sea waves, raindrops, snowflakes and hail.  Clutter is therefore often the limiting factor for detection of targets of interest.

#### Sea Clutter

Sea return is caused by the reflection of the radar signal from water surface irregularities and sea return is therefore composed of a very large number of echoes, originated by the random wave reflection from scan to scan.

Sea and wind relations vary depending on the geographical location.  Ocean areas differs from other waters, and the same seems to be the case for the definition of sea states, which have been seen to be defined by the Beaufort scale (wind), the GIT (Georgia Institute of Technology) model (used by the CARPET radar performance evaluation program) or the Douglas scale.  The GIT model has been used throughout this document.

#### Sea breezes

The large temperature variations between day and night in desert areas, and the associated pressure differences between land and sea, tend to cause strong sea breezes during summertime.  This, combined with layered distribution of air, generates eddies over the sea, often forming distinctive sea clutter patterns.

#### Land clutter

Clutter caused by land will not normally affect the VTS radar service.

### Precipitation

Precipitation in the form of rain, snow, hail and other hydro-meteorological particles affects radar range and tends to give unwanted returns or clutter.

#### Rain clutter and penetration

The radar signal’s ability to penetrate the atmosphere depends on the amount of precipitation and the amounts of precipitation (or rain) clutter returned are dependent on the:

- resolution cell size – influence is reduced with decreasing antenna beam width and  with decreased transmitter pulse width;

- antenna polarization, where circular polarization is preferred over vertical or horizontal polarization;

- frequency, where lower frequencies penetrate precipitation better than higher frequencies and give less clutter return.  However, lower frequencies are, in most cases, associated with severe drawbacks in respect to resolution and cost.  Therefore, the use of higher frequencies and circular polarized antennas in combination with processing techniques tend to be preferred for VTS.  An exception to this would be for long-range installations in areas with extreme tropical rainfall.

#### Hail and snow

Hail and snow produce effects similar to that of rain clutter.  Dense snow has a greater effect than that of light flurries which, owing to its small reflecting surface, has minimal effect.

### Man-made noise

Man-made noise consists primarily of signals from other nearby radars (interference).  However, applying noise reduction techniques allows for several radars to operate on the same frequency in close proximity without interference.

### Multiple reflections (ghost images)

Multiple reflections can result in several types of ghost images.  Sometimes these have the appearance of normal targets.  However, in most cases, they are intermittent and poorly defined.  Ghost images maintain a fixed relationship with respect to the true image and have more of an arc-like appearance than the true image, as well as a tendency to smear.  This is especially the case when the radar is located in the vicinity of large structures.

One type of ‘ghost’ is caused by multiple reflections from objects that are close together.  For example, two vessels that are passing within a few hundred metres of each other may, because of reflections of the electrical impulse between the vessels, produce a smaller third, and fourth image at a longer range.  These images will usually appear only momentarily beyond the vessel.

Another type of ‘ghost’ image is caused by secondary reflection from bridges, buildings and other structures.  Such reflections are especially common when the antenna is mounted close to such structures.

Careful notice should be taken of any false image encountered to determine the probable reason for its creation and to assist in future recognition.  For safe operation, if there is ever any question as to whether a particular image is true or a ‘ghost’, always assume it to be a true image corresponding to an actual object.

## Ice

The echoes obtained from ice depend largely on the form and shape of the ice encountered.

The typical characteristics of the various types of ice floes are as follows:

- Smooth flat ice – Most of the radar energy is reflected at its angle of incidence, providing little or no return signal.  It is sometimes an advantage to set up the radar controls to obtain sea clutter right up to the edge of the ice.  If there is sufficient wind to disturb the surface of the water, patches of water in a smooth ice field are often revealed by sea clutter return.

- Pack ice – Strong multiple echoes are usually obtained from this condition producing a pattern on the display not unlike excessive sea clutter.  The ice left in the wake of a vessel passing through an ice field can be distinguished clearly.

- Ice walls – These objects will give strong echoes depending on the angle that they are to the sea surface to scatter the reflected energy.

- Icebergs – The sharp angles of the faces of icebergs usually give a poor return signal.  Their smooth surface will often scatter the reflected signal at acute angles.

- Growlers – The detection of growlers by radar is uncertain due to their small surface area above water.  Detection of icebergs and growlers may require sophisticated techniques, similar to those for detection of small targets performing illegal activities.

## Target discrimination and position accuracy

The ability to discriminate targets depends on the radar range and azimuth resolution as shown in Figure 4.5.

### Range resolution

For traditional pulse radars, the range resolution depends on transmitter Pulse Width (PW), rise time, fall time, allowance for jitter and stretch in the receiver.

In a pulse compression or continuous wave radar, the effective pulse width can be made very short providing high resolution. The equivalent peak power may be very big. The drawback is added complexity and unwanted effects such as time side lobes.

Figure 4.5  Range and Azimuth resolution.

### Azimuth resolution

The azimuth characteristics of a radar return from a point target are almost entirely determined by the characteristics of the antenna.  The azimuth beam width is inversely proportional to the antenna length that, in turn, directly affects the radar's angular resolution (and range performance, as antenna gain increases with length).

The angular width of small target echoes (at the - 3 dB) is equal to the horizontal beam width of the antenna (defined at the - 3 dB points). However, this is a highly theoretical definition of azimuth resolution that is mainly used for comparison of equipment.

Most targets, especially at short or medium ranges, are detected when they are also below the - 3 dB points and, as a result, most echoes appears with an angular width in excess of the antenna beam width.

### Range accuracy

Determination of the range from an antenna to a target (the slant range) is normally dependent on precise, reliable clocks. The range accuracy is accordingly precise in most modern radar systems.

### Azimuth accuracy

The azimuth accuracy depends mainly on the turning gear, the resolution and accuracy of its associated bearing transmitter.

## Signal processing, extraction and tracking

The radar return signal down-converted to video level contains wanted signals from objects of interest, unwanted signals (clutter – for example sea clutter) and some basic system noise (for example, thermal noise) all mixed together.  The purpose of the signal processing is to enhance the signals from the objects of interest relative to the clutter and noise.

Techniques vary with the level of service implemented. Effective methods include:

- Sweep-to-sweep correlation: Normally, the pulse repetition rate of a radar is so high that a target is hit several times while the antenna beam turns past it.  In contrast, noise will pop up randomly.  Therefore, by correlation of radar return signals from consecutive transmissions, noise can be suppressed efficiently, relative to targets (and clutter).

- Signal differentiation: Extended areas of clutter (e.g. from rain showers) can be reduced by signal differentiation (in radars this is called Fast Time Constant (FTC)).

- Signal integration: The multiple hits on a target as described above can be added together.  This will enhance the returns from the objects of interest relative to noise and, to a lesser extent, relative to clutter.  Further, it will reduce the fluctuations of target echoes.

- Frequency Diversity: Frequency diversity radars employ two transmission frequencies.  The radar returns on the two frequencies from a target will normally be similar, while radar returns from clutter will be different.  If an antenna with frequency dependent squint such as a slotted waveguide antenna is used, there will also be a temporal difference between the returns on the two frequencies.  This means that the target will be present in both radar returns, while it is very likely that the clutter will be present in only one of the returns.  The total effect of using frequency diversity will be an enhancement of target radar returns, relative to clutter radar returns.

- Scan-to-scan correlation: By comparing radar returns from consecutive rotations of the antenna, fast disappearing echoes (e.g. sea waves) can be filtered away while persisting echoes (e.g. ships) can be retained for further processing.  Again, the overall effect is to enhance target radar returns, relative to clutter radar returns.

The output of the signal processing is the processed radar data in a polar grid, i.e. a circular area divided into radar cells, with each cell being characterized by a range value, a range depth, an azimuth value, an azimuth width and a radar return intensity value.  Plot extraction is the process of identifying groups of neighbouring radar cells containing radar return intensity values above a defined threshold value.  Such a group of radar cells is called a plot, and is intended to correspond to a physical target.

Plots are updated on each rotation of the radar.  Plots which persist from rotation to rotation, and which move within reasonable physical limits of speed and acceleration, are intended to correspond to physical targets.  A series of plots associated in this way is called a track.  Erratically appearing plots are rejected as representatives for physical objects and, therefore, as candidates for tracks.

Tracking is the process of associating plots from consecutive rotations of the radar to existing tracks and to initiate new, tentative tracks on plots not associated to existing tracks.

## Security implications

Using a VTS for security applications is outside the IALA mandate and this document only discusses the implications briefly.

A VTS authority asked to provide information for security purposes must realise that radar is the only VTS tool that can potentially detect small, non-cooperative targets at long distances and in all weather conditions.  Detection and identification of such objects is a difficult task likely to impose requirements to:

- Substantially increase sensitivity of the radar services, also dictating increased dynamic range, additional requirements to suppression of noise and clutter, increased resolution requirements and increased data rates; and

- Identify revised functional requirements for extraction, tracking, data handling and display presentation that are likely to be so different from normal VTS operations that they should be separated between the normal VTS operation and operation specific for security.

The requirements for security applications are therefore, in general, additional to the Advanced set of requirements outlined in this document.

# Performance verification methods

The dynamic characteristics and dependence on propagation conditions adds uncertainty to tests on installed radar systems and it can be difficult to reproduce tests as conditions may vary substantially from day to day.  In addition the complexity of radar systems call for a stepwise approach.  Therefore the combination of tests and calculations listed in Table 5-2 is recommended.

Table 5-2  Performance verification methods.

|  | Design / verification method |  |  |  |
| --- | --- | --- | --- | --- |
|  | During system design | Factory Acceptance Test | Site Acceptance Test | In Operation Test |
| Overall functionality | Analysis | Functional test | Functional test |  |
| Coverage | Analysis including weather influence and propagation |  | Functional test |  |
| Detection performance and dynamic requirements | Simulation including weather influence and propagation | Measurement of antenna and transmitter-receiver parameters, losses etc. | Functional evaluation, compared to weather condition | Analysis of statistical data from the system |
| Target separation | Analysis | Measurement of individual antenna and transceiver parameters | Check using known structures or controlled targets. |  |
| Tracking performance | Analysis or Simulation |  | Functional test Analysis of statistical data from the system | Analysis of statistical data from the system |
| Environmental and Safety issues | Analysis and type tests | Check of workmanship and measurement of parameters | Check of Installation |  |
| Availability | Analysis |  |  | Record and analysis of events |