SHIP DOMAINS

1.  The current draft definition proposed for the new VTS manual reads as follows:

Ship Domain A vessel may be allocated a moving safety zone, or “domain”, which is centred on the ship and from which all other shipping should remain clear. The size of the domain will vary depending upon the dimensions of the waterway, ship size, and characteristics, and the degree of risk caused by its cargo; the dimensions selected should be determined taking account of these

details and a relevant risk assessment.

- The SIGTTO document “LNG Operations in Port Areas” has, at paragraph 3.5.3, a heading of “Moving” Safety Zone" which opens with the sentence “It is sound practice to establish a cordon sanitaire or exclusion zone around a transiting gas tanker.

- From publications such as the Royal Institute of Navigation’s publication “The Journal of Navigation” it is also clear that the term Ship Domain is also used with a more theoretical connotation and in expressing theories in research projects.

.

- It is suggested that the current definition proposed for the VTS Manual when combined with the SIGTTO definition, effectively makes the terms Ship Domain, Safety Zone and Exclusion Zone synonymous.  It is considered that there is considerable benefit, both for clarity and for the development of theory and simulation and their translation into operational procedures, that these terms be differentiated and more precisely defined.  The definitions below have been developed in consultation with a UK Maritime training college, a Marine Risk Assessment consultant and a VTS peripatetic trainer.   It is believed that the revised definitions for a Ship Domain and a Safety Zone

provide a link between the dynamic variations of a Ship Domain (which a VTS Operator should still be invited to consider as well as being used as a research definition) and a Safety Zone which is more part of an operational procedure introduced to implement these variations into a rule or guidance.  The Exclusion Zone is different in that it is seen as a zone based on a fixed geographical point rather than a moving ship and can be used for such purposes as excluding traffic from environmentally sensitive areas or the scene of an incident.  The proposed revised definitions are as follows

Ship Domain.  An operational zone around, above or below a vessel within which an incursion by another fixed or moving object, or another domain, may trigger reactions or processes.  The size of a domain may vary for the same vessel dependent on a number of circumstances such as: the dimensions of the waterway; traffic density; ship size; ship characteristics; ship speed; and aspect of encounter.  A Ship Domain is widely used in traffic simulation models, encounter criteria, traffic lane design criteria, VTS planning, risk assessment, collision avoidance, and for other applications such as establishing operational procedures and the dimensions of a Safety Zone.

Safety Zone.  An area around a vessel within which all other vessels should remain clear unless authorised.  The size of the Safety Zone may vary depending upon such factors as: the dimensions of the waterway; ship size; ship characteristics; and the degree of risk caused by its cargo.  The dimensions selected should be determined taking into account these details and a relevant risk assessment.  This safety zone may be moving or stationary.

Exclusion Zone.  A geographical area, within which all other vessels should remain clear unless authorised.  The size and shape of the area may vary depending on the risks involved.

5.  The VTS Committee is invited to consider these revised definitions and, if agreed for inclusion in the VTS Manual, to consider whether they should be circulated more widely.

Barry Goldman

VTS Manager

Port of London Authority

for

International Harbour Masters’ Association