SECRETARIAT – FIX UP NUMBERING THRO DOC

Annex 3

Performance requirements for Automatic Identification System (AIS) in VTS

Table of Contents

1.	Introduction	2

1.1	Description of AIS as a VTS sensor	2

2.	References	3

3.	Definitions	4

4.	Functional requirements	5

1.2	Application	5

1.3	Objectives of AIS	5

1.4	Equipment	6

5.	Operational requirements	7

1.5	General requirements	7

1.6	Use of virtual MMSI for VTS communication	8

1.7	Short safety-related messages	8

1.8	Use of binary messages for data exchange	8

1.9	Use of the VTS Targets message	8

1.10	Assigned Mode	9

1.11	Graphical presentation	9

1.11.1	Symbol usage	9

1.11.2	Interaction with radar tracks	9

1.12	Data Validity	10

6.	Design and installation	10

1.13	Transmission Layer	10

1.14	Coverage aspects	10

7.	Interfacing	10

8.	Documentation	11

# Introduction

AIS is intended to enhance safety of life at sea, the safety and efficiency of navigation, and the protection of the marine environment.  In addition, AIS may contribute to maritime security.  SOLAS Regulation V/19 requires that AIS exchange data ship-to-ship and with shore-based facilities.  Therefore, the purpose of AIS is to help identify vessels; assist in target tracking; simplify information exchange (i. e. reduce ship reporting using radiotelephony); and provide additional information to assist situational awareness.  In general, data received via AIS will improve the quality of the information available to the VTSO or OOW.  AIS is a useful source of supplementary information to that derived from other navigational systems and sensors, including radar.

## Description of AIS as a VTS sensor

Automatic Identification System (AIS) is a system that makes it possible to monitor and track ships from suitably equipped ships, and  shore  stations.  AIS transmissions consist of bursts of digital data ‘packets’ from individual stations, according to a pre-determined time sequence.  AIS data consists of shipboard information such as position, time, course over ground (COG), speed over ground (SOG), heading, etc..  AIS uses a broadcast and interrogation technology that operates ship-to-ship and ship-to-shore and includes limited communication capabilities.

Shore stations receive the same information from AIS equipped ships within VHF range. .

The International Maritime Organization (IMO) has established carriage requirements for merchant ships.  The International Telecommunication Union (ITU) has defined the technical characteristics and ratified the global frequencies.  In addition, the International Electrotechnical Commission (IEC) has developed methods for testing AIS for global interoperability.

AIS  makes navigation safer by enhancing situational awareness and increases the possibility of detecting other ships, even if they are behind a bend in a channel or river or behind an island in an archipelago.  AIS also solves the problem inherent with  radars, by detecting smaller craft, fitted with AIS, in sea and rain clutter .

# References

| [1] | IMO MSC. 74(69), Annex 3 | IMO Recommendation on Performance Standards for a Universal Shipborne Automatic Identification System (AIS) |
| --- | --- | --- |
| [2] | SOLAS Convention | Chapter V Safety of Navigation, Regulation 19 |
| [3] | ITU Radio Regulations | Appendix S18, Table of Transmitting Frequencies in the VHF Maritime Mobile Band |
| [4] | ITU-R M.1371-1 | ITU Recommendation on the Technical Characteristics for a Universal Shipborne Automatic Identification System (AIS) Using Time Division Multiple Access in the Maritime Mobile Band |
| [5] | IEC Standard 61993 Part 2 | Universal Shipborne Automatic Identification System (AIS) Operational and Performance Requirements, Methods of Testing and required Test Results. |
| [6] | IALA Technical Clarification | IALA Technical Clarifications of Recommendation ITU-R M.1371-1 |
| [7] | IALA Recommendation A-124 | IALA Recommendation on AIS Shore Stations and Networking Aspects Related to the AIS Service |
| [8] | Resolution A.694(17) | General Requirements for Shipborne Radio Equipment forming Part of the Global Maritime Distress and Safety System (GMDSS) and for Electronic Navigational Aids |
| [9] | Resolution A.953(23) | World-Wide Radionavigation System |
| [10] | Resolution A.688(17) | Code on Alarms and Indicators |
| [11] | Resolution MSC.39(63) | Adoption of amendments to the Code on Alarms and Indicators |
| [12] | IALA Recommendation V-125 | IALA Recommendation on the Integration and Display of AIS and other information at a VTS Centre. |

# Definitions

AIS		Automatic Identification System

ASM 		AIS Service Management

BSC		Base Station Controller

FATDMA	Fixed Access Time Division Multiple Access

LSS		Logical Shore Station

OOW		Officer of the Watch

PSS		Physical Shore Station

SOTDMA	Self Organising Time Division Multiple Access

VTSO		Vessel Traffic Services Operator

# Functional requirements

For VTS purposes, an AIS service provides AIS information from one or several base stations to users. In addition to vessel data, an AIS service provides status on AIS equipment and management functions for the control of the AIS network.  The AIS Service may consist of one or more PSS or AIS Networks, or a combination of both.

## Application

These performance requirements are for the use of AIS in a Vessel Traffic Service (VTS).  AIS should improve the safety of navigation by assisting in the efficient navigation of ships, protection of the environment, and operation of VTS, by satisfying the following functional requirements:

- in a ship-to-ship mode for collision avoidance;

- as a means for littoral States to obtain information about a ship and its cargo; and

- as a VTS tool, i.e. ship-to-shore (traffic management).

AIS should provide ships and competent authorities, information from the ship, automatically and with the required accuracy and frequency, to facilitate accurate tracking.

Mandating AIS carriage and establishing a service to receive, process and distribute the AIS signals received from vessels enhances safety and security and improves the ability to manage traffic.

Some shore facilities may need to act on the information, others may need to monitor AIS and maintain an information database.  For these reasons, a nationwide or regional network may be set up.

The service should also be capable of information exchange and distribution among several users ashore and afloat.  Government agencies, allied services and commercial maritime interests may have justifiable needs for AIS data.

In order for VTS to take full advantage of AIS, access to the capabilities of an AIS Base Station is required.  This access should preferably be through an AIS service.  With this access, the VTS may change the reporting rate or AIS channel, send short safety-related messages, or perform other functions as necessary

## Objectives of AIS

AIS shall:

- provide information automatically to appropriately equipped shore stations, other ships and aircraft, including the ship’s identity, type, position, course, speed, navigational status and other safety-related information;

- receive automatically such information from similarly fitted ships;

- monitor and track ships;

- exchange data with shore based facilities;

- assist in ensuring the highest possible level of safety and efficiency for vessel traffic in the designated area.

The requirements should not be applied to cases where international agreements, rules or standards provide for the protection of navigational information.  AIS should be operated taking into account the guidelines developed by the International Maritime Organization.

## Equipment

AIS equipment is described in detail in IALA Recommendation A-124 on Automatic Identification System (AIS) Shore Station and networking aspects relating to the AIS Service.

# Operational requirements

## General requirements

The AIS service should provide timely, relevant and accurate information to users to support the decision-making processes of a VTS.  The AIS service may also support port operations by providing information to appropriate shore facilities.  It provides automatic vessel position reports and movement information as it is received at remote sites throughout the service area. In support of incident response, the AIS service, in conjunction with the port authority, can provide information about traffic and the corresponding situational information.  Utilizing the AIS Service, the VTS will monitor safety and security zone boundaries that are established in connection with an incident. The AIS service also provides information to allied services to support their tasks.

a. AIS information needs of the VTS.  Basic information needs for vessel tracking and port security related missions are:

- Up-to-date knowledge regarding the route to be transited.

- Timely, relevant, and accurate information about vessels within the area that might affect safety, security, or the decision making of the VTSO.

- Timely information about emergency and environmental conditions that might affect safety or the decision making of the VTSO.

- Where required, the transmission of relevant information to the mariner in a manner that does not distract from the task at hand, particularly in narrow, confined channels where there is heavy traffic.

AIS, as well as existing aids to navigation and tools, pilotage systems, navigation management systems, and regulations provide information to the mariner but these systems require integrity monitoring to ensure the information they impart is accurate.

b. Integrated Operations.  Where applicable, AIS should support regional Vessel Traffic Services between adjoining VTS Centres.

c. Incident Analysis Support.  The AIS service should support incident response and analysis.

- Immediately after notification of an incident, the AIS service should alert vessels in or planning to enter the area of concern with particulars of the incident.

- The AIS service may provide for review of supporting information and events that occurred before, during, and after an incident.  Otherwise, the VTS should have the ability to support recording and replay of AIS data.

## Use of virtual MMSI for VTS communication

Addressing of AIS units is accomplished via the Maritime Mobile Service Identity (MMSI).  Every base station will have a unique MMSI number. However, when a VTS is operating several AIS Base Stations, it can present a single address, known as a virtual MMSI number.  This will enable the ships to send and receive AIS messages in the VTS area using only one MMSI regardless of the number of base stations in use.

This functionality has to be supported through the AIS Network.  However, shipboard AIS units can not currently respond to messages from virtual MMSIs.

## Short safety-related messages

Short safety-related messages are  free format text messages.  They can be addressed either to a specified destination (MMSI) or broadcast to all ships in the area.  Their content should be relevant to the safety of navigation, e.g. an iceberg sighted or a buoy not on station.  Such messages can contain a maximum of 158-162 characters.  Although unregulated, these messages should be kept as short as possible.

Cautionary note: The VTSO should not assume that all short safety-related messages have been read onboard.

## Use of binary messages for data exchange

IMO has selected seven binary messages, as shown below, to be used as a trial set.  The intention is to use this set for a period of 4 years (May 2004-May 2008) with no change.

The messages are:

- meteorological and hydrological data

- dangerous cargo indication

- fairway closed

- tidal window

- extended ship static and voyage related data.

- number of persons on board

- pseudo-AIS targets.

For further details, see IMO SN/Circ.236, dated 28 May 2004, Guidance on the Application of AIS Binary Messages.

## Use of the VTS Targets message

One of the IMO approved binary messages is for broadcast pseudo AIS targets.  This contains information on non-AIS targets derived by means other than AIS. Typically these will be targets that are tracked by a VTS-system based on radars. Sometimes the VTS Targets message is also referred to as VTS Footprint.

The AIS message for VTS Targets can be filled with information for a maximum of 7 targets..  At present, IMO restricts this to a maximum of 4.  Each target comprises:

- Target id (MMSI, IMO number or Callsign)

- Latitude

- Longitude

- Course (COG)

- Speed (SOG)

- UTC time stamp

A VTS Targets message can either be sent as an addressed message to one target or as a broadcast message to all targets that are inside the VHF coverage area of one or more shore stations (i.e., PSS).

For additional information, refer to IMO SN Circ/236 – Guidance on the application of AIS Binary messages.

## Assigned Mode

VTS may use the AIS Service capability to change the reporting mode (from autonomous to assigned mode, for example) of selected shipboard AIS units.  This will enable the ship station to operate according to a specific transmission schedule, as determined by a competent authority.

## Graphical presentation

In the VTS Centre, AIS-data is usually viewed on an electronic chart, either separately or combined with the radar data.

### Symbol usage

The IALA Guidelines on AIS include a description of the recommended AIS target symbols, but these were originally intended for the onboard ECDIS/ECS systems. As the guidelines point out, the given symbols are not necessarily adequate in the VTS context. The main difference is that a VTS-operator may prefer a much wider range of information than is necessary onboard a ship. For example traffic management may necessitate the use of symbols which depict different types and sizes of vessels. Further it may be necessary to show which vessel have pilots embarked, and which do not.

### Interaction with radar tracks

A target that is tracked by radar and also carries an AIS transponder may be displayed with one symbol based on a fusion of the dynamic information received from the two sensor types. The user may have the option to display the input from each sensor with two different symbols. It should also be possible to identify which sensor(s) are used to derive the target position.

## Data Validity

The validity of AIS data received from ships is dependent on the proper installation of AIS, correctly interfaced and functioning ship’s equipment, and correct manual input of static and voyage-related data.

Until further regulations dictate stricter data accuracy requirements in the AIS mobile units, caution has to be taken when using AIS data for processing.

VTS may contribute AIS service data to government agencies, allied services and commercial maritime interests.  This determination will be made by the relevant Competent Authority.

# Design and installation

## Transmission Layer

AIS is an unencrypted broadcast system and as such, its data is accessible to anyone with a suitable receiver.  National regulations may regulate the use of AIS frequencies. AIS may be susceptible to interference from adjacent channels.  Due consideration should be given to frequency allocations adjacent to AIS channels to avoid possible service disruption.

## Coverage aspects

In general, AIS design coverage ranges should approximate VHF voice communication ranges.  However, actual vessel traffic density or geographic considerations (i.e., mountains or other VHF occlusions) may determine the need for additional base stations

When estimating the size of the operational coverage (operational cell) for shore facilities, an important consideration is the traffic load – number of mobile AIS stations within the area.

For example, calculations in one port have indicated that an AIS Base Station could accommodate less than 300 active AIS units.

For further information, please refer to IALA GUIDELINES ON THE UNIVERSAL AUTOMATIC IDENTIFICATION SYSTEM (AIS) Volume 1, Part II – Technical Issues, Edition 1.1

# Interfacing

The VTS will select features of the AIS network it requires via the AIS Service .  These features are the Basic AIS Services defined in IALA Recommendation A-124.

# Documentation

Documentation should be provided in accordance with the IALA VTS Manual.