| From: VTS26 | VTS26-output-10 |
| --- | --- |
| To: e-NAV3 | September 2007 |

Liaison Note to e-Navigation Committee

- VHF Spectrum Management

- Introduction

Clear and effective VHF communications remains a key factor in VTS operations.  The use of VTS as a key mitigation measure in marine operations is becoming increasingly recognised.  This has resulted in more VTS Centres with their associated VTS Sectors being established which in turn is placing a greater demand on Marine VHF channels available for VTS communications.  The limited number of channels in the marine VHF band is widely recognised.  VTS Authorities are having to resort to the use of VHF channels that are not dedicated to ship/shore use.  As a result, there is evidence of increasing interference on VTS VHF channels either from ship/ship transmissions or from other VTS Centres or port related services.  This has the potential of creating dangerous situations.

- Discussion

Whilst the problem of interference in the Marine VHF band has been recognised for some years there has been a tendency to live with the problem notwithstanding the potential impact on safety.  There are a number of issues that now make it timely to review the management of the Marine VHF band.  These include:

- Greater expectation of VTS in its contribution to risk control

- Greater reliance by ships on VTS communications

- Competing demands from new technologies such as AIS.

- Potential future demands for data transfer to meet the e-Navigation initiatives

- Possibilities created by new technologies for more efficient use of the available spectrum

- Challenges to the use of the Marine VHF spectrum from non-marine related use.  (It is understood that this part of the spectrum is not always reserved in the same way in which corresponding aircraft control channels are allocated).

- The need to protect AIS frequencies from interference

There are two clear measures that could be introduced to alleviate these issues: firstly the creation of more channels within the Marine VHF band, and secondly better management of the existing spectrum.

Increased VHF Channels.  It is understood that new digital technologies are becoming available that will allow the reduction in channel spacing for voice circuits whilst data channels may require similar or greater bandwidth.  Clearly it should be possible to make more efficient use of the spectrum.  However, this would require a fundamental review of the entire Marine VHF Channel allocations beyond that proposed in Recommendation ITU R M.1084-4 ; it would be a long term project involving international agreements and long lead times to implementation.  Whilst the challenges are significant, the potential long term benefits are enormous; there is even the potential for identifying a greater number of channels within a smaller part of the existing spectrum which could benefit both marine and other potential users.

Spectrum Management.  Whilst most countries have authorities in place to administer the spectrum, there is little evidence of effective cross-border management of spectrum issues.  Better liaison between national authorities would offer the potential for a reduction in interference even in the short term.

- Recommendation

The e-Nav Committee is invited:

- to note the impact of these issues on Maritime safety and,

- to consider appropriate actions that may be taken to increase awareness within the responsible administrative organisations to resolve these problems

- to consider how national authorities could be encouraged to manage the spectrum more effectively and to improve cross-border liaison and coordination