A REVIEW OF THE NAVIGATIONAL ASSISTANCE SERVICE

A PROPOSAL BY IHMA

Introduction

- The draft Chapter 5 to the new IALA VTS Manual introduces the concept of a Local Port Services and also attempts to clarify the difference in function between the types of service involving the management and organisation of the port or VTS Area  (i.e. an Information Service (INS) and a Traffic Organisation Service (TOS) ) and the type of service involving the safe navigation within the VTS Area (i.e. a Navigational Assistance service(NAS) ).  The present draft also differentiates between NAS that is provided because the VTS Operator notices that intervention is appropriate or “Observed” and NAS that is provided “On Request”.  The former is likely to be provided at short notice whilst, for the latter, there is more likely to be time to plan and agree the VTS support that is required.

- The provision of a NAS “On Request” is a service that is likely to require high levels of experience and marine understanding, whilst the provision of NAS when “Observed” is the sort of intervention that should be expected from any reasonably capable and competent VTS.  There is a concern that lumping these two aspects of the NAS service together deters VTS Authorities from advertising the provision of an NAS lest there be an expectation of a higher level of service provision than is intended.  In addition, the promulgation of a NAS may raise liability issues.

Proposal

- The description above indicates that there are two significantly different types of service being offered under the same listing of a Navigational Assistance Service.  It is proposed that an additional service type be introduced entitled a “Navigational Safety Service” (NSS).  This would broadly equate to the Navigational Assistance Service “Observed”.  It represents the sort of service that VTS operators should more routinely be expected to deliver whilst authorisations for operators may take into account whether the “Method” by which this might provided should be limited to the provision of Information, or whether Advice or Instruction may be authorised.  The term “Navigational Safety Service” is also considered to be more descriptive than a “Navigational Assistance Service – Observed” of the service that is actually being delivered.

- A revised draft Chapter 5 is attached that defines this change in more detail.  This draft revises the table in paragraph 0502 and is marked up in track change on the existing draft to indicate where amendments would become necessary.

Recommendation

- The VTS Committee is invited to consider whether there is a case for introducing a Navigational Safety Service (NSS) as a new and additional service type.  If agreed, consideration would have to be given to how it might be introduced since an amendment to A857(20) would probably also be required.

# CHAPTER 5:   TYPES AND FUNCTIONS OF TRAFFIC SERVICES

## 0501    Introduction

In many waterways vessels can operate independently under any conditions of traffic and weather that are likely to be encountered.  In these circumstances there is no requirement for a VTS and vessels operate unaided.  However, there are many waterways where vessels rely on interaction with shore authorities to conduct their movements safely and efficiently.

The purpose of this chapter is to identify the contribution of traffic services and to set out the options available to a Competent Authority and the traffic services that may be provided.

## 0502	Services

The diagram below gives a pictorial overview of service types and functions.

Information Service

Providing

Port:

INFORMATION

Safe and efficient port operations

Traffic Organisation Service

Navigational Safety

Service

Safe use of the waterway

Efficiency of traffic movement

Protection of the marine and adjacent environment

Informing or managing  marine traffic in the VTS Area

(broadcast, addressed, or on request)

Local Port Service

Providing Traffic:

INFORMATION

Providing

Navigational:

INFORMATION

ADVICE

INSTRUCTION

When observed to be necessary

Providing local information, eg:

Environmental data and Port Information

SERVICE

TYPES

FUNCTIONS

Vessel Traffic Services

Local Port Services

Navigational Assistance Service

Managing  Navigational Safety of marine traffic in the VTS Area

Providing

Navigational:

INFORMATION

ADVICE

INSTRUCTION

On Request or in special situations to an agreed plan

Providing Traffic:

INFORMATION

ADVICE

INSTRUCTION

METHODS

Fig xx  Types and Functions of Traffic and Port Services

OBJECTIVES

## 0503	Prerequisites

The prerequisites for Vessel Traffic Services (VTS) and Local Port Services (LPS) are:

Vessel Traffic Services

- Authorised by the Competent Authority;

- Staffed by V-103 certificated personnel;

- Equipped as appropriate to provide INS/NAS/TOS;

- Interacts with traffic;

- Responds to traffic situations.

Local Port Services

- Does not require to be authorised by the Competent Authority;

- Staffed and trained appropriate to task;

- Equipped appropriate to task.

The Competent Authority is the Authority made responsible, in whole or in part, by the Government for the safety, including environmental safety, and efficiency of vessel traffic and the protection of the environment.                 IMO Resolution A.857(20)

## 0504  	Local Port Services

Local Port Services (LPS) is an organisation ashore that provides information to the bridge team.  LPS is designed to improve port safety and co-ordination of port services by dissemination of port information with vessels and berth or terminal operators. It is mainly concerned with the management of the port, by the supply of information on berth and port conditions.  LPS can also provide liaison between vessels and allied services.

Examples of LPS may include:

- Shipping schedules;

- Meteorological and Hydrological data;

- Berthing information;

- Availability of port services.

LPS is not an authorized VTS.  It is not required to have the ability and/or the resources to respond to developing traffic situations, nor is there a requirement for a traffic image.  In addition, the training requirement for personnel is less comprehensive and the operators are not normally certificated to the V-103 standard but should be trained to meet local requirements.  It should be noted that LPS are outside of the scope of this manual as they do not meet international standards, although they will invariably meet the standards of a lower level of capability sufficient to meet local needs.  In consequence LPS is not considered further in this manual.

## 0505  Vessel Traffic Services

An authorized VTS will be capable of offering one or more of the following service types:

- Information Service (INS).  An Information Service provides essential and timely information to assist the on-board decision-making process.  An Information Service does not participate in on-board decision-making;

- Traffic Organisation Service (TOS).  A Traffic Organisation Service is a service to provide for the safe and efficient movement of traffic and to identify and manage potentially dangerous traffic situations.  A Traffic Organisation Service provides essential and timely information to assist the on-board decision-making process and may advise, instruct or exercise the authority to direct movements;

- Navigational Safety Service (NSS).  A Navigational Safety Service may be provided in addition to an Information Service or Traffic Organisation Service.  It is a service to assist in the on-board navigational decision-making process and is provided when the VTS observes that it is necessary to provide navigational information, advice or instruction.

- Navigational Assistance Service (NAS).  A Navigational Assistance Service may be provided in addition to an Information Service, Traffic Organisation Service or Navigational Safety Service.  It is a service to assist in the on-board navigational decision-making process and is provided at the request of a vessel, or when deemed necessary by the VTS.

## FUNCTIONS

The functions of each service type are outlined below and identify the major activities that may be expected from each service type, together with an indication of the role that they can be expected to undertake:

## 0506	Information Service (INS)

This service type involves maintaining a traffic image and allows interaction with traffic and response to developing traffic situations.  An INS provides essential and timely marine information to assist the on-board decision-making process, which may include:

- the position, identity, intention and destination of vessels;

- amendments and changes in promulgated information concerning the VTS area such as boundaries, procedures, radio frequencies, reporting points;

- the mandatory reporting of movements;

- meteorological and hydrological conditions, notices to mariners, status of aids to navigation; limited manoeuvrability that may impose restrictions on the navigation of other vessels, or any other potential hindrances.

## 0507   Traffic Organisation Service (TOS)

A Traffic Organisation Service provides essential and timely information to assist the on-board decision-making process.  It may involve the provision of advice and instructions.  Traffic Organisation concerns the forward planning of movements to maintain vessel safety and to achieve efficiency.  This service may involve:

- the position, identity, intention and destination of vessels;

- amendments and changes in promulgated information concerning the VTS area such as boundaries, procedures, radio frequencies, reporting points;

- the mandatory reporting of movements;

- information such as meteorological and hydrological conditions, notices to mariners, status of aids to navigation;

- specific information such as traffic congestion and special vessels with limited manoeuvrability which may impose restrictions on the navigation of other vessels or any other potential hindrances;

- the allocation of waterspace;

- establishing and operating a system of traffic clearances - all or certain classes of vessels may be required to participate in this service and shall not proceed without clearance;

- establishing routes to be followed and speed limits to be observed and such other measures as may be considered necessary and appropriate by the VTS;

- specific information, such as traffic congestion and advice about vessels with

- co-ordinating VTS sailing plans.  These are an instrument of traffic organisation and the ability of a VTS to contribute to vessel safety.  It is a major source of information to the VTS.  The category of vessels for which a VTS sailing plan is compulsory and the details required should be clearly identified.  A VTS sailing plan normally includes the intended route, the estimated time of arrival in the VTS area or the departure from a berth or an anchorage in the VTS area.  It may also take into account the general flow of traffic, efficiency and co-ordination with allied services.

## 0508      Navigational Safety Service (NSS)

A Navigational Safety Service may be provided in addition to an Information Service or Traffic Organisation Service.  It is a service that may be provided when a potentially dangerous navigational situation is observed or is developing, and intervention by VTS is deemed necessary.   It provides essential and timely navigational information to assist in the on-board navigational decision-making process, and may also involve navigational advice and/or instruction.

Clear operational procedures should be in place for the provision of a Navigational Safety Service.  A Navigational Safety Service is unlikely to be pre-planned and will generally involve the need for early intervention to prevent a navigational incident; it may involve the provision of information, such as:

- course and speed made good by a vessel;

- position relative to fairway axis, navigational features and/or way-points;

- proximity to navigational hazards;

- positions, identities, intentions and any restrictions of surrounding traffic.

A Navigational Safety Service may require firmer intervention through the provision of advice and/or instruction.

## 0509      Navigational Assistance Service (NAS)

A Navigational Assistance Service may be provided in addition to an Information Service or Traffic Organisation Service.  It is a service that provides essential and timely navigational information to assist in the on-board navigational decision-making process.  It may also involve navigational advice and/or instruction.

Navigational Assistance may be provided at the request of a vessel or when a navigational situation is observed and intervention by VTS is deemed necessary.   Such assistance requires positive identification and continuous communication throughout the process.  It is important that the provision of Navigational Assistance is agreed between the vessel and the VTS providing the service.  Acceptance by the vessel of the Navigational Assistance Service should be established, and the beginning and the end of Navigational Assistance should be clearly stated.

Clear operational procedures should be in place for the provision of a Navigational Assistance Service.  Navigational Assistance may involve the provision of information as for a Navigational Safety Service in paragraph 0508 above

Navigational Assistance may also involve the additional provision of advice and/or instruction, and may include or require:

- an assessment of the suitability of the vessel to respond to the advice provided including an assessment of linguistic ability;

- a review of vessel characteristics including manoeuvrability relative to the area in which the service is provided and any defects or deficiencies;

- an assessment of the environmental conditions;

- an assessment of the implications of the cargo carried;

- a review of the proposed sailing plan;

- recommendations on measures to maintain the sailing plan noting that any advice on courses and speeds should be result orientated;

- the use of message markers;

- the use of a dedicated frequency;

- restriction of other traffic movement.

When the VTS is authorised to issue instructions to vessels, these instructions should be result-oriented only, leaving the details of the execution to the master or pilot ….

IMO Resolution A.857(20)

## 0510	  Promulgation of Information and Categorisation of Services

The services offered to the mariner by a VTS should be promulgated to vessels in internationally recognised marine publications.  This should include details of the VTS, its capabilities, rules, regulations, requirements and procedures. The information promulgated should be verified, or up-dated, at least at annual intervals. Further information can be found in Chapter 13.

## 0511	Certification of VTS

The responsibility for determining VTS service type required to mitigate identified hazards lies with the Competent /VTS Authority who are accountable for the standards they set.  This includes the resources, staffing levels, training, qualifications and equipment capability.

The authorisation of VTS personnel to provide the types of service above should also be clearly identified.

Appropriate and adequate operational and administrative procedures should be in place.  The Competent Authority should ensure that the operational and administration procedures used by a VTS Authority are appropriate for the advertised services through certification.  Certification can be achieved by an appropriate auditing and accreditation process.  IALA Guideline 1055 provides advice on preparing for audit of the delivery of VTS services.

It is a national responsibility to determine whether the Competent Authority has taken action to ensure that the VTS Authority/Centre has appropriate procedures and practices in place to meet the declared standards.