VTS 27-output-03

RESULT- ORIENTED VTS INSTRUCTIONS

In developing task 8 “Review/provide guidance on navigational assistance service”, the VTS Committee identified worrying indications that differing guidance was being given to VTS Operators in the interpretation and implementation of the part of the IMO Resolution A 857 (20) relating to “result-oriented instructions”.  Recordings and experiences have been shared and it is the view of the VTS Committee that this specific issue requires clarification.  It is considered that this will ultimately require an update to A 857 (20) but, in the interim, it is considered that action is required to incorporate such guidance within the draft being prepared as part of task 8.  It is considered that IMO could regard the promulgation of such a guideline contentious and it is recommended that the IALA Council seek ratification of the intention to issue such guidance from MSC in advance.  A proposed letter to MSC is attached below:

DRAFT NOTE FOR COUNCIL TO SEND TO IMO MSC

Feedback from VTS practitioners within the IALA VTS Committee have identified concerns that the guidance contained within IMO Resolution A857 (20) relating to the term “result-oriented” is open to misunderstanding.  In the worst cases, examples have been identified where VTS operators have felt inhibited by interpretation; on occasions, advice to avoid dangerous situations has been less clear and less timely than might have been desired.  The specific section of the resolution reads:

“2.3.4 When the VTS is authorized to issue instructions to vessels, these instructions

should be result-oriented only, leaving the details of execution, such as course to

be steered or engine manoeuvres to be executed, to the master or pilot on board

the vessel. Care should be taken that VTS operations do not encroach upon the

master's responsibility for safe navigation, or disturb the traditional relationship

between master and pilot.

The particular concern has been a certain reluctance of some VTS Operators to advise on a recommended heading to avoid danger.  At best this is overcome by a recommendation of a “course to make good”, but all too frequently alternatives are used such as to “make ground” in a particular direction, which is not only a convoluted means of achieving the desired action but has on occasions been open to misinterpretation and unexpected and dangerous reactions.  This is made worse by language difficulties.  This situation contributes to information to assist on-board decision making not being made available to vessels in a timely manner.

The IMO Resolution A. 857 (20) relates specifically to the danger of an “instruction” to steer a course as a specific action that might be considered to compromise the master’s position of responsibility.  The key part of this section in the IMO Resolution is the final sentence that highlights the pre-eminence of the master and the importance of preserving the master/pilot relationship.

IALA has been presented with examples where “advice” to steer a course has resulted in clear and unambiguous action by the vessel standing into danger and early appreciation by VTS operators that their advice has been understood.  In addition, the advent of AIS since the original promulgation of the Resolution A.857 (20) has allowed more immediate and effective appreciation by VTS Operators that their advice has been followed.  It is considered that issuing advice or a recommendation for a heading is both in the spirit and the word of this article, and IALA would wish to issue appropriate guidance in the short term to this effect.  This also facilitates achieving the intent of Resolution A.857 (20) in terms of ensuring that information to assist onboard decision making is timely.

In the longer term, IALA would suggest that an amendment of the text in IMO Resolution A. 857 (20) for greater clarification would be appropriate and will propose a textual amendment.  In the near term, IALA believes that clarification is required and seeks the endorsement of IMO to issue a formal IALA Guideline to ensure that this important issue is resolved indicating that advice or a recommendation for a particular heading or course to steer is an acceptable interpretation of the IMO Resolution A. 857 (20).