# VTS27 / 20 / 01

# Formerly VTS26-WP7

# LIAISON DOCUMENT TO THE e-NAVIGATION COMMITTEE

# Defining e-Navigation requirements for shore based users

1.	Introduction

Taking into account the decisions made at IMO/NAV53 (doc. NAV53, WP.4 and WP.8) to focus first on defining the potential e-Navigation users and the user requirements it was clear that first the users should be identified. The (draft) user requirements for mariners already has been identified by the IALA eNAV Committee (doc. eNAV3 06/03). In the latter document also a first step was made to identify user requirements for ashore. The Operational WG of VTS26 focussed on the users ashore, especially concentrating on the users in and around the environment of VTS and proactive defining users and requirements in a broader perspective: possible users in a future VTS /VTM.

Taken into account were

- The definition of e-Navigation

- The aims of the primary shore based parties

- The tasks of VTS/VTM developing

- The key functionalities

- The identification of the needed e-Navigation components ashore

2.	eNavigation

IMO/NAV53 provisionally finalized the definition for e-Navigation as a concept based on harmonization of marine navigation systems and supporting shore services driven by users needs:

“e-Navigation is the harmonized collection, integration, exchange, presentation and
  analysis  of maritime information onboard and ashore by electronic means to enhance
  berth to berth navigation and related services, for safety and security at sea and 
  protection of the marine environment”

In order to define the user requirements ashore the specific items concerning information should be considered:

- Collection

- Integration

- Exchange

- Presentation

- Analysis

3.	Identification of the users

At the intersessional meeting of the Operational and Strategic Working Group of the e-Navigation Committee (St Germain-en-Laye, July 2007) a first attempt was made to identify the shorebased users.

In order to categorize all potential users ashore the distinction was made based on the ways the information exchange (one of the primary tasks of a future VTS / VTM) will be handled:

- Ship to shore / Shore to ship	(ship with VTS/VTM or Coastguard-centre vv)

- Shore (VTS/VTM) to shore 1	(maritime/nautical information towards maritime
                                                organizations and stakeholders)

- Shore (VTS/VTM) to shore 2   	(public and transport/cargo information supporting the
                                                         logistic chain)

It should be noted that the information for both maritime/nautical and public/transport/cargo can be made available for parties inside and outside the defined area.

The table included below is not a comprehensive list of  the possible users devided over the above smentioned three categories.

| SHIPSIDE | SHORESIDE (1) | SHORESIDE (2) |
| --- | --- | --- |
| Mariners – Merchant vessels (incl. all types of passenger vessels) | VTM - Organizations | Ships owners & operators, Logistic stakeholders |
|  | VTS – Organizations (centres) - VTS operators - Management (strategic) | Media (news organizations) |
| Mariners - Coastguard vessels | Coastguard Organizations (or equivalents) | Integrated coastal management authorities |
|  | Search and Rescue organizations | Marine accident investigation organizations |
|  |  | Health & Medical Organizations |
| Law Enforcement vessels - police - customs - bordercontrol - immigration - fishery inspection | Law Enforcement Organizations - Police - Customs - Bordercontrol - Immigration - Fishery inspection | Insurance & Financial institutions |
|  | National administration (Flag State) | Governmental and other national policy organizations |
|  | Port authorities (operational) | Port authorities (strategic) |
|  | Security organizations | Ministries |
|  | Port State Control | Marine environment management |
| Mariners – Pilots | Pilot Organizations |  |
| Mariners – Military vessels | Military Organizations |  |
| Mariners – Fishing vessels |  | Fisheries management |
| Mariners – Pleasure craft |  |  |
| Mariners – Tourism (water buses) |  | Tourism agencies |
| Mariners - Ferries |  |  |
|  |  | Training organizations |
| Mariners - Dredging & AtoN operations | Counter Pollution | Fairway maintenance organi-zations (dredging, AtoN etc) |
|  |  | Equipment manufacturers & system maintenance organizations |
| Mariners – Ice patrol/breakers | Meteorological organizations |  |
| Offshore operations (Oil-Gas Rigs / Windmillparks) | Lighthouse & AtoN authorities |  |
| Dispatch activities (nautical services: tugs/tenders etc.) |  |  |
| Tourism (commercial, private) |  |  |
| Hydrographic Survey vessels | Hydrographic organizations |  |
| Oceanographic Research vessels |  | Oceanic Research Institutes |

4	Identification of  primary users and their requirments

4.1	Primary users ashore in e-Navigation		
The most important parties ashore interacting with ships are the Vessel Traffic Service/Management (VTS/M) organisations and their centres of operation; namely VTSs and Coastguard or other monitoring centres.

Currently, VTSs are provided world-wide.  There are IMO and IALA documents which offer guidance on design and operation of VTS systems.

Another function of Vessel Traffic Service/Management (VTS/M) organisations in the overall e-Navigation concept is to backup the integrity of a ship’s own data systems and the offering of advice and support when relevant.

All the foregoing shorebased VTS functions are capable of being executed and communicated by electronic means.  A standardised VTS may be defined under the e-Navigation concept, however it should be recognized that local and regional circumstances may differ.

4.2.	The aims of VTS/VTM as developing

The primary user requirements for parties ashore are to satisfy the key service/management aims and functionalities supporting these aims of VTS and/or VTM:

1)  safety of shipping and the environment

- to manage vessel traffic efficiently, to respond effectively to changing circumstances

- meeting the demands of all parties concerned for reliable and current information in
      order to optimise 
 			            - the efficient utilization of maritime water infrastructure.
 			            - the operation of or support to other tasks (i.e. security) 
 			            - the efficient and logistic and/or nautical processes

4.3	The tasks of VTS/VTM as developing

The key functionalities are also necessary to support the execution by an optimised information exchange of present and future tasks of VTS/VTM as developing. The next table is reflecting this.

| TASKS | CONNECTION | TYPE INFORMATION | DESCRIPTION / REMARKS |
| --- | --- | --- | --- |
| Information Exchange | Ship to Ship, ship to shore, | Information from the vessel | - Navigational information (e.g. position, speed, course, ETA etc.) - Cargo information (dangerous goods etc.) - Requests for assistance when appropriate |
|  | Shore to ship | Traffic Information | - Actual (reactive), - Future (proactive) |
|  |  | Infrastructure Information | - Fairway conditions, locks, bridges etc. - Technical systems (status of operation like GNSS, comms etc.) |
|  |  | Nautical Information | - Hydrographic information - Meteorological information - Tide information - Voyage planning (interaction) - Fairway availability, obstructions and events following traffic behaviour (warnings, advices and eventually mandatory instructions) |
|  | shore to shore, | - maritime/nautical information | - towards maritime organizations and stakeholders inside and outside defined area (shore1) |
|  |  | - public and transport/-cargo information supporting the logistic chain | - towards all other interested parties inside and outside defined area (shore 2) |
|  | Central data maintenance |  | - Central role VTS |
| Navigational Assistance | Shore to ship | - Related to the individual vessel on request - Additional to INS & TOS | - E.g. position information, course and speed made good, tide, en route warnings (see VTS Manual) |
| Traffic Organization Service | Ship to shore Shore to ship | Allocation of space, e.g. the overall Traffic Image for tactical/operational decision making onboard/ashore, safety related and efficient transport | - Traffic Monitoring - (Voyage & efficient fairway) Planning - Admission & Berth management |
| Support Enforcement and Inspection | Ship to shore Shore to ship shore to shore |  | - Pro-active information collection - Supportive information distribution |
| Support (port) Security | Shore to shore |  | - Pro-active information collection - Supportive information distribution |
| Support Port State Control | Shore to shore |  | - Pro-active information collection - Supportive information distribution |
| Support Incident Management | Shore to ship Shore to shore |  | - Pro-active information collection - Supportive information distribution |
| Support SAR and calamity operations | Shore to ship Shore to shore | - Information centre - Facilitating operational platform and infrastructure | - Pro-active information collection - Actual information collection - Supportive information distribution |
| Central co-ordination of allied services/instruments and availability of nautical service providers | Ship to shore Shore to shore | - E.g. pilots, tugs, linesmen | - Pro-active information collection for planning - Final availability check - Exchanging and updating information |

4.4		Key functionalities

The key functionalities to meet these aims and to execute present and future tasks properly are: 
                        1. Detect, identify and track targets

2. Traffic management

3. Receipt and provision of Maritime Safety Information

4. Information management including data maintenance and recording

5. Communications

6. Risk assessment and management

4.4.1	Detect, identify and track targets

- A positive means of detection, identification and tracking of all targets of interest (e.g. from multiple sources)

- A display of an overall image of the area of responsibility, including the traffic image and all relevant regional information (i.e. the Recognised Maritime Picture (RMP) which would normally include the information contained within official nautical charts and other relevant static and dynamic information layers)

4.4.2 	Traffic management

- A capability to assimilate, coordinate and prioritise all necessary information so as to effectively manage vessel traffic within the area of responsibility

4.4.3  	Receipt and provision of Maritime Safety Information (MSI)

- A capability to receive, formulate and disseminate MSI to vessels within the area of responsibility

4.4.4 	Information management including data recording

- A capability for information recording and archiving VTS/M information in accordance with local information archiving requirements, [consideration needs to be given to legal and protection aspects, such as the meeting of any legal or evidentiary requirements]

- A capability to retrieve and replay VTS/M information

- The diversity of information systems, tools and databases should be compatible and/or interfaced on international/supranational standards

- To reduce workload ashore and to optimize the operational use it’s necessary that the collection, handling, maintenance, exchange and presentation of information should also be formatted on international recognized standards

4.4.5	Communications

- Sufficient standardised and harmonised means to communicate all needed digital and analogue information between all relevant participants (e.g. ship/ship, ship/shore and shore/shore)

- A positive and reliable means for a VTS/M operator to initiate a process to alert a mariner of potential or developing risks (e.g. collision, grounding), through the provision of advice, the issuing of warnings and the activation of an onboard alert/alarm

- A standardised format be used for the transmission and sharing of VTS/M information

4.4.6  Risk assessment and management

- Capability to feed and monitor internationally recognised risk factor models/tools

- Effective planning, modelling and simulation tools for traffic management and risk analysis

- A capability to automatically determine if a potential incident is developing and a means to alert the operator

5	e-Navigation components to meet the shore based user requirements

Reference is made to the document VTS25-output-14 (IALA eNAV concepts) and document VTS26 WP1-revised, also taking into account document VTS 26 WP2.

The technical infrastructuur and components necessary to meet the user requirements in order to respond to the aims of VTS/VTM and carry out the responsibilities, tasks, services and to fullfill the key functionalities are shore  based facilities for:

| - The interaction between ships and shore |  |
| --- | --- |
| - Communications (sensor systems) | - VHF, MF and HF voice communications - NAVTEX - AIS - LRIT2 |
| - Communications (network systems) | - Satellite communications Inmarsat > GMDSS) - Broadband networks (GSM, GPRS, UMTS) |
| - Monitoring traffic / Navigation (sensor systems) | - Radar - AIS - LRIT - CCTV - VHF DF |
| - Navigation systems | - GNSS and terrestrial navigation systems (incl. availability monitoring) - Marginal ships navigator (Portable Pilot Unit) - Traditional Aids to Navigation - Virtual Aids to Navigation |
| - Presentation systems | - ECS/ENC/ECDIS - Visual (ECS) - Assisted docking |
| - Information systems | - Compatible strategic, tactical and operational information systems (data storage and exchange) |
| - Additional information systems | - Meteo and Hydro sensors (e.g., weather conditions, ice, fog, tidal) |
| - Storage systems | - Voice and data recording (incl. images) |
| - The interaction between shore and shore 1 (VTS/VTM to other parties within the own area, maritime/nautical information) |  |
| - Communications (sensor systems) | - VHF, MF and HF voice communications |
| - Communications (network systems) | - Public and internal telephone network (incl. fax) - Broadband networks (GSM, GPRS, UMTS) |
| - Information systems | - Compatible strategic, tactical and operational information systems (data storage and exchange) - Compatible administrative information systems |
| - Presentation systems | - ECS/ENC/ECDIS - Visual (ECS) - Assisted docking |
| - The interaction between shore and shore 2 (VTS/VTM with supportive information to parties inside the own area and relevant information to other parties outside the own area) |  |
| - Communications (network systems) | - A public telephone network (incl. fax) - Broadband networks (GSM, GPRS, UMTS) |
| - Information systems | - Compatible strategic, tactical and operational information systems (data storage and exchange) - Сompatible administrative information systems |
| - The interaction between within the VTS / VTM organization |  |
| - Communications (network systems) | - Public and internal telephone network (incl. fax) - Broadband networks (GSM, GPRS, UMTS) |
| - Information systems | - Compatible strategic, tactical and operational information systems (data storage and exchange) - Compatible administrative information systems |
| - Presentation systems | - ECS/ENC/ECDIS - Visual (ECS) |

6.	Challenges/future considerations to finalize shore based requirements

During the defining of the user requirements the VTS Committee concluded that, in order to finalize the requirements for the primary users ashore, VTS / VTM, it is essential that the following issue will be addressed: the interface/connection between eNavigation (where it comes ashore) and the already defined other concepts for information management (e.g. VTMIS, which is providing the services and standards).

The eNAV Committee is requested to consider and provide input to this topic to the VTS Committee. It is the view of the VTS Committee that the development of the user requirements cannot be completed if this issue is nog completely resolved.

The following diagram can be seen as a starting point for further discussions in and between the VTS and eNAV Committees.

shoreline

eNAV

VTS

Maritimewaters

VTMIS

Inland waters

VTM

- Request to the eNAV Committee

The eNAV Committee is requested to

- Take the presented identification of users and user requirements on board in the further development of the eNavigation concept

- To address the issue described in chapter 6 of this document and to report the views of the Committee before the next session of the VTS Committee, in which the user requirements can be finalized.

Depending on geographical circumstances, area of responsibility, to be decided by the CA

here set as a communication tool (which AIS and LRIT in essence are)

here set as a monitoring tool (data exchanged through AIS can be fused with other data to
  establish traffic image)

here set as a monitoring tool (data exchanged through LRIT can be fused with other data to
   establish traffic image)

depending on geographical circumstances, area of responsibility, to be decided by CA