VTS in Particular Areas

1. SCOPE

This guideline aims at defining a special application of measures and procedures for establishing and managing vessel traffic in particular areas where limitations to normal navigation may need to be or have been established. In order to better consider the effects or impact to shipping the VTS authority should participate in the process of establishing any kind of restrictions to navigation.

The purpose of these guidelines is to assist VTS authorities in defining appropriate procedures to manage traffic around and inside areas with particular restrictions to navigation.

2. REFERENCES

Reference documentation is IMO Resolution A.857(20), the IALA VTS Manual.

3. DEFINITIONS AND CLARIFICATIONS

Limited or restricted areas are intended to further enhance the common protection of shipping, other environmental interests and third party interests by addressing the unique risks and vulnerabilities in these areas, and to ensure these interests are better protected from potential threats and that vulnerabilities are limited by those protections. Examples of such are offshore installations, offshore renewable energy installations (OREI), national parks, military exercise areas, Sites of Special Scientific Interest and the overall environmental protection.

The conditions that lead an authority to establish a restricted area are by their nature factors of potential danger to navigation and to the environment or any kind of installations and require strict attention to be paid by VTS.

Frequency and/or type of shipping in the vicinity of such areas can potentially increase risks and vulnerabilities and therefore should be taken into consideration when determining the restricted or limited area. Particular care should be given to make sure that control measures will prevent/limit shipping to enter this area and to monitor and report possible violations/infringements to such limitations

4. ROLE OF A VTS IN ESTABLISHING A RESTRICTED OR LIMITED ACCESS AREA

In many cases an authority other than a VTS has the responsibilty to establish a restricted or limited access area (e.g. Harbour Master, Coastguard, Ministry of Environment), however the VTS should have a consultative or supportive role in the establishment process of such areas.

As VTS has the most expertise and experience regarding traffic movement in its area, it should advise the establishing authority about the consequences for shipping and for the access to harbours. In some cases a VTS Authority may advise that establishing such an area is not appropriate.

An analysis of statistical traffic data and of potential dangerous situations that may occur in the area affected may be provided by the VTS Authority to evaluate the consequences related to shipping interests and/or the goods to be protected (i.e. shipping, human, environment, offshore installations, etc) in terms of frequency and outcome. Consideration must be given to the location of the area; in particular the applicability of legislation (e.g. UNCLOS).

A VTS should also participate in the evaluation process so that adjustments can be made to the areas/limitations

While assessing the need to establish restrictions to navigation, responsible authorities should consider the necessity to make modifications (for example: increase VTS area, install new equipment, change services the VTS provides...) to an existing VTS system or even to establish a new one, in order to minimise all the risks to as low as reasonably practicable.

5. ROLE OF A VTS IN THE MANAGEMENT OF RESTRICTED OR LIMITED ACCESS AREA

In circumstances where the risk related to navigation is increased, VTS is one of the most efficient instruments through which such a risk may be dealt with in order to reduce it to an acceptable level.

The objectives are protecting the traffic through monitoring, enforcing the regulations, protecting the interests of other authorities, stakeholders and third parties.

To this end the VTS centre may need to adopt particular provisions such as to;

- impose specific navigation measures (e.g. recommended route)

- implement specific reporting obligations

- arrange for the exchange of related information with other organisations involved

- monitor/co-ordinate safety zone

- insert boundary co-ordinates and set appropriate alarms for the area.

- update existing operating procedures

- arrange appropriate response measures

- provide information of infraction to enforcement authorities

- prevent unauthorized access

Notes:

- End product is a guideline

- Develop an annex listing types of areas, measures, restrictions: what it is, where it might be, what its purpose is and what authorisation is required to establish

- Identify problem (through risk assessment) – determine management method – develop requirements to implement

- Risk assessment – role in determining need for measures and assessment of their effectiveness.

- Add references: World VTS Guide, others