VTS 27 – WP4

WORKING PAPER ON NAV ASSISTANCE SERVICE (OPS WG)

Review / provide guidance on navigational assistance service (Task 8*)

A sub-workgroup of the Ops WG reviewed input paper VTS 27/13/01, IMO resolution 857(20) and VTS Manual chapter 0508 (2008).

The sub working group considered that there is a need of clarification of the text used in the manual.

Navigational Assistance Service (NAS)

A Navigational Assistance Service may be provided in addition to an Information Service or Traffic Organisation Service. It is a service that provides essential and timely navigational information to assist in the on-board navigational decision-making process. It may also involve navigational advice and/or instruction.

The Navigational Assistance is used in different navigational or meteorological circumstance or in case of defects or deficiencies. Such assistance requires positive identification and continuous communication throughout the process. The beginning and the end of navigational assistance should be clearly stated by the VTS. The service is normally rendered at the request of a vessel or when the VTS realize that a vessel is in a crucial situation. For example, potential collision between vessels, potential collision with hazard, risk of grounding, vessel off course, critical overtaking and assistance to anchor area.

The use of message markers should be used to avoid misunderstandings. Both IMO Res A857 and the IALA Manual state the principle of result orientation. There is no conflict between this principle and advising or instructing altering off the course. It is recommended to primarily use advice, unless the situation dictates otherwise e. g. emergencies situations, where VTS might be forced to use advice or instructions although this is not at service delivered by the particular VTS. This to avoid any misunderstanding with regard to navigational responsibility and any language or other communication difficulties.

VTS Authorities should give careful consideration to staffing levels, their qualifications, operational procedures and equipment capability when implementing this type of service.

Navigational Assistance may involve the provision of information, such as:

- Course and speed made good by a vessel;

- Position relative to fairway axis, navigational features and/or way-points;

- Proximity to navigational hazards; and

- Positions, identities, intentions and any restrictions of surrounding traffic.

Navigational Assistance may also involve the additional provision of advice and/or instruction, and may include or require:

- An assessment of the suitability of the vessel to respond to the advice provided including an assessment of linguistic ability;

- A review of vessel characteristics including manoeuvrability relative to the area in which the service is provided and any defects or deficiencies;

- An assessment of the environmental conditions;

- An assessment of the implications of the cargo carried;

- A review of the proposed sailing plan;

- Recommendations on measures to maintain the sailing plan noting that any advice on courses and speeds should be result orientated; (leaving the details of execution, such as course to be steered or engine manoeuvres to be executed, to the master or pilot on board the vessel. Care should be taken that VTS operations do not encroach upon the master's responsibility for safe navigation, or disturb the traditional relationship between master and pilot.)

- The use of IMO message markers; e.g. ADVICE and INSTRUCTION

- The use of a dedicated frequency; and

- Restriction of other traffic movement.

Depending on the type of service delivered by VTS the information/advice/instruction should be result-oriented only, leaving the details of the execution to the master or pilot.

Notes:

- Provide practical guidance on delivery of NAS – how do you do it?

- More details on specifying the beginning and end of NAS

- Provide best practices on use of message markers during NAS – e.g., use at all times or in certain circumstances

- Emphasize that NAS is vessel specific – NAS provided to a particular vessel