IALA Guideline No. ####

On

A minimum comprehensive mix of AtoN in fairways including dredged channels and canals

Edition 1

[May 2009]

AISM Association Internationale de Signalisation Maritime      IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone: +33 1 34 51 70 01  Fax:  +33 1 34 51 82 05

e-mail:  iala-aism@wanadoo.fr       Internet:  www.iala-aism.org

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |

Table of Contents

Document Revisions	2

Table of Contents	3

Index of Tables	4

Index of Figures	5

Guidance on a minimum comprehensive mix of AtoN in fairways including dredged channels and canals	6

1	Introduction	6

2	Minimum requirements	6

2.1	General	6

2.2	Relevant IMO documentation	8

2.2.1	SOLAS, Chapter V, regulation 10	8

2.2.2	SOLAS, Chapter V, regulation 13	8

2.2.3	IMO-resolution A.915(22) “Revised Maritime Policy and Requirements for a Future Global Navigation Satellite System (GNSS)”, adopted on 29 Nov 2001	8

2.2.4	IMO Resolution A.953(23) Adopted on 5 December 2003 WORLD-WIDE RADIONAVIGATION SYSTEM	9

2.2.5	IMO Resolution A.423(XI) Radar beacons and transponders	9

2.3	Relevant IHO Documentation	9

2.3.1	IHO STANDARDS FOR HYDROGRAPHIC SURVEYS 5th Edition, February 2008 Special Publication No. 44	9

2.4	Relevant IALA Documentation	9

2.4.1	IALA Recommendations	9

2.4.2	IALA Guidelines	9

3	Accuracy	10

3.1	IMO-resolution A.915(22)	10

3.2	Navigation Accuracy – Position Accuracy of the vessel	10

3.2.1	General	10

3.2.2	Radionavigation	11

3.2.3	Visual and other short range A to Ns	11

3.2.4	Channel design - the hydraulic engineering aspect	11

3.3	Positioning Accuracy of the A to N	11

4	Types and principles of Marking Fairways	12

4.1	Purpose of Marking	12

4.2	General Principles of Marking	13

5	Perception of A to N	13

5.1	Visual A to Ns	13

5.1.1	Useful range	14

5.1.2	Lights	14

5.1.3	Daymarks and unlighted AtoN	15

5.1.4	Classes of buoys	15

5.1.5	Perception with shipborne Radar	15

5.1.6	Perception by means of additional electronic devices on A to Ns	16

5.2	Principles for different Types of Marking (of a Fairway)	16

5.2.1	General	16

5.2.2	Design principles for buoys and beacons on the fairway	16

5.2.3	Leading lines	18

5.2.4	Sector lights	18

5.2.5	Electronic aids to navigation	19

5.3	Special Requirements for different user groups	19

5.3.1	Categorisation	19

5.3.2	High speed craft (HSC)	19

6	Examples of Marking of fairways	20

6.1	Rio de La Plata, Argentina	20

6.2	Finland	20

6.3	Seekanal Rostock	20

7	Procedures for determining the requirements for short-range marine aids	20

7.1	Types of analysis and associated procedures	20

7.1.1	Site Analysis (refer to 5.1.1)	21

7.1.2	Needs Analysis (refer to 5.1.2)	21

7.1.3	Operational Analysis (5.1.3)	21

7.1.4	Cost-Benefit Analysis (5.1.4)	21

7.1.5	Site analysis	22

7.1.6	Needs analysis	22

7.1.7	Operational analysis	23

7.1.8	Simulation and GIS	23

7.1.9	Cost-benefit analysis	24

8	Conclusion	24

9	References	25

ANNEX 1	Desired Positioning Tolerance of Floating A to N	26

ANNEX 2	A to N design for the Rio de La Plata Navigation Channel	28

ANNEX 3	Finland	30

ANNEX 4	A to N design for the “Seekanal Rostock”	31

Index of Tables

Table 1	Minimum maritime user requirements for general navigation	8

Table 2	Horizontal uncertainty, requirements:	12

Table 3	Desired Positioning Tolerance	27

Table 4	Title required	33

Index of Figures

Figure 1	VTS Coverage – Pilotage Services	29

Figure 2	Canal Section with two types of floating aids: Deep Water buoy and Hinge Beacon	32

Figure 3	AtoN scheme of the inner part of “Seekanal Rostock”	33

Figure 4	Sea Chart of a part of the outer and inner channel “Seekanal Rostock”:	34

Guidance on a minimum comprehensive mix of AtoN in fairways including dredged channels and canals

# Introduction

Until recently, mariners have used the radio aids to navigation systems (also referred as electronic aids to navigation) and the short range systems in two distinct geographic areas.  The short range system was mostly used close to shore and in restricted waterways.  On the opposite, vessels were able to navigate offshore safely using less accurate radionavigation systems.

However, considering the availability, reliability and relative low cost of high precision electronic aids systems (e.g. GPS/DGPS and electronic charting programs) available today, these two areas of navigation are beginning to overlap.  This is especially true in those transitional areas where mariners shift from the low accuracy requirements of ocean navigation to the high accuracy needs of coastal and inshore piloting.  As electronic aids continue to improve, their use will increase in areas where previously only short range aids provided the necessary accuracy.  This evolutionary change must be recognized and accounted for when conducting waterway analyses.

Safe navigation by electronic means, does require a certain level of accuracy, availability and integrity in order to accurately portray the fairway/waterway, dredged channel or canal.  There should also be an adequate level of integration with the control of the ship either automatically or through the navigator.  As there are uncertainties connected with this, visual navigation eventually combined with the use of radar is in principle either a primary or a secondary means of navigation.

So, despite the technical changes there remains a significant need to provide mariners with navigational information when piloting within restricted waters.  The best manner in which to provide information to this level of precision is through the use of a short range aids to navigation system consisting of buoys, beacons, sound signals and electronic systems such as Radar beacons (RACONS) and Automated Information System (AIS) devices on A to Ns and through radionavigation systems like GNSS and DGNSS.

This guideline caters for this and provides guidance for Aids to Navigation Design regarding fairways/waterways including dredged channels and canals which should be both relevant to the mariner and cost effective to the National Authorities. In order to cover different situations, the guideline is quite broad.

Aids to navigation systems may be provided for the safety of navigation in various kinds of fairways, dredged channels and canals such as in:

- Waters adjacent to the coast

- Archipelagic waters, in pristine and/or improved condition

- Estuarine rivers

- River systems

- Straits

- Narrow stretches of land.

- Open Sea with Ship Routeing Systems

Different users are considered, as there are different types of vessels and boats facilitating commerce, fishing, leisure and a number of other activities that can typically be found in marine environments.

# Minimum requirements

## General

The primary goal of the design of waterway aids to navigation systems is to facilitate safe, economical and efficient movement of vessels.  Aids to navigation systems are designed to assist navigation and not to replace normal prudent navigational practices or the use of onboard navigational equipment. The responsible provision of aids to navigation systems requires that systems be designed to meet the essential minimum requirements for safe and expeditious navigation through specific waters in accordance with the type and volume of traffic.

The aids to navigation provided are normally intended to function in systems and therefore mariners should make use of the full information provided.  These systems are intended to be used by the prudent mariner who knows and understands the functioning of aids to navigation, uses them in conjunction with charts and other published information, and conforms to the applicable rules of the waterway.

Whether designing a new waterway system or evaluating an existing one, there are many factors that must be considered.  The identification of these factors allows National Authorities to develop a greater understanding of the risks and threats that are present within a particular waterway.  Section 7 of this guideline provides guidance for completing this analysis.  Recognizing that waterways are different, the site analysis, needs analysis, simulation, and operational analysis provide the necessary framework to generically evaluate the overall risks that may be present and provide mitigation measures.

These factors include the consideration of

- User “needs”;

- Shoreside characteristics;

- Physical and environmental condition of the waterway;

- Significant traffic types and density using the waterway.

Once the evaluation has been completed, National Authorities should use this information to design the aids to navigation system that mitigates the risk to safe transit or reduces it to an acceptable level.  In completing the design it is important to note that the entire waterway must be viewed using a systematic approach, recognizing that each individual element of the waterway design by itself will not reduce transit risk.  So while individual regions of the waterway must be considered, the overall aids to navigation system must support a smooth transit of the waterway as a whole. This way, the waterway must be examined “end-to-end” to ensure that an acceptable and uniform level of risk is present throughout the waterway.  While the analysis process is a tool for the assessment of risk, it should be the National Authorities objective to design an aid to navigation system best able to manage those risks.

The specific aids to navigation system implemented should enable waterway users to transit an area safely and efficiently, while avoiding groundings, obstructions to navigation, and collisions with other vessels.  Therefore, in order to satisfy the information requirement of the user, a system of aids to navigation must:

- Be available at the time it is needed

- Provide timely warning of danger from channel limits and fixed obstructions to navigation

- To enable the mariner to determine quickly their location within the channel, relative to fixed obstructions to navigation, and relative to other vessels

- Enable a safe course for the vessel to be determined.

Once the system has been established, maintaining the availability of this system is critical to controlling overall risks.  Please refer to IALA Recommendation O-130 on short range AtoNs for additional information related to the categorization of individual aids to navigation, the calculation of availability targets, and recommended availability objectives.

Duplication in function of an AtoN may be appropriate and necessary to provide a degree of safety in the event of a discrepancy and to avoid excessive costs of emergency repairs. Moreover temporary duplication may be provided when new or alternative types of aids are being introduced in order to allow a safe transitional period.

## Relevant IMO documentation

### SOLAS, Chapter V, regulation 10

Ships' routeing systems contribute to safety of life at sea, safety and efficiency of navigation and/or protection of the marine environment. Ships' routeing may be established by the coastal state within its territorial sea. However, routeing systems are not always marked with physical AtoNs.

In the Exclusive Economic Zone and on the High Seas IMO is the competent body for Ships' routeing measures. Normally countries also submit Ships' routeing proposals that are within their territorial sea to IMO for adoption.

### SOLAS, Chapter V, regulation 13

Each of the Contracting Governments undertakes to provide, as it deems practical and necessary, Aids to Navigation as density of traffic and the level of risk requires. IMO member states commit themselves to take into account the international recommendations and guidelines when establishing such aids.

According to the same regulation, contracting Governments undertake to arrange for information relating to aids to navigation to be made available to all concerned. So, information on A to Ns has to be distributed to all users who could be affected.

Changes in the transmissions of position-fixing systems which could adversely affect the performance of receivers fitted in ships shall be avoided as far as possible and only be effected after timely and adequate notice has been promulgated.

### IMO-resolution A.915(22) “Revised Maritime Policy and Requirements for a Future Global Navigation Satellite System (GNSS)”, adopted on 29 Nov 2001

Future GNSS(s) should meet the maritime user's operational requirements for general navigation, including navigation in harbour entrances and approaches and other waters where navigation is restricted. The minimum maritime user requirements for general navigation are given in Appendix 2 to the annex of the resolution as follows.

- Minimum maritime user requirements for general navigation

|  | System level parameters |  |  |  | Service level parameters |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Absolute Accuracy | Integrity |  |  | Availability % per 30 days | Continuity % over 3 hours | Coverage | Fix interval² (seconds) |
|  | Horizontal (metres) | Alert limit (metres) | Time to alarm² (seconds) | Integrity risk |  |  |  |  |
|  |  |  |  | (per 3 hours) |  |  |  |  |
| Ocean | 10 | 25 | 10 | 10-5 | 99.8 | N/A1 | Global | 1 |
| Coastal | 10 | 25 | 10 | 10-5 | 99.8 | N/A1 | Global | 1 |
| Port approach and restricted waters | 10 | 25 | 10 | 10-5 | 99.8 | 99.97 | Regional | 1 |
| Port | 1 | 2.5 | 10 | 10-5 | 99.8 | 99.97 | Local | 1 |
| Inland waterways | 10 | 25 | 10 | 10-5 | 99.8 | 99.97 | Regional | 1 |

Notes:	1: Continuity is not relevant to ocean and coastal navigation.

2: More stringent requirements may be necessary for ships operating above 30 knots.

### IMO Resolution A.953(23) Adopted on 5 December 2003 WORLD-WIDE RADIONAVIGATION SYSTEM

It defines the operational requirements for radionavigational systems, for each navigational area.

### IMO Resolution A.423(XI) Radar beacons and transponders

## Relevant IHO Documentation

### IHO STANDARDS FOR HYDROGRAPHIC SURVEYS 5th Edition, February 2008 Special Publication No. 44

Usually, the uncertainty of positioning an AtoN should not be bigger than the uncertainty in survey and charting.

In CHAPTER 2 of this Standard, - ”POSITIONING” - 2.1 Horizontal Uncertainty the uncertainty of a position is defined as the uncertainty at the position of the sounding or feature within the geodetic reference frame.  Positions should be referenced to a geocentric reference frame based on the International Terrestrial Reference System (ITRS) e.g. WGS84.  The position uncertainty at the 95% confidence level should be recorded together with the survey data.

The position of soundings, dangers, other significant submerged features, navaids (fixed and floating) - these are called AtoNs in this IALA Guideline - , features significant to navigation, the coastline and topographical features should be determined such that the horizontal uncertainty meets the requirements specified in Table 1.  This includes all uncertainty sources not just those associated with positioning equipment.  Some figures of Table 1 regarding aids to navigation will be discussed in the section “

## Relevant IALA Documentation

Following lists to be checked and be confirmed short before approval of this Guide:

The IALA Maritime Buoyage System (set in force by IMO) must be observed. However, for the inland waterways, there might be different legislation, other rules and other marking systems established by national authorities.

Example: SIGNI (Signs and Signals on Inland Waterways, Revision 1, from 2005, New York and Geneva, worked out by the Working Group on Inland Water Transport of the Inland Transport Committee of the Economic Commission for Europe of the United Nations).

The IALA Navguide, offers overall guidance. In particular, chapters 3 on Aids to Navigation and 6.1.3 on Risk Management.

### IALA Recommendations

A-126 “On the use of AIS in Marine Aids to Navigation Services”, Edition 1.3, June 2007

E-110 “For the rhythmic characters of lights on aids to navigation” (Edition 2.0 December 2005)

E-111 "For port traffic signals" May 1998.

E-105 "On the need to follow national and international standards", May 1998

E-112 “For leading lights”, MAY 1998

E-200-Series   include here

O-130 "On categorization and availability objectives for short range aids to navigation", December 2004,

O-113 The Marking of fixed bridges over navigable waters, MAY 1998

O-138 on the use of simulation.

### IALA Guidelines

1010 “On Racon range performance”, Ed. 2, JUNE 2005

1018 “On risk management”, December 2000

1023 “For the design of leading lines”, December 2001

1033 On the provision of aids to navigation for different classes of vessels, including high speed craft, December 2003.

1041 “Sector Lights”, December 2004

1058 Use of Simulation as a Tool for Waterway Design and Aids to Navigation Planning, December 2007,

1051 On the provision of Aids to Navigation in Built-up Areas; December 2005

Other IALA recommendations and guidelines on VTS, AIS, Racons, DGPS should be considered.

# Accuracy

## IMO-resolution A.915(22)

The IMO-resolution A.915(22) “Revised Maritime Policy and Requirements for a Future Global Navigation Satellite System (GNSS)”, adopted on 29 Nov 2001, defines the following terms in its Appendix 1 “Terms used in the GNSS”:

- Absolute accuracy (Geodetic or Geographic accuracy). The accuracy of a position estimate with respect to the geographic or geodetic co-ordinates of the Earth.

- Geodetic or Geographic accuracy  See Absolute accuracy.

- Predictable accuracy  The accuracy of the estimated position solution with respect to the charted solution.

- Relative accuracy  The accuracy with which a user can determine position relative to that of another user of the same navigation system at the same time.

- Repeatable accuracy  The accuracy with which a user can return to a position whose co-ordinates have been measured at a previous time using uncorrelated measurements from the same navigation system.

- Alert limit (or threshold value)  The maximum allowable error in the measured position during integrity monitoring - before an alarm is triggered.

As indicated in IMO-resolution A 915(22), the position accuracy (95 %) for Global Navigation Satelite Systems (GNSS) for general navigation should be 10 m for most of the different types of waterways according to Appendix 2 of IMO-resolution A 915(22), which can be found in section 2.2 of this guideline.

## Navigation Accuracy – Position Accuracy of the vessel

### General

In general the position accuracy for the user (the vessel) should be good enough to fulfil the required navigation accuracy. However, there will be a level where, depending on the properties of vessel and waterway, the navigation accuracy cannot be improved further by improving the position accuracy.

For the mariner it is relevant to know exactly the distance from the vessel to a certain point or line, like a critical hazard or the limitation of a fairway.

The distance can be found as the difference between two absolute positions. Then it is determined by the absolute accuracies of both positions.

The distance can also be found directly, if there is a visual aid or a radar target or any other device showing directly the relevant point or line. This is described by the relative accuracy.

Both types of accuracy can be relevant for practical navigation and for the ability of a ship to stay on track within a fairway/waterway with the available aids.

For both instances, the position of the AtoN is referenced to the nautical chart. The concept of absolute accuracy is in general more developed for the nautical chart and the GNSS.  That is the case where nautical charts are surveyed to high accuracy and the ships position is available throughout the voyage and with high accuracy by the use of GNSS.

### Radionavigation

Most of the Radionavigation systems provide a very good absolute accuracy for determination of  the position based on a geodetic datum. Using the navigational chart the mariner can navigate safely along a waterway.  The navigation accuracy is limited not only by the Radionavigation  itself, but also by the accuracy of the navigational chart and the navigational equipment and performance on the ship.

However, there is a limitation in the safe use of GNSS position data caused by the Alert limit (or threshold value).  The maximum allowable error in the measured position – during integrity monitoring - before an alarm is triggered, is 25 m according to Appendix 2 of IMO-resolution A 915(22), which can be found in section 2.2 of this guideline. So the user cannot rely absolutely on an accuracy of better than 25 m.  So, this is one more reason for having visual and short range AtoNs for reference, also from the point of view of the Radionavigation.

### Visual and other short range A to Ns

Short  Range A to Ns including most of the visual aids in many cases do not provide  such a good absolute accuracy.  But they provide a good relative accuracy, provided the aids are properly positioned. So the mariner can use them directly to navigate safely along a waterway.

The position obtained by direct use of visual and radar aids without continuous tracking on the sea chart or on a modern radar screen is restricted by the relative accuracy and will not remain constant along the track. The concept of accuracy is in this case the ability to keep the intended track and to locate the movement of the ship along this track.

Visual and other Short Range AtoNs are – as already mentioned before -  necessary for providing a reference for the use of GNSS.

### Channel design - the hydraulic engineering aspect

Waterways planners as for instance hydraulic engineers and mariners in the port and waterways authorities should consider the option of a significant contribution to the efficiency of major dredging projects and waterways maintenance by improving positioning accuracy and navigational accuracy due to proper use of Aids to Navigation.  Thus in some cases the required channel width could be reduced and so the costs for major and maintenance dredging.  In the PIANC Guide “Approach Channels” which is under review now, there can be found some ideas regarding this issue.

## Positioning Accuracy of the A to N

The positioning accuracy of the AtoN itself is a basic factor and limitation for the attainable position accuracy for the vessel.

The position of the aids to navigation should be accurate and at least in accordance with IHO standards so that a ship sufficiently can establish its position and follow a track in the fairway by visual means, radar or Radionavigation.  Generally the position for the location of A to Ns should be first determined on the nautical chart.  This may be different for direct local positioning of a small buoy or a withy.

The aids to navigation should be surveyed and positioned using the same accuracy as the nautical chart.  Table 2 gives an extract from TABLE 1 of the IHO STANDARDS FOR HYDROGRAPHIC SURVEYS (S-44) 5th Edition February 2008 15, shows Minimum Standards for Hydrographic Surveys as far as A to Ns are concerned (To be read in conjunction with the full text set out in the IHO document).

- Horizontal uncertainty, requirements:

| Reference | Order | Special | 1a | 1b | 2 |
| --- | --- | --- | --- | --- | --- |
| Chapter 1 | Description of areas. | Areas where under-keel clearance is critical | Areas shallower than 100 metres where under-keel clearance is less critical but features of concern to surface shipping may exist. | Areas shallower than 100 metres where under-keel clearance is not considered to be an issue for the type of surface shipping expected to transit the area. | Areas generally deeper than 100 metres where a general description of the sea floor is considered adequate. |
| Chapter 2 and note 5 | Positioning of fixed aids to navigation and topography significant to navigation. (95% Confidence level) | 2 metres | 2 metres | 2 metres | 5 metres |
| Chapter 2 and note 5 | Mean position of floating aids to navigation (95% Confidence level) | 10 metres | 10 metres | 10 metres | 20 metres |

However, if an AtoN is used directly to provide very precise relative positioning to the user on the vessel, e.g. a leading line, the horizontal uncertainty of 2 metres will not meet the requirements for this type of AtoN.

The position and therefore the navigation accuracy of floating short range aids to navigation is sometimes difficult to define absolutely.  The required level of accuracy is dependent upon the navigation information the aid is designed to provide, the availability of other aids and landmarks within the immediate vicinity and the overall level of risk that may be present within a particular section of the waterway.

Floating aids to navigation are subject to a great degree of variation due to water level, tides and current, to the capabilities of the servicing vessel.  Specifically, the servicing vessel or craft may not always be capable of returning a floating aid to navigation to its charted position.

If precise surveying systems like DGPS or precise DGPS are available in the area and the floating aids are easy accessible by the service vessel the positioning accuracy should be realized as good as possible.

But there will still be some inaccuracy of the position of the floating aid because it will have a swinging circle radius, depending on the chain length, ground chain, thrash and riding chain and the water depth at low water and high water.  If the swinging circle radius is bigger than the desired positioning tolerance, elastic moorings or hinge beacons (Section 6.3 and Annex 6 of this guideline) or the employment of beacons should be considered.

Annex 1 provides an example – differing from the IHO definitions, how the Desired Positioning Tolerance (DPT) can be defined. This could be used by IALA members, if applicable, especially under conditions when the horizontal uncertainty of 20 metres cannot be practically achieved.

In case of very narrow fairways, the DPT should be defined in % of fairway width or % of beam of maximum vessel for the fairway rather than absolutely in m.

# Types and principles of Marking Fairways

## Purpose of Marking

In fairways, channels and restricted waterways AtoNs indicate generally the limits of the navigable waters and/or preferred routes.

Furthermore AtoNs are – in conjunction with a marked fairway or separately - used to:

- Assist landfall;

- Mark of approaches to ports and waterways;

- Mark tracks;

- Mark hazards;

- Identify positions;

- Show courses;

- Indicate preferred routes;

- Separate traffic (e.g. to mark traffic separation schemes or the middle of a fairway);

- Indicate special areas such as anchorages;

- Mark restricted and/or prohibited areas.

## General Principles of Marking

The types of buoys are described in the IALA Maritime Buoyage System, which has to be consulted as the mandatory system for buoyage.  Thus the systematic, consistent and clear use of floating Aids to Navigation can be ensured.

It is essential, that the Aids to Navigation are identified and located on charts (ENC, paper chart, synthetic radar screen).  The references to IMO- and IHO-requirements, as already described in preceding sections of this guideline, are also essential.

In narrow or winding passages it may be difficult for mariners to correlate the vessel’s position with chart information in a appropriate time.  Therefore the navigational marks should be explicit and clear so that navigational decisions can be made directly by using visual and/or radar information.

Reliance upon a single aid to navigation may result in a higher availability requirement which may prove difficult for the National Authority to meet.  If this is the case, then the implementation of multiple aids may be used to provide redundancy during periods of unavailability by certain aids.

Aids to navigation systems should be designed to assist mariners under various weather, sea and ice conditions found during the local navigation season for the various types of vessels.

Short Range Aids to Navigation, especially buoys, should be designed regarding their visual information, radar information (active or passive) and other modes of information (Radionavigation, AIS).  Design elements must take into account the visibility and radar availability implications. Designing for worst case visibility is not practical. Reduced visibility due to haze and fog must be considered.

In designing and redesigning systems, past incidents such as groundings, collisions or near-misses must be carefully considered.  Such incidents should be well documented to ensure accuracy of the information used for a decision to change or not to change the configuration of aids in a system.

During the design phase the required level of availability should be considered.  The level of availability is dependent upon the threats and risks that are mitigated through the use of a particular aid to navigation.  In those areas in which the level of risk has been determined to be high, the use of certain types of aids to navigation may prove to provide greater risk mitigations. However, the planner must also consider the higher availability objectives that may be required.

# Perception of A to N

## Visual A to Ns

### Useful range

To describe the navigational requirement for the visual perception of A to Ns the useful range can be used as a parameter.  This is the practical convenient range for a mariner to identify an A to N. This range can be found using scientific knowledge about the light, the atmosphere and the human eye and using experience.

The IALA expertise on this issue is laid down in some IALA documents which will be referenced in the relevant sections of this guideline.

Generally there are different qualities of visual perception:

- The object is visible (Pure visibility).  The aid can be seen but appears still too small to be recognized as a certain type of A to N according to IALA buoyage system;

- The object can be identified (identification): The aid can be identified, e. g. as a certain type of buoy of the IALA buoyage system. Usually this will be the quality of perception which is needed for a practical use of the A to N for the navigator. So this quality will determine the useful range.

In some cases the Conspicuity of an object is relevant or limiting for its visibility and particularly for the identification and for the use for the navigator.  An object is conspicuous if it appears outstanding in a complex visual scene.

In every case it is necessary that the visual information is confirmed (Confirmation, Verification).

This process can take time and could thus be a limiting factor, if the visual information emitted from the aid is influenced or interrupted by intermittent influences like movement of a floating aid by waves.

But this process can also provide a kind of redundancy, which means an increase in the safety of use of the aid.  The aid sends out its information in different ways (light, shape, colours, marking by letters and numbers, other means).  As long as the information of one aid, received by different means, is conflicting, there is no confirmation of the information from the aid.

The conditions of perception can be different in:

- Good visibility at daytime;

- Good visibility at night;

- Poor visibility.

### Lights

The parameter to define the user requirement for a light is the luminous range. This is the distance from which – under defined conditions - the user can identify the light.

The identification and confirmation of a light includes also its colour and character.

The parameter to describe the light itself is the intensity, the colour and the distribution of the light. The IALA-Recommendations of the E-200 series provide methods how to calculate the luminous range which can be obtained by a light with a given intensity.

The calculation includes the following factors:

- Intensity;

- Light character;

- Colour;

- Transmissivity of the atmosphere;

- Daytime / night time;

- Background illumination.

The conspicuity in the specific situation must be considered.

The details can be found in the IALA Recommendation series E 200.

The prevailing visibility conditions will vary over different geographical locations.  Therefore, when selecting a light, this should be taken into account.  Selection and calculation of the useful range should be based on a practical luminous range value and not on nominal range.

However, the luminous range will be promulgated in nautical charts and publications for mariners, lists of lights etc.

### Daymarks and unlighted AtoN

The distance in which a daymark can be identified depends on:

- Size;

- Shape;

- Colours (one or more on one aid).

A simple rule for the estimation of the distance in which a daymark of a simple shape can be identified – provided the contrast to the background is sufficient - is as follows:

The object can be identified when it appears at the eye of the user under an angle of 3’ (three minutes).

Other distances can be defined for identification of topmarks and identification of numbers.

The contrast to the background and the identification of the colours depends on the:

- Chromaticity of the paint of the aid, the specific meteorological visibility in the area;

- Colour and illumination of the background;

- Conspicuity.

The details can be found in the “IALA Guideline on Daymarks for Floating Aids-to-Navigation”..Draft Edition 1ab 4. April 2008  new reference?

It has to be considered that there is poor visual perception of unlighted A to N at night. There are some tools that assist navigation using unlighted AtoN at nighttime:

- Searchlight on board;

- Reflective sheets on the aid (buoy, beacon, withy);

- Shipborne Radar;

- Radar reflector on A to N   see also next section (reference number required, please)

### Classes of buoys

It might be considered to define classes of buoys (size and shape) with certain identification distances.  If fitted with a light, this should have the same useful range.  Thus the right buoy can be chosen from this “toolbox” instead of designing a new one for each case.

### Perception with shipborne Radar

The distance in which an AtoN can be detected in on the radar screen on the bridge of a ship depends on the:

- Height of the ship’s radar;

- Observation area chosen by the mariner on the radar screen;

- Size and the shape of the AtoN;

- The existence of radar reflectors and there performance parameter;

- RACONs on the AtoN.

Only if a RACON is installed an identification of the aid by means of shipborne radar; if only the other tools are used, there is no individual identification of the aid by means of shipborne radar.

### Perception by means of additional electronic devices on A to Ns

Also for short range AtoN the perception can be improved by electronic means as A to N-AIS. More Details can be found in IALA Recommendation A-126 “on the use of AIS in Marine Aids to Navigation Services”, Edition 1.3, June 2007.

## Principles for different Types of Marking (of a Fairway)

### General

The limitation of a fairway indicates in general the area of navigational waters and the general track for the vessels in this area.  Additional short range aids to navigation and electronic aids to navigation may be used in a right mix to indicate:

- Critical points;

- The (middle) of the fairway;

- Change of direction;

- Marking of different dangers;

- Marking of different areas.

Each of these types of marking can use:

- Lateral, cardinal or safewater marks, which are physically directly on the fairway or on the point they indicate - or very close to it.

These can be floating (buoys) or fixed (beacons);

- Especially single markers for warning purposes, curve turning points and longitudinal positioning;

- Paired markers for both longitudinal and lateral positioning;

One advantage of this type of marks is that they are well visible with shipborne radar. One disadvantage of this type of marks is that they can be damaged by ice, current, collision.

- Leading marks, which are not physically in that place they indicate, especially leading lines along the centre line of the fairway or as powerful tool for lateral positioning in the fairway;

- Sector lights as a tool for lateral positioning in the fairway (direction lights);

- Sector lights with oscillating boundary a powerful tool for lateral positioning in the fairway;

- Sector lights abeam or leading lines crossing the fairway at turning points or other relevant positions (longitudinal positioning in the fairway);

- Other aids to navigation including radio aids;

- AIS.

### Design principles for buoys and beacons on the fairway

A fairway shall be marked in principle by lateral marks.

There shall be buoys or beacons at least at bends and junctions of the fairway.

Light buoys or beacons should be generally used for marking:

- Approach buoys;

- First and last pair of buoys;

- Changes of direction;

- Dangers or hazards for shipping;

- Special marks (for example, measuring instruments in the field of shipping routes).

The sections between the bends and junctions should be divided into the same distances between buoys.

In general, the useful range (identification) of buoys at daytime, night time and on the on-board-radar screen should be greater than the distance between the buoys.

The mostly used observation area on the on-board-radar is 3 nm. So, the buoy distance of a continuous buoyage should not exceed 3 nm.

Simulation tests for lighted bouys for narrow waters with 150 m to 600 m width have shown buoy separation distances of 1 - 1.5 M as the best for navigation.

The distances of unlighted buoys are based on their size and daytime visibility.

In general, the buoys or beacons of one fairway section should have the same distance from the axis of the channel.

Generally the buoys of a fairway shall be conducted as pairs (“gates”), especially if high navigation accuracy or a very clear and continuous buoyage.

The design of a continuous buoyage of a fairway which is formed by straight lines and bends can be done in three steps:

- Select a conspicuous buoy or pair of buoys for the point where the fairway starts;

- Place buoys on points.

  - Where ships have to divert their course or;l

  - Where the fairway boundary line or the middle line has a bend or;

  - Where critical shallows and rocks or other hazards form the boundary of the fairway and at;

  - Fairway intersections.

- Distribute buoys between these points regarding the distance, in which they can be detected and identified (section 3.3 of this guideline and other relevant IALA documents).

The distance, in which the marks should be detected and identified, may be different in relation to the length of the fairway section for different cases as follows.  The buoy separation distance should be chosen in such way, that the next one or two buoys are recognized ca. 100 or more before the nearest one has been passed or in other cases ca. 100 m before the last buoy can no longer be detected and identified.

Examples:

(D is the buoy separation distance, L is the the distance, in which the marks can be detected and identified)

D < L / 2, if high navigation accuracy or a very clear and continous buoyage is required

D < L, “normal” for fairways

D < 2 L for some purposes in the open sea, e. g. for a Ship Routeing System

If a certain buoy separation distance is desired, the proper mark has to be found or constructed. If a certain type of marks is desired, the separation distance has to be determined.

Usually, this will be an iterative process. ore than one option should be examined and assessed from the nautical, economical and technical point of view.

In general the more fairway or channel markers are there the easier and more accurate is navigation.  However, there is a saturation point where adding AtoNs does not help positioning further. Too many markers may be confusing and weaken the result.  To find this point, usually simulation and risk assessment will be necessary or at least very useful in the detailed design phase.

### Leading lines

Leading lines (lighted or unlighted) can be established if there are stretches that form a straight line. Leading lines should be established if:

- The buoyage could be affected by ice;

- There is a channel inside the fairway, which has to be used by ships with big draft;

- Strong cross currents occur (harbour entrances);

- There is a protected or special area to be marked as restricted waters, in order to provide a separation of the traffic area from the protected area (e.g. fish farms, aquaculture farms, or naval devices).

The preliminary decision in designing a leading line is to specify the segment of water to be defined by the leading line.  Generally, it is costly to build a leading line to serve a long channel, as the rear leading light must be of sufficient height to be clearly visible above the front structure.  Leading marks must also be large enough to be visible from the far end of the channel.  Both of these conditions result in increases to the required height of the rear structure marking a long channel. The structures must also be sturdy enough to support the leading mark under wind loads.  The use of other aids (fixed beacons or buoys) may reduce the overall cost of marking the waterway by reducing that portion of the channel marked by the leading line.

More Details can be found in IALA Guideline No. 1023 “For the design of leading lines”.

### Sector lights

A sector light is an aid to navigation that displays different colours and/or rhythms over designated arcs. The colour of the light provides directional information to the mariner.

A sector or a limit between two sectors may indicate a fairway, a turning point, a junction with other channels, a hazard or something else of importance for the navigator.

When a fairway is covered by a white sector, the convention to a vessel approaching the light is a green sector to starboard and a red sector to port (IALA Maritime Buoyage System colour convention for Region A, for Region B the colours are reversed).  The white sector indicates safe passage, however this does not always hold true in the entire radial length of the sector.

A sector light may indicate one or more of the following boundaries and aspects of a navigable

Waterway:

- Change of course position;

- Shoals, banks, etc.;

- An area or position (e.g. an anchorage;)

- The deepest part of a waterway;

- Position checks for floating aids;

More Details can be found in IALA Guideline No. 1041 “Sector Lights”.

### Electronic aids to navigation

AIS as applied to aids to navigation (AtoN), improves and enhances services provided to mariners as far as their vessels are equipped with AIS. The AIS applied to an AtoN can provide a better:

- Integrity of the aid, because it is able to show Off-Position and Off-Function Messages to the mariner; and

- Accuracy, because it shows the accurate position on the electronic chart or on a modern radar screen.

So, AIS applied to AtoNs should be considered for floating and fixed AtoNs if they are:

- Major aids;

- On positions with a high risk of damage or impairment by ice, current, collision.

More Detais can be found in IALA Recommendation A-126 “on the use of AIS in Marine Aids to Navigation Services”, Edition 1.3, June 2007

## Special Requirements for different user groups

### Categorisation

There are different ways to categorize the vessels.  It depends on the issues that shall be highlighted which categorisation will be applied.  For the purpose of chapter 5 of this guideline, the level of on-board navigational aids is the differentiating factor that leads to three basic categories:

- Certified commercial vessels;

- Uncertified commercial vessels;

- Pleasure craft;

Certified Commercial vessels are vessels, used for commercial activities, which operate with charts, sailing directions and which are equipped with certified on-board navigational aids such as a standard compass, radar and radio positioning equipment (in accordance with applicable regulations) to support long-range and/or low visibility navigation.  These vessels are operated by professionally trained and certified personnel. High Speed Craft are included here.

Uncertified Commercial vessels are vessels being used for commercial activities, to which the applicable regulations do not apply in terms of requirement for on-board navigational aids.  Such vessels operate with charts and are normally equipped with a compass and timepiece suitable only for short range navigation however may be equipped with navigational aids prudent to safe navigation in the vessel’s operation area.  These vessels are normally equipped with a searchlight where night-time use of reflective short range marine aids is intended or may be necessary and are operated by personnel who may or may not have a certificate and who rely to a considerable degree on local knowledge to navigate safely.

Pleasure Craft are vessels being operated for pleasure activities by the owner or under charter, rent or loan.  They operate with charts and are normally equipped with a compass and a searchlight where night-time use of short range marine aids is intended or may be necessary.

### High speed craft (HSC)

According to the definition of the International Maritime Organization (IMO) HSC are defined as:

“craft capable of maximum speed, in metres per second (m/s), equal to

or exceeding:

3.70.1667

where:

 = volume of displacement corresponding to the design waterline (m3)

excluding craft the hull of which is supported completely clear above the water surface in non-displacement mode by aerodynamic forces generated by ground effect.”

When assessing the provision of AtoN for HSC there are many factors to take into consideration.  There are responses that can be used to mitigate these concerns, and it is likely that a combination of such responses will be required to address the specific requirements in an Authority’s area of responsibility.  It was noted that some Authorities illuminate the buoy number by LED (Light Emitting Diode) with good effect. In addition, with the trend towards plastic and aluminium buoys, Authorities should take into account the particular reflective qualities of these materials and should consider the need for passive or active radar reflectors on these AtoN.

Considerations to address AtoN requirements for HSC include:

- For coastal passages, Authorities should consider shortening the period of flashing lights and increasing the flash length and the use of floodlight;

- For coastal passages and port approaches, the provision of AIS (Automatic Identification System) on buoys and beacons is considered to be most advantageous. The availability of additional information such as meteorological and hydrological data is particularly important for HSC;

- For harbours and port approaches, lights should have a multiple flash character rather than a single flash, for example, Fl (3) 15 sec rather than Fl 15 sec. for more rapid identification;

- For harbours and port approaches, Authorities may consider the use of laser lights;

- For channels without turns or bends, a form of synchronisation of lights should be considered;

- Authorities should consider the divergence of lights. (e.g. it should be noted that when in close proximity and when viewed from above, there may be difficulty in seeing an LED buoy light or beacon which is fitted with a solar panel on top.);

- In channel or inter-island navigation, indirect lighting of beacons is considered particularly effective.

More details can be found in IALA Guideline No. 1033 “On the provision of aids to navigation for different classes of vessels, including high speed craft”.

# Examples of Marking of fairways

## Rio de La Plata, Argentina

## Finland

## Seekanal Rostock

These examples can be found in the annexes 2, 3, 4 and 5.

# Procedures for determining the requirements for short-range marine aids

Procedures for determining the requirements for aids to navigation at any site are based upon four types of analysis.

## Types of analysis and associated procedures

The design methodology can be described as a step by step procedure organized in a logical sequence in order to justify the conclusion and recommendations in determining the requirements for short-range aids to navigation.  These procedures consist of the following four analyses:

### Site Analysis (refer to 5.1.1)

An analysis of users, traffic patterns, weather, possible navigational threats and existing aids to navigation if present at the site.

The physical and environmental condition of a waterway, as well as the size of the vessels using the waterway, must be considered.

### Needs Analysis (refer to 5.1.2)

An analysis of user needs for navigational markers and signals at the site to help mariners cope with navigational threats.

### Operational Analysis (5.1.3)

An analysis of outputs of various combinations of short-range aids that could be provided at the site to identify feasible means of meeting user needs.  For this analysis, simulations and GIS may be used.

### Cost-Benefit Analysis (5.1.4)

A comparative analysis of cost effective combinations of aids (cost-effectiveness analysis) to select between viable alternatives.

In general terms, the four analyses consist of the following procedures:

- Specify the navigation site to be analyzed by identifying the user’s traffic patterns and plotting the track or tracks on the chart.  Gather all user and site information such as size, type and number of vessels, type, size, output and characteristics of all established aids to navigation, weather and sea conditions, previous reviews, marine incidents, requests, etc.

- Identify and classify the size and number of users according to the three basic categories i.e. Certified commercial, Uncertified commercial and pleasure craft.

- Rate the threats at each site for each type of user, using guidelines and professional judgement.

- Identify any local means, other than short-range aids to navigation, that may meet any of the needs identified.

- For each of the threats in the waterway, identify the related navigation needs and the aid or aids which address these needs.

- Identify any threats that are not adequately reduced and the recommended aids or other means that could meet the navigational need.

- Identify any aids to navigation that are provided where the threats are not significant or service is redundant and recommend action to reduce or discontinue aids.

- Calculate the coverage and hazard marking requirements at any open water landfall situation, for each user group.

- Calculate the equipment output required to achieve the required visual, aural and radar coverage at the specified level of availability.  Identify where the requirements are not being met with established equipment and propose viable options.

- Calculate the size and output requirements for each aid to navigation established or proposed, in the confined section of the waterway, to meet the proposed availability for each user group. Identify where the proposed levels are not being met and propose viable options.

- Establish the comparative direct costs - including maintenance costs - of each proposed alternative, to assist in determining the most cost-effective system of aids to navigation.

### Site analysis

The purpose of this first analysis is to gather site specific data for the review work.  In this phase, the following steps should be followed:

- Identify and classify the size, types & number of vessels;

- Assess the volume of traffic and identify traffic patterns including navigation seasons and or trends for each users category;

- Obtain and record weather & sea conditions including tidal and current information, wind and reduced visibility (needed to determine the design visibility);

- Obtain information on the physical geography of the river, waterway or shoreline such as silting, distinctive shore features, existing charted landmarks, etc.;

- Retrieve related historical information such as previous reviews, marine incidents, experience and skills of mariners, requests for aids to navigation, etc.;

- Prepare an inventory of all aids provided by the authority at the site if applicable. This may include private aids to navigation;

- Identify and rate specific threats to identify potential hazards for each type of users. This initial rating considers each of the following threats individually:

- Minimum depth allowance (considering the vessel draft, wave height and under keel clearance and bottom/seabed structure);

- Distance from hazards and from other vessels when passing;

- Minimum channel width and angle of turn in channel; and

- Wind speed, wave height, current along and across track and restricted visibility.

### Needs analysis

Once the site analysis has been completed, the authority should proceed with the analysis of the mariners’ needs.  This is where the authority establishes user requirements for safe and accurate navigation.  It is at this stage of the process that the authority will:

- Consider the interactive effects and unique features of the site by rating the following composite threats according to a pre-determined scale:

  - Sea conditions;

  - Proximity of hazards;

  - Complexity of track/ channel;

  - Diminished room to manoeuvre;

  - Traffic density / mix / crossing;

  - Reduced visibility;

  - Darkness;

  - Ice and freezing spray;

  - Channel siltation;

  - Lack of distinctive features for positioning / pilot boarding station.

- Identify related needs, noting the most appropriate generic types of aids to reduce the threats (e.g. visual marker on relevant hazards, lighted visual marker for lateral motion, visual boundary marker, aural signal on relevant hazard, radar target, etc.) as well as rate their relative effectiveness in reducing each type of threat for each category of users.

- Note and assess to which extent local means and natural features (e.g. towers, spires, landmarks, etc.) can meet the needs.

- Note the specific aids provided (if there is an aids system already in place) and assess against the threats and/or the generic needs.

A five point scale is used for the purpose of rating composite threats. Under this scale, threats are rated as follows: 1 - Overriding Importance; 2 - Highly Significant; 3 - Significant; 4 - Little Significance; and 5 - Not Applicable.

The generic types of markers and signals that serve to reduce the various categories of composite threats have been rated in consultation with users for their effectiveness in reducing these threats for each user category.  These ratings are:

A. 	Greatly reduces the threat.

B. 	Significantly reduces the threat.

C. 	Somewhat reduces the threat or reduces the threat for some vessels in the category (e.g., radar targets will assist those uncertified commercial vessels who are equipped with radar but do not qualify as Category I vessels because of their lack of other electronic navigation aids, etc.).

### Operational analysis

Through the operational analysis, the authority is able to examine the outputs of various combinations of aids to navigation, compare the end results in relation to the needs of mariners for the specific waterway and consider the best possible option.

The operational analysis is conducted by performing a series of steps to:

- Determine the system requirements (landfall or confined waters);

- Determine  the perception requirement (coverage);

- Compare the existing aids against the calculated requirements;

- Determine the needs to meet design visibility (Design Visibility is the lowest level of visibility at which the system can be used visually);

- Examine and test options and or combination of aids to navigation;

- Recommend final optimal system design;

- When reviewing an existing system, compare optimum design with existing system making appropriate observation and/or recommendations.

To complete the above steps, the following actions are also required:

- Calculation of the coverage and hazard marking requirements (perception requirement) for each type of users ensuring that said coverage is sufficient to allow the largest user time to complete an evasive manoeuvre;

- Calculation of the equipment output required to achieve the required visual, aural and radar coverage to meet the calculated perception requirement;

- Calculation of the size and output requirements for each aid to navigation established or proposed to meet the design availability for each user group.

### Simulation and GIS

Prior to implementing a new AtoN system or changing an existing one, National Authorities can use simulation techniques to assess the overall safety and effectiveness of these changes.  The use of Geographic Information System (GIS) technology can improve efficiency of AtoN deployment and waterway layout which, with volume of traffic overlaid (e.g. taken from AIS data), can be used to assess risks and plan the disposition and type of AtoN to mitigate such risks and improve the provision of AtoN for all users.  Having planned aids to navigation in this manner, the authority can use modern simulation tools where passage and combinations of various types of vessels can be simulated, in order to conduct a validation process, in consultation with appropriate stakeholders.  To achieve a high level of realism in the simulations, GIS data can be integrated to the waterway models used in the simulator.

One of the purposes of simulation in relation to waterway design is to evaluate planning of placement and design of AtoN in a specific waterway, channel or port area.  Other purposes could be e.g. ensuring sufficient channel width, channel depth, and optimal orientation and design of breakwaters as well as ensuring that the lay-out of a channel and port is suitable seen from a manoeuvring point of view.

Simulation offers a method to help ensure that AtoN are appropriate and cost effective. Sophisticated computer simulation techniques are becoming increasingly available, and they provide an excellent mechanism for decision making as follows:

- Generating ‘what if’ scenarios – feasibility studies;

- Obtain input to placement and design in consultation with stakeholders;

- Validation of final design and lay-out;

- Training in a variety of areas.

Simulating the placement and operation of AtoN by day and night, and under various conditions of visibility assists in ensuring that AtoN are effective and provided in a cost effective manner that suits the purpose of providing a predetermined level of safety.  This is particularly important as aids to navigation become more sophisticated (synchronised and sequential lights, LED with flicker, and other new light characteristics).

Please see IALA Recommendation (O-138) for further discussion on the use of simulation in waterway design and placement of AtoN. The recommendation includes a reference to a guideline that amongst other outlines how to select the right simulation tool in relation to the objectives and expected outcome of the simulations.

### Cost-benefit analysis

The last phase of the design methodology consists of a cost-benefit analysis which is a comparative analysis of costs of the various scenarios evaluated during the previous steps.  This cost-benefit analysis should be done using values and cost factors as accurate as possible.

At the end of the process, the design methodology produces various options for aids system, various combinations of aids and associated costs ready for consultations with the affected mariners.

# Conclusion

The art of navigation has evolved over many years, as well as the properties of ships, vessels and various other craft.

In principle navigation comprises:

- Planning a track / passage for a ship by the use of a nautical chart.

- Monitoring or establishing the ships position or advance along a predetermined track / passage, and

- The control of the ship, e.g. that to ensure that it follows a predetermined track / passage.

To navigate there are a number of "tools" available:

- clock

- log

- compass

- magnetic compass

- gyro compass

- radar

- nautical charts

- paper chart

- electronic navigational charts

- visual and radar aids to navigation

- single point aids (lights and daymarks)

- floating aids

- sector lights

- leading lights

- sound signals

- ranges

- racons and radar reflectors

- AIS as AtoN

- flood lightning of conspicuous objects

- Global Navigation Satellite Systems (GNSS)

- local and regional electronic navigation systems

- auto pilot

- shore based (radar)assistance

- pilotage

Of these "tools" some can be alternate to another tool while some can be complementary to another tool.  That is the tools can be used in various numbers and combinations. Subsequently principally a primary and secondary method of navigation is often used.  Depending on the situation the primary method could be either a visual/radar based method or a GNSS based method.

The appropriate nautical chart is of fundamental importance to find a safe track/passage and later as a reference during the ships movement. It is also fundamental to have a compass, clock and log to be able to follow and monitor the advance along the track.  To establish a position visual or radar aids can be used either independently or in combination, although for instance a Global Navigation Satellite System can also be used.

The navigation process is normally performed by the navigator integrating the chart, navigational data and the control of the ship.  In this case visual and radar aids normally play an important role. On some ships this process is fully automated, relying completely on electronic/digital "tools."  This put a demand on accuracy on the chart, the navigation system and the control system, that cannot be fulfilled under all circumstances today.  In general it is considered that proper marking of waterways/fairways, dredged channels and canals by visual and radar aids is important to reduce the risk posed by the waterway and the traffic.

# References

- Tbd.

- Desired Positioning Tolerance of Floating A to N

Desired Positioning Tolerance (DPT) is the radius of a circle that describes the maximum allowed error for positioning the aid.  It is based on the navigational needs of the mariner and the service that the aid provides.

The first consideration is the Waterway Risk Level as determined through analysis of different factors as shown detailed in section 5.  For this method of determining the DPT three waterway risk levels are used: great, moderate and low.

The waterway risk level is based on the type and volume of traffic prevalent at the aid location, the environmental sensitivity of the water way, and the bottom type (hard or soft), and the width of the channel compared to the beam of the average vessel that will transit the waterway.  The Width to Beam (W/B) ratio relates the size and manoeuvrability of the vessel to the physical size of the waterway.

Great Risk:

- Traffic Density.  The amount of vessel traffic is substantial. Vessel congestion can be a problem;

- Type of Traffic.  Vessel traffic consists of various types including large commercial vessels.  Vessels containing hazardous cargo whose grounding and subsequent discharge would pose a substantial threat to public health or welfare, or result in critical public concern;

- Environment.  The waterway has environmentally sensitive areas and endangered species areas;

- Waterway Conditions.  Wind, wave, tide and current, water depth;

- Bottom Conditions.  Hard bottom conditions (rock, coral, wreck, etc.) exist where the likelihood of holing a vessel is great;

- W/B Ratio.  Less than 3 in one-way channels or less than 5 in two-way channels.

Moderate Risk.

- Traffic Density.  The amount of vessel traffic is moderate. Vessel congestion is not a problem under normal waterway conditions;

- Type of Traffic.  Vessel traffic consists of commercial fishing fleets and smaller commercial vessels carrying non-hazardous cargo;

- Protection of Environment.  The waterway has little effect on environmentally sensitive areas;

- Waterway Conditions.  Wind, wave, tide and current, water depth;

- Bottom conditions are such that a possibility of holing a vessel exists if a grounding occurs;

- W/B Ratio.  Between 3-5 in one-way channels or between 5-8 in two-way channels.

Low Risk.

- Traffic Density.  The amount of vessel traffic is light. Little or no vessel congestion exists;

- Type of Traffic.  Vessel traffic consists of small pleasure craft, fishing boats and small commercial vessels carrying non-hazardous cargo;

- Environment. A marine casualty on the waterway will have little or no effect on environmentally;

- Bottom Conditions. A soft "forgiving" bottom makes the likelihood of holing a vessel remote if a grounding occurs;

- W/B Ratio: Greater than 5 in one-way channels or greater than 8 in two-way channels.

The second consideration is the Geographic Area Type. There are three area types:

- Area Type 1: Area Type 1 is a narrow or restricted waterway such as channels, rivers, canals and straits;

- Area Type 2: Area Type 2 has more vessel manoeuvring room but may still be restrictive such as harbours and coves;

- Area Type 3: Area Type 3 has little restriction on the manoeuvrability of vessels which transit the area such as coastal regions and fairways.

After these considerations the DPT can be determined in three steps:

- Locate the Area Type;

- Compare the waterway risk level;

- Use the following table:

- Desired Positioning Tolerance

| Desired Positioning Tolerance (DPT) shown in meters |  |  |  |
| --- | --- | --- | --- |
|  | Great Risk | Moderate Risk | Low Risk |
| Area Type 1 | 10 | 25 | 50 |
| Area Type 2 | 25 | 50 | 75 |
| Area Type 3 | 50 | 75 | 100 |

- A to N design for the Rio de La Plata Navigation Channel

The Rio de La Plata Navigation Channel is a partly straight and partly curved channel with buoys from a position of 129.1 miles seawards of the harbour entrance of Buenos Aires.

More data can be seen on the following chart.

Some basic data:

Waterway in analysis

Length:	63.8 M

Width:	100 m (one way channel)

(there is a  two way channel sector of 160 m)

Design Depth:	34 feet

Tidal range	0,60 m

The Vessel Traffic

Total Traffic in the Area in 2006:	7.760  vessels

including

Containerships:	1.726

Bulk Carriers:	3.134

General cargo:	962

Oil Tanker:	1.386

Other:	552

Vessels Dimensions. Maximum size of:

Length BP	Beam	Design Draught

Containership 	261 m	40 m	41 feet

Bulkcarrier	260 m	42 m	42 feet

Tanker	241 m	32,2 m	49 feet

Aids to Navigation

The buoyage system is designed as “paired buoys” with the following parameters:

Buoy separation distance	average 3000 m  (max 5.000 m, min 1.100 m)

Buoy types	Floating buoys	 and Spar Buoys

Floating Buoy size above water level	4 m

Spar Buoys size above water level:	8 m

Additionally there are the following AtoNs:

DGPS coverage (on demand- private service)

VTS and AIS control by Prefectura Naval Argentina (National Coast Guard)

Mandatory Pilotage services

- VTS Coverage – Pilotage Services

- Finland

- A to N design for the “Seekanal Rostock”

The Seekanal Rostock is a straight channel with buoys from a position 6 miles seawards of the harbour entrance of Rostock/ Warnemünde leading into the mouth of the river Warnow with following design data:

Waterway

Outer part:

Length:	5.7 M

Width:	225 … 120 m

Depth:	>= 14,5 m

Inner part:

Length:	1.5 M

Width:	120 m

Depth:	>= 14,5 m

Vessel Traffic

Total Traffic in the Area in 2006:	25.200 vessels

including

Passenger- and Ro/Ro Ferries:	13.000

Cargo vessels:	4.500

Oil Tanker: 	1.800

The area is also one of the major places for pleasure and fishing craft in the region.

Vessels Dimensions

Terms for two lane traffic passing 120 m width area:

- Beam	< 40 m – sum of both passing vessels and

Draught	< 8.5 m

- If wind < 6 Bft and both captains accept a two lane traffic passing and

Beam	< 60 m – sum of both passing vessels beams and

Draught	< 8.5 m

- vessel with draft > 8,5 m and using the leading line

X = 60 m – 0.5 * beam of this vessel

MAX beam of the meeting vessel X/5 (opposite direction)

All other vessels have to pass by “one lane traffic” under responsibility of the captain with assistance of VTS.

Maximum size of  vessel in “Seekanal Rostock” :

Length	296 m

Beam	  32 m

Draft	    8.5 m

The channel is narrow in relation to the vessels dimensions.

Aids to Navigation:

The buoyage system is designed as “paired buoys” with the following parameters:

Buoy separation distance	Outer Part: 1500 … 1000 m

Inner Part:   600 m

Buoy type	Outer Part: deep water light buoy “LT 81”, steel mooring chains

Inner Part: hinge beacon with steel tube, with a minimal swinging circle radius, synchronised lights

Buoy size above water level	Outer Part: 2,5 x 5 m

Inner Part: 1 x 4 m

SC Tonne

EWF Bake

Middle Line Fairway

Canal section

EWF Bake

SC Tonne

- Canal Section with two types of floating aids: Deep Water buoy and Hinge Beacon

Notes

EWF = effective width of fairway

SC = swinging circle

The inner part of the canal is designed with Hinge Beacons because they have a smaller swinging circle and thus cause less reduction of the effective width of fairway.

Additionally there are the following AtoNs:

- Leading Light over 4 miles, DGPS coverage, VTS coverage, AIS coverage

- The Leading Lights are synchronized (same character for front light and rear light).

To avoid misleading information in case of failure of one of the lights by mirroring a light with the same character on the water surface and be interpreted as a front light in the wrong place, in case of failure of either the front light or the rear light the other one will also be switched off.  This is ensured by connecting the remote control devices of the front light and the rear light.

- AtoN scheme of the inner part of “Seekanal Rostock”

The Leading Lights are synchronized with the mole lights and the buoys as shown in Table 4.

- Title required

|  | 1s | 2s | 3s | 4s | Flash code |
| --- | --- | --- | --- | --- | --- |
| Leading lights | X | On | On | On | Int. 4s |
| Mole lights at canal entrance | X | X | On | On | Iso. 4s |
| Articulated lights on buoys | X | X | X | On | Fl. 4s |

- Sea Chart of a part of the outer and inner channel “Seekanal Rostock”: