ARM3	Working paper

Agenda item

Task Number

Author(s)

# Use of Pictogram on Aids to Navigation

# summary

As Special Marks defined in MBS as “Special Marks are used to indicate a special area or feature may be apparent from reference to a chart or other nautical publication.” There has been a proliferation in the use of Special Marks in some regions as an ‘all-purpose’ aid, and it is desirable to distinguish the different uses of Special Marks. IALA pictogram rule to define its multi functions is needed.

## Purpose of the document

To invite the Committee to discuss the needs of IALA rules on this matter, share the experiences and develop guidance on use of pictogram on Aids to Navigation.

## Related documents

IALA MBS Chapter 6. Special marks

# BACkGROUND

Special Marks are defined in MBS as “Marks used to indicate a special area or feature whose nature may be apparent from reference to a chart or other nautical publication”

There has been a proliferation in the use of Special Marks in some regions as an ‘all-purpose’ aid, and it is desirable to distinguish the different uses of Special Marks.

Special Marks are no longer special as originally intended. The MBS defines a number of different uses of Special Marks for which it is convenient, and the proliferation of its use is consequently what can be expected. Its yellow colour is conspicuous at sea and contributes to its versatility.

# Discussion

## Needs for IALA rules

When Special Marks are used for different purposes adjacent to each other it may be difficult for mariners to distinguish between them. And when they are used with different rules from the members, it will confuse the mariners.

## Use of pictogram

### IALA MBS

Special Marks are not generally intended to mark channels or obstructions where other marks are more suitable.

Some examples of uses of Special Marks are;

Ocean Data Acquisition Systems (ODAS) marks.

Traffic separation marks where use of conventional channel marking may cause confusion

Spoil Ground marks

Military exercise zone marks

Cable or pipeline marks

Recreation zone marks

Boundaries of anchorage areas

Structures such as offshore renewable energy installations

Aquaculture

Location of pictogram

Description of Special Marks

| Description |  |
| --- | --- |
| Colour | Yellow |
| Shape of buoy | Optional, but not conflicting with lateral marks |
| Top-mark (if any) | Single yellow “X” shape |
| Light (when fitted) |  |
| Colour | Yellow |
| Rhythm | Any, other than those reserved for cardinal, isolated danger and safe water marks. |
| Pictogram | The use of pictograms is authorized, as defined by a competent authority. |

# Input from members

## Korea

Incheon port area

Incheon port(chart 0306)

Special Marks in Incheon port

# pictogram

## Examples

Incheon port area

Sports

Pictogram example

Inland waterway transport

# References

ANM19-WG2-WP1 MBS issues

# Action requested of the Committee

The Committee is requested to:

- Note of the needs for IALA rule for use of pictogram on AtoN

- Share the experience and cases on pictogram

- Develop guidance on uses of pictogram on AtoN