Input paper:   DTEC3-5.2.3.4

Input paper for the following Committee(s): 		Purpose of paper:

(Select as appropriate)

☐ ARM	☐  ENG		☐ PAP			☐ Input

☒ DTEC	☐ VTS					☒ Information

Agenda item 			5.2

Technical domain/ Task number 	6.3.10

Author(s)/Submitter(s)			China MSA

# VDES sat-ship application tests

# Summary

In the context of the rapid development of e-Navigation and intelligent shipping, the applications of satellite VDES system is increasingly visible. VDES provides a solution to current AIS overloading problem and alleviates AIS data communication pressure to improve maritime communication quality. Satellite VDES is seen as an additional way to supplement shore-ship information exchange in the e-Navigation data communication. China Maritime Safety Administration (MSA) has carried out a number of satellite-ship VDES application scenario tests in multiple sea areas with multiple VDES satellites to verify the application capabilities of VDES satellites.

## Purpose of the document

The VDES terminal was on-board the ship during the sea navigation tests, during which the ship's positioning data was transmitted back through the VDES satellites; the hydrological information was broadcast to the ship via satellites and the recommended route information was broadcast to the ship on demand. The satellite-ground data link, communication protocol and application capability of VDES system in some application scenarios in IALA G1117 guideline were verified.

## Related documents

[1] IALA COMMITTEE WORK PROGRAMME 2023-2027

[2] IALA G1117 VHF Data Exchange System (VDES) Overview Edition 3.0 December 2022

[3] ITU-R M.2092-1, Technical characteristics for a VHF data exchange system in the VHF maritime mobile band, February 2022

[4] ITU-R M.1371-5, Technical characteristics for an automatic identification system using time division multiple access in the VHF maritime mobile frequency band, February 2014

# Background

The satellite VDES system supports real-time navigation and data communication, which can improve maritime traffic safety and information services on a global scale and meet the needs of e-Navigation and intelligent shipping for maritime data communication. Up to now, several satellite operators have launched VDES satellites and carried out tests. China MSA has conducted a series of tests based on a China's current VDES constellation to verify the application capabilities of VDES satellites in various scenarios.

# DISCUSSION

China MSA, together with VDES application partners, has completed real ship testing of multiple satellite VDES system application scenarios, which includes uplink and downlink communication link test, satellite-ground system communication protocols test and specific business application test.

## Data communication link test

### routes and frequency used in the tests

The data link tests include LinkID21 uplink information reporting at frequency 157.3125MHz, with a bandwidth of 50KHz and LinkID33 downlink information broadcasting at frequency 161.9125MHz, with a bandwidth of 50KHz. The tests were done following Sankulilang port - Zhenjiang route (May 8th,2024 to May 20th, 2024) and the Rizhao - Tianjin route (June 16th, 2024 to June 20th, 2024).

### satellites in the sat-ship test

The orbit height and inclination of the satellites are listed below in Table 1.

Table1 Satellites table

| Satellites | Orbit height | Orbit inclination |
| --- | --- | --- |
| HEAD-2E | 500km | 98° |
| HEAD-2F | 500km | 98° |
| HEAD-2H | 500km | 98° |
| HEAD-3A | 700km | 45° |
| HEAD-3B | 700km | 45° |
| HEAD-3C | 700km | 45° |
| HEAD-3D | 700km | 45° |

### SAT-SHIP TEST WORKFLOW

The VDES SAT-SHIP test workflow is shown in Figure 1.

Figure 1 on-orbit test flowchart

### ship-borne devices installation

Some VDES terminal and antenna installation pics are shown below:

(a)VDES terminal installation                 (b) Antenna installation scene

Figure 2 VDES terminal and antenna installation scenes

### data link test results

- Sankulilang Port - Zhenjiang route

The navigation track during the test of Sankulilang Port - Zhenjiang route is shown in Figure 3.

Figure 3 Sankulilang Port - Zhenjiang route

A single satellite route coverage map over Sankulilang Port - Zhenjiang route is shown in Figure 4.

Figure 4 Satellite coverage (15:51:45 May 13th, 2024)

The navigation positioning data excerpts on the VDES terminal backhauled by the satellites are summarized in Table 2.

Table 2 Uplink positioning data received by the satellite

| Satellite name | Date | Time |
| --- | --- | --- |
| HEAD-3A | 2024-05-07 | 04:47:13~04:49:54 |
| HEAD-3A | 2024-05-08 | 05:30:49~05:34:54 |
| HEAD-3B | 2024-05-13 | 01:44:33~01:49:49 |
| HEAD-3B | 2024-05-14 | 02:21:33~02:27:26 |
| HEAD-3A | 2024-05-14 | 02:56:30~02:57:54 |

2) Rizhao-Tianjin Route

The navigation route from Rizhao to Tianjin is shown as Figure 5.

Figure 5 Tianjin-Rizhao Route

A single satellite route coverage map over Rizhao-Tianjin Route is shown in Figure 6.

Figure 6 Satellite coverage (5:34:27 June 20th,2024)

Part of the satellite downlink broadcast data is shown in Table 4.

Table 4 Downlink broadcast data received

| Satellite name | Date | Time |
| --- | --- | --- |
| HEAD-2H | 2024-06-20 | 03:22:42~03:26:02 |
| HEAD-3D | 2024-06-20 | 05:32:00~05:37:41 |

## Communication protocol tests

### SHIP TO SAT ADDRESSED MESSAGES

1. Test steps:

(a) VDES terminal received 1#SBB at slots 14, 29, and 44; slot 104 received #10MAC;

(b) After receiving the above packets, the terminal generated its own data, obtained a random time slot in the RAC channel of the same VDE subframe, and sent the #20 resource request packet;

(c) After receiving the resource request message, the satellite will determine whether there is available resource allocation. If there is available resource allocation, the satellite will broadcast the #12 resource allocation message in the nearest ASC channel.

(d) After receiving the resource allocation packet, the terminal will uplink the #30, #31, and #32 packets data in the assigned DC logical channel.

Ship to SAT addressed message is shown in Figure 7.

Figure 7 Ship to SAT addressed message

2. Test results:

(a)Ship sends resource request#20 to SAT；

(b)After receiving resource request from a ship, SAT sends resource configuration #12;

The ship received the red circled message in the resource allocation #12 message sent by the satellite. Its decoding is shown in Table 5.

Table 5 The red-marked resource allocation #12 message description received by the ship

| Field number | Size （Byte） | Function | Content |
| --- | --- | --- | --- |
| 1 | 1 | Type | Type = 12 (0X0C) |
| 2 | 2 | Data payload size | Field size, ranging from 3 to 22. Data payload size = 32 (0X00 0X20, little-endian) |
| 3 | 4 | Ship station ID | The Unique Identifier of the ship station, as described in § 2.4. 0 for broadcast. (0X78 0X56 0X34 0X0D, little-endian) |
| 4 | 1 | Logical Channel 1 | Allocates logical channels for data transmission. Applicable to data time slots only. LC of 255 indicates no resource. (0X02) |
| 5 | 1 | Link ID 1 | Link ID that should be used in Logical Channel 1. Transmission direction can be inferred from link ID. Here it used Link ID 21 (0X15) |
| 6 | 1 | Session ID 1 | The Session ID associates with satellite allocation, ranging from 1 to 255 0 indicates short message (0X02) |
| Field number | Size （Byte） | Function | Content |
| 7 | 1 | Uplink CQI 1 | Received CQI is defined in Annex2, § 1.2.8 in ITU-R M.2092. (0X02) |
| 8 | 4 | Ship station ID 2 | The Unique Identifier of the ship station, as described in § 2.4. 0 for broadcast. |
| 9 | 1 | Logical channel 2 | Logical Channel assigned for data transmission. Only applies to data slots. LC of 255 indicates no resource. |
| 10 | 1 | Link ID 2 | Link ID that should be used in Logical Channel 2. Transmission direction can be inferred from link ID. |
| 11 | 1 | Session ID 2 | Satellite assigned session ID |
| 12 | 1 | Uplink CQI2 | Received CQI is defined in Annex2, § 1.2.8 in ITU-R M.2092. |
| 13 | 4 | Ship station ID 3 | The Unique Identifier of the ship station. See in § 2.4, Annex 1 in ITU-R M.2092. 0 indicates broadcast. |
| 14 | 1 | Logical channel 3 | Logical Channel assigned for data transmission. Only applies to data slots. LC of 255 indicates no resource. |
| 15 | 1 | Link ID 3 | Link ID that should be used in Logical Channel 3. Transmission direction can be inferred from link ID. |
| 16 | 1 | Session ID 3 | Assigned session ID. |
| 17 | 1 | Uplink CQI 3 | Received CQI is defined in Annex2, § 1.2.8 in ITU-R M.2092 |
| 18 | 4 | Ship station ID 4 | The Unique Identifier of the ship station, as described in § 2.4, Annex 1. 0 for broadcast. |
| 19 | 1 | Logical channel 4 | Logical Channel assigned for data transmission. Only applies to data slots. LC of 255 indicates no resource. |
| 20 | 1 | Link ID 4 | Link ID that should be used in Logical Channel 4. Transmission direction can be inferred from link ID. |
| 21 | 1 | Session ID 4 | Assigned session ID. |
| 22 | 1 | Uplink CQI 4 | Received CQI is defined in Annex2, § 1.2.8 in ITU-R M.2092 |

(c)Ship station sends start segment #30 to satellite;

(d)Ship sends continue segment #31 to satellite；

(e)Ship sends end segment#32 to satellite.

Test results are consistent with ITU-R M.2092-1

### SAT SENDS ADDRESSED MESSAGE TO SHIP STATION WITH ACKNOWLEDGEMENT

- Test steps:

(a)Terminal receives 1# SBB at slots14, 29, and 44; slot 104 receives #10 MAC;

(b) #12 Resource allocation message is received at ASC, where ship station ID is the unique identifier for the ship and LC is assigned for data transmission, not 0Xff.Session ID is not 0xFF;

(c)Terminal receives #30，#31，#32 messages at assigned DC logical channel;

(d)After receiving the #32 packet, the terminal sends the #29 downlink data reply packet in the DC logical channel reply slot assigned in the nearest DSCH channel.

Satellite to ship addressed message is shown as Figure 8.

Figure 8 Satellite to ship addressed message

- Test results:

(a)Satellite sends resource allocation #12 to ship；

The ship received the red-marked message in the resource allocation #12 message sent by the satellite, and the decoding is shown in Table 6:

Table 6 Description of red-marked message in the resource allocation #12 message sent by the satellite

| Field Number | Size (Byte) | Function | Content |
| --- | --- | --- | --- |
| 1 | 1 | Type | Type = 12 (0X0C) |
| 2 | 2 | Data payload size | Field size, ranging from 3 to 22. Data payload size = 32 (0X00 0X20, little-endian) |
| 3 | 4 | Ship station ID | The Unique Identifier of the ship station, as described in § 2.4. 0 for broadcast. (0X00 0X00 0X00 0X01, little-endian) |
| Field Number | Size (Byte) | Function | Content |
| 4 | 1 | Logical channel 1 | Allocates logical channels for data transmission. Applicable to data time slots only. LC of 255 indicates no resource. (0X02) |
| 5 | 1 | Link ID 1 | Link ID that should be used in Logical Channel 1. Transmission direction can be inferred from link ID. Here it used Link ID 33. (0X21) |
| 6 | 1 | Session ID 1 | The Session ID associates with satellite allocation, ranging from 1 to 255. 0 indicates short message. (0X02) |
| 7 | 1 | Uplink CQI 1 | Received CQI is defined in Annex2, § 1.2.8 in ITU-R M.2092. (0X02) |
| 8 | 4 | Ship station ID 2 | The Unique Identifier of the ship station, as described in § 2.4. 0 for broadcast. |
| 9 | 1 | Logical channel 2 | Allocates logical channels for data transmission. Applicable to data time slots only. LC of 255 indicates no resource. |
| 10 | 1 | Link ID 2 | Link ID that should be used in Logical Channel 2. Transmission direction can be inferred from link ID. |
| 11 | 1 | Session ID 2 | Satellite assigned session ID |
| 12 | 1 | Uplink channel CQI 2 | Received CQI is defined in Annex2, § 1.2.8 in ITU-R M.2092. |
| 13 | 4 | Ship station ID 3 | The Unique Identifier of the ship station, as described in § 2.4. 0 for broadcast. |
| 14 | 1 | Logical channel 3 | Allocates logical channels for data transmission. Applicable to data time slots only. LC of 255 indicates no resource. |
| 15 | 1 | Link ID 3 | Link ID that should be used in Logical Channel 3. Transmission direction can be inferred from link ID. |
| 16 | 1 | Session ID 3 | Satellite assigned session ID |
| 17 | 1 | Uplink CQI 3 | Received CQI is defined in Annex2, § 1.2.8 in ITU-R M.2092. |
| 18 | 4 | Ship station ID 4 | The Unique Identifier of the ship station, as described in § 2.4. 0 for broadcast. |
| 19 | 1 | Logical channel 4 | Allocates logical channels for data transmission. Applicable to data time slots only. LC of 255 indicates no resource. |
| 20 | 1 | Link ID 4 | Link ID that should be used in Logical Channel 4. Transmission direction can be inferred from link ID. |
| 21 | 1 | Session ID 4 | Satellite assigned session ID |
| 22 | 1 | Uplink CQI 4 | Received CQI is defined in Annex2, § 1.2.8 in ITU-R M.2092. |

(b)Satellite sends #30 message to ship;

The red-marked message in the start message #30 received by the ship from the satellite is interpreted as shown in Table 7:

Table 7 The red-marked message in the start message #30 received by the ship from the satellite

| Field number | Size (Byte) | Function | Content |
| --- | --- | --- | --- |
| 1 | 1 | Type | Type = 30 (0X1E) |
| 2 | 2 | Data payload size | Field size, ranging from 3 to 9. Data payload size = (0X02 0X10, little-endian) |
| 3 | 4 | Source station ID | The Unique Identifier of the ship station, as described in § 2.4, Annex 1.(0X78 0X56 0X34 0X0D, little-endian) |
| 4 | 1 | Satellite ID | Satellite ID (0X00) |
| 5 | 1 | Session ID | 1~255 (0X02) |
| 6 | 4 | Destination ID | The unique identifier of the receiving station, as described in § 2.4, Annex 1. (0X00 0X00 0X00 0X01, little-endian) |
| 7 | 2 | Fragment number | Fragment number of the payload in this VDE-SAT data session. First fragment should start at 0, increment with any additional fragment and wrap at 65535. (0X00 0X00, little-endian) |
| 8 | Variable | Payload | (0X55 0X55............, little-endian) |
| Note: Used for uplink and downlink data. The data transfer direction can be inferred from the used link ID. For uplink data, the source station is the ship station; for downlink data, the destination station is the ship station. |  |  |  |

(c)Satellite sends #31 message to ship;

The red-marked message in the start message #31 received by the ship from the satellite is analyzed as shown in Table 8:

Table 8: Star message #31 received by the ship from the satellite description

| Field number | Size (Bytes) | Function | Content |
| --- | --- | --- | --- |
| 1 | 1 | Type | Type = 31 (0x1F) |
| 2 | 2 | Data payload size | Field size, ranging from 3 to 8. Data payload size = (0X02 0X10, little-endian) |
| 3 | 4 | Source station ID | The Unique Identifier of the ship station, as described in § 2.4, Annex 1 (0X78 0X56 0X34 0X0D, little-endian) |
| 4 | 1 | Satellite ID | Satellite ID (0X00) |
| 5 | 1 | Session ID | 1~255 (0X02) |
| 6 | 4 | Destination ID | The unique identifier of the receiving station, as described in § 2.4, Annex 1. (0X00 0X00 0X00 0X01, little-endian) |
| 7 | 2 | Fragment number | Fragment number of the payload in this VDE-SAT data session. First fragment should start at 0, increment with any additional fragment and wrap at 65535. (0X00 0X01, little-endian) |
| 8 | Variable | Payload | (0X55 0X55............, little-endian) |

(d)Satellite sends #32 to ship;

The red-marked message in the message #32 received by the ship from the satellite is described as shown in Table 9.

Table 9 The red-marked message in the message #32 received by the ship from the satellite

| Field number | Size (Bytes) | Function | Content |
| --- | --- | --- | --- |
| 1 | 1 | Type | Type = 32 (0X20) |
| 2 | 2 | Data payload size | Field size, ranging from 3 to 8. Data payload size = (0X01 0X3D, little-endian) |
| 3 | 4 | Source station ID | The Unique Identifier of the ship station, as described in § 2.4, Annex 1 (0X78 0X56 0X34 0X0D, little-endian) |
| 4 | 1 | Satellite ID | Satellite ID (0X00) |
| 5 | 1 | Session ID | 1~255 (0X02) |
| 6 | 4 | Destination ID | The unique identifier of the receiving station, as described in § 2.4, Annex 1. (0X00 0X00 0X00 0X01, little-endian) |
| 7 | 2 | Fragment number | Fragment number of the payload in this VDE-SAT data session. First fragment should start at 0, increment with any additional fragment and wrap at 65535. (0X00 0X0C, little-endian) |
| 8 | Variable | Payload | (0X55 0X55............, little-endian) |

(e)Ship sends feedback message #29 to satellite.

Test results are consistent with ITU-R M.2092-1.

### Satellite sends broadcast message to ship

- Test steps：

(a) Terminal receives 1# SBB in slot 14, 29, 44; Slot 104 receives #10 MAC

(b) The ASC receives a #12 resource allocation packet. In the resource allocation packet, the ship station ID is 0, LC is not 0xFF, and is the specified DC logical channel number. LinkID is the downlink LinkID assigned by the satellite, and the session ID is 0xFF.

(c) Packets #30, #31, and #32 are received in the DC logical channel specified in resource allocation. Figure 9 shows the example flow:

Figure 9 Satellite originated broadcast

- Test results

(a)Satellite sends resource allocation #12 message to ship;

The ship received the red-marked part in the resource allocation #12 message sent by the satellite, which is described as shown in the following table:

Table 10: The red-marked part description in the resource allocation #12 message sent by the satellite to the ship

| Serial number | Size (Bytes) | Function | Content |
| --- | --- | --- | --- |
| 1 | 1 | Type | Type = 12 (0X0C) |
| 2 | 2 | Data payload size | Field size, ranging from 3 to 22 Data payload size = 32 (0X00 0X20, little-endian) |
| Serial number | Size (Bytes) | Function | Content |
| 3 | 4 | Ship station ID | The Unique Identifier of the ship station, as described in § 2.4. 0 for broadcast. (0X78 0X56 0X34 0X0D, little-endian) |
| 4 | 1 | Logical Channel 1 | Allocates logical channels for data transmission. Applicable to data time slots only. LC of 255 indicates no resource. (0X02) |
| 5 | 1 | Link ID 1 | Link ID that should be used in Logical Channel 1. Transmission direction can be inferred from link ID. Here it used Link ID 21. (0X15) |
| 6 | 1 | Session ID 1 | The Session ID associates with satellite allocation, ranging from 1 to 255. 0 indicates short message. (0X02) |
| 7 | 1 | Uplink CQI 1 | Received CQI is defined in Annex2, § 1.2.8 in ITU-R M.2092. (0X02) |
| 8 | 4 | Ship station ID 2 | The Unique Identifier of the ship station, as described in § 2.4. 0 for broadcast. |
| 9 | 1 | Logical Channel 2 | Allocates logical channels for data transmission. Applicable to data time slots only. LC of 255 indicates no resource. |
| 10 | 1 | Link ID 2 | Link ID that should be used in Logical Channel 2. Transmission direction can be inferred from link ID. |
| 11 | 1 | Session ID 2 | Satellite assigned session ID |
| 12 | 1 | Uplink CQI 2 | Received CQI is defined in Annex2, § 1.2.8 in ITU-R M.2092. |
| 13 | 4 | Ship station ID 3 | The Unique Identifier of the ship station, as described in § 2.4. 0 for broadcast. |
| 14 | 1 | Logical channel 3 | Allocates logical channels for data transmission. Applicable to data time slots only. LC of 255 indicates no resource. |
| 15 | 1 | Link ID 3 | Link ID that should be used in Logical Channel 3. Transmission direction can be inferred from link ID. |
| 16 | 1 | Session ID 3 | Satellite assigned session ID |
| 17 | 1 | Uplink CQI 3 | Received CQI is defined in Annex2, § 1.2.8 in ITU-R M.2092. |
| 18 | 4 | Ship station ID 4 | The Unique Identifier of the ship station, as described in § 2.4. 0 for broadcast. |
| 19 | 1 | Logical Channel 4 | Allocates logical channels for data transmission. Applicable to data time slots only. LC of 255 indicates no resource. |
| 20 | 1 | Link ID 4 | Link ID that should be used in Logical Channel 4. Transmission direction can be inferred from link ID. |
| 21 | 1 | Session ID 4 | Satellite assigned session ID |
| 22 | 1 | Uplink CQI 4 | Received CQI is defined in Annex2, § 1.2.8 in ITU-R M.2092. |

(b)Satellite sends #30 message start fragment to ship；

(c)Satellite sends #31 continue fragment to ship;

(d)Satellite sends #32 end fragment to ship.

Test results are consistent with ITU-R M.2092-1.VDES application tests.

## Application scenarios test

We tested two application scenarios, i.e. marine environmental data broadcast and navigation exchange request and response using VDES satellites.

### MARINE ENVIRONMENT DATA BROADCAST AND TRANSMISSION (link id 33)

A total of 30kb of environmental data collected in Bohai Bay was tasked to ground control station. The ground station then uploaded the data to the VDES satellites, and the satellites broadcast the marine environmental data to the VDES terminal, as shown in Figure 10:

Figure 10 Marine environmental data receival and broadcast test diagram

- Test process

The 30kb (62 packets) data was divided into 3 segments, marked as data segment 1 (28 packets), data segment 2 (28 packets), and data segment 3 (6 packets).

Each minute is divided into three 20s, and data segment 1 (or data segment 2/ data segment 3) is repeatedly broadcast within each 20s. The duration of data broadcasting is 3 min in total, i.e., 30k bytes data has been repeated for 3 times, as shown in Figure 9:

Figure 11 Marine environment broadcast test workflow

- Test result

All marine environment data were received.

### Route exchange request and response service test

The ship sends the route request to the satellite, and the satellite downlinks its request to the ground T&C station. The ground T&C station renders the optimal route design by careful calculation and comparison, and ultimately sends the favorable route design to the satellite. The ship then receives the route design via VDES downlink broadcast channel. The workflow is shown in Figure 12:

Figure 12 Route exchange request and response service workflow

- Test flow

- Simulate navigation request data and send such data regularly to the satellite via VDES terminal. The navigation request data model is shown in below:

// Occupies 16 bits

test_data.vpfi = 8; // Fixed=8

// Occupies 16 bits

test_data.message_id = 0; // Fixed=0

// Occupies 5 bits

// 0 = Not available = default

// 1 = Compulsory route，2 = Recommended route，3 = alternative route

// 4 = Ice zone recommended route, 5 = Vessel nav route plan, 6 - 31（reserved for future use）

test_data.line_type = 2;

// occupies 4 bits

// 0~3 bit：month range 1~12（13~15, not used）

test_data.start_mon = 6;

// Occupies 5 bits

// 4~8 bit：date range 1~31（32, not used）

test_data.start_day = 27;

// occupies5 bits

// 9~13 bit hour range 0~23（24~31, not used）

test_data.start_hour = 16;

// Occupies 6 bit

// 14~19 bit：minute range 0~59（60~63, not used）

test_data.start_min = 56;

// Occupies 18 bit

// Nav route valid duration time

// Unit by minute

test_data.continue_times = 187;

// Start latitude and longitude N39.002298,E117.694582

// End latitude and longitude N38.359375,E117.925143.

Corresponding HEX data:

00080000136E1C0017686B0D605944E810DE8C20AF98645555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555

(b) In the navigation response service, the returned route data is tasked to the satellite through the ground T&C station. The satellite broadcasts the route data to the VDES terminal. The specific response route data is shown as follows:

Route original data:

// Occupies 16 bit

line_change.vpfi = 8; // Fixed=8

// Occupies 16 bit

line_change.message_id = 1; // Fixed=01

// Ocupies 3 bits

// 0 = Ship side = default

// 1 = ADM

// 2 - 7 (reserved for future use)

line_change.tx_type = 1;

// Occupies 5 bits

// 0 = Not available = default

// 1 = Compulsory route，2 = recommended route，3 = alternative route

// 4 = ice zone recommended route, 5 = vessel route plan, 6 – 31(reserved for future use)

line_change.line_type = 2;

// Occupies 4 bit

// 0~3 bit：month range 1~12（13~15, not used）

line_change.start_mon = 6;

// occupies 5 bit

// 4~8bit：date range 1~31（32, not used ）

line_change.start_day = 27;

// Occupies 5 bit

// 9~13bit：hour range 0~23(24~31, not used）

line_change.start_hour = 16;

// Occupies 6 bit

// 14~19bit：minute range 0~59（60~63, not used）

line_change.start_min = 56;

// Occupies 18 bits

// route valid duration time

// unit by minute

line_change.continue_times = 187;

- Test result:

The terminal can exactly broadcast the uplink route request information and receive the downlink route response data, and the final route information broadcast by the satellite is as follows:

00080001226DC38002EED3D24E361C048599AE606B0EAC0B0E19B0D29FCA15D1AD51A09A202B7E5DC34130BC5667CF06826178A8BD220D04C2F144AE001A09A2026D714E33C88F04F6CB886667508A669870CBA4AE154F7D219BF1282BA39C4345D7D559E562C6B0EAC0B748C40D8710BD720E1E1BA30E82ED6B1237DB0965D187406C38798BAC5E18D870F3176B56B1B335CC2ED6AD63666B985DAD5AC6CCD730BB5AB58D99AE6176B56B1B335CC3FFC5683FF15A07FF8AD07FE2B40FFF15A0FFC5681FFE2B41FF8AD03FFC5683FF15A07FF8AD07FE2B40FFF15A0FFC5681FFE2B41FF8AD03FFC5683FF15A07FF8AD07FE2B405555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555555

## Test conclusions

The VDES SAT-SHIP communication link, protocol link and application scenario tests verified satellite to ship data interaction and navigation aids service feasibility. Such services would facilitate VDES future inclusion into Chapter V under the SOLAS convention.

# REFERENCES

IALA G1117, VHF Data Exchange System (VDES) Overview Edition 3.0 December 2022

ITU-R M.2092-1, Technical characteristics for a VHF data exchange system in the VHF maritime mobile band, February 2022

ITU-R M.1371-5, Technical characteristics for an automatic identification system using time division multiple access in the VHF maritime mobile frequency band, February 2014

# ACTION REQUESTED OF THE COMMITTEE

The committee is requested to note the information in this document.