Input paper:   DTEC3-5.2.3.5

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	□  ENG		□  PAP					□  Input

X  DTEC	□  VTS							X  Information

Agenda item 			5.2

Technical Domain / Task Number 2	6.3.10

Author(s) / Submitter(s)			Space Norway / Kongsberg Discovery / European Space Agency (ESA)

# RHCP Ship Antenna for Improved VDES and AIS Performance

# Summary

A Space Norway led consortium with Kongsberg Discovery - Seatex, and Comrod has under a European Space Agency (ESA) contract developed and tested a new omni RHCP VDES ship antenna called the Slim Quadrifilar Helix (QFH). The new circular polarized antenna (Figure 1-2) has been tested alongside a standard dipole antenna used on ships for VHF (Figure 1-1).

The new antenna significantly improves upon the amount of received packets, signal strength and reduces multipath specular fading compared to the standard dipole antenna. In optimal conditions for both antennas, the QFH antenna receives nearly double the number of packets compared to a standard dipole antenna. The QFH antenna has shown its ability to operate in noisy environments such as harbours, in these cases a standard dipole tends to lose multiple packets and parts of the satellite pass, while the QFH is able to establish a stable connection, with little loss of packets.

Figure 1-1 Comrod AV7 Dipole antenna

Figure 1-2 Comrod QFH antenna

# introduction

Space Norway and Kongsberg Discovery - Seatex have tested and demonstrated a number VDE-SAT services using forward and return link since 2017 using the NorSat-2 and NorSat-TD VDES satellites. The testing has shown Packet Error Rate (PER) degradation caused by interference and deep fading caused by multipath. VDE-SAT protocols use selective Automatic Repeat Request (ARQ) to handle some packet loss, but the consortium wanted to investigate service quality and throughput improvements replacing the standard ship VHF dipole antenna with a circularly polarized antenna optimized for VDE-SAT, while still supporting VDE-TER and AIS usage. Following award of an ESA contract, around 100 candidate antennas were simulated to optimize gain, rejection of cross polar specular multipath and bandwidth.

Our antenna partner in this project, Comrod, has been producing high quality antennas for many years. Comrod designed and manufactured an antenna that increased the gain by more than 4 dB and suppressed specular multipath by more than 10 dB in the whole maritime VHF band.

Our measurements using a standard dipole has shown a signal variation (st. dev) of 4.2 dB, and with a target PER of 1%, a fade margin of 10 dB would be required. Halving the signal variation would reduce the required margin by more than 5 dB. Thus, the improved signal strength and fade margin reduction could improve the link more than 9 dB. Interference is likely to be linearly polarized, and a circularly polarized ship antenna may provide some suppression in harbours and noisy areas.

The new VDES antenna is expected to improve maritime communication links by enhancing PER and throughput. The antenna design aims to mitigate cross-polar specular multipath fading at negative elevations and ensure strong and consistent satellite signals at positive elevations, accommodating pitch and roll variations up to ±10°.

An important driver for our VDES activities is to enhance maritime safety and communication efficiency in Arctic regions, particularly beyond the geostationary satellite coverage area. However, the antenna can benefit satellite VDES users globally.  An improved antenna will increase service quality and throughput.  The VDES system already has methods for automatically adjusting the throughput based on link quality.

Field tests conducted across multiple locations in Norway, including Tau, Trondheim, and Svalbard, have demonstrated the antenna’s capability to outperform systems with vertical dipoles by reducing interference and improving signal reliability.

Figure 2-1 QFH antenna mounted on Kongsberg Ocean Space Lab vessel

# Objectives

## Concept of Operations

To improve the satellite communication link for VDES, it was decided that the easiest and most cost-effective way would be to improve the VHF antenna used on ships. The common antenna used for marine VHF today is a vertically polarized dipole antenna. This antenna works well for terrestrial communication, is used on AIS/VDES base stations, and is the reference antenna used in this project.

## Increased Throughput

The QFH antenna is expected to reduce multipath fading. This in turn could allow the use of satellite downlink waveforms that deliver higher bitrates. Table 3-1 shows expected throughput increase in the satellite downlink waveforms relative to the default link ID, as a function of link quality improvements and physical layer Link IDs.

Table 3-1 Throughput and link quality improvements for some of the Link ID defined in  [1]

| Link ID | Throughput (kbps) | C/N0 and fading improvement (dB) | Throughput increase |
| --- | --- | --- | --- |
| 32 | 0.78 | 0 | 1 (current) |
| 25 | 1.99 | 3 | 2.6 |
| 28 | 8.8 | 6 | 11.3 |
| 29 | 13.88 | 6 | 17.8 |

# System Specification

In this project Comrod developed an Iso Level Circularly Polarized antenna with good specular multipath suppression. This antenna was expected to improve the link C/N0 by around 4 dB (3 polarizations loss + 1 dB improved gain) at 0 degrees elevation. The reduced multipath was expected to improve the received Packet Error Rate (PER) significantly.

## Polarization Gain

The most obvious way to improve the antenna is to make one that has the same polarization as the satellites it communicates with. The Norwegian satellites NorSat-2 and NorSat-TD are two satellites owned by the Norwegian Space Agency, with VDES payloads owned by Space Norway and operated by Statsat. These satellites use right hand circularly polarized (RHCP) Yagi antennas to deliver the VDES service.

By making a new antenna with RHCP the link should be improved by 3 dB relative to the standard VHF dipole antenna, since receiving RHCP signals with a linear polarised antenna introduces a polarization loss of 3 dB because of the polarization mismatch.

## Gain vs elevation

The range from a LEO satellite varies from around 550 km to 2800 km as a function of elevation angle.  The link performance is improved for antennas that have a gain pattern that compensates for the free space loss variation. SPN measurements on the Norsat-2 satellite, show that about 80% to 90% of the pass-time the satellite elevation angle is lower than 30°, the antenna radiation pattern should therefore be optimized for these lower elevations.

## Multipath Fading Suppression

It is well known that a circular signal changes polarization upon reflection from a flat surface.  To handle this multipath, it has been common in maritime geostationary systems to use antennas with low gain at negative elevation angles and define the service area down to 5 degrees elevation. This reduces pass time significantly (around 25%) for a LEO satellite with a median elevation angle of 10°. In our opinion, it is better at VHF where we experience specular multipath, to use an antenna with good RHCP performance down to –10° elevation and good LHCP suppression at negative elevation angles.

## Environmental Requirements

The QFH antenna has been designed for the tough conditions of the maritime environment:

- The antenna shall handle relative humidity of 0% - 100%

- The antenna shall survive wind speed up to 200km/h

- The operating temperature range shall be between -40°C to +70°C

# Performance Analysis

## Iso Level Mask

Figure 5-1 shows measured antenna gain for the QFH and a standard dipole antenna placed in an Iso Level Mask (ILM) plot. The figure shows the antenna performances at the frequencies allocated for AIS and VDES around 157 MHz and 162 MHz, the QFH has been optimized for these frequencies. The gain of the QFH curve is about 1 – 3 dB better than the dipole antenna throughout the range when only looking at the RHCP gain, gain is considerably better than dipole antenna at elevations of 70° and higher.

Figure 5-1 Measured RHCP gain for the QFH and a standard dipole antenna at 157 MHz and 162 MHz

## Specular multipath suppression

It is custom to specify antenna cross polarization discrimination (XPD), as XPD is a good figure of merit. It is valuable to look at when a RHCP and LHCP signal are both received at the same angle but is not an accurate measure of multipath suppression for omni-antennas. For that reason, the Specular Multipath Suppression (SMS) has been used as a figure of merit to give a better picture of the antennas ability to suppress the reflected signal relative to the direct signal, which determines fading depth.

An example of calculating SMS can be seen below using the EZNEC simulation of a QFH antenna in Figure 5-3. On the left the RHCP (green) gain at 15 degrees has been selected, and on the right the LHCP gain (red) at negative 15 degrees has been selected. For this example the SMS and XPD at 15 degrees would be:

SMS =RHCPgain @15°-LHCPgain @-15°=2.2+6.3 dB=8.5 dB

XPD= RHCPgain @-15°-LHCPgain @-15°= -1.3+6.3 dB =5.0 dB

Figure 5-3 Specular multipath suppression example

When looking at the measured data for the QFH frequency of 160 MHz in Figure 5-3 we found that the XPD and SMS for the QFH antenna is approximately:

SMS = RHCPgain @15° − LHCPgain @−15° = 1.5 + 13.8 dB = 15.3 dB

XPD = RHCPgain @−15° − LHCPgain @−15° = −1.3 + 13.8 dB = 12.5 dB

As seen, the SMS is more than 15 dB and 2.8 dB higher than the XPD, indicating that the antenna is able to handle multipath fading very well.

Figure 5-3 Measured RHCP gain (left) and LHCP gain (right) for the QFH antenna in dBic

# Test Setup

The project included several field measurement campaigns at various locations in Norway, including Tau, Trondheim, and Svalbard, to test and verify the antenna's performance against the standard dipole. Also, the produced antennas were tested and characterized at Comrod’s test facilities. For all locations except for the sea vessel MS Polarsyssel, the antennas were tested on the same location, at the same time, under the same conditions.

## Space Infrastructure

Space Norway owns and operates multiple satellites used for AIS and VDES. For this project the NorSat-2 satellite has been used. The NorSat-2 satellite can be seen in Figure 6-1. The NorSat-2 satellite has a proven VDES payload made by Kongsberg Discovery - Seatex (Figure 6-2). NorSat-2 uses an 8 dBi RHCP Yagi antenna which is pointed at the earth limb towards a defined Service Area Centre. The satellite rotates around its nadir axis during a pass. The Yagi boresight pointing during an example pass is shown in Figure 6-3 in yellow. The antenna boresight line goes through a point above the defined Service Area Centre (blue dot). The Service Areas Center was set for each experiment.

| Figure 6-1 NorSat-2 satellite | Figure 6-2 Kongsberg VST x50 VDES transceiver |
| --- | --- |

The EIRP at 0 degrees elevation was nominally set to -1.1 dBW/50 kHz in order to meet ITU PFD limits as contained in [1]. The coverage area using a standard dipole antenna relative to the subsatellite point is shown in Figure 6-3. The contour for a nominal receive power level of -118.8 dBm at the dipole connector at 0 degrees elevation contour is shown.

Figure 6-3 Limb pointing (yellow) and Service Area Centre (blue)

## Tau

The general test setup for this project can be seen in the diagram in Figure 6-4. This particular setup was used during testing at Tau and consists of the reference dipole and the new antenna with 1.5 – 2.4 meters separation. The antennas were connected to a Kongsberg RF relay, switching every 0.4 seconds timed to the VDES slot boundaries.

The relay was connected and controlled by a Kongsberg VDES 300 terminal (Figure 6-5). By switching between the two antennas during alternate packets, the conditions for the antennas will be as similar as possible, making comparisons of data much easier as the receiver hardware is identical.

The VDES transceiver recorded data files that for each antenna included parameters such as:

- UTC time

- Signal to noise ratio (Es/N0)

- Received signal strength (RSSI)

- Doppler

- Timing offset

- CRC OK/NOK

The isolation between the antennas was measured as -27.4 dB and -30.5 dB, at 156 and 162 MHz respectively, which shows that any antenna coupling effects are minimized.

VDES 300
transceiver

Coaxial Relay

1m RG58

-0.2dB

QHX

RHCP

V-pol Dipole

2.4m

Floor

2m

GPS antenna

Two identical cables

-1.55dB 31.5m LMR400

Roof

2

1

Figure 6-4 General test setup

Figure 6-5 Kongsberg VDES 300 mobile station

## Trondheim

For the test campaign in Trondheim fjord, Kongsberg’s Ocean Space Lab vessel was used (Figure 2-1). Figure 6-6 shows the antenna placement of the standard dipole and the QFH antennas. The rest of the test setup was identical to Figure 6-4 except cable losses, which were corrected for.

Figure 6-6 Antenna positions on Ocean Space Lab vessel

## Svalbard

At Svalbard, the QFH antenna was installed on MS Polarsyssel, replacing an existing standard VHF dipole antenna. Data was logged before and after the installation, to be able to assess the performance before/after the change.

Figure 6-7 Antennas mounted topside MS Polarsyssel

# Test Results

The data comparing the performance of the QFH and dipole antenna indicates that the QFH antenna consistently receives more packets per pass than a dipole antenna at both the Polarsyssel and Ocean Space Lab locations. This is evident despite issues with satellite pointing and VDES payload commanding affecting overall system performance. The maximum number of packets received in a single pass with the QFH antenna is about twice that of a standard VHF dipole antenna at both sites, as seen in Table 7-1. Detailed analysis of specific passes shows that the QFH antenna not only receives more packets, but also performs better in terms of signal quality, with higher mean Es/No and slightly lower RSSI compared to a standard dipole antenna.

Figure 7-3 and Figure 7-4 shows an example of two good passes with a similar trajectory and a high number of packets received for a standard dipole antenna and the QFH antenna during the Polarsyssel campaign. Comparing these two passes with good performance for both antennas, there is an improvement of 3.4 dB in Es/No and reduction in RSSI of 1.5 dB in favor of the QFH antenna.

Figure 7-1 shows a histogram with number of packets received per pass for Polarsyssel and Ocean Space Lab. The Ocean Space Lab test campaign did measurements with both antennas at the same time. The Polarsyssel measurements were done over two separate test campaigns, where a dipole antenna was initially mounted on the vessel, before it was replaced by the QFH antenna. The reason the blue plot in Figure 7-1 is overrepresented compared to the other colours is because more measurements have been performed for the QFH in the Polarsyssel test campaign. It should also be noted that the QFH in the Polarsyssel test campaign was able to pick up very weak signals that the dipole could not, and therefore have many passes with 0-50 packets. Figure 7-2 shows the travel path for the Polarsyssel vessel during the two test campaigns, note that during the QFH test campaign the vessel was mostly in the Longyearbyen harbour.

Figure 7-1 Histogram of number of packets received per pass. Note that the QFH antenna has most passes with 0-50 packets, as the standard dipole (AV7) has many passes with no decoded data at all

Table 7-1 Maximum number of packets received in a pass

|  | Polarsyssel (packets in one pass) | Ocean Space Lab (packets in one pass) |
| --- | --- | --- |
| Standard Dipole antenna | 402 | 153 |
| QFH antenna | 749 | 289 |

Figure 7-2 Sailing path for Polarsyssel during standard dipole and QFH antenna test campaign

Figure 7-3 Satellite and ship position, distribution of Es/No and RSSI, elevation and frequency offset in one of the passes of the summer 2023 measurement campaign using the dipole antenna.

Figure 7-4 Satellite and ship position, distribution of Es/No and RSSI, elevation and frequency offset in one of the passes of the spring 2024 measurement campaign using the QFH antenna.

In especially noisy environments, such as the inner Trondheim harbour, the QFH antenna was able to significantly suppress the noise. During 6 passes, the standard dipole lost quite a lot of packets and was not able to establish a connection for two of these passes, while the QFH antenna was able to establish a connection in each pass. The total number of packets received for each antenna during these passes were 95 and 495 respectively. The results are summarized in Table 7-2.

Table 7-2 Measurements from noisy environment in Trondheim harbour

|  | Number of passes | Number of passes received | Total packets received |
| --- | --- | --- | --- |
| Standard dipole antenna | 6 | 4 | 95 |
| QFH antenna | 6 | 6 | 495 |

# Achievements and lessons learned

The QFH antenna has proven to be a significant improvement compared to a standard VHF dipole antenna. It is able to sustain a stable connection and lose less packets than a standard dipole under the same conditions. The QFH is also able to reduce the received interference in noisy environments compared a standard dipole antenna.

A challenge during this project has been to reach a firm conclusion on the overall performance of the QFH vs a standard dipole antenna in a maritime environment. The antenna has been tested extensively on ships close to or in the harbor and on land, but there has been less testing with the antenna on the open sea or in motion. An extended test campaign could include testing on vessels in motion and testing on several vessels over longer time duration.

The issue with loss of packets for a standard dipole antenna makes it challenging to make proper comparisons of the two antennas; raising the output power for the satellite could improve this and give us more representable data on the standard dipole antenna.

The main achievement of the project is the development of a product ready circular polarised antenna with improved performance compared to the standard VHF dipole antenna. This new antenna has already been released on the market by Comrod [2].

# Related developments and future work

The antenna is planned to be used in future VDES projects. Future testing could involve testing with other satellite Link IDs with higher throughput and terrestrial testing of VDE and AIS.

Further measurements with the antenna are expected as part of a new propagation measurement activity funded by ESA. These measurements will help determine in more detail the improved VDES performance of the new QFH antenna compared to a traditional dipole.

To make the antenna even more desirable for customers, Comrod is looking into making a combi antenna variant with an integrated active GNSS antenna. The GNSS antenna would be added internally and use the same single coaxial cable. Power for the GNSS antenna, 5VDC, will be feed through the coaxial cable, hence only one RF cable would be needed for the full installation.

# Proposed action

The consortium asks IALA to take the presented work into consideration for future developments. In particular, the presented work should be considered in the development of guidelines for VDES and AIS.

# Acknowledgements

This work is funded by the European Space Agency, ESA-ESTEC, Noordwijk, The Netherlands, under contract no. 4000137156-22-NL-NR [3]. Opinions, interpretations, recommendations and conclusions expressed herein are those of the authors and are not necessarily endorsed by the European Space Agency.

Special thanks to the Norwegian Space Agency for the use of the NorSat-2 satellite for this project.

# References

Technical characteristics for a VHF data exchange system in the VHF maritime mobile band, ITU-R M.2092-1, 02/2022

Comrod VHF156162QFH, datasheet included in Annex A

https://connectivity.esa.int/projects/rhcp-vhf-antenna

# Annex A 	Datasheet Comrod VHF156162QFH

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain