VTS32	Information paper

Agenda item	5.2 - Use and impact of risk and decision making tools (M2*)

14 - Any Other Business

Task Number	13 - Produce a Guideline on assessing and auditing the overall performance of VTS Centres with respect to their effectiveness in mitigating risk and as described in Chapter 18, Quality Management, in the VTS Manual

Author(s)	Catherine Beadley – Maritime Safety Queensland, Australia

Port and Waterway Safety Assessment (PAWSA) - The Path from Final Report to Implementation Projects and Actions

# Summary

This paper outlines the project management approach used by Maritime Safety Queensland (the Australian Government maritime safety regulator for the State of Queensland) to prioritise, coordinate and implement:

- Port and Waterways Safety Assessment (PAWSA) Report for the Port of Gladstone 28 Workshop 28 – 29 July 2009;

- Port and Waterways Safety Assessment (PAWSA) Report from the Waters of the Great Barrier Reef Between Palm and Hydrographers Passages 4 – 5 August 2009.

The aim is to encourage PAWSA Workshop participants’ organizations to commit to implementing the ideas translated into practical actions and/or projects.

## Purpose of the document

The purpose of this document is to inform VTS Committee members considering IALA risk management tools of a case study example for translating a PAWSA Final Report recommendations and ideas into a practical implementation program. The intention is to distil lessons learned for the information of other port/waterway regulators/operators considering the qualitative PAWSA tool to:

- Identify and mitigate risk;

- Ensure pre-workshop planning delivers a final report which is practical to implement;

- Encourages workshop participant organisations to commit to implementing risk mitigation strategies;

- Assess Workshop participant ideas from the perspective of which agency/organisation has the lead and supporting roles to translate ideas into practical actions and/or projects;

- Support senior management in lead/support role implementation agencies by giving them a convenient means to assess resource/funding priorities for projects/actions.

## Related documents

VTS 24/4/6,  Report on the 52nd session of the IMO Sub-Committee on Safety of Navigation (NAV), Para 3 IALA Risk Management Tool for Ports and Restricted Waterways.

VTS32/4/3 Formerly PAP20/output/1. Report of the 20th Session of the IALA Policy Advisory Panel (PAP). 18, 19 and 20 October 2010. Para11 IALA and Risk Management – Sub-para 11.2 PAWSA, IWRAP Mk2 and Simulation Training Seminar.

8.8.3a, IALA Training Seminar on Risk Management PAWSA, IWRAP Mk2 & Simulation - 15-19 November 2010 St Germain en Laye, France Final Report.

VTS32/1/1. 32nd Meeting of the Vessel Traffic Services Committee - Agenda for the 31st meeting of the VTS Committee 7-11 March 2011 Item 14 any Other Business 14.1 Workshops & Seminars (2010 – 2014)

# Background

## Geographic Maritime Context

Maritime Safety Queensland (MSQ) is a division of the Department of Transport and Main Roads. Our role is to protect Queensland's waterways and the people who use them — providing safer, cleaner seas. Maritime Safety Queensland is responsible for:

- Improving maritime safety for shipping and small craft through regulation and education;

- Minimising vessel-sourced waste and responding to marine pollution;

- Providing essential maritime services such as pilotage for regional ports and aids to navigation;

- Planning and managing boating infrastructure and state boat harbours, including dredging of channels for small craft;

- Encouraging and supporting innovation in the maritime industry.

Queensland has a 7080klms coast with a further island coast length of 6435klms which includes: 15 trading ports; four non commercial trading ports (i.e. significant fishing vessel and small commercial/tourist and recreational craft activity but not foreign going commercial shipping); and two community ports serving remote indigenous communities. The Great Barrier Reef (GBR) and Torres Strait region have international environmental and cultural significance and are located off this coast.

MSQ, in partnership with the Australian Maritime Safety Authority (AMSA) manages a coastal Vessel Traffic Service (REEFVTS) for the Great Barrier Reef and Torres Strait.  REEFVTS encompasses a mandatory Ship Reporting System REEFREP and monitoring and surveillance systems including radar, Automatic Identification System (AIS), Automated Position reporting via Inmarsat C (APR) and VHF reporting. IMO designated the Great Barrier Reef Marine Park Area as the world’s first Particularly Sensitive Sea Area (PSSA). REEFREP was established in accordance with SOLAS and adopted under a resolution from MSC52.66 in 1996.

## The Decision to Engage in a PAWSA Process

The port of Gladstone is one of the world’s largest coal ports and between January 2010 and January 2011 had 1648 ship arrivals. Increasing construction activity within Gladstone harbour including the introduction of an LNG trade has heightened the risk of marine incidents occurring. The risks arising from increased shipping activity in the coal export port of Abbot Point and coastal waters covering the Whitsundays, Palm and Hydrographers Passages environs also warranted a reassessment.

Based on previous positive experience with a PAWSA workshop for the Torres Strait, two workshops were planned for Gladstone and Townsville (closest major city to the port of Abbot Point and home to the REEFVTS centre) in 2009 in partnership with AMSA. Local expert opinion was sourced for workshop participants from industry, community and public sector maritime/emergency services with approximately 30 plus participants invited to each Workshop. The reports are available at:

- Ports and Waterways Safety Assessment Workshop Report for the Port of Gladstone 28 – 29 July 2009. Refer www.amsa.com.au.

- Ports and Waterways Safety assessment Workshop Report for the Waters of the Great Barrier Reef (GBR) Between Palm and Hydrographers Passages 4 – 5 August 2009. Refer www.amsa.com.au

# Discussion

3.1 Port of Gladstone and GBR Waters PAWSA Workshops

PAWSA as a qualitative process tasks workshop participants to identify risk and mitigation ideas using risk factors which consider the probability and consequences of incidents occurring. 24 risk factors were grouped into six risk categories and were drawn from both causes and effects of waterway casualties.

Whilst many PAWSA Report related documents refer to recommendations, the two Queensland PAWSA reports include participant statements which are more aspirational goals and ideas rather than concrete recommendations e.g.

- IALA Risk Factor - Subsequent Consequences: Health and Safety’ - the idea ‘Establish Plans’;

- IALA Risk Factor - Immediate Consequences: Personal Injuries - the idea ‘Prepare local guidelines;’

- IALA Risk Factor - Vessel Conditions: Deep Draft Vessel Quality’ - ‘the idea to ‘Promulgate and educate’

- IALA Risk Factor - Vessel Conditions: Deep Draft Vessel Quality’ - ‘AMSA address crew competency through IMO’

- IALA Risk Factor - Subsequent Consequences: Aquatic Resources - ’Achieve planning balance between environmental and development’

3.2 Assessing the ‘Ideas’ Generated by PAWSA Workshops

The two Workshop reports identified 123 ‘New Ideas’ to mitigate risks. An analysis of these ‘New Ideas’ highlighted the following:

- Many ‘Ideas’ duplicate common themes /goals

- Some ideas are global idealism where 100% delivery is not humanly possible, however projects and actions can of course move matters towards achieving the goal;

- Some ‘Ideas’ are relevant to projects/actions already underway;

- Some ‘Ideas’ can be delivered by more than one project and/or action: and

- In contrast some ‘Ideas’ need no further action either because they are already in place or impractical e.g.  IALA Risk Factor – Vessel conditions: Commercial Fishing Vessel Quality – ‘ Enable VTS transmission of non AIS data.’

3.2.1 Comment

During the initial phase of engaging PAWSA consultants it is suggested close attention be paid to the need for quite proscriptive requirements for the final Workshop Report listing of ‘New Ideas’ and recommendations. These tighter requirements should also be reflected in the guidance provided to workshop participants and how they record their work team combined ideas on ‘butchers paper’ prior to amalgamation in the plenary wrap-up sessions. These butchers’ paper lists provide critical input to the PAWSA Workshop facilitators when they write up the final report. If the risk mitigation ideas are too open ended it can impact negatively on the calibre of the final report.

Whilst ‘Ideas’ identified in the final Gladstone and GBR waters PAWSA Reports demonstrated workshop participants’ knowledge of the risk/issues their suggested risk mitigation ideas were offered from the perspective of their own organisations. This revealed a broad spectrum of responses and differences in approaches to solving essentially what is the same problem. For example the IALA Risk factor: Traffic Conditions: Congestion can prompt some workshop participants to recommend solving the problem by employing more pilots, tugs etc whilst others recommended vessel scheduling be reviewed to exact greater efficiency from existing resources.

An essential maxim in delivering a worthwhile result from a PAWSA Workshop is to have a broad cross section of expert opinion amongst participants. Whilst representatives are invited from a cross section of organizations, regrettably there may be a number of late cancellations with no replacement representative sent. Intensive and thorough pre workshop planning cannot always guard against this happening.

It is important for the final PAWSA Workshop Report to clearly note disproportionate numbers from one sector of the maritime community and the absence or minimal input from other representative organisations. This may skew results and flow through to the calibre of the final report recommendations. It is suggested that non-attendance of key expert opinion representatives should be taken into account and transparently recorded to enable any future final report reader to make an informed assessment.

3.3 PAWSA Implementation Coordination Officer Role

Coordinating the PAWSA Report implementation process requires a point of contact officer conversant with risk management, the maritime industry and port waterway issues relevant to the PAWSA Workshop. The person who manages and organizes a PAWSA Workshop does not necessarily have to be the same person tasked with coordinating the implementation phase. However it is considered essential they have a good knowledge of the PAWSA process and have attended the Workshop as either a participant or observer.

The implementation coordinator needs to be able to project manage the process from analysis of the PAWSA Report to developing and tracking a path to implementation. Factored into this should be internal and external project governance ie

- Periodic situation/progress reports to senior management;

- Liaison with those responsible for program budgeting/funding;

- consultation with those tasked with delivery of individual projects;

- Appropriate record keeping.

It is important that the terms of reference for the implementation coordination officer role not succumb to project scope creep. They should be:

- Responsible for analyzing the PAWSA Reports;

- Translating Report Ideas into practical actions or projects ;

- Securing management approval to progress;

- Handing over the task of running the individual implementation actions and/or projects to those responsible for the subject area of the project/action eg pollution response.

PAWSA as a risk management tool supports management priority decision making. The coordination implementation officer role should have a clear divide between the PAWSA Workshop risk mitigation ideas analysis, translation into projects/actions and those who will be responsible for delivering individual projects/actions.

3.4 Keeping Track of Risk Mitigation Ideas/Recommendations

From a record keeping perspective PAWSA Reports lend themselves to xls format spreadsheets. The following worksheet categories were used by MSQ:

- Worksheet 1 - IALA Waterway Risk Model Table

- Worksheet 2 - PAWSA Workshops Gladstone (GLT) and Great Barrier Reef (GBR) Recommended ideas

- Worksheet 3 - PAWSA Workshops GLT and GBR Recommended Ideas Implementation Project Stakeholder Issues Analysis Table

- Worksheet  4 - PAWSA Workshops GLT and GBR Recommended Ideas - MSQ Project Implementation Suitability Analysis for Senior Management Approval Tracking

- Worksheet  5 - PAWSA GLT Workshop  Ideas linked to Existing or New Actions or Projects and Those Requiring No Further Action ( due to either  being completed, considered contrary to the maritime regulatory standards or impractical)

- Worksheet 6 - Tracking links between Ideas to Actions or Projects and Consultation Feedback from those responsible for Implementing the Action or Project

To keep track of the 123 Ideas’ listed in the two PAWSA Queensland Workshop Reports each  ‘Idea’ was assigned a unique alpha/numeric reference eg Gladstone Idea 12 = GLT idea 12.

Of these 123 Ideas, some were identified in the Reports as of higher value and priority and have been referenced as ‘GLT or GBR (Great Barrier Reef) Top Ideas 1 thru 29.

The Gladstone Report identified 46 ideas i.e GLT Ideas 1 thru 46 and the Townsville – Workshop identified 48 Great Barrier Reef GBR ideas 1 thru 48.

3.5 The Path from Idea to Implementation

To coordinate implementation of the 123 Ideas into actions and/or projects MSQ assessed our ability to deliver on the Ideas by applying the following filter questions:

- What can we directly control;

- What can we influence and or engage in partnerships with agencies/organizations;

- Idealistic goals to aim for;

- What is outside our ability to influence and thus in the category of a watching brief.

Each ‘Idea’s’ unique alpha/numeric reference was then linked to a project and/or action which was also given a subject title / unique reference e.g.

- Existing Project No. 1 - Ship Sourced Pollution/ Emergency Management Arrangements; or

- New Action No. 3 – Letter to Gladstone Ports Corporation (GPC)

3.6 Tracking the Ideas Path to Implementation – Analysis

Defensible and transparent stakeholder analysis with respect to the choice, priority and type of projects chosen to implement the PAWSA ideas encourages continuing commitment across the port/waterway community to the PAWSA process. Any organization coordinating implementation must make decisions able to stand up to external scrutiny.

The application of project management stakeholder analysis from the perspective of a coordinating agency’s capacity to directly control and deliver implementation enables senior management to make approval decisions based on easy recognition of projects / actions which:

- Enjoy a high degree of stakeholder consensus and support;

- Those which are least likely to encounter impediments to their implementation;

- Those which will require significant infrastructure / project funding;

- Those which are susceptible to project scope creep, vulnerable to over ambitious goal setting ,cost overruns or degenerating into a paper chase with minimal practical value;

- Those which will require legislative change;

- Those which involve other agencies / organizations who have a history of being reliable and easy to work with;

- Those which involve other agencies / organizations, some of whom may have a history of being a ‘challenge’ to work with;

- Those which the coordinating organization can only gently influence rather than control the major levers needed to deliver an effective outcome.

For the purposes of subject grouping implementation projects/actions the following categories are suggested (this is not an exhaustive list as other subject headings relevant to each PAWSA location can be developed):

- Communications - Human Factors i.e.  public education, professional networking etc. e,g. example public awareness education campaigns targeting small craft interaction with large commercial shipping , Port User Groups;

- Emergency management / pollution response arrangements;

- Environmental management;

- Substandard shipping - mitigation tools;

- Vessel Traffic Management (VTM) - Human Factors i.e. scheduling, pilotage etc;

- Vessel Traffic Management (VTM) - Infrastructure ie. dredging, large Aids to Navigation etc;

- Vessel Traffic Management (VTM) - Technical support equipment and services - AIS, radar technology etc;

- Regulatory Role - Legislation and Compliance

# Conclusion

MSQ’s approach to progressing PAWSA Workshop Final Reports through to the implementation phase has essentially been to apply project management principles of:

- Define project scope;

- Stakeholder analysis;

- Consultation;

- Documentation/records management;

- Senior management approval processes which support priority decision making;

- Internal and external governance reporting.

By pushing each PAWSA risk mitigation idea/recommendation through the above process it is easier to:

- Identify those recommendations which will require resource ‘hungry’ and absorb large amounts of funding/personnel with minimal prospect of success;

- Identify high profile project/action ‘champions’ which showcase PAWSA implementation progress to all stakeholders;

- Encourage the future commitment of Workshop participant organisations/stakeholders in a partnered approach to tackling the more contentious and difficult to deliver recommended risk mitigation projects/actions.

# References

- Ports and Waterways Safety Assessment Workshop Report for the Port of Gladstone 28 – 29 July 2009. Refer www.amsa.com.au

- Ports and Waterways Safety assessment Workshop Report for the Waters of the Great Barrier Reef between Palm and Hydrographers Passages 4 – 5 August 2009. Refer www.amsa.com.au

# Action requested of the Committee

The Committee is requested to:

- Note this paper for information