Annex 9

Functional and Performance Requirements for Data Processing and Display in VTS

# Functional Requirements

- Maintaining the traffic situation

    - Getting Sensor information (real-time, semi-real-time)

    - Data fusion

    - Target extraction and maintenance

    - Target identification

    - Internal and External Databases (Loyd’s, SafeSeanet, single-hull database, various incident/black lists) – semi-static information

    - Distribution/Acces level/”Need to know”

  - Traffic situation display – real-time and non-realtime, mixing???

    - sensor information (radar, AIS, RDF, Camera)

    - (identified/unidentified/likely/lost/DR) targets

    - environment – dynamic (hydro/meteo, lock status)

    - environment – static (ENC)

  - Support Tools – Automatic and/or manual

    - Operator assistance

      - CPA

      - anchor watch

      - grounding alert

      - collision alert

      - area penetration

      - route adherence

    - analysis and prediction

    - incident recording, replay and analysis

  - (Legal) recording and replay

  - Simulation and Training

# Data Processing

## Introduction

## Sensor Information

### Radar

Digitisation

Dynamic range

In most cases, a VTS system will use one or more radar sensors to generate a real-time traffic situation. Radar is a non-cooperative sensor in the sense that targets are usually detected without any particular action on the side of the target. Note that, especially small targets like sailing boats, may increase their visibility as a radar target by using specially crafted radar reflectors.

The radar image may be presented to the operator without further processing. This allows a trained operator to get a better perception of the target and its environment. If the targets are sufficiently close to the radar, such as during port approach, the resolution of radar allows target features  like an approximate length, width or even orientation can be distinguished. This warrants presentation of the radar image on its own.

On the other hand, radar provides range and bearing measurements, relative to the sensor position. These measurements can be used to automatically determine the position, groundspeed and course over ground of a target. Typical VTS radars use a rotating antenna with a 1s to 10s rotation period. This means that a target is observed once about every rotation. This observation period matches with the expected movements and accelerations of VTS targets.

### AIS

AIS is basically a digital VHF communication system that provides information from sensors onboard cooperative targets (i.e. targets carrying an AIS transponder). AIS information is sent in packets. This is position information, but also information that identifies the ship (MMSI and ship name) and whether it carries any dangerous cargo or not. Apart from this, AIS can be also used to exchange safety-related information and other information, as detailed in [A.124].

AIS position information is obtained from the ship navigational system and is, generally, a GNSS-derived position, which may or may not be differentially corrected. This latter fact is indicated in the AIS message though.

### Long-range sensors

This concerns Long-Range Identification and Tracking (LRIT), satellite AIS and information of AIS networks beyond the area of direct interest of the VTS. Long range sensors are not ususally necessary for VTS operation (INF, NAS or TOS). They may provide information, though, that assists in long-term planning and may be relevant for allied services as well.

### Electro-optical

### Data fusion

Local tracking/centralised fusion

multi-sensor tracking

### Target extraction and maintenance

### Target identification

### Internal and External Databases (Loyd’s, SafeSeanet, single-hull database, various incident/black lists) – semi-static information

### Distribution/Acces level/”Need to know”

### 4.2.4  Plot Extraction

Plot extraction (see IALA Guideline 1056) should be automatic.  The plot extraction process should be able to handle a minimum of plots per rotation (see Table 55.1).

### 4.2.5 Track Initiation

Track initiation should be automatic, automatic except in selected areas, automatic in selected areas, or manual depending on the concept of operations.

In automatic track initiation modes, all plots in a scan should be considered potential targets.  Some of the plots will be associated with previously established tracks, while the remaining plots should be considered as candidates for new tracks, i.e. tentative tracks.

Tentative tracks will become confirmed tracks if plots from consecutive scans “fit into the picture” within reasonable physical manoeuvrability limits, otherwise the tentative tracks are discarded.

The tracking system should be able to handle at least a certain number of tentative tracks and to initiate tracks and eventually to confirm tracks under certain conditions of PD and PFA.

It should also be possible to initiate a track manually.  In manual track initiation a plot on the radar display is selected by the operator using a graphical tool.  When selected this plot should form the starting point for a tentative track which eventually should be confirmed or discarded, as in the automatic case described above.

### 4.2.6 Maintaining Track

If automatically or manually created tentative tracks persist over a certain length of time the tracks should be promoted to confirmed tracks.  Confirmed tracks should be shown on the display.  The tracking system should be able to handle at least a certain number of confirmed tracks (Table 55.1) and to maintain tracks under certain conditions of PD (Table 55.2) and PFA ≤ 0.01.

### 4.2.7 Track Termination

If a confirmed track either moves outside a user defined maximum range, into a user defined non-tracking area, if the quality of the track falls below a predefined minimum, or if the track cannot be updated with new plots over a certain length of time, then the track should be terminated.  In certain cases the operator should receive a warning as defined by the VTS Authority.

## Plot extraction and tracking performance

The requirements in respect of plot extraction and tracking should be defined by the individual VTS authority, on the basis of local conditions, number of radar sensors in a system etc.  Table 55.1 suggests values for each individual radar sensor in a system.

Table 55.1  Radar tracking performance parameters.

| Plot Extraction and Tracking Performance for each individual radar in a system |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter |  | Recommendation level |  |  |
|  |  | Basic | Standard | Advanced |
| Number of plots per antenna rotation |  |  1000 |  2500 |  5000 |
| Number of confirmed tracks |  |  100 |  200 |  300 |
| Time for confirmation of tentative track |  |  1 minutes |  |  |
| Time from track confirmation to achievement of specified accuracy |  |  2 minutes |  |  |
| Time from data loss to automatic track termination |  |  1 minutes |  |  |
| Speed of tracked objects |  |  50 knots |  | 70 knots |
| Turn rate of tracked objects |  |  10°/second |  |  20°/second |
| Accuracy in track position | Range |  0.75 % of range covered by the individual radar or 10m + selected pulse length, whichever is the greater |  |  0.5 % of range covered or 5m + pulse length |
|  | Bearing a |  1, X-band  2, S-band |  |  0.5, X-band  1, S-band |
| Accuracy of track data | Speed a |  2 knots |  1 knot |  1 knot |
|  | Course a |  5° |  2° |  2° |

### 5.3.1  Track initiation and track maintenance

The radar PD should be adapted to the role of the VTS.  The automatic track initiation and track maintenance is optimised accordingly.

Recommendation for the minimum radar PD for track initiation is given in Table 55.2.  For track maintenance a lower minimum radar PD can apply, depending on the tracking principles used by the manufacturers.

Table 55.2  Track initiation

| Minimum radar PD for track initiation |  |  |  |
| --- | --- | --- | --- |
| Priority of the VTS | Recommendation level |  |  |
|  | Basic | Standard | Advanced |
| Surveillance and/or traffic monitoring | 0.9 | 0.8 | 0.7 |
| Safety | 0.9 |  |  |

### 5.3.2 False tracks

False tracks may appear as a result of noise, clutter (including wakes) and ghost echoes.  However, the number should not be significant if the recommended values given in Table 55.1 and Error: Reference source not found are respected.

The maximum number of false tracks allowed is dependant on role of the VTS.  False tracks should be avoided in safety critical areas and occasionally accepted in areas where surveillance and traffic monitoring is the priority.

There is a trade-off between the time for confirmation of tentative track and the number of false tracks.  A longer confirmation time implies less false tracks and it should be possible to balance this trade-off in the setup of the VTS.

### 5.3.3 Track loss

Track loss may occur as a result of PD < 1 in combination with targets manoeuvring, especially in the vicinity of obstructions such as bridges.

A level generally accepted is that each operator should correct up to one track loss per hour in all areas where the recommended values given in Table 55.1 and Table 55.2 are respected.

The VTS authority should address critical areas, such as the vicinity of bridges, and explain expectations to tracking to allow VTS suppliers to make solutions accordingly.

### 5.3.4Track swap

Swapping of track identity may occur as a result of targets moving close together or even merging for a period of time, especially if targets are overtaking with small difference in speed and course.

A simple method of manual correction should be employed.

In the case of AIS information being available for the radar track(s) in question, automatic correction should be performed.

The problem may also be addressed by implementing operational procedures to separate targets or to prevent overtaking in critical areas.

# Data Display

## Introduction

Where both AIS and radar data are available they should be fused and presented to the VTSO as one unambiguous target using best available target data.  However, the VTS Operator should have the ability to choose whether to display the information on a sensor basis.  That is, whether individual targets are to be displayed using the AIS derived data only or the radar derived data only or from both subject to track fusion in accordance with the requirements of the VTS Authority.

If AIS information shows differences when compared to other sources, VTSOs should use appropriate procedures and evaluate the obtained information and where applicable advise the vessel immediately.

## Control of Displayed Information

To ensure the best possible traffic image is maintained the duty VTSO should have the capability to select the primary target source being displayed for individual vessels or all vessels.  For example, selecting the radar target for a vessel with a faulty AIS unit or the AIS network is unreliable.

## Emergency Situations

To optimize the operations of the VTS Centre the data information should be easily transferred to another location capable of maintaining the VTS service in the event of an emergency situation resulting in temporary closure of the VTS Centre.

Figure 55.1 Long range image

## List of Participating Vessels

The list of participating vessels should include static information and dynamic information concerning the vessel, for example Vessels Name, Call Sign, IMO Number, MMSI, ETA, ETD, Draft, Course, Speed and Position, if required.

An example of a typical list is shown in Error: Reference source not found. This example is taken from the Baltic AIS Network which is integrated to the VTS system

Figure 55.2 Example of List of Participating Vessels –Source: Archipelago VTS, FMA

Within one standard deviation (Gaussian distribution) when sailing on a straight course.  Note that verification may require simulated tracks or other methods due to the fact that it may be impossible to direct a test target to sail with sufficient accuracy.