Input paper:   DTEC4-6.2.3.5

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	□  ENG		□  PAP					□  Input

X  DTEC	□  VTS							X  Information

Agenda item 			6.2

Technical Domain / Task Number 2	…………………………………

Author(s) / Submitter(s)			Kongsberg Discovery, Space Norway

# AIS Performance using RHCP Ship Antenna

# Summary

A Space Norway led consortium with Kongsberg Discovery - Seatex, and Comrod recently has developed and tested a new omni RHCP VDES ship antenna called the Slim Quadrifilar Helix (QFH), which was presented at DTEC3 [1]. This circularly polarized antenna (Figure 1-2) has now been tested at an AIS base station alongside a standard dipole antenna used on ships for VHF (Figure 1-1).

The dataset of 60 000 AIS position reports shows as expected around 3 dB polarization loss for the QFH antenna, but both antennas received a similar number of AIS reports and AIS targets. There was no measurable difference in maximum range. Other comparative measurements in Trondheim harbour in Norway has shown reduced interference levels using for the circularly polarized antenna.

The used of circular polarized antennas for VDE-SAT can enable significantly higher throughput on the VDE-SAT link and our measurements indicate no reduction in AIS performance (range, success rate) when used on the vessel.

Figure 1-1 Comrod AV7 Dipole antenna

Figure 1-2 Comrod QFH antenna

# introduction

Figure 2-1 shows the test-up used for testing in the Trondheimsfjord in Norway. Both vertical and circularly polarized antennas are installed on the roof 15 m above sea level in Trondheim harbour with a vertical separation of 5 m. A Kongsberg VDES MS620 monitoring station (Figure 2-2) was used to receive and process the AIS messages. The monitoring station is part of Space Norway’s VDE-SAT Quality Monitoring Node network.

Figure 2-1 AIS terrestrial testing using linear and circularly polarized base station antennas

The antennas were switched every 750th timeslot (20 s) by an RF switch, consecutively over multiple TDMA frames, resulting in equal receive duty cycle for each antenna. The Maritime Mobile Service Identity (MMSI) and position data were logged together with demodulator metadata such as antenna used and received signal strength.

Figure 2-2 Kongsberg VDES MS620 installed in the VDE-SAT Quality Monitoring Node with RF switch

# Test Results

The dataset contained 88 000 decoded AIS messages, of which approximately 60 000 were position reports (Figure 3-1) from which a simple analysis was performed and graphs generated. Vector mathematics was used to calculate the great circle distance between the vessel and base station location.

Figure 3-2 shows a scatter plot of received signal strength for all vessels arriving and departing Trondheim harbour. We have used logarithmic scale for distance to better show the expected logarithmic reduction in signal level, which can be seen as a straight line. We have also plotted as solid lines a moving average of 1000 positions. We clearly see that the vertically polarized dipole antenna (red line) is 3 to 6 dB stronger than the circularly polarized (blue line). There was no measurable difference in maximum range (56.6 km)

The reports were from ships in harbour vary significantly and can clearly be seen as vertical lines. There are also dense points between 10 and 15 km, this represents fast catamaran vessels traversing frequently.

Figure 3-1 Map of AIS position reports received during the test, receiver site is marked in red.

Figure 3-3 shows the variation in range for vessels with Norwegian MMSIs. To improve plot clarity we have added 50 000 to the MSSI for vertically polarized receptions (orange). There are no measurable differences in range. The AIS report density (success rate) shows no difference.

The number of received AIS messages and the number of AIS stations is shown in Table 3-1. The QFH antenna showed a similar performance to the standard dipole antenna, both in terms of vessels tracked and number of messages received. More detailed analysis showed QFH received 51.6% of the messages and the dipole 48.4%.

Table 3-1 Number of received AIS messages and AIS stations

|  | Total number of AIS messages received | Position reports (AIS message type 1-3) | AIS stations (total/unique) |
| --- | --- | --- | --- |
| Standard dipole antenna | 42 532 | 22 604 | 64 / 1 |
| QFH antenna | 45 323 | 24 220 | 66 / 3 |

Figure 3-2 RSSI vs distance for 60 000 AIS messages in Trondheim harbour

Figure 3-3 AIS RSSI measurements comparing vertical and circularly polarized antennas

# Analysis

In terms of received AIS messages, the QFH antenna showed a similar performance to the standard dipole antenna. This is believed to be due to the environmental noise-reduction properties of the circularly polarized antenna, rejecting linearly polarized interference compensating for the polarization loss in noisy harbours.

More testing is needed before concluding that a circularly polarized ship antenna does not result in degraded AIS/VDES performance under all conditions, but these tests indicate no reduction in performance in interference environments. In quiet environments we expect the circularly polarized antenna to be 3 dB worse than when linear antennas are used at both ends of the link.  This raises the question if it is better to improve the links in noise environments or to maximize the range in quiet environments. For the AIS reception on vessel, maximum range is not believed to be of utmost importance, while for an AIS base station site usage of the circular polarized antenna might limit the coverage area slightly.

# Conclusion

For VDE-SAT a much higher throughput is expected with a circularly polarized antenna. For AIS, the circularly polarized antenna may reduce the detection range slightly in low-noise environments due to the polarization loss. As shown in the results the performance is similar to the linear antenna in a noisy harbour environment for this limited dataset. For VDE-TER higher throughputs may be achieved in similar environment, but this is yet to be tested in the field.

# Proposed action

The consortium asks IALA to take the presented work into consideration for future developments.

# References

DTEC3-5.2.3.5, RHCP Ship Antenna for Improved VDES and AIS Performance

Input document number, to be assigned by the Committee Secretary

Leave open if uncertain