Task 5.1.1 – Develop Guidance on Maritime Services

This paper reflects discussions on an initial strategy to develop Maritime Services within the IALA domain.

Background

When developing the International Maritime Organization (IMO) e-Navigation strategy to improve safety and efficiency of maritime transportation, it became clear that information services provided from coastal states to ships would be an essential part of this initiative.

The draft guideline developed with support from IALA members was submitted to the IMO/IHO Harmonisation Group on Data Modelling (HGDM) and was the foundation of the IMO resolution (MSC.467(101) ).

The resolution is a reproduction of the IALA draft guideline (“Guidance on the definition and harmonization of the format and structure of maritime services in the context of e-Navigation”).  IMO MSC 101 also endorsed the circular on initial descriptions of Maritime Services (MS) in the context of e-Navigation(MSC1.Circ.1610).

The basic descriptions were submitted by IALA, IMPA, WMO and IHO. The MS without coordination body were submitted by Norway. The template and context of the MS is basically harvested from the IALA draft guideline.

The ARM Committee recognises the latest developments in IMO related to operational descriptions of MS. MSC 101 adopted the draft MSC resolution on Guidance on the definition and harmonisation of the format and structure of Maritime Services in the context of e-Navigation. The IMO resolution (MSC.467(101) contains a guideline, as well as a template for describing the MS. The ARM committee has followed the development of MS descriptions in IMO, e.g. most recently through IALA representation and attendance by ARM WG2 members at NSCR7 (Navigation Workgroup), where an agenda item on this subject was discussed (Provide additional guidance for further development of Maritime Services (MS) descriptions (MSC.1/Circ.1610), including technical services and product specifications, to guide the work being undertaken by domain coordinating bodies according to resolution MSC.467(101)). NSCR7 endorsed a requirement for an IMO intersessional harmonization group, with a scope which could include some inputs from IMO FAL, as this would effectively enable the Organization leading the work to progress the development of descriptions of Maritime Services. MSC 102 has been asked to adopt this requirement and a provision for a ToR for this harmonisation group.

Based on the new, central role/initiative in IMO, as expressed in the MSC resolution, the ARM committee recognises that IALA can contribute and provide subject matter expertise to the development and implementation of MS, both within IALA domain, to the IMO and to other coordinating bodies.

Focus / strategy / task?

Currently, the IMO MS related to VTS is divided into three services, MS1, MS2 and MS3. The IALA VTS committee’ work on a revised guideline for VTS (IMO Resolution A.857(20)) was adopted by NCSR7 with an expectation for endorsement by IMO Assembly, and thus in the future this might be merged into one MS service containing all known maritime services for VTS.

The ARM Committee has the capacity to coordinate the development within already existing relationships and cooperation with experts from coordinating bodies, industrial and national members. The ARM committee will support IMO in the development towards operationalization of MS.

The descriptions of the MS form the basis for the development of product specifications. Some of the product specifications are under development in ARM, with support from experts and IHO.

The committee should be tasked to focus on the support and delivery to IMO progress. This also includes the need for IALA to liaison with relevant coordinating bodies, e.g. through the IMO lead Harmonisation Group referenced further up in this document

VTS and ARM committees are responsible for content and development (operational requirements) of Maritime Services within their expertise, e.g. Maritime Services related to VTS is the responsibility of the VTS committee and Aids to Navigation is the responsibility of ARM committee. ARM is the main committee of IALA on management of product specifications and the support and delivery to IMO progress.

New Maritime Services under IALA

The work on ship reporting has already been fruitful. The committee has provided input to IMO NCSR and IMO FAL regarding Maritime Service no. 8 (MS8 - ship reporting). IALA members have emphasised the need for coordination of MS8. This has also been part of the background for including this task in the ARM Committee work program. The ARM committee will continue to develop a guideline and a new Maritime Service.

IALA has been developing S-201 (AtoN Product Specification) (and in process of its derivative S-125 (Nautical Services Product Specification)). It is recommended that the ARM Committee continues to support IHO on the provision of MS11 (Nautical Chart Service) and MS12 (Nautical Publications Service). MS17 (AtoN Service) is to be developed within the ARM committee.

Similarly, MS18 (PNT Service) will also be developed. To that end, one Product Specification has been created (S-240 – DGNSS Station Almanac Product Specification) which supports further development of this Service.

Additional considerations should focus on interoperability as an important aspect of e-navigation and the implementation of Maritime Services as well as include mechanism to identify and submit new services based on user needs revealed by members.

Recommendations / conclusions

The ARM Committee will provide a roadmap that reflect the support and development towards operationalisation of Maritime Services, using practical solutions based on user needs. Examples of successful test and evaluation of Maritime Services, such as Smart Navigation project in Korea, STM and SESAME projects, among others, should serve as role models and examples of effective implementations of Maritime Services derived from specific user needs.

A development and delivery roadmap and strategy should take into considerations and/or include the elements highlighted in this document.

The roadmap work and development of a new Maritime Service on AtoN service commended in ARM 11.

Annex New Maritime Services description on AtoN service

MS 17 Aids to navigation services (AtoN)

### Submitting organisation

IALA

### Coordination body

IMO and IALA

### Description of the Maritime Service

This Maritime Service describes the provision of Marine Aids to Navigation (AtoN) that deployed to enhance and improve the level of safety at sea. AtoN is a device, system or service, external to vessels, designed and operated to enhance safe and efficient navigation of individual vessels and/or vessel traffic.

AtoN should not be confused with a navigational aid. A navigational aid is an instrument, device, chart, etc., carried on board a vessel for the purpose of assisting navigation.

Scope of AtoN in this description is limited to visual and other physical AtoN in current use and provides comments on the application and performance of the various technologies.

### Purpose

IALA Maritime Buoyage System (MBS) is the major guidance on buoyage systems.

The purpose of this Maritime Service is to mitigate transit risks and to promote the safe, economic, and efficient movement of vessels by assisting navigators with determining their position, a safe course, and warning them of dangers and obstructions, especially when used in conjunction with other aids within visual, audio, or radar range of the mariner.

Within the Maritime Buoyage System there are six types of marks, which may be used alone or in combination. The mariner can distinguish between these marks by identifiable characteristics. Lateral marks differ between Buoyage Regions A and B, as described below, whereas the other five types of marks are common to both regions.

### Operational Approach

Visual marks for navigation can be either natural or man-made objects. they include structures specifically designed as short range AtoN, as well as conspicuous features such as headlands, mountain-tops, rocks, trees, church-towers, minarets, monuments, chimneys, etc. Short range AtoN can be fitted with a light if navigation at night is required, or left unlit if daytime navigation is sufficient.

Common examples of visual AtoN include lighthouses, beacons, leading (range) lines, buoys (lit or unlit), lightvessels, daymarks (dayboards) and traffic signals.

MBS provides six types of marks:

- Lateral marks

- Cardinal marks

- Isolated danger Marks

- Safe Water Marks

- Special Marks

- Emergency Wreck Marking Buoy

MBS region A: a port lateral marks and lights are coloured red and starboard lateral marks and lights are coloured green. MBS region B port lateral marks and lights are coloured green and starboard lateral marks and lights are coloured red.

IALA has produced a recommendation on the characters for light and color on AtoN. Tables of classifications and specifications of AtoN characters are provided in IALA Recommendation E-110.

Figure 17-1: Maritime Buoyage System Region A

Figure 17-1: Maritime Buoyage System Region B

As part of AtoN service, virtual AtoN service could be provided where AtoN does not physically exist but is a digital information object promulgated by an authorised service provider that may be presented on navigational systems. The purpose of the Virtual AtoN System is to provide a near-instantaneous warning to the mariner of a new danger, such as a wreck, obstruction or floating debris. This warning must be provided in a form that can be received, interpreted and displayed by any class of vessel in the required operational area and carrying appropriate equipment.

Virtual AtoN can provide early notification to the mariner of urgent, temporary or dynamic information. Virtual AtoN should not, in general, be considered as a replacement for other forms of MSI but can provide a valuable supplementary delivery mechanism, enabling an automated graphical display of MSI otherwise only available in textual form. In certain circumstances, refer to the IALA Recommendation O‐143 on the Provision of Virtual Aids to Navigation, Virtual AtoN can be also used as permanent marking.

### User Needs

AtoN is any sort of marker that guides to mark safe waters and also help mariners in determining their position with respect to land or any navigational hazard or hidden danger.

New danger information should be provided to all stakeholder by all means.

### Information to be Provided

17.7.1 General

IALA Maritime Buoyage System (MBS), Guideline 1081 on provision of virtual Aids to Navigation provide the set of information and guidance needed for the service.

17.7.2 Virtual AtoN

IMO SN/Circ. 243 Annex 2 provides guidance on the use of navigation related symbols on all shipborne navigational systems and equipment.

The symbol for an AIS AtoN

Shore based AIS networks provide competent authorities with means to operate an information service for shore‐based VTS, traffic management schemes, ship reporting systems and other shore‐based safety related services, including virtual AtoN services. This service consists of information delivery between ships and shore and vice versa.

Technical details of the AIS technology and of the layout and local configuration of shore‐based AIS are described in IALA Recommendation A‐124.

In the longer term, authorities may expect that other media for information services will become available. This would enable virtual AtoN services to be transmitted via means other than AIS, for example: sitcom / internet; WiMAX; LORAN; DGPS ‐ IALA maritime beacon system; GPRS cell phone data transmission protocol.

In the near term, not all vessels can be expected to be able to effectively display virtual AtoN information. As of July 2008 SOLAS Radars are required to interface to AIS and display AIS information, as in MSC Resolution 192(79). Although ECDIS is capable of displaying AIS information, it is not required to do so. Based on current rates of navigation system upgrades, it is expected to take at least some years before the substantial percentage of the international commercial fleet will have this capability. Changes in mandatory carriage requirements may accelerate this development for SOLAS vessels. Portable Pilot Navigation Systems may facilitate implementation.

Economic benefit to the shipping industry could also provide an impetus to early adoption of this technology. The nominal report rate of virtual AIS AtoN broadcasts is specified in ITU‐R‐M.1371. However, due to the limitations in data link capacity IALA Recommendation A‐126 recommends a more flexible approach to be considered, taking data link capacity and power consumption at transmitting stations into account. In the longer term, different report rates may be implemented on different communication media, as appropriate.

The IMO e‐Navigation concept includes the need to harmonise the presentation of both shipboard and shore side safety related information. Consideration should be given in implementing virtual AtoN to the harmonisation process within e‐Navigation for future applications.

17.7.3 Technical service

17.7.4 Product specifications S-201, S-124 and S-125

The Aids to Navigation (AtoN) Information Product Specification provides a common structure for the exchange of information about AtoN. This includes buoys, beacons, racons, lights, sound signals and AIS. The product contains the positions, properties, operational status and general comments related to an AtoN.

The Product Specification can be used to exchange AtoN information in a consistent form between Lighthouse Authorities, Hydrographic Offices and other organizations, including commercial and professional agencies.

17.7.5 User and system requirements?

### Associated Technical Services

| Name | ID (MRN) | Description | Architect(s) | Standardisation Body |
| --- | --- | --- | --- | --- |
| Marine navigational services | urn:mrn:iho:S125: | S-125 |  | IHO |
| Navigation warning service | urn:mrn:iho:S124: | S-124 |  | IHO |
| AtoN information | urn:mrn:iala:aton: | S-201 |  | IALA |

### Relation to other Maritime Services

MS 17 has a relationship with other Maritime Services where it affects the AtoN

Examples may be different depending on coastal states arrangement.

| Description | Examples of information related to MS 17 |
| --- | --- |
| MS 1 VTS Information Service (INS) | Navigational hazard |
| MS 2 VTS Navigation Service (NAS) | Exchange of routes, navigational advices and assistance |
| MS 3 VTS Traffic Organisation Service (TOS) | Waterway management |
| MS 4 Port Support Service (PSS) | ETA/ATA, Access to the port, availability of port services |
| MS 5 Maritime Safety Information (MSI) Service | MSI information |
| MS 6 Pilotage Service | Pilot and boarding arrangements |
| MS 7 Tug Service | Update on information |
| MS 8 Vessel Shore Reporting | Position information |
| MS 9 Telemedical Assistance Service (TMAS) | Vessels position, delays, weather information |
| MS 10 Maritime Assistance Service (MAS) | Notifications, routing, places of refuge |
| MS 11 Nautical Chart Service | Local area updates, chart updates |
| MS 12 Nautical Publication Service | Updates to publication |
| MS 13 Ice Navigation Service | Ice routes, ice conditions, ice breaking assistance |
| MS 14 Meteorological Information Service | Weather information |
| MS 15 Real Time Hydro and Environmental Information Service | Horizontal and vertical tidal information in VTS area, available water column |
| MS 16 Search and Rescue (SAR) Service | Search pattern and vessels of opportunity |