IALA Guideline

Navigation Officers’ Awareness of VTS

Edition 2.0

Document date

Revisions to this IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |
| month/year approved by Council |  |  |

1.	Intorduction	7

1.1.	BACKGROUND	7

1.2.	APPLICATION	7

2.	AIMS AND OBJECTIVES	7

2.1.	AIMS	7

2.2.	OBJECTIVES	8

3.	OVERVIEW OF VTS	8

3.1.	TERMS USED IN VTS	8

3.2.	AIMs, GOALs, ROLEs AND FUNCTIONs OF VTS	8

3.2.1.	AIMS	8

3.2.2.	GOALS	8

3.2.3.	roles	9

3.2.4.	FUNCTIONS	9

3.3.	WHAT CAN VTS DO FOR NAVIGATION OFFICER	9

3.4.	TYPES OF VTS	10

3.4.1.	COASTAL VTS	10

3.4.2.	PORT /HARBOUR VTS	10

3.4.3.	VTS IN INLAND WATERS	10

3.4.4.	INTEGRATED VTS	11

3.5.	DIFFERENCES AND CONNECTIONS BETWEEN VTS AND TRAFFIC SEPARATION scHEMES & VESSEL REPORTING SYSTEMS	11

3.5.1.	DIFFERENCES	11

3.5.2.	CONNECTIONS	11

3.6.	When a vessel is required to participate in VTS in international waters, it can be achieved by setting up the Vessel Reporting System in the relevant area or by a Ship routeing system adopted by the IMO.TYPES OF SERVICE PROVIDED BY VTS	11

3.6.1.	INFORMATION SERVICE	11

3.6.2.	TOS	12

3.6.3.	NAS	14

3.6.4.	ALLIED SERVICES AND OTHER SERVICES	15

3.7.	APPLICATION OF VTS	15

3.7.1.	MANDATORY VTS	15

3.7.2.	VOLUNTARY VTS	15

4.	THE BASIC WORK OF VTS	16

5.	COMMUNICATION BETWEEN THE NAVIGATION OFFICER AND THE VTS CENTRE	16

5.1.	TOOLS	16

5.2.	LANGUAGES	16

5.3.	REQUIREMENTS OF COMMUNICATIONS	16

5.4.	SMCP AND MESSAGE MARKERS	16

5.5.	COMMUNICATION PROCEDURES	17

5.6.	COMMUNICATION SKILLS AND METHODS	18

6.	ACQUISITION OF VTS INFORMATION	18

7.	HOW TO RAISE THE VTS AWARENESS OF NAVIGATION OFFICER	18

7.1.	ADDING VTS RELATED CONTENT SUBJECTS TO THE BRIDGE RESOURCE MANAGEMENT (BRM) TRAINING	18

7.1.1.	VTS KNOWLEDGE IN THEORY TRAINING	18

7.1.2.	ADDING VTS CONTENTS IN PRACTICAL TRAINING	18

7.2.	ADDING THE VTS CONTENTS IN NAVIGATIONAL EQUIPMENT TRAINING	19

7.2.1.	PRACTICAL TRAINING OF VHF	19

7.2.2.	ECDIS PRACTICAL TRAINING	20

7.3.	CONSIDERATION OF VTS FACTORS IN PRACTICAL TRAINING of VOYAGE PLANNING AND PREPARATION	20

7.4.	ADDING VTS CONTENTS TO NAVIGATION SIMULATOR TRAINING	21

7.5.	COMMUNICATION ACTIVITY BETWEEN NAVIGATION OFFICERS AND VTS CENTRE	22

7.6.	STRENGTHENing PILOT'S FAMILIARITY WITH VTS	23

8.	ACRONYMS & DEFINITIONS	23

8.1.	ACRONYMS	23

8.2.	DEFINITIONS	23

9.	REFERENCES	23

ANNEX A	OPERATING GUIDE of VESSELs IN VTS AREA	25

ANNEX B	BASIC WORDS	30

List of Tables

Table 1	Examples of the types of information that may be provided by the VTS operating an INS	11

Table 2	Examples of types of information that may be provided by the VTS within a TOS	13

Table 3	Examples of the types of information that may be provided by a VTS operating NAS	15

Table 4	Message marks used by VTS operator	21

Table 5	Message marks used by Navigation Officers	22

Table 6	List of headings – guide to type of information	26

Table 7	Examples of practical training projects in BRM	27

Table 8	Examples of a communication training project using vhf and the VTS Centre	28

Table 9	Examples of VTS factors that can be considered in AIS practical training	29

Table 10	Examples of VTS factors that can be considered in ECDIS practical training	29

Table 11	Examples of VTS factors that can be considered in voyage planning and preparation practical training	30

Table 12	Examples of VTS training in Marine Simulator training	31

Table 13	Examples of reporting to VTS centre	35

Table 14	Example of Navigation Officer operating in NAS	36

Table 15	Example of Navigation Officer operating in TOS	37

Table 16	Examples of Navigation Officer operating in SAR	37

Table 17	Example of Navigation Officer operating in other emergency situations	38

Table 18	Apply for weather information	40

Table 19	Apply for traffic information	40

Table 20	Table 20 Basic words and it’s meaning	41

Table 21	Spelling of letters	42

Table 22	Number Spelling Pronunciation	42

Table 23	Table 23 Example of Number Pronunciation	43

List of Figures

Figure 1	The implemention process of INS	12

Figure 2	The implementation  process of TOS	14

Figure 3	The implementation process of NAS	16

Figure 4	Vessel calling process	24

Figure 5	Vessel responding process	25

# Intorduction

## BACKGROUND

There is a consensus within the industry that VTS is now a mature and reliable partner to improve the safety and efficiency of navigation, the safety of life at sea and to provide a central risk reduction measure regarding oil spill and environmental damage.

IMO res. A.857 (20), SOLAS Chapter 5 Regulation 12 Vessel Traffic Services, MSC/Circ. 1065, STCW 1978 resolution 10 mention VTS and stress the importance of it.

The efficiency of VTS depends upon the effectiveness of the communications involved. The effectiveness of the information exchanges between the VTS, the master or the bridge team depends upon each party's understanding of their respective and mutual function and responsibility. It is useful to establish effective coordination between them for enforcing the safe and efficient navigation. The VTS centre can be considered as an extension of the bridge resource management team. When the master and the bridge team have more knowledge about a VTS it would contribute to better teamwork between VTS and vessels to achieve safe navigation.

However, there is another consensus within the industry that Navigation Officers’ knowledge of VTS is insufficient. Indeed, the lack of knowledge of masters and the bridge personnel concerning VTS capabilities limits the vessels to use services provided by VTS, resulting in confusion, hazardous situations and significantly weakening situational awareness. Meanwhile, there is little requirement for VTS training within STCW and it is only mentioned briefly in Table A-II/1 “Maintain a safe navigational watch” and the importance of VTS is not reflected in STCW.

It is necessary to develop a guideline to help Navigation Officer better grasp VTS functions, responsibilities and procedures, and to instruct the training institutions to conduct VTS awareness training; it also helps mariners to be on duty as required by the STCW Convention, thereby promoting the safety of navigation.

## APPLICATION

This Guideline applies to Maritime Training Centres, shipping companies, harbours and Pilot stations, flag State certification administrations and other relevant stake holders who have a role or assist in improving awareness of the Navigation Officers and their understandings of VTS.

# AIMS AND OBJECTIVES

## AIMS

This guideline presents guidance to what information should be made available to Navigation Officers as part of their basic Navigational training, whether this training is provided through Maritime Training Centres or any other stake holders.

## OBJECTIVES

The objective is to raise the awareness of the Navigation Officers about VTS through training and to facilitate effective communication and exchange of information between the bridge team and VTS Operators through a greater mutual understanding between the two parties of their respective functions and responsibilities.

# OVERVIEW OF VTS

## TERMS USED IN VTS

.1.Vessel traffic service (VTS) - a service implemented by a Competent Authority, designed to improve the safety and efficiency of vessel traffic and to protect the environment. The service should have the capability to interact with the traffic and to respond to traffic situations developing in the VTS area.

.2.Competent authority - the authority made responsible, in whole or in part, by the Government for safety, including environmental safety, and efficiency of vessel traffic and the protection of the environment.

.3.VTS authority - the authority with responsibility for the management, operation and co-ordination of the VTS, interaction with participating vessels and the safe and effective provision of the service.

.4. VTS area - the delineated, formally declared service area of the VTS. A VTS area may be subdivided in sub-areas or sectors.

.5. VTS operator—an appropriately qualified person performing one or more tasks contributing to the services of the VTS.

.6. VTS reporting line - an electronic virtual line established by the competent authority to report to the VTS centre as required by the specified procedures and requirements when passing through the line.

.7. VTS centre - the centre from which the VTS is operated. Each sub-area of the VTS may have its own sub-centre.

.8. VTS voyage plan - a plan which is mutually agreed between a VTS Authority and the master of a vessel concerning the movement of the vessel in a VTS area.

## how to obtain VTS INFORMATION

The services offered to the mariner by a VTS should be promulgated to vessels in internationally recognised marine publications. This should include details of the VTS, its capabilities, rules, regulations, requirements, radio frequencies and procedures. The information promulgated should be verified, or updated, at least annually. VTS areas should also be clearly defined on navigational charts.

## AIMs, GOALs, ROLEs AND FUNCTIONs OF VTS

### AIMS

VTS contribute to safety of life at sea, safety and efficiency of navigation and protection of the marine environment, adjacent shore areas, work sites and offshore installations from possible adverse effects of maritime traffic.

### GOALS

The goals of implementing VTS are as follows:

- Aid the Navigation Officer in the safe and efficient use of navigable waterways;

- Afford unhindered access to pursue commercial and leisure activities, subject to any restrictions that may exist;

- Contribute to keeping the seas and adjacent environment free from pollution.

### roles

As an auxiliary means for navigation, VTS is an aid to navigation. By monitoring the movements of vessels the VTS Operator is able to interact with participating vessels in accordance with established regulations and procedures within the VTS area.

### FUNCTIONS

VTS functions can be subdivided into internal and external functions. Internal functions are the preparatory activities that have to be performed to enable a VTS to operate. These include data collection, data evaluation and decision-making. External functions are activities executed with the purpose of influencing the traffic characteristics. They relate to the primary traffic management functions of rule-making, allocation of space, routine control of vessels and manoeuvres to avoid collisions, as well as to other management functions such as enforcement, remedial and ancillary activities.

Amongst the most important functions that a VTS may carry out are those related to, contributing to and thereby enhancing:

- Safety of life at sea;

- Safety of navigation;

- Search and Rescue;

- Efficiency of vessel traffic movement;

- Protection of the marine environment;

- Supporting maritime security;

- Supporting law enforcement; and

- Protection of adjacent communities and infrastructure.

## WHAT CAN VTS DO FOR Navigation Officers

As an active aid to navigation, VTS has real-time knowledge and may therefore provide the following benefits for Navigation Officers:

- Providing information about coastal, port and inland waterway infrastructure and its key and vulnerable locations, thereby improving maritime traffic efficiency.

- Providing information about vessel activities, including vessel position and intention, etc., thereby improving the awareness of information between vessels.

- Providing navigational assistance in difficult navigation conditions to ensure the safety of vessels.

- Providing knowledge of conventional port operations, including vessel route and port interface activities, thereby reducing the pressure in route planning and help Navigation Officers fully understand the navigation environment in complex traffic situations.

- Providing information concerning the coordination of medical assistance in emergency situations.

## TYPES OF VTS

### COASTAL VTS

The coastal VTS mainly involves vessel traffic through the VTS area. A coastal VTS can assist the safe and expeditious passage of shipping through coastal waters. It may also be involved in providing assistance to vessels before embarkation or disembarkation of a pilot. The types of service provided by a coastal VTS will depend on its legal basis.

### PORT /HARBOUR VTS

Port/Harbour VTS are mainly related to vessel traffic in ports, wharves or harbours. In order to ensure safety and efficiency, the movement of vessels within port areas may demand specific traffic management which not only minimizes incidents but also promotes economic results. A Port/Harbour VTS help prevent unnecessary delays and reduce the number of close quarter situations between vessels that may jeopardize the ports integrity.

Port VTS around the world ensure the economic profitability of ports and may also assist in, the safety of human life and the protection of the marine environment. The techniques used in port VTS may be different from a coastal VTS, because the navigation is in itself different. In port areas both the separation between vessels and the required under-keel clearances may be reduced. These two factors directly affect the navigation of vessels and therefore port VTS procedures.

A Port/Harbour VTS may promulgate port entry requirements and may issue instructions. Examples of such requirements/instructions are:

- The need to obtain clearances for entering/exiting VTS areas;

- The obligation of taking pilot;

- The obligation of taking tugs;

- The maximum speed in the area;

- The minimum separation distance between vessels;

- The assignments of anchor positions;

- The need to obtain overtaking permissions in narrow channel.

These functions are normally rendered under a TOS (TOS). In a port area, VTS may also provide a NAS (NAS), where vessels are assisted in complex navigational situations.

### VTS IN INLAND WATERS

The purpose of VTS in inland waters is to improve the safety and efficiency of navigation, safety of life and the protection of the marine environment and/or the adjoining waterway banks, nearby residents and enterprises from possible adverse effects of vessel traffic.

A part of the objectives of a VTS in inland waters may include the support of efficient transport and the collection of data and information that, as a consequence, may be required.

The benefits of implementing a VTS in inland waters are that it allows identification and monitoring of vessels, the strategic, tactical and operational planning of vessel movements, managing traffic by allocation of space, as well as the provision of navigational information and assistance.

A VTS in inland waters may provide an information service (INS), a NAS (NAS) and a TOS (TOS). Further information may be found in IALA Recommendation V-120, - ‘Vessel Traffic Services in Inland Waters’.

## DIFFERENCES AND CONNECTIONS BETWEEN VTS & Ship REPORTING SYSTEMS

### DIFFERENCES

In the VTS area VTS is proactive, VTS Operators provide Vessel Traffic Services to vessels through real-time monitoring of vessels.

Ship Routeing System and Ship Reporting System are passive in that vessel traffic management is conducted through predetermined rules.

### CONNECTIONS

- VTS, Traffic Separation Scheme and Ship Reporting System can be used independently or in combination.

- Ship Reporting System provides data for VTS traffic images.

- VTS is mandatory only within the territorial sea. When a vessel is required to participate in VTS in international waters, it can be achieved by setting up the Ship Reporting System in the relevant area or by a Ship Routeing System adopted by the IMO.

### TYPES OF SERVICE PROVIDED BY VTS

### INFORMATION SERVICE

#### Definition

An Information Service provides relevant information at appropriate time and on request for the promulgated VTS area.

#### General

- Information Service objects are vessels within the VTS area;

- The information promotes the safe navigation of the vessel;

- VHF working channel is often the preferred communication method;

- The VTS centre may provide the broadcast at fixed time and intervals.

#### Examples of Information Service

- Updated information about vessel traffic and routes contributing to safety;

- Navigational warnings relating to the safety of vessels;

- Meteorological and hydrological information affecting navigational safety of vessels;

- Failure of navigational aids affecting the navigational safety of vessels;

- Other information relating to the navigational safety of vessels.

#### Occasion to Provide Information Service

- Information broadcasted at fixed time or intervals;

- When deemed necessary by the VTS;

- When requested by vessels.

#### Information Service Content

- Examples of the types of information that may be provided by the VTS operating an INS

| Information related to | Examples |
| --- | --- |
| Navigational situations (including traffic and route information) | Position, identity, destination of vessels and the intention of other traffic; Amendments and changes in promulgated information concerning the VTS area such as boundaries, procedures, radio frequencies, reporting points; the mandatory reporting of movements; Limited manoeuvrability that may impose restrictions on the navigation of other vessels, or any other potential hindrances; Suspension or change of routes; etc. |
| Navigational warnings | Dangerous wrecks, obstacles not otherwise promulgated, diving operations, vessels not under command, etc. |
| Meteorology | Information that will include the speed and direction of the prevailing wind, direction and height of the waves, visibility, atmospheric pressure, the formation of ice, etc. |
| Meteorological warnings | Gale, storm, tsunami, restricted visibility, etc. |
| Hydrography | Information that will include factors such as the stability of the seabed, sea depth, the accuracy of surveys, tidal ranges, tidal streams, prevailing currents and swell, etc. |
| Electronic navigational aids | The availability of electronic navigational aids such as: GNSS, Loran, LRIT, DGPS, AIS, RACON etc. |
| Other information | Port information, pilot or tug request, cargo information, health condition, PSC, ISPS, etc. |

### TOS

#### Definition

A TOS (TOS) is a service to prevent the dangerous maritime traffic situations and to provide for the safe and efficient movement of vessel traffic within the declared VTS area.

#### General

A TOS may be provided in part, or all, of the declared VTS area.

While VHF may be the primary means of providing a TOS, any available communication method may be used.

#### Examples of TOS provided

A TOS should be provided when the VTS is authorized to provide such services, for instance when:

- vessel movements need to be planned or prioritised to prevent congestion or dangerous situations;

- special transports or vessels with hazardous or polluting cargos may affect the flow of traffic which then need to be organised;

- an operating system of traffic clearances or sailing plans, or both, have been established;

- the allocation of space needs to be organised;

- mandatory reporting of movements in the VTS area has been established;

- special routes should be followed;

- speed limits should be observed;

- the VTS observes a developing situation and deems it necessary to interact and coordinate vessel traffic;

- nautical activities (e.g. sailing regattas) or marine works in‐progress (such as dredging or submarine cable‐ laying) may interfere with the flow of vessel movement.

#### TOS Content

- Examples of types of information that may be provided by the VTS within a TOS

| Information related to | Examples |
| --- | --- |
| Traffic clearance | Empower authorization under conditional circumstances to a vessel when: prior to or entering a VTS area; - departing from a berth or an anchorage position within a VTS area; - entering into a fairway within a VTS area; or - prior to commencing a manoeuver that may be detrimental to safe navigation. Examples of conditions: - a VTS sailing plan before entering a VTS area; - lock and bridge passage planning; - report position at determined reporting point/line/pilot station; - use a second fairway in case of bad visibility/weather; - use a tug boat in case of strong wind; - dredging or compass swing in confined waterway. |
| Anchorage | Examples of anchorage situations: - organizing the movements to/from an anchorage position/area; - assignment of an anchorage position; - assisting vessels into anchorage position. |
| Enforcement | Examples of enforcement: - speed limits; - adherence to rules regarding traffic routeing measures; - pilotage requirements; - other traffic regulations and possibly local by‐laws |
| Waterway (sea, channels and fairway) management | Examples of management measures: - the use of one‐way traffic as an alternative of two way traffic, depending on the dimensions of vessel or the weather conditions; - organizing other traffic when a vessel has passed point of no return; - slot management to allocate vessels in a time window; - organizing the traffic concerning vessel dimensions in comparison to fairway restrictions; - instruct vessels when overtaking is not permitted; - establish and organise vessel safety zones in case of particular operations; - establish and organise exclusion zones; - instruct vessels to keep clear from special areas/positions; - organizing the traffic as regards to meteorological, hydrographical or other restrictions such as visibility, wind speed, current, sea state and under keel clearance. |

### NAS

#### Definition

A NAS is a service that provides essential and timely navigational information to assist in the on board navigational decision-making process and to monitor its effects. It may also involve the provision of navigational advice and/or instruction.

#### General

- NAS requires positive identification and continuous communication throughout the process. If time permits, checks should usually be made prior to commencement of the provision of NAS to assess the capability of the vessel to respond to the guidance given.

- NAS should be provided within the declared VTS area and/or in specified areas or sectors within the VTS area.

- It is recommended as best practice that message markers are always used when delivering NAS irrespective of the language ability of the recipient.

#### Examples of NAS provided

- NAS should be provided when the VTS is authorised to provide such services, and may be provided when deemed necessary by the VTS, or upon request from the vessel.

- NAS may for instance when a vessel:

- Is in risk of grounding;

- is deviating from the VTS sailing/passage plan;

- is unsure of its position or unable to determine its position;

- is unsure of the route to its destination;

- needs assistance to find suitable anchoring position;

- experiences defects or deficiencies, such as navigation or manoeuvring equipment failure;

- experiences severe meteorological conditions (e.g. low visibility, strong winds);

- is facing a risk of colliding with a fixed or floating object – or with another vessel;

- needs assistance to support the unexpected incapacity of a key member of the bridge team.

#### Contents of NAS

- Examples of the types of information that may be provided by a VTS operating NAS

| Information related to NAS | Examples |
| --- | --- |
| Request and identification | - availability of NAS, start and end of NAS; - request for vessel identification such as position, course made good and speed over the ground; - status of vessel's equipment; etc. |
| Navigational information (including position and course information) | Examples provided to an individual vessel: - provide range and bearing from fixed objects, fairway/channel or way‐points; proximity to navigational hazards, etc. - provide information related to navigating into a channel/fairway/lane (i.e. track is parallel/diverging/converging with/from/to reference line); etc. |
| Advice (or instruction) | - advise (or instruct) a vessel to alter the course, speed; - advise (or instruct) to keep clear from area/position, close up/drop back on/from vessels; etc. |
| Warning | Diverging from the recommended track towards dangerous wrecks, obstacles not otherwise promulgated; diving operations; vessels not under command; etc. |

### ALLIED SERVICES AND OTHER SERVICES

#### Definition

Allied services are services actively involved in the safe and efficient passage of the vessel through the VTS area. It is supported by ports, pilotage, salvage, customs, military, etc. to increase the safety and efficiency of transportation, protect the environment, and improve the effectiveness of VTS.

Other Services refer to services other than the allied services, which may use VTS data to more effectively undertake their work (e.g. ensuring local security or preventing illegal imports).

#### Interaction with Allied Services and Other Services

VTS may establish procedures to deal with incidents such as

- Search and Rescue

- Marine emergency events

- Natural Disaster

- Security

For further details, see annex C to IALA Guideline 1102 (VTS Interaction with Allied or Other Services)

## Participation in VTS

Within the territorial sea, VTS implemented and operated by coastal States may be mandatory. Whether the VTS is mandatory depends on the governing rules and regulations of the coastal State to which it belongs. In the mandatory VTS area, VTS may be applicable to all vessels or there may be exemptions to certain types and sizes of vessels. It should be noted that vessels not required to participate in a VTS may still choose to do so.

# COMMUNICATION BETWEEN THE NAVIGATION OFFICER AND THE VTS CENTRE

## TOOLS

VHF is often the main communication tool, and it is generally performed on the channel specified by the VTS centre. Other tools include AIS binary messages, DSC, email and radiotelephony on MF/HF etc.

## LANGUAGE

Communication may be executed in English or in the common language of the Navigation Officers and VTS operator.

## REQUIREMENTS OF COMMUNICATIONS

The following requirements should be paid attention to when Navigation Officers are communicating with the VTS centre:

- The vessel should maintain watch on VTS working channel in VTS area.

- The specific implementation of information sent by the VTS centre should be made by the Navigation Officers since information is result-oriented.

- Communication language should be concise and unambiguous.

- SMCP including Message markers should be used.

- It should be noted that information issued by the VTS centre does not affect the statutory responsibilities and duties of Navigation Officers.

## SMCP AND MESSAGE MARKERS

To ensure the communication efficiency, Navigation Officers should have a good grasp of SMCP and use them correctly. Eight message markers are defined in SMCP. In the process of communication, the appropriate message mark should be used so as to increase the probability of correctly understanding the message.

In table 4 and 5 below the different uses and expected responses of the message markers are shown. It should be noted that a distinction should be made between communication from a VTS to a Navigation Officer – and from a Navigation Officer to a VTS.

- 7 Message markers used by VTS operator to the Navigation Officer

| Message marker | Definition | The response of the Navigation Officer |
| --- | --- | --- |
| INSTRUCTION | A communication whereby the message implies the intention of the sender to influence the recipient by a regulation | Without affecting the safety of the vessel or the safety of navigation, the vessel shall carry out and comply with the content of the instructions. If it is not possible to follow the instruction, the Navigation Officer should report and explain the situation to the VTS centre. |
| ADVICE | A communication whereby the message implies the intention of the sender to influence the recipient by a recommendation. | The vessel should consider the proposal carefully and decide whether to carry out it. If it is not possible to carry out the advice, the Navigation Officer should report this to the VTS centre and explain it. |
| WARNING | A communication whereby the message implies the intention of the sender to inform others about danger. It may be used to convey potentially dangerous situations or observed developing situations | The vessel should pay great attention to it and respond immediately and take measures to deal with the danger. |
| INFORMATION | A communication whereby the message is restricted to observed facts, situations, etc. and is preferably used for navigational and traffic information. | The Navigation Officer should pay attention to the information and ascertain whether the information has an impact on navigational safety. |
| QUESTION | A communication whereby the message marker indicates that the following message is of an interrogative character. | The Navigation Officer should understand that the following is a question which requires an answer. |
| ANSWER | A communication whereby the message marker indicates that the following message is the reply to a previous question. | The VTS is responding to a question posed by the Navigation Officer |
| REQUEST | A communication whereby the message marker indicates that the following message is asking for action from others with respect to the vessel. | The Navigation Officer should acknowledge the content of the request. |

- 5 Message markers used by Navigation Officers to the VTS centre

| Message Marker | Definition | When the Navigation Officers should use it |
| --- | --- | --- |
| INFORMATION | A communication whereby the message is restricted to observed facts, situations, etc. and is preferably used for navigational and traffic information. | Providing information to the VTS Centre |
| QUESTION | A communication whereby the message marker indicates that the following message is of an interrogative character. | Sending question to the VTS Centre |
| ANSWER | A communication whereby the message marker indicates that the following message is the reply to a previous question. | Answering the inquiries from the VTS Centre |
| REQUEST | A communication whereby the message marker indicates that the following message is asking for action from others with respect to the vessel. | Sending information to a VTS Centre for requests, needs or assistance. |
| INTENTION | A communication whereby the message marker indicates that the following message informs others about immediate navigational action intended to be taken. | Delivering the ship’s upcoming action to the VTS and/or to other ships |

## COMMUNICATION PROCEDURES

Communications between Navigation Officers and VTS centre should be carried out in the following procedures:

- The communication process should clearly indicate the identity of the vessel and the VTS centre.

- The Navigation Officers should be aware of the assigned frequencies/channels of the VTS.

- The Navigation Officers should be aware of the reporting procedures of the VTS.

- The Navigation Officer should be aware of the limitations within radio telecommunication, both when receiving and transmitting communication with a VTS.

# HOW TO RAISE THE VTS AWARENESS OF NAVIGATION OFFICER

## VTS and navigation officer TRAINING

As an important external resource in the VTS area, VTS should be effectively utilized by the Navigation Officer. In the meanwhile, effective communication and cooperation between the Navigation Officer and VTS would be more beneficial to navigational safety and efficiency. Therefore, VTS should be included in the mandatory training for Navigation Officers, and be reflected in the examination and assessment. Further, VTS should also be included in other types of training – such as Bridge Resource Management (BRM) etc.

### VTS KNOWLEDGE IN THEORY TRAINING

VTS theory training includes the following contents:

- Introduction of VTS, (i.e. types, functions, responsibilities, procedures and requirements of VTS);

- General requirements for transiting through the VTS area

- Benefits of VTS to the Navigation Officer

- Communicating methods and requirements between the Navigation Officer and VTS.

### VTS knowledge IN PRACTICAL TRAINING

Once a vessel enters a certain VTS area, it would be affected by VTS to some extent. The introduction of VTS in practical training is helpful for Navigation Officers to be familiar with the operation of vessels in the VTS area.

- Examples of practical training projects

| Training project | specific items | Evaluation elements about VTS | Evaluation requirements |
| --- | --- | --- | --- |
| Passage plan | 1)Develop a passage plan to pass through the VTS area | (1) Identify the corresponding VTS tasks; (2) Obtain the necessary VTS information; (3) Utilization and arrangement of VTS resources; (4) Organization and arrangement of correspondence personnel with VTS Centre. | VTS influencing factors should be fully considered and reflected in the formulation of the passage plan. |
|  | 2)Foreseeing accidental events and Countermeasures | (1) Reasonably foresee possible emergency events in the VTS area and draw up corresponding measures (2) Identify possible contingencies (3) Procedures of responding to occasional incidents, including reporting and receiving information from the VTS Centre; (4) Response to incidents including a request for help from the VTS Centre; (5) Organization and arrangements for incidents, including coordination with the emergency response of the VTS Centre. |  |
| Actual operation through a certain VTS area | Resources and management | (1) Identify available VTS resources; (2) Coordination and use of VTS Resources. | In the VTS area, vessels should report to VTS as required, receive service, seek help from VTS centre, and cooperate with VTS centre to deal with the occasional incident. |
|  | Bridge team work | (1) Keep smooth communication with VTS Centre; (2) Maintain a high level of VTS awareness. |  |
|  | Contingency and emergency | (1) Emergency responding procedures include reporting to the VTS Centre; (2) VTS Centre participates in the decision-making of contingencies.; (3) VTS Centre participates in the process of handling accidental events; (4) Processing results are reported to the VTS Centre. |  |

## ADDING THE VTS CONTENTS IN NAVIGATIONAL EQUIPMENT TRAINING

### PRACTICAL TRAINING OF VHF

VHF is the main tool for communication between bridge and VTS centre. The simulating training of communication with VTS centre during VHF practical training is helpful to the communication and coordination between vessels and VTS centre in VTS area.

- Examples of a communication training project using VHF with the VTS Centre

| Training project | Evaluation elements |
| --- | --- |
| Communication language | 1) Common language used with the VTS centre; 2) English. |
| Communication methods and skills | 1) Active listening skills, including inquiries, feedback and rewording. 2) Transpositional consideration |
| Communication requirements | 1) Clear, concise, accurate, timely and meaningful communication; 2) Reword the information received 3) Appropriate use of message markers 4) Note that any VTS information is results-oriented. |
| Reporting on vessel intention | 1) Report vessel dynamic information; 2) Report vessel voyage information; 3) Answering queries from the VTS Centre. |
| Applying for INS | 1) Clarify the types and requirements of the information required; 2) Call the VTS Centre and apply for Information Services; 3) Receive relevant information; 4) End communication. |
| Applying for NAS | 1) Clarify circumstances of the required NAS; 2) Call VTS Centre and apply for NAS; 3) Confirmation of commencement of NAS; 4) Maintain continuous communication with the VTS centre; 5) Confirm termination of NAS. |

### ECDIS PRACTICAL TRAINING

The use of ECDIS makes it easy to identify VTS coverage, and location of the VTS reporting line / point.

- Examples of VTS factors that can be considered in ECDIS practical training

| Training project | Evaluation elements |
| --- | --- |
| Route design through the VTS area | 1)Clarify coverage of VTS and location of VTS reporting lines / reporting points; 2) Passage planning should meet the requirements of the VTS area; 3) Proper use of VTS as a navigation aid. |
| Navigational Monitoring in VTS area | 1) Annotate VTS area and VTS reporting line; 2) Accept the monitoring of the VTS Centre; 3) Navigational decision based on the Information provided by VTS Centre. |

## CONSIDERATION OF VTS FACTORS IN PRACTICAL TRAINING of VOYAGE PLANNING AND PREPARATION

In the voyage plan and voyage preparation training, the ship may be set to sail through a VTS area, and the VTS implication should be fully considered during the training process.

- Examples of how VT may be considered in voyage planning and preparation practical training

| Training project | Evaluation elements |
| --- | --- |
| Obtain VTS Information via navigational publications | 1) Look up VTS related navigational publication appropriately. 2) Obtain full details of requirements to be met and procedures to be followed in the VTS area. |
| Estimate the time entering and leaving VTS area | 1) The time of entering into VTS area is in compliance with the relevant regulations; 2) Report and requirements of vessels entering or leaving VTS area; |
| Passage planning in VTS area | 1) Routes should meet navigational requirements in VTS area. 2) Sailing should take into account the services provided by the VTS Centre and its impact 3) prepare suggestions for a sailing plan |
| Considerations for ships in the VTS area | 1)Estimated arrival / departure from VTS area，noting the format, contents, time, and manner of the estimated arrival / departure report; 2) Entering VTS area，Noting the establishment of contacts with the VTS Centre, initial information exchange requirements, communications and reporting requirements, and dynamic planning arrangements; 3)When vessels passing through VTS area，it is necessary to note the mandatory requirements for ships in the VTS area, types of services provided by the VTS, communication and reporting requirements; 4) Anchoring in VTS area, noting the designated anchorage, communications and reporting requirements; 5) Berthing / unberthing in VTS area, noting the arrival berth report, maintenance of communication watch, restrictions on other passing-by vessel , and the report before unberthing ; 6) Departure from VTS area, taking note of the pre-departure report; 7) Traffic control in VTS area, paying attention to requirements of traffic control under adverse weather conditions in VTS area, Such as poor visibility, strong currents or tidal currents, strong winds and underwater activities, etc., which may affect navigation safety, and noting restrictions on vessel movements, prohibitions or additional requirements of reporting. |

## ADDING VTS CONTENTS TO NAVIGATION SIMULATOR TRAINING

Marine simulator training is a comprehensive practical training for Navigation Officers. VTS training may simulate navigating, mooring, berthing, unberthing, anchoring and responding to emergencies in the VTS area.

The use of active VTS Operators, playing the role of either VTS Operator or navigator on other ships, may contribute to the outcome of the course and enhance the Navigation Officers’ knowledge of VTS in general.

- Examples of VTS training in Marine Simulator training

| Training project | specific items | Evaluation elements |
| --- | --- | --- |
| Sailing | 1) Vessel estimated arrival / departure from VTS area | Submission of estimated arrival / departure reports in accordance with required format, content, time, manner, etc. |
|  | 2) Entering VTS area | (1) Establish contact with the VTS centre; (2) Exchange Initial information; (3) Communications and reporting; (4) Comply with dynamic planning arrangements. |
|  | 3) Passing through VTS area | (1) Receive vessel traffic services from the VTS Centre; (2) Communications and reporting. |
|  | 4) Departure from the VTS area | (1) reporting before departure; (2) Develop VTS sailing plan. |
|  | 5) Traffic control in VTS area | (1) Comply with traffic organization requirements; (2) Restrictions on vessel dynamic movements; (3) Communications and reporting. |
| Anchoring | Anchoring in VTS area | (1) designated Anchorage appointed by VTS Centre, pilot or ship; (2) Communications and reporting. |
| Berthing/ unberthing | berthing / unberthing in VTS area | (1) Arrival berth report; (2) Stand-by on VTS working VHF channel; (3) Report before unberthing. |
| Responding to emergencies | 1)SAR MOB | (1) Timely reporting to VTS Centre, including the basic information of the vessel, the time and position of the person falling into the water, the current environment, the hydrometeorological conditions, the rescue measures and the assistance needed, etc. (2) Issue a navigational warning; (3) Take full consideration of instructions from VTS Centre; (4) Take SAR actions with the VTS centre (5) Maintain communications and reporting. |
|  | 2) Encountering poor visibility | (1) Report visibility around the vessel to VTS Centre timely; (2) Apply for information as appropriate (3) Apply for NAS as appropriate (4) Comply with the traffic organization (5) Maintain communications and reporting. |
|  | 3) Main engine/ Steering gear out of order | (1) Timely reporting to VTS Centre, including the time main engine/ steering gear out of order, vessel position, reasons and assistance needed. (2) Apply for information as appropriate (3) Apply for NAS as appropriate (4) Comply with the traffic organization (5) Take necessary measures to cooperate with VTS Centre (6) Maintain communications and reporting. |
|  | 4)collision | (1) Timely reporting to VTS Centre, including the time of the collision, the position of the vessel, the casualties, the damage to the vessel, the condition of the cargo, the name of colliding vessel, and the assistance required. (2) Apply for information as appropriate (3) Apply for NAS as appropriate (4) Cooperate with VTS Centre to carry out rescue operation (5) Maintain communications and reporting. |
|  | 5) Grounding | (1) Timely reporting to VTS Centre, including the time of grounding, vessel position, casualties, damage to the vessel, cargo situation, draft in fore, aft and middle ship , the part which the vessel ran aground, the geology of the bottom of grounding, and the assistant required. (2) Apply for information as appropriate (3) Cooperate with VTS Centre to carry out rescue operation (4) Maintain communications and reporting. |

## COMMUNICATION ACTIVITY BETWEEN NAVIGATION OFFICERS AND VTS CENTRE

Strengthening the communication between Navigation Officers and VTS centres is helpful to understand the purpose of VTS and thereby to enhance their VTS awareness.

Benefits of communication activities：

- VTS Centre provides information directly;

- Navigation Officers can give feedback directly to VTS personnel;

- Deepen the understanding of Navigation Officers of VTS duties and responsibilities.

The main means of communication include visiting to the VTS Centre and its facilities. In addition, visiting the website of the VTS authorities, and pay attention to the publication of the VTS centre or the VTS authorities, etc.

## STRENGTHENing PILOT'S FAMILIARITY WITH VTS

The participation of the pilot is both as a user and provider of information - is of benefit to the VTS. Therefore, VTS familiarisation for pilots will strengthen their awareness of VTS and their cooperation with VTS will enhance the safety of navigation.

# ACRONYMS & DEFINITIONS

## ACRONYMS

IALA	International Association of Marine Aids to Navigation and Lighthouse Authorities – AISM

IMO	International Maritime Organization

VTS	Vessel Traffic Service

VTSO	Vessel Traffic Service Operator

INS 	Information Service

TOS	TOS

NAS	NAS

ISPS	International Vessel And Port Facility Security (Code)

BRM	Bridge Resource Management

VHF	Very High Frequency

AIS	Automatic Identification System

ECDIS	Electronic Chart Display And Information System

SMCP	Standard Marine Communication Phrases

CPA	Closest Point Of Approach

DCPA	Distance to Closet Point Of Approach

TCPA	Time to Closet Point Of Approach

SAR	Search And Rescue

MOB	Man Overboard

## DEFINITIONS

The definition of terms used in this Guideline can be found in the International Dictionary of Marine Aids to Navigation (IALA Dictionary) at http://www.iala-aism.org/wiki/dictionary.Acronyms.

# REFERENCES

- IALA VTS MANUAL 2016

- IMO RESOLUTION A.857(20) GUIDELINES FOR VESSEL TRAFFIC SERVICES

- ITU RADIO REGULATIONS, VOLUME IVE, RECOMMENDATION ITU.R M.1171.0 AND SUBSEQUENT CHAPTERS

- IMO RESOLUTION A.918(22) IMO STANDARD MARINE COMMUNICATION PHRASES (SMCP)

- SOLAS 1974- INTERNATIONAL CONVENTION FOR THE SAFETY OF LIFE AT SEA CHAPTER V, REGULATION 12 VESSEL TRAFFIC SERVICES.

- STCW 1978 - (11) CONVENTION AND CODE as amended

- IALA GUIDELINE 1089 PROVISION OF VTS SERVICES (INS,TOS&NAS);

- IALA GUIDELINE 1071 ESTABLISHMENT OF A VESSEL TRAFFIC SERVICE BEYOND TERRITORIALSEAS;

- IALA GUIDELINE 1102 VTS INTERACTION WITH ALLIED OR OTHER SERVICES;

- IALA RECOMMENDATION V-120 VESSEL TRAFFIC SERVICES IN INLAND WATERS (EDITION2);

- IALA RECOMMENDATION V-127 Operational Procedures for Vessel Traffic Services  (EDITION2);

- ADMIRALTY LISTOF RADIO SIGNALS,ALRS

- INTERNATIONAL CONVENTION ON MARITIME SEARCH AND RESCUE,1979

- 1132-ED.1-VTS-VHF-VOICE-COMMUNICATION

- STW 43-9 - VTS TRAINING FOR NAVIGATION OFFICERS (IALA)

- 1131-ED.1-SETTING-AND-MEASURING-VTS-OBJECTIVES

- OPERATING GUIDE of VESSELs IN VTS AREA

- REPORTING AND ACCEPTING ENQUIRY AS REQUIRED

  - REPORTING TO VTS CENTRE

  - Examples of reporting to VTS centre

| Type of Report | Content of Report |
| --- | --- |
| Arrival/Departure Report | Sailing Plan, ETA/ETD Vessel’s Length/Draft, Cargo’s name, etc. |
| Passing Reporting Line Report | Vessel’s particular, such as LOA, Breadth, GT, etc. Voyage data, such as Draft, Last/Next Port, Cargo’s Name, etc. |
| Berthing/Unberthing Report | Sailing Plan, ETA/ETD, Vessel’s Length/Draft, Last/Next port of call, etc. |
| Dropping/Heaving up Anchor Report | Time of dropping/heaving up anchor, anchor position, length of shackles released, distance to other vessel, meteorological and hydrological conditions, etc. |
| Other report | The reporting content depends on the specific circumstances, such as occurring emergency situations, finding aids to navigation abnormal, etc. |

  - ACCEPTING ENQUIRY FROM VTS CENTRE

When the vessel navigating、anchoring or operating in the VTS area, the Navigation Officers should keep a continuous watch on the designated radio frequencies / channel, accept enquiry from VTS centre at any time, and report concerning information to VTS centre.

- ACCEPTING/REQUESTING INFORMATION

  - ACCEPTING INFORMATION

The vessel can receive the broadcasting information at fixed time and intervals from VTS centre, accept the point-to-point information such as intention of vessels, meteorological and hydrological conditions, navigational warning or any safety information for navigating by information、warnings、advice or instructions.

  - REQUESTING INFORMATION

Examples of Navigation Officers requesting information from VTS centre:

- The Navigation Officers should confirm the content and types of information they need;

- Connecting with VTS centre and requesting information;

- Receiving the information from VTS centre;

- Applying for the information to assist the on board decision-making process.

- ACCEPTING/REQUESTING NAS

- Examples of Navigation Officer operating in NAS

| Operating Items | Content and Procedure |
| --- | --- |
| Request and identification | Availability of NAS, assure the time of starting and ending NAS. Report to VTS centre for vessel identification such as position, course made good and speed over ground Report to the VTS centre the status of vessel’s equipment Ensure continuous watch on the designated radio frequencies /channel |
| Receiving navigational information (including position and course information) | Keeping a continuous communicating with VTS centre, navigating based on the information provided by VTS centre, such as the range and bearing from fixed objects, fairway/channel or way-points、the information concerning danger and a channel/fairway/ lane. |
| Receiving advice (or instructions) | Follow the advice or instructions from VTS centre to alter the course、speed. Accepting the advice、instructions、warning information issued by VTS centre and applying for the information to assist in the on board navigational decision-making process. Following the advice or instructions to keep clear from hazards、close up /drop back on/from vessels. |
| Continuance and end the operation | Feeding back efficiently and timely and reporting the problems occurred in the whole process to VTS centre. Confirming the time of end with VTS centre |

- ACCEPTING TOS

- Examples of Navigation Officer operating in TOS

| Type of TOS | Content and Procedure |
| --- | --- |
| Traffic Clearance | Request entering or departing VTS area; Request departing from a berth or an anchorage position within VTS area; Request entering into a fairway within VTS area; Report VTS sailing plan, sluice gate and bridge passing plan to VTS centre before entering VTS area; Report position at determined reporting point/line/pilot station ; Using a backup fairway in case of bad visibility/weather. |
| Anchorage | Report the movements arriving/leaving an anchorage position/area; Proceed to the anchor position assigned by VTS centre, pilot or ship; Request assisting vessels in entering anchor position from VTS centre. |
| Waterway (sea, channels and fairway) management and enforcement | Comply with the speed limits; Comply with the rules regarding traffic routeing measures; Comply with the pilotage requirements; Comply with other traffic regulations and relevant local rules; Keep clear of the vessel safety zones which established and organized for keeping clear of particular operation; Keep clear of the special area/position; Do not overtake in the area where overtaking is prohibited. |

- COOPERATION WITH VTS CENTRE IN ALLIED SERVICE

  - COOPERATION WITH SEARCH AND RESCUe ACTIONs

  - Examples of Navigation Officer operating in SAR

| Type of operating | Contents and procedure |
| --- | --- |
| Search and rescue the crew of your own vessel when encountering accidents. | (1) Report MOB to VTS centre, including Time, Vessel’s position, Wind speed and direction, etc. Broadcasting MOB information on VHF in order to get assistance from other vessel around the waters. (2) Keep in touch with VTS centre and report the latest search and rescue information to VTS centre in time. |
| Search and rescue the crew of other vessel when the vessel encounters accidents. | (1) The master should be informed timely when receiving the MOB information. The master shall actively cooperate with VTS centre to participate in the search and rescue operations without serious danger to the vessel, crewmember, and passengers. (2) Report the scene situation to the VTS centre including own vessel’s name, call sign and position. Following the arrangements and instructions issued by on-scene coordinator, and following the mission assigned by VTS centre. (3) Keep in touch with VTS centre and on-scene coordinator. Report the latest search and rescue situation to VTS centre in time. (4) When VTS centre or on-scene coordinator issues an order of terminating the search and rescue operation or the vessel is no longer required to take part in the actions, the vessel can carry out the original sailing plan. |

  - COOPERATION IN OTHER EMERGENCY SITUATIONS

  - Examples of Navigation Officer operating in other emergency situations

| Type of emergency | Content and procedure |
| --- | --- |
| Collision | (1)The Navigation Officer should report the time, position, casualties, cargos on board, vessel damage condition, risk of sinking and assistance needed to VTS centre. (2) If there is a risk of sinking, it should prepare to abandon the vessel based on muster list. If there is no risk of sinking, the damage of the vessel, the cargo holds, ballast tank and oil tank should be checked. And the information should be reported to the VTS centre timely. (3) Broadcasting the navigational warning, requesting the vessels around stay away or seeking assistance. (4) Keeping in touch with VTS centre and cooperating with VTS centre to carry out rescue actions. |
| grounding | (1) The Navigation Officer should report the time, position, casualties, part of aground, damage of vessel, cargo on board, any oil spill pollution and assistance needed to the VTS centre. (2) Sounding the vessel’s six-sided draft and the depth of surrounding area. Checking whether flooding, damage of vessel, part of flooding, amount of flooding water and reporting to VTS centre timely. (3) Keeping in touch with VTS centre and cooperating with VTS centre to carry out rescue action. |
| Fire | (1)The Navigation Officer should report the time, position, casualties, cargo on board, risk of sinking and assistance needed to the VTS centre. (2) Taking action as per muster list and reporting the on-scene fighting situation to VTS centre. (3) Keeping in touch with VTS centre and cooperating with VTS centre to carry out rescue action. |
| Pollution | (1) The Navigation Officer should report the time, position, casualties, cargo on board, risk of sinking and assistance needed to the VTS centre. (2) Taking action as per muster list and reporting the on-scene situation to VTS centre. (3) Keeping in touch with VTS centre continuously and cooperating with VTS centre to carry out rescue actions. (4) Proceed to the designated area according to the VTS centre. |

  - COOPERATION IN VESSEL SECURITY INCIDENT

When vessel security incident occurs in the VTS area, the Navigation Officer should take action as the following procedure:

- Reporting the number of casualties and basic situation;

- Reporting the number of pirates or robbers, the type, number, size, speed of vessel they using and what weapons, explosives they are using.

- Reporting the measures taken by the vessel and assistance needed.

- Reporting the damage of the machine, the condition of equipment and the ability to manoeuvre.

- Taking appropriate actions under the guidance of the VTS centre.

- Keeping in touch with VTS centre.

- BASIC WORDS

The proper use of the basic words improves unambiguous communication from shore to vessel and vice versa.

- Table 20 Basic words and it’s meaning

| Basic Word | Meaning |
| --- | --- |
| Approved | Permission for proposed action granted |
| Check | Examine (something) in order to determine its accuracy, quality, or condition, or to detect the presence of something |
| Clearance | Authorised to proceed under the conditions specified |
| Confirm | I request verification of: (clearance, instruction, action, information) |
| Contact | Establish communications with… (your details have been passed) |
| Correction | A change that rectifies an error or inaccuracy |
| Disregard | Ignore last message |
| Incorrect | That is not correct |
| I repeat | I will state my message again |
| I spell | Phonetic spelling follows |
| Maintain | Continue in accordance with the condition(s) specified or in its literal sense, e.g.‘’Maintain your course’’ |
| Out | End of transmission. No answer is required or expected |
| Over | End of transmission. An answer is expected |
| Read back | Read back to me the message as you received it |
| Report | Pass requested information |
| Say again | A request to retransmit all or a portion of a transmission |
| Standby | Wait and I will call you |