IALA Guideline

DraFT G1111-3

Producing Requirements for RADAR

Functions, Performance And radar specifIC AcceptANce

VTS 51-52 intersessional work, VTS52 input.

Updated with outstanding’s from VTS 51 including:

- Completion of calculation methods, also including Magnetron radars and ll relevant frequency bands.

- Comparison of technologies, guideline evaluation of dynamic range, formulas for evaporation ducting and formulas for target separation added.

- examples of performance calculations to be included for imaginary, simplified radar configurations developed

- Editorial changes and clarifications including restructuring of Annexes to avoid repletion of arguments and reduce page count. Acceptance chapter moved to Annex to reduce main body of document.

Edition x.x

Date (of approval by Council)

urn:mrn:iala:pub:g1111-3(2’nd draft)

Revisions to this document are to be noted in the table prior to the issue of a revised document.

| Date | Details | Approval |
| --- | --- | --- |
|  | Edition 1.0 This document originated from Guideline G1111 which has been subdivided into 13 sub-guidelines, including this document. Document structure revised, Basic, Standard and Advanced substituted with guidance on specific areas including Inland VTS, Ports, Ports Approach and Coastal VTS. Guidance on offshore related VTS and Acceptance of VTS Radar Systems added. Measurements in Metric terms adopted for Inland Waterways only. (Note - G1111 originated from annex of Recommendation V-128 Ed 3 in May 2015) |  |

1	INTRODUCTION	7

1.1	The IALA G1111 guideline series	7

2	DEFINITIONS	8

2.1	General Terms	8

2.2	Specific Terms	8

2.3	Speciffic IALA Definitions	10

2.3.1	Target Types	10

3	References	11

4	Abbreviations	12

5	Operational Areas	12

5.1	Inland Waterway	13

5.2	Port	13

5.3	Port Approach	13

5.4	Coastal Area and OFF-SHORE	14

6	Producing Functional and Performance requirements	14

6.1	Core Radar Performance	15

6.1.1	Radar Coverage	15

6.1.2	Target detection and range performance	16

6.1.3	Target Separation	16

6.1.4	Target Positional Accuracy	17

6.2	Radar Performance Versus the Dynamic EnvirOnment	17

6.2.1	False Echoes	17

6.2.2	Handling of Dynamic Range, targets and surroundings	17

6.2.3	Handling of environmental Conditions	18

6.3	Radar Functions and Other Features	18

6.3.1	Target Positional Update rate	18

6.3.2	Operator Functions	18

6.3.3	Operational Outputs	18

6.3.4	Clutter and Noise Reduction / Management	18

6.3.5	Target tracking	19

6.3.6	Interfaces	19

6.3.7	robustness, availability, and serviceability	19

ANNEX A	VTS Radar Considerations	21

A.1.	Radar Technologies	21

A.1.1.	Pulse Radar	21

A.1.2.	Pulse Compression Radar	21

A.1.3.	Frequency Modulated Continuous Wave Radar	22

A.2.	Frequency Bands	22

A.3.	Antennas	23

A.3.1.	Antenna Principles	23

A.3.2.	Antenna Polarisation	23

A.4.	Target caracteristics	24

A.4.1.	Radar Cross Section	24

A.4.2.	Target Models versus Detailed Characteristics	24

A.4.3.	Multipath Effects	26

A.5.	Radar coverage	26

A.5.1.	Radar Configuration Examples	26

A.5.2.	Minimum detection range	27

A.5.3.	Radar Horizon	28

A.5.4.	Analysis of Radar Detection Performance	28

A.6.	Target separation	36

A.6.1.	Azimuth Separation	36

A.6.2.	Range Separation	37

A.7.	Target Positional Accuracy	38

A.8.	Target Positional update rate	39

A.9.	Dynamic characteristics	39

A.9.1.	Dynamic range	39

A.9.2.	Sidelobes	42

A.10.	Environmental Influence	43

A.10.1.	Precipitation and Sea	43

A.10.2.	Reduced Visibility, Fog, sand, Dust	44

A.10.3.	Air Mass (Propagation)	44

A.11.	Other Influencing Factors	48

A.11.1.	Shadowing Effects	48

A.11.2.	Interference	48

A.11.3.	Radar Susceptibility	49

A.11.4.	Radar Compatibility with Other Users	49

A.11.5.	Influence from Wind Farms	49

A.12.	Signal Processing and Tracking	50

ANNEX B	Acceptance of VTS Radar Systems	51

B.1.	TEST Targets	51

B.1.1.	Target of opportunity	51

B.1.2.	Controlled target	51

B.1.3.	Controlled and calibrated target	52

B.2.	Lobing	52

B.3.	Test Metodology	53

B.3.1.	Radar Functions and other features	53

B.3.2.	Core radar performance	53

B.3.3.	Radar Performance versus the Dynamic Environment	58

B.3.4.	Target tracking	58

ANNEX C	Test Targets and their Callibration	59

List of Tables

Table 1 IALA Target Types	11

Table 3 Detailed Target Characteristics	24

Table 2 Imaginary, simplified VTS Radar System configurations and associated key parameters	27

Table 4 Calculated minimum range as a function of antenna characteristics and antenna hight	28

Table 5 Calculation of radar horizon.	28

Table 6 Atmospheric attenuation for various radar frequencies.	31

Table 7 Swerling Cases	32

Table 8  Example of performance across various radar types and weather situations	34

Table 9  Example of performance across various radar types, longer range	35

Table 10 Comparison between Horizontal and Circular polarization	36

Table 11 Typical azimuth separation as a function of system type and range.	37

Table 12 Typical Target Positional Accuracies for pulse compression radars	39

Table 13 Update rate	39

Table 14 Example of a dynamic range evaluation for an imaginary pulse compression radar	41

Table 15  Dynamic range typical for VTS applications	42

Table 16 Typical requirements to maximum Side Lobe Level Relative to Non-saturating Target Signals	43

List of Figures

Figure 1 Elements included in producing requirements	15

Figure 2 Illustration of radar coverage	16

Figure 3 Target separation and resolution	17

Figure 4 Minimum detection range as a function of antenna height and lookdown angle.	27

Figure 5 Geometry, antenna height and target height	28

Figure 6 Example of received power versus range	30

Figure 7: Probability of detection	33

Figure 8 Example of Return from two identical targets placed at the 0.6° and 1.4° angles	37

Figure 9 Range discrimination	38

Figure 10 Definition of trueness and precision	38

Figure 11 Dynamic characteristics of signal received versus target RCS and target range	40

Figure 12 Range and Antenna (Azimuth) side lobe effects	42

Figure 13 Coverage diagram, in standard atmosphere (left) and including an evaporation duct (right).’	45

Figure 14 Example of simulated radar coverage in surface based + evaporation ducting conditions	46

Figure 15 Coverage diagram, elevated duct	47

Figure 16 Coverage diagram based on a measured condition	47

Figure 17 One hour of recordings with trials (snail tracks) shown in red.	48

Figure 18 Multipath propagation resulting in Ghost images	50

Figure 19 Possible scenarios for ghost target analysis	50

Figure 20 Multi-path reflections (lobing)	52

Figure 21 Example of a video capture with “snail tracks”	53

Figure 22 Example of a test trajectory for range detection testing	54

Figure 23 Measurement of range separation	56

Figure 24 Luneberg reflectors	59

Figure 25 Target aspect angle relative to the radar antenna	60

# INTRODUCTION

The intent of this Guideline is:

- To be a common source of information on VTS radar.

- To assist VTS Providers in the understanding of radar performance.

- To support the design of a radar service and its contribution to the VTS traffic image; and

- To assist VTS Providers in establishing appropriate Radar Functional-, Performance- and associated Acceptance Requirements.

The guideline considers radar surveillance in different operational areas (e.g., inland waterways, Harbours, Coastal regions and offshore). This includes considerations, related to environmental conditions such as weather and sea state, geographical constraints, and obstructions, which all pose challenges to the detection and coverage of radar sensors.

## The IALA G1111 guideline series

This Guideline is one of the G1111 series of guideline documents. The purpose of the G1111 series is to assist the VTS Provider in preparing the definition, specification, establishment, operation, and upgrades of a VTS system.  The documents address the relationship between the operational requirements and VTS system performance (technical) requirements and how these requirements affect system design and sub system requirements.

The G1111 series of guideline documents present system design, sensors, communications, processing, and acceptance, without inferring priority. The guideline documents are numbered and titled as follows:

- G1111		Establishing Functional & Performance Requirements for VTS Systems

- G1111-1	Producing Requirements for the Core VTS System

- G1111-2	Producing Requirements for Voice Communications

- G1111-3	Producing Requirements for RADAR

- G1111-4	Producing Requirements for AIS and VDES

- G1111-5	Producing Requirements for Environment Monitoring Systems

- G1111-6	Producing Requirements for Electro Optical Systems

- G1111-7	Producing Requirements for Radio Direction Finders

- G1111-8	Producing Requirements for Long Range Sensors

- G1111-9	Framework for Acceptance of VTS Systems

# DEFINITIONS

## General Terms

For general terms used throughout this section refer to IEEE Std 686-1997 IEEE Standard Radar Definitions.

## Specific Terms

Specific terms in this document are defined as follows:

Azimuth (Antenna) Side Lobes - antenna responses (in azimuth) outside the intended radiation beam.  Weighting of the illumination function allows a significant reduction of these lobes, but some response outside the intended direction is unavoidable, normally presenting an irregular pattern with 'peaks' and 'nulls.  The side lobes may produce responses from targets in unwanted directions, allowing disturbing signals (intentional or not) to enter the receiver, and in addition makes the radar detectable by receivers, which are not illuminated by the main beam.

Availability - is the probability that a system will perform its specified function without failure when required.

Blind Spots – typically resulting from either blind range (the Range corresponding to an echo delay of one or more pulse repetition intervals: the echo then arrives at the receiver while the radar is transmitting a new pulse and the receiver is blanked) or Blind speed (target speeds which produce Doppler shift which are integer multiples of the radar pulse repetition frequency (PRF), which are therefore aliased to zero Doppler and cancelled by the clutter rejection filtering).  Blind spots can also arise behind significant obstructions in the field of view (buildings, land masses, oil tankers).

BITE: Built in Test Equipment

Chirp - frequency modulation of the carrier frequency applied within the radar pulse to increase its bandwidth and therefore the range resolution (see also pulse compression).

Coherence - capability of a system to keep a stable phase reference during the target illumination time in order to properly exploit the received phase information for moving target indication (MTI), pulse Doppler processing or other purposes.

Doppler Shift - shift in frequency of a wave due to the relative motion between the transmitter and the receiver.  Frequency shift is relative target velocity/wavelength.  Radar echoes are shifted twice this value because this shift must be accounted for in both the forward and the return path.

Doppler Side Lobes - when using Doppler processing (or MTI) the integrated ideal pulse always presents a response outside the integration peak (across all Doppler filters) known as Doppler side lobes.  Their main effect is to limit the capability to discriminate weak returns in proximity of strong returns (with side lobes of the same order of magnitude as the primary response of the weak return).

FMCW - Frequency Modulation - Continuous Wave - A type of radar where a continuous wave instead of pulse is transmitted.  The range information is derived by frequency modulating the carrier with a saw tooth waveform and comparing the echo FM modulation envelope with the reference.

Gap Filler - A radar used to supplement the coverage of a principal radar in areas where coverage is inadequate.

Ghost Targets (Ghost Echoes) – undesirable radar echoes resulting from a number of sources.  For example, multipath related wave reflections caused by large structures or surface reflections, time sidelobes, antenna azimuth sidelobes, and Doppler sidelobes.

Interference Rejection - this function is included to seek to reduce or eliminate interference received from transmitters utilising the same or nearby frequencies.  One common technique is to compare adjacent range cells in the present 'live' video signal with the video signal from the previous sweep.  The output video signal to the display device is inhibited should the comparison indicate the presence of interference.

Normal Weather and Propagation Conditions - are normal conditions as defined by the individual VTS Provider.  The rest of the time is considered having adverse weather and propagation conditions.

Plot Extraction – the process of determining the likely target related radar returns from the radar video signal.  This typically consists of comparing the video level with a threshold which can be (dynamically) adapted to local background noise and clutter conditions.

Polarisation – of a radar signal is determined by the orientation of the electrical field.  In the case of circular polarisation, the field rotates left or right.

Pulse – typically a pulse (which is modulated in the case of pulse compression radar) of RF energy transmitted from the radar.

Pulse Compression – A technique used to achieve a wide pulse bandwidth (and, therefore, enhanced range resolution) using long pulse (for high pulse energy with limited peak power) by introducing an intra-pulse modulation (e.g., chirp frequency modulation or Barker discrete phase modulation) and performing a correlation on the received echo.

Radar Cross Section (RCS) – an assessment of the cross-sectional area presented by a reflector (typically a target or unwanted “clutter”) to the transmitted radar energy.  The RCS can vary with frequency and target attitude.

Radar Information – a generic term potentially referring to the radar picture/video, target data, clutter data, topographical data, aids to navigation SARTs etc.

Radar PD – is the probability of target detection at the output of a radar, after plot extraction, but prior to tracking, and presentation.  Note, in some systems the boundary of the radar and its achieved PD complicate this definition – clarification may be required to avoid misunderstanding arising from, for example, data compression or video processing.

Radar PFA – is the probability of false alarm at the output of a radar, subsequent to plot extraction, but prior to tracking, and presentation.  In this context, the PFA is defined as relating to the number of false target declarations per radar cell (range cell x azimuth cells), arising from a noise plus clutter environment (only).  Note, in some systems the boundary of the radar and its achieved PFA complicate this definition – clarification may be required to avoid misunderstanding arising from, for example, noise related threshold crossings vs. unwanted radar energy reflections (unwanted targets, ghost targets etc.).

Radar Plot – is the generic term to describe the report resulting from a radar sensor observation. Each report contains positional information, possibly supplemented by other data.

Radar Target – an object about which information is sought with radar equipment.

Radar Video – a time-varying signal, proportional to the sum of the radio frequency (RF) signals being received and the RF noise inherent in the receiver itself.  Traditionally, radar video is an analogue signal with associated azimuth reference information.  Recently, radar systems have become available which provide equivalent data in digital format.

Radar Sensor – the transmitting, receiving and signal handling apparatus, delivering radar information to the tracking and presentation features of VTS.

Radar Service – a service that delivers all radar-derived data, such as radar image, radar plots and radar tracks.

Radar Track – an estimate of a target’s position and speed vector, resulting from the correlation, by a special algorithm (tracking), of present and past radar-reported positions (radar plots).  The report normally contains an identification (e.g., a track number), a timestamp, the estimated position, and a speed vector.  Additional information may include e.g., estimated errors, the last associated radar plot and perceived track quality

Range Ambiguous Returns – the measured range of a target typically assumes that the target true range is less than the first range ambiguity (the Range corresponding to an echo delay of one pulse repetition interval) whereas large targets beyond this range can be detected but typically with (incorrect) ambiguous range measurement.  Techniques exist for the resolution of range ambiguity if required.  See also blind spots above.

Range Side Lobes – see Time Side Lobes (below).

Receiver Dynamic Range – essentially the range of signal levels over which a receiver can operate.  The low end of the range is governed by its sensitivity whilst, at the high end, it is governed by its overload or strong signal handling performance.

Reliability – the probability that a system, when it is available performs a specified function without failure under given conditions for a given period of time.

Sea Characteristics – often described by sea state but additional parameters can also be of interest.  Sea characteristics include wave/swell height, direction and speed of waves/swell, distance between waves/swell, salinity etc.

Standard Atmospheric Condition – the International Commission of Air Navigation uses a definition for a standard atmosphere, defining temperature and pressure relative to the height.  In the troposphere (0 metres to 11,000 metres), the temperature is defined to be 15 °C at the surface and changing -6.5 °C/km.

Squint – the potential angular difference between antenna broadside and the antenna beam pointing direction.  This angular difference may change with transmission frequency.  The effect can be fully compensated.

Swerling Cases – a series of mathematical models representing RCS fluctuations to characterise the statistical behaviour of reflected radar signals from a target (see also target fluctuations).

Target Fluctuations – (also known as Glint or Swerling characteristic) - Fluctuation of a target radar cross section (RCS) (and, therefore, of the received echo amplitude) due to changes in the target attitude and illuminating frequency.  For complex targets (consisting of a number of reflecting surfaces), RCS is normally strongly dependent on the angle of observation.

Target Separation – (also known as Target resolution) – the ability to successfully identify two discrete detectable, similarly sized targets when closely spaced in either range or azimuth.

Track Swapping – the (usually unwanted) transfer of a track identity (track label) to another track.  This can break the intended association between a track and a physical object.

Time Side Lobes – when using pulse compression, the correlated pulse always presents responses outside the correlation peak (before and after it) known as time (or range) side lobes.  Their main effect is to limit the capability to discriminate weak returns in proximity of strong returns (with side lobes of the same order of magnitude as the primary response of the weak return).

## Speciffic IALA Definitions

### Target Types

For calculation purposes, the IALA simplified target types are defined in Table 1.

Typical targets of interest are modelled as point targets with conservative estimate of Radar Cross Section (RCS) and height.  This is normally sufficient for estimation of detection range for consideration in VTS radar sensor coverage.

The term fluctuation in the table means that the signal returns will vary in amplitude, from pulse to and from antenna rotation to antenna rotation. This, the hight of the target, the radar cross section and other parameters affects the ability for the radar to detect specific targets as further discussed in ANNEX A

Table 1 IALA Target Types

| IALA Target Types |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Target Type | Typically Representing | Radar Cross Section |  |  | Height (ASL) | Fluctuation |
|  |  | S‐Band | X‐Band | Ku-Band |  |  |
| 1 | AtoN without radar reflector. Small open boats, fiberglass, wood or rubber | <<1 m2 | 1 m2 | 1 m2 | 1 m | Rapid, depending on sea state and target movement |
| 2 | In‐shore fishing vessels, sailing boats and speedboats. | 1 m2 | 3 m2 | 4 m2 | 2 m |  |
| 3 | Aids to Navigation with radar reflector. | 4 m2 | 10 m2 | 12 m2 | 3 m |  |
| 4 | Small metal ships, fishing vessels and patrol vessels. | 40 m2 | 100 m2 | 120 m2 | 5 m | Moderate |
| 5 | Small coasters and large fishing trawlers. | 400 m2 | 1,000 m2 | 1,200 m2 | 8 m |  |
| 6 | Large coasters, bulk carriers, and cargo ships. | 4,000 m2 | 10,000 m2 | 12,000 m2 | 12 m | Negligible |
| 7 | Container carriers and tankers. | 40,000 m2 | 100,000 m2 | 120,000 m2 | 18 m |  |

Notes:

- The RCS values assume linear polarized antennas.

  - A general reduction of 5 dB can be assumed in the case of circular polarized antennas.

  - The reduction of RCS will typically be even larger for non-metallic targets, trihedral reflectors and most Lunenburg reflectors, often omitting detection of such targets in the case of circular polarized antennas.

- The indicated values include allowance for the RCS-limiting effect of the cell size in the case of radars with high-resolution. See Table 2 for detailed target characteristics.

# References

- IEEE Std 686-2017 – “IEEE Standard Radar Definitions”

- Merrill I. Skolnik – “Introduction to Radar Systems”, McGraw-Hill Higher Education, ISBN 0-07-290980-3

- P.D.L. Williams, H.D, Cramp and Kay Curtis – “Experimental study of the radar cross section of maritime targets”, ELECTRONIC CIRCUITS AND SYSTEMS, July 1978. Vol. 2. No 4

- Ingo Harre – “RCS in Radar Range Calculations for Maritime Targets” (http://www.mar-it.de/Radar/RCS/RCS_18.pdf)

- International Maritime Organisation (IMO) – “Performance Standards for radar reflectors”

- ITU-R SM.1541 – “Unwanted emissions in the out-of-band domain”

- ITU-R SM.329-9 – “Spurious emissions”

- ISO 8729 Ships and marine technology – “Marine radar reflectors”

- International Commission for Air Navigation – “Definition of the Standard Atmosphere”

- AREPS: “Advanced Refractive Effects Prediction System”, Space and Naval Warfare Systems Center, San Diego, http://sunspot.spawar.navy.mil

- IALA Guideline G1150 – “ESTABLISHING, PLANNING, AND IMPLEMENTING VTS”

- “Radar Handbook”, M. I. Skolnik, McGraw Hill, 2008

- “Influence of evaporation ducts on radar sea return”, J. P. Reilly and G. D. Dockery, IEE Proceedings Vol. 137, Part F, No. 2, p.80 – 88, April 1990.

- “Radar Target Detection”, D. P. Meyer and H. A. Mayer, Academic Press, 1973.

- “An Analysis of X-Band Calibrated Sea Clutter and Small Boat Reflectivity at Medium-to-Low Grazing Angles”, P. L. Herselman et al, International Journal of Navigation and Observation Volume 2008, Article ID 347518, 14 pages.’

- “Land Clutter Models for Radar Design and Analysis”, D. K. Barton, Proceedings of the IEEE, Vol. 73, p. 198-204, 1985.

# Abbreviations

Please refer to IALA G.1111 Establishing Functional and Performance Requirements for VTS systems for an extensive list of abbreviations and acronyms covering the entire G1111 series

# Operational Areas

This guideline considers application of radar to different operational areas. These areas vary in the types of risks, vessels and their interactions, the required sensor ranges and the types of services. Within the total VTS area, there are generally several different operational areas.

The necessary Functional and Performance requirements may differ across the VTS, so that setting the Functional and Performance Requirement should be conducted precisely for each operational area to make sure the navigational risks are mitigated and VTS Operators can provide services smoothly.

The VTS Provider should identify the Operational Requirements that may impact the Functional and Performance Requirements of radar sensor(s) within the VTS system.

Operational Requirements arise from:

- Risk assessment and needs assessment

- The identified operational areas within the area of interest

- The types and size of vessels operating within the area of interest

- The services intended to be provided by the VTS

From a surveillance perspective, operational areas may be characterised as follows:

- Inland waterways, like rivers and canals

- The port area

- The port approach area incl. anchorages

- Coastal areas and offshore, including shipping lanes, platforms, and windfarms

Radar Functional and Performance requirements will differ in these operational areas, so identifying the appropriate Radar Functional and Performance Requirements should be a careful process, balancing cost and, sometimes opposing, performance requirements (e.g., probability of detection versus false alarm rate).

## Inland Waterway

Inland waterways, like rivers and canals, are confined waters that are used by a large variety of vessels, ranging from sea-going vessels (e.g., a river section as part of the port approach), river-trade, allied services vessels, ferries and even recreational vessels. Traffic separation may be as low as a few metres. This requires a high position accuracy and target resolution performance.

Locks, bridges, and waterway intersections may be present that will limit target visibility, affect position accuracy or force reduced target separation.

Although the required sensor range is limited, the sensor may not have a full view of the waterway due to obstructions. Such obstructions may be permanent or temporary e.g., moored large vessels. For that reason, Radar location will require proper attention (especially when location options are limited).

Maximum target detection ranges typically vary from 1km to 10km. Considering these ranges, radar detection performance is generally not a critical factor.

The addition of short-range gap filler radars may be an option to cover specific areas, such as locks.

In this operational area, accuracy and resolution are the critical performance parameters. At larger ranges, angular separation becomes the limiting factor.

Given the required high accuracy and resolution, S-band radar will generally not be able to meet these requirements.

Note: The distances are normally measured in kilometres for inland waterways. Other areas measure distances in Nautical Miles.

## Port

The port area consists of confined waters. Traffic includes sea-going vessels, allied services vessels and, possibly, river-trade vessels (trans-shipping). Required position accuracy and target resolution performance are comparable to those of inland waterways. The required radar range is usually very limited, but the radar view will be obstructed or mirrored, even if only temporarily, by cranes, moored or passing vessels, cargo, and buildings. For this reason, special attention should be given to radar siting and the possible occurrence of unwanted reflections.

For port areas, required accuracy and resolution performances are like those for inland waterways.

Measures to reduce the occurrence and/or impact of false targets may be required in this operational area. This may include:

- Reduction of emitted power in certain directions (blanking or low power emitted)

- Identification of reflection areas

- Short-range gap filler radars to cover specific areas

## Port Approach

The port approach area is usually within the 12NM zone but may extend beyond. There may be several types of operations within this area:

- Vessels entering and leaving port, possibly crossing coastal traffic lanes

- Pilot (dis-)embarkation

- Anchorages with supply vessel traffic and, possibly, trans-shipping operations

- Fishery

- Maritime traffic exclusion areas including renewable energy harvesting, fish farms, etcetera.

- Recreation

Coverage of this area translates into a medium required sensor range. Traffic of interest may range from small sailing boats up to >20k TEU container vessels. In dense traffic areas, e.g. pilot boarding areas, radar resolution performance is important to be able to separate individual vessels.

## Coastal Area and OFF-SHORE

The coastal area may extend well beyond the 12NM zone and the number of offshore installations in the coastal areas are increasing world-wide. Ranges to be covered are in general long, however, radar installation on offshore platforms is typically set up for shorter range than coastal.

Typical operations are traffic monitoring for

- Protection of oil/gas platforms and renewable energy harvesting, like windfarms.

- Protection of the environment

- Regular traffic monitoring and Search and Rescue (SAR)

Coverage of coastal areas usually requires long-range sensors. Traffic of interest may include fishing vessels and larger vessels, but also significantly smaller vessels or even helicopters in case of SAR operations. Monitoring of ATON positions including Racons may also be an operational requirement.

Weather, including varying propagation conditions and weather causing clutter, is often challenging.

Resolution may be important in busy coastal area, especially with many SOLAS vessels navigating near each other.

Depending on siting and the extent of e.g. a windfarm, this requires a short to long sensor range. Traffic of interest may include fishing vessels and larger vessels as well inter-turbine traffic in the form of small vessels.

Special attention should be given to siting to minimise multipath reflections between wind turbines and larger vessels in the coverage area.

# Producing Functional and Performance requirements

- Producing functional and performance requirements for VTS radar is a challenging task involving iterations, including evaluation of achievable performance versus overall system cost.

- The requirements should be based on a Business Case and a Feasibility study (risk assessment, operational feasibility, legality, technical capability, available budget, and time) as described in IALA Guideline G1150. The risk assessment should address the specific risks within areas of interest and the way to handle or mitigate the risks.

- The use of radar sensors is one of the options to mitigate such risks by providing dynamic maritime situational awareness. When used, the radar sensor's functional and performance requirements, see Figure 1, should be established in consultation with VTS personnel, other stakeholders, and the supplier. The planning or design of the VTS System should specify the requirement or role of the radar sensor(s) to present and track all detectable targets of interest simultaneously in normal and predefined conditions.

- The requirements should be established for the identified areas of interest and with characteristics including:

- Core radar performance:

Radar coverage, target detection and range performance

Target separation

Target positional accuracy

- Radar performance versus the dynamic environment:

Elimination of false echoes, sidelobes

Handling of dynamic characteristics of targets and surrounding

Handling of local environmental conditions including environmental impact on target detection

Seasonal impact

Handling other influencing factors, e.g. obstructions

- Radar functions and other features:

Target positional update rate

Operator functions

Operational outputs

Clutter, noise, and interference reduction/handling

Target tracking and false alarm rate

Robustness, availability, and serviceability

Maximum Detection Range

Angular Separation

Range Separation

Minimum Detection Range

Radar performance to deal with the dynamic environment

Radar Coverage

Figure 1 Elements included in producing requirements

Different suppliers are likely to have unique solutions to the same functional and performance requirements. Therefore, the VTS Provider should avoid prescribing specific technical solutions. In turn, suppliers should propose solutions that meet the operational and functional requirements, as specified by VTS Provider

## Core Radar Performance

### Radar Coverage

The radar coverage is the area of interest where VTS Operator needs to have a radar video picture on the User interface of the VTS System. This includes minimum and maximum coverage and the radar coverage required for a VTS may consist of multiple radar sensors that are furthermore distributed to optimise system availability, radar data integrity, security constraints and possible equipment access considerations.

Careful consideration should be given to optimise radar location(s) and antenna height(s) to ensure appropriate radar coverage and radar target separation. Refer to Annex A.5 for further guidance on determination of radar coverage.

A single radar coverage can be designed by identifying the necessary coverage and then determine coverage taking antenna height, targets to be detected and environmental conditions into account.

Figure 2 Illustration of radar coverage

To provide coverage over large areas and/or to mitigate shadow effects of other vessels, multiple radar sensors may be utilised.  Combined processing of images from two or more radars may also be utilised for elimination of false (ghost) echoes and to improve target discrimination.

Possible radar location(s), obstructions etc. are often limiting factors for the establishment of radar stations and such limitations should be considered when determining requirements.

GIS tools will typically be used to provide graphical illustrations e.g., as illustrated in Figure 2

### Target detection and range performance

Target detection is determined by acquisition of radar plots of the specific targets within the VTS coverage area. The requirements for target detection and range performance can be identified by the target types of interest found within the VTS area, including their minimum and maximum operational range.

The minimum detection range for the radar depends on the antenna height above sea level, the length of the waveguide and the pulse (chirp) length of the transmitted signal.  The lookdown angles for the antenna depend on the vertical coverage diagram, e.g. an inverse cosecant beam will typically have look down angles in excess of fan beam antennas.

The Requirements of target detection and range performance can be determined by:

The target types mentioned by Table 1

Minimum and maximum range to be detected

Antenna height

Weather (such as fog, sea state and rain) and atmospheric propagation conditions.

### Target Separation

The required target separation, ref Figure 3, should be based on a business case and feasibility study which is conducted in the planning phase. The Feasibility study on Risk should derive the minimum separation requirement between surface objects or vessels for the areas covered by radar and specified for the target types of interest to the VTS.

Azimuth

Resolution

Distance from radar to target

Azimuth separation

Range Separation

Range Resolution

Figure 3 Target separation and resolution

Requirements can be determined by:

The target types mentioned by Table 1

Minimum separation of above targets

Range to require target separation, which should be within detection coverage

### Target Positional Accuracy

Target Positional Accuracy should be based upon the results of the business case and a feasibility study.

The operational requirements in terms of accuracy should consider:

- Limitations of ships in the VTS area that may impose restrictions on the navigation of other ships (e.g. manoeuvrability), or any other potential hindrances

- VTS space allocation

- Route advice

- Navigation assistance

- Responding to unsafe situations

- Refer to Annex A for further guidance on the subject

## Radar Performance Versus the Dynamic EnvirOnment

### False Echoes

False echoes are undesirable radar echoes resulting from various sources, including:

- Multipath caused by large structures and surface reflections

- Radar range (time) sidelobes and antenna azimuth sidelobes

- Refer to Annex A for further guidance on the subject

### Handling of Dynamic Range, targets and surroundings

The dynamic range of the radar, ref. Annex 0, should detect and process the surface objects specified by the VTS Provider weather and propagation conditions normal for the VTS area.  This should be achieved without significant side lobes, degradation of target appearance, degradation of detection or degradation of separation capability.

- Refer to Annex A for further guidance on the subject

### Handling of environmental Conditions

The environmental conditions of the VTS area should be described, including the annual cycle.  This should include environmental conditions to be expected as normal over twelve months as well as extreme events. The Environmental information can be obtained from meteorological services and be used to set requirements (e.g., temperature ranges, operational/sustainable wind, and sea conditions.

Seasonal impact and restrictions with respect to operation and access to site(s) due to weather should also be included.

The VTS Provider should define the required radar performance requirements in clear conditions as well as for precipitation conditions normal for the VTS area.  Guidance of what is possible is provided as follows.

#### Impact from precipitation and sea clutter

Precipitation and sea clutter can significantly affect the radar coverage of the VTS system due to reduction of radar signal propagation and backscatter from rain as well as backscatter from the sea.

Note that rainfall is rare in dry/hot regions, maybe only once or twice per year and the VTS Provider should consider if rain should be specified at all.

Also note that designing a system to perform in tropical rain showers will typically call for S-band radars.  However, the worst rain conditions may only be present for a few hours per year, and reduction in performance on other parameters, may not justify the additional investment.

Refer to ANNEX A for guidance on what is possible and formulas for calculations.

## Radar Functions and Other Features

The Radar service in a VTS should, as a minimum, support the operational functions specified by G.1111-1 (Target tracking included) as well as the functions listed below.

Note that the tracking functions may be hosted by the radar sensors or other parts of the VTS system.

### Target Positional Update rate

The VTS operation and especially the tracking function is dependent of timely update rates of the radar information. Refer to ANNEX A.8

### Operator Functions

Radar functions should be designed and implemented to optimise performance and minimize VTSO workload to the level practical.  The VTSO typically need to control basic functions of radar(s) such as start and stop.

Each radar site should be designed and equipped to reduce the adverse effects of rain and sea clutter and enhance the probability of target detection.

It may be necessary to adapt radar settings to the changing weather conditions, etc., automatic and/ or manual adaption to reduce VTSO workload. However, it might be necessary to implement dedicated operational modes e.g. for adaptation to weather.

### Operational Outputs

The output from a radar service should include radar image data and track data.  In addition, the output from the radar service may include clutter data to enable identification of squalls, oil spills, ice detection, wave height, etc.

### Clutter and Noise Reduction / Management

Appropriate clutter reduction or clutter management should be available to meet the performance requirements.

This will typically include:

- White noise suppression

- Interference rejection

- Sea and rain clutter processing

- Adaptation to varying propagation conditions.

The features should preferably be automatic. These functions may be hosted by the radar sensors or other parts of the VTS system; however, they are normally most efficient and saves bandwidth in the transmission of data if hosted within the radar sensor.

### Target tracking

Target tracking is often part of the core VTS system but may also be part of the individual radar. Refer to IALA G1111-1 for guidance on the subject.

However, the ability to handle target speeds and manoeuvres is also dependent on basic radar characteristics and these requirements are handled by this document.

### Interfaces

When establishing of a new VTS system, all internal interfacing between the radars, other VTS Equipment and the overall VTS System should be well defined.

When updating or expanding existing VTS System or VTS Equipment, careful attention needs to be given to include requirements of radar sensor interfaces, including protocols, standards, and bandwidth, to ensure radar and updating or expanding VTS System or VTS Equipment interact as intended.

Cybersecurity considerations for interfaces to open data networks should always be included.

### robustness, availability, and serviceability

The radar systems should be designed taking robustness, availability, and serviceability into account.

Special safety precautions for radar should include but not be limited to those applicable to Rotating Machinery, Radiation Hazards and Electrical Shock.

Special precautions should also consider lightning protection, wind load on antennas and access to the systems, including antennas for installation and maintenance.  Turbulence, asymmetrical wind and vertical wind components should be considered with reference to the descriptions in ANNEX A

Waveguides should in general be kept as short as possible.  Their length and associated losses need to be included when determining system performance.  Waveguides should always be equipped with dehumidifiers or simple desiccators.

The radars and their installation condition should be designed taking robustness, availability, and serviceability into account. Referring to the descriptions in ANNEX A, the consideration could be given including applicable safety precautions such as Rotating Machinery, Radiation Hazards and Electrical Shock.

#### Antenna Robustness Against Wind Load

The requirement of radars should consider potential problems resulting from high winds.  In some cases, it may be appropriate to separately specify both the survival wind limits and a lower operational wind limit within which the system should not be degraded due to the normal weather conditions specified for that location.  High winds can affect the antenna’s motor and gearbox design and the instantaneous rotation rate at varying angles to the predominant wind direction. The build-up of ice in some climates should also be a consideration.

High winds can affect plot accuracy, and the overall system azimuth accuracy should consider torsional errors arising from high winds. As a rule of thumb, the maximum torsion of the radar tower should not exceed 25% of the antenna’s horizontal beamwidth at the maximum operational wind.

Note that vertical wind components or asymmetrical wind loads may cause stresses to rotating antennas. This may restrict how antennas are geographically and physically installed, how restrict tower constructions are made or call for other mitigation methods. Turbulent wind gradients nearby large structures and mountain lee waves may also stress equipment beyond its design limits

In extreme conditions, it may be appropriate to house the rotating radar antenna within a static radome. However, this can increase overall costs, RF losses and maintenance costs and difficulties.

#### Choice of Up-mast versus Down-mast Transceivers

Careful consideration may be given whether to locate the transceiver up-mast or down-mast. A down-mast transceiver might benefit from an environmentally controlled location and consequent improvement in equipment reliability.

Conversely, an up-mast transceiver installation may be more difficult to access and conduct maintenance and service than a down-mast transceiver.

Further issues might include the need to transfer high bandwidth video (on copper, fibre or microwave link etc.) which can influence the radar designer’s selection of whether to use an up-mast or down-mast transceiver.  Incorporation of transceiver redundancy can also influence the choice of transceiver location.

#### Waveguide Considerations

The waveguide length between the transceiver and antenna, and associated losses need to be considered when determining radar detection performance. Waveguides should always be equipped with dehumidifiers or simple desiccators.

#### Position of the lightning rod

Position of the lightning rods are typically preferable to place in a blanked sector or a direction of low practical importance.

- VTS Radar Considerations

This annex is descriptive and intended to supplement previous sections in this document and introduce the reader to common radar topics and knowledge specific to VTS radar as well as guidance on what is possible to achieve from radar configurations typical for VTS.

Calculation methods for the determination of performance are described and examples of that possible with various technologies and simplified configurations are provided.

Radar Technologies

VTS radars could be based on the following technologies:

- pulse radar (usually Magnetron based)

- pulse compression radar (usually Solid State)

- Frequency Modulated Continuous Wave, FMCW (usually Solid State)

Pulse Radar

A Magnetron based pulse radar typically transmits high peak power RF pulses (10 to 50 kilowatt (kW)) of very short duration (50 to 1000 nanoseconds).  The transmission is made with a pulse repetition frequency (PRF) of typically 1000 to 4000 pulses per second.  Upon reception, the returned signal is amplified, demodulated, and processed.

Main characteristics include:

it is a well-known and proven technology.

it has fixed pulse lengths.

increased pulse duration translates into longer-range detection, but reduced range resolution and reduced ability to penetrate precipitation due to increased backscatter.

normally with a fixed transmission frequency (given by the magnetron characteristics).

the need to reduce out-of-band transmissions.

- Note: ITU requirements for land-based radar are more stringent than for shipborne radar in respect to out of band emissions.

Pulse Compression Radar

A pulse compression radar transmits low peak power modulated chirps (typically up to 200-300 Watt (W), in some cases higher) with a typical pulse duration of up to 100 microseconds (s).  The transmission is made with a chirp repetition frequency of typically 1000 to 20000 chirps per second.  The longer chirps are converted into short pulses upon reception by the process of pulse compression; therefore, improved range resolution can be achieved at all ranges within a single radar mode.

The average transmitted energy of a pulse compression radar is comparable to that emitted by a magnetron radar.

Main characteristics include:

it is based on well-known and proven principles

no need for magnetron replacement, reducing the need for periodic maintenance

increased ability to penetrate adverse weather conditions facilitating smaller target detection

transmission frequencies can be programmed

cleaner spectrum than magnetron radars, with reduced emissions outside the allocated frequency band(s)

Additional challenges compared to magnetron radars include:

the need for sophisticated interference rejection due to the longer chirps transmitted

the need for simultaneous short- and long-range detection increases complexity

high-power solid-state amplifiers operate with large currents therefore requiring careful design to obtain high reliability

by nature, the pulse compression radar creates so-called time (or range) side lobes.  Avoiding such side lobes, requires sophisticated techniques, alternatively side lobes suppression may imply a reduced detection of small targets in the vicinity of larger targets

additional focus on dynamic range to avoid saturation which will create very distinct sidelobes in range and reduce sensitivity in front of and behind large objects

inability or less sensitivity in the detection to trigger RACONS and SART transponders

Note: There may be legal restrictions (dual use, catch-all etc.) limiting the compression ratio and other parameters when importing (pulse compression) radars to certain countries.  This may limit the availability of this technology to some VTS Authorities.

Frequency Modulated Continuous Wave Radar

Frequency modulated continuous wave radar transmits low peak power continuous wave forms (typically up to 50 W).  The waveforms are repeated with a typical rate of 500 to 2000 per second.  The waveforms are converted into pulses upon reception, therefore high range resolution can be maintained at all ranges.

The energy in a frequency modulated continuous wave radar is comparable to the energy emitted in a pulse from a magnetron radar.

Main characteristics include:

well-known and proven principles

no need for magnetron replacement, reducing the need for periodic maintenance

the ability to detect at very short range

transmission frequencies can be programmed

cleaner spectrum than magnetron radars, with reduced emissions outside the allocated frequency band(s)

Additional challenges compared to other types of radar include:

dynamic limitations restrict the ability to handle small and large targets simultaneously and also affects long range detection

target revisit rate may be low compared to other technologies

the need for sophisticated interference rejection, even more than for pulse compression radars

by nature, FMCW creates so-called time side lobes and suppressing the side lobes may reduce detection of small targets in the vicinity of larger targets

more complicated antenna system, 2 antennas or complicated antenna feed

the inability to detect RACONS and SART transponders

Frequency Bands

The choice between available frequency bands will always be a compromise depending on the operational area, weather and balancing several parameters. Typical capabilities are provided later in this Annex. Frequency bands allocated for radar and suitable for VTS lie in the following range:

S-band, 2.0 – 4.0 GHz: Frequency band occasionally used for VTS Coastal radar to minimize the effect of rain on longer ranges but is for modern radars a less dominating argument as X-band radars improve in capabilities. Frequency allocation (predefined by ITU) is challenged by other users, e.g., Cellular phone operators aiming at expanding band allocation. By nature, S-band radars provide less resolution in azimuth (limited antenna size).

C-band, 4.0 – 8.0 GHz: Not used for VTS

X-band, 8.0 – 12.0 GHz: The commonly used frequency band across VTS operational areas – Frequency allocation (sub-bands for radiolocation and radionavigation services) predefined by ITU. X-band is often a good compromise for balance between resolution, weather penetration and cost across application areas.

Ku-band, 12.0 – 18.0 GHz: Frequency band occasionally used to improve resolution and minimize antenna size. Ku-band radar is generally more sensitive to weather (rain) than S- and X-band radar.

Note: There is no Ku-band Frequency allocation for VTS predefined by ITU, meaning that frequencies shall be allocated by individual countries.

Antennas

The selection of antenna parameters (height, gain, side lobes, rotation rate, polarisation etc.) for a given installation is key to the resulting radar performance. VTS authorities are, however, advised to avoid specifying detailed antenna characteristics and should preferably specify operational requirements such as:

- coverage area and range performance based on need and risk assessments

- typical target manoeuvres in high density areas

- need for overlapping and redundant coverage.

The identified operational requirements will allow the radar vendor some flexibility to achieve the best solution within the given constraints and considering cost and location options.

Antenna Principles

Typically, the VTS radar design includes an antenna, which provides a narrow beam in azimuth and a wide beam in elevation.  Thus, the VTS antenna is not designed to measure the target elevation (from which target height might be determined) and to separate targets based on elevation angle difference.

The installed antenna height is determined based on avoidance of physical obstructions, and the compromise between the need for close range coverage vs. long-range performance.

The radar designer, in his selection of antenna characteristics, needs to optimise the compromise between antenna size (and cost), track update rate, integration time on target (related to rotation rate and azimuth beamwidth and contributing to target detection range) and azimuth target separation and accuracy.  In addition, the choice of transmission frequency influences the size vs. beamwidth compromise.

Flat panel electronic scanning antennas (phased array) may provide advantages in terms of no rotating parts, however, adding to technical complexity.

Antenna Polarisation

Radio waves are polarised, and objects (target and clutter) will often reflect differently dependent on the polarisation used.  This can be utilised by radar system designers, where rules of thumb are that:

target returns from linear polarisation, (horizontal or vertical) in general will be stronger than returns from circular polarisation

non-metallic and natural objects, such as human beings will return linear polarised radio waves much better than circular polarised radio waves

most radar reflectors will be poor reflectors for circular polarised radio waves

distant ships with vertical masts tend to give the strongest return for vertical polarisation, whereas the opposite tends to be the case for modern ship designs without tall masts.

- In addition, target detection in clutter can be affected by the polarisation.  For instance:

linear polarisation (horizontal or vertical) will result in higher rain clutter returns than circular polarisation

vertical or circular polarisation will result in higher sea clutter returns than horizontal polarisation, especially for lower sea states.

Complex designs are possible in which operators may select the polarisation.  However, this adds to equipment costs and adds to the VTSO workload.

In general, the best cost/performance combined with ease of operation is achieved by horizontal polarisation.

Note: In case of a radar station also being used for oil spill detection, the preferred polarization is vertical.

Target caracteristics

VTS radar targets are defined by their height above sea level, their speed and manoeuvrability, their polarisation characteristics, their radar cross section (RCS) and the fluctuations in RCS.

Radar Cross Section

The amount of radar energy reflected by a target is proportional to its effective area called the Radar Cross Section (RCS). The RCS defined as the ratio between the power (in W) scattered by the target back towards the radar receiver and the power density (in W/m2) hitting the target. Thus, the RCS has the dimension of an area, being measured in m2. In practical terms, the RCS can be interpreted as the cross-sectional area of a perfectly reflecting metal sphere that would give rise to the same reflected energy as the target in question.

Being an effective area, the RCS of a target is not necessarily related to the physical cross-sectional area of the target, but rather depends strongly on the nature and shape of the surfaces exposed to the radar beam. Consequently, targets of similar physical sizes may exhibit vastly different RCS, and additionally, their RCS may vary drastically with aspect angle (see [2]) as the exposed surfaces change with orientation. Finally, the apparent RCS of a target is subject to fluctuate over time, for instance due to minor changes in orientation between exposures to the radar beam.

To obtain quantitative and reproducible results during performance testing, it is therefore necessary to use physically small and rather simple targets and to measure and calibrate their RCS against that of a well-known reference target. During these measurements, the dependence of the RCS on the aspect angle can additionally be mapped out. A method to perform such a measurement is described in 0.

Target Models versus Detailed Characteristics

The point target characteristics, ref Table 1, may be sufficient for range calculation of specific targets of interest in VTS.

However, the design of a radar system should consider the overall characteristics of all objects within coverage range of the individual radar and Table 2 provides an overview of such characterises for targets, typically of interest to VTS radar.

Table 2 Detailed Target Characteristics

| Target | Typical characteristics at X-band |  |  |
| --- | --- | --- | --- |
|  | RCS | Height | Fluctuations etc. |
| Aids to Navigation without radar reflector. | Up to 1 m2 | 1 to 4 m ASL | Rapidly fluctuating, highly dependent on sea characteristics. Polarisation characteristics will often vary depending on wind. |
| Aids to Navigation with radar reflector. | 10 – 100 m2 |  | Rapidly fluctuating, wind and currents may tilt to blind angles and lobing may cause reflectors to be in blind spots. Most radar reflectors will be poor radar targets in case of circular polarisation |
| Small open boat, fibreglass, wood or rubber with outboard motor and at least 2 persons on board, small speedboat, small fishing vessels or small sailing boats. | 0.5 – 5 m2 | 0.5 to 1 m ASL | Rapidly fluctuating may be hidden behind waves up to 50% of the time. Slow moving targets tend to lie lower in the water than fast moving ones and therefore RCS visible to the radar tends to increase with speed. Humans and non-metallic targets will give poor radar return in case of circular polarisation |
| Inshore fishing vessels, sailing boats and speedboats, equipped with radar reflector of good quality. | 3 – 10 m2 | 1 to 2 m ASL | Rapidly fluctuating. |
| Small metal ships, fishing vessels, patrol vessels and other similar vessels. | 10 – 100 m2 | 2 to 4 m ASL | Moderately fluctuating. |
| Coasters and other similar vessels. | 100 – 1000 m2 | 6 to 10 m ASL | RCS is highly dependent on aspect angle of the individual vessel. Rate of fluctuations is typically moderate. |
| Large coasters, Bulk carriers, cargo ships and other similar vessels. | 1000 – 10,000 m2 | 10 to 25 m ASL |  |
| Container carriers, tankers and other similar vessels. | 10,000 – 2,000,000 m2 | 15 to 40 m ASL |  |
| Buildings, cranes. Stacks of containers and other large structures. | Up to 1,000,000 m2 | Depends on site | Insignificant. |
| Floating items, oil drums and other similar items. | Up to 1 m2 | 0 to 0.5 m ASL | Rapidly fluctuating, highly dependent on sea characteristics and target movements. |
| Birds, floating or flying. |  | Sea level and up |  |
| Flocks of birds. | Up to 3 m2 | Sea level and up | Rapidly fluctuating, flight paths tend to be characteristic of given species in given areas of interest. |
| Jet Skis and other personal watercraft | Up to 0.5 m2 | 0 to 1 m ASL | Rapidly fluctuating but virtually independent of aspect angle |
| Wind turbines (onshore and offshore) | Up to 10,000,000 m2 | Up to 300 m ASL | Fluctuations for towers are insignificant. Rotating parts give a wide spectrum of Doppler shifts with RCS up to hundreds of m2 |

Data is primarily based on references [3] + [4] , supplemented by data obtained from the experiences of IALA VTS committee members.

Note: Modern warship design seeks to minimise RCS and, as a result, the above figures cannot be related to such vessels.

In case the physical size of the target exceeds the size of the resolution cell of the radar, the target gets extended into other resolution cells. In such a case, the RCS values, as mentioned above, may be incorrect (as the target reflection is now distributed across more than one resolution cell).  Additionally, the extension of the target poses extra challenges on detection and tracking performance. Extension of targets and the consequences thereof should especially be considered in situations where the radar is positioned close to the targets to be detected and tracked and/or in case of high-resolution radars.

RCS on targets using S-band is typically 40% and Ku-band is typically 120% of the RCS in X-band except for small non-metallic targets where the RCS difference between the 3 bands can be much higher.

Target RCS Fluctuations

For VTS target types, statistical RCS fluctuations can adversely affect target detection.  To predict range performance more realistically, such fluctuations can be mathematically modelled using the different Swerling cases.  The radar design and the location of radars can compensate for the consequences of target RCS fluctuations.

Multipath Effects

Avoidance or reduction of effects from reflections does require in depth analysis, often combined with site surveys and calculations by the experienced radar engineer to determine acceptable radar positioning.

Reflections from large objects

Multipath conditions resulting from reflections from large buildings and large vessels can still impact on VTS radar performance resulting in the possibility of target signal cancellation or enhancement.  This effect is hard to predict, and it is unreasonable to expect to model multipath affected performance unless a (potentially expensive) site-specific radar model is developed.

Mechanisms in Port areas will often include reflection from large surfaces, e.g. large buildings, piers etc  and it can result in very challenging situations, even calling for more than one radar to cover certain areas.

Probability of Detection and False Alarm Rate

The probability of detection and the false alarm rates, used for calculations, should comply with those required to meet the operational performance.  Please note the definitions of radar PD and radar PFA, refer to section 2.1

It should also be noted that improved system performance may be obtained by allowing a higher radar PFA in combination with subsequent, enhanced plot extraction and tracking.

At specified maximum coverage ranges, the single-scan probability of detection values for VTS will typically lie in the range from 70% to 90%.

It is normally not desirable to have noise and clutter spikes presented to the VTSO in each scan.  Classically, optimal false alarm rates for VTS applications normally lie in the range from 10-4 to 10-5 for the radar video display.  Different values may apply for the tracking, on the condition that tracking requirements are met.

With modern high-resolution technology there is a tendency to allow higher false alarm rates and let the extraction and tracking discriminate between noise and targets of interest.

False alarms considered in the calculations, include unwanted information from noise and clutter, as presented to the VTSO or to the tracker (after signal processing), but not signals from other unwanted objects or other targets.

Radar coverage

Radar Configuration Examples

The following imaginary radars with simplified system characteristics and key parameters was used for the example calculations and illustrations made for this document.

Table 3 Imaginary, simplified VTS Radar System configurations and associated key parameters

This gives a good idea of the baseline coverage, but will, however, not cover all VTS’s and individual manufacturer characteristics, site specifics and settings for various operational areas will differ. The calculation examples can therefore only be used as a guideline, to be supplemented by specific calculations made for the individual VTS radar site.

Minimum detection range

Radars used for VTS will typically have a blind range close to the radar station depending on the pulse characteristics, the waveguide length and processing. In addition, the minimum detection range depends on antenna height and antenna vertical beam “look down angle” as illustrated in Figure 4., and with corresponding figures per Table 4.

Figure 4 Minimum detection range as a function of antenna height and lookdown angle.

The lookdown angle can as a rule of thumb be determined as the -20 dB point on the vertical antenna patten. Inverse cosecant beam techniques are used to improve the minimum detection range.

Antenna tilt mechanism will only have marginal influence on the minimum range, and it will also have the negative effect of increasing sea clutter returns.

Table 4 Calculated minimum range as a function of antenna characteristics and antenna hight

Radar Horizon

Figure 5 illustrates the basic geometry in target detection associated with antenna height above sea level (ASL) and the effect of the earth curvature, determined by the following formula:

Target Horizon=2.2×htarget0.5+hantenna0.5NM.

Figure 5 Geometry, antenna height and target height

Table 5 provides the radar Horizon for specific antenna heights and target heights corresponding to IALA targets type1-7

Table 5 Calculation of radar horizon.

I addition to minimum range and radar horizon, the target detection range is highly dependent on several other factors including target characteristics, weather (such as fog, sea state and rain) and atmospheric propagation conditions, calculated and expressed in statistical terms.

This is used to calculate range coverage taking several radar parameters, installation, and the environment into account. Formulas and calculations are provided as follows.

Analysis of Radar Detection Performance

The requirements for radar coverage and range detection performance can be determined by a combination of site inspections and radar system performance predictions, including:

evaluation of the effects from propagation conditions and obstructions in the on-site environment

calculation of range detection performance focused on the smallest targets of interest in poor weather conditions. All applicable losses (atmospheric and system related) should be included in calculations

evaluation of dynamic range requirements.

It will typically not be possible to cover all combinations of variables, and calculations are therefore calculations will be made based on simplified models of the targets and the environment based on statistical information with associated limitations and tolerances.

Furthermore, in the design of radar systems it is very important not only to focus on maximum detection range but also on perceived radar image quality and target separation at all ranges, the ability to handle clutter, the ability to suppress interferences and the ability to simultaneously handle of small and large targets in the VTS area covered by radar.

Notes: The use of radar prediction models requires a full understanding of their applicability and limitations. The calculations should be performed by radar specialists using a suitable tool and radar formulas given in this document.

It should also be noted that radar performance predictions are indications and not guarantees of real-world performance.  There are many variables and explicit or implicit assumptions within most known prediction tools and the results obtained can only cover approximations of target, radar, and environment behaviour.

Live testing during Site Acceptance may be included to evaluate the actual performance against expectations.

Warning: Cumulative Detection seen in some modelling tools can be very misleading for determination of VTS related radar detection performance.

Symbols

The followings symbols are used in the document to represent mathematical quantities. SI units are used throughout.

|  | Symbol |  | Symbol |
| --- | --- | --- | --- |
| Pt | Transmitter peak power | ϕ | Incidence angle |
|  | Chirp duration | Ai | Interference term |
| Lt | Transmitter loss | Au | Upwind term |
| Lr | Receiver loss | Aw | Wind term |
| Lp | Processing loss | σHH0 | Surface clutter cross-section |
|  | Wavelength | η | Volume clutter cross-section |
| G | Antenna gain | Pn | Received noise power |
| kB | Boltzmann constant | Psc | Received power from surface clutter |
| T | Receiver temperature | Pvc | Received power from volume clutter |
| N | Receiver noise figure | x | Signal to (noise + clutter) ratio |
|  | Radar cross section | Nb | Number of incoherently integrated pulses |
| r | Slant range | Y | Normalized detection threshold |
| Fp | Propagation factor | Pd | Probability of detection |
|  | Attenuation factor | Pr | Received power from target |
| B | Chirp Bandwidth | rr | Rain rate [mm/h] |

received powers

Received power from target

The power received from a given target, for a pulse compression radar, is computed from the following

Pr    =    PtBτLtLrLpλ2G24π  σ4π2r4 Fp4e-2αr

And for a pulse radar, such as a magnetron-based radar, the power received is

Pr    =    PtLtLrLpλ2G24π  σ4π2r4 Fp4e-2αr

The antenna gain depends on the angle, considering that both a direct path and an indirect, reflected path exists between transmitter antenna and target. The propagation factor (Fp) is a coherent addition of the phase terms between the direct path between target and antenna, and the reflected path, where the reflected path includes the reflectivity of the Earth surface. For observations of targets over sea, the reflection coefficient depends on the water salinity and sea state. For target ranges exceeding the radar horizon, a linear transition is used from the optical to the diffraction region. The target return power will be a function of target altitude and slant range. The radar cross section is a function of the radar wavelength as described elsewhere in this document, refer to Table 1 and Table 2.

Figure 6 give an example of received power versus range, X2 system, 20-meter antenna height, IALA type 4 target, Sea State 3 and 4 mm/hour uniform rain

Figure 6 Example of received power versus range

Received power from sea clutter

The power received from surface clutter is computed using the de-facto industry standard model of sea clutter from Georgia Institute of Technology (GIT) [13]. Based on the size of the illuminated area and wind speeds, the total received power from sea clutter is calculated using a radar equation similar to that of the target. The model uses a surface clutter cross section (in m2 / m2) of

σHH0= 10log3.9∙10-6λ ϕ0.4 AiAuAw  [1-10 GHz] (S and X-band)

σHH0= 10log5.78∙10-6λ ϕ0.547 AiAuAw  [10-100 GHz] (Ku-band)

The illuminated area depends on the antenna beam width and the duration of the pulse (for pulse compression radars it is the duration of the compressed pulse). The exact shape of the Interference term, Upwind term and Wind terms are different in the two frequency ranges.

Note: The models described here have been derived for pulse radars, and may be less representative for modern pulse compression radars with ability to resolve individual sea clutter spikes.

Received power from land clutter

The power received from land clutter is computed using the constant-gamma model of [16] and the illuminated area. The total received power from land clutter is calculated using a radar equation similar to that of the target and a surface clutter cross section (in m2 / m2) of

σHH0=γ sin⁡(ϕ)

Received power from volume clutter

The power received from volume clutter (rain) is computed from the radar equation and the illuminated volume based on the antenna beam width in azimuthal and elevation plane. The radar cross section of rain is taken from [12], Chapter 19. The clutter is modelled as stratiform rain resulting in a volume clutter cross section (in m2 / m3) of

η=π5λ41.86∙10-14∙rr1.6

Valid from S-band to Ku-band. The power received depends on the clutter cross section and the illuminated column of rain, the size of which is given by the antenna beam width and height of the rain cloud ceiling.

Thermal noise

The thermal noise contribution in the received power is given by the simple thermal noise term

Pn=kB TBN

Attenuation

The radar signals are attenuation by the atmospheric air and any potential water in the shape of droplets. Many highly detailed studies of attenuation exist, and the exact values of attenuation for both atmosphere and rain must be read of from a curve or table such as those given in [12]. In particular, the atmospheric attenuation does not have a simple dependence on wavelength due to molecular absorption bands in the Ku band.

Approximate 2-way values for VTS-radar relevant frequencies are given in Table 6 below.

Table 6 Atmospheric attenuation for various radar frequencies.

| Frequency [GHz] | Atmospheric loss [dB/km] | Dense Fog loss, 50m visibility [dB/km] | Rain rate 1 mm/h, loss [dB/km] | Rain rate 2 mm/h, loss [dB/km] | Rain rate 4 mm/h, loss [dB/km] | Rain rate 10 mm/h, loss [dB/km] | Rain rate 20 mm/h, loss [dB/km] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3 (S) | 0.016 | 0.007 | 0.001 | 0.001 | 0.003 | 0.007 | 0.015 |
| 9.5 (X) | 0.032 | 0.080 | 0.013 | 0.031 | 0.045 | 0.248 | 0.606 |
| 14 (Ku) | 0.038 | 0.181 | 0,056 | 0.124 | 0.276 | 0.793 | 1.762 |
| 17 (Ku) | 0.120 | 0.240 | 0.095 | 0.205 | 0.440 | 1.210 | 2.598 |

Evaporation Ducting

Ducting can severely affect radar range in clear conditions as described in ANNEX A, Section A.10. The effect of evaporation ducting is to reduce the loss in the diffraction region. A very simplified ducting model consists of the Naval Research and Development evaporation duct model where a fit to a waveguide solution is used to model the diffraction loss. All ranges and heights are scaled to an X-band solution. The dimensionless range scale factor is

R=3f/9600

And the dimensionless height scale factor is

Z=3f/96002

The fitted solution allows modelling of ducts up to a scaled height of 23.3 meters. For scaled duct heights less than 10.25 meters, the height gain function in the diffraction region is

Fz=C1zC2+C3zC4+C5

Where z is the scaled transmitter / target height. The coefficients C1 to C5 have a complicated form dependent on the scaled duct height. For well trapped modes with scaled heights between 10.25 and 23.3 meters, the height gain function is

F(z)=C1lnsinC1zC2+C41<z< zmaxC5zC6+C7z> zmax

The maximum scaled altitude of the first solution is dependent on the duct height. The various numeric constants in the waveguide fit model have complicated forms and require specialized simulation tools.

The effects of other than evaporation ducting phenomena will also require specialized simulation tools.

Detection probability

From the computed power terms, the final ratio of signal to noise and clutter at a given target position (slant range and altitude) is

x=PrPn+Psc+Pvc

This is converted to a probability of detection using the statistical models first developed by Swerling [14]. The choice of Swerling case depends on the target geometry as well as the decorrelation from sweep to sweep.

Large complex targets such as ships are classically modelled as Swerling case 1 targets with a large number of reflectors and small variation from sweep to sweep, with only variation from scan to scan. A complex target with fast variation is represented by Swerling case 2. Small point targets (missiles etc) are modelled as Swerling case 3 targets, with a single dominant reflector and little variation from sweep to sweep.

However, using modern radars with multiple frequencies, each frequency hitting the target more than once, leads to a partial decorrelation over the sweeps hitting the target. Swerling case 3 can be used as an approximation for complex surface targets when multiple carrier frequencies are used by the radar, since the curve for Pd versus signal-to-noise for Swerling case 3 lies between Swerling cases 1 (perfect correlation, many roughly identical scatterers) and 2 (perfect decorrelation, many roughly identical scatterers) For single frequency radars, complex surface targets are represented by Swerling case 1.

Table 7 Swerling Cases

| Target type | Single carrier frequency radar (magnetron or similar) | Multiple carrier frequency radar |
| --- | --- | --- |
| Large complex targets (larger ships, aircraft etc) | Swerling case 1 | Average between Swerling case 1 and 2 (represented by Swerling case 3) |
| Small point targets (buoy) | Swerling case 3 | Swerling case 3 |

A false alarm rate is specified, and the false alarm rate and number of incoherently integrated pulses determines the detection threshold from

Pfa=1-Γ(NB,Y)

After determining the detection threshold, an intermediate term, c, is first defined as

c=11+NBx2

The probability of detection can subsequently be calculated as

Pd=1-ΓNB-1,Y+cYNB-1NB-2!e-Y+

e-cY1-cNB-21-cNB-21-c+cYΓNB-1,Y(1-c)

This is the final output of the model providing a probability of detection for given target, antenna, and weather conditions

The example given by Figure 7 illustrates calculated coverage for an IALA type 4target in Sea State 3 and clear conditions assuming 20-, 40, 80 and 200-meter antenna height respectively.

Figure 7: Probability of detection

Calculation examples, comparison of technologies

Range performance

The radar technologies available for VTS has all their strength and weaknesses, especially dependant on weather and, range and antenna hight and the below examples are intended as a simplified guideline to what is possible and what should be avoided in specific situations.

reflect simplified coverage calculations for the system types defined per Table 3 – all set up for 24 nautical mile coverage and various weather conditions. Targets are modelled as a vertical line to minimize the effect from lobing on the calculations.

Table 8  Example of performance across various radar types and weather situations

| Radar Performance Calculation per IALA G111-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Radar type: Pulse Compression Compressed Pulse length: 50nS Antenna Hight ASL: 40 m Instrumented Range: 24 NM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Target detection range in SS3 |  | IALA1 target: AtoN without radar reflector. Small open boats, fiberglass, wood or rubber |  |  |  |  | IALA 2 target: In‐shore fishing vessels etc |  |  |  |  | IALA 3: AtoN with radar reflector |  |  |  |  | IALA 4 target: Small metal ships etc. |  |  |  |  |
|  |  | Clear + 7m duct | Clear | 4 mm/h rain | 10 mm/h rain | 20 mm/ rain | Clear + 7m duct | Clear | 4 mm/h rain | 10 mm/h rain | 20 mm/ rain | Clear + 7m duct | Clear | 4 mm/h rain | 10 mm/h rain | 20 mm/ rain | Clear + 7m duct | Clear | 4 mm/h rain | 10 mm/h rain | 20 mm/ rain |
| Radar Type | S1 | Target RCS <<1 m2 at this frequency may not allow for reliable detection |  |  |  |  | 4.3 NM | 4.2 NM | 4.2 NM | 4.2 NM | 4.1 NM | 5.9 NM | 5.9 NM | 5.9 NM | 5.9 NM | 5.8 NM | 9.0 NM | 8.9 NM | 8.9 NM | 8.9 NM | 8.8 NM |
|  | S2 |  |  |  |  |  | 6.7 NM | 6.4 NM | 6.3 NM | 6.2 NM | 5.8 NM | 8.7 NM | 8.5 NM | 8.4 NM | 8.3 NM | 8.1 NM | 12.1 NM | 12 NM | 11. 7 NM | 11. 7 NM | 11.5 NM |
|  | X1 | 4.1 NM | 4.1 NM | 3.9 NM | 2.9 NM | < 1 NM | 6.1 NM | 6.2 Nm | 5.9 NM | 5.4 NM | 2 NM | 8.2 NM | 8.1 NM | 7.8 NM | 7.2 NM | 3 NM | 11.9 NM | 11.4 NM | 11 NM | 10.4 NM | 8.8 NM |
|  | X2 | 6.8 NM | 6.3 NM | 5,6 NM | 4,5 NM | 3,5 NM | 9.5 NM | 8.9 NM | 8.5 NM | 7.7 NM | 6.6 NM | 12.1 NM | 10.8 NM | 10.5 NM | 9.8 NM | 8.7 NM | 14.9 NM | 13.7 NM | 14.4 NM | 12,8 NM | 11,6 NM |
|  | X3 | 10.0 NM | 8.6 NM | 6.7 NM | 5.3 NM | 4.3 NM | 13.5 NM | 11.2 NM | 10.0 NM | 8.8 NM | 7.7 NM | 16.5 NM | 13.0 NM | 12.2 NM | 11.2 NM | 10.2 NM | 21.9 NM | 15.8 NM | 15.2 NM | 14.3 NM | 13.3 NM |
|  | Ku1 | 5.0 NM | 5.0 NM | 4.1 NM | < 1 NM | < 1 NM | 7.8 NM | 7.7 NM | 6.6 NM | 2.9 NM | < 1 NM | 10.0 NM | 9.6 NM | 8.3 NM | 4.3 NM | 3.9 NM | 15.3 NM | 12.8 NM | 11.6 NM | 6.6 NM | 5.9 NM |
|  | Ku2 | 9.9 NM | 8.3 NM | 6.3 NM | 4.4 NM | < 1 NM | 14.9 NM | 11.0 NM | 9.8 NM | 8.2 NM | 2.9 NM | 18.8 NM | 12.7 NM | 11.6 NM | 10.1 NM | 4.4 NM | 24.0 NM | 15.3 NM | 14.3 NM | 12.7 NM | 6.8 NM |

The calculations are simplified and indicative for typical achievable performance. Targets are modelled as point targets and calculations use equivalent pulse power and compressed pulse width as specified per Table 3

Table 9  Example of performance across various radar types, longer range

| Radar Performance Calculation per IALA G111-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Radar type: Pulse Compression Compressed Pulse length: 50nS Antenna Hight ASL: 200 m Instrumented Range: 48 NM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Target detection range in SS3 |  | IALA1 target: AtoN without radar reflector. Small open boats, fiberglass, wood or rubber |  |  |  |  | IALA 2 target: In‐shore fishing vessels etc |  |  |  |  | IALA 3: AtoN with radar reflector |  |  |  |  | IALA 4 target: Small metal ships etc. |  |  |  |  |
|  |  | Clear + 7m duct | Clear | 4 mm/h rain | 10 mm/h rain | 20 mm/ rain | Clear + 7m duct | Clear | 4 mm/h rain | 10 mm/h rain | 20 mm/ rain | Clear + 7m duct | Clear | 4 mm/h rain | 10 mm/h rain | 20 mm/ rain | Clear + 7m duct | Clear | 4 mm/h rain | 10 mm/h rain | 20 mm/ rain |
| Radar Type | S1 | Target RCS <<1 m2 at this frequency may not allow for reliable detection at any sea state |  |  |  |  | 4.9 NM | 4.9 NM | 4.8 NM | 4.7 NM | 4.3 NM | 7.7 NM | 7.7 NM | 7.7 NM | 7.6 NM | 7.5 NM | 14.1 NM | 14.1 NM | 14.1 NM | 14 NM | 13.9 NM |
|  | S2 |  |  |  |  |  | 10.9 NM | 10.9 NM | 10.8 NM | 10.7 NM | 10.5 NM | 15 NM | 15 NM | 15 NM | 14.9 NM | 14.7 NM | 21.7 NM | 21.7 NM | 21.6 NM | 21.5 NM | 21.5 NM |
|  | X1 | Poor detection at all ranges at sea state 3 or higher |  |  |  |  | 2.6 NM | 2.6 NM | 2 NM | NIL | < 1 NM | 4.1 NM | 4.1 NM | 3.7 NM | 2.6 NM | 1.3 NM | 8.9 NM | 8.9 NM | 8.1 NM | 6.8 NM | 5.1 NM |
|  | X2 | 3.6 NM | 3.6 NM | 1.7 NM | < 1 NM | < 1 NM | 8 NM | 8 NM | 5.3 NM | 2.1 NM | < 1 NM | 12.5 NM | 12.5 NM | 11.3 NM | 7.3 NM | 2.6 NM | 24.8 NM | 24.8 NM | 21.4 NM | 16,6 NM | 8.5 NM |
|  | X3 | 13,6 NM | 11.5 NM | 10.8 NM | < 1 NM | < 1 NM | 20.6 NM | 20.6 NM | 18.7 NM | 4 NM | < 1 NM | 23.9 NM | 23.9 NM | 22.7 NM | 19.1 NM | 7.3 NM | 28.1 NM | 28.1 NM | 26.3 NM | 25.7 NM | 17.3 NM |
|  | Ku1 | < 1 NM | < 1 NM | < 1 NM | < 1 NM | < 1 NM | 1.7 NM | 1.7 NM | 1.4 NM | < 1 NM | < 1 NM | 6.9 NM | 6.9 NM | 1.6 NM | 1.4 NM | < 1 NM | 3.5 NM | 3.5 NM | 3 NM | 2.6 NM | 2.1 NM |
|  | Ku2 | 8.3 NM | 8.3 NM | < 1 NM | < 1 NM | < 1 NM | 13.2 NM | 13.2 NM | 5.3 NM | < 1 NM | < 1 NM | 21.9 NM | 21.9 NM | 10.8 NM | 3.7 NM | < 1 NM | 28.6 NM | 28.6 NM | 20 NM | 11.3 NM | 6.6 NM |

Note that 20 MM/h uniform rain is rare over such long distances (48 NM) and evaporation ducting has lite influence when the antenna is high elevated. The values in those fields are therefore Greyed.

Circular versus linear polarization

Utilisation of Circular polarisations has traditionally been done to improve performance in rain; however, it also reduces the ability to detect small non-metallic targets and some types of reflectors. Furthermore, modern pulse compression radars and narrow beamwidth antennas reduces the sensitivity to rain

Table 13 reflects calculations of various combinations. The Pulse radar is set up with a pulse length of 0,5 uS in the examples. 10 mm/h uniform distributed rain was assumed as more dense wides spread rain is rare.

Table 10 Comparison between Horizontal and Circular polarization

| Radar Performance Calculation per IALA G111-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Radar type: X2 Antenna Hight ASL: 40 m |  |  |  |  |
| Radar type / Polarization | Detection probability for combinations of precipitation and polarization Instrumented Range: 24 Nautical miles Propagation: 10 mm/h rain Sea State: 3 |  |  |  |
|  | X2 pulse comp / Horizontal | X2 pulse comp / Circular | X2 pulse (magnetron) / Horizontal | X2 pulse (magnetron) / Circular |
| IALA1: AtoN without radar reflector. Small open boats, fiberglass, wood or rubber | 4,5 NM | This type of target may not give returns from circular polarized radar | < 1 NM | This type of target may not give returns from circular polarized radar |
| IALA 2: In‐shore fishing vessels etc. | 7.7 NM | 7.4 NM | 4.7 NM | 8.0 NM |
| IALA 3: AtoN with radar reflector | 9.8 NM | 9.1 NM | 8.5 NM | 10.1 NM |
| IALA 4: Small metal ships etc. | 12.8 NM | 12.0 NM | 12.5 NM | 13.0 NM |
| IALA 5: Small coasters etc | 15.4 NM | 12.8 NM | 15.7 NM | 15.7 NM |
| IALA 6: Large coasters etc. | 18.1 NM | 17.4 NM | 18.6 NM | 19.0 NM |
| IALA 7: Large coasters, bulk carriers, and cargo ships. | 20.9 NM | 20.2 NM | 21.7 NM | 21.2 NM |

Target separation

The ability to separate two small point targets in Range and Angle (azimuth) is illustrated  by Figure 3, where the range resolution and separation are, for traditional magnetron radars, linked to the transmitted pulse length and linked to the compressed pulse length for the pulse compression (solid-state) radars.

The angular resolution and separation is given by the antenna characteristics. The range separation performance is generally equal over distance.

Increasing frequency will improve target separation and increase gain for antennas with the same physical size and thereby increase range but will also increase susceptibility to rain, however, pulse compression radars are less sensitive to this than pulse radars.

Azimuth Separation

Figure 8 shows an example of returns from two identical targets using an antenna with 0.45° -3dB beamwidth and separated by 0.8 o. The echoes of T1 and T2 are added, which in average is illustrated by the red. However, the separation will also be dependent on integration processes and the phase of the signals received and the resulting separation can vary between the dotted and the dashed lines in the figure.

To calculate the resulting azimuth separation, the antenna beamwidth and sampling interval must also be considered where the sample interval as a rule of thumb can be added. I.e. 0.08° should be added if azimuth is sampled every 360°/4096.

Figure 8 Example of Return from two identical targets placed at the 0.6° and 1.4° angles

In the worst-case azimuth separation for point targets are:

- Approximately 1.75 times the – 3dB antenna beam width + 1 resolution sample to achieve 6 dB displayed separation.

- Approximately 2.5 times the – 3dB antenna beam width + 1 resolution sample to achieve 20 dB displayed separation.

The first will normally be sufficient for visual separation of targets and the later should be sufficient to avoid swap of tracked targets.

Table 11 Typical azimuth separation as a function of system type and range.

Range Separation

Figure 9 illustrate an example of returns from two targets separated in range, and the sum (power) of the two. The range separation is determined by the frequency of range samples and the bandwidth. The echoes of T1 and T2 are added, which in average is illustrated by the red. However, the separation will also be dependent on integration processes and the phase of the signals received and the resulting separation can vary between the dotted and the dashed lines in the figure.

To calculate the range separation, the effective bandwidth and sampling interval must be considered, where the worst-case range separation for point targets are:

- Approximately 2 times the – 3dB pulse width + 1 resolution sample to achieve 6 dB displayed separation.

- Approximately 3 times the – 3dB pulse beam width + 1 resolution sample to achieve 20 dB displayed separation.

Figure 9 Range discrimination

The sampling resolution should be added to obtain a figure for the achieved range separation, i.e., 3 meters in the case of 50 MHz sampling rate.

This will i.e., for a 5o nS effective pulse width (7,5 m) and 50 MHz sampling rate result in typical range separation figures of

- 18 m at the -6dB points

- 24 m, at -20 dB points

Note that for larger targets and for separation of dissimilar sized targets, the definition of separation is highly dependent on physical size, aspect angles, and other radar characteristics.

Target Positional Accuracy

The “accuracy” is characterized in accordance with ISO 5725-1 by two parameters; trueness (systematic error) and precision (random error), which are illustrated in Figure 10.

Figure 10 Definition of trueness and precision

The “Overall Accuracy” is the average deviation between sets of single measurements and the corresponding set of reference values. Overall accuracy thus equals the definition of trueness provided that enough measurements are averaged.

The overall RMS accuracy is calculated by:

AccuracyRMS=Trueness2+Precision2

The impact of the antenna height on the measuring accuracy (slant range vs. plan range) is additional.

Typical requirements for VTS are listed Table 12. Values are valid for stable tracks on uniformly moving, small point targets.

Table 12 Typical Target Positional Accuracies for pulse compression radars

| Operational area | Typical obtainable Slant Range and Azimuth Target Positional Accuracies, RMS |  |  |
| --- | --- | --- | --- |
|  | Systems designed for areas without dense traffic | Intermediate | Systems designed for areas with dense traffic |
| Inland waterways, like rivers and canals | 20 meters/ 0,5 degree | 15 meters/ 0,35 degree | 10 meters/ 0,25 degree |
| Port with berths and anchorages |  |  |  |
| Port approach | 30 meters/ 0,5 degree | 20 meters/ 0,35 degree | 15 meters/ 0,25 degree |
| Coastal areas and offshore | 100 meters/ 0,5 degree (S-band: 1 degree) | 50 meters/ 0,35 degree (S-band: 1 degree) | 25 meters/ 0,25 degree (S-band: 0,5 degree) |

As a rule of thumb, the range inaccuracy will be increased by 20 % of the pulse length for pulsed magnetron radars.

Target Positional update rate

The VTS operation and especially the tracking function is dependent of timely update rates of the radar information and the required radar update rate is determined by the behaviour of the expected target types.  Typical update rates(antenna rotation rates) are listed in Table 13

Table 13 Update rate

| Operational area | Typical Positional Update Rate |
| --- | --- |
| Inland waterways, like rivers and canals | 1.5 – 3 seconds |
| Port with berths and anchorages | 1.5 – 3 seconds |
| Port approach | 1.5 – 3 seconds |
| Coastal areas and offshore | 2-6 seconds |

Dynamic characteristics

Dynamic range

The required dynamic range is determined by:

The ratio between the largest nearby objects expected and the smallest distant objects to be detected.

Target return signal fluctuations including multipath.

Requirements to the radar(s) can be determined from the characteristics of the objects in the coverage area of the individual radar.  This is shown as a function of RCS and detection range in Figure 11.

The figure represents targets in free space, which normally is sufficient for VTS radar requirements when combined with 10 dB allowance for target fluctuations.

Figure 11 Dynamic characteristics of signal received versus target RCS and target range

It should also be noted that the vide span of signals makes it very difficult to measure the dynamic range of a given radar and it is recommended to use analytical methods i.e. considering the following for a pulse compression radar:

- The effective dynamic range of the ADC where each bit corresponds to 6 dB, i.e:

  - The max dynamic range for a 14-bit ADC 84 dB.

  - However, inaccuracies in the ADC limit the dynamic range down to the so called effective dynamic bit that may be 13bit, equal to 78 dB.

- The required minimum bit-level of sampled noise in ADC (full sampling bandwidth), i.e:

  - To make the ADC sampling noise small compared to the thermic noise in the RF receiver, the received thermic noise is gained up to be 10 dB above the quantization noise of the ADC. This uses 10 dB of the effective dynamic range of the ADC

- Noise power reduced by the ratio of processing bandwidth relative to sampling bandwidth i.e:

  - The noise power level sampled by the ADC is received in a full Nyquist bandwidth of 50 MHz. As the digital match filter reduces the bandwidth defined by the range resolution, the noise power level is equally reduced. The bandwidth ratio is up to 50 to 8, giving a reduction factor of 6 times or 8 dB. This is referred to as over-sampling.

- Range, azimuth or area dependent sensitivity (swept gain) prior to ADC

  - Typically, a feature of advanced radars adding to the effective dynamic range

  - May be used to attenuate specific areas based on previously scans.

- Range dependent sensitivity of different pulse compression gain of short and long chirps.

  - Typically, a feature of advanced radars adding substantially to the effective dynamic range

- Reduction of the effective RCS of large structure when they are in close range

  - When the RCS of an object is defined, it is normally to assume that the object is in far field of the radar. However, the phase difference to all parts of a distributed target is significant at X-band when the object is in close range to the radar.

  - Likewise, the radar beam may not illuminate all of a nearby target, e.g., a large ship.

  - As an example, the RCS of a 100m high, 6 to 4m tapered cylinder (the tubular tower of wind turbines) is 66dBsqm in the far field, but in 20km, it is reduced by 13dB to 53dBsqm. In 2km it is reduced by 23dB to 43dBsqm.

  - The effect depends of the object type, but it will not be an exaggeration to include 10dB in the dynamic range on this account.

- Transmitter power variations

  - Depending on geographical conditions it may be possible to reduce power towards areas with the largest targets. (Sector power transmission)

- Reduction of the antenna gain when operating in the antenna near field.

  - The effect will only be significant on very close object, typically less than 100m.

- Acceptable saturation handled without violation of specified range side lobe level.

  - Minor saturation i.e. 5 dB can for advanced radars be accepted and handled by the CFAR, especially in the case of short chirps.

Actual values will be dependent on the individual manufacturer solution. It is furthermore unlikely that all elements can be considered simultaneously.

Table 14 gives a calculation example and Table 15 state values typical for VTS applications.

Table 14 Example of a dynamic range evaluation for an imaginary pulse compression radar

| 1 | The effective dynamic range of the ADC | 78 dB |
| --- | --- | --- |
| 2 | The required minimum signal level of the sampled noise | -10 |
| 3 | Bandwidth reduction factor, oversampling | +8 |
| 4 | Swept gain prior to ADC | +0 |
| 5 | Difference in pulse compression gain of near range and long-range chirps | +26 |
| 6 | Sector power transmission | +0 |
| 7 | Near range RCS reduction | +10 |
| 8 | Near range antenna gain reduction | +0 |
|  | Unsaturated dynamic range | 112 dB |
| 9 | Saturation giving acceptable range side lobe levels | +5 |
|  | Dynamic range incl acceptable saturation | 117dB |

Table 15  Dynamic range typical for VTS applications

| Operational area | Typical Dynamic Range |  |  |
| --- | --- | --- | --- |
|  | By systems designed for areas without large structures, or very large ships | Intermediate | By systems designed for areas with large structures and/or very large ships |
| Inland waterways, like rivers and canals | 90 dB | 100 dB | 110 dB |
| Port with berths and anchorages | 100 dB | 100 dB | 100 dB |
| Port approach | 90 dB | 100 dB | 110 dB |
| Coastal areas and offshore | 80 dB | 100 dB | 120 dB |

Sidelobes

Side lobes, ref. Figure 12 are unwanted, as they will limit the size of a small Radar Cross Section (RCS) target that can be detected when located next to a large RCS target.

Range (time) side lobes

Antenna side lobes

Target

Target

.

Unwanted side lobes

Clean target echo

Figure 12 Range and Antenna (Azimuth) side lobe effects

The ratio between the peak level of the target and the highest side lobe is called the Peak Side Lobe Ratio (PSLR).

Typically, azimuth (antenna) side lobes are the only contributor in traditional magnetron pulse radars.  FMCW and pulse compression radars are subject to range as well as azimuth side lobes.

Side lobes need to be sufficiently low to avoid masking of smaller targets in the proximity of large returns from targets or clutter.  In addition, low side lobes minimise the probability of false targets arising from other large returns.  However, it is normally not possible to eliminate false echoes from range and time sidelobes completely.

Table 16 recommends the typical allowed signal presented to the display and plot extractors.

The figures account for two-way propagation, therefore the antenna side lobe requirements (one way) equal half the values indicated (dBs divided by 2).

Table 16 Typical requirements to maximum Side Lobe Level Relative to Non-saturating Target Signals

| All Operational Areas | Typical side lobe Level requirements |  |  |
| --- | --- | --- | --- |
|  | Areas without large structures, or very large ships | Intermediate | Areas with large structures and/or very large ships |
| Maximum near side lobe level (within +/- 10 in azimuth and +/- 250 m in range from any target) | - 52 dB (-26 dB one way) | - 54 dB (-27 dB one way) | - 56 dB (-28 dB one way) |
| Maximum far side lobe level (outside +/- 10 in azimuth and +/- 250 m in range from any target) | - 66 dB (-33 dB one way) | - 68 dB (-34 dB one way) | - 70 dB (-35 dB one way) |

Note that objects within the near field of the antenna may increase azimuth side lobe levels. This should also be carefully considered when defining locations or radars.

Antenna Side Lobes

The antenna designer uses an amplitude weighting function to control the azimuth side lobe levels to a level which recognises the requirements for close in (< 10° from the main lobe peak) azimuth side lobes and the requirements for side lobes beyond this region.  Side lobes are specified as a ratio (in dB) relative to the antenna beam peak.

The antenna gain defined from a specific point in the radar system is specified as a ratio above ‘isotropic’ (dBi) and can be increased by increasing the physical size of the antenna.  This will also reduce the beamwidth (azimuth, elevation or both).

Sidelobes exist in Azimuth and elevation, however, elevation side lobes are unlikely to be a major contributor to the performance of the VTS radar system. So called grating lobes are of special concern for array antennas and these should be included in consideration and measurement of side lobe levels.

Also note that structures near antennas (in some cases up to 100 metres) tend to distort wave propagation and, thereby, increase the azimuth side lobe level in the direction of such structures.

Range Side Lobes

Pulse compression and FMCW uses an amplitude weighting function alike that used for antennas to control the range side lobe levels to a level which recognises the requirements.

Note: Suppression values for range side lobes should in principle be the same as for the antenna sidelobes. However, it might not be economically feasible to obtain more than 55-60 dB suppression by such weighting and follow-on suppression/cancellation of range sidelobes by signal processing may be required in order to avoid false tracks and poor video quality.  The drawback from such measures is reduced dynamic range locally which my limit the ability to detect small targets near to larger ones.

Environmental Influence

There is a strong and complicated relationship between radar performance, geographical constraints and environmental conditions and it is highly recommended that individual assessments, involving radar and meteorological experts, are made for each VTS site separately.  The sections below discuss typical conditions covering the majority of VTS installations.

Precipitation and Sea

Rain will attenuate signals and backscatter from sea and rain will “compete” with the signals received from targets and thereby reduce detection ranges.

Reduced Visibility, Fog, sand, Dust

Reduced visibility due to Fog, Sand and Dust will in general have small influence on radar coverage, compared to that of clear conditions. However, Sand and dust storms are often associated with abnormal propagation conditions as discussed below

Air Mass (Propagation)

The performance of surface-based radar systems is strongly influenced by the electromagnetic properties of the atmosphere and the surface of the Earth.  In free space, electromagnetic waves propagate in straight lines from the antenna toward the targets and back.  However, radars located near the Earth’s surface should deal with, and adjust to the diffraction and refraction of the propagating wave.

Performance should, in all cases, be evaluated assuming Standard Atmospheric Conditions, combined with precipitation and sea state information for the individual location.  Evaluation of the effects from adverse propagation should in addition be considered for hot, dry areas of the world, e.g. the Arab Gulf.

Propagation in the Standard Atmosphere

An electromagnetic wave observed at a target consists of a summation of an infinite number of contributions from different paths to the target leading to constructive and destructive contributions at the target.  The return path suffers from similar effects.  For small low targets this results in shorter detection ranges, than the distance calculated by simple line of sight calculations.

In addition, the barometric pressure and water vapour content of the standard atmosphere decreases rapidly with height, and the temperature decreases slowly with height.  This causes the electromagnetic waves to bend a little towards the Earth’s curvature.

Radar parameters, losses in transmission lines (not only waveguide), processing losses, clutter and precipitation add to the complexity and, even for the Standard Atmosphere, it is necessary to combine this with propagation factors by radar calculation tools in order to determine the predicted performance for a VTS radar sensor.  This is typically modelled by increasing the radius of the Earth by a multiplier (typically 1.33) and assuming straight-line propagation.

Sub-refraction and Super-refraction

Sub-refraction, bending the electromagnetic waves up, and super-refraction, bending the electromagnetic waves down, exists when the atmosphere deviates from the standard.

Sub-refraction can be caused by fast reduction of temperature and slower reduction of water vapour content with height, bending the electromagnetic waves towards space.  However, this phenomenon occurs rarely in nature.

Super-refraction can be caused by temperature increase with height (generally by temperature inversion) and/or rapid decrease of water vapour with height, decreasing the refractivity index (N).  Decreasing the refractivity gradient will eventually cause it to reach the critical gradient, at which point an electromagnetic wave will travel parallel to the Earth’s curvature.

Ducts and Trapping Layers

Super-refraction will develop into trapping layers, if the refractivity gradient decreases beyond the critical gradient, at which point the electromagnetic wave will be trapped and follow the Earth’s curvature.

Ducts act like waveguides for propagating waves bordered by trapping layers or the Earth’s surface.  To couple into a duct and remain in a duct, the angle of incidence must be small, typically less than 1°.

Ducting can be categorized into three main types:

Evaporation duct:

- Weak, caused by evaporation from the sea surface, and only at low levels (maximum of 40 metres ASL).

- Generally increasing the radar horizon, especially for low mounted antennas.

Surface-based duct:

- Surface ducts caused by low level inversion (increase of temperature /decrease of humidity with height), up to 500 metres ASL.

- Increase of radar range, depending on duct and antenna height.

Elevated duct:

- 0.2-2 km above the surface.

- Typical no effect on surface-based antennas.

The effects are typically increased range but also increased amounts of noise and 2nd / multiple time around returns which may appear as false radar returns.

Notice that the electromagnetic waves are refracted (bent) and not reflected by the trapping layers.

Evaporation Ducts

Evaporation ducts exist over the ocean to some degree, almost all the time.  A change in the moisture distribution without an accompanying temperature change will lead to a trapping refractivity gradient.  The air in contact with the ocean surface is saturated with water vapour, creating a pressure that is decreasing to some value above the surface. This results in bending down of the electromagnetic waves as illustrated by Figure 13

Figure 13 Coverage diagram, in standard atmosphere (left) and including an evaporation duct (right).’

This will cause a steep refractivity gradient (trapping) near the surface but will gradually equalise towards normal refractivity gradient at a certain height, which is defined as the evaporation duct height.

Evaporation ducts are generally increasing the radar detection range and the antenna can be located above the duct and still have extended propagation strength.  The effect of evaporation ducts is usually more noticeable for higher frequency radars. This means that the radar range extension caused by an evaporation duct will in general be more noticeable for a X-band radar than for a S-band radar.

For typical coastal radar installations, evaporation duct heights of 6 – 15 metres lead to the longest detection ranges.  Evaporation duct heights of more than 10 metres will also introduce an increased amount of clutter, setting additional demands to clutter processing and noise reduction capabilities.

For example, investigations of weather data from the Arab Gulf area shows that evaporation ducts exist all the time with typical duct heights of 5 to 25 metres, resulting in increased radar range in 80% of the time and increased clutter in 50% of the time.

Surface-based Duct

Surface based ducts can be much stronger than evaporation ducts.  They occur when the air aloft is hot (and dry) compared to the air at the Earth’s surface.  Over the ocean and near land masses, dry continental air may be advected over the cooler water surface; either directly from leeward side of continental land masses or by circulation associated with sea-breeze.

In addition to the temperature inversion, moisture is added to the cool marine air by evaporation, increasing the trapping gradient.  In coastal areas, this leads to surface trapping ducts.  However, away from land, this trapping layer may well rise from the surface thereby creating an elevated duct.

The electromagnetic wave will be refracted towards the surface of the Earth and be trapped in the duct like in a waveguide.  This kind of trapping condition greatly increases the surface detection range – and the amount of noise received.  Note that surface detection may occur far beyond the radar horizon with a ‘dead zone’ in between.

Surface based ducts are often combined with evaporation ducts and examples of radar performance in such conditions, as illustrated by Figure 14

Figure 14 Example of simulated radar coverage in surface based + evaporation ducting conditions

Elevated Duct

Generally, elevated ducts occur from descending, compressed and thereby heated air, from anticyclones, approaching the marine boundary layer and causing ducts like that simulated in Figure 14Figure 15.  Elevated ducts may also occur from elevating surface-based ducts.

Surface detection might also occur in this case far beyond the radar horizon with a ‘dead zone’ in between, adding noise to the radar image.

Figure 15 Coverage diagram, elevated duct

Severe Ducting at Coastlines Adjacent to Hot Flat Deserts

The large temperature variation between night and day in desert areas and the associated pressure differences between land and sea tend to cause very strong temperature inversion during night-time and result in strong sea breeze in the afternoons.  This can result in very severe ducts.  This type of duct can be very low, as little as 15 metres has been experienced

Range performance is very different for an antenna positioned inside or above these ducts, and radar systems with an antenna positioned within such a duct will have substantial increase in the detection range for surface targets.  If the antenna is positioned above the trapping layer (outside the duct), the electromagnetic wave will be refracted and the detection range for surface targets will be substantially reduced.

Figure 16 show a coverage diagram based on a measured condition at a coastline adjacent to hot flat deserts- 15 m antenna height (left) and 55 m antenna height (right).

Figure 16 Coverage diagram based on a measured condition

Note.  The radar detection using antennas positioned inside the duct (left) and above the duct (right) corresponded to the simulated coverage diagram.

The sea breeze also causes eddies over the sea, forming distinctive sea clutter patterns.  The eddy results in a ‘snake’ like pattern moving forth and back for a few hours in the afternoon on hot days with strong sea breezes (see 0). Of course, this may disturb display and tracking.

Eddy (yellow)

History (red)

Figure 17 One hour of recordings with trials (snail tracks) shown in red.

Note.  The yellow ‘snake’ at sea is an eddy moving back and forth with a speed of approximately 4 knots.

Other Influencing Factors

Obstructions, e.g., topography, buildings and other man-made structures may block or reflect radar signals.  Other radars and sources of electromagnetic radiation may cause interference.

Inland and harbour VTS will often require special considerations as the number of structures, their density and their close ranges can create very complex distortions.  Additional care should be taken to assess and mitigate effects caused by natural and man-made structures such as bridges, buildings, riverbanks, sheet metal pilings, and steep bends.

Shadowing Effects

Radar detection may be blocked by shadowing effects that, to the extent possible, should be avoided.  Such effects include:

Targets being hidden by larger targets or other obstructions.

Masking of small targets by the effects of range and time side lobes.

These effects can be minimised by the appropriate siting of radars and selection of equipment with low side lobes.

Interference

Interference can be split into susceptibility (received interference) and compatibility (transmitted interference).

To minimise interference, separation between wanted and unwanted transmissions has to be optimised – this can be achieved by a combination of frequency separation, physical separation of transmission sites, antenna directionality, sector blanking, separation by time and also by ensuring that all the systems competing for the same or adjacent spectrum do not transmit excessive and unnecessary transmit power levels or transmit time periods.  The radar receiver design will typically be very sensitive (to achieve the required performance), although gain control techniques (swept gain or Sensitivity-Time Control (STC)) offers further resistance.  Waveform designs incorporating staggered PRFs, and processing schemes designed to reject known interference patterns can also aid the radar receiver to suppress unwanted returns.

Radar Susceptibility

In the case of any radar installation, (e.g. a permanent VTS installation of a radar or of each radar in a VTS network), the performance of that radar can be detrimentally affected by received emissions from other radiating sources (physically adjacent VTS radars, maritime shipborne radars, and other users (legitimate or otherwise) of the electromagnetic spectrum.  Typically, local legislative regulations and restrictions will control and minimise unwanted received signals, but elimination of such signals is likely to be impossible.  National spectrum allocation authorities should always be approached by a VTS integrator when considering any changes to a VTS network (radar, microwave link, communications etc.) to enable a holistic view of the changes and how they might affect all users.

The radar design can assist in minimising the susceptibility to unwanted received interference, e.g. by utilising low antenna side lobes, avoiding large reflecting surfaces, minimising receiver front end bandwidth etc.

Note that FMCW and pulse compression radars may typically require larger front-end receiver bandwidths than conventional magnetron systems.  The multi-pulse waveforms transmitted (and consequently received) by pulse compression radars have to achieve a compromise between pulse chirp bandwidth (related to range cell size and hence range resolution), use of frequency diversity (to optimise performance in clutter), unwanted pulse to pulse interaction and so on versus spectrum usage and hence unwanted susceptibility with other transmitting spectrum users.  FMCW radars transmit and receive (at low levels) for 100% of the time across a swept bandwidth.

Radar Compatibility with Other Users

In the case of any radar installation (e.g., a permanent VTS installation of a radar or of each radar in a VTS network), the performance of adjacent systems can be detrimentally affected by transmitted emissions from the radar in question (physically adjacent VTS radars, maritime shipborne radars, and other users (legitimate or otherwise) of the electromagnetic spectrum).  Typically, local legislative regulations and restrictions will control and minimise unwanted transmitted signals but elimination of the influence of such signals is likely to be impossible.  As with susceptibility above, National spectrum allocation authorities should always be approached by a VTS integrator when considering any changes to the RF sub- systems within a VTS network.

The radar design can assist in minimising the impact of transmitted signals, for example by utilising low antenna side lobes, avoiding large reflecting surfaces, minimising transmit power, consideration of peak and mean power levels, sector blanking, physical location of the radar etc.

Note that conventional magnetron radars require larger peak power levels than pulse compression and FMCW radar systems.  The magnetron technology can result in unnecessary wideband spectral emissions unless steps are taken to include frequency band pass filtering (which has an inherent loss to the wanted signal).  However, pulse compression radars and FMCW radars, although utilising lower peak powers, use techniques, which may include frequency modulation, pulse-to-pulse frequency variation, frequency diversity etc. all of which increase the use of the spectrum and increase the chances of unwanted degradation of adjacent systems.

Influence from Wind Farms

Wind turbines produce large static target-like returns which, from a VTSO’s perspective, are normally easy to distinguish from vessel traffic.

Multipath propagation where signals are reflected by objects within the radar coverage can generate so called ghost echoes as illustrated by Figure 18.

To calculate the estimated amount of ghost targets, as well as the ghost targets size in RCS, there are basically three scenarios to consider, which for a wind turbine situation is shown in Figure 19.

The complex return from a wind turbine is made up of two key elements:

The tower and generator housing offering a large static zero-Doppler RCS, in some cases up to 1 million m2;

The rotating blades of the turbine offering a complex spread of non-zero-Doppler RCS, typically up to 100 m2, which will vary with wind direction and speed.

This composite return will be seen as a large static target by a conventional pulse radar, whereas FMCW and coherent radars using Doppler processing will see a complex target spread across the Doppler domain.

The influence, independent of the applied radar technology, will be reflections causing unwanted ghost echoes and suppression of nearby targets.  The large RCS may also result in antenna side lobe returns, resulting in reduced detectability.  The symmetrical layout of wind farms may add further to the disturbance

Figure 18 Multipath propagation resulting in Ghost images

Figure 19 Possible scenarios for ghost target analysis

Signal Processing and Tracking

Modern radars use a variety of signal processing techniques to discriminate targets of interest from noise and clutter, often based on statistical properties of the signal. This will increase sensitivity of the radar by allowing for more noise in the incoming signal. In reverse it will also result in higher dynamic range of the incoming signals setting additional requirements to the receiver. The techniques will often include pulse compression, correlation, and automatic adaptation to the environment.

The tracking functions in VTS will often offer fully automatic target acquisition and functions to correlate with information from other sources i.e., AIS.

Tracking stationary objects or slowly moving surface objects like boats is normal done without Doppler based processing, whereas Doppler based processed video provides enhanced discrimination between moving targets and background clutter for surface targets with radial speeds significantly different from the radial speed of nearby clutter.

The tracker function may be part of the individual radar sensors, hosted as part of the overall VTS system or a combination. Please refer to G1111-1 for further discussion on the subject

- Acceptance of VTS Radar Systems

IALA Guideline 1111-9 Acceptance Framework of VTS System provide general acceptance steps and key area of considerations related to acceptance of VTS System and VTS Equipment.

The radar systems could be tested to evaluate the fulfilment to the specific requirement in the agreed requirement according to Test Plan and Test Procedure.

The radar system should be set up in normal operational mode, and its receiver gain may be adjusted to meet the specified false alarm probability requirements.

The meteorological and hydrological conditions significantly impact radar performance. These testing conditions should resemble the agreed requirements for the specific target type and be recorded during the test.

It might also be desirable to verify the availability and include (part off) an annual cycle in the acceptance process and perform fine tuning underway to ensure good long-term operation.

TEST Targets

Qualitative site acceptance performance test for radar systems could be conducted using any of the following three targets:

Target of opportunity

Controlled target

Controlled and calibrated target

The selection of test target depends on the agreement, test requirement, test method, budget, time span and other resources.

Target of opportunity

The target of opportunity means a random target navigating at the operational area and maritime constructions (buoy, meteorological and hydrological construction and wind farm). Radar systems cannot identify the target of opportunity; however, AIS information, charts and other resources are typically used to identify the target of opportunity during testing.

The selecting target of opportunity for test target enables to:

Use real targets for VTS operation

Use various types of targets

Test for an extensive period

Test under different meteorological and hydrological conditions of opportunity

Controlled target

Controlled target means target which is under control to accomplish specific testing. A controlled target should be selected to reflect the RCS specified in the agreed requirements. It is an almost impossible task to judge the RCS of a target by inspection and quantitative measurements. The determination of selected targets can be carried out using IALA target types, ref. Table 1.

The selecting controlled target for test target enables to:

Use specific targets of interest which is compatible with the requirements

Control target to navigate a specified route and move

Repeat the same test

It is preferable to secure communication between ship to shore to conduct acceptance test smoothly.

Controlled and calibrated target

A controlled and calibrated target means the controlled target with the required ability to reflect electromagnetic waves, such as a target with RCS of 1 m2 to 10 m2 is typically used. The calibration improves the accuracy of the used target RCS.

Lobing

One of the most common pitfalls during verification of radar systems is that the echo intensity from a point target can vary drastically with height. As illustrated in Figure 20, the radar signal will reach the target by travelling along different paths of which the most important ones are the direct path and the path where the radar signal has been reflected by the sea surface. The radar signals travelling along the two paths are added to form a resultant signal, a phenomenon known as multipath reflections. At some target heights, the two waves interference constructively, leading to increased signal levels, whereas the waves interference destructively leading to decreased or even vanishing signal levels at other target heights.

Figure 20 Multi-path reflections (lobing)

As a result, the coverage area exhibits a “lobed” structure where the detection probability alternates between high and low as a function of target height. Hence, if two point-targets of equal RCS are placed at the same distance from the radar, their echoes can differ significantly if their heights happen to be picked such that one is situated in a multipath peak (top left pane) while the other is at a multipath minimum (top right pane). Thus, the appearance of multipath reflections must be accounted for when selecting target heights and positioning the targets.

Multipath reflections are severe at low sea states, elevated heights, and at long ranges. Furthermore, the lobing will depend on atmospheric conditions making range detection tests with point targets, e.g., traditional radar reflectors, very difficult and in practice impossible to reproduce. The practical solution is to use targets distributed in height, such as small vessels with measured RCS.

Test Metodology

Radar Functions and other features

The verification of Operator Functions, Operational outputs and Update rate can normally be made as functional tests, whereas verification of Clutter and noise reduction functions can be part of the performance testing as outlined below.

Core radar performance

Radar Coverage

The continuous coverage of the VTS can be checked utilising of targets of opportunity which aligns to the requirement, as illustrated by Figure 21 giving an example of video capture with “snail tracks” lasting several hours and showing uninterrupted vessel traffic within the coverage of a radar sensor. This may be complemented by use of small, controlled targets determining the maximum range of a target with specific RCS, but also ensuring the ability to continuously monitor small targets.

Figure 21 Example of a video capture with “snail tracks”

Testing Minimum detection range

A test target could be selected from:

- A controlled target, which is compatible with the requirement using the IALA target ref. Table 1 or

A controlled and calibrated target having as small RCS as possible (<5m2) and a low profile.

Test sequence: Observe the radar detection performance on the radar display. The following steps are carried out:

1) Test target should be positioned at least two times beyond the expected minimum detection range

2) Ask the test target to move towards radar site,

3) For each antenna scan, observe whether radar video is detected on the radar screen, and

4) Record the horizontal distance from the boat to the radar station when the radar echo is lost.

5) Repeat the same test several times to find the average value, the minimum detection range.

Testing Maximum detection range

A test target could be selected from:

A controlled target, which is compatible with the requirement using the IALA target ref. Table 1 or

A controlled and calibrated target.

The test trajectory can be in a radial direction along free line of sight and extending from close to the radar to at least 10% beyond the expected detection range. An example of such a trajectory is shown in Figure 22. This includes that the test target can be asked to perform a manoeuvre, like sailing along a large circle, after traversing a certain distance. The manoeuvre shown enables identifying the controlled target in the radar video, and it shows the radar echo as a function of aspect angles.

Test sequence: Observe the radar performance from the radar display and establish tracking of the test target. The following steps are then carried out:

1) Ask the controlled target to move along the predetermined test trajectory including the manoeuvres repeated every 1-2 NM

2) For each antenna scan, observe whether the target is visible and/or tracked in the radar video. Record the result

3) Re-establish tracking if target tracking is lost, e.g. as a result of increased clutter levels, shadowing effects due to large ship, or similar

4) Continue the test until target tracking is permanently lost, and until video plots of the test targets are seen in less than 3 out of 10 antenna scans

5) Repeat the test with the target sailing inbound

Maximum detection range is normally determined by the range the target is seen in e.g., 8 out of 10 scans.

The maximum detection range can be determined by the test target proceeding to the radar station. When the echo of this target is almost disappeared on the radar screen, the horizontal distance from the boat to the radar station should be measured by the radar.

Figure 22 Example of a test trajectory for range detection testing

Test during given Weather situations

Naturally, the weather plays a significant role for target detection and conditions during testing should preferably resemble those described for the specific target type in requirements. However, tests are often determined and planned for a certain period (of the year), implying that the test is forced to be carried out under the actual weather conditions at the given time. In this situation, the test results can be used to benchmark simulation tools which then can be used to predict range detection performance under the specified weather conditions, using the calculation formulas in ANNEX A

Before any testing, ensure that the targets used meets the requirements and that radar is in normal operational mode with the receiver gain is adjusted to meet the specified false alarm probability requirements.

Weather information should be recorded at the beginning and during the test activities.

Target separation

Verification of target separation aims to determine the minimum distance, in range and bearing, where a clear separation between the returns from two small, close targets is obtained.

It is important to use as small targets as possible for the test (in principle point targets having equal RCS). The performance figures stated in this document assume the utilisation of 10 m2 point reflectors with a height of 1-3 meters.

As the distance between the two targets must be changed during testing, at least one of the targets must be moveable. It can be cumbersome to perform the test at sea, as it requires floating, non-reflecting supports for the test targets which must be moved relative to each other on the sea surface. Furthermore, the test should not be performed in higher sea states than 2.

If geographical conditions allow, a much easier and valid approach is to perform the test on land e.g. at a beach or flat paved area, thus providing much better control of the positioning of the reflectors.

Range Separation

          - Test using controlled targets

Two controlled targets specified in the agreed requirement or test procedure could be selected for the test target to test range separation.

Test sequence: Observe the range separation performance on the radar display. The following steps could be carried out:

1) One controlled target is placed to keep the same position. The other controlled target approaches towards the other target at the same bearing and with clear separation. (Speed is preferably less than 10 knots). The distance between the two targets along the range direction should give a clear separation of the targets.

2) Record the minimum distance where the two targets can be seen as two separate targets on the radar display.

3) As an option, the distance can be measured using measurement equipment on-site (e.g., laser range finder).

          - Test using a controlled and calibrated target

Test sequence: Observe the range separation performance on the radar display. The following steps are carried out:

1) The two reflectors are placed in the test area at the same bearing and with clear separation. One reflector is kept fixed throughout the test.

2) Record the minimum distance where the two reflectors can be seen as two separate targets.

The test is passed if the recorded distance is less than the range separation required.

Figure 23 Measurement of range separation

Bearing Separation

          - Test using controlled targets

Two controlled targets specified in the agreed requirement or test procedure could be selected for the test target to test range separation.

Test sequence: Observe the range separation performance on the radar display. The following steps could be carried out:

1) One controlled target is placed to keep the same position. The other controlled target approaches towards the other target at the same radial and with clear separation. (Speed is preferably less than 10 knots). The distance between the two targets along the azimuthal direction should give a clear separation of the targets.

2) Record the minimum distance where the two targets can be seen as two separate targets on the radar display.

3) As an option, the distance can be measured using measurement equipment on-site (e.g., laser range finder).

          - Test using a controlled and calibrated target

Test sequence:

1) The two reflectors are placed in the test area at the same radial distance beyond 1 NM from the radar (to ensure the reflectors are placed in the far field region of the antenna). The distance between the two reflectors along the azimuthal direction should give clear separation of the targets. One reflector is kept fixed throughout the test.

3) Record the angle in bearing where the two reflectors can be seen as two separate targets.

The test is passed if the recorded distance is less than the bearing separation stated in the requirements.

Target Positional Accuracy

Measurement of the target positional error is a combination of the trueness (systematic error) as well as the precision (random error), ref ANNEX A.7.

If possible, the measurement should be done with fixed targets. The optimum is to use the centre/leading edge of physically small structures with known positions.

It is recommended to verify the maximum target position errors utilising at least two reference targets at known positions. It is normally sufficient to measure at one or two points in the distance.

Precise determination of errors can be determined by making a high number of observations and calculate as follows.

Range accuracy

Testing method: Stationary targets should be selected. The distance of the target measured by high-precision locator (Its accuracy is at least one order of magnitude higher than radar’s accuracy) is taken as the true value. Compared that with the distance measured by radar. The range error of stationary and moving targets can be determined separately according to the following formula

（1）The range trueness (systematic error) is:

Ri－radar range observation value.

Rt－range observation value of high precision locator (true value).

n－number of observations, n≥20.

（2）The range precision error (random error) is:

（3）The root mean square error of range is

（4）The Maximum (sigma 3) range error is

Bearing accuracy

Testing method: Stationary targets should be selected. The bearing of the target measured by high-precision locator (Its accuracy is at least one order of magnitude higher than radar’s accuracy) is taken as the true value. Compared that with the bearing measured by radar. The bearing error of stationary and moving targets can be determined separately according to the following formula.

（1）The bearing trueness (systematic error) is:

ai－radar bearing observation value.

at－bearing observation value of high precision locator (true value).

n－number of observations, n≥20.

（2）The bearing precision error (random error) is:

（3）The root mean square error of bearing is

（4）The Maximum (sigma 3) bearing error is

Radar Performance versus the Dynamic Environment

The verification of performance versus the dynamic environment will typically be based on observations of the radar screen by experienced operators.

The magnitude of False echoes due to Multipath or sidelobes can often be measured in the video and it is thereby possible to evaluate if specifications are meet.

Evaluation of Noise and clutter behaviour including Rain suppression and Sea clutter suppression is normally indirect by checking if targets of known size can be detected in clutter as specified.

Dynamic Range

It may only be possible to measure the dynamic range of a radar in a laboratory environment and on-site evaluation will often be subjective using targets of opportunity where with an evaluation of the display of returns from the largest nearby objects and the smallest distant objects.

A qualitative on-site evaluation can be carried out utilizing traffic of opportunity, possibly combined with the verification of continuous coverage, ref B.3.2.1.

Test sequence: Ensue that set up of the radar is correct and observe the radar performance from the radar display. The following steps are then carried out:

- Check that small distant targets are detected as specified.

- Evaluate the appearance of large objects within the area and check for malfunctions. The typical malfunctions in case of insufficient dynamic range includes the loss of small targets near larger ones, time sidelobes due to saturation of receivers and/or longitudinal stretch of large targets due to saturation.

- Repeat same test in different angle and distance.

Target tracking

Refer to IALA G1111-1 for guidance on the subject.

Test Targets and their Callibration

Test targets, also referred to as the controlled target, is assumed to be a small vessel which typical for test of VTS systems.

The RCS of the test targets are needed to obtain quantitative and reproducible results during testing of range performance, and it is necessary to calibrate the RCS of the test targets against a known reference reflector.

A prerequisite for performing RCS calibration is furthermore that the radar receiver characteristics is known, or more specifically that the relationship between the received power and the output video level is known. If not, the characteristics must first be mapped out.

The calibration procedure must be performed in calm weather with a low sea state and a well-known reference reflector must be at hand. An example of a reliable reference reflector is a calibrated Luneberg reflector mounted on a floating support as shown in Figure 24. It is important that the height of the reference reflector resembles that of the test target to be calibrated. If not, the measurements can be influenced by lobing as discussed in section 0 which leads to erroneous RCS estimates for the test targets.

Figure 24 Luneberg reflectors

Note that the RCS of the individual target is dependent on the radar frequency in use.

Calibration sequence:

The controlled target and the reference reflector should be placed within 2 NM from the radar sensor and preferably within 0.5 to 1 NM. Moreover, the targets should be positioned within free line of sight of the radar sensor and away from objects yielding intense radar echoes.

With the reference target on board, the controlled target is positioned in the test area where the following steps are carried out:

1) The reference reflector is placed in the water on its floating support.

2) The controlled target moves away from the reflector until the two targets are clearly separated on the radar display.

3) The radar gain is adjusted to obtain a clear picture of the test area with the test and reference targets clearly visible and stable in the water, both in azimuth and range. Make sure that any automatic gain adjustment is disabled such that the gain will remain constant throughout the calibration procedure.

4) Record the intensity of the reference target echo during at least 30 consecutive scans. Convert all recorded intensities to received power levels using the receiver characteristics. Calculate the average value and variance of the received echo power.

5) The controlled target is then positioned with its stern pointing the direction of the radar, see Figure 25.

6) Record the intensity of the controlled target echo during at least 30 consecutive scans. Convert all recorded intensities to received power levels using the receiver characteristics. Convert the received power, to an RCS value. Calculate the average value and variance of the measured RCS.

7) The controlled target changes its orientation in steps of 45 degrees and after each orientation change, the measurement in 6) is repeated. The last measurement is performed with the controlled target seen from astern.

8) Let the controlled target move in a circle cantered at the reference target and with a radius of approximately 100 meters. In each scan, record the intensity of the controlled target echo. Convert all recorded intensities to received power levels using the receiver characteristics and further to RCS values. Calculate the average value and variance of the measured RCS.

The RCS of the target has now been measured for several different orientations (step 6 and 7), and in addition a value for the overall RCS (the RCS averaged over all orientations) has been obtained in step 8.

Figure 25 Target aspect angle relative to the radar antenna

------------------ END ---------------------