IALA Guideline

GXXX

Human Factors and Ergonomics in VTS

Edition 0.0

Date (of approval by Council)

Revisions to this IALA document are to be noted in the table prior to the issue of a revised document.

| Date | Details | Approval |
| --- | --- | --- |

1.	Introduction	5

1.1.	Background	5

1.2.	Aims and objectives	5

2.	VTS as socio-technical system	6

2.1.	Overview	6

2.2.	Characteristics	8

3.	Human factors and ergonomics	9

3.1.	Human – Technology – Organization (HTO)	9

3.2.	Situation awareness	10

4.	Human factors and ergonomics in VTS operations	11

4.1.	Human aspects	12

4.1.1.	Sensory inputs	12

4.1.2.	Muscle response	12

4.1.3.	Memory	12

4.1.4.	Cognition factors	13

4.2.	Technological aspects	14

4.3.	Organisational aspects	14

4.4.	Environmental aspects	15

5.	Recommendations to organise Human factors activities in VTS	16

5.1.	Making Human factors part of VTS activities	16

5.1.1.	Human-centric design process	16

5.1.2.	Training human factors	17

5.1.3.	Quality assurance	18

5.2.	VTS setup recommendations	18

5.2.1.	Working environment	18

5.2.2.	Workspaces, including anthropometric aspects	18

6.	References	20

ANNEX A	Sample checklist of workspace ergonomics factors	21

ANNEX B	Risky situations related to human factors	23

ANNEX C	Quality and performance metrics	27

List of Tables

Table 1	Characteristics of socio-technical system applied to VTS	8

Table 2	Human factors in VTS	11

Table 3	Cognition factors	13

Table 4	Technological aspects	14

Table 5	Organisational aspects	14

List of Figures

Figure 1	VTS as socio-technical system	6

Figure 2	VTS Operator in northern European VTS centre	7

Figure 3	Information sources and communication means for single VTS Operator	7

Figure 4	Potential benefits of HTO-approach	9

Figure 5	Steps of human-centric design process	16

# Introduction

## Background

IALA conducted a workshop on Human Factors and Ergonomics in VTS at Chalmers University of Technology in Gothenburg, Sweden (12 - 16 October 2015), in conjunction with Chalmers University of Technology, the Dutch VTS Operator Training Foundation and the Port of London Authority.  A conclusion of that workshop highlighted the need for guidance on raising awareness about safety culture and human factors, including incorporating the work into existing and developing IALA documentation.

The IALA VTS Committee has been tasked with developing guidance on human factors and ergonomics in VTS.  The objective of this task is to provide awareness regarding the role of the human factor in the performance of a VTS to competent authorities and VTSOs. In addition, this guidance would support the implementation of human factors in the training cycle of initial training, sector training, recurrent training, updating training and adaptation training.

This document builds upon previous work to further develop VTS guidance on human factors and ergonomics. Within the scope of this work the VTS is identified and understood as a complex socio-technical system where operators, organization and technology and the interactions between them play a crucial role for quality, safety and efficiency of VTS services. Human factors and ergonomics here is understood as a framework that can help to categorise information about work and work settings, how to collect information about a work setting, what to do to make it better, and how to explain and measure the possible benefits to the change.

## Aims and objectives

This Guideline provides awareness regarding the role of the human factor in the performance of a VTS. It is intended to present a source of information to assist Competent and VTS Authorities in the preparation and implementation of systems to support improved operational performance. It is not intended to provide specific information on the safety culture or portrayal of data.

Furthermore, this guideline provides guidance in implementing human factors in the training cycle of initial training, sector training, recurrent training, updating training and adaptation training.

# VTS as socio-technical system

## Overview

Traditionally, technological advancements have been a driver for research and development activities in the VTS domain. Thus, many activities have foremost focused on the development of technical equipment and information sources within the domain. However, in the past 15 years an increasing body of work has taken a human factors approach rooted in a socio-technical systems perspective to increase the understanding for interactions across operators, technology, and organization. In recent years, an increased focus on future VTS operation and the potential of human-autonomy interactions within shore operation centers has developed[1].

VTS as socio-technical system

VTS can be considered as a socio-technical system (Figure 1) where operators provide services to a larger variety of customers. While staffing and local organization might differ, VTS is very often, but not always, part of a port system including VTS, pilot, tugs, ship/s and crew/s. Studies [2][3] have shown that all the actors in this system need the same information but perhaps at different levels of granularity. Thus, coordination and cooperation among the different actors within the port system becomes a key for the service’s quality, and the overall safety and efficiency of traffic movements within the area.

Figure 2 draws attention to the complexity of the work environment for VTS operators. The operator provides information services in an area that is located on the port entrance of one of northern Europe’s largest ports. Within the center the operator is collocated with operators for the pilotage ordering service and the port. Thus, information for service provision can be gained both through the decision support system, VHF radio and from other services.

VTS Operator in northern European VTS centre

For the service provision to the vessels, the information from all the different sources available (Figure 3) are merged by the operator thus drawing attention to both physical, such as proximity to other services or workstation layout, and cognitive aspects, e.g., decision making, interpretation of current traffic picture in the VTS area and reactions with multiple communication means. All it forms the daily routine of VTS.

Information sources and communication means for single VTS Operator

Therefore, the operator’s expertise and experience based on technical and non-technical skills become crucial for the service provision [4][5]. In addition to the physical and cognitive aspects of the work, the organizational framework of the VTS, including training, service and staffing levels, operational procedures and integration with other services, also shape the precondition of work [6].

## Characteristics

Table 1 (adopted from [7]) presents an overview of the characteristics of VTS as a socio-technical system. As there are many different organizational, human, and technical aspects to the VTS operations, human factors and ergonomics become an important facilitator to design a system that can balance quality, safety, efficiency, and economic aspects of operations.

Characteristics of socio-technical system applied to VTS

| Characteristic | VTS |
| --- | --- |
| Interactions of various layers | - External interactions with vessels, allied services, general public - Interactions with multiple equipment, including operational procedures and maintenance/service |
| Large problem domain | Complexity of everyday work based on: - subject domain knowledge (regulations, best practices, etc.) - uncountable number of variables (ship-related, environmental, geographical, technical etc.). |
| Dependence on communication and coordination | Multiple means of communication: - VHF as main means of coordination through communication between vessels and shore - Phone, email, information systems of allied services |
| Distributed | - VTS personnel operating in co-located/different centres, communicating to each other - Dependency on remote sources of information |
| Dynamic | Multiple dynamic elements in VTS affecting: - Ships moving independently - Changing shifts of VTS personnel - Dynamic environment (weather, hydrographic situation, navigational constraints — for instance, military exercises) - Technical constraints (evolving and changing systems, equipment failures and maintenance, etc.) |
| Couplings | Complex net of technical, human, and organisational functions |
| Automation | - Decision support system - Technical innovation - New developments affecting VTS (i.e., MASS) |

# Human factors and ergonomics

Human factors and ergonomics (HF/E) describe the scientific discipline and domain of practice concerned with the understanding of interactions among humans and other elements of a system. Human factors and ergonomics as terms are used interchangeably along with user experience (UX). Human factors are often referred to in American settings, while the ergonomics is more commonly used within Europe. In this document we will use the term Human Factors.

Human factors apply theory, principles, data, and methods to design in order to optimize human well-being and overall system performance.

Human factors is a systems approach to understand human work in complex environments. As approach, HF/E is holistic and concerns the design and evaluation of tasks, jobs, products, environments, and systems considering physical, cognitive, and organizational factors.

Work and work settings are analysed taking human operators, systems, and work environment into concern.

Based on approach of International Ergonomics Association (IEA) [8], three main areas of human factors include:

Cognitive factors: include functions related to perception, memory, reasoning and motor responses, human-computer interaction, communication, and teamwork.

Physical factors: relate to human anatomy, physiology, anthropometrics, and biomechanics.

Organizational factors: relate to operating procedures, shift schedules, administration, regulations.

Each of these factors have overlapping areas which need to be considered in interactions within the operational environment.

## Human – Technology – Organization (HTO)

The human-technology-organization is an analytical framework within the human factors domain. The approach is rooted within the nuclear power industry. It was in the late 1990s as a response to the increasing need to create a deeper understanding for the complex interplay between human operators, technology, and organization in safety critical operations [9]. The HTO perspective emphasizes the importance of understanding the interactions and interdependencies between a sociotechnical system’s human, technical and organizational parts. Since the late 1990s this perspective has gained an increased acknowledgement and is now applied across many different industries.

Potential benefits of HTO-approach

Within this document, we suggest adopting a HTO perspective with specific focus on system performance can be understood and support operator well-being. The benefits of applying an HTO-perspective to system design and analysis are exemplified in Figure 4 (adopted from [10]). The figure shows potential improvements in three categories: Human, Technology and Organization.

Adopting an HTO to the VTS, may help to structure discussions on how (work) environment, technology, operator, and organization are coupled to each other and emphasize the need to acknowledge the interactions and interdependencies among these to shape the precondition for safe and efficient service provision in the VTS domain.

## Situation awareness

In context of VTS systems additional major systematic characteristic requires particular attention — situation awareness. It is the perception of environmental elements and events with respect to time or space, the comprehension of their meaning, and the projection of their future status [11].

Supporting proper level of situation awareness across all stakeholders (including VTS staff, vessels crews, allied services personnel, etc.) is crucial for safety of seafaring, and the role of human factors in it is extremely important, because it relies on cognitive and physical particularities, working environment affordance, practical implementation of information delivery, etc.

# Human factors and ergonomics in VTS operations

As aforementioned, VTS can be considered as a complex socio-technical system. There are multiple aspects, which may affect operator work, system performance and operator well-being. The following table presents key areas to address within the design, evaluation and assessment of VTS systems based on the HTO framework — and they are covered with more details later in this section.

To acknowledge the difference between VTS systems world-wide, a fourth category, Environment, has been added to the list below as service provision, service level and demands may vary. These factors are taken into consideration noting the VTS Operational area, including the area specific aspects of the fairway, traffic, meteorological and hydrographic elements.

Human factors in VTS

| Category | Key Areas to Address |
| --- | --- |
| Human | Performance and shaping factors (workload, stress, fatigue) Attitudes and commitment Technical skills (expertise/operational experience, e.g., conflict detection and management, service provision) Non-technical skills (communication, decision-making, situation awareness/sensemaking, leadership, teamwork, task management, problem solving, creativity) |
| Technology | Human-centric design User needs Information presentation/representation Level of automation and human-automation interaction Information structure and integration of sensors within the VTS system Workplace design Workstation design Physical workstation arrangements/spatial design |
| Organization | Preconditions for work (work schedule, staffing, selection criteria) Resilience/High reliability Procedures – SOPs etc. Training provision Training needs analysis Assessment and evaluation techniques On the job training Safety and quality management Appraisal procedures Reporting system (e.g. incidents/accidents) and follow-up procedures including organizational learning Working conditions (light, noise, temperature) |
| Environment | Operational environment/service integration within port system Particularities of geographic error Weather and other environmental factors |

## Human aspects

Humans and machines differ by nature, they have their own particularities that affect the overall organisation, and purely technical approach is limited. To improve efficiency of socio-technical VTS system, understanding of particularities of both humans and machines is needed.

### Sensory inputs

Humans get the information from external world by sensory inputs. The most critical are vision and hearing because most of the information is received by them, while other senses, like touch and smell are also important, but are used less consciously (for instance, touch response helps to quickly identify hardware controls without direct observation, while smell allows to identify hardware failures because of short circuits or before to respond to fire before it is too late). Every sensory input has its own particularities, possibilities, and limitations.

It is important to consider the relative bandwidth — the limitation of the information, that can be perceived in time, but compared to the machines, it differs depending on the circumstances and human state (e.g., fatigue). If there is more information provided, than current bandwidth allows, the human gets overloaded, and critical signals may be ignored.

### Muscle response

Humans interacts with external world with their muscles, including voice. For VTS these are mostly manipulations with hardware (dedicated buttons, keyboards, mouse and touchscreens) and changing the position to get better access to specific input and output devices.

Muscle motions are separated into two major types: gross motor skilled actions, that require less efforts, and more precise fine motor actions, that require more attention. For example, swipe gesture on touch screens is gross motor actions, while pointing to specific area on the display with mouse is fine action. Properly designed workflows and layouts considering these types of actions allow to increase the speed of interaction and avoid fast fatigue.

### Memory

Human memory both with cognition is used for decision making. Three types of memory are distinguished:

Short term — very fast compared to other types, but has low volume and duration, that affects the amount of perceived information we can process at a moment. The information in it is easily replaced with new inputs, and typically we don’t recognize that something is lost

Mid term — this memory has more volume, but it is harder to access. Can be subject of multiple distortions and biases. It helps to keep the overall context of the situation

Long term — the knowledge that we have. Can be fast but requires regular updates to be used effectively. Also, it Is easier to put the information with strong emotional connection into the long term memory, what is critical for trainings — it is naïve to expect, that a single formal training will be enough for further effective work process.

### Cognition factors

The table below briefly highlights basic cognition shaping factors, that critically affect human performance and their ignorance typically result in drop of efficiency.

Cognition factors

| Factor | Description |
| --- | --- |
| Single focus of attention | Humans are not able to perform several tasks simultaneously, it is possible only to switch with relatively slow speed between tasks, depending on their complexity. Also, under stress, the focus on specific (recognised as critical) task can lead to “tunnel perception” — any other inputs can be ignored, even if they are distinct, strong and lead to dangerous situations. |
| Learning curve and automated actions by reflexes | Some time is needed to start using the system in the most effective way — conscious actions are transformed into subconscious, automatic actions, that are based on the mechanics of conditional reflexes. It can significantly increase the speed, but reflexes can lead to their own issues, because they result in loss of conscious awareness. |
| Fatigue | Long activities, especially monotonous ones, decrease the efficiency of the humans because of fatigue, that affect all human capabilities: perception, cognition and decision making, response etc. Multiple incidents across all industries are caused by fatigue and improper fatigue management at organizational level. |
| Stress | Stress is a situation of internal resources mobilization under the threat of dangerous consequences. In short term, it can significantly raise the efficiency of operator, but right after it the efficiency decreases significantly because of fast fatigue. As it was mentioned, stress can also cause tunnel perception. |
| Errors | Humans are prone to errors. The following types of errors are differentiated [14]: Slips of actions (“not doing what is expected”) Lapses of memory (“forgetting to do something midway through the task”) Rule-based and knowledge-based mistakes (“decision making failures”) Biases Biases relate to the “mistakes” type of the error, and they are based on multiple perception and cognition particularities, that are result of evolution. Basically, biases are based on “survival best-practices”, but they tend to affect the performance in the irrelevant situations, that are not recognized as such. Informing during trainings about cognitive biases as well as other types of errors is a critical step for their prevention, but their minimization requires systems approach, including proper organization and design of the system etc. |

## Technological aspects

Some aforementioned technological aspects are covered in more details in the table below:

Technological aspects

| Aspect | Description |
| --- | --- |
| Automated systems | Automated systems are used to decrease workload on humans, but they are also source of subsequent side effects, that require higher technical level of expertise, dependency on the uncontrolled (hidden) technical factors, etc. More sophisticated systems require more maintenance, trainings, and if they are designed in non human-centric way, it creates potential of human errors and major incidents by their own reasons. |
| Ergonomics and safety of workplace | Ergonomics is the study of people’s efficiency in the working environments — it covers the factors, related to organisation of workspaces, including working room conditions and arrangement, workplaces layout, safety and comfort to provide the environment for optimum performance. |

## Organisational aspects

Organisational aspects

| Aspect | Description |
| --- | --- |
| Subject domain | Subject domain defines top-level formal aspects of VTS activities: Formal VTS objectives and role Regulations and procedures defined by top-level authorities Language that is used (including standard terms, phrases, professional slang, multiple languages usage and probable language barriers etc.) |
| Organisation | Standard operating procedures (SOPs) Resource management procedures — similar in some extent to Cockpit Resource Management (CRM) in aviation and Bridge Resource Management (BRM) in shipping companies that support effective usage of technical and human resources during 24/7 routine and emergency activities Approach to initial and regular training procedures, how formal or goal-oriented they are |
| Culture | Traditions, that exist in every organisation and affect formal and informal relations between VTS staff, especially between operators and their supervisors, less experienced and more experienced colleagues. One particular aspect is openness to discussions — culture of best practices exchange and error tolerance in organisation. In some organisational cultures less experienced colleagues that recognise human errors made by more experienced tend to consider that this is their false misinterpretation and don’t raise their concerns, taking passive role in evolving situations. |
| Financial | Even with non-commercial nature of VTS systems, availability and limitations of financial resources affect the overall effectiveness and safety of VTS organisation in following ways: Amount of available personnel and, as consequence, their workload, expertise and motivation Availability and coverage by technical equipment, including regular upgrades Availability of comfortable working conditions |

## Environmental aspects

Environmental aspects cover wide range of contexts related to particular area of specific VTS:

Geographic extent and particularities of area

Weather and other environmental conditions, including forecasts

Military and Coast Guard divisions, MRCCs, port infrastructure and other organisations operating in the same area that require interaction between operators

# Recommendations to organise Human factors activities in VTS

This section of document contains practical recommendations to support human factors and corresponding activities in VTS centres.

## Making Human factors part of VTS activities

This section provides recommendations how to embed human factors in VTS organisation and activities.

### Human-centric design process

All decisions related to organisation and technology used by particular VTS must consider human factors. It is based on human-centric design and implementation process, that is basically irrelevant to application — whether these are the user interfaces of VTS information systems, working area spatial design or development of standard operational procedures.

The figure 5 illustrates human-centric design (HCD) process (adopted from MSC Circ. 1512):

Steps of human-centric design process

The process is iterative, and is divided into the following steps:

Analysis
During this step all the requirements to the product (as final outcome of process) are gathered from stakeholders and other sources, analysed and defined, including high-level goals, business and functional requirements, user roles and needs.

Concept development
The conceptual decisions are made how to organise the product design, what are its principles, primary workflows and scenarios, already considering human particularities, limitations and needs. Detailed aspects are not covered, to get the bird’s-eye view of the final product and check the vision with stakeholders.

Planning
Based on conceptual design, the real development activities are organised and planned, including dedication of human and financial resources, risks assessment, definition of the schedule.

Detailed design
At this phase all necessary details are defined and developed.

Integration and testing
All the decisions made during the previous phases are to be verified internally for human factors and validated with real users and operators. This guarantees that they are not isolated or irrelevant to particular aspects of human factors and ergonomics, and that sufficient feedback is gathered and considered.

Operation
Only after previous steps starts the real operation/usage of the resulting product. The feedback is gathered regularly, and based on it, next iteration of product development/improvement can be started.

This process illustrates, that human factors activities are not just a single “did-and-forgot-it” sequence, it is recommended to perform them consistently during the whole lifecycle of the system, especially when there are major changes in design, regulations, or after incidents occurred.

### Training human factors

Trainings play significant role to emphasize human factors and their role. With proper training activities, all the VTS personnel becomes aware of the role of human factors in safety and efficiency of daily routine and prevention of accidents.

It is recommended to consider the following training activities:

Initial trainings for new VTS operators, that cover human factors in general, their role in accidents, best practices to prevent them, review of critical elements and risky situations based on the historical incidents and accidents.

Regular trainings to keep the awareness of human factors at the appropriate level, that briefly cover topics from initial training.

Ad hoc trainings that pay particular attention to the topics that are identified as necessary to be improved based on previously held analysis of recent VTS activities and/or new incidents occurred both in the particular VTS and outside it. It requires to have information channels organized to get fresh and important reviews about human factors related topics, including incidents — for instance, review of official investigation reports.

These trainings may include the following recommended topics:

- General explanation of human factors role in VTS

- Components of human factors (the ones, that are mentioned in previous sections of this Guideline)

- Typical human errors to be aware of:

  - Errors in perception

  - Errors in cognition

- Typical risky situations (see ANNEX B) where HF play significant role and affect the safety and performance.

The trainings materials and procedures should not be limited to formal approach, providing practical guidance, examples (like reviews of past situations and accidents occurred) and best practices.

### Quality assurance

Human factors are to be included in quality assurance procedures of VTS centre — with special role of human factors specialist dedicated among VTS personnel, who will supervise the state of human factors, considering all the aforementioned aspects, including operational workplace conditions, trainings, communication with manufactures related to equipment ergonomics and usability, etc.

To track the state of human factors in VTSO, the specialist can regularly use several quality and performance metrics, that are provided in ANNEX C.

Special procedures and authority to affect human factors in VTS organisation should be defined for the role of HF specialist, not limiting it with formal and passive role.

## VTS setup recommendations

In this section some practical recommendations about VTS setup are provided. Based on them ANNEX A contains general checklist of workplace ergonomics, providing additional details.

### Working environment

Important part of human factors activities must be related to organise comfortable environmental conditions, considering lightness, noise, temperature and quality of air:

Avoiding direct sunlight, and glares from artificial light on shared wall-mounted and workstation displays.

Minimisation of ambient and equipment sound noise, usage of sound absorption materials, panels and carpets to minimize reverberations.

The organization of workplaces and its partitioning to avoid audio distractions from neighbor operators

Minimisation of visual noise (calm interior colours with low contrast, minimalist furniture, proper movement ways organisation to avoid peripheral vision distractions by someone walking from one place to another, etc.)

Optimal temperature range, proper ventilation and air conditioning

Tracking rates of noise, lightness and temperature, air quality (CO2)

### Workspaces, including anthropometric aspects

The layout and spatial design of workplaces is another point of HF activities, to provide comfortable working area:

Proper methods to organise the workstation layouts depending on the priorities of the tools and information provided

Recommended set up (static chart displays for situational awareness, dynamic screens for contextual tasks)

Selection of hardware (displays, input devices) to fit workstation ergonomic criteria.

Features to support work both when the operator seats in front of the workstation and when he/she stands — it helps to decrease the fatigue and keep the concentration.

Selection of VTS systems with usable user interfaces and hardware controls and that support frequent operations, changes in day/night lightness conditions, etc. The identification of primary controls should be available without direct observation. Groups of controls to differ by form and material texture to avoid confusion

#### Support of situation awareness

Shared access displays (videowalls) are used to support situation and operational context sharing among operators and supervisors.

Critical alerts are to be duplicated by sound signals because they could be ignored when the operator is disturbed and is not observing the displays. The signals should be clear and definitive, not too loud to avoid irritations, but differ from ambient noise.

The practice of duplication of alerts by using dedicated wall mounted signal panels can be considered, to cross-check that the situation is not ignored by multiple operators or head of the watch.

# References

Relling, T., Praetorius, G., Hareide, O.S. (2019). A socio-technical perspective on the future Vessel Traffic Services. Necesse. 4. 112-129

Mansson, J., Lutzhoft, M., & Brooks, B. (2017). Joint Activity in the Maritime Traffic System: Perceptions of Ship Masters, Maritime Pilots, Tug Masters, and Vessel Traffic Service Operators. Journal of Navigation, 70(3), 547-560. doi:10.1017/S0373463316000758

van Westrenen, F., Praetorius, G. (2014). Maritime traffic management: a need for central coordination? Cognition, Technology & Work. 16. 59-70.

Costa, N.A., Lundh, M. & MacKinnon, S.N. Non-technical communication factors at the Vessel Traffic Services. Cogn Tech Work 20, 63–72 (2018). https://doi.org/10.1007/s10111-017-0448-9

Brodje, A., Lundh, M., Jenvald, J. et al. Exploring non-technical miscommunication in vessel traffic service operation. Cogn Tech Work 15, 347–357 (2013). https://doi.org/10.1007/s10111-012-0236-5

Praetorius, G., Hollnagel, E., Dahlman, J. (2015). Modelling Vessel Traffic Service to understand resilience in everyday operations. Reliability Engineering & System Safety. 141. 10-21.

Vicente, K.J. (1999). Cognitive Work Analysis: Toward Safe, Productive, and Healthy Computer-Based Work

International Ergonomics Association () What is ergonomics? https://iea.cc/what-is-ergonomics/; retrieved 2020-08-20

Karltun, A., Karltun, J., Berglund, M., & Eklund, J. (2017). HTO – A complementary ergonomics approach. Applied Ergonomics, 59, 182–190. https://doi.org/10.1016/j.apergo.2016.08.024

Anastassova M. (2018), Evaluation of job satisfaction: a psychological approach, http://ace-factories.eu/wp-content/uploads/MANUWORK_2018_Job_satisfaction.pdf, 3.

Endsley M. (2003) Design for situation awareness: An Approach to User-Centered Design.

Stanton N., et al. (2005) Handbook of Human Factors and Ergonomics Methods.

Wise J. A., Hopkin V. D., Garland D. J. (2009), The Handbook of human factors in aviation, s.16-1.

Reason J. (1990), Human error.

Sample checklist of workspace ergonomics factors

This appendix provides the checklist to be used for organisation of personal workplaces for VTS operators:

Workspace position

- Workspace position is located in such way, that no direct sun light or bright light form artificial sources interferes observation of displays and working surfaces by operator.

- The workstation is accessed easily by the operator

- It is possible to have another assisting person or trainee to stand or sit close together and observe the displays of the workstation (during shift, critical/emergency situations, trainings)

- The workstations of operators allow simultaneous voice communication without interference

- Shared working area (including videowalls) is observable from different workspaces

- Local air quality and temperature conditions (including heating, air conditioning, ventilation equipment selection and location) are considered to provide balanced working environment that will maximize comfort and avoid probability of “battles for conditioning/thermostat”.

Workstations & displays

- Workstations are located that they don’t disturb operator (for instance, their location don’t result in less space for legs under table desk).

- Noise pollution from workstations is minimized.

- Workstations are regularly upgraded.

- All the displays are observed from the seat without distortions.

- The displays are ordered by priority/importance — most critical in central vision field, less important in peripheral area.

- The size of the displays corresponds to the information presented; information is readable by operators.

- The brightness of all the displays is the same.

- It is possible to adjust the brightness of all the displays by single control

Office chair

- It is possible to easily move the chair in necessary position or to move it away

- The chair height, seat and back can be adjusted

- Feet are fully supported by the floor or stand

- The chair supports lower back

- No pressure is felt by back of the knees

- Armrests allow to sit close the workstation surface

- Shoulders are relaxed

Work surface (desk)

- When working, the upper arms are relaxed

- The angle between forearms and upper arms is 90° or greater

- The keyboard, pointing and other control devices are accessed from the seat without its movement and are positioned at approximately the elbow level

- During typing, the wrists are in line with forearms and not bent

- There is a space to keep arms relaxed on the work surface

Risky situations related to human factors

This annex is intended to highlight the role of human factors in VTS in potentially risky situations and examples of incidents occurred during them with references to official investigation reports. Some examples are not related to VTS, but still added, because although the situations are irrelevant to subject domain, they are related to human factors and are important for illustration — it is better not to wait for incidents to occur but be well prepared.

It is important to emphasize, that accidents never happen because of single cause, and the provided examples can be easily interrelated in a single situation. Human factors require holistic, systems approach.

False assumptions

Proper situational awareness means adequate understanding of current situation status and synchronisation between all the stakeholders participating in it. Multiple accident reports mention insufficient communication between VTS and vessel personnel, that result in loss of situation awareness (like being aware of ongoing traffic, anchored ship drifting, etc.)

In many cases the reason is in fault assumptions, that another side of activity is aware of current and upcoming events and reacts accordingly — for instance VTS operator assumes, that crews of the vessels are making procedures correctly and are aware of each other. It results in passive observation of evolving dangerous situation and when it is clear, that previous assumptions are wrong, it is too late to respond.

Recommendation:

- During trainings, pay specific attention to the need that all assumptions must be cross-checked with available means and by other personnel.

VTS-related accidents:

- Helga Ingstad frigate collision and grounding (November 8, 2018)

- Collision of Donald Redford with the pier (November 1, 2003) — fault assumption that slurred speech of junior captain answers via VHF is not related to alcohol intoxication.

Vague communication style

Using informal, spoken-language communication between VTS and vessels may result in distortion of transferred messages and misunderstanding between stakeholders, ignoring direct orders, or shadowing the reasons/intentions behind the perceived messages.

Recommendation:

- Clear, unambiguous phrases that do not allow more than a single interpretation, using, especially for communication with foreign crews, Standard marine communication phrases.

- During trainings emphasize the role of repeating orders and “closing communication loops” methods, voice actively expectations, concerns and assumptions.

VTS-related accidents:

- MV Full City grounding at Sastein, Norway (July 31, 2009) — miscommunication resulted in different expectations of VTS operator and vessel crew and subsequent loss of situation awareness.

- Maersk Kendal grounding (September 16, 2009). One of the reasons was misinterpretation by Maersk Kendal master and chief officer of information about the vessels that were referenced by VTS operator.

Other accidents:

- Costa Concordia grounding (January 13, 2012), particularly the situation of communication between the ship’s master and helmsman on the bridge, when the order to turn starboard was perceived as to turn port by the helmsman, and it was not identified.

- Collision of two Boeing 747 airplanes on the runway of Los Rodeos Airport, Tenerife Island (March 27, 1977). One of the reasons was mutual misunderstanding between crew of one of the airplanes and ATC operator.

Overload and fatigue of VTS operator

If there is an insufficient amount of operators handling the dynamic traffic situation, or an inadequate organisation of working schedule, the operators can or get quickly tired or overloaded, resulting in loss of awareness and communication effectiveness, ignoring otherwise obvious signs of evolving dangerous situation.

Recommendation:

- Procedures assigned to proactively change the working schedule if the amount of operators available is affected by sickness, personal problems, etc., or switch operators on the watch if there are signs of fatigue, loss of concentration, etc.

VTS related accidents:

- Collision of Donald Redford with the pier (November 1, 2003) — personal problems and fatigue of VTS operator resulted in ignorance of behavior of the ship and slurred speech of its master over VHF.

Other accidents:

- Collision of DHL and Bashkirian Airlines airplanes, (July 1, 2002) — ATC operator was put alone for night shift in dense air traffic environment, also with reduced technical equipment support, resulting in fatigue and loss of situation awareness because of insufficient cognitive resources.

Workflow interruptions and watch handover

Watch handover is a critical procedure, because the situation that is clear and obvious to the VTS operator leaving the watch can be not so obvious to the upcoming operator. Especially it is critical if the dangerous situation occurs exactly during watch change.

Recommendation:

- The watch change procedure should be organised including active communication of current situation from one operator to another, but not causing the loss of awareness/responsibility during it.

- Technical VTS systems should be designed and set up to support single actions to return to previous states and/or activities that had been interrupted.

“It is not related to me”, the role of personal relationship in staff

Sometimes dangerous situation starts to evolve when it is behind the operator responsibility area. It results in the ignorance of signs of danger, or the operator, who noticed the danger, is not wishing to address his considerations to another operator who is responsible for the area, relying too much on formalities and fearing to affect the personal relationship with other members of VTS.

Recommendations

- Organizational culture should support active assistance and open-mindedness. If another operator observes that the operator (even with higher rank) ignores some “obvious” information, he or she proactively announces his concerns without risks of being ashamed.

VTS related accidents

- MV Full City grounding at Sastein, Norway (July 31, 2009) — among other reasons, the ship was anchored in the area, that was behind of VTS area, so the operator didn’t consider it’s duty to provide instructions about anchorage, while the crew expected to receive them.

- Helga Ingstad Frigate collision and grounding (November 8, 2018) — VTS operators considered that the situation is under control of vessels’ crews.

Tunnel perception under stress conditions

When some complex task or dangerous situation makes an operator concentrated on it, it is easy under stress to ignore simple activities to resolve it, or ignore another evolving dangerous activity.

Recommendations:

- During operator trainings, attention to limitations and consequences of stress conditions must be paid.

- Resource management activities need to be organized, similar to BRM (Bridge resource management) on vessel’s bridges or CRM (Cockpit resource management) in aircraft’s cockpits, especially considering that no single operator should participate in problem resolving without supervisor’s assistance. For example, if one operator is concentrated on the evolving problem, another operator keeps situation awareness for all remaining activities in area of responsibility of first operator.

Other accidents

- Eastern Air Lines Flight 401 (December 29, 1972) — the problem of burned-out landing gear bulb was being solved by the whole aircraft crew, resulting in loss of situation awareness and crash.

- Air France Flight 447 (June 1, 2009) — stress conditions because of insufficient training resulted that no standard actions to put the airplane from stall were performed.

Degradation of technical equipment

During lifecycle of VTS there is always a probability of losing functional capabilities of VTS technical systems — by incident or intentionally (e.g., during maintenance and upgrade procedures), and the overall support of situation awareness may be degraded significantly.

While cases of degradation that happened by incidents are typically covered by alert management systems, in some cases it can be better tracked by VTS staff, compared to planned degradation, which can be poorly organised and lead to issues in awareness about existing degradation.

Even if there are no issues in with notification, limited functionality increases workload of operators, leading to stress and fatigue.

Recommendations:

- Define specific procedures to inform VTS personnel and other stakeholders about planned and current maintenance procedures and resulting degradation of system’s function.

- In case of degradation, it is recommended to pay specific attention to fatigue management, because degradation typically causes excessive workload of operators.

- VTS systems should support explicit notification about new functional limitations, including those that were actively triggered by service and technical staff.

Accidents:

- Crash of V-22 Osprey (October 1, 2014) — the aircraft was left in maintenance mode with limited engines power, and the pilots didn’t notice it before the take-off, because of missing indications and flight manual SOPs covering this aspect.

Quality and performance metrics

For quantitative measurement of the quality of VTS organization and work process with respect to human factors, several quality and performance related metrics can be used.

While it is hard or even impossible to measure quality in absolute values, the comparison of numeric results provides the opportunity to evaluate relative changes in human factors in time, trigger proactive research of reasons of changes, affect them and measure their effectiveness.

This section provides some examples of these metrics.

Errors

These metrics assist to estimate the error rate during routine and critical operations of VTS:

- Errors made by operators per period in general

- Errors for specific types of operational scenarios

- Errors by severity

- Errors by cause (unproper training, stress and fatigue, poor usability, etc.)

The metrics are used to identify the crucial parts of the system and factors to be reviewed. Also they can help to estimate the general progress of human factors approach.

Cognitive load

Cognitive load metrics help to identify the issues with excessive load of the operator during daily procedures, that can cause stress, fatigue with ongoing efficiency decrease and increasing amount of human errors.

Interpretations

This aspect is used to determine the amount of cognitive (internal) efforts to understand the information provided by VTS technical sources. For example, if timestamps of alerts are presented to the operator, he has to interpret by his own how old are they compared to current time. Or if targets speeds and courses are provided, how high the risk of collision is.

By measuring the amount of data occurrences and their probable interpretations, what time it takes to interpret, and which additional information sources (like current time in example above) are need to close the “perception -> interpretation -> decision making -> action” loop, we can estimate the overall cognitive efforts and identify areas for optimisation of interaction between stakeholders, including VTS operators, vessel personnel, and VTS systems.

There is always a risk of too narrow predefined interpretations.

Clutter (signal/noise ratio)

There is a need to find the proper balance between the amount of information provided and its relevance to current situational context and tasks. Excessive and useless information can distract operator from more critical data, and lead to the ignorance of relevant messages and loss of situation awareness (known as “Boy who cried the wolf” problem). But too filtered information can also lead to the same result.

Example: “Loss of target” alert, that triggers every time the radar loses the acquired target because of waves. But we can result with cases, when the loss of the vessel was not identified.

During test sessions, the events of information updates (both visual and audible, including VHF contacts, visual indications, sound alerts, VTS notifications, etc.) and responses to them can be measured, grouped by type, source, severity, modality (visual, audio) etc., and the relevance and usefulness can be estimated., allowing to count the signal/noise ratio.

Usage speed

One of the simplest and most important metrics, that helps to measure how many actions and for how long it takes to perform certain operational scenarios, from the beginning to the end, including time for perception, actions to reach necessary information at the workstation, interpretation and decision making.

In general, the less is the best, but It is also important to estimate the risks of awareness loss and incorrect interpretations during every step.

The most basic methods to measure usage speed are Critical path analysis and GOMS [12].

Based on the results, technical methods to minimize, for instance, time to access information or trigger commands can be considered, like the usage of multiple displays, designing dedicated hardware controls for frequent actions.

Learning curve

The learning curve metrics are used to estimate effectiveness of continuous usage of general and advanced procedures of VTS in connection with trainings, and identify areas of expertise to improve:

- Affordance rate: How many procedures are completed before training, after training and after some period of system usage (or extra trainings)

- Experience rate: The ratio between time required to complete the specified task at the first time right after the training, and the time for the same task after some period of system usage — helps how the system supports experienced users and forming of strong habits

- Decay rate: How many tasks from the training operators are able to complete without errors after some time period.

Emotional response

Emotional response highlights the general satisfaction with work environment and tools provided. This is identified by general anonymous questionnaires provided to VTS operators, with closed and open questions like:

- How are you satisfied with the workroom conditions (workstations, design of the interior, etc.)?

- How often you feel negative emotions (including rage) to the system you are working with?

- If you had negative emotions on your workplace recently, please describe them and their cause.

The higher (or less negative) response is observed, the better is, because emotional factors easily lead to errors, issues in communication and effectiveness of operations, accumulation of stress and consequent burn-out.

Additional attention can be added to the aesthetical factor, because there are evidences [13], that the systems, perceived by operators as “ugly” or “fancy” can lead to the drop of usage efficiency.