ANM1 / 10 / 2

RNAV17 / output / 04

Radionavigation Committee

17th Session								19 Sept 2002

GNSS Vulnerability & Mitigation Measures

## Background

A study of the vulnerability of the US transportation infrastructure relying on Global Positioning System (GPS) signals to intentional and unintentional interference was carried out by the U. S. Department of Transportation Volpe Center in 2001. [1]

The increasing reliance on GPS in all types of position finding and navigation, including position and time input to AIS, underlines the importance of an objective consideration of possible areas of vulnerability and a consideration of measures to reduce or mitigate such effects. This applies to the rest of the world just as much as the US and will apply to other satellite systems such as Galileo just as much as GPS. The need for measures to counteract vulnerability will become particularly important with the phasing out of other systems and should be taken into account in the formulation of a Radio Navigation Plan.

## Scope

Although this paper concentrates on navigation applications in the maritime sector, its findings might also be relevant to other transport sectors, specialised positioning and timing applications.

## Reasons for Vulnerability

Some failure modes are common to all types of electronic navigation system. The system itself can fail, for example because of deliberate or accidental damage to the ground infrastructure. Given the military nature of present Global Navigation Satellite Systems (GNSS – GPS & GLONASS), it may be assumed that security levels are high and that standby equipment is provided. Experience with GPS bears this out and system failure can be assumed to be a very rare event. In contrast, failure of the users' receiving equipment is not unusual, either due to power supply failure or a temporary or permanent fault in the receiver itself. The measures to counteract this problem are the same as for other onboard systems, namely standby power supplies and redundancy of equipment.

The reason for the particular vulnerability of GNSS is the extremely low level of the signal, making it susceptible to accidental or malicious interference.

Unintentional sources of interference or interruption in the GNSS service include ionospheric variability, the effects of solar activity, spectrum congestion and adjacent channel intrusion.  Intentional causes of interference include radiation of deliberate broad-band jamming signals, or “spoofing” in which a false GNSS signal is radiated.

### Interference Sources

Ionospheric variability and the effects of solar activity on GNSS are the subject of studies currently underway.

Sources of unintentional interference identified include television broadcasts, microwave communications - both fixed links and satellite uplinks, radars and non-type approved onboard equipment such as active TV antennas. The television broadcasts and microwave fixed links are in some ways the most serious as they can affect all vessels within a significant area or on a particular waterway. Satellite uplinks, radars and other onboard equipment will affect individual vessels and correct installation should eliminate them as interference sources.

### Jamming & Spoofing

Jamming of GNSS signals can be achieved quite easily using relatively low-cost equipment. This is because of the extremely low power levels of the signals at the earth’s surface (minimum -160 dBW). Spread spectrum signals such as that of GPS are in theory less vulnerable to a single frequency jammer, but a broad-band jammer could block out GNSS signals over a significant area. Frequency division systems such as GLONASS could, in theory still give a service when a narrow band jammer affects one or two satellites' signals.

Spoofing is much more difficult to achieve as it is necessary to simulate the signals in order to make the receiver lock on to the false signals. Simulators are sophisticated and expensive pieces of equipment. However, the consequences of spoofing are far more serious than those of jamming. If the false signals are indistinguishable from the real ones and give a position close enough to be believable then the user may not be aware of the deception and could be led into danger.

Given the relative difficulties of jamming and spoofing, jamming is far more likely to be encountered.

The sort of area that could be affected with a simple jammer would not just be a single port approach. GNSS service could be denied over a critical area such as the Straits of Dover.

## Risks/Consequences

Incidence of external and onboard interference to GNSS on ships is relatively easy to assess. It is much more difficult to quantify the risk of events such as jamming and spoofing. A table is presented below giving subjective assessments of the different threats in terms of their risk of occurrence, consequences and the difficulty and cost of mitigation.

Risk / Consequences Table

| Threat | Risk | Consequences | Mitigation difficulty/cost |
| --- | --- | --- | --- |
| GNSS System failure | L | H | H |
| Onboard Power supply failure | H | H | L |
| Onboard Receiver/ antenna failure | M | H | L |
| Onboard interference | M | M | L |
| External interference | L | H | M |
| Ionospheric | L | M | M |
| Jamming | L | H | M |
| Spoofing | L | H | H |

L = Low 	  M = Medium		H = High

## Mitigation

Awareness of the problem and changes in the design of future systems such as greater radiated power, increased receiver sophistication and added operating frequencies can serve to mitigate the impact of some of these effects. However system vulnerability, particularly to deliberate attack, cannot be fully eliminated. This message was clear and repeated several times within the Volpe Report. Modification of the present systems can reduce the effect of natural and inadvertent sources of noise and interference. Calculated attempts to jam or otherwise deny the user community the navigation and timing services of GNSS will be far more difficult to anticipate and combat. Therefore maintenance of adequate alternative systems is essential.

## Alternative Systems

Alternative means of navigation may be provided at various levels. A truly redundant system provides the same functionality as the primary system, allowing a seamless transition with no change in procedures. A backup system ensures continued operation, but not necessarily with the full functionality of the primary system and may necessitate some change in procedures by the user. Contingency systems allow safe completion of a manoeuvre, but may not be adequate for long-term use.

## Redundant Systems

An example of a truly redundant systems would be GPS and Galileo. At present there is no redundant system for GPS, since GLONASS is not fully deployed. It must also be noted that such similar systems may also have common failure modes. For example a jammer or interference source could deny both GPS and Galileo, as could the masking effects of terrain.

## Backup Systems

Backup systems could include terrestrial radionavigation systems such as Loran, or radar and radar aids to navigation. Both have limitations, which prevent them from being used as fully redundant systems.

### Loran

In the case of Loran, coverage is limited to North America, the Far East and Northwest Europe and parts of India and Russia. Loran receivers are not a required fit and very few vessels have them. However, Loran can provide a position input compatible with electronic charts and other onboard systems. In order to make Loran into a backup system throughout the world a large number of additional stations would be needed and this would be hard to justify. However, there may be some parts of the world where the addition of a small number of stations and modernisation of existing stations would provide Loran coverage at a reasonable cost, for example in the Mediterranean and Barents Seas.

### Radar

Radar does not provide a compatible input to an electronic charting system, therefore different procedures are necessary for its use. However, radar is a required fit on SOLAS vessels and radar map-matching techniques are well developed. Radar in combination with Radar Aids to Navigation provides a system of position fixing totally independent of GNSS.  The most serious limitation of radar as a backup is in areas with low-lying, featureless coastlines, such as the Netherlands or the East coast of England. In order to make radar a universal backup, such areas would need to be marked with sufficient radar aids to navigation. The cost of this is difficult to estimate.

### Universal Mobile Telecommunications System (UMTS)

Another possible backup in the future may be positioning using the Universal Mobile Telecommunications System.  This is the third generation cellular system and the first networks are likely to become operational in 2002. One of the main features of 3G cellular will be Location Based Services (LBS) and location of the user will be achieved by GNSS, triangulation on the cellular base-stations or most probably a combination of the two. Deployment of such networks is likely to start with the more densely populated areas and coverage of coastlines is unlikely to be a priority, however, it may be possible to use base-station triangulation as a backup in some areas.

## Contingency Systems

The most obvious contingency system, allowing safe completion of a manoeuvre, is the system of lights and buoys already provided. Reliance on these visual aids as a contingency system is a strong argument for their retention, at least in some form. This function needs to be borne in mind when users and service providers are assessing the continuing need for visual aids.

Another contingency system that could become available is the inertial navigation system (INS). INS has previously been too expensive for non-military vessels, but lower cost devices with acceptable short-term performance are now becoming available. Integrated with a GNSS receiver an INS could provide continuity of service to electronic charts and autopilots, but only for a limited period, depending on the rate of drift. It would be very important that the user was made aware of the change of input source from GNSS to INS as confidence in position accuracy will degrade with time.

## Action Plan

Just as the Volpe Report led to an action plan for the US, this study provides the outline of such a plan for marine navigation generally. This outline plan may be taken into account in national and regional radionavigation plans.

1. Maintain and improve liaison and partnerships between service providers.

2. Encourage the transfer of mitigation technology from the military for civil use.

3. Develop improved receiver performance standards in co-operation with industry.

4. Raise awareness among users about the vulnerability of GNSS.

5. Assess the level of threat from unintentional and intentional interference with GNSS for marine traffic.

6. Determine the level of a) redundant systems b) backup systems & c) contingency systems, required to meet the perceived threat.

7. Assess the currently available and potential systems for maritime navigation.

8. Assess the cost of providing and operating:

- redundant systems

- backup systems

- contingency systems to meet the threat.

9. Identify the most appropriate mix of systems, in terms of capabilities and cost, for the next 10 years and beyond.

## References

[1] John A.Volpe National Transportation Systems Center, Vulnerability assessment of the Transportation Infrastructure relying on the Global Positioning System. Final report, August 2001