IALA ANM Committee,

Working Group 1

Paris / London / Kiel, the 18th December 2007

- IALA ANM Committee; channel design and risk assessment

- Task 1: Develop Guidance (manual) on channel design and a minimum comprehensive mix of A to N, promoting Marine Electronic Highways especially in congested waters

- Your input paper ANM 10/12/01, formerly 9/12/1

“IALA Risk Management Tool – Lateral Distribution of Traffic”

To

Mr. Peter Kent

peter-kent@btconnect.com

Dear Peter,

at first I want to thank you very much – in the name of the whole ANM Committee - for your input paper ANM 9/12/1 “IALA Risk Management Tool – Lateral Distribution of Traffic”, which we discussed at ANM 10 (ANM 10/12/01).

I send this answer to your paper as the task leader for task 1 in working group 1 of the committee.

We found some major aspects in your paper, which we would like to follow in order to develop IALA risk assessment and the minimum comprehensive mix of Aids to Navigation for navigation channels.

- Review and Validation of your attached document and other related IALA papers

Please let me first ask some questions regarding the paper “IALA Risk Management Tool – Risk reduction factors for buoys, leading lights and sector lights”, that you added as Annex 1 to your input paper. If I go through that paper I find that the risk reduction factors are calculated using  the “standard number of buoys”  and assuming a dependence between navigation accuracy and lateral distribution of traffic.

The concept of a “standard  number of buoys” goes back to the IALA paper “Navigation Accuracy by the use of buoys and leading lights in restricted waterways  (SG 3-4-1 Navigation Accuracy for IWRAP.doc)”.

I repeat some figures from this paper:

“The standard number of paired buoys for a section of a channel is twice the length of the section divided by the optimum separation of each pair of buoys.

The standard number of single buoys for a section of a channel is the length of the section divided by the optimum separation of each buoy.

- The optimum separation between paired buoys along a straight section is 3 times the average channel width;

- The optimum separation between paired buoys along a curved section is 2.8 times the average channel width;

- The optimum separation between single buoys along a straight section is 2.1 times the average channel width;

- The optimum separation between single buoys along a curved section is 2 times the average channel width”

These  figures can also be found in the NAVGUIIDE Edition 5 2006 under 3.1.15.1.

For a small waterway of 100 m width I found a navigation accuracy of 10 m according to the paper “Navigation Accuracy by the use of buoys and leading lights in restricted waterways  (SG 3-4-1 Navigation Accuracy for IWRAP.doc)”(Table 1) and a buoy separation distance of 300 m. Regarding Table 3 and 5 of this paper I can also chose a buoy separation distance of 500 m to 600 m, if the mariner can be content with a navigation accuracy of 17 m for that channel.

For a big waterway of 400 m width I found a navigation accuracy of 100 m according to the paper “Navigation Accuracy by the use of buoys and leading lights in restricted waterways  (SG 3-4-1 Navigation Accuracy for IWRAP.doc)”(Table 1) and a standard buoy separation distance of 1200 m. But the navigation accuracy is already fulfilled with a certain percentage fo the standards number of buoys. So, regarding Table 2 and 4 of this paper I should chose a buoy separation distance of 1500 m to 1600 m, if the mariner can be content with a navigation accuracy of 60 m for that channel.

So, after this little test, I find I could trust these figures, if I knew a little bit more about the method how these figures were found.

But in the paper you submitted to ANM 9/10 as an annex, IALA Risk Management Tool – Risk reduction factors for buoys, leading lights and sector lights, on page 1 we find quite different figures:

- “The optimum separation between paired buoys along a straight section is 1.7 times the average channel width;

- The optimum separation between paired buoys along a curved section is 1.9 times the average channel width;

- The optimum separation between single buoys along a straight section is 1.2 times the average channel width;

- The optimum separation between single buoys along a curved section is 1.3 times the average channel width”

As note “i” in this paper we find that the separation distance of paired buoys along a straight section was found from Portsmouth Harbour approach. Which accuracy  was assumed to be required in Portsmouth Harbour approach? What is the maximum ships beam in this 200 yards wide channel?

As note “ii” in this paper we find that the separation distance of paired buoys along a curved section was derived from the idea that the mariner should see at least the next pair of buoys throughout the turn.

But the distance from which a buoy can be seen depends on size, shape and colour, might be rather than on any factor relating to fairway width. And will it be enough to see only one pair of buoys in order to follow a turn? There may be another dependence on the radius of the turn.

So, regarding these questions, the assumptions that the spacing between buoys on a curved section should be greater than on the straight might be not correct.

There might also be a general problem with this approach of the paper “IALA Risk Management Tool – Risk reduction factors for buoys, leading lights and sector lights” that the factors relate purely to channel width. But increase in risk can often be associated with areas where there is no channel as such,  for example where vessels are approaching, converging or crossing at a particular location and increased positional awareness may be necessary.

It was reported that when the IWRAP model was run during the Copenhagen ( Oresund ) seminar that the overall waterway was divided into small sections to make the model fit. As such the use of the tables and formula relate only to that particular section and not the transit between sections or the waterway as a whole.

The overall passage must be considered, taking into account all variables, including types, numbers and constraints of traffic using the waterway, proximity of dangers and particular difficulties with confirming the position of the vessel during all sections of the passage.

It is generally agreed that when a vessel is approaching or is conducting a turn in a channel, positioning becomes more significant and hence physical reference becomes more necessary. It would therefore be logical that in these areas an increase in the number of physical AtoNs or other assistance may be required.

The consequential reduction in overall Risk of grounding or collision by changes to the Physical AtoN, provision of VTS and/or pilot assistance need to be assessed within the quantitative Risk Assessment. This should allow for overall waterway assessment or for transit form one section to another, joining or leaving the section or other outside influences.

ANM Committee would appreciate if you could provide some help for validation of the papers mentioned above before it will promote the use of these papers for Channel Design (A to N s) and Risk Assessment.

- The dependence of channel dimensions, ship dimensions and the role of the PIANC guide on design of approach channels

The Guide for Design of Approach Channels (PIANC Report of Working Group II-30, issued in June 1997)  provides an analytic approach to the determination of Channel Dimensions. The methodology is used in some national guidance for the safe design of commercial shipping channels, for instance in Part 1 (a) “Guidelines for the Safe Design of commercial Shipping Channels” of “Safe Waterways” (A Users Guide to the Design, Maintenance and Safe Use of Waterways) of the Fisheries and Oceans, Canada.

The analytic methodology might help us to find the “reserve” between the ship and the bank of the navigation channel which we could use to determine the required Navigation Accuracy. There is a PIANC working group to review the PIANC Guide. We will be in contact with this working group to develop our channel design (regarding A to N s).

We can use the analytic PIANC methodology for existing channels and compare with their existing mix of A to Ns and we can use it for a simplified channel model in order to determine the mix of A to Ns we would propose.

- The use of AIS data for channel design and for risk analysis

Last not least, Peter, we come to the core message of your input paper: To use AIS data to find the lateral distribution of different vessels in existing channels. This can be described by the Standard Deviation.

Using the findings of analysis of historical AIS data for channel design and risk assessment, we would, as you proposed, like to find the dependencies and interferences between the Standard Deviation of Vessel Traffic and other factors like:

Waterways dimensions

Straight / curved sections

Weather and tidal conditions, water level, currents

Visibility, day / night, winter / summer

Vessel size, type, speed, traffic patterns

Existing Aids to Navigation

Causation factors (ships using space, but only if space is not occupied by other ships)

To find these dependencies and interferences we should follow your suggestion and do even more. We should analyze AIS data from different waterways and different situations. If possible we should find situations where as many factors as possible are constant, and some significant factors are different.

On the Rio de La Plata we have two straight sectors directly connected but with different width. So we should have the same weather and tidal conditions and the same visibility and other meteorological conditions for each passage and the same vessels, but different channel dimensions and different buoy separation distances.

We could define two or more gates for this part of the waterway and compare the standard deviation by analyzing AIS data. Well, I’m still not yet quite sure, if we then are searching for a dependency of the Standard Deviation from the waterway width, the ships beam or the buoy distance or two or more of these three together. Before we go into analyzing AIS data in detail, we need a concept.

The good news is that Argentina followed the IALA Guideline 1050 “On the Management and Monitoring of AIS Information” and collected AIS data for the Rio de La Plata for a certain period. They are waiting for our questions!

Another situation with two straight sectors of different width with different buoyage directly following each other is the “Seekanal Rostock” in Germany. This could also be an interesting place for obtaining relevant results for our questions.

At last, Peter let me thank you very much for your input to ANM and IALA Risk Assessment Tools! I see your questions and your answers to our questions as a start to a very fruitful and important discussion.

In the name of the IALA ANM Committee I wish you Merry Christmas and send you all our best wishes for a Happy and Prosperous New Year and Health and best Luck for you!

Yours sincerely

With my best regards

Hendrik Eusterbarkey

Hendrik Eusterbarkey

Telematics and Traffic Technology Division

Waterways and Shipping Directorates North and Northwest

Postal adress:

Wasser- und Schifffahrtsdirektion Nord

Hindenburgufer 247

24106 Kiel

Germany

Tel. ++49 (431) 3394 5703

E-Mail: HEusterbarkey@wsd-nord.de