Commissioners of Irish Lights

| To: | Inspector and Marine Superintendent | Ref: | Guard duties on wreck “Ice Prince” |
| --- | --- | --- | --- |
|  |  | Date: | 20th January 2008 |

Sir

On the 17th January 2008 at 0500hrs Granuaile took up wreck guarding duties at the site of the sunken merchant vessel Ice Prince. The wreck is located 17 nm NE of the SW entrance to the Casquets traffic scheme in 53 meters of water (CD) and skirts the path of traffic proceeding towards the SW shipping lane.

Although the wreck is sited on the United Kingdom side of the channel the local vessel traffic management scheme is French. Navigational warnings were promulgated from the British and French authorities regarding the position of the wreck and the French VTMS  (Jobourg Radio) was proactive in instructing vessels to give the site a wide berth. Granuaile remained close to the site and transmitted a “Delta” Racon signal to warn vessels of the danger. Granuaile also maintained an active navigational guard on the site and cautioned vessels by VHF that threatened to pass close to the wreck.

Despite the general navigational warnings, physical Racon transmissions and VHF communications from the French VTMS many transiting ships persisted in maintaining courses towards the wreck site.

On the 18th January at approximately 1630hrs the French authorities transmitted from a shore station radio signals that produced four virtual buoys onto the wreck site as observed on the AIS MKD and electronic chart. This action had an immediate positive effect on shipping approaching the site and a subsequent reduction in VHF contacts warning vessels of the wreck site.

One cannot be certain that all the passing ships had electronic charts but it is reasonable to conclude that of those that did their navigators would have observed the diamond symbols and deemed it prudent, if not to interrogate, then at least to avoid them. The signals were not imposed on Radar sets.

Other observations

- The transmissions emanated from a shore station that was at least twenty three miles from the site. As the wreck is equidistant from the British and French coasts this would imply that there is a good chance that a large section of the English Channel could be covered for transmitting virtual buoyage via AIS to wreck sites.

- Not all ships would have electronic charts but of those that do one can be sure that today’s navigator is using it as a prime source to fix his position and to make himself spatially aware of his surroundings particularly with regard to other ships. The contents of the new Marine Notice 316- Maintenance of ECDIS software imply that we are at the initial stages of allowing ships to replace paper charts with approved ECDIS.

- In the case of the Ice Prince the main concern was that deep draft vessels would run foul of the wreck if she was in the upright condition. One would expect deep draft vessels to have an electronic chart on board.

- The information shown on the MKD was very limited and only showed the name and range and bearing of the aid. When the identification field was entered only the MMSI number and the lat and long was given but this could be down to the particular set used on board Granuaile. In any event I do not believe that the virtual buoyage signals received on MKD’s would have had the same effect on navigators as observing the actual diamond symbols on the electronic chart.

- As can be seen from the accompanying chartlet the symbols are less than prominent when compared with the signals shown from nearby vessels. Technology permitting I believe that when using virtual buoys to mark wrecks the actual symbol should be more evident and perhaps with some accompanying text such as the word “DANGER”.

- The information shown on the virtual buoys when selected from the electronic chart was as follows:

Isolated Danger over the wreck position.

| Detail | Entry Received |
| --- | --- |
| Feature | AIS Aid to Nav |
| Source | AIS |
| MMSI No | 992275201 |
| Name | EPAVE- Ice Prince |
| Bearing | Yes |
| Range | Yes |
| CPA | Yes |
| TCPA | Yes |
| Lat | Yes |
| Long | Yes |
| Position | Autonomous |
| EPFS | Surveyed |
| Size | 2m x 2m |
| Type | Floating Isolated Danger |
| Off Position | Entry of “No” |
| Racon Status | No entry |
| Light Status | No Entry |
| Health Status | No Entry |
| Virtual | Yes |
| Assigned Mode | Yes |

East Cardinal

| Detail | Entry Received |
| --- | --- |
| Feature | AIS Aid to Nav |
| Source | AIS |
| MMSI No | 992275202 |
| Name | Ice Prince Card E |
| Bearing | Yes |
| Range | Yes |
| CPA | Yes |
| TCPA | Yes |
| Lat | Yes |
| Long | Yes |
| Position | Autonomous |
| EPFS | Surveyed |
| Size | 2m x 2m |
| Type | Floating Card E |
| Off Position | Entry of “No” |
| Racon Status | No entry |
| Light Status | No Entry |
| Health Status | No Entry |
| Virtual | Yes |
| Assigned Mode | Yes |

East Cardinal 2

| Detail | Entry Received |
| --- | --- |
| Feature | AIS Aid to Nav |
| Source | AIS |
| MMSI No | 992275203 |
| Name | Ice Prince Card E2 |
| Bearing | Yes |
| Range | Yes |
| CPA | Yes |
| TCPA | Yes |
| Lat | Yes |
| Long | Yes |
| Position | Autonomous |
| EPFS | Surveyed |
| Size | 2m x 2m |
| Type | Floating Card E |
| Off Position | Entry of “No” |
| Racon Status | No entry |
| Light Status | No Entry |
| Health Status | No Entry |
| Virtual | Yes |
| Assigned Mode | Yes |

South Cardinal

| Detail | Entry Received |
| --- | --- |
| Feature | AIS Aid to Nav |
| Source | AIS |
| MMSI No | 992275204 |
| Name | Ice Prince Card S |
| Bearing | Yes |
| Range | Yes |
| CPA | Yes |
| TCPA | Yes |
| Lat | Yes |
| Long | Yes |
| Position | Autonomous |
| EPFS | Surveyed |
| Size | 2m x 2m |
| Type | Floating Card S |
| Off Position | Entry of “No” |
| Racon Status | No entry |
| Light Status | No Entry |
| Health Status | No Entry |
| Virtual | Yes |
| Assigned Mode | Yes |

Regards

H.McClenahan

Master ILV Granuaile.