LIAISON NOTE TO ANM 11

Proposal for a revision of the IALA standard for cardinal buoy lights.

# Introduction

During the Floataid workshop in Brest, concerns were expressed about the potential disruption of the rhythmic character of cardinal buoy lights due to buoy motion. It was shown that this disruption could cause the rhythmic characters of cardinal buoys to be indiscernible for long periods. During the workshop, a proposal was made to introduce a combination of coloured flashing lights to replace the existing cardinal mark flashing white characters, colours being more rapidly discernable than rhythmic characters. This proposal was discussed during EEP11 and committee members expressed some concerns. The proposal and the concerns of EEP members are given below.

# Proposal

The speed of modern ships continues to increase with an increasing workload on smaller bridge crews. Consequently it is desirable to minimise the effort required to identify critical AtoNs and thereby reduce the risk of error.

Tests have clearly indicated that character recognition for Cardinal buoys can become very ambiguous due to the buoy motion. A paper presented at the Floataid workshop in Brest on 1 April 2008 demonstrated how a South cardinal and a North cardinal character can be mistaken with a light on a 3m diameter buoy in the presence of buoy motion arising from sea conditions with wind SW 25 knots, tide NE 1.5 knots, depth: 20 metres, scope: 75 metres.

Calculations indicate that probability of detection of the light character with a light character period of 7 sec is 63% in a 21 sec observation period.

However, recognition of the colour of a light is unaffected by buoy motion.

It is therefore suggested that advantage be taken of modern technology with LED-lanterns on cardinal buoys to use colour to as the means of character recognition. IALA has recently considered alternating blue and yellow lights to mark wrecks and it is proposed that a similar concept be used to identify cardinal lights.

Lateral buoys are identified by red and green for port and starboard marks respectively. A consistency of approach could be introduced by using a combination of white/red and white/green to mark the West and East cardinals respectively. South cardinal could be identified with a white/yellow/ green flash with no interval between the individual colours within the flash. The North cardinal remains unchanged. In all cases the white light is retained to mark the light as a cardinal. The proposal is summarised as follows:

| LP | Red, as current standard |
| --- | --- |
| LS | Green, as current standard |
| NC | White, QFl |
| SC | White / Yellow/ Green approx 0.3 sec each, repeating 10 sec |
| EC | White / Green approx 0.3 sec each, repeating 5 sec |
| WC | White / Red approx 0.3 sec each, repeating 5 sec |
| Wreck | Blue / Yellow, as current standard |
| Special mark | Yellow, as current standard |
| Safe water | White with special character as current |

This proposal has the advantage that a mariner can quickly identify cardinal marks by colour combinations, without the need for counting individual flashes and period time. It avoids long and complex identification characters that may be compromised by buoy motion.

This proposal would imply review of the IALA Buoyage System A/B standard.

# Considerations of the Proposal during EEP11

During EEP11, the EEP Committee considered this proposal and had several concerns as follows:

- the use of red and green in cardinal marks would

  - lead to possible confusion with lateral marks, particularly in the event of partial failure

  - cause problems because of differences in the use of lateral colours in Region A and Region B ;

- differential atmospheric scattering of different colours may cause only the dominant colour to be seen, it may also cause a white light to appear yellow;

- yellow is used for special marks of no navigational significance, therefore its use as a cardinal mark is questionable;

- at threshold of perception, green could be confused with white;

- loss of one of the colours due to equipment failure could lead to confusion;

- since the use of red and green could cause confusion with lateral marks, blue is only used for emergency wreck marking and the use of yellow is questionable, white seems to be the only practical colour to use for cardinal buoys;

- concerns were also voiced about the cost of replacing existing lights.

Because of the above concerns, the EEP Committee recommends that this proposal should not be pursued further.

# Further Work

Other options are currently being considered to address the problem of rhythmic character disruption on cardinal buoys. These are:

- use lanterns with very wide vertical divergence (at the expense of reduced range or increased power consumption);

- a combination of white fixed and flashing lights (Japan Coast Guard are experimenting with this concept using pulse-width modulation of LEDs).

- white rhythmic characters with shorter periods and shorter/quicker flashes (the period being within half the roll period of the buoy).  One suggestion for alternative faster rhythmic characters is:

| NC | White, QFl (one of the existing options) |
| --- | --- |
| EC | White Fl 3s |
| SC | White VQFl (2) 3s |
| WC | White VQFl (3) 3s |

10 April 2008